3GPP TSG-RAN Meeting #107	RP-250056

Incheon, Korea, 12–14 March, 2025

Agenda item:	15.2.5

Source:	Samsung

Title:	View on Rel-20 IoT NTN

Document for:	Discussion and Decision

# Introduction

3GPP RAN had a workshop to discuss the candidate items for Rel-20 last December, and RAN Chair summarized the discussion in RP-243292 [1].

For IoT NTN, we observe the strong interest to support voice service in Rel-20 as summarized below [1].

In this contribution, we discuss the possible objective in Rel-20 for the item.

# Discussion and justification

Several companies provided the proposals on the support of voice in IoT-NTN in RAN#106 (e.g., RP-242550, RP-242627, RP-242893, RP-242968 and RP-243127).

From detailed proposals, the following two topics are the main proposals besides impact to SA side:

-	Support of SPS (semi-persistent scheduling) (RAN2/RAN1)

-	Protocol overhead reduction for voice packets (RAN2)

SPS is currently not supported for NB-IoT data transmission, and this can be part of Rel-20 to support the voice over IoT-NTN. In addition, both control plane (CP)-based approach and user plane (UP)-based approach would be beneficial to specify for the flexible implementation of the feature.

However, protocol overhead reduction would have a significant impact on the specification, and we have not found much benefit from overhead reductions. From our assessment, the most important factor to meet the data rate in IoT-NTN for the voice service would be the voice codec which is not the domain of RAN side, so RAN does not need to spend too much effort optimizing the overhead at this stage.

# Detailed objectives

The following can be the objectives for the WI:

-	Support of semi-persistent scheduling for NB-IoT in DL and UL (RAN2/RAN1)

To accommodate the objectives above, 1 TU and 0.5 TU would be needed in RAN2 and RAN1, respectively.

# Reference

RP-243292, RAN Chair’s Summary of 5G-Advanced in Rel-20, RAN Chair, December 2024