3GPP TSG RAN meeting #107	RP-250066

Incheon, South Korea, March 12-14, 2025

## Status Report to TSG

Agenda item:			9.2.1

| WI / SI Name | Study on channel modelling enhancements for 7-24GHz for NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: No | Performance part: No |  | Testing part: No |
| Acronym | FS_NR_7_24GHz_CHmod |  |  |  |  |
| Unique ID | 1020081 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-234018 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: 06/2025 | Core part: n/a | Performance part: n/a | Testing part: n/a |  |
| Overall Completion level | Study Item: 75 % | Core part: n/a | Performance Part: n/a | Testing part: n/a |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur (Primary) | Name | LEE Daewon |
|  | Company | Intel Corporation |
|  | Email | daewon.lee@intel.com |
| Rapporteur | Name | ZHANG Nan |
|  | Company | ZTE |
|  | Email | zhang.nan152@zte.com.cn |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

Conclusion

- To provide measurement data, and/or simulation results, and/or available publications with measurement information for frequencies 7 to 24 GHz to validate/update the channel model.

- For frequency continuity of the channel models, Measurement information outside 7 to 24 GHz is also encouraged

Agreement

The following provides list of modelling parameters for 7 – 24 GHz frequencies that could be further studied for validation. The parameters listed are starting point for further discussions and does not imply the parameters require validation nor imply parameters require updates for 7 – 24 GHz frequencies.

- Antenna modelling parameters (e.g. radiation power patterns, directional gain values, etc.)

- Pathloss

- LOS probability

- O-to-I penetration loss

- Delay spread (mean, variance)

- AoD spread (mean, variance)

- AoA spread (mean, variance)

- ZoA spread (mean, variance)

- ZoD spread (mean, variance)

- ZoD offset

- Angle distribution characteristics (e.g. exponential, Gaussian, Laplacian distributions)

- Shadow fading

- K factor (mean, variance)

- LSP cross correlations

- Delay scaling parameter

- XPR

- Number of clusters

- Number of rays per cluster

- Cluster delay spread

- Cluster ASD

- Cluster ASA

- Cluster ZSD

- Cluster ZSA

- Per Cluster shadowing

- Correlation distances

- LSP correlation type (e.g. site-specific or all correlated)

- Oxygen absorption

- Correlation distance for spatial consistency

- Blockage region parameters/blocker parameters

- Spatial correlation for blockages

- Material properties for ground reflector model

- Spatial consistency model A/B

Conclusion

RAN1 to continue discussion on the need for new modelling parameters/scenarios and modelling procedure. The following modelling parameters/aspects for 7 – 24 GHz frequencies that are currently not available in TR38.901 have been identified by companies in RAN1#116bis. At least the following is for further study, but does not imply parameters/scenarios and modelling procedure are required for 7 – 24 GHz frequencies.

- Intra-cluster K factor

- Random power variability in each polarization

- Addition of SMa deployment scenario

Conclusion

- RAN1 to compile measurement/simulation descriptions from companies into a Tdoc to be added as reference to TR38.901.

  - Rapporteur to update the Tdoc in each meeting based on inputs from companies.

- Rapporteurs to provide a template for the measurement/simulation descriptions capture to RAN1 #117 for initial review and endorsement.

Agreement

To check and review the following results and measurement data provided in RAN1 #117 and RAN1#116bis for further discussion in next RAN1 meeting. R1-2405646 contains the list of data sources for the results and measurements provided in RAN1 #117.

- measurements for penetration loss for various materials, including drywall/wood, clear glass, IRR glass, and concrete

- measurements for pathloss for following scenarios: InH_office LOS, InH-Office NLOS, InF LOS, InF NLOS, UMi LOS, UMi NLOS, UMa LOS, UMa NLOS, [Outdoor courtyard], RMa LOS, RMA NLOS, SMa NLOS

- measurements for polarization for UMa deployment scenario

- measurements for DS for following scenarios: InH-Office LOS, InH-Office NLOS, UMi LOS, UMI NLOS, UMa LOS, UMa NLOS, InF LOS, Inf NLOS.

- Measurements for angular distributions, such as ZOD, ZOA, AOD, AOA for following scenarios: InH, UMi, UMa

- Measurements for number of clusters for following scenarios: InH, UMi

- Simulations for LOS probability for SMa deployment scenario

- Measurement results regarding near-field model for following deployment scenarios: InH-Office LoS, UMa

- Measurement results regarding spatial non-stationarity for following deployment scenarios: UMa, [UE side]

- Simulation results regarding spatial non-stationarity for UMa deployment scenario

Observation

- Some companies provided information that sub-urban deployments cannot be represented by existing deployments in TR38.901 (such as UMi, UMa, RMa).

Conclusion

The following parameters are used as a starting point for aligning companies understanding of channel model parameters related to suburban use cases.

- BS height: [22.5] m

- Layout:	Hexagonal grid, 19 Macro sites, 3 sectors per site, ISD = [1732] m

- Typical building heights: [Up to two floors for residential buildings, up to five floors for commercial buildings]

- UT height: [1.5 or 4.5 m for residential buildings], [1.5/4.5/7.5/10.5/13.5 m for commercial buildings]

- UT distribution: [Uniform horizontally, 70% indoor residential users are on ground floor, 30% are on upper floor]

- FFS: ratio between residential and commercial buildings

- Indoor/Outdoor: [80% indoor and 20% outdoor, FFS on in-car users]

- LOS/NLOS: LOS and NLOS

- Min BS - UT distance(2D): [25] m

Conclusion

- To provide information about motivation and reasons why changes to the channel model are essential.

Agreement

- Further study whether the following parameters for existing deployment scenarios is necessary to be updated:

  - Delay spread, pathloss, penetration loss, AoD/AoA/ZoA/Zod spreads, ZoD offset, Angle distribution characteristics (e.g. exponential, Gaussian, Laplacian distributions), XPR, number of clusters, number of rays per cluster, LSP correlations type (e.g. site-specific or all correlated), UE antenna modeling parameters, K factor (mean, variance)

- Study of updates to other parameters are not precluded and subject to further study.

Agreement

- Further study whether/how to reflect absolute delay between links, or whether/how correlation type of the delay needs to be changed from site-specific to all-correlated type in the model

  - Note: site-specific and all-correlated definitions are provided in TR38.901 Section 7.6.3.4.

  - FFS: impact of ISD on correlation type for the deployment scenario

Agreement

- Further study on correcting the scaling of the angles and other alternative to address angle scaling in TR38.901 Section 7.7.5 to enable accurate desired angle spread.

Agreement

- Further study of whether/how to model the variability of the co- and cross polar powers, in both diagonal and anti-diagonals of the polarization matrix, in the TR 38.901 model.

  - FFS: variability is applied for per ray or per cluster or per link

  - FFS: impact of antenna configurations

  - For example, variability may be random with an i.i.d. zero-mean Gaussian with some standard deviation, via changes to step 9 and eq (7.5-22) and (7.5-28) in clause 7.5 in TR 38.901.

Agreement

- Further study whether/how to model intra-cluster K factor to the TR38.901 models, such as power re-normalization among intra-cluster rays of a cluster so that first intra-cluster ray has K times more average power compared to rest of the intra-cluster rays.

  - FFS: whether same or different intra-cluster K factor is applied for each clusters

  - FFS: which applicable deployment scenarios

Agreement

- Further study whether/how following UE antenna modelling aspects should be considered in the modelling:

  - UE antenna placement, e.g. placement along edges of a rectangle reflecting UE form factor,

  - UE antenna orientation of individual antenna elements, e.g. randomize UE antenna element orientation,

  - Antenna radiation pattern, e.g. consider more realistic antenna patterns, including a phase component, potential reuse the parabolic pattern,

  - Antenna imbalance

- Note: this is only used for calibration.

Agreement

The antenna array is assumed for the near-field study.

Agreement

For the study of near-field channel modelling, at least following aspects should be considered:

- Whether/How to define the near-field region

- The parameters variation for each ray/cluster across different antenna element pairs

Agreement

The following scenarios defined in TR38.901 should be considered for the study/modelling of near-field.

- UMa,UMi, Indoor office and Indoor factory

- FFS: RMa and other new scenarios

Agreement

For the assumption on the aperture size of antenna array, the following is considered as reference for channel model study.

- up to [TBD] m, or  [TBD] lambda for UMi

- up to [TBD] m, or [TBD] lambda for UMa

- up to [TBD] m, or [ TBD] lambda for Indoor office

- up to [TBD] m, or [TBD] lambda for Indoor factory

Agreement

For the near-field channel model:

- The impact of the assumption of wavefront is only considered from the perspective of antenna array.

- The near field for each element within the antenna array is not considered in this SI.

Agreement

For near-field channel model, RAN1 strives to design a unified model to explicitly reflect the new properties of near- and existing properties of far-field under the structure of existing stochastic model TR 38.901.

- FFS: whether the same or different implementations, e.g., procedures/equations, are used for near- and far-field channel realization

Agreement

The near- or far-field condition should be studied for the direct path and non-direct paths between BS and UE.

- The near-/far-field condition for the direct path may be assessed by using the 3D BS-UE distance.

  - FFS: The determination of near-/far-field condition for the non-direct paths

- Note: The direct path is referring to the LoS ray in the TR 38.901 in principle.

- Note: The non-direct paths are referring to the cluster/ray(s) without including LoS ray in the TR 38.901.

Agreement

For near-field channel, if necessary, to model the following antenna element-wise channel parameters of direct path between TRP and UE,

- Angular domain parameters (i.e., AoA, AoD, ZoA, ZoD), Delay, initial phase, Doppler shift, Amplitude

- FFS: Impacts on the polarization

The following options are considered:

- Option-1: Determined by the locations of both TRP and UE.

- Option-2: Determined by the antenna element locations of both TRP and UE

Agreement

The following scenarios defined in TR38.901 should be considered for studying/modelling of spatial non-stationarity

- UMi, UMa, Indoor office and Indoor factory

- FFS: RMa and other new scenarios

Agreement

For the modelling of spatial non-stationarity, at least the following options can be studied to identify the impacted ray/cluster and element-pair link:

- Option 1: Introducing per ray/cluster the visible probability, or visibility region for set of antenna element

- Option 2: Introducing the physical blocker to emulate the blockage impact on the link for each element-pair

- Note: The consistency across antenna elements and across clusters should be guaranteed.

Agreement

For the assumption on the aperture size of antenna array, the following is considered for near-field and spatial non-stationarity channel model study, e.g., simulation/measurement and calibration:

- Up to 1.5 m for UMa with maximum antenna elements in the array is [5k] for single Polarization.

- Up to 1 m for UMi with maximum antenna elements in the array is [2.22k] for single Polarization.

- Up to [0.71] m for Indoor factory with maximum antenna elements in the array is [1.12k] for single Polarization.

- Up to [0.25 (for rectangular antenna array), 0.5 (for linear antenna array)] m for Indoor office with maximum antenna elements in the array is [138, 24] for single Polarization, respectively.

Working Assumption

For the near-field channel modeling, no changes are expected on both value and parameter generation procedure of at least following large-scale parameters in existing TR 38.901:

- Pathloss model, SF, LOS probability

- FFS:DS, ASA, ASD, ZSA, ZSD, K factor

Agreement

For near-field channel, if necessary, to model the following antenna element-wise channel parameters of direct path between TRP and UE,

- Phase

with Option-2 “Determined by the antenna element locations of both TRP and UE”.

Agreement

For near-field channel, if necessary, to model the following antenna element-wise channel parameters of non-direct path between TRP and UE,

- Angular domain parameters (i.e., AoA, AoD, ZoA, ZoD), Delay, phase, Doppler shift, Amplitude

- FFS: Impacts on the polarization

The following options are considered:

- Option-1: The cluster location-based approach, wherein the cluster location is obtained with following alternatives:

  - Alt-1: cluster location is derived based on at least the distance between the BS/UE and clusters.

  - FFS: How to obtain the distance.

  - FFS: Other parameters.

  - Alt-2: cluster location is directly dropped and generated.

- Option-2: The parameter-based approach with following detailed alternatives:

  - Alt-1: Introduce the model of variation rate of parameter over antenna elements.

  - Alt-2: Modelling the variation by taking the existing spatial consistency procedure of TR 38.901 as baseline.

- Option-3: The curvature-based approach.

Agreement

For the modelling of spatial non-stationarity, if necessary, the variation (e.g., reduction) of power for the impacted ray/cluster within the element-pair link should be modelled.

- FFS: The value for power variation

- FFS: Impacts on the phase

Agreement

For the modelling of spatial non-stationarity, if necessary, if visible probability (VP) or visibility region (VR) is adopted, at least the following aspects should be considered for definition of VR/VP:

- Granularity of visible probability or visibility region (e.g., per cluster or per ray)

- Determination of visible probability (e.g., distribution) or visibility region (e.g., size, location)

Agreement

For the modelling of spatial non-stationarity, if necessary, if physical blocker-based approach is adopted, the following aspects should be considered for definition of blocker:

- Blocker size/type:

  - FFS: Additional blocker size/type compared to the Table 7.6.4.2-5 in TR 38.901.

  - FFS: Different blocker sizes/types are considered to emulate the antenna element-wise blockage effect at the BS and UE side

- Blocker location, e.g. distribution of the blocker, relative distance between blocker and BS or UE

- FFS: Number of physical blockers to be considered.

Agreement

To align the understanding of the terminology for channel model study, the following figures are considered as the reference:

- For non-direct path:

- For direct path:

Conclusion

For near-field channel, no changes are expected on the following parameters for direct path.

- Amplitude, polarization matrix

Agreement

To check and review the following results and measurement data provided in RAN1 #118 for further discussion in next RAN1 meeting. R1-2407251 contains the list of data sources for the results and measurements provided in RAN1 #118, RAN1 #117, and RAN1 #116-bis.

Agreement

- Enable necessary model to support “sub-urban macro deployments”. Scenario of interest can be implemented when necessary by one of the following options:

  - Option-1: Define alternate parameters for UMa based on different assumption on building density/heights and corresponding BS and UE height/distributions, ISD, other aspects related to sub-urban macro deployments.

  - Option-2: Define a new scenario, e.g., SMa.

  - FFS parameter commonality and differences between Uma and scenario of interest

  - FFS whether and how to enable frequency continuity beyond 7-24 GHz for scenario of interest

Agreement

- At least for calibration purposes, introduce new UE antenna model that potentially provides updates to following parameters:

  - UE antenna placement

    - E.g. placement along edges of a rectangle reflecting UE form factor.

  - UE antenna orientation

    - E.g. randomize UE antenna orientation

  - Antenna radiation pattern

    - E.g. consider more realistic antenna patterns, including a phase component

    - Consider reusing the parabolic pattern

- Note: not all parameters are necessarily updated from current calibration antenna model.

Agreement

- At least for calibration purposes, for a new UE antenna model, additionally

  - FFS: Antenna imbalance

Observation

- ‘standard’ Glass penetration loss model in TR38.901 seems to be aligned well with measurements in 7-24 GHz conducted by companies.

- Some exceptional cases are wood, concrete and IRR glass penetration losses. It should be noted that assumption of at least material thickness has impact to penetration loss and further alignment of at least material thickness is useful for further validation.

- Note that these are initial observations from RAN1 #118 and does not represent conclusive observations for the SI. RAN1 study and validation is expected to continue.

Conclusion

- Continue study on penetration loss at least for the wood, concrete and IRR glass penetration loss and provide details of experimental setup used for penetration loss measurements.

Observation

- Preliminary study shows pathloss updates may not be needed at least for the following scenarios:

  - InH-Office LOS/NLOS, UMi Street Canyon LOS/NLOS, InF LOS/NLOS, RMa LOS/NLOS

- Note that these are initial observations from RAN1 #118 and does not represent conclusive observations for the SI. RAN1 study and validation is expected to continue.

Conclusion

- Continue study on at least pathloss for the following applicable scenarios, UMa LOS/NLOS.

Observation

- Preliminary study shows delay spread updates may not be needed at least for the following scenarios:

  - InH-Office NLOS

- Preliminary study shows delay spread updates may be needed at least for the following scenarios:

  - UMi LOS/NLOS

- Preliminary study shows different conclusions from different sources on whether delay spread updates are needed at least for following scenarios and further study and validation is needed.

  - InH-Office LOS

  - UMa LOS/NLOS/O2I

- Note that these are initial observations from RAN1 #118 and does not represent conclusive observations for the SI. RAN1 study and validation is expected to continue.

Conclusion

- Continue study on at least delay spread for applicable scenarios, including any frequency dependency analysis of delay spread.

- Companies are encouraged to provide methodology of frequency dependence analysis of delay spread, if performed

Observation

- Preliminary study shows some updates may be needed for azimuth and zenith angular spread for at least following scenarios:

  - InH LOS/NLOS, UMi LOS/NLOS, UMa LOS/NLOS

- Note that these are initial observations from RAN1 #118 and does not represent conclusive observations for the SI. RAN1 study and validation is expected to continue, including frequency continuity aspects.

Conclusion

- Continue study on angular spread for applicable scenarios. The following are preliminary examples for identified scenarios:

| Scenario |  | InH @10 GHz |  | UMi @10 GHz |  | UMa @13 GHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | LOS | NLOS | LOS | NLOS |
| ASD lgASD=log10(ASD/1°) | μlgASD | 1.21 | 1.27 | 1.04 | 1.14 | 1.08 | 1.25 |
|  | σlgASD | 0.18 | 0.14 | 0.2 | 0.07 | 0.21 | 0.3 |
| ASA lgASA=log10(ASA/1°) | μlgASA | 1.29 | 1.5 | 1.19 | 1.37 | - | - |
|  | σlgASA | 0.13 | 0.23 | 0.13 | 0.08 | - | - |

| Scenario |  | InH @10.1 GHz |  | UMi @10.1 GHz |  | UMi @ 10GHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | LOS | NLOS | LOS | NLOS |
| ASD | μlgASD | 8.3 | 24.0 | 16.6 | 22.8 | 36.7 | 17.4 |
|  | σlgASD | 4.8 | 13.7 | 15.8 | 19.3 | 20.4 | 24.0 |
| ASA | μlgASA | 28.4 | 47.6 | 32.8 | 60.2 | 19.2 | 31.7 |
|  | σlgASA | 7.3 | 20.6 | 16.0 | 12.6 | 11.0 | 9.3 |
| ZSD | μlgASD | 10.5 | 6.6 | 6.8 | 7.9 | - | - |
|  | σlgASD | 8.6 | 10.5 | 2.8 | 1.3 | - | - |
| ZSA | μlgASA | 4.4 | 8.3 | 13.5 | 12.6 | - | - |
|  | σlgASA | 1.8 | 6.2 | 3.2 | 3.8 | - | - |

| Scenarios |  | UMa @ 3.5 GHz, 13 GHz, 28 GHz |  |  |
| --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | O2I |
| ASD lgASD=log10(ASD/1) | µlgASD | 0.39 + 0.1114 log10(fc) | 0.83 - 0.1144 log10(fc) | 0.58 |
|  | σlgASD | 0.4 | 0.7 | 0.7 |
| Cluster ASD () in [deg] |  | 1.5 | 1.5 | 1.5 |

| Scenario |  | InH @15 GHz |  |
| --- | --- | --- | --- |
|  |  | LOS | NLOS |
| ASA lgASA=log10(ASA/1°) | μlgASA | 1.57 | 1.78 |
|  | σlgASA | 0.15 | 0.15 |
| ZSA lgASA=log10(ZSA/1°) | μlgASA | 0.94 | 0.94 |
|  | σlgASA | 0.05 | 0.06 |

| Scenario |  | UMa @6.5 GHz |  |
| --- | --- | --- | --- |
|  |  | LOS | NLOS |
| ASD lgASD=log10(ASD/1°) | μlgASD | 0.82 | 1.26 |
|  | σlgASD | 0.28 | 0.27 |
| AOA spread (ASA) lgASA=log10(ASA/1°) | μlgASA | 1.67 | 1.72 |
|  | σlgASA | 0.19 | 0.15 |

Observation

- Preliminary study shows some updates may be needed for number of cluster and/or the threshold for removing lower powered clusters in the channel model for at least following scenarios:

  - InH LOS/NLOS, UMi LOS/NLOS, UMa LOS/NLOS

- Note that these are initial observations from RAN1 #118 and does not represent conclusive observations for the SI. RAN1 study and validation is expected to continue, including frequency continuity aspects.

Conclusion

- Continue study on handling of number of clusters for applicable scenarios.

- The following are preliminary examples for identified scenarios:

  - InH LOS/NLOS number of clusters

    - LOS @ 10 GHz, 500 MHz measurement BW: 15 ⇒ 10

    - LOS @ 10.1 GHz, 500 MHz measurement BW: 15 ⇒ 6

    - NLOS @ 10 GHz, 500 MHz measurement BW: 19 ⇒ 11

  - UMi LOS/NLOS number of clusters

    - LOS @ 10 GHz, 500 MHz measurement BW: 12 ⇒ 5

    - LOS @ 10.1 GHz, 500 MHz measurement BW: 12 ⇒ 8

    - NLOS @ 10 GHz, 500 MHz measurement BW: 19 ⇒ 7

    - NLOS @ 10.1 GHz, 500 MHz measurement BW: 19 ⇒ 10

  - UMa LOS/NLOS number of clusters

    - LOS @ 13 GHz, 400 MHz measurement BW: 12 ⇒ 9

    - NLOS @ 13 GHz, 400 MHz measurement BW: 20 ⇒ 14

- Companies are encouraged to disclose measurement methodology and conditions for determining number of clusters, such as measurement noise floor, capture dynamic range, measurement bandwidth, angular/spatial resolution, etc.

Observation

- 1 source observed existence of dominant angle for each cluster angular power distribution in 13 GHz UMa NLOS scenario. The source noted that dominant angle representation of the channel may be done by introducing an intra-cluster power factor.

- Note that these are initial observations from RAN1 #118 and does not represent conclusive observations for the SI. RAN1 study and validation is expected to continue, including frequency continuity aspects.

- The following is an example of potential changes:

Hu,s,nNLOSt=ICP∙PnICP+1Frx,u,θθn,1,ZOA,ϕn,1,AOAFrx,u,ϕθn,1,ZOA,ϕn,1,AOATexpjΦn,1θθκn,1-1expjΦn,1θϕκn,1-1expjΦn,1ϕθexpjΦn,1ϕϕFtx,s,θθn,1,ZOD,ϕn,1,AODFtx,s,ϕθn,1,ZOD,ϕn,1,AOD

expj2πrrx,n,1T∙drx,uλ0expj2πrtx,n,1T∙dtx,sλ0expj2πrrx,n,1T∙vtλ0

+PnICP+1M-1m=2MFrx,u,θθn,m,ZOA,ϕn,m,AOAFrx,u,ϕθn,m,ZOA,ϕn,m,AOATexpjΦn,mθθκn,m-1expjΦn,mθϕκn,m-1expjΦn,mϕθexpjΦn,mϕϕFtx,s,θθn,m,ZOD,ϕn,m,AODFtx,s,ϕθn,m,ZOD,ϕn,m,AOD

expj2πrrx,n,mT∙drx,uλ0expj2πrtx,n,mT∙dtx,sλ0expj2πrrx,n,mT∙vtλ0

where ICP is the intra cluster power ratio.

Conclusion

- To potentially reflect the channel angular domain sparsity, continue study of unequal intra-cluster power distribution for applicable scenarios.

Observation

- Preliminary study shows LOS probability do not need to be updated for the frequency range of interest for all existing scenarios. Further study is needed on LOS probability if existing scenario building/node deployment statistics are updated or if new deployment scenario(s) are adopted.

- Note that these are initial observations from RAN1 #118 and does not represent conclusive observations for the SI. RAN1 study and validation is expected to continue, including frequency continuity aspects.

Observation

- 3 source observed variability in power between co-polarized and cross-polarized antennas.

  - Among the 3 sources, 1 source observed ground reflection model in 38.901 may be used to introduce the polarization variability.

- 1 source observed variability in power between co-polarized and cross-polarized antennas for fix antenna setup. The source noted that UE relative polarization is not fixed in practice and questioned the applicability of the measurements for deployments of interest.

- 1 source observed that power ratio for co-polarization and cross-polarization is influenced by the depolarization effects of the environment and the antenna assumptions. Furthermore, under ±45° antenna slant assumption, the ratios of cross-polarization received power to co-polarization received power have a consistent mean value and therefore existing polarization modeling in TR 38.901 is sufficient.

Conclusion

- Continue study on handling of polarization for applicable scenarios.

Observation

- Preliminary study shows inconclusive results on whether updates may be needed for shadow fading for pathloss parameters of the channel model.

  - 1 source observed shadow fading values, are frequency-independent based on measurements at 6.75, 16.95, 28, and 73 GHz for InH-Office scenario. The discrepancies in shadow fading values are less than 0.3 dB for LOS and less than 2 dB for NLOS channel conditions across the entire frequency range of 0.5-100 GHz.

  - 1 sourced observed frequency dependency of the shadow fading in UMa LOS/NLOS scenario in 7-24 GHz, which the current channel modeling does not have.

- Preliminary study shows inconclusive results on whether updates may be needed for shadow fading per cluster parameters of the channel model.

  - 1 source observed 3dB reduced per cluster shadowing standard deviation for UMi LOS scenario, 1 dB reduced per cluster shadowing standard deviation for InH LOS scenario.

- Note that these are initial observations from RAN1 #118 and does not represent conclusive observations for the SI. RAN1 study and validation is expected to continue.

Conclusion

- Continue study on handling of shadow fading parameters for applicable scenarios.

Observation:

- Preliminary study shows some updates may be needed for K-Factor parameters of the channel model for at least following scenarios:

  - InH LOS, UMi LOS

- Note that these are initial observations from RAN1 #118 and does not represent conclusive observations for the SI. RAN1 study and validation is expected to continue, including frequency continuity aspects.

Conclusion

- Continue study on handling of K-factor parameters for applicable scenarios. The following are preliminary examples for identified scenarios:

  - InH LOS K-factor

    - Mean 9, std-dev 5 ⇒ Mean 5.1, std-dev 3.2

  - UMi LOS K-factor

    - Mean 7, std-dev 4 ⇒ Mean 8.5, std-dev 3.5

Observation:

- Inconclusive on whether CDL model angle calculation update is needed.

- Note that these are initial observations from RAN1 #118 and does not represent conclusive observations for the SI. RAN1 study and validation is expected to continue, including frequency continuity aspects.

Conclusion

- Continue study on handling of other channel modeling aspects. The following are examples of suggested changes for angle calculation for CDL model by sources:

  - Example 1)

      - ϕn,scaled=ϕn,intermediate-μϕ,intermediate+μϕ,desired (7.7-5)

      - where

      - ϕn,intermediate=ScalingFactor(ASdesired)∙Angleϕn,model-μϕ,model,

      - and

      - Anglex=x,if x is a zenith angleWrapTo180x,if x is an azimuth angle.

      - ScalingFactorASdesired=minx≥0{x: ASx=ASdesired}.

      - ASx=-2lnn=1NexpjxAngleϕn,model-μϕ,modelPnn=1NPn

  - Example 2)

      - ϕn,scaled=ϕn,intermediate-μϕ,intermediate+μϕ,desired, and

      - ϕn,intermediate=s* ϕn,model-μϕ,model, where s is a scale factor to achieve desired angular spread.

Conclusion

- Continue study on handling of channel delays between different UE-TRP links. The following are examples of how absolute delays between different UE-TRP link may be applied in 38.901 provided by companies.

  - Example 1) introduce a new correlation type called “physically consistent” that takes the individual UE-TRP distances into account when generating the link-specific delays.

  - Example 2) Reuse absolute delay modelling in section 7.6.9 in TR 38.901 with extension for other scenarios as follows:

Table 7.6.9-1: Parameters for the absolute time of arrival model

| Scenarios |  | InF-SL, InF-DL | InF-SH, InF-DH | UMi | UMa |
| --- | --- | --- | --- | --- | --- |
| lgΔτ=log10Δτ1s | μlgΔτ | -7.5 | -7.5 | -7 | -6.8 |
|  | σlgΔτ | 0.4 | 0.4 | 0.4 | 0.6 |
| Correlation distance in the horizontal plane [m] |  | 6 | 11 | 15 | 50 |

Agreement

The following assumptions are modeling parameters related to suburban use case. For aspects with multiple options, FFS which option(s) to support.

- BS height:

  - Option 1: 22.5 m

  - Option 2: 20 m – 25 m

  - Option 3: 15 m

  - Option 4: 35 m

  - Option 5: 25 m

- Layout:	Hexagonal grid, 19 Macro sites, 3 sectors per site,

  - Option 1: ISD = 1732 m

  - Option 2: ISD = 500 m

  - Option 3: ISD <= 1000 m

  - Option 4: 500 ~ 1000 m

  - Option 5: 1299 m

  - Option 6: ISD < 500 m

- Building density

  - Option: [373, 440, others] buildings/km2

- UT height: 1.5 or 4.5 m for residential buildings, 1.5/4.5/7.5/10.5/13.5 m for commercial buildings

  - Typical building heights: Up to two floors for residential buildings, up to five floors for commercial buildings

- UT distribution:

  - Option 1: Uniform horizontally, 70% indoor residential users are on ground floor, 30% are on upper floor

  - Option 2: 0.5 m for outdoor; 1.5+3(Nfl-1) m for indoor, where Nfl is the floor and Nfl is uniform (1, 2) for residential buildings or is uniform (1, 5) for commercial buildings. (framework in 36.873 for UMa with changes to distribution of Nfl (number of floors).)

- Indoor/Outdoor:

  - Option 1: 80% indoor and 20% outdoor,

    - FFS: ratio between residential and commercial buildings

    - FFS on in-car users

  - Option 2: 40% indoor in residential buildings, 40% indoor in commercial buildings, and 20% outdoor

- LOS/NLOS: LOS and NLOS

- Min BS - UT distance (2D):

  - Option 1: 25 m

  - Option 2: 10 m

  - Option 3: 35 m

- FFS: Penetration model: low-loss penetration model

- FFS: clutter height parameter, e.g. foliage

Conclusion

Study at least the following channel modelling aspects of suburban use case. The sub-bullets describing the detailed equations of the modelling aspect are examples for consideration:

- Pathloss

  - NLOS: A+B⋅log10d+20⋅log10fc+10⋅log10minfc,10

  - LOS: Reuse pathloss model of UMa scenario in TR 38.901

  - LOS: PLLoS=22.9log10(d3D)+19.6log10f+28.6 dB, σ=3.48 [dB]

  - For LOS and NLOS, reuse Winner II

- LOS probability

  - PrLOS=1d2D-out≤10 mexp-d2D-out-1040010 m<d2D-out

- ASD

| Scenarios |  | SMa |  |  |
| --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | O2I |
| AOD spread (ASD) lgASD=log10(ASD/1°) | µlgASD | 0.55 | 0.55 | 0.55 |
|  | σlgASD | 0.25 | 0.25 | 0.25 |
| Cluster ASD () in [deg] |  | 1.5 | 1.5 | 1.5 |

- ZSD

| Scenarios |  | SMa |  |  |
| --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | O2I |
| ZOD spread (ZSD) lgZSD=log10(ZSD/1°) | µlgZSD | 0.55 | 0.55 | 0.55 |
|  | σlgZSD | 0.25 | 0.25 | 0.25 |

Agreement

Previous agreement made in RAN1#117 is updated as:

For the assumption on the aperture size of antenna array, the following is considered for near-field and spatial non-stationarity channel model study, e.g., simulation/measurement and calibration:

- Up to 1.5 m for UMa with maximum antenna elements in the array is 5k for single Polarization.

- Up to 1 m for UMi with maximum antenna elements in the array is 2.22k for single Polarization.

- Up to 0.71 m for Indoor factory with maximum antenna elements in the array is 1.12k for single Polarization.

- Up to 0.25 (for rectangular antenna array), 0.5 (for linear antenna array) m for Indoor office with maximum antenna elements in the array is 256, 80 for single Polarization, respectively.

Agreement

Confirm the following working assumption made in RAN1#117.

- Working Assumption

  - For the near-field channel modeling, no changes are expected on both value and parameter generation procedure of at least following large-scale parameters in existing TR 38.901:

  - Pathloss model, SF, LOS probability

  - FFS:DS, ASA, ASD, ZSA, ZSD, K factor

Agreement

For the near-field channel modeling, no changes are expected on both value and parameter generation procedure of at least following large-scale parameters in existing TR 38.901:

- DS, ASA, ASD, ZSA, ZSD, K factor

Agreement

The spatial non-stationarity characteristics, i.e., the antenna element-wise power variation at least at BS side, is supported in the channel modelling.

- FFS: the antenna element-wise power variation at UE side

- FFS: the causes and details of methodology for modelling the spatial non-stationarity characteristics

Agreement

For near-field channel, no changes are expected on following parameters of the non-direct path between TRP and UE:

- Polarization matrix

- FFS: Amplitude

Agreement

For near-field channel, the following formula is adopted to model the phase of direct path between TRP and UE as antenna element-wise channel parameter:

exp⁡(-j2πd3Dλ0)exp-j2π|ru,s|-d3Dλ0,

where the ru,s refers to the vector determined by the location of antenna element u and s. The d3D refers to the 3D distance between reference antenna at TRP and UE side.

Agreement

For near-field channel, if necessary, the following parameters of the non-direct path between TRP and UE should be modeled as antenna element-wise parameter.

- Phase

- FFS: Doppler shift, Angular domain parameters, delay

Observation

According to the inputs from multiple sources, partial blockage effect may cause the spatial non-stationarity.

Agreement

For the modelling of spatial non-stationarity, the variation (e.g., reduction) of power for the impacted ray/cluster within the element-pair link should be modelled as:

- If visible probability (VP) or visibility region (VR) is adopted,

- A power attenuation factor within [0 1] is introduced.

- FFS: Details on how to determine the exact value for each cluster

- If physical blocker-based approach is adopted:

- Adopt the existing knife edge attenuation model in TR 38.901 in blockage Model-B to model the power attenuation per element with following update as:

- For each ray/cluster, rotating the blocker to ensure the arrival/departure direction at each Receive/Transmit antenna element is always perpendicular to the screen, respectively.

- FFS: applicability and details for blockage Model-A

Agreement

For the modelling of spatial non-stationarity, if physical blocker-based approach is adopted, the following additional blocker type can be considered for blockage model B:

- Building edge for outdoor scenario

- Small object, e.g., billboard, street lamp, pillar, for either indoor or outdoor scenario

- FFS: UE-side (self-blockage) blocker for both indoor and outdoor scenario

- Details, e.g., blocker types such as Single hand grip, dual-hand grip, and head with one hand grip.

FFS: the number and the location of the blocker between BS and one specific UE.

FFS: applicability and details for blockage Model-A

Agreement

For near-field channel, if necessary, the antenna element-wise channel parameters of non-direct path between TRP and UE can be determined by one of the following candidate options:

- Option-1: The antenna element-wise channel parameters are derived based on at least the distance between the BS/UE and a point associated with cluster.

  - FFS: How to obtain the distance.

  - FFS: Other parameters.

  - FFS: association between a point and a cluster

- Option-2: The antenna element-wise channel parameters are determined based on the existing spatial consistency procedure of TR 38.901 with updates.

  - FFS: Details to obtain the antenna element-wise parameters.

Note: Companies are encouraged to check the Option-3 including the similarity/difference with Option-1.

Agreement

For the modelling of spatial non-stationarity, if visible probability or visibility region is adopted,

- Visible probability or visibility region is modelled per cluster

- FFS Ratio of UEs and clusters that have SNS impact

- Rectangle can be considered as starting point for shape of VR with following alternatives to define the size:

- Alt 1: VR size is defined as number of elements generated by a distribution

- FFS distribution

- Alt 2: VR size is derived based on distance between antenna array of BS and UE/cluster

- Note: Cluster location is required in this alternative

- Alt 3: VR size is randomly generated with a minimum size limit

Conclusion

Investigation and potential update of the propagation characteristics of the channel model for 6-7 GHz are part of the study for this item.

Agreement

For new UE antenna model, antenna placement candidate locations relative to centre of the handheld device is as follows:

- UE antenna modeling (at least for calibration), the UE antennas are placed along the edges of a rectangle reflecting a UE form factor.

- The size of the device can be set to be X cm x Y cm x Z cm

  - FFS (X,Y, Z), e.g., (X,Y,Z) can be15cm x 6 x 0 cm,

- The four corners and the centres of each edge are identified as potential locations of the UE antenna.

- Antennas are assumed to be oriented along the direction determined by the vector connecting the centre of the rectangle to the antenna location

  - FFS: antenna field pattern with respect to the direction of antenna

- FFS: how antenna elements and antenna modules are mapped to antenna location candidates, e.g., for FR1, antenna location candidates are for antenna elements, and for FR2, antenna location candidates are for antenna array modules.

FFS: how to handle polarization aspects of the antenna element/module.

Agreement

For UE antenna radiation pattern modeling, consider the following options for further study:

- Option 1) Adapt the Table 7.3-1: Radiation power pattern of a single antenna element

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) | AE,Vθ″=-min12θ″-90°θ3dB2,SLAV,θ3dB=FFS°,SLAV=FFS dB |
| Antenna element horizontal radiation pattern (dB) | AE,Hϕ″=-min12ϕ″ϕ3dB2,Am,ϕ3dB=FFS°,Am=FFS dB |
| Combining method for 3D antenna element pattern (dB) | A″(θ″,ϕ″)=-min-AE,Vθ″+AE,Hϕ″,Am |
| Maximum directional gain of an antenna element GE,max | FFS dBi |
| Note: max direction gain and half power beam widths should be selected such that total antenna efficiency should be equal or less than 0dBi. |  |

- Option 2) half-wavelength dipole.

  - The normalized E-field radiation pattern of a half-wavelength dipole can be expressed as: E(θ) = cos(cos(θ)*π /2) / sin(θ), where θ is the angle relative to the +Z-axis

    - FFS: range of θ

  - Option 2b) half-wavelength dipole

    - The normalized E-field radiation pattern of a half-wavelength dipole can be expressed as: sin(π/2*cos(θ))

    - range of θ = -90° to +90°

- Option 3) omni-directional antenna pattern

Agreement

The following assumptions are updated modeling parameters related to SMa use case.

| Parameters |  | SMa | Options for FFS |
| --- | --- | --- | --- |
| Cell layout |  | Hexagonal grid, 19 micro sites, 3 sectors per site (ISD = FFS) Up to two floors for residential buildings, up to five floors for commercial buildings. FFS: ratio between residential and commercial buildings | ISD Option 1: 1732 m ISD Option 2: 1299 m ISD Option 3: 1000 m ISD Option 4: 250 m Building Type Ratio Option A: 50% residential, 50% commercial Building Type Ratio Option B: 90% residential, 10% commercial |
| BS antenna height hBS |  | FFS | BS Height Option 1: 35 m BS Height Option 2: 25m BS Height Option 3: 15m |
| UT location | Outdoor/indoor | Outdoor and indoor | - |
|  | LOS/NLOS | LOS and NLOS | - |
|  | Height hUT | 1.5 m for outdoor, 1.5 or 4.5 m for residential buildings, 1.5/4.5/7.5/10.5/13.5 m for commercial buildings | - |
| Indoor UT ratio |  | 80% indoor and 20% outdoor |  |
| UT mobility (horizontal plane only) |  | FFS |  |
| Min. BS - UT distance (2D) |  | FFS | Min Distance Option 1: 20m Min Distance Option 2: 25m Min Distance Option 3: 35m |
| UT distribution (horizontal) |  | Uniform | - |
| UT distribution (vertical) |  | FFS | Vertical Distribution Option 1: 70% indoor residential users are on ground floor, 30% are on upper floor, uniform distribution across all floors for commercial Vertical Distribution Option 2: uniform distribution across all floors for a building type |
| Penetration model |  | FFS: low-loss penetration model | - |

Agreement

Further study and down-select cell layout, BS antenna height, and building type ratio for modeling parameters related to SMa use case. The Following are sets of option combinations prioritized for study and down-selection. Other option combinations are not precluded.

- Alt 1) ISD 1299m + Building Type Ratio 90% residential, 10% commercial + BS Height 35m

- Alt 2) ISD 1000m + Building Type Ratio 90% residential, 10% commercial + BS Height 25m

- Alt 3) ISD 250m + Building Type Ratio 90% residential, 10% commercial + BS Height 25m

- Alt 4) ISD 1299m + Building Type Ratio 90% residential, 10% commercial + BS Height 25m

- Alt 5) ISD 1732 m + Building Type Ratio 90% residential, 10% commercial + BS Height 35m

- Alt 6) ISD 1732 m + Building Type Ratio 90% residential, 10% commercial + BS Height 25m

Agreement

Further study methods of achieving frequency continuity and handling frequency dependency parameters of the channel model.

Observation:

- 4 sources showed wood penetration loss model in TR38.901 seems to with acceptable range of deviation with measurements in 7-24 GHz. 1 source from RAN1 #117 showed wood penetration loss model in TR38.901 deviates from measurement by more than 5 dB for certain frequency of interest within 7 – 24 GHz.

- 3 sources and 1 source from RAN1 #117 showed Concrete penetration loss seems to deviate from measurements by more than 25 dB for certain frequency of interest within 7 – 24 GHz.

- 3 sources show IRR glass penetration loss seems to deviate from measurements for certain frequency of interest within 7 – 24 GHz.

  - It is noted that the spread of measurements from companies could be due to selection of the glass window type (e.g. fixed, sliding, swinging, etc), thickness of the window, and number of panes.

Continue study on penetration loss for Wood, Concrete, and IRR glass taking into account measurement study conducted for the original TR38.901 (available in http://www.5gworkshops.com/5GCMSIG_White%20Paper_r2dot3.pdf)

Agreement

Study possibility of introduction of additional O2I building penetration loss model type that consider the following:

- concrete penetration loss model that is potentially different from existing model

- Note: other aspects are not precluded

Companies are asked to provide information on the need for the different penetration loss model. Companies are encouraged to provide penetration loss per unit thickness (1 cm) of the material.

Observation

- 1 Source suggested updates to UMa NLOS pathloss based on measurement of UMa

- 1 Source suggested updates to UMi LOS and NLOS pathloss based on measurement of UMi

  - Note methodology to obtain the curve fit for measurement data has been questioned.

- For other scenarios InH and RMa, multiple sources reported reasonable alignment of measurements to current model in TR38.901.

Continue study on pathloss for applicable scenarios, UMa NLOS. The following are example of suggests changes:

- UMa NLOS: 13.54 + 38.6 39.09 log10( d3D ) + 20 log10( fc ) – 0.6 (hUT – 1.5)

Observation

- The following delay spread results were observed by 5 sources for UMi and UMa scenarios.

  - The center of the range of the values for a specific source in the figure represent the mean of the log delay spread.

  - The line range of the values for a specific source in the figure represent the standard deviation of the log delay spread.

Conclusion

RAN1 concludes LOS probability is validated for existing deployment scenarios defined in TR38.901.

- Note: this does not preclude introduction of new LOS probability for new scenarios of interest and/or different BS height to be supported for new/existing scenarios.

Observation

- 2 sources observed aligned shadow fading parameters in the frequency ranges of interest for InH LOS and NLOS scenarios.

- 1 source observed (from RAN1 #118) shadow fading parameters to deviate from TR38.901 by more than 1 dB for InH LOS, InH NLOS, and UMi LOS scenarios.

- 2 sources observed no need for updates to shadow fading parameters in the frequency ranges of interest.

Continue discuss on shadow fading parameters for InH and UMi scenarios.

Observation

- 1 source observed frequency dependency of K-Factor for UMi LOS scenario.

- 1 source observed (in RAN1 #118) deviation of mean K-factor by 4 dB for UMi LOS scenario and 1.5 dB for InH LOS scenario.

Continue study of K-factor parameters for InH LOS and UMi LOS scenarios.

Agreement

- Introduce absolute delay for UMi and UMa scenarios.

  - Absolute delay is enabled by re-using the absolute time of arrival for InF scenarios.

  - Details of the absolute delay modeling for UMi and UMa FFS.

  - Note: this does not preclude potential introduction of absolute delay for other scenarios.

  - The following parameters are examples:

- Table 7.6.9-1: Parameters for the absolute time of arrival model

| Scenarios |  | InF-SL, InF-DL | InF-SH, InF-DH | UMi | UMa |
| --- | --- | --- | --- | --- | --- |
| lgΔτ=log10Δτ1s | μlgΔτ | -7.5 | -7.5 | -7 | -6.8 |
|  | σlgΔτ | 0.4 | 0.4 | 0.4 | 0.6 |
| Correlation distance in the horizontal plane [m] |  | 6 | 11 | 15 | 50 |

Observation

- 1 Source observed decorrelation distance for DS and KF for UMi LOS are different from measured results, and possibly have frequency dependency.

Continue study on correlation distances for applicable scenarios.

Agreement

Study the possibility of supporting a < 500 m ISD for UMa

- FFS whether any other modeling parameter is required to be updated

Agreement

Further study LOS probability assumption for SMa

- The following options are example of LOS probability assumption for SMa

| Scenario | LOS probability (distance is in meters) |
| --- | --- |
| SMa | Option1: PrLOS=exp-d2D-outexpα∙hBSβ+γhUEδ - FFS: α, β, γ, δ parameters Option 2: (ITU M.2135-1) PLOS=1, d2D-out≤10exp-d2D-out-10200, d2D-out>10 - BS height of 35m Option 3: PLOS=1, d2D-out≤10exp-d2D-out-10562, d2D-out>10 - For BS height of 25m, and UE height of 1.5m |

Agreement

Further study the following modeling parameters and associated parameter assumption values for SMa:

- Correlation distance for spatial consistency

- Spatial correlation distance

- Blocking region parameters (For Blockage Model A)

- Fast fading parameters

Conclusion

The spatial non-stationarity characteristics at UE side (e.g., due to the impact of hand(s) and/or head), i.e., the antenna element-wise power variation, is supported in the channel modelling in TR 38.901 with potential updates.

Agreement

For near-field channel, if necessary, to model the following antenna element-wise channel parameters of direct path between TRP and UE,

- Angular domain parameters

- FFS: Doppler shift, Delay

with Option-2 “Determined by the antenna element locations of both TRP and UE”.

Note: Angular domain parameters can be optionally modelled.

Conclusion

For near-field channel, no changes are expected on following parameters of the non-direct path between TRP and UE:

- Amplitude

Agreement

For near-field channel, if necessary, the antenna element-wise channel parameters of non-direct path between TRP and UE is modelled by:

- For the non-direct paths, the antenna element-wise channel parameters are derived based on at least the distance between the BS/UE and a point associated with cluster, i.e.,

  - d1 is used to determine the 1st point associated wit h cluster to calculate the parameter at least for BS side;

  - FFS: d2 is used to determine the 2nd point associated with cluster to calculate the parameter at least for UE side;

  - FFS: whether one point is sufficient

  - FFS: the details to generate the d1, d2,e.g.,if the d2 is needed, both d1 and d2 will be generated pair wise.

  - FFS: The ratio of non-direct path which show the phenomenon with spherical wavefront

  - FFS: The determination of the point associated with cluster is conducted in Cluster-level

Agreement

For near-field channel, if necessary, the following parameters of the non-direct path between TRP and UE should be modeled as antenna element-wise parameter.

- Angular domain parameters, Delay (both can be optionally modelled)

  - Working Assumption

    - The existing model in Clause 7.6.2 of TR 38.901 is used to model the delay.

- FFS: Doppler shift

Agreement

For near-field channel, if necessary, to model the following antenna element-wise channel parameter of direct path between TRP and UE,

- Delay (can be optionally modelled)

  - Working Assumption

    - The existing model in Clause 7.6.2 of TR 38.901 is used to model the delay.

with Option-2 “Determined by the antenna element locations of both TRP and UE”.

Conclusion

RAN1 confirms that the modelling of near-field propagation characteristics (i.e., characteristics of spherical wavefront) is taken as an additional modelling component, and the relevant progress (e.g., the agreement(s) with/without “if necessary”) in AI 9.8.2 will be captured in TR 38.901.

Agreement

For the modelling of spatial non-stationarity, if physical blocker-based approach is adopted, at least for blockage model B, the following new blocker type/size can be introduced in the Table 7.6.4.2-5 in TR 38.901:

|  | Typical set of blockers | Blocker dimensions | Mobility pattern |
| --- | --- | --- | --- |
| Outdoor | Billboard | Cartesian: w=2.4m; h=3.6m | Stationary |
| Outdoor | Street lamp | Cartesian: w=0.4m; h=0.8m | Stationary |
| Outdoor | Building edge | Cartesian: w=X m; h=Y m | Stationary |
| Indoor | Pillar | Cartesian: w=0.3m; h=3m | Stationary |

- FFS: the value of X and Y for the blocker dimensions of building edge is needed.

- FFS: the details related to the user hand/head:

| Indoor;Outdoor | FFS: User hand | Cartesian: w=[0.2]m; h=[0.1]m | Stationary |
| --- | --- | --- | --- |
| Indoor;Outdoor | FFS: User head | Cartesian: w=[0.24]m; h=[0.20] m | Stationary |

- FFS:The details of blockage model B to implement the impact of user hand and head.

- FFS: The location of the user hand/head

Agreement

For the modelling of spatial non-stationarity at BS side, if physical blocker-based approach is adopted, for blockage model B, the procedure to determine the number of blockers and locations are same as existing blockage model B.

For the modelling of spatial non-stationarity at UE side, if blocker-based approach is adopted, an attenuation per antenna element is introduced, the following options can be considered with down-selection by RAN1#120:

- Option-1: generated by leveraging the existing blockage Model-B with potential updates.

  - At most 2 hand type blockers and one head type blocker for a specific UE is assumed.

- Option-2: generated by leveraging the existing blockage Model-A with potential updates

- Option-3: generated by a distribution or a fixed value due to the hand grip and/or head proximity

FFS: the details, e.g., how to determine the value of loss for the case as one hand grip, a dual hand grip and/or head proximity

Note:

To check and review the following results and measurement data provided in RAN1 #119 for further discussion in next RAN1 meeting. R1-2410631 contains the list of data sources for the results and measurements provided in RAN1 #119, #118-bis, #118, #117, and #116-bis.

Agreement

For suburban scenario, adopt the following assumption for calibration purposes:

- Building type ratio: 90% residential, 10% commercial

Agreement

For suburban scenario, adopt the following assumption for calibration purposes:

- UT Vertical Distribution Option 2: uniform distribution across all floors for a building type

Agreement

For suburban scenario, adopt the following assumption for calibration purposes:

- UT mobility in horizontal plane only: Indoor UTs: 3 km/h, outdoor UTs: 40 km/h

Agreement

For handheld devices adopt the following device dimensions for UE antenna modeling:

- 15 cm x 7 cm x 0 cm

Conclusion

For any new channel modeling parameter updates, replace the existing parameters values (whenever applicable). For parameters that were replaced TR should add information that the parameter value/model equation/procedure was changed in Rel-19.

- Note: If deemed important, it should be still possible to discuss how specific channel model parameters need to be updated.

Conclusion

- Based on measurement data provided, RAN1 concludes pathloss models in TR38.901 for the following scenarios are validated and no updates to TR are made.

  - InH-Office LOS

  - InH-Office NLOS

  - RMa LOS

  - RMa NLOS

  - InF LOS

  - InF NLOS

Observation

- For the following scenarios, some sources have observed some differences in delay spread in the TR and measurement taken for 6 – 24 GHz frequency range. Other sources have observed delay spread consistent with the model in the TR.

  - InF LOS

  - InF NLOS

Conclusion

- For the following scenarios, there is no consensus to update delay models due to lack of consistent and significant observed difference between model and measurements.

  - InF LOS

  - InF NLOS

Agreement

- Study further on the need for introduction of intra-cluster power profile.

  - Potential options for introduction of intra-cluster power profile:

    - Introduce unequal power per ray within a cluster and change the unequal angle offset to equal angle offset among rays within a cluster to maintain the same PAS characteristic as in TR 38.901 (example in R1-2409724)

    - Power scaling within a cluster based on DS associated with the cluster (example in R1-2410213)

Agreement

Further study penetration loss modeling, including the following aspects:

- Need for change and details of the updates to the material penetration loss, for example, if any

|  | Concrete | Wood |
| --- | --- | --- |
| Existing TR | Lconcrete=5+4f | Lwood=4.85+0.12f |
| New Updates | Lconcrete=α∙(5+4f) α=Tconcrete23cm | Lwood=α∙(4.85+0.12f) α=Twood6cm |
| Tconcrete and Twood represent reference (default) thickness of the material. For concrete and wood, the reference thickness are 23 cm and 6 cm, respectively. |  |  |

  - Whether penetration loss can be scaled linearly with thickness for each material

  - For standard multi-pane glass, FFS whether and how to reflect the penetration loss resonation effect, which may relate to thickness of the glass.

  - FFS the impact of updates to material penetration loss to total build penetration loss

- Need for change and details of total building penetration loss, e.g. low-loss and high-loss model, if any

- Impact of material composition on material penetration loss

Agreement

- For the following scenarios, further study path loss model parameters for at least 6 – 24 GHz frequency range.

  - UMi LOS

  - UMi NLOS

  - UMa LOS

  - UMa NLOS

Agreement

- For the following scenarios, further study delay spread parameters for at least 6 – 24 GHz frequency range.

  - UMi LOS

  - UMi NLOS

  - InH-Office LOS

  - InH-Office NLOS

  - UMa LOS

  - UMa NLOS

Agreement

- Based on measurement data provided, RAN1 concludes angular spread parameters in TR38.901 that following scenarios are validated and no updates to TR are made.

  - UMa LOS: ZSD

  - UMa NLOS: ZSD

Observation

- For the following scenarios, some sources have observed some differences in angular spread in the TR and measurement taken for 6 – 24 GHz frequency range. Other sources have observed angular spread consistent with the model in the TR.

  - InH LOS and NLOS: ZSA, ZSD

  - UMi LOS: ZSA, ZSD

  - UMi NLOS: ZSD

  - UMa LOS: ZSA

Conclusion

- For the following scenarios, there is no consensus to update angular spread models due to lack of consistent and significant observed difference between model and measurements.

  - InH LOS and NLOS: ZSA, ZSD

  - UMi LOS: ZSA, ZSD

  - UMi NLOS: ZSD

  - UMa LOS: ZSA

Agreement

- For the following scenarios, further study angular spread parameters for at least 6 – 24 GHz frequency range.

  - InH LOS/NLOS: ASA, ASD

  - UMi LOS: ASA, ASD

  - UMi NLOS: ASA, ASD, ZSA

  - UMa LOS: ASD, ASA

  - UMa NLOS: ASA, ASD, ZSA

  - InF LOS and NLOS: ASD, ASA, ZSA, ZSD

Agreement

- Further study number of clusters for existing deployment scenarios (InH, InF, UMi, UMa, RMa) taking into consideration the following:

  - Consistency/continuity across frequencies below 6GHz and above 24 GHz, including whether to update values for other frequencies based on measurements

  - Whether changes to number of clusters result in meaningful impact to system performances and evaluation of features

Conclusion

- Based on measurement data provided, RAN1 concludes shadow fading parameters in TR38.901for the following scenarios are validated and no updates to TR are made.

  - InH LOS

  - InH NLOS

  - UMi NLOS

Observation

- For the following scenarios, some sources have observed some differences in shadow fading in the TR and measurement taken for 6 – 24 GHz frequency range. Other sources have observed shadow fading with the model in the TR.

  - UMi LOS

Conclusion

- For the following scenarios, there is no consensus to update shadow fading due to lack of consistent and significant observed difference between model and measurements.

  - UMi LOS

Agreement

- Further study the need for updating the K-factor parameters for existing deployment scenarios (InH, InF, UMi, UMa, RMa) taking into consideration the following:

  - Consistency/continuity across frequencies below 6 GHz and above 24 GHz, including whether to update values for other frequencies based on measurements.

  - Whether changes to K-Factor result in meaningful impact to system performances and evaluation of features.

Working Assumption

- Adopt the following absolute delay for UMi and UMa scenarios.

Table 7.6.9-1: Parameters for the absolute time of arrival model

| Scenarios |  | UMi | UMa |
| --- | --- | --- | --- |
| lgΔτ=log10Δτ1s | μlgΔτ | -7.5 | -7.4 |
|  | σlgΔτ | 0.5 | 0.2 |
| Correlation distance in the horizontal plane [m] |  | 15 | 50 |

Agreement

- Introduce absolute delay for InH scenarios.

  - Absolute delay is enabled by re-using the absolute time of arrival for InF scenarios.

  - FFS details of the parameter for InH. Consider the following for further study.

Table 7.6.9-1: Parameters for the absolute time of arrival model

| Scenarios |  | InH |
| --- | --- | --- |
| lgΔτ=log10Δτ1s | μlgΔτ | Option 1) -7.9 Option 2) -8.6 |
|  | σlgΔτ | Option 1) 0.3 Option 2) 0.1 |
| Correlation distance in the horizontal plane [m] |  | 10 |

Agreement

- To address the angle-scaling issue when generating CDL channels for link-level evaluations, adopt the following two-step approach to compute the scaled angles.

  - ,

  - is the mean angle of the intermediate ray angles, calculated using the definition in Annex A.

  - is a function which wraps an azimuth angle to the half-open interval (-180, 180].

- Down-select among options.

  - Option 1)

    - ϕn,intermediate=s* WrapTo180ϕn,model-μϕ,model,

    - where s is a scale factor chosen to change the distribution of the angles (up to each company to provide information, e.g. s=ASdesiredASmodel).

  - Option 2a)

    - ϕn,intermediate=s* WrapTo180ϕn,model-μϕ,model,

    - s=minx≥0{x: ASx=ASdesired}.

    - ASx=-2lnn=1NexpjxAngleϕn,model-μϕ,modelPnn=1NPn

  - Option 2b)

    - ϕn,intermediate=s* WrapTo180ϕn,model-μϕ,model,

    - s is chosen such that AS(ϕn,intermediate) = AS_desired

    - ASx=-2lnn=1NexpjxAngleϕn,model-μϕ,modelPnn=1NPn

  - Option 3)

    - ϕn,intermediate=ASdesiredASmodel* WrapTo180ϕn,model-μϕ,model,

    - Note: The resulting scaled ray angle may or may not achieve the desired angle mean and the desired rms angle spread.

- FFS whether and how to enable specular rays of CDL-D and CDL-E to be in the direction of the desired vector.

Agreement

For suburban scenario, further consider following options and down-select a value for BS height assumption for calibration purposes:

- Option 1) BS height = 25m

- Option 2) BS height = 35m

Note: It should be understood that BS height selected here is for calibration purposes and the channel model developed for suburban macrocell scenarios may be applicable for a range of BS heights [25 – 35]m.

Agreement

For suburban scenario, further consider following options and down-select a value for ISD assumption for calibration purposes:

- Option 1) ISD = 1299 m

- Option 2) ISD = 1732 m

Note: It should be understood that ISD selected here is for calibration purposes and the channel model developed for suburban macrocell scenarios is applicable for a range of ISD [1200 – 1800]m.

Agreement

For suburban scenario, further study the LOS probability equation.

- Example of LOS probability equation for study:

  - PrLOS=exp-d2D-out-dmexpα∙hBSβ+γhUEδ, where dm, α, β, γ, δ are modeling parameters

- Study includes whether distinction of LOS probability should be made for UEs in residential and commercial buildings, which have different building heights.

- Study includes whether or not vegetation should be explicitly considered and modeled for determining LOS probability, if determined to be explicitly modeled, how to model vegetation impact, e.g. reuse of InF clutter to change the LOS probability.

Agreement

Further study the need of additional low loss outdoor-to-indoor (O2I) building penetration loss model for suburban scenario.

- Potential example of the new low loss O2I building penetration loss:

PLtw=5-10log100.3⋅10-Lglass10+0.7⋅10-Lwood10

Agreement

- UMa can also be used for other ISDs between 200m and 500m, including the ISD proposed in ITU-R M.2412 -0

- One ISD value will be selected for calibration with ISD = 500m

- No change to any other parameters due to other ISD values

- FFS: How to reflect it in TR38.901

Agreement

Capture channel modeling updates regarding suburban macrocell scenario as follows:

- Define a new scenario, e.g., SMa

- The key differentiating aspects of SMa (compared to UMa and RMa) in high level description will be highlighted in Section “6.2 Scenario of Interest” of the TR.

Agreement

Further study device dimension and antenna placement applicability for customer premise equipment (CPE) devices, include the need for defining CPE device type dimensions.

Agreement

For UE antenna modeling of handheld devices,

- Further study on default antenna placement locations for calibration purpose for 2, 3, 4, 6, and 8 antenna elements at least for FR1 and 6-24 GHz, including the need to define default antenna placement locations, and applicability of frequency ranges, e.g. FR1, FR2.

  - FFS on handling of antenna array panels

Agreement

For handheld UE device type antenna radiation pattern, further study details of antenna field pattern orientation with respect to the direction of antenna for each antenna candidate locations.

- For example, maximal gain directivity of the antenna radiation pattern to be aligned with direction of the antenna candidate location.

Agreement

For near-field channel, the following formula is adopted to model the angular domain parameters of direct path between TRP and UE as antenna element-wise channel parameter:

Frx,u,θθLOS,ZOA,u,s,ϕLOS,AOA,u,sFrx,u,ϕθLOS,ZOA,u,s,ϕLOS,AOA,u,sT100-1Ftx,s,θθLOS,ZOD,u,s,ϕLOS,AOD,u,sFtx,s,ϕθLOS,ZOD,u,s,ϕLOS,AOD,u,s

Where θLOS,ZOA,u,s, ϕLOS,AOA,u,s, θLOS,ZOD,u,s, ϕLOS,AOD,u,s are the respective antenna element-wise elevation arrival angles, azimuth arrival angles, elevation departure angles and azimuth departure angles of LoS path between the transmit antenna element s and receive antenna element u.

Working Assumption

- For the modelling of spatial non-stationarity at BS side, if physical blocker-based approach is adopted, the nearest K blockers from BS are selected.

- For the modelling of spatial non-stationarity at BS side, if physical blocker-based approach is adopted, at least for blockage model B, the following parameters to define the building edge are introduced in the Table 7.6.4.2-5 in TR 38.901:

|  | Typical set of blockers | Blocker dimensions | Mobility pattern |
| --- | --- | --- | --- |
| Outdoor | Building edge | Cartesian: w=50 m; h=20 m | Stationary |

- In the blockage model B, the following equation is used to calculate the attenuation caused by the building edge:

  - LdB=20log100.5-F, where F represents one of Fh1, Fh2, Fv1, and Fv2 defined in equation 7.6-29 in section 7.6.4.2 of TR 38.901.

Agreement

For the modelling of spatial non-stationarity at BS side, if stochastic based approach, at least for unified visible probability (VP) and visibility region (VR), is adopted, the following steps are considered:

- Step 1: Determine whether a cluster is impacted by SNS

- Step 2: Generate the visibility region

- Step 3: Calculate the power attenuation factor

FFS: the details of each step

Note: Merge of some of the steps is not precluded

Working Assumption

For the modelling of spatial non-stationarity at UE side mainly due to close proximity, if blocker-based approach is adopted, the Option-3 is supported with following details:

- Three typical cases are introduced: one hand grip, dual hand grip, head and one hand grip.

  - FFS: probability of each case in the simulation.

- For each case, to model the attenuation for each of antenna in the candidate antenna location defined in the UE antenna model for handheld device with one of following alternatives:

  - Alt 1: Introduce a fixed value for each candidate antenna position per case.

  - Alt 2: Introduce a distribution for candidate antenna position.

    - FFS: distribution per antenna position(s) and per case.

Agreement

For near-field channel, to generate the channel parameter, the reference point is defined as:

- The physical center of the antenna array/center of the device.

Conclusion

For near-field channel, no changes are expected on following parameter for direct path between TRP and UE:

- Doppler shift

Conclusion

For near-field channel, no changes are expected on following parameter for non-direct path between TRP and UE:

- Doppler shift

Agreement

For near-field channel, the following formula is adopted to model the angular domain parameters of non-direct path between TRP and UE as antenna element-wise channel parameter:

Frx,u,θθn,m,ZOA,u,ϕn,m,AOA,uFrx,u,ϕθn,m,ZOA,u,ϕn,m,AOA,uTexpjΦn,mθθkn,m-1expjΦn,mθϕkn,m-1expjΦn,mϕθexpjΦn,mϕϕFtx,s,θθn,m,ZOD,s,ϕn,m,AOD,sFtx,s,ϕθn,m,ZOD,s,ϕn,m,AOD,s

where θn,m,ZOA,u,ϕn,m,AOA,u, θn,m,ZOD,s,ϕn,m,AOD,s are the ray-wise angular domain parameters of ray m cluster n between the transmit antenna element s and receive antenna element u.

Agreement

For the modelling of spatial non-stationarity at BS side, if physical blocker-based approach is adopted, the rotation and power variation calculation are conducted in ray level.

Agreement

For near-field channel, for the non-direct paths, the distance d1 between the BS and the 1st point associated with cluster is generated with one of following options that to be selected in RAN1#120:

- Option-1: The distance d1 is generated directly following a specific distribution.

  - For the k1 clusters, d1 = speed of light times the absolute delay of the cluster

  - For other k2 clusters, the distance  d1 is equal or less than the speed of light times the absolute delay of the cluster and generated following a specific distribution

    - FFS: The specific distribution, e.g., log-normal distribution for UMi

    - FFS: The values of k1 and k2

  - The generation of absolute delay of the cluster is according to the procedure defined in Section 7.6.9 in TR 38.901

    - FFS: The value for UMi, UMa, InH-Office.

- Option-2: The distance d1 is generated by a scaling factor multiplied by the absolute distance corresponding to the absolute delay of the cluster, where the scaling factor is generated from a specific distribution.

  - For the k1 clusters, d1 = speed of light times the absolute delay of the cluster (i.e., the scaling factor is 1)

  - For other k2 clusters, the distance  d1 is equal or less than the speed of light times the absolute delay of the cluster and the scaling factor generated following a specific distribution (i.e., the scaling factor is equal or less than 1)

    - FFS: Distribution of scaling factor, e.g., Beta distribution for UMa

    - FFS: The values of k1 and k2

  - The generation of absolute delay of the cluster is according to the procedure defined in Section 7.6.9 in TR 38.901

    - FFS: The value for UMi, UMa, InH-Office.

Note 1: If d2 is agreed to be considered, the generation of d2 can follow the same principle with the generation of d1.

Working Assumption

For near-field channel, for the non-direct paths, the impact of spherical wavefront is optionally considered at UE side.

Agreement

For near-field channel, in principle, one of following options is considered to calculate the antenna element-wise delay of direct path between TRP and UE:

- Option-1: The following formula is used to calculate the antenna element-wise delay of direct path:

τ1,u,s=τ1-d3D-|ru,s|c

where ru,s refers to the vector determined by the location of receive antenna element u and transmit antenna element s, d3D refers to the 3D distance between reference antenna at TRP and UE side.

  - The above formula is applied with the same principle as existing model in Clause 7.6.2 of TR 38.901, and the channel response of LOS ray between Rx antenna u and Tx antenna s at delay τ at time t is given by:

Hu,s,1LOSt;τ=Frx,u,θθLOS,ZOA,u,s,ϕLOS,AOA,u,sFrx,u,ϕθLOS,ZOA,u,s,ϕLOS,AOA,u,sT100-1Ftx,s,θθLOS,ZOD,u,s,ϕLOS,AOD,u,sFtx,s,ϕθLOS,ZOD,u,s,ϕLOS,AOD,u,s

exp-j2πd3Dλfexp-j2πru,s-d3Dλfexpj2πrrx,LOST∙vλ0tδτ-τ1,u,s

    - The part of the equation above, exp-j2πd3Dλfexp-j2πru,s-d3Dλf, is working assumption

- Option-2: The antenna element-wise delay of direct path is directly modeled based on the distance of direct path between antenna elements at TRP and UE side.

  - Following formula is adopted to model the antenna element-wise delay in the channel coefficient as:

δτ-τ1,u,s,  τ1,u,s=τ1-d3D-|ru,s|c

where ru,s refers to the vector determined by the location of receive antenna element u and transmit antenna element s, d3D refers to the 3D distance between reference antenna at TRP and UE side.

- Option-3: Confirm the following working assumption made in RAN1#118BIS:

  - “The existing model in Clause 7.6.2 of TR 38.901 is used to model the delay.”

- Note: τ1 is same as that is defined in 7.5-30 in TR38.901

Conclusion

- Based on measurement data provided, RAN1 concludes pathloss parameters in TR38.901 that following scenarios are validated and no updates to TR are made.

  - UMi LOS/NLOS

Agreement

- Further study whether/how to handle the LOS pathloss convergence regardless of the frequency band beyond the breakpoint distance phenomena.

Conclusion

- Based on measurement data provided, RAN1 concludes that following angular spread parameters for 6 – 24 GHz frequency range are validated and no updates to TR are made.

  - UMa LOS: ZSD

  - UMa NLOS: ZSD

Conclusion

- For the following scenarios, there is no consensus to update angular spread models due to lack of consistent and significant observed difference between model and measurements.

  - InH LOS : ASD, ASA

  - InH NLOS: ASD, ASA

  - UMi LOS: ASD

  - UMi NLOS: ASD, ZSA

Agreement

- Confirm working assumption (made in RAN1#119) for proposed absolute delay parameters for UMi and UMa scenarios.

Table 7.6.9-1: Parameters for the absolute time of arrival model

| Scenarios |  | UMi | UMa |
| --- | --- | --- | --- |
| lgΔτ=log10Δτ1s | μlgΔτ | -7.5 | -7.4 |
|  | σlgΔτ | 0.5 | 0.2 |
| Correlation distance in the horizontal plane [m] |  | 15 | 50 |

Agreement

Adopt the following description for suburban scenario:

- In suburban macro-cells base stations are located above the surrounding environment to allow wide area coverage, and mobile stations are outdoors at street level and within commercial and residential buildings. Buildings are typically low residential detached houses with one or two floors, or blocks of apartments/condos or commercial buildings with a few floors. Occasional open areas such as parks or playgrounds between the houses make the environment rather open. Streets do not form urban-like regular strict grid structure.

- FFS: vegetation

Agreement

For suburban scenario, use the BS height and ISD assumption for calibration purposes:

- BS height = 35 m

Agreement

For suburban scenario, adopt the following assumption for calibration purposes:

- Minimum BS to UT distance in 2D:

  - 35m

Agreement

Assume enumeration of candidate antenna placement locations as follows:

Agreement

For calibration, the antenna placements for 4 elements of handheld devices are defined as follows:

Agreement

Define candidate antenna locations for CPE devices

FFS: Device dimensions for CPE antenna modelling.

Agreement

- For cases when a candidate antenna placement location is used for one antenna field pattern (i.e., not used for two distinct antenna polarization field patterns):

- Reference radiation pattern of the UE antenna model is vertically polarized with all the gain in the theta filed component,

  - Fθ″″θ″,ϕ″=A″θ″,ϕ″ and Fϕ″″θ″,ϕ″=0.

- For cases when a candidate antenna placement location is used for two distinct antenna polarization field pattern:

  - Reference radiation pattern of the UE antenna model is FFS,

    - For first antenna field pattern: Fθ″″θ″,ϕ″=FFS and Fϕ″″θ″,ϕ″=FFS.

    - For second antenna field pattern: Fθ″″θ″,ϕ″=FFS and Fϕ″″θ″,ϕ″=FFS.

- Each polarized field component of the reference radiation pattern Fθ″″θ″,ϕ″ and Fϕ″″θ″,ϕ″ should be rotated according to the orientation of the each of UE antennae to get Frx,u,θ''(θ',ϕ'), Frx,u, ϕ''(θ',ϕ') and based on the orientation of the UE in the global coordinate system to get Frx,u,θ(θ,ϕ) and Frx,u,ϕ(θ,ϕ) using the methods already specified in TR 38.901, equation (7.3-3)/(7.1-11).

Agreement

- Further study and consider update of power angular spectrum (PAS) of the current cluster modeling from equal powered angle offset to unequal power angle offsets.

  - For the update of power angular spectrum (PAS) of the current cluster modeling from equal powered angle offset to unequal power angle offsets. Consider the following options:

    - Option 1) introduce the update as an optional additional modeling component

    - Option 2) update the existing intra-cluster power distribution and angles

  - Consider power adjustments made by “large bandwidth and large antenna array” additional component and review whether measurement observed can be replaced and modeled by large bandwidth and large antenna array” model.

  - The following are examples of changes

Example 1)

Finally add offset angles m from Table 7.5-3 to the cluster angles , (7.5-13) where cASA is the cluster-wise rms azimuth spread of arrival angles (cluster ASA) in Table 7.5-6. Table 7.5-3: Ray offset angles within a cluster, given for rms angle spread normalized to 1 Ray number m; Basis vector of offset angles m; Ray power ratio βm; Basis vector of offset angles m. 1,2; ± 0.0447; 0.1193; ± 0.1134. 3,4; ± 0.1413; 0.0933; ± 0.3403. 5,6; ± 0.2492; 0.0729; ± 0.5671. 7,8; ± 0.3715; 0.0570; ± 0.7940. 9,10; ± 0.5129; 0.0446; ± 1.0208. 11,12; ± 0.6797; 0.0348; ± 1.2477. 13,14; ± 0.8844; 0.0272; ± 1.4745. 15,16; ± 1.1481; 0.0213; ± 1.7014. 17,18; ± 1.5195; 0.0166; ± 1.9282. 19,20; ± 2.1551; 0.0130; ± 2.1551. <unchanged text omitted> Hu,s,nNLOSt=PnMm=1MβmFrx,u,θθn,m,ZOA,ϕn,m,AOAFrx,u,ϕθn,m,ZOA,ϕn,m,AOATexpjΦn,mθθκn,m-1expjΦn,mθϕκn,m-1expjΦn,mϕθexpjΦn,mϕϕ Ftx,s,θθn,m,ZOD,ϕn,m,AODFtx,s,ϕθn,m,ZOD,ϕn,m,AODexpj2πrrx,n,mT∙drx,uλ0expj2πrtx,n,mT∙dtx,sλ0expj2πrrx,n,mT∙vλ0t (7.5-22)

Example 2)

Finally add offset angles m from Table 7.5-3 to the cluster angles , (7.5-13) where cASA is the cluster-wise rms azimuth spread of arrival angles (cluster ASA) in Table 7.5-6. Table 7.5-3: Ray offset angles within a cluster, given for rms angle spread normalized to 1 Ray number m; Basis vector of offset angles m; Ray power ratio βm; Basis vector of offset angles m. 21; ICPn/(ICPn+1); 0. 1,2; ± 0.0447; 0.2029/(ICPn+1); ± 0.1134. 3,4; ± 0.1413; 0.1679/(ICPn+1); ± 0.3403. 5,6; ± 0.2492; 0.1390/(ICPn+1); ± 0.5671. 7,8; ± 0.3715; 0.1151/(ICPn+1); ± 0.7940. 9,10; ± 0.5129; 0.0952/(ICPn+1); ± 1.0208. 11,12; ± 0.6797; 0.0788/(ICPn+1); ± 1.2477. 13,14; ± 0.8844; 0.0653/(ICPn+1); ± 1.4745. 15,16; ± 1.1481; 0.0540/(ICPn+1); ± 1.7014. 17,18; ± 1.5195; 0.0447/(ICPn+1); ± 1.9282. 19,20; ± 2.1551; 0.0370/(ICPn+1); ± 2.1551. Value and Distribution of ICPn is FFS <unchanged text omitted> Hu,s,nNLOSt=PnMm=1Mβm,nFrx,u,θθn,m,ZOA,ϕn,m,AOAFrx,u,ϕθn,m,ZOA,ϕn,m,AOATexpjΦn,mθθκn,m-1expjΦn,mθϕκn,m-1expjΦn,mϕθexpjΦn,mϕϕ Ftx,s,θθn,m,ZOD,ϕn,m,AODFtx,s,ϕθn,m,ZOD,ϕn,m,AODexpj2πrrx,n,mT∙drx,uλ0expj2πrtx,n,mT∙dtx,sλ0expj2πrrx,n,mT∙vλ0t (7.5-22)

Example 3)

Finally add offset angles m from Table 7.5-3 to the cluster angles , (7.5-13) where cASA is the cluster-wise rms azimuth spread of arrival angles (cluster ASA) in Table 7.5-6. Table 7.5-3: Ray offset angles within a cluster, given for rms angle spread normalized to 1 Ray number m; Basis vector of offset angles m; Ray power ratio βm,n; Basis vector of offset angles m. 1,2; ± 0.0447; ICPn/(ICPn+1); -0.0901. 2; 1/(19ICPn+1); -0.3611. 3,4; ± 0.1413; 1/(19ICPn+1); ±0.2729. 5,6; ± 0.2492; 1/(19ICPn+1); ±0.3614. 7,8; ± 0.3715; 1/(19ICPn+1); ±0.4626. 9,10; ± 0.5129; 1/(19ICPn+1); ±05808. 11,12; ± 0.6797; 1/(19ICPn+1); ±0.7227. 13,14; ± 0.8844; 1/(19ICPn+1); ±0.9004. 15,16; ± 1.1481; 1/(19ICPn+1); ±1.1383. 17,18; ± 1.5195; 1/(19ICPn+1); ±1.4995. 19,20; ± 2.1551; 1/(19ICPn+1); ±2.2763. ICPn=b∙10-a∙τnDS FFS values of a and b for each deployment scenario <unchanged text omitted> Hu,s,nNLOSt=PnMm=1Mβm,nFrx,u,θθn,m,ZOA,ϕn,m,AOAFrx,u,ϕθn,m,ZOA,ϕn,m,AOATexpjΦn,mθθκn,m-1expjΦn,mθϕκn,m-1expjΦn,mϕθexpjΦn,mϕϕ Ftx,s,θθn,m,ZOD,ϕn,m,AODFtx,s,ϕθn,m,ZOD,ϕn,m,AODexpj2πrrx,n,mT∙drx,uλ0expj2πrtx,n,mT∙dtx,sλ0expj2πrrx,n,mT∙vλ0t (7.5-22)

Conclusion

For new scenarios, and if changes are made to existing scenarios, interested companies in RAN1 to perform large scale and full calibration based on the following simulation assumptions. Use the following updates to TR38.901 as starting point for further discussion for calibration.

### 7.8.1 Large scale calibration For large scale calibration, fast fading is not modeled. The calibration parameters can be found in Table 7.8-1. The calibration results based on TR 38.900 V14.0.0 can be found in R1-165974. Table 7.8-1: Simulation assumptions for large scale calibration Parameter; Values. Scenarios; UMa, UMi-Street Canyon, Indoor-office (open office). Sectorization; 3 sectors per cell site: 30, 150 and 270 degrees. BS antenna configurations; Mg = Ng = 1; (M,N,P) = (10, 1, 1), dV = 0.5λ. BS port mapping; The 10 elements are mapped to a single CRS port. BS antenna electrical downtilting; 102 degrees for UMa and UMi Street Canyon 110 degrees for indoor. Antenna virtualization; DFT precoding according to TR 36.897 with application of panning and tilting angles. BS Tx power; 44 dBm for UMi-Street Canyon, 49 for UMa at 6GHz 35 dBm at 30GHz and 70 GHz for UMa and UMi-Street canyon 24 dBm for Indoor for all carrier frequencies. Bandwidth; 20MHz for 6GHz, and 100MHz for 30GHz and 70 GHz. UT antenna configurations; 1 element (vertically polarized), Isotropic antenna gain pattern. Handover margin (for calibration); 0dB. UT distribution; Following TR36.873 for UMa and UMi, (3D dropping) uniform dropping for indoor with minimum distance (2D) of 0 m. UT attachment; Based on pathloss considering LOS angle. UT noise figure; 9 dB. Fast fading channel; Fast fading channel is not modelled. O2I penetration loss; 50% low loss and 50% high loss. Carrier Frequency; 6 GHz, 30 GHz, 70GHz. Wrapping method for UMa and UMi; geographical distance based wrapping (mandatory) radio distance (optional). Metrics; 1) Coupling loss – serving cell (based on LOS pathloss). 2) Geometry (based on LOS pathloss) with and without white noise. Additional calibration parameters can be found in Table 7.8-1A. It is assumed that parameters from Table 7.8-1 is used if unspecified by the additional calibration parameters in Table 7.8-1A. Table 7.8-1A: Simulation assumptions for large scale calibration Parameter; Values. Scenarios; UMa, UMi-Street Canyon, SMa. BS antenna electrical downtilting; FFS degrees for SMa. BS antenna configurations; Config 0 for 6GHz: M = 10,N = 1,P=1; Mg = Ng = 1; Mp = 10, Np = 1; dV = 0.5λ Config 1 for 7GHz : M = 8, N = 16,P = 2; Mg = 1, Ng = 1; Mp = 8, Np = 16; dH = dV = 0.5λ. BS Tx power; FFS dBm for SMa. Bandwidth; 20MHz for 6GHz only for UMa and UMi-Street Canyon, 200 MHz for 7 GHz. UT distribution; For SMA, 20% of UT outdoor, 80% of UT indoor. Among indoor UT, 90% of indoor UT are within residential buildings, and 10% of indoor UT in commercial buildings. Indoor UTs are uniformly distributed across all floors for a building type.. O2I penetration loss; For SMa, FFS. Carrier Frequency; 6 GHz, 7 GHz. ### 7.8.2 Full calibration The calibration parameters for full calibration including the fast fading modelling can be found in Table 7.8-2. Unspecified parameters in Table 7.8-2 are the same as those in Table 7.8-1. When P=2, X-pol (+/-45 degree) is used for BS antenna configuration 1 and X-pol (0/+90 degree) is used for UT antenna configuration. The calibration results based on TR 38.900 V14.0.0 can be found in R1-165975. Table 7.8-2: Simulation assumptions for full calibration Parameter; Values. Scenarios; UMa, UMi-street Canyon, Indoor-office (open office). Carrier Frequency; 6 GHz, 30 GHz, 60GHz, 70GHz. Bandwidth; 20MHz for 6GHz, and 100MHz for 30GHz, 60 GHz and 70 GHz. BS Tx power; 44 dBm for UMi-Street Canyon, 49 for UMa at 6GHz 35 dBm at 30GHz, 60 GHz and 70 GHz for UMa and UMi-Street canyon 24 dBm for Indoor for all carrier frequencies. BS antenna configurations; Config 1: M = 4,N = 4,P = 2, Mg = 1, Ng = 2, dH = dV = 0.5λ, dH,g = dV,g = 2.5λ … calibration metrics 1), 2), 3) are calibrated Config 2: Mg = Ng = 1, M = N = 2, P = 1 … calibration metrics 1), 2), 4) are calibrated. BS port mapping; Config 1: all 16 elements for each polarization on each panel are mapped to a single CRS port; panning angles of the two subarrays: (0,0) degs; same downtilt angles as used for the large-scale calibrations Config 2: each antenna element is mapped to one CRS port. UT antenna configurations; Mg = Ng = 1, M = N = 1, P = 2. UT distribution; Following TR36.873 for UMa and UMi, (3D dropping) uniform dropping for indoor with minimum distance (2D) of 0 m. UT attachment; Based on RSRP (formula) from CRS port 0. Polarized antenna modelling; Model-2 in TR36.873. UT array orientation; ΩUT, uniformly distributed on [0,360] degree, ΩUT,= 90 degree, ΩUT, = 0 degree. UT antenna pattern; Isotropic. Metrics; 1) Coupling loss – serving cell. 2) Wideband SIR before receiver without noise. 3) CDF of Delay Spread and Angle Spread (ASD, ZSD, ASA, ZSA) from the serving cell (according to circular angle spread definition of TR 25.996). 4) CDF of largest (1st) PRB singular values (serving cell) at t=0 plotted in 10*log10 scale. CDF of smallest (2nd) PRB singular values (serving cell) at t=0 plotted in 10*log10 scale. CDF of the ratio between the largest PRB singular value and the smallest PRB singular value (serving cell) at t=0 plotted in 10*log10 scale. Note: The PRB singular values of a PRB are the eigenvalues of the mean covariance matrix in the PRB.. Additional full calibration parameters can be found in Table 7.8-2A. It is assumed that parameters from Table 7.8-2 is used if unspecified by the additional full calibration parameters in Table 7.8-2A. Table 7.8-2A: Simulation assumptions for full calibration Parameter; Values. Scenarios; UMa, UMi-Street Canyon, SMa. Carrier Frequency; 6 GHz, 7 GHz. BS antenna electrical downtilting; FFS degrees for SMa. BS antenna configurations; Config 1 for 6GHz and only for UMa and UMi: M = 4,N = 4,P = 2, Mg = 1, Ng = 2, dH = dV = 0.5λ, dH,g = dV,g = 2.5λ … calibration metrics 1), 2), 3) are calibrated Config 2 for 6GHz and only for UMa and UMi: Mg = Ng = 1, M = N = 2, P = 1 … calibration metrics 1), 2), 4) are calibrated Config 3) for 7: M = 8, N = 16,P = 2, Mg = 1, Ng = 1, Mp = 8, Np = 16,dH = dV = 0.5λ … calibration metrics 1), 2), 3), 4) are calibrated Config 4) for 7: M = 64, N = 16,P = 2, Mg = 1, Ng = 1, Mp = 8, Np = 16,dH = dV = 0.5λ … calibration metrics 1), 2), 3), 4) are calibrated. BS Tx power; FFS dBm for SMa. Bandwidth; 20MHz for 6GHz and only for UMa and UMi-Street Canyon, 200 MHz for 7 GHz. UT distribution; For SMA, 20% of UT outdoor, 80% of UT indoor. Among indoor UT, 90% of indoor UT are within residential buildings, and 10% of indoor UT in commercial buildings. Indoor UTs are uniformly distributed across all floors for a building type.. UT antenna configurations; Config A for 6GHz and only for UMa and UMi: Mg = Ng = 1, M = N = 1, P = 2 Config B for 7GHz: reference 4 antenna port with single polarization for calibration based on handheld device antenna model described in Clause 7.3 Config C for 7 GHz: Mg = Ng = 1, M = 2, N = 2, P = 2. UT antenna pattern; Config A for 6GHz: FFS Config B and C for 7 GHz: Based on [directional antenna] for UT described in Clause 7.3.. O2I penetration loss; For SMa, FFS. ### 7.8.3 Calibration of additional features The calibration parameters for the calibration of oxygen absorption, large bandwidth and large antenna array, spatial consistency, and blockage, near field channel modeling, and spatial non-stationarity can be respectively found in Table 7.8-3, 7.8-4, 7.8-5, and 7.8-6, 7.8-7, and 7.8-8. Unspecified parameters in these tables are the same as those in Tables 7.8-1 and 7.8-2. When P=2, X-pol (+/-45 degree) is used for BS antenna configuration 1 and X-pol (0/+90 degree) is used for UT antenna configuration. The calibration results based on TR 38.900 V14.0.0 can be found in R1-1700990. Table 7.8-7: Simulation assumptions for calibration for near field channel modeling Parameter; Values. Scenarios; UMi-street Canyon, [Indoor-office (open office)]. Carrier Frequency; [6 or 7 GHz]. Bandwidth; 20 MHz. BS antenna height; UMi: 10m [Indoor-office: 3m]. BS antenna downtilt; 102 degrees for UMi Street Canyon [110 degrees for indoor]. BS Tx power; 44 dBm for UMi-Street Canyon [24 dBm for Indoor for all carrier frequencies]. BS antenna configurations; For UMi scenario: - Config1: (M, N, P,Mg, Ng,MP,NP) = (24,16,2,1,1, 8,16), dh = 0.5λ,dv = 0.7λ; - Config2: (M, N, P,Mg, Ng, MP,NP) = (24,32,2,1,1,8,32), dh = 0.5λ,dv = 0.7λ; [For Indoor-office scenario:FFS]. UE Location; Outdoor/indoor; UMi and UMa: 100% outdoor Indoor-office: 100% indoor. LOS/NLOS; 100% LOS. UE antenna height; UMi and UMa: 1.5m Indoor-office: 1m. UT antenna configurations; - Config 1: Mg = Ng = 1, M = 2, N = 1, P = 2 - Config 2: Mg = Ng = 1, M = 2, N = 2, P = 2 - Config 3: 4 antenna ports with single polarization based on the following antenna placements - Config 4: Mg = Ng = 1, M = 4, N = 2, P = 2. Calibration method; Drop multiple users in the multiple cells at the fixed horizontal distance Z from the BS, and collect the metric per distance; - Note: the value Z can be (1,2,3,4,5,10)* minimum distance of BS and UT - For UMi, the minimum distance is 10 m (ref. Table 6-1: Scenarios in TR36.873) Component of NF channel to be considered: - Phase of Direct path; - Phase of Non-direct path (optionally);. Metric; 1) CDF of the ratio between the 2nd,3rd,4th,...,xth singular value and the 1st singular value (serving cell) at t=0 plotted in 10*log10 scale with/without SNS at UE side Note: The value of x are the minimum value of (number of BS antenna ports, number of UE antenna ports) Note: The singular value are calculated in the M PRBs, e.g., M=4;. Table 7.8-7: Simulation assumptions for calibration for spatial non-stationarity Parameter; Values. Scenarios; UMi-street Canyon. Carrier Frequency; [6 or 7 GHz]. Bandwidth; 20 MHz. BS Antenna height; UMi: 10m. BS Tx power; 44 dBm for UMi-Street Canyon. BS Ant Config.; M = 4,N = 4,P = 2, Mg = 1, Ng = 2, dH = dV = 0.5λ, dH,g = dV,g = 2.5λ. BS port mapping; Config 1: all 16 elements for each polarization on each panel are mapped to a single port; panning angles of the two subarrays: (0,0) degs; same downtilt angles as used for the large-scale calibrations Config 2: each antenna element is mapped to one port. UT Ant Config.; 4 antenna ports with single polarization based on the antenna placements for 4 elements of handheld devices. UT distribution; Following TR36.873 for UMi, (3D dropping). Metric; 1) CDF of coupling loss (serving cell) received per UE port with/without SNS at UE side 2) CDF of the ratio between the 2nd,3rd,4th,...,xth singular value and the 1st singular value (serving cell) at t=0 plotted in 10*log10 scale with/without SNS at UE side Note: The value of x are the minimum value of (number of BS antenna ports, number of UE antenna ports) Note: The singular value are calculated in the M PRBs, e.g., M=4;.

Note: Email discussion on the simulation assumption for calibration until Mar.5th .

Conclusion

To check and review the following results and measurement data provided in RAN1 #120 for further discussion in next RAN1 meeting. R1-2501426 contains the list of data sources for the results and measurements provided since RAN1 #116-bis until RAN1 #120.

Agreement

- Adopt the following absolute delay parameters for InH scenarios.

  - Down-select between option 1, 2 and 3.

Table 7.6.9-1: Parameters for the absolute time of arrival model

| Scenarios |  | InH |
| --- | --- | --- |
| lgΔτ=log10Δτ1s | μlgΔτ | Option 1) -7.9 Option 2) -8.6 Option 3) for 40% of the NLOS cluster, Δτ=0; for other 60% of NLOS cluster μlgΔτ = -8.9 |
|  | σlgΔτ | Option 1) 0.3 Option 2) 0.1 Option 3) for 40% of the NLOS cluster with Δτ=0, σlgΔτ is not applicable; for other 60% of the NLOS cluster σlgΔτ = 0.7 |
| Correlation distance in the horizontal plane [m] |  | 10 |

Working Assumption

- Intermediate angle values of the CDL channel is computed by

  - ϕn,intermediate=s* WrapTo180ϕn,model-μϕ,model,

  - where s is a scale factor chosen to change the distribution of the angles. Table X shows required scale factor for typical desired angular spread values for AOD, AOD, ZOA, and ZOD.

Table X: Scale factor values for each CDL model

| CDL Type | Desired AOD Spread（°） | Scale Factor (AOD) | Desired AOA Spread（°） | Scale Factor (AOA) | Desired ZOA Spread（°） | Scale Factor (ZOA) | Desired ZOD Spread（°） | Scale Factor (ZOD) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CDL-A | 5 | 0.068 | 30 | 0.353 | 5 | 0.240 | 1 | 0.035 |
|  | 10 | 0.136 | 45 | 0.527 | 10 | 0.480 | 3 | 0.106 |
|  | 15 | 0.204 | 60 | 9.525 | 15 | 0.723 | 5 | 0.176 |
|  | 25 | 0.341 |  |  |  |  |  |  |
| CDL-B | 5 | 0.124 | 30 | 0.542 | 5 | 0.652 | 1 | 0.194 |
|  | 10 | 0.248 | 45 | 0.808 | 10 | 1.302 | 3 | 0.582 |
|  | 15 | 0.371 | 60 | 1.071 | 15 | 1.948 | 5 | 0.971 |
|  | 25 | 0.617 |  |  |  |  |  |  |
| CDL-C | 5 | 0.128 | 30 | 0.431 | 5 | 0.648 | 1 | 0.364 |
|  | 10 | 0.257 | 45 | 0.645 | 10 | 1.297 | 3 | 1.093 |
|  | 15 | 0.386 | 60 | 7.749 | 15 | 1.950 | 5 | 1.822 |
|  | 25 | 0.651 |  |  |  |  |  |  |
| CDL-D | 5 | 0.323 | 30 | 9.869 | 5 | 4.327 | 1 | 0.448 |
|  | 10 | 0.665 | 45 | N/A | 10 | 8.887 | 3 | 1.347 |
|  | 15 | 1.059 | 60 | N/A | 15 | 14.034 | 5 | 2.258 |
|  | 25 | 9.354 |  |  |  |  |  |  |
| CDL-E | 5 | 0.395 | 30 | 9.743 | 5 | 6.919 | 1 | 0.971 |
|  | 10 | 0.801 | 45 | N/A | 10 | 14.838 | 3 | 2.918 |
|  | 15 | 1.233 | 60 | N/A | 15 | 27.285 | 5 | 4.877 |
|  | 25 | 7.780 |  |  |  |  |  |  |

Note: the values are computed by formula option 2A defined in RAN1#119.

Agreement

- Further study updates to the mean angle calculation in Annex used for calculation of the mean angle values for CDL-D/E channel model

  - Potential update of the power weighted mean angle is

Agreement

- Discuss further on the following suggested text changes:

< Unchanged text omitted > #### 7.7.5.1 CDL extension: Scaling of angles The angle values of CDL models are fixed, which is not very suitable for MIMO simulations for several reasons; The PMI statistics can become biased, and a fixed precoder may perform better than open-loop and on par with closed-loop or reciprocity beamforming. Furthermore, a CDL only represents a single channel realization. The predefined angle values in the CDL models can be generalized by introducing angular translation and scaling. By translation, mean angle can be changed to and angular spread can be changed by scaling. The translated and scaled ray angles can be obtained according to the following equation: (7.7-5) in which: is the tabulated CDL ray cluster angle is the rms angular spread of the tabulated CDL including the offset ray angles, calculated using the angular spread definition in Annex A is the mean angle of the tabulated CDL, calculated using the definition in Annex A is the desired mean angle is the desired rms angular spread is the resulting scaled ray cluster angle. The angular scaling is applied on the ray angles including offsets from the tabulated cluster angles. Typical angular spreads for different scenarios can be obtained from the system-level model. Example scaling values are: - AOD spread (ASD) for each CDL model: {5, 10, 15, 25} degrees. - AOA spread (ASA) for each CDL model: {30, 45, 60} degrees. - ZOA spread (ZSA) for each CDL model: {5, 10, 15} degrees. - ZOD spread (ZSD) for each CDL model: {1, 3, 5} degrees. The angular scaling and translation can be applied to some or all of the azimuth and zenith angles of departure and arrival. Note: The azimuth angles of ϕn,model-μϕ,model should be wrapped around to be within [-180, 180] degrees. Note: The azimuth angles may need to be wrapped around to be within [0, 360] degrees, while the zenith angles may need to be clipped to be within [0, 180] degrees. Note: The resulting scaled ray cluster angle may or may not achieve the desired angle mean and the desired rms angle spread.

Agreement

- RAN1 has identified that standard multi-panel glass penetration loss model may require updates at least for 6-24 GHz frequency range. Further discuss changes.

- For standard multi-panel glass penetration loss model, study the following model changes:

  - Option 1) thickness dependent model without resonation effect

    - Lglass=α∙(2+0.2f)

    - where α=Tglass3 cm+0.41.4

    - Default reference thickness for standard multi-panel glass is 3 cm

    - If model without resonation effect is selected, add note in TR that while glass material measurement may showcase resonation effect as a function of frequency and thickness, the model abstracts away the effect.

  - Option 2) thickness dependent model with resonation effect

    - Lglass=0.08∙α∙f+sin1.05∙α∙f-7.8/α+0.2-0.3α+1.8

      - For α=3, Lglass=0.9+ 0.24∙f+sin3.15∙f-7.8/3.2

    - where α=Tglass1 cm

    - Default reference thickness for standard multi-panel glass is 3 cm

  - Option 3) thickness dependent model without resonation effect and with air interface loss

    - Lglass=a+b∙α∙f, α=TglassX cm

    - Where X is the default thickness of the glass.

    - Parameters a, b, and X are FFS.

  - Note: for all options, consider the impact to overall building exterior penetration loss from the potential changes to glass penetration loss.

Agreement

- RAN1 has identified that IRR glass penetration loss model may require updates at least for 6-24 GHz frequency range. Further discuss changes For IRR glass penetration loss model, study the following model changes:

  - Option 1) coating dependent model without resonation effect

    - LIRRglass=0.11f-0.9α2+8.3α+18

      - For α=1, LIRRglass=25.4+ 0.11f

    - where α=1 for single silver-coating; α=2 for double silver-coating; α=3 for triple silver-coating.

    - Default reference coating for standard multi-panel glass is single coating, i.e. α=1

    - Default reference thickness for IRR glass is 3 cm

    - If model without resonation effect is selected, add note in TR that while glass material measurement may showcase resonation effect as a function of frequency and thickness, the model abstracts away the effect.

  - Option 2) coating dependent model with resonation effect

    - LIRRglass=0.11f-3sin0.5f+1.36-0.9α2+8.3α+18

      - For α=1, LIRRglass=25.4+ 0.11f-3sin0.5f+1.36

    - where α=1 for single silver-coating; α=2 for double silver-coating; α=3 for triple silver-coating.

    - Default reference coating for IRR glass is single coating, i.e. α=1

  - Option 3) thickness dependent model without resonation effect and with air interface loss

    - LIRRglass=a+b∙α∙f, α=TIRRglassX cm

    - Where X is the default thickness of the glass.

    - Parameters a, b, and X are FFS.

  - Note: for all options, consider the impact to overall building exterior penetration loss from the potential changes to glass penetration loss.

  - FFS: whether and how to incorporate thickness dependent affect for the IRR model. In this context, incorporation of thickness impact is considered a minor refinement of option 1 or 2 to better match the measurement data.

Agreement

- Discuss further on potential update of concrete penetration model based on measurement provided by companies.

  - For updates of the concrete penetration loss model, further study between the following model changes:

    - Option 1) model with no separate air to interface insertion loss

      - Lconcrete=α(5+4f)

      - α=Tconcrete23 cm

      - Default reference thickness for concrete is 23 cm

    - Option 2) model with separate air to interface insertion loss

      - Lconcrete=1.4+4.6αf

      - α=Tconcrete23cm

      - Default reference thickness for concrete is 23 cm

Note: for all options, consider the impact to overall building exterior penetration loss from the potential changes to concrete penetration loss.

Agreement

- Discuss further on potential introduction of new material mode “cinder block” penetration model based on measurement provided by companies for concrete based walls.

Agreement

- Discuss further on potential update of wood penetration loss model based on measurement provided by companies. The following is an example of a potential update:

  - Option 1) thickness dependent model without air interface loss

    - Lwood=α(4.85+0.12f)

    - α=Twood6cm

    - Default reference thickness for wood is 6 cm

  - Option 2) thickness dependent model with air interface loss

    - Lwood=a+b∙α∙f

    - α=TwoodX cm

    - Where X is the default thickness of the wood.

    - Parameters a, b, and X are FFS.

Agreement

Discuss further on whether to introduce additional model to handle LOS probability impact from vegetation for SMa and details of the additional modeling component.

Agreement

For suburban scenario, the down-select among the following LOS probability

- Option 1)

PrLOS=exp(-d2D-out-10exp(0.02044hBS+5.746))

- Option 2)

PLOS=1, d2D≤10exp-d2Dkcommercialexp-d2Dkresidentialexp-d2Dkvegetation, d2D>10

kcommercial=-dcommercialln1-rcommericial∙hBS-hUThcommercial-hUT

kresidential=-dresidentialln1-rresidential∙hBS-hUThresidential-hUT

kvegetation=-dresidentialln1-rresidential∙hBS-hUThresidential-hUT

dcommercial,residential,vegetation=clutter bin size

rcommercial,residential,vegetation=clutter density

hcommercial,residential,vegetation=clutter height

| Parameter |  |
| --- | --- |
| dcommercial,residential,vegetation | 30 m |
| hcommercial | 20 m |
| hresidential | 8 m |
| hvegetation | 15 m |
| rcommercial | 2% |
| rresidential | 18% |
| rvegetation | 0% (no vegetation), 10% (sparse vegetation), or 20% (dense vegetation) |

- Option 3)

PLOS =1,  &d2D-out≤10exp(-d2D-out-10200),  &d2D-out>10

Working Assumption

- For parameters with multiple options, choose from one of the options in meeting RAN1#120-bis.

- For information: O2I are barrowed from TR38.901 UMa O2I, LOS and NLOS parameters are barrowed from ITU M.2135 SMa if values are available. For colored values listed, values are based on measurement data from companies.

| Scenarios |  | SMa |  |  | Reference – ITU M.2135 SMa |  |  | TR38.901 UMa |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | O2I | LOS | NLOS | O2I | LOS | NLOS | O2I |
| Delay spread (DS) lgDS=log10(DS/1s) | µlgDS | –7.23 | Option 1) -7.12 Option 2) -7.2 Option 3) -6.98 | -6.62 | –7.23 | –7.12 | N/A | -6.955 - 0.0963 log10(fc) | -6.28 - 0.204 log10(fc) | -6.62 |
|  | σlgDS | 0.38 | Option 1) 0.33 Option 2) 0.58 Option 3) 0.74 | 0.32 | 0.38 | 0.33 | N/A | 0.66 | 0.39 | 0.32 |
| AOD spread (ASD) lgASD=log10(ASD/1) | µlgASD | 0.55 | 0.55 | 0.55 | 0.78 | 0.90 | N/A | 1.06 + 0.1114 log10(fc) | 1.5 - 0.1144 log10(fc) | 1.25 |
|  | σlgASD | 0.25 | 0.25 | 0.25 | 0.12 | 0.36 | N/A | 0.28 | 0.28 | 0.42 |
| AOA spread (ASA) lgASA=log10(ASA/1) | µlgASA | Option 1) 1.48 Option 2) 1.75 | Option1) 1.65 Option 2) 1.74 Option 3) 1.27 | 1.76 | 1.48 | 1.65 | N/A | 1.81 | 2.08 - 0.27 log10(fc) | 1.76 |
|  | σlgASA | Option 1) 0.20 Option 2) 0.04 | Option 1) 0.25 Option 2) 0.24 Option 3) 0.86 | 0.16 | 0.20 | 0.25 | N/A | 0.20 | 0.11 | 0.16 |
| ZOA spread (ZSA) lgZSA=log10(ZSA/1) | µlgZSA | 1.31 | Option 1) 1.32 Option 2) -0.388 | 1.01 | N/A | N/A | N/A | 0.95 | -0.3236 log10(fc) + 1.512 | 1.01 |
|  | σlgZSA | 0.02 | Option 1) 0.08 Option 2) 1.17 | 0.43 | N/A | N/A | N/A | 0.16 | 0.16 | 0.43 |
| Shadow fading (SF) [dB] | σSF | 4 | 8 | 7 | 4 | 8 | N/A | See Table 7.4.1-1 | See Table 7.4.1-1 | 7 |
| K-factor (K) [dB] | µK | 9 | N/A | N/A | 9 | N/A | N/A | 9 | N/A | N/A |
|  | σK | 7 | N/A | N/A | 7 | N/A | N/A | 3.5 | N/A | N/A |
| Cross-Correlations | ASD vs DS | 0 | 0 | 0.4 | 0 | 0 | N/A | 0.4 | 0.4 | 0.4 |
|  | ASA vs DS | 0.8 | Option 1) 0.7 Option 2) 0.5 | 0.4 | 0.8 | 0.7 | N/A | 0.8 | 0.6 | 0.4 |
|  | ASA vs SF | –0.5 | Option 1) 0 Option 2) -0.25 | 0 | –0.5 | 0 | N/A | -0.5 | 0 | 0 |
|  | ASD vs SF | –0.5 | -0.4 | 0.2 | –0.5 | –0.4 | N/A | -0.5 | -0.6 | 0.2 |
|  | DS vs SF | –0.6 | Option 1) -0.4 Option 2) -0.13 | -0.5 | –0.6 | –0.4 | N/A | -0.4 | -0.4 | -0.5 |
|  | ASD vs ASA | 0 | 0 | 0 | 0 | 0 | N/A | 0 | 0.4 | 0 |
|  | ASD vs K | 0 | N/A | N/A | 0 | N/A | N/A | 0 | N/A | N/A |
|  | ASA vs K | 0 | N/A | N/A | 0 | N/A | N/A | -0.2 | N/A | N/A |
|  | DS vs K | 0 | N/A | N/A | 0 | N/A | N/A | -0.4 | N/A | N/A |
|  | SF vs K | 0 | N/A | N/A | 0 | N/A | N/A | 0 | N/A | N/A |
| Cross-Correlations 1) | ZSD vs SF | FFS | FFS | 0 | N/A | N/A | N/A | 0 | 0 | 0 |
|  | ZSA vs SF | FFS | -0.47 | 0 | N/A | N/A | N/A | -0.8 | -0.4 | 0 |
|  | ZSD vs K | FFS | N/A | N/A | N/A | N/A | N/A | 0 | N/A | N/A |
|  | ZSA vs K | FFS | N/A | N/A | N/A | N/A | N/A | 0 | N/A | N/A |
|  | ZSD vs DS | FFS | -0.06 | -0.6 | N/A | N/A | N/A | -0.2 | -0.5 | -0.6 |
|  | ZSA vs DS | FFS | FFS | -0.2 | N/A | N/A | N/A | 0 | 0 | -0.2 |
|  | ZSD vs ASD | 0.5 | 0.5 | 0.5 | N/A | N/A | N/A | 0.5 | 0.5 | -0.2 |
|  | ZSA vs ASD | FFS | FFS | 0 | N/A | N/A | N/A | 0 | -0.1 | 0 |
|  | ZSD vs ASA | FFS | FFS | 0 | N/A | N/A | N/A | -0.3 | 0 | 0 |
|  | ZSA vs ASA | FFS | 0.36 | 0.5 | N/A | N/A | N/A | 0.4 | 0 | 0.5 |
|  | ZSD vs ZSA | FFS | FFS | 0.5 | N/A | N/A | N/A | 0 | 0 | 0.5 |
| Delay scaling parameter r |  | 2.4 | 1.5 | 2.2 | 2.4 | 1.5 | N/A | 2.5 | 2.3 | 2.2 |
| XPR [dB] | µXPR | 8 | 4 | 9 | 8 | 4 | N/A | 8 | 7 | 9 |
|  | σXPR | N/A | N/A | 5 | N/A | N/A | N/A | 4 | 3 | 5 |
| Number of clusters N |  | 15 | 14 | 12 | 15 | 14 | N/A | 12 | 20 | 12 |
| Number of rays per cluster M |  | 20 | 20 | 20 | 20 | 20 | N/A | 20 | 20 | 20 |
| Cluster DS (cDS) in [ns] |  | N/A | N/A | 11 | N/A | N/A | N/A | max(0.25, 6.5622 -3.4084 log10(fc)) | max(0.25, 6.5622 -3.4084 log10(fc)) | 11 |
| Cluster ASD (cASD) in [deg] |  | Option 1) 5 Option 2) 1.5 | Option 1) 2 Option 2) 1.5 | 1.5 | 5 | 2 | N/A | 5 | 2 | 5 |
| Cluster ASA (cASA) in [deg] |  | 5 | 10 | 8 | 5 | 10 | N/A | 11 | 15 | 8 |
| Cluster ZSA (cZSA) in [deg] |  | N/A | N/A | 3 | N/A | N/A | N/A | 7 | 7 | 3 |
| Per cluster shadowing std ξ [dB] |  | 3 | 3 | 4 | 3 | 3 | N/A | 3 | 3 | 4 |
| Correlation distance in the horizontal plane [m] | DS | 6 | 40 | 10 | 6 | 40 | N/A | 30 | 40 | 10 |
|  | ASD | 15 | 30 | 11 | 15 | 30 | N/A | 18 | 50 | 11 |
|  | ASA | 20 | 30 | 17 | 20 | 30 | N/A | 15 | 50 | 17 |
|  | SF | 40 | 50 | 7 | 40 | 50 | N/A | 37 | 50 | 7 |
|  | K | 10 | N/A | N/A | 10 | N/A | N/A | 12 | N/A | N/A |
|  | ZSA | N/A | N/A | 25 | N/A | N/A | N/A | 15 | 50 | 25 |
|  | ZSD | N/A | N/A | 25 | N/A | N/A | N/A | 15 | 50 | 25 |

| Scenarios |  | SMa LOS | SMa NLOS | SMa O2I | For reference - UMa LOS | For reference - UMa NLOS | For reference – RMa O2I |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ZOD spread (ZSD) lgZSD=log10(ZSD/1) | µlgZSD | Option 1) 0.14 Option 2) 0.55 | Option 1) 0.14 Option 2) 0.55 | 0.55 | max[-0.5, -2.1(d2D/1000) -0.01 (hUT - 1.5)+0.75] | max[-0.5, -2.1(d2D/1000) -0.01(hUT - 1.5)+0.9] | max[-1, -0.19(d2D/1000) -0.01(hUT - 1.5) + 0.28] |
|  | σlgZSD | Option 1) 0.16 Option 2) 0.25 | Option 1) 0.16 Option 2) 0.25 | 0.25 | 0.40 | 0.49 | 0.30 |
| ZOD offset | µoffset,ZOD | FFS | FFS | FFS | 0 | e(fc)-10^{a(fc) log10(max(b(fc), d2D)) +c(fc) -0.07(hUT-1.5)} | arctan((35 - 3.5)/d2D) -arctan((35 - 1.5)/d2D) |
| Note: For NLOS ZOD offset: a(fc) = FFS; b(fc) = FFS; c(fc) = FFS; e(fc) = FFS. |  |  |  |  | Note: For NLOS ZOD offset: a(fc) = 0.208log10(fc)- 0.782; b(fc) = 25; c(fc) = -0.13log10(fc)+2.03; e(fc) = 7.66log10(fc)-5.96. |  |  |

Agreement

Further study on introduction of a loss outdoor-to-indoor (O2I) building penetration loss model for SMa:

- PLtw=5-10log100.3⋅10-Lglass10+0.7⋅10-Lwood10

- Consider the impact to overall building exterior penetration loss

Working Assumption

| Scenario | LOS/NLOS | Pathloss [dB], fc is in GHz and d is in meters | Shadow fading std [dB] | default values | Applicability range |
| --- | --- | --- | --- | --- | --- |
| SMa | LOS | PL1=20log1040πdfc3+min0.03h1.72,10log10d-min0.044h1.72, 14.77+0.002log10hd, h = 10 m, | σ=4 σ=6 | 10 m < d< dBP dBP < d < 5000 m | 0.8 ≤fc≤ 37 GHz 25 m < hBS < 35 m 1 m < hUT < 14 m |
|  | NLOS | PL=161.04-7.1log10W+7.5log10h-24.37-3.7hhBS2log10hBS+43.42-3.1log10hBSlog10d-3+20log10fc-3.2log1011.75hUT2-4.97 h = 10 m, W = 10 m FFS potential changes: PL=161.04-7.1log10W+7.5log10h-24.37-3.7hhBS2log10hBS+43.42-3.1log10hBSlog10d-3+20log10fc+10⋅log10minfc,10-10⋅log1010-3.2log1011.75hUT2-4.97 h = 10 m, W = 10 m | σ=8 | 10 m < dBP < 5000 m |  |

Agreement

For UE antenna modeling of handheld devices, at least support directional antenna radiation pattern for calibration purposes.

- Further study the following directional radiation pattern parameters

| For directional radiation pattern |  |  |
| --- | --- | --- |
| Vertical Radiation Pattern | Horizontal | Max Gain |
| θ3dB=90° SLAV= 25 dB | ϕ3dB=120°~180° Am= 25 dB |  |
| θ3dB=115° SLAV=25 dB | ϕ3dB=85° Am=25 dB |  |
| θ3dB=131° SLAV=20 dB | ϕ3dB=115° Am=20 | 4.5 ~ 6.5 dBi |
| θ3dB=90° SLAV=25 dB | ϕ3dB=90° Am=25 | ≤ 5 dBi |
| θ3dB=90~120° SLAV=15~25 dB | ϕ3dB=90°~120° Am=15~25 dB | 0 ~ 5 dBi |
| θ3dB=131° SLAV=15 dB | ϕ3dB=131° Am=15 dB | 5 dBi |

Agreement

RAN1 has identified that angular spread for following scenarios require necessary updates at least for 6-24 GHz frequency range. Further discuss necessary changes. Note: For UMa ASD, weak dependency to frequency was observed and the potential necessary changes may include changes to values across wide frequency ranges.

- UMi LOS ASA

- UMi NLOS ASA

- UMa LOS ASD

- UMa LOS ASA

- UMa NLOS ASD

- UMa NLOS ASA

- UMa NLOS ZSA

Agreement

- Conclude in RAN1 #120bis among the following:

- Alt 1)

  - RAN1 has identified that delay spread for UMi LOS/NLOS and UMa LOS/NLOS scenario is smaller compared to delay spread in the TR at least for 6-24 GHz frequency range. Further discuss, necessary changes for UMi LOS/NLOS and UMa LOS/NLOS

  - Proponent companies to provide detailed information on potentially necessary changes to DS for scenarios in question.

    - Data samples for discussion on necessary changes for UMi LOS

| Delay spread For UMi LOS | TR38.901 @ 6.5 GHz | Measured @ 6.5 GHz | TR38.901 @ 10 GHz | Measured @ 10 GHz | TR38.901 @ 13 GHz | Measured @ 13 GHz | TR38.901 @ 13.5 GHz | Measured @ 13.5 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| μDS (log_10⁡〖DS/1s〗 ) | -7.35 | -8.16 | -7.39 | -7.47 | -7.42 | -7.85 | -7.41 | -8.19 |
| σDS | 0.38 | 0.83 | 0.38 | 0.28 | 0.38 | 0.54 | 0.38 | 0.91 |

- Alt 2)

  - For the following scenarios, there is no consensus to update delay spread models due to lack of consistent and significant observed difference between model and measurements.

    - UMi LOS/NLOS and UMa LOS/NLOS

Agreement

- Conclude in RAN1 #120bis among the following:

- Alt 1)

  - RAN1 has identified that number of clusters spread for [InH LOS/NLOS, UMi LOS/NLOS, and UMa LOS/NLOS] scenario is smaller compared to delay spread in the TR at least for 6-24 GHz frequency range. Further discuss, necessary changes for [InH LOS/NLOS, UMi LOS/NLOS, and UMa LOS/NLOS].

    - RAN1 to determine which scenarios among [InH LOS/NLOS, UMi LOS/NLOS, and UMa LOS/NLOS] requires necessary changes.

    - Data samples for discussion on necessary changes

| Scenario | InH @10 GHz |  |  |  | Umi @10 GHz |  |  |  | UMa @6.5 GHz |  |  |  | UMa @13 GHz |  |  |  | UMa @15GHz |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TR 38.901 |  | Measurement |  | TR 38.901 |  | Measurement |  | TR 38.901 |  | Measurement |  | TR 38.901 |  | Measurement |  | TR 38.901 |  | Measurement |  |
|  | LOS | NLOS | LOS | NLOS | LOS | NLOS | LOS | NLOS | LOS | NLOS | LOS | NLOS | LOS | NLOS | LOS | NLOS | LOS | NLOS | LOS | NLOS |
| Number of clusters | 15 | 19 | 10 | 11 | 12 | 19 | 5 | 7 | 12 | 20 | 13 | 17 | 12 | 20 | 11 | 15 | 12 | 20 | 7 | 13 |

  - Proponent companies to provide detailed information on potentially necessary changes to number of clusters for scenarios in question.

- Alt 2)

  - For the following scenarios, while sources have observed smaller number of clusters for various deployment scenario from measurement taken for 6 – 24 GHz frequency range, due to impact on frequency continuity outside 6 – 24 GHz frequency range, RAN1 concludes that there is no census to update number of clusters for all existing scenarios.

Agreement

- Conclude in RAN1 #120bis to either (1) introduce an optional modeling component for polarization variability for each cluster for NLOS component of the channel or (2) no consensus to introduce polarization variability for each cluster for NLOS component of the channel.

Agreement

For the modelling of spatial non-stationarity at UE side mainly due to close proximity, if blocker-based approach is adopted, the Option-3 is supported with following details:

- Three typical cases are introduced: one hand grip, dual hand grip, head and one hand grip.

  - probability of each case in the simulation is defined with the following table:

| Usage scenario | one hand grip | dual hand grip | head and one hand grip |
| --- | --- | --- | --- |
| Proportion | 58% | 29% | 13% |

- For each case, to model the attenuation for each of antenna in the candidate antenna location defined in the UE antenna model for handheld device with Alt 1:

  - Alt 1: Introduce a fixed value for each candidate antenna position per case.

- The percentage of the UE without impact from the SNS at UE side is 10%.

Related Working Assumption made in RAN1#119 doesn’t need to be confirmed.

Agreement

For the modelling of spatial non-stationarity at UE side, the fixed values of attenuation for candidate antenna locations for handheld device will be defined.

Note:

The fixed values of attenuation for candidate antenna locations for handheld device can be defined as:

| Antenna index | Power attenuation (dB) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | One hand grip |  |  |  | Dual hand grip |  |  |  | Head and one hand grip |  |  |  |
|  | Set-1@6GHz | Set-2@ [0.7-8.4] GHz | [Set-3@15GHz] |  | Set-1@6GHz | Set-2@ [0.7-8.4] GHz | [Set-3@15GHz] |  | Set-1@6GHz | Set-2@ [0.7-8.4] GHz | [Set-3@15GHz] |  |
|  |  |  | Alt-1 | Alt-2 |  |  | Alt-1 | Alt-2 |  |  | Alt-1 | Alt-2 |
| 1 | 0 | 1 | 1.0 | 0 | 7.31 | 9 | 3.8 | 9.68 | 10.57 | 1 | 4.0 | 14.16 |
| 2 | 0 | FFS | FFS | 0 | 0 | FFS | FFS | 0 | 7.08 | FFS | FFS | 9.87 |
| 3 | 0 | 3 | 3.8 | 0 | 7.31 | 9 | 3.8 | 10.6 | 0 | 3 | 3.3 | 0 |
| 4 | 6.15 | FFS | 3.6 | 9.13 | 6.15 | FFS | FFS | 9.12 | 0 | FFS | 3.3 | 9.13 |
| 5 | 7.31 | 12 | 3.8 | 10.6 | 0 | 5 | 1.0 | 0 | 6.55 | 12 | 3.3 | 10.6 |
| 6 | 0 | FFS | FFS | 0 | 0 | FFS | FFS | 0 | 10.12 | FFS | FFS | 9.70 |
| 7 | 0 | 1 | 1.0 | 0 | 0 | 5 | 1.0 | 0 | 10.57 | 3 | 4.0 | 14.16 |
| 8 | 0 | FFS | 1.0 | 0 | 6.15 | FFS | FFS | 10.62 | 10.67 | FFS | 4.0 | 14.27 |

- Note1: The selection of number of antenna and antenna location is determined as part of assumption in the calibration and simulation.

- Note2: For both one hand grip and Head and one hand grip cases, the handheld is assumed to be held by right hand as example.

- The standardized CTIA hand grip can be considered as reference.

Conclusion

For near-field channel, no changes are expected on following parameter for both direct and non-direct path between TRP and UE:

- Delay

Agreement

For near-field channel, for the non-direct paths, the distance d1 between the TRP and the 1st point associated with cluster is generated by Option-2:

- The distance d1 is generated by a scaling factor (sTRP,n) multiplied by the absolute distance corresponding to the absolute delay of cluster, where the scaling factor (sTRP,n) is generated by:

- For UMa scenarios, a Beta distribution with [α=1.8,β=1.22]:

- For UMi scenarios, a Beta distribution with [α=2.35,β=1.45]:

- For Indoor Office scenarios, a Beta distribution with [α=1.13,β=1.42]

- For the value of  k1 and  k2

- For UMa, the value of  k1 is 2.

- For UMi, the value of  k1 is 2.

- For InH, the value  k1 is [75%, 10.9%] of the total number of clusters for InH

- Note: the  k1 clusters are the strongest of the total number of clusters in the channel

- Note: the value of  k2 is equal to total number of clusters minus k1.

Agreement

Confirm the following working assumption.

- Working Assumption (made in RAN1#119)

  - For near-field channel, for the non-direct paths, the impact of spherical wavefront is optionally considered at UE side.

Agreement

For near-field channel, for number of  k2  non-direct paths, the distance d2 between the UE and the 2nd point associated with cluster is generated by:

- a scaling factor (sUE,n) multiplied by the absolute distance corresponding to the absolute delay of cluster.

Agreement

- For the modelling of spatial non-stationarity at BS side, if physical blocker-based approach is adopted, the nearest K blockers from BS are selected.

- For the modelling of spatial non-stationarity at BS side, if physical blocker-based approach is adopted, at least for blockage model B, the following parameters to define the building edge are introduced in the Table 7.6.4.2-5 in TR 38.901:

|  | Typical set of blockers | Blocker dimensions | Mobility pattern |
| --- | --- | --- | --- |
| Outdoor | Building edge | Cartesian: w=50 m; h=20 m | Stationary |

  - In the blockage model B, the following equation is used to calculate the attenuation caused by the building edge:

- LdB=-20log100.5-F, where F represents one of Fh1, Fh2, Fw1 and Fw2 defined in equation 7.6-29 in section 7.6.4.2 of TR 38.901.

Note: Corresponding Working Assumption made in RAN1#119 does not need to be confirmed.

Agreement

For near-field channel, the following formula is adopted to model the phase parameters of non-direct path between TRP and UE as antenna element-wise channel parameter:

- For the TRP side, the element-wise phase is calculated as:

expj2πd1,n-d1,n∙ rtx,n,m-dtx,sλ0

where d1,n is the generated distance of the cluster n. rtx,n,m is the spherical unit vector with azimuth departure angle and elevation departure angle for ray m of cluster n. dtx,s is the vector pointing from reference point to transmit antenna element s, wherein the reference point is the physical center of the antenna array/center at Tx side.

- For the UE side, the element-wise phase is calculated as:

expj2πd2,n-d2,n∙ rrx,n,m-drx,uλ0

where d2,n is the generated distance of the cluster n. rrx,n,m is the spherical unit vector with azimuth arrival angle and elevation arrival angle for ray m of cluster n. drx,u is the vector pointing from reference point to receive antenna element u, wherein the reference point is the physical center of the antenna array/center at Rx side.

#### 2.1.2	Remaining Open issues

Validate using measurements the channel model of TR38.901 at least for 7-24 GHz.

Adapt/extend as necessary the channel model of TR38.901 at least for 7-24 GHz, including at least the following aspects for applicable scenarios:

- Near-field propagation (with consideration being given to consistency between near-field and far-field)

- Spatial non-stationarity

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

#### 2.4.2	Remaining Open issues

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

- R1-2402009, “Considerations on the 7-24 GHz channel model validation,” Huawei, HiSilicon, Tongji University

- R1-2402010, “Considerations on the 7-24 GHz channel model extension,” Huawei, HiSilicon, Tongji University

- R1-2402090, “On Channel Model Validation of TR38.901 for 7-24GHz,” InterDigital, Inc.

- R1-2402091, “On Channel Model Extension of TR38.901 for 7-24GHz,” InterDigital, Inc.

- R1-2402128, “Work Plan for Study on 7-24 GHz Channel Modeling Enhancements,” Intel Corporation, ZTE

- R1-2402129, “Discussion on channel modeling verification for 7-24 GHz,” Intel Corporation

- R1-2402130, “Discussion on channel model adaptation/extension,” Intel Corporation

- R1-2402256, “Views on channel model validation of TR38.901 for 7-24GHz,” vivo

- R1-2402257, “Views on channel model adaptation/extension of TR38.901 for 7-24GHz,” vivo

- R1-2402397, “Discussion on channel model validation of TR38.901 for 7-24GHz,” CATT

- R1-2402398, “Discussion on channel model adaptation/extension of TR38.901 for 7-24GHz,” CATT

- R1-2402407, “Channel Model Validation of TR 38.901 for 7-24 GHz,” SHARP, NYU WIRLESS

- R1-2402480, “Discussion on channel model validation of TR38.901 for 7-24GHz,” Samsung

- R1-2402481, “Discussion on channel model adaptation/extension of TR38.901 for 7-24GHz,” Samsung

- R1-2402500, “Discussion of FR3 channel model,” Lenovo

- R1-2402601, “Discussion on Channel model validation of TR38.901 for 7-24GHz,” Nokia

- R1-2402602, “Discussion on Channel model adaptation/extension of TR38.901 for 7-24GHz,” Nokia

- R1-2402613, “Discussion on validation of channel model,” Ericsson

- R1-2402614, “Discussion on adaptation and extension of channel model,” Ericsson

- R1-2402619, “Skeleton of the CR for TR 38.901,” ZTE, Intel Corporation

- R1-2402620, “Discussion on the channel model validation,” ZTE

- R1-2402621, “Discussion on the channel model adaptation and extension,” ZTE

- R1-2402853, “Channel model validation of TR 38901 for 7-24 GHz,” NVIDIA

- R1-2402854, “Channel model adaptation of TR 38901 for 7-24 GHz,” NVIDIA

- R1-2402899, “On Channel Model Validation of TR 38.901 for 7-24 GHz,” Apple

- R1-2402900, “On Channel Model Adaptation/Extension of TR 38.901 for 7-24 GHz,” Apple

- R1-2402938, “Discussion on channel modelling enhancements for 7-24GHz for NR,” MediaTek

- R1-2403066, “Channel model adaptation/extension of TR38.901 for 7-24 GHz,” CEWiT

- R1-2403086, “Views on Channel Model Adaption/Extension of TR 38.901 for 7-24 GHz,” SHARP

- R1-2403144, “Discussion on Validation of the Channel Model in 38901,” AT&T

- R1-2403208, “Channel Model Validation of TR38.901 for 7-24 GHz,” Qualcomm Incorporated

- R1-2403209, “Channel Model Adaptation/Extension of TR38.901 for 7-24GHz,” Qualcomm Incorporated

- R1-2403261, “Changes to TR 38 901,” Spark NZ Ltd

- R1-2403267, “Discussion on channel model validation of TR38.901 for 7-24GHz,” LG Electronics

- R1-2403268, “Discussion on channel modelling adaptation/extension for 7-24GHz,” LG Electronics

- R1-2403280, “Discussion on channel model validation of TR38.901 for 7-24GHz,” BUPT, Spark NZ Ltd

- R1-2403285, “Discussion on modeling near-field propagation and spatial non-stationarity in TR38.901 for 7-24GHz,” BUPT, CMCC

- R1-2403366, “Channel model adaptation/extension of TR38.901 for 7-24 GHz,” Keysight Technologies UK Ltd

- R1-2403441, “Summary of issues for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation)

- R1-2403442, “Summary #1 of discussions for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation)

- R1-2403443, “Summary #2 of discussions for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation)

- R1-2403541, “Summary#1 of channel model adaptation and extension,” Moderator (ZTE)

- R1-2403608, “Summary#2 of channel model adaptation and extension,” Moderator (ZTE)

- R1-2403609, “Summary#3 of channel model adaptation and extension,” Moderator (ZTE)

- R1-2403630, “Summary #3 of discussions for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation)

- R1-2403631, “Summary #4 of discussions for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation)

- R1-2403665, “Session notes for 9.8 (Study on channel modelling enhancements for 7-24GHz for NR) ,” Ad-Hoc Chair (CMCC)

- R1-2403718, “Summary#4 of channel model adaptation and extension,” Moderator (ZTE)

- R1-2403856, “Discussion on Channel Model Validation of TR38.901 for FR3,” InterDigital, Inc.

- R1-2403857, “Discussion on Channel Model Extension of TR38.901 for FR3,” InterDigital, Inc.

- R1-2403878, “Views on Channel model validation of TR38.901 for 7-24GHz,” Sharp, NYU WIRELESS

- R1-2403907, “Discussion on channel model validation of TR38.901 for 7-24GHz,” LG Electronics

- R1-2403908, “Discussion on channel modelling adaptation/extension for 7-24GHz,” LG Electronics

- R1-2403925, “Considerations on the 7-24GHz channel model validation,” Huawei, HiSilicon

- R1-2403926, “Considerations on the 7-24GHz channel model extension,” Huawei, HiSilicon

- R1-2403962, “Discussion on channel modeling verification for 7-24 GHz,” Intel Corporation

- R1-2403963, “Discussion on channel model adaptation/extension,” Intel Corporation

- R1-2403969, “Template for Data Source Descriptions,” Intel Corporation, ZTE

- R1-2403991, “Discussion on validation of channel model,” Ericsson

- R1-2403992, “Discussion on adaptation and extension of channel model,” Ericsson

- R1-2403996, “Discussion on Channel model validation of TR38.901 for 7-24GHz,” Nokia, Anritsu

- R1-2403997, “Discussion on Channel model adaptation/extension of TR38.901 for 7-24GHz,” Nokia

- R1-2404129, “Discussion on channel model validation of TR38.901 for 7 - 24 GHz,” Samsung

- R1-2404130, “Discussion on channel model adaptation/extension of TR38.901 for 7 - 24 GHz,” Samsung

- R1-2404191, “Views on channel model validation of TR38.901 for 7-24GHz,” vivo

- R1-2404192, “Views on channel model adaptation/extension of TR38.901 for 7-24GHz,” vivo

- R1-2404212, “Discussion on the channel model validation,” ZTE

- R1-2404213, “Discussion on the channel model adaptation and extension,” ZTE

- R1-2404304, “Initial Measurement Results for Channel Model Validation,” Apple

- R1-2404305, “Channel Model Adaptation and Extension of TR38.901 for 7-24 GHz,” Apple

- R1-2404330, “Discussion on modeling near-field propagation and spatial non-stationarity in TR38.901 for 7-24GHz,” BUPT, CMCC, vivo

- R1-2404331, “Discussion on channel model validation of TR38.901 for 7-24GHz,” BUPT, Spark NZ Ltd, vivo

- R1-2404340, “Discussion of FR3 channel model,” Lenovo

- R1-2404415, “On channel model validation of TR38.901 for 7-24GHz,” CATT

- R1-2404416, “On channel model adaptation/extension of TR38.901 for 7-24GHz,” CATT

- R1-2404437, “Discussion on channel modeling for single road bridge (SRB) scenario,” China Telecom, BJTU

- R1-2404514, “Discussion on channel model validation of TR38.901 for 7-24GHz,” Sony

- R1-2404521, “Discussion on validation of channel model,” Vodafone, Ericsson

- R1-2404543, “Channel model validation of TR 38901 for 7-24 GHz,” NVIDIA

- R1-2404544, “Channel model adaptation of TR 38901 for 7-24 GHz,” NVIDIA

- R1-2404925, “Discussion on Validation of the Channel Model in 38901,” AT&T

- R1-2405082, “Discussion on channel modelling enhancements for 7-24GHz for NR,” MediaTek Inc.

- R1-2405169, “Channel Model Validation of TR38.901 for 7-24 GHz,” Qualcomm Incorporated

- R1-2405170, “Channel Model Adaptation/Extension of TR38.901 for 7-24GHz,” Qualcomm Incorporated

- R1-2405250, “Channel model adaptation/extension of TR38.901 for 7-24 GHz,” CEWiT

- R1-2405339, “Views on Channel model validation of TR38.901 for 7-24GHz,” Sharp, NYU WIRELESS

- R1-2405360, “Summary of issues for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation)

- R1-2405361, “Summary #1 of discussions for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation)

- R1-2405362, “Summary #2 of discussions for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation)

- R1-2405443, “Summary#1 of channel model adaptation and extension,” Moderator (ZTE)

- R1-2405444, “Summary#2 of channel model adaptation and extension,” Moderator (ZTE)

- R1-2405545, “Summary#3 of channel model adaptation and extension,” Moderator (ZTE)

- R1-2405546, “Summary#4 of channel model adaptation and extension,” Moderator (ZTE)

- R1-2405588, “Summary #3 of discussions for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation)

- R1-2405589, “Summary #4 of discussions for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation)

- R1-2405646, “Data source descriptions for 7 – 24 GHz SI,” Moderator (Intel Corporation)

- R1-2405698, “Session notes for 9.8 (Study on channel modelling enhancements for 7-24GHz for NR) ,” Ad-Hoc Chair (CMCC)

- R1-2405865, “Considerations on the 7-24GHz channel model validation,” Huawei, HiSilicon

- R1-2405866, “Considerations on the 7-24GHz channel model extension,” Huawei, HiSilicon

- R1-2405884, “On Angle Scaling for MIMO CDL Channel,” InterDigital, Inc.

- R1-2405885, “On Channel Model Extension for FR3,” InterDigital, Inc.

- R1-2405895, “Channel Model Validation of TR 38.901 for 7-24 GHz,” Sharp

- R1-2406007, “Discussion on channel modeling verification for 7-24 GHz,” Intel Corporation

- R1-2406062, “Discussion on channel modelling adaptation/extension for 7-24GHz,” LG Electronics

- R1-2406128, “Discussion on the channel model validation,” ZTE Corporation, Sanechips

- R1-2406129, “Discussion on the channel model adaptation and extension,” ZTE Corporation, Sanechips

- R1-2406139, “Discussion on Channel model validation of TR38.901 for 7-24GHz,” Nokia

- R1-2406140, “Discussion on Channel model adaptation/extension of TR38.901 for 7-24GHz,” Nokia

- R1-2406198, “Views on channel model validation of TR38.901 for 7-24GHz,” vivo

- R1-2406199, “Views on channel model adaptation/extension of TR38.901 for 7-24GHz,” vivo

- R1-2406252, “Discussion on channel model validation for 7~24GHz,” OPPO

- R1-2406253, “Discussion on channel model adaptation and extension,” OPPO

- R1-2406384, “Views on channel model validation of TR38.901 for 7-24GHz,” CATT

- R1-2406385, “Views on channel model adaptation/extension of TR38.901 for 7-24GHz,” CATT

- R1-2406393, “New measurement results for TR38.901 channel model validation and adaptation/extension consideration,” Keysight Technologies UK Ltd

- R1-2406485, “Further discussion on channel model validation of TR38.901 for 7-24 GHz,” Sony

- R1-2406490, “Channel model validation of TR 38901 for 7-24 GHz,” NVIDIA

- R1-2406491, “Channel model adaptation of TR 38901 for 7-24 GHz,” NVIDIA

- R1-2406518, “Discussion on channel model adaptation/extension for 7-24 GHz,” Fujitsu

- R1-2406666, “Discussion on channel model validation of TR38.901 for 7 - 24 GHz,” Samsung

- R1-2406667, “Discussion on channel model adaptation/extension of TR38.901 for 7 - 24 GHz,” Samsung

- R1-2406717, “Discussion on validation of channel model,” Ericsson

- R1-2406742, “Discussion on adaptation and extension of channel model,” Ericsson

- R1-2406743, “Discussion on near-field propagation and spatial non-stationarity,” BUPT, CMCC

- R1-2406744, “Discussion on channel model validation of TR38.901 for 7-24GHz,” BUPT, Spark NZ Ltd

- R1-2406766, “Discussion on channel modelling enhancements for 7-24GHz for NR,” MediaTek Inc.

- R1-2406858, “Discussion on validation of channel model,” Apple

- R1-2406859, “Channel Model Adaptation and Extension of TR38.901 for 7-24 GHz,” Apple

- R1-2406869, “Discussion on Validation of the Channel Model in 38901,” AT&T

- R1-2406946, “Discussion on channel model validation for 7-24 GHz,” NTT DOCOMO, INC.

- R1-2407045, “Channel Model Validation of TR38.901 for 7-24 GHz,” Qualcomm Incorporated

- R1-2407046, “Channel Model Adaptation/Extension of TR38.901 for 7-24GHz,” Qualcomm Incorporated

- R1-2407073, “Channel model adaptation/extension of TR38.901 for 7-24 GHz,” CEWiT

- R1-2407106, “Measurements of the angular spread in a suburban macrocell,” Vodafone, Ericsson

- R1-2407201, “Discussion on channel model adaptation/extension,” Intel Corporation

- R1-2407251, “Data source descriptions for 7 – 24 GHz SI,” Moderator (Intel Corporation)

- R1-2407252, “Summary of issues for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation)

- R1-2407253, “Summary #1 of discussions for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation)

- R1-2407254, “Summary #2 of discussions for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation)

- R1-2407255, “Summary #3 of discussions for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation)

- R1-2407290, “Summary#1 of channel model adaptation and extension,” Moderator (ZTE)

- R1-2407291, “Summary#2 of channel model adaptation and extension,” Moderator (ZTE)

- R1-2407292, “Summary#3 of channel model adaptation and extension,” Moderator (ZTE)

- R1-2407293, “Summary#4 of channel model adaptation and extension,” Moderator (ZTE)

- R1-2407467, “Summary #4 of discussions for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation)

- R1-2407481, “Session notes for 9.8 (Study on channel modelling enhancements for 7-24GHz for NR) ,” Ad-Hoc Chair (CMCC)

- R1-2407494, “Summary #5 of discussions for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation)

- R1-2407683, “Considerations on the 7-24GHz channel model validation,” Huawei, HiSilicon

- R1-2407684, “Considerations on the 7-24GHz channel model extension,” Huawei, HiSilicon

- R1-2407802, “Discussion on validation of channel model,” Ericsson

- R1-2407803, “Discussion on adaptation and extension of channel model,” Ericsson

- R1-2407874, “Views on channel model validation of TR38.901 for 7-24GHz,” vivo, BUPT

- R1-2407875, “Views on channel model adaptation/extension of TR38.901 for 7-24GHz,” vivo, BUPT

- R1-2407931, “Discussion on the channel model validation,” ZTE Corporation, Sanechips

- R1-2407932, “Discussion on the channel model adaptation and extension,” ZTE Corporation, Sanechips

- R1-2408060, “Discussion on channel model validation of TR38.901 for 7-24GHz,” CATT

- R1-2408061, “Discussion on channel model adaptation/extension of TR38.901 for 7-24GHz,” CATT

- R1-2408089, “Discussion on channel modelling adaptation/extension for 7-24GHz,” LG Electronics

- R1-2408095, “Discussion on channel model validation of TR38.901 for 7-24GHz,” BUPT

- R1-2408096, “Discussion on modeling near-field propagation and spatial non-stationarity in TR38.901 for 7-24GHz,” BUPT, CMCC

- R1-2408156, “Channel model adaptation and extension for 7-24GHz,” OPPO

- R1-2408195, “Discussion on Angle Scaling for MIMO CDL Channel,” InterDigital, Inc.

- R1-2408196, “Discussion on Channel Model Extension for FR3,” InterDigital, Inc.

- R1-2408244, “Channel model validation of TR 38.901 for 7-24 GHz,” Sharp

- R1-2408270, “Validation of Deterministic Radio Channel Model by 10 GHz Microcell Measurements,” Keysight Technologies UK Ltd

- R1-2408283, “Discussion on channel modeling verification for 7-24 GHz,” Intel Corporation

- R1-2408284, “Discussion on channel model adaptation/extension,” Intel Corporation

- R1-2408388, “Channel model validation of TR 38901 for 7-24 GHz,” NVIDIA

- R1-2408389, “Channel model adaptation of TR 38901 for 7-24 GHz,” NVIDIA

- R1-2408421, “Discussion of channel model validation of TR38.901 for 7–24GHz,” Sony

- R1-2408422, “Discussion of channel model adaptation/extension of TR38.901 for 7–24GHz,” Sony

- R1-2408484, “On Validation of Channel Model,” Apple

- R1-2408485, “On Channel Model Adaptation and Extension of TR38.901 for 7-24 GHz,” Apple

- R1-2408499, “Discussion on channel model adaptation/extension for 7-24 GHz,” Fujitsu

- R1-2408589, “Measurements of the angular spreads in a urban and suburban macrocells,” Vodafone, Ericsson

- R1-2408659, “Discussion on channel model validation of TR38.901 for 7 - 24 GHz,” Samsung

- R1-2408660, “Discussion on channel model adaptation/extension of TR38.901 for 7 - 24 GHz,” Samsung

- R1-2408705, “Discussion on channel modelling enhancements for 7-24GHz for NR,” MediaTek Inc.

- R1-2408757, “Discussion on Validation of the Channel Model in 38901,” AT&T

- R1-2408799, “Discussion on channel model validation for 7-24 GHz,” NTT DOCOMO, INC.

- R1-2408863, “Channel Model Validation of TR38.901 for 7-24 GHz,” Qualcomm Incorporated

- R1-2408864, “Channel Model Adaptation/Extension of TR38.901 for 7-24GHz,” Qualcomm Incorporated

- R1-2408895, “Discussion on Channel model validation of TR38.901 for 7-24GHz,” Nokia

- R1-2408896, “Discussion on Channel model adaptation/extension of TR38.901 for 7-24GHz,” Nokia

- R1-2408984, “Discussion on channel model validation of 3GPP TR 38.901 for 7-24 GHz,” Southeast University, Purple Mountain Laboratories, China Telecom

- R1-2408998, “Data source descriptions for 7 – 24 GHz SI,” Moderator (Intel Corporation)

- R1-2408999, “Summary of issues for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation)

- R1-2409000, “Summary #1 of discussions for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation)

- R1-2409001, “Summary #2 of discussions for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation)

- R1-2409002, “Summary #3 of discussions for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation)

- R1-2409003, “Summary #4 of discussions for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation)

- R1-2409152, “Summary#1 of channel model adaptation and extension,” Moderator (ZTE Corporation)

- R1-2409153, “Summary#2 of channel model adaptation and extension,” Moderator (ZTE Corporation)

- R1-2409154, “Summary#3 of channel model adaptation and extension,” Moderator (ZTE Corporation)

- R1-2409155, “Summary#4 of channel model adaptation and extension,” Moderator (ZTE Corporation)

- R1-2409402, “Considerations on the 7-24GHz channel model validation,” Huawei, HiSilicon

- R1-2409403, “Considerations on the 7-24GHz channel model extension,” Huawei, HiSilicon

- R1-2409441, “Discussion on validation of channel model,” Ericsson

- R1-2409442, “Discussion on adaptation and extension of channel model,” Ericsson

- R1-2409467, “Further Discussion on Angle Scaling for MIMO CDL Channel,” InterDigital, Inc.

- R1-2409468, “Further Discussion on Channel Model Extension for FR3,” InterDigital, Inc.

- R1-2409491, “Discussion on channel modelling adaptation/extension for 7-24GHz,” LG Electronics

- R1-2409610, “Discussion on channel model validation of TR38.901 for 7 - 24 GHz,” Samsung

- R1-2409611, “Discussion on channel model adaptation/extension of TR38.901 for 7 - 24 GHz,” Samsung

- R1-2409694, “Views on channel model validation of TR38.901 for 7-24GHz,” vivo, BUPT

- R1-2409695, “Views on channel model adaptation/extension of TR38.901 for 7-24GHz,” vivo, BUPT

- R1-2409724, “Discussion on the channel model validation,” ZTE Corporation, Sanechips

- R1-2409725, “Discussion on the channel model adaptation and extension,” ZTE Corporation, Sanechips

- R1-2409738, “Discussion on channel modeling verification for 7-24 GHz,” Intel Corporation

- R1-2409739, “Discussion on channel model adaptation/extension,” Intel Corporation

- R1-2409778, “Channel model validation of TR 38901 for 7-24 GHz,” NVIDIA

- R1-2409779, “Channel model adaptation of TR 38901 for 7-24 GHz,” NVIDIA

- R1-2409819, “Validation of Channel Model,” Apple

- R1-2409821, “Channel Model Adaptation and Extension of TR38.901 for 7-24 GHz,” Apple

- R1-2409954, “Discussion on channel model validation for 7-24GHz,” CATT

- R1-2409955, “Discussion on channel model adaptation/extension for 7-24GHz,” CATT

- R1-2410015, “Channel model validation for 7-24 GHz,” Lenovo

- R1-2410016, “Channel model extension for 7-24 GHz,” Lenovo

- R1-2410099, “Channel model adaptation and extension for 7-24GHz,” OPPO

- R1-2410131, “Discussion on channel model adaptation/extension for 7-24 GHz,” Fujitsu

- R1-2410170, “Draft CR/TP to incorporate the agreement for Rel-19 7-24GHz Channel model,” ZTE Corporation,Intel Corporation

- R1-2410213, “Discussion on channel model validation of TR38.901 for 7-24GHz,” BUPT

- R1-2410214, “Discussion on modeling near-field propagation and spatial non-stationarity in TR38.901 for 7-24GHz,” BUPT, CMCC

- R1-2410236, “Further Discussion of channel model validation of TR38.901 for 7–24GHz,” Sony

- R1-2410237, “Further discussion of channel model adaptation/extension of TR38.901 for 7–24GHz,” Sony

- R1-2410254, “Instantaneous directional channel measurements and parameter estimation at 14 GHz,” ROHDE & SCHWARZ

- R1-2410299, “Measurements of the angular spreads in a urban and suburban macrocells,” Vodafone, Ericsson

- R1-2410319, “Channel model validation of TR38.901 for 7-24 GHz,” Sharp, Nokia

- R1-2410320, “Views on channel modelling adaptation/extension for 7-24GHz,” Sharp, NYU WIRELESS

- R1-2410334, “Discussion on Validation of the Channel Model in 38901,” AT&T

- R1-2410402, “Discussion on channel model validation for 7-24 GHz,” NTT DOCOMO, INC.

- R1-2410491, “Channel Model Validation of TR38.901 for 7-24 GHz,” Qualcomm Incorporated

- R1-2410492, “Channel Model Adaptation/Extension of TR38.901 for 7-24GHz,” Qualcomm Incorporated

- R1-2410520, “Discussion on channel modelling enhancements for 7-24GHz for NR,” MediaTek Inc.

- R1-2410594, “Discussion on Channel model validation of TR38.901 for 7-24GHz,” Nokia

- R1-2410595, “Discussion on Channel model adaptation/extension of TR38.901 for 7-24GHz,” Nokia

- R1-2410628, “Channel model adaptation/extension of TR38.901 for 7-24 GHz,” CEWiT

- R1-2410631, “Data source descriptions for 7 – 24 GHz SI,” Moderator (Intel Corporation)

- R1-2410632, “Summary of issues for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation)

- R1-2410633, “Summary #1 of discussions for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation)

- R1-2410634, “Summary #2 of discussions for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation)

- R1-2410635, “Summary #3 of discussions for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation)

- R1-2410636, “Summary #4 of discussions for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation)

- R1-2410655, “Discussion on Validation of the Channel Model in 38901,” AT&T

- R1-2410656, “Discussion on channel model adaptation/extension,” Intel Corporation

- R1-2410767, “Summary#1 of channel model adaptation and extension,” Moderator (ZTE Corporation)

- R1-2410768, “Summary#2 of channel model adaptation and extension,” Moderator (ZTE Corporation)

- R1-2410769, “Summary#3 of channel model adaptation and extension,” Moderator (ZTE Corporation)

- R1-2410910, “Summary #5 of discussions for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation)

- R1-2500087, “Considerations on the 7-24GHz channel model validation,” Huawei, HiSilicon

- R1-2500088, “Considerations on the 7-24GHz channel model extension,” Huawei, HiSilicon

- R1-2500122, “Discussion on Evaluation of FR3 Channel Modeling,” InterDigital, Inc.

- R1-2500123, “Discussion on Extension of FR3 Channel Modeling,” InterDigital, Inc.

- R1-2500236, “Views on channel model validation for 7-24GHz,” CATT

- R1-2500237, “Views on channel model adaptation/extension for 7-24GHz,” CATT

- R1-2500252, “Discussion on channel modelling adaptation/extension for 7-24GHz,” LG Electronics

- R1-2500362, “Views on channel model validation of TR38.901 for 7-24GHz,” vivo, BUPT

- R1-2500363, “Views on channel model adaptation/extension of TR38.901 for 7-24GHz,” vivo, BUPT

- R1-2500380, “Discussion on the channel model validation,” ZTE Corporation, Sanechips

- R1-2500381, “Discussion on the channel model adaptation and extension,” ZTE Corporation, Sanechips

- R1-2500393, “Channel model validation of TR38.901 for 7-24 GHz,” Sharp

- R1-2500409, “Discussion on channel modeling verification for 7-24 GHz,” Intel Corporation

- R1-2500410, “Discussion on channel model adaptation/extension,” Intel Corporation

- R1-2500411, “Draft CR for Rel-19 7-24GHz Channel model,” Intel Corporation, ZTE Corporation

- R1-2500464, “Channel model adaptation and extension for 7-24GHz,” OPPO

- R1-2500506, “Discussion on validation of channel model,” Ericsson

- R1-2500507, “Discussion on adaptation and extension of channel model,” Ericsson

- R1-2500541, “Discussion on channel model validation of TR38.901 for 7-24GHz,” BUPT, Spark NZ Ltd

- R1-2500542, “Discussion on modeling near-field propagation and spatial non-stationarity in TR38.901 for 7-24GHz,” BUPT, CMCC

- R1-2500561, “Discussions on FR3 Channel Modelling,” Lekha Wireless Solutions

- R1-2500627, “Near-field channel modelling for FR3,” VIAVI Solutions

- R1-2500628, “Near-field channel modelling for FR3,” VIAVI Solutions

- R1-2500640, “Measurements of the angular spreads in urban and suburban macrocells,” Vodafone, Ericsson

- R1-2500661, “Discussion of channel model validation of TR38.901 for 7-24GHz,” Sony

- R1-2500677, “Channel model validation for 7-24 GHz,” Lenovo

- R1-2500678, “Channel model extension for 7-24 GHz,” Lenovo

- R1-2500693, “Channel model validation of TR 38901 for 7-24 GHz,” NVIDIA

- R1-2500694, “Channel model adaptation of TR 38901 for 7-24 GHz,” NVIDIA

- R1-2500798, “On Validation of Channel Model,” Apple

- R1-2500799, “On Channel Model Adaptation and Extension of TR38.901 for 7-24 GHz,” Apple

- R1-2500862, “Discussion on channel model validation of TR38.901 for 7 - 24 GHz,” Samsung

- R1-2500863, “Discussion on channel model adaptation/extension of TR38.901 for 7 - 24 GHz,” Samsung

- R1-2501028, “Discussion on channel modelling enhancements for 7-24GHz for NR,” MediaTek Inc.

- R1-2501048, “Intra cluster power profile,” Keysight Technologies UK Ltd

- R1-2501083, “Discussion on Validation of the Channel Model in 38901,” AT&T

- R1-2501168, “Channel Model Validation of TR38.901 for 7-24 GHz,” Qualcomm Incorporated

- R1-2501169, “Channel Model Adaptation/Extension of TR38.901 for 7-24GHz,” Qualcomm Incorporated

- R1-2501213, “Discussion on channel model validation for 7-24 GHz,” NTT DOCOMO, INC.

- R1-2501281, “Discussion of channel model adaptation/extension of TR38.901 for 7-24GHz,” Sony

- R1-2501348, “Discussion on Channel model validation of TR38.901 for 7-24GHz,” Nokia

- R1-2501349, “Discussion on Channel model adaptation/extension of TR38.901 for 7-24GHz,” Nokia

- R1-2501371, “Discussion on Validation of the Channel Model in 38901,” AT&T

- R1-2501426, “Data source descriptions for 7 – 24 GHz SI,” Moderator (Intel Corporation)

- R1-2501427, “Summary of issues for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation)

- R1-2501428, “Summary #1 of discussions for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation)

- R1-2501429, “Summary #2 of discussions for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation)

- R1-2501430, “Summary #3 of discussions for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation)

- R1-2501431, “Summary #4 of discussions for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation)

- R1-2501482, “Summary#1 of channel model adaptation and extension,” Moderator (ZTE)

- R1-2501483, “Summary#2 of channel model adaptation and extension,” Moderator (ZTE)

- R1-2501484, “Summary#3 of channel model adaptation and extension,” Moderator (ZTE)

- R1-2501485, “Summary#4 of channel model adaptation and extension,” Moderator (ZTE)

- R1-2501549, “Session notes for 9.8 (Study on channel modelling enhancements for 7-24GHz for NR),” Ad-Hoc Chair (CMCC)

- R1-2501638, “Summary#5 of issues for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation)

16.02.2024		minor adaptations for RAN #103

10.11.2023		minor adaptations for RAN #102

02.08.2023		minor adaptations for RAN #101

26.04.2023		minor adaptations for RAN #100

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template