3GPP TSG RAN Meeting #107						RP-250082

Incheon, Korea, March 12-14, 2025

Agenda item:	16

Source:	Ericsson

Title:	Technical Performance Requirements (TPR) for IMT-2030

Document for:	Approval

# Background

At TSG RAN#106, a study was initiated for 6G Scenarios and requirements [1], where in the first phase the objective is to investigate a candidate set of items for minimum TPRs based on the Recommendation ITU-R M.2160 [9] and to share the outcome with ITU-R WP5D as suitable. Based on the discussions at TSG RAN#106, a first LS response was sent to ITU-R WP5D in RP-243325 [2] with a list candidate TPR, plus statements on the carrier frequency and bandwidth to use for evaluation.

Meanwhile, the work in ITU-R WP5D continues on the new report on Technical Performance Requirements (TPR) for IMT-2030 [7] that corresponds to ITU-R report M.2410 [10] for IMT-2020.

This contribution gives input to the continuing work on the 6G scenarios and requirements study for TSGRAN, based on the ongoing discussions in WP5D on the TPR Working document [7] and WP5D input documents [3,4,5,6]:

- Candidate IMT-2030 TPR (Section 2)

- TPR Values (Section 3)

- TPR evaluation and mapping to test environments (Section 4)

- Evaluation configuration and parameters (Section 5)

- Requirements beyond the IMT-2030 TPR (Section 6)

The accompanying contribution in RP-250083 [12] gives further rationale, detailed discussions and numerical examples are given for four of the new TPR.

Text proposals for the new TR 38.8xx/9xx covering the proposed TPR are given in RP-250084 [13].

# Candidate IMT-2030 TPR

The ITU-R WP5D Working document for the draft new Report on TPR [7] lists technical performance requirements, based on the capabilities in ITU-R Recommendation M.2160 [9]. Definitions of the TPR in [7] are derived from that recommendation, the legacy definitions for IMT-2020 in Report ITU-R report M.2410 [10], plus additional input made to WP5D. Being a “working document” means that the text in [7] is under discussion has varying levels of stability, ranging from text with wide consensus to some proposed TPR with weak support within WP5D. No text is formally agreed.

The following is the preliminary list of TPR produced at WP5D:

- AI-related capabilities (TBD)		(not proposed in TSG RAN LS)

- Area traffic capacity (Quantitative)

- Average Spectral efficiency (Quantitative)

- Peak spectral efficiency (Quantitative)

- 5th percentile user Spectral efficiency (Quantitative)

- Bandwidth (Quantitative)

- Connection density (Quantitative)

- Control plane latency (Quantitative)

- User plane latency (Quantitative)

- Energy Efficiency (Quantitative)

- Mobility (Quantitative)

- Mobility interruption time (Quantitative)

- Peak data rate (Quantitative)

- User Experienced Data Rate (Quantitative)

- Sensing-related capabilities (Quantitative) 	(not proposed in TSG RAN LS)

- Positioning (Quantitative) 			(not proposed in TSG RAN LS)

- Reliability (Quantitative)

Of the TPR listed by WP5D above, we support all becoming IMT-2030 TPR (marked green). There are some questions concerning AI-related capabilities, but it has potential to be describes as a TPR, at least for inspection. We support AI to be evaluated by Inspection, but do not see how it could be quantified in a useful way.

Additional TPRs under consideration in WP5D are (not proposed in TSG RAN LS):

- [Joint requirement on data rate, latency, reliability and capacity]

- [Coverage]

- [Security]

- [Resilience]

- [Sustainability]	( Covered by Energy Efficiency)

- [Interoperability]

The “Joint requirement” would be a suitable addition to the ITM-2030 TPR, which could demonstrate new use cases for 6G and related performance. The name would need improvement though, where “Composite requirement” could be an alternative. For more details, see the background, discussion and numerical examples given in [12].

The candidate items marked grey above are proposed to not become TPR. Security, Resilience and Interoperability are could instead be covered by the submission template for IMT-2030. The same is true for Coverage, which will most likely be covered by link budgets and as such be provided in the submission template. Sustainability would be covered by the TPR for Energy Efficiency.

Further details on the three revised/new TPR (Energy Efficiency, Positioning, Sensing) are given in a separate contribution [12].

# TPR values

The LS from ITU-R WP5D in RP-243202 [14] asked for candidate items for TPR and also “proposed associated target values for the above candidate items for the minimum technical performance requirements which are requested to be provided preferably by WP 5D #49 (24 June – 3 July 2025, TBC)”. The RAN Chair’s Guidance for 6G Study (ITU Focused) in RP-243276 [15] states that initial discussion on the associated values of the identified TPRs should be held at RAN4#107. Table 1 below with proposed TPR values is an input to that discussion.

NOTE: 	In order for an LS to ITU-R to be treated at WP 5D#49 in June, it would have to be sent from 3GPP to the ITU-R Secretariat latest on 12 June 1600 UTC. This is on the fourth day of the RAN#108 meeting, so it should be doable to respond to ITU-R with TPR values with good preparation.

As a background to setting values for the TPRs, it is observed that:

- Traffic in mobile networks continues to increase.

- IMT-2030 should enable new use cases.

- Different emerging services for IMT-2030 rely on a different combination of TPR performance, such as data rate, delay, and reliability.

- The uptakes of the usage categories mMTC and URLLC introduced in IMT-2020 do not seem to be limited by the capabilities of the RIT.

Based on these observations, the TPR values in Table 1 below are proposed.

Table 1: Proposed TPR values for quantitative TPRs

| TPR | IMT-2020 req | IMT-2030 proposal | Comments |
| --- | --- | --- | --- |
| Peak data rate | 20 Gbps 10 Gbps | DL: 50 Gbps UL: 25 Gbps |  |
| User experienced data rate | 100 Mbps 50 Mbps | TBD | Proposed to revisit the evaluation methodology before setting values. |
| Peak spectral efficiency | 30 bps/Hz 15 bps/Hz | DL: 50 bps/Hz UL: 25 bps/Hz |  |
| Average and 5%tile spectral efficiency | DL: 3.3-9.0, 0.12-0.3 UL: 1.6-6.75,0.045-0.21 (bits/s/Hz) | 2-3x IMT-2020 | This is enabled with more antenna elements than used for IMT-2020. Different factors of improvements could be considered in different test environments. |
| Area traffic capacity | 10 Mbps/m2 | 30 Mbps/m2 |  |
| Connection density | 106/km2 | 106/km2 | Not seen as limiting for mMTC/MC usage. |
| Mobility | 10-500 km/h and 1.5-0.45 bps/Hz | TBD | Air-to-ground links may require higher speeds. Possibly that does not need to be represented by an IMT-2030 TPR. |
| Mobility Interruption Time | 0 ms |  |  |
| Latency (UP and CP) | 1 ms URLLC, 4ms MBB 20ms (10ms optional) | Same as IMT-2020 | It is not seen that the IMT-2020 values are limiting usage. Proposed to focus on the composite requirement on bitrate, delay and reliability TPR instead. |
| Reliability | 99.999% for 32B in 1 ms | Same as IMT-2020 | We do not see this as limiting for URLLC usage. |
| Bandwidth | 200 MHz | 2x IMT-2020 |  |
| Positioning | - | TBD |  |
| Sensing | - | TBD |  |
| Energy efficiency | (Inspection only) | TBD |  |
| Composite requirement |  | For XR, 30Mbps DL, 10Mbps UL, 40ms two-way delay with 99% reliability. For video conferencing, 1Mbps DL, 1Mbps UL, 100ms two-way delay with 98% reliability. | SNR and bandwidth TBD. |

# TPR mapping and evaluation

The test environments refer to the usage scenarios for IMT-2030. Usage scenarios that are expanded from IMT-2020 are Immersive communication (IC), Hyper Reliable and Low-Latency Communication (HRLLC), and Massive communication (MC). New usage scenarios are Ubiquitous connectivity (UC), Artificial Intelligence and Communication (AIAC) and Integrated Sensing and Communication (ISAC).

For the TPR proposed for IMT-2030, the type of evaluation and mapping to (provisional) test environments are shown in Table 2.

Table : Evaluation of TPR and mapping to Test environments

| TPR | Type | Evaluation | Test environments / Usage scenario |
| --- | --- | --- | --- |
| Peak data rate | Quantitative | Analytical | IC |
| User experienced data rate | Quantitative | Analytical, Simulation | Dense Urban – IC |
| Peak spectral efficiency | Quantitative |  | - |
| Average and 5th percentile spectral efficiency | Quantitative | Simulation | Indoor hotspot – IC Dense Urban – IC Rural – IC |
| Area traffic capacity | Quantitative | Analysis | Indoor hotspot – IC |
| Connection density | Quantitative | Simulation | MC |
| Mobility | Quantitative | Simulation | Indoor hotspot – IC Dense Urban – IC Rural – IC |
| Mobility Interruption Time | Quantitative | Analysis | - |
| Latency (UP and CP) | Quantitative | Analysis | eMBB HRLLC |
| Reliability | Quantitative | Simulation | [Indoor factory HRLLC] (to be considered) |
| Composite requirement | Quantitative | Simulation | Dense Urban – IC [Indoor factory – HRLLC] (to be considered) |
| Security | Submission template | - | - |
| Bandwidth | Quantitative | Inspection | - |
| Coverage | Submission template | - | - |
| Resilience | Submission template | - | - |
| Positioning | Quantitative | Simulation | Indoor Hotspot – IC Dense Urban – IC |
| Sensing | Quantitative | Simulation | Urban Macro – ISAC Rural - ISAC |
| AI | Qualitative | Inspection | - |
| Energy efficiency | Quantitative | Analysis | [IC] |
| Inter-operability | Submission template | - | - |

# Evaluation configurations and parameters

For the TPR listed above, several parameters need to be set to define the requirements while others will be defined for the evaluation. In ITU-R WP5D, a new report on Guidelines for the evaluation of IMT-2030 is being developed [8] that corresponds to ITU-R report M.2412 [11] for IMT-2020. The document is presently at an early stage, with much of the evaluation methodology inherited from IMT-2020.

As a first input to the update of the evaluation parameters, it is proposed to:

- Add a frequency band in the range ~7GHz to the existing set of IMT-2020 frequency bands.

- For the frequency band in the range ~7GHz, use up to four times more antennas elements than used for 4 GHz for IMT-2020, i.e. up to 1024 antenna elements.

- Consider using more antenna elements also for 30 GHz, at least the same number as for 7 GHz.

- Consider more than 10 users per cell for full buffer spectral efficiency evaluations.

# Requirements beyond the ITU-R IMT-2030 TPR

Requirements not covered by the IMT2030 TPRs, but still relevant for 3GPP include:

- Possible stricter requirement values for TPRs defined by ITU-R. As some of the values defined by the ITU-R may not be limiting, it is possible to refine these requirements in some of the evaluation scenario to improve on the ones proposed for the IMT-2030 evaluation.

- Maintaining the coverage footprint of LTE and NR when the same frequency bands and antennas are used. This can be verified by a new RAT supporting at least the same coupling loss as LTE and NR for control channels and data channels with the same data rate.

- Integration of terrestrial and non-terrestrial networks. Users should be able to experience seamless mobility between TN and NTN in remote areas. NTN should also be enabling connectivity for IoT devices almost everywhere.

- FWA is a key 5G use case that will benefit from potential 6G new spectrum and improvements of the RAN. Since FWA is not covered by the ITU-R, it is crucial that 3GPP define a set of requirements and scenarios for this use case.

# Proposals

The following is proposed:

- the items marked green in Section 2 to be pursued as TPR for IMT-2030.

- the values in Section 3 as minimum requirements for the IMT-2030 TPR.

- the new parameters in Section 4 for the evaluation of IMT-2030 TPR.

- the additional items in Section 5 for requirements beyond IMT-2030 TPR.

# References

- “New SID: Study on 6G Scenarios and requirements”, RP-243327, Source: CMCC, NTT DOCOMO, INC., Deutsche Telekom.

- “DRAFT LTI Response to LS on Minimum requirements related to technical performance for IMT-2030 radio interface(s) (ITUR_WP5D_TEMP_167”, RP-243325.

- “Proposals for the Working Document Towards a Preliminary Draft New Report ITU-R M.[IMT-2030.TECH PERF REQ]”, Source: Telefon AB – LM Ericsson, Document 5D/218, 13 June 2024.

- “Proposals for the Working Document Towards a Preliminary Draft New Report ITU-R M.[IMT-2030.TECH PERF REQ]”, Source: Telefon AB – LM Ericsson, Document 5D/387, 23 September 2024.

- “Proposals for the Working Document Towards a Preliminary Draft New Report ITU-R M.[IMT-2030.TECH PERF REQ]”, Source: Telefon AB – LM Ericsson, Document 5D/541, 24 January 2025.

- “Proposals for the working document towards a preliminary draft new Report ITU-R M.[IMT-2030.EVAL]”, Source: Telefon AB – LM Ericsson, Document 5D/540, 24 January 2025.

- “Working Document towards a Preliminary Draft New Report ITU-R M.[IMT-2030.TECH PERF REQ]: Minimum requirements related to technical performance for IMT-2030 radio interface(s)”, Annex 5.9 to Document 5D/563 (Working Party 5D Chairman’s Report).ITU-R Report M.2410-0 (11/2017), “Minimum requirements related to technical performance for IMT-2020radio interface(s)”.

- “Working Document towards Preliminary Draft New Report ITU-R M.[IMT-2030.EVAL]: Guidelines for evaluation of radio interface technologies for IMT-2030”, Annex 5.2 Document 5D/563 (Working Party 5D Chair’s Report).

- ITU-R Recommendation M.2160-0 (11/2023), “Framework and overall objectives of the future development of IMT for 2030 and beyond”.

- ITU-R Report M.2410-0 (11/2017), “Minimum requirements related to technical performance for IMT-2020radio interface(s)”.

- ITU-R Report M.2412-0 (11/2017), “Guidelines for evaluation of radio interface technologies for IMT-2020”.

- RP-250083, “Discussion of new TPR for IMT-2030”, Source: Ericsson.

- RP-250084, “TR 38.8xx/9xx text proposal for IMT-2030 TPR”, Source: Ericsson.

- RP-243202, “LS on Minimum requirements related to technical performance for IMT-2030 radio interface(s) (ITUR_WP5D_TEMP_167)”, Source: ITU-R WP5D.

- RP-243276, “RAN Chair’s Guidance for 6G Study: ITU Focused”, Source: RAN Chair.