3GPP TSG RAN Meeting #107						RP-250083

Incheon, Korea, March 12-14, 2025

Agenda item:	16

Source:	Ericsson

Title:	Discussion of new TPR for IMT-2030

Document for:	Discussion

# Background

In ITU-R WP5D, a new report on Technical Performance Requirements (TPR) for IMT-2030 is being developed [1] that corresponds to ITU-R report M.2410 [7] for IMT-2020. The report will define a set of minimum technical performance requirements for IMT-2030 candidate radio interface technologies.

The contribution in [2] proposes candidate TPRs, based on the ongoing discussions in WP5D on the TPR Working document [1] and WP5D input documents [4,5,6]. This contribution presents more details and proposed evaluation methodologies for the following TPR:

- Joint requirement on Data rate, Latency, and Reliability

- Positioning

- Sensing

- Energy efficiency

Detailed text proposals for the Working Document related to the TPR are given in [3].

# Composite requirement for Immersive Communication (IC) [and HRLLC] (Joint requirement)

In a paper to the previous TSG RAN [9], the background and rationale for a “Joint requirement” was given. There are presently several requirements discussed in the ITU-R that cover multiple TPR for various use cases proposed in the working document, including:

- Joint requirement on data rate, latency and reliability.

- Joint requirement on data rate, latency, reliability, and capacity.

- XR connection Density/connection Capacity.

- XR Area Capacity.

- XR Area Efficiency.

The idea is similar in that they evaluate TPR for a specific set of parameters based on example use cases and, in this way, intend to cover the expanded usage scenarios of IMT-2030. These proposals could all be covered by a single “Joint requirement” selecting the appropriate parameters..

Since the scope of the requirement would be broad and may cover multiple use cases and scenarios, as well as multiple “sub TPR”, the title should be generic and not include reference to any specific TPR. A new title “Composite requirement for Immersive Communication (IC) [and HRLLC]” is proposed.

In a previous contribution [Doc. 5D/387], numerical examples for a “joint requirement” covering data rate, latency and reliability were given, together with evaluation methodology with an example. The evaluation methodology should be expanded to cover also capacity, in order to cover the various proposals made to WP 5D. Possibilities of such an evaluation are shown below. A possible step-by-step evaluation is shown for the Capacity part in Section 2.2 with a numerical example in Section 2.3.

## Definition and evaluation including capacity

The joint requirement as described in [Doc. 5D/387] addresses the capability of the RIT to support the required bitrate, delay and reliability at given SNR and bandwidth. Specifying the SNR and bandwidth is a way to measure link efficiency; fulfilling the requirements cannot rely on infinite bandwidths or SNRs. There is however no direct measure of capacity, or how many simultaneous users with links fulfilling the requirements the RIT can support.

Capacity for ‘joint requirement’ services can be addressed in different ways. Candidates, in increasing order of detail and complexity, but also effort, include:

- No specific capacity evaluations. We assume that the joint requirement together with high full buffer spectral efficiency lead to system designs enabling high capacity for joint requirement services.

- A simple capacity assessment based on dividing the available system bandwidth W with the per-user bandwidth Wuser consumed for fulfilling the requirement. N = W / Wuser.

- As above, but also taking into account the fraction of time ftime consumed for fulfilling the requirement. N = W / (Wuser * ftime). Here ftime can be calculated as the traffic per time unit T generated by the user divided by its data rate when active R, i.e. ftime = T/R.

- As above, but also taking into account that the bandwidth and/or fraction of time depends on the SNR (the mandated SNR is not representative of all users in a cell) and combining that with the distribution of SNRs in a cell.

    - N = W / sum( Wuser(SNR) * ftime(SNR) *P(SNR)).

  - This is similar to the procedure used for connection density evaluations for IMT-2020.

- Full system level evaluations.

Of the four “ambition levels” for doing an evaluation, it is proposed to define Level 3 and 4. Allowing two evaluation approaches is similar to what was done for Connection density in IMT-2020. A step-by-step description of a Level 3 evaluation, with a numerical example is given below.

## Evaluation description based on Ambition level 3

Ambition level 3 for the joint requirement capacity evaluation given above is based it on the full-buffer evaluation methodology for connection density for IMT-2020 in ITU-R Report M.2412 [8].

The full-buffer evaluation methodology for connection density is described as follows in ITU-R Report M.2412 [8].

The following steps are used to evaluate the connection density based on full-buffer system-level simulation followed by link-level simulation. Traffic model used in this method is defined in Table 8-3 in § 8.4 of ITU-R Report M.2412 [8]:

Step 1: Perform full-buffer system-level simulation using the evaluation parameters for Urban Macro-mMTC test environment, determine the uplink SINRi for each percentile i=1…99 of the distribution over users, and record the average allocated user bandwidth Wuser.

In case UE multiplexing on the same time/frequency resource is modelled in this step, record the average number of multiplexed users Nmux. Nmux = 1 for no UE multiplexing.

Step 2: Perform link-level simulation and determine the achievable user data rate Ri for the recoded SINRi and Wuser values.

In case UE multiplexing on the same time/frequency resource is modelled in this step, record the average number of multiplexed users nmux,i under SINRi . The achievable data rate for this case is derived by Ri = Zi/nmux,i, where aggregated bit rate Zi is the summed bit rate of nmux,i users on Wuser. nmux,i = 1 for no UE multiplexing.

Step 3: Calculate the packet transmission delay of a user as Di = S/Ri, where S is the packet size.

Step 4: Calculate the traffic generated per user as T = S/Tinter-arrival, where Tinter-arrival is the inter-packet arrival time.

Step 5: Calculate the long-term frequency resource requested under SINRi as Bi = T/(Ri/Wuser).

Step 6: Calculate the number of supported connections per TRxP, N = W / mean(Bi). W is the simulation bandwidth. The mean of Bi may be taken over the best 99% of the SINRi conditions.

In case UE multiplexing is modelled in Step 1, N = Nmux × W / mean(Bi). In case UE multiplexing is modelled in Step 2, N = W / mean(Bi/nmux,i).

Step 7: Calculate the connection density as C = N / A, where the TRxP area A is calculated as A = ISD2 × sqrt(3)/6, and ISD is the inter-site distance.

When applied for the joint requirement, Step 3 can be removed, since the delay is given as a parameter to the joint requirement. Step 7 does not apply either if we measure capacity per TRxP.

## Example capacity evaluation for downlink XR

The above approach can be reused for the joint requirement. An example evaluation is presented here for XR in the downlink in a Dense Urban environment. An NR TDD system with W=100MHz bandwidth and a DDDSU TDD pattern is assumed.

In accordance with step 1 of the methodology, the first subplot in Figure 1 below shows an example distribution of SINRi.

The second subplot shows the 99th percentile delay as a function of SNR. This represents the first part of the joint requirement evaluation. A bandwidth of 50MHz is assumed in the link simulations (Wuser = 50MHz).

In subplot 3, the achievable datarate (R) is shown as a function of SNR. This is logged in the same link evaluation used to determine the delay. This is relation is used to determine Ri from SINRi.

The fourth subplot shows a distribution of Bi, calculated according to step 6. The average, mean(Bi) = 7.85MHz, is also noted. Using this together with the assumption of the system bandwidth W = 100MHz, we can calculate N = 100/7.85 = 12.8 simultaneous users.

Figure 1: Example full-buffer-based capacity evaluation for XR in the downlink

# Energy efficiency

## Background and choice of ambition level

In a previous contribution [9], it was noted that there are several possibilities for the ambition level of a requirement on Energy efficiency:

- An evaluation to demonstrate the capability to support a high sleep ratio and long sleep duration (Inspection; Similar to IMT-2020 evaluation).

- A quantitative evaluation to demonstrate the level of sleep ratio and long sleep duration supported by the RIT (Analysis; With TPR limits on sleep ratio and sleep duration).

- A quantitative evaluation to demonstrate the power savings of sleep ratio and sleep duration for the RIT using power models (Analysis without user data traffic; With TPR limits on relative energy savings).

- A quantitative evaluation to demonstrate the power savings of sleep ratio and sleep duration for the RIT using power models (Simulation with user data traffic; With TPR limits on relative energy savings).

It was argued in[9], that ambition Level 3 should be used for further work on Energy efficiency. Level 1) above is the same TPR as for IMT-2020, while Level 2) is based on the same type of evaluation, but with limits for the TPR (sleep ratio and duration). Level 3) improves over this considerably by applying a power model for the evaluation. This provides a tangible measure of power savings, and the way power is saved is concretized.

It was also argued in [9] that while ambition Level 4) would also model user data traffic in the system, that does not give a better estimate of energy efficiency, since the average physical resource utilization of a typical network is low and thus energy consumption is dominated by the energy cost of providing coverage. To make meaningful evaluations using ambition Level 4 requires detailed modelling of deployment, including multiple frequency bands and heterogenous networks consisting of more than one cell layer, as well as a detailed characterisation of realistic traffic is required. Defining such deployment and traffic models is complex and can become time consuming. Without sharing simulator code to ensure that the same assumptions are used in each part of the scenario definition of the simulation it is difficult to compare results from such detailed simulators.

For the reasons above, ambition Level 3 should be used for further work on Energy efficiency.

## Power model for evaluation

A power model used for evaluating the energy saving potential of a future 6G RIT should be future-looking and demonstrate that a RIT has potential for exploiting future advances on efficient power savings. One example of a power model with such properties is the future-looking 3GPP power model defined in 3GPP TR 38.864 [10]. This model could be used when evaluating the energy performance potential of a candidate RIT. Optionally, proponents may also present results using some other power models.

An evaluation example and proposed step-by-step procedure is given below.

## Evaluation methodology and example

A simple example evaluation of the energy consumption using ambition level 3 above is depicted in Figure 2 (from [Doc. 5D/387]). The leftmost bar shows a 5G NR reference system with 20 ms periodicity between SSB transmission, SIB1 transmissions, and PRACH receptions. The different colours indicate the cost in normalized power units of transmitting or receiving different signals; operating in different sleep states; and of waking up from different sleep states. In the rightmost bar another reference case is provided where the base station is in a continuous deep sleep state and no signals are transmitted or received. The intermediate six bars show the relative power used when increasing the SSB periodicity, SIB1 transmission periodicity, and PRACH reception periodicity to 40 ms, 80 ms, 160 ms, 320 ms, 640 ms, and 1280 ms, respectively. Interesting to note is that once the sleep duration exceeds 50 ms the base station can enter a deep sleep state. The relative power consumption obtained by extending the default transmission and reception periodicity of the system from 20 ms in 5G NR (ref) is written out above each bar.

Figure 2: Example energy consumption evaluation of different hypothetical RIT alternatives using ambition level 3. The left most bar represent a 5G NR reference case with 20 ms periodicity of transmissions and receptions and the rightmost bar represent a deep sleep reference level where nothing is transmitted or received.

Performing this type of “ambition level 3” evaluation is fairly simple, and it captures the majority of the potential energy consumption benefits a RIT has on a nationwide scale over time and does not require any complex definition of user traffic models and network deployment. The basic evaluation procedure can be summarized as follows:

- Define the required transmission and reception events on a timeline. The result of this step is a time domain representation of a signal to be transmitted and a time domain representation of signal reception attempts. The time domain transmit signal shall consist of all required signal components defined by the RIT when no users are connected or active. Typically, this includes all signals used by UEs in idle mode such as system synchronization signals, system information broadcast signals, and paging signals. In addition, the time domain reception attempts shall include processing to detect any signal that an idle mode UE may transmit to transit from an idle state (where no data is transmitted to and from the UE) to a connected or active state (where data is transmitted or may be transmitted to and from the UE), such as random-access transmissions.

- Jointly examine the transmission and reception timelines from the previous step and divide the timeline into sleep times separated by active times. Sleep times are characterized by no transmission and no reception occurring during a consecutive time window. Active times are defined by either something that needs to be transmitted, or some reception that needs to occur.

- For each active time, use the power model to determine the total required energy to transmit and receive all signals defined in this active time duration.

- For every sleep time, determine the sleep-state resulting in the lowest total energy, considering the duration of the sleep time, the required sleep state power, and the energy required to transition back and forth between the sleep state and an active state. In this step only sleep states with an activation time shorter than the sleep time duration are considered. Then apply this sleep state and calculate the total energy required for the sleep duration.

- Calculate the total energy by aggregating the energy over all active and sleep times. The average power can then be calculated as the total energy divided by the length of the total timeline.

The parameters of the power model used in steps 3 and 4 above will depend on the test environment that is selected. For energy efficiency analysis the following test environment are proposed to be used:

- Dense Urban-IC: An urban environment with high user density and traffic loads focusing on pedestrian and vehicular users.

- Rural-IC: A rural environment with larger and continuous wide area coverage, supporting pedestrian, vehicular and high speed vehicular users.

The test environment Dense Urban-IC and Rural-IC would use power model parameters as defined in 3GPP TR 38.864 [10].

Scaling of the power model parameters shall be applied according to the description in 3GPP TR 38.864 [10] in case any of the following parameters differing between the test environment (as defined in ITU-R M.[IMT-2030.EVAL])  and the “Reference scenario” (as defined in in 3GPP TR 38.864) [10]:

- number of used physical antenna elements, or TX/RX radio units (RuUs)

- occupied bandwidth (BW) / resource blocks (RBs) for DL and/or UL in a slot/symbol in one component carrier (CC)

- number of component carriers (CCs) in carrier aggregation (CA)

- number of transmission points (TRPs)

- Power spectral density (PSD) or transmit power

- number of DL and/or UL symbols occupied within a slot.

# Positioning

## Test environments for positioning

Positioning use cases will typically coexist with communication cases and/or will use the wireless network infrastructure – designed and deployed primarily for connectivity services – also for positioning services. Therefore, the positioning capability should be defined and evaluated assuming the usage scenarios that are identical or as close as possible to those used for the evaluation of communication services. More specifically, it is proposed to evaluate positioning in two representative usage scenarios in indoor and outdoor environments:

- Indoor Hotspot - Immersive Communication.

- Dense Urban - Immersive Communication.

In our view, it is sufficient to select these two geographical environments to test and evaluate the positioning capabilities, and these can be aligned with those used to evaluate communication related capabilities.

Note that the objectives, participating entities and associated propagation and channel models – and, consequently, some details of the suitable test environments –are markedly different for sensing and positioning. Specifically, the objectives of sensing include detecting and estimating the parameters of passive objects (targets) rather than localizing and tracking connected user equipment devices. Therefore, while positioning focuses on determining the location of a device with an active Tx/Rx circuitry and processing capabilities, testing of sensing requires some assumptions on the characteristics of the object of interest (such as radar cross-section).

Given that positioning shares and/or reuses test environments with communication, the guidelines for the evaluation of radio interface technologies must take positioning-specific evaluation needs into account. The channel model for indoor hotspot in IMT-2020 is detailed in ITU-R Report M.2412 [8]. The cluster power delay profile (PDP) model of ITU-R Report M.2412 [8] targets mobile broadband scenarios and lacks an excess delay needed to reflect the absolute time of arrival for each path in multipath environments. However, for testing and realistically evaluating the performance of positioning algorithms, especially in non-line-of-sight (NLoS) environments, more information embedded in the details of PDP becomes very important. Specifically, the relevant signal characteristics include signal strength, signal time delay, or a multi-path power delay profile, which shows the power and the arrival times of the different paths of the multi-path propagation channel between the selected Tx and Rx nodes. Location estimation is facilitated by the information provided by the PDP profile (containing e.g. the amplitudes and delays of the dominant multi-path components) of the environment.

The following positioning specific parameters have been identified:

- Propagation channel model: the evaluation of high accuracy methods requires a channel model capturing absolute time of flight. As an example, 3GPP TR 38.901 clause 7.6.9 models the excess delay using a random lognormal distributed variable.

- Carrier frequency: to capture a broad range of applications and devices, the requirement could be set for both low and high frequency bands, where lower bands would have priority.

- Antenna parameters for both outdoor and indoor usage scenarios: for indoor cases, transmitting nodes – set with short ISDs in dense deployment scenarios – assuming large antenna size are not feasible. To facilitate testing and evaluating both indoor and outdoor usage scenarios, the antenna configuration should be left flexible, specifying only the maximum number of antenna elements. This will facilitate testing multiple antenna configurations, e.g. 4x2, 8x4 antenna arrays.

# Sensing-related capabilities

## Selected tasks and related KPIs

In a previous contribution to TSG RAN [9], a set of sensing tasks enabling a number of use cases were defined. Based on those, KPIs were defined for each task, forming a sensing TPR.

Those recommended KPIs are listed here, in the order of their priority for inclusion into the set of sensing TPRs, in the order of their priority:

- KPIs related to presence/absence detection of a passive object (correct detection probability, PD, and false alarm probability, PFA).

- KPIs related to velocity estimation (accuracy).

- KPIs related to localization of passive objects (horizontal and vertical accuracies).

- KPIs related to object classification (correct classification probability).

- KPIs related to mapping (RMSE).

A minimum set for a sensing TPRs should include the passive object detection sensing task and its KPIs (shown without square brackets in Table 1). The inclusion of other sensing tasks and their KPIs can be further discussed but have lower priority. For this reason, they are shown in square brackets in Table 1.

Table 1: Sensing requirements

|  | SENSING TASKS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test environment | Object presence detection |  | [Velocity] | [Passive object localization] |  | … |
|  | PD | PFA | [Accuracy] | [Horizontal location accuracy] | [Vertical location accuracy] | … |
| [Urban macro – ISAC @ Ground level] | TBD1 | TBD1 | [TBD] | [TBD] | [TBD] | … |
| [Urban macro – ISAC @ Aerial vehicles] | TBD2 | TBD2 | [TBD] | [TBD] | [TBD] | … |
| [Rural – ISAC @ Ground level] | TBD1 | TBD1 | [TBD] | [TBD] | [TBD] | … |
| [Rural – ISAC @ Aerial vehicles] | TBD2 | TBD2 | [TBD] | [TBD] | [TBD] | … |
| NOTE 1: Applicable for ground-level objects. NOTE 2: Applicable for low-altitude aerial vehicles. |  |  |  |  |  |  |

If localization is to be a part of TPR, then both horizontal and vertical accuracy is to be considered, in cartesian coordinates. Polar coordinates could also be suitable for mono-static sensing, where each point on a plane is determined by a distance from a reference point and an angle (2D polar system) or two angles (3D polar system) from a reference direction. However, since the TPR should be generic and not limited to a specific sensing deployment, cartesian coordinates for passive object localization are proposed.

## TPR applicability

The requirements shall apply per sensing task supported by the RIT. In a network deployment, some of the sensing tasks can be mandatory for a communication network supporting sensing service. Other sensing tasks can be optional and indicated by the capability.

The TPR should be agnostic to the type of sensing deployment (e.g., mono-static and bi-/multi-static), i.e., the same requirements should apply regardless of whether it is mono-static or bi-/multi-static.

## Test environments

The following test environments are proposed to be prioritized for sensing performance evaluations (see [1] for more details):

- Urban macro – ISAC @ Ground level

- Urban macro – ISAC @ Aerial vehicles

- Rural – ISAC @ Ground level

- Rural – ISAC @ Aerial vehicles

## Performance evaluation procedure and parameters

Evaluation procedure for sensing can follow the same basic principles described for IMT-2020 in section 7.1, ITU-R Report M.2412 [8], e.g. based on a network layout with multiple three-sector sites over a regular grid as shown in. Figure 2 in ITU-R Report M.2412 [8],. In scenarios with UE involvement, if such agreed, random dropping of fixed-location UEs can be modelled.

However, sensing-specific modelling aspects need to be also included in the simulation assumptions, e.g.:

- Sensing signal model

- Dropping rules for passive (non-connected) sensing target objects

- Selecting relevant transmitters and receivers for each sensing target

- Sensing target object characteristics, e.g., mobility, size

- Sensing channel model capturing the sensing object reflections and the clutter impact,

- Interference modelling.

Simulation parameters in addition to the deployment parameters for the test environments can be as shown in Table 2.

Table 2: Additional simulation parameters for sensing

| Characteristic |  | Comments |  |
| --- | --- | --- | --- |
| Layout |  | As in § 8.3 of the Working Document (see 5D/413 Annex 5.2) |  |
| Link level abstraction |  | Explicit simulation of all links. |  |
| Radio sensing signal |  | Tx: BS1, Rx: BS2 | [Tx: BS, Rx: UE] |
| Radio sensing channel model |  | E.g. geometry-based stochastic channel model. Needs to capture the sensing object reflections and the clutter impact. |  |
| BS | BS antenna radiation pattern and configuration | Directional (e.g., reuse network planning for communication without direction optimization for sensing, otherwise configuration is to be clarified by proponent) |  |
|  | BS antenna height |  |  |
|  | BS tx power for sensing signals | No boosting 49 dBm (20 MHz) |  |
| UE (for scenarios with UE involvement) | UE dropping procedure | - | [Random, based on some rules for fixed-location UEs] |
|  | UE antenna radiation pattern and configuration | - | [Omni (0dBi) or directional] |
|  | UE height | - | [3m] |
|  | Min. BS-UE distance (2D), m | - | [35m] |
|  | UE mobility (for modelling Doppler effects) | - | [0km/h (fixed UEs)] |
| Passive (non-connected) sensing target object | Object dropping procedure | Random, based on some rules. E.g. N per cell (N≥1). |  |
|  | Object height | Depends on object type. UAV: Random within a range. |  |
|  | Object mobility | Depends on object type |  |
|  | (tx,rx) pairs for the object | [Rule-based TBD] |  |
| Interference model |  | TBD |  |

# Summary

The discussion in this paper presents background and rationale concerning four of the TPR proposed for IMT-2030, to be used in the continued work on TPR within the TSG RAN “Study on 6G Scenarios and requirements”. Related proposals concerning all TPR are made in [3] with accompanying text proposals for a Technical Report in [3].

# References

- “Working Document towards a Preliminary Draft New Report ITU-R M.[IMT-2030.TECH PERF REQ]: Minimum requirements related to technical performance for IMT-2030 radio interface(s)”, Annex 5.7 to Document 5D/413 (Working Party 5D Chairman’s Report).

- RP-250082, “Technical Performance Requirements (TPR) for IMT-2030”, Source: Ericsson.

- RP-250084, “TR 38.8xx/9xx text proposal for IMT-2030 TPR”, Source: Ericsson.

- “Proposals for the Working Document Towards a Preliminary Draft New Report ITU-R M.[IMT-2030.TECH PERF REQ]”, Source: Telefon AB – LM Ericsson, Document 5D/218, 13 June 2024.

- “Proposals for the Working Document Towards a Preliminary Draft New Report ITU-R M.[IMT-2030.TECH PERF REQ]”, Source: Telefon AB – LM Ericsson, Document 5D/387, 23 September 2024.

- “Proposals for the Working Document Towards a Preliminary Draft New Report ITU-R M.[IMT-2030.TECH PERF REQ]”, Source: Telefon AB – LM Ericsson, Document 5D/541, 24 January 2025.

- ITU-R Report M.2410-0 (11/2017), “Minimum requirements related to technical performance for IMT-2020radio interface(s)”.

- ITU-R Report M.2412-0 (11/2017), “Guidelines for evaluation of radio interface technologies for IMT-2020”.

- RP-242025, “Proposed Technical Performance Requirements (TPR) for IMT-2030”, Source: Ericsson.

- 3GPP TR 38.864 V18.1.0 (2023-03), Technical Report, “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on network energy savings for NR (Release 18)”.