3GPP TSG RAN Meeting #107																RP-240201

Incheon, Korea, March 12-14, 2025

Source:	vivo

Title:	Scope of MIMO phase 6 in 5G-A Rel-20

Agenda Item:	15.2.2

Document for:	Discussion and decision

Introduction

In [1], RAN chair summarized following on NR MIMO enhancement in Rel-20.

As noted, enhancement on uplink performance, especially robust uplink transmission in moderate to high speed scenario is of utmost importance. In [2], discussion on scope of Rel-19 NR MIMO Phase 5 scope extension was summarized and “item E” was supported by many operators from different regions. In [3], we provided detailed analysis with simulation results on open-loop sub-band precoder cycling to exploit multiple transmit antennas at the UE and multiple Tx antennas are non-coherent in real deployment. The evaluation results are provided in appendix of this contribution for reference.

There were other items proposed in contributions submitted for Rel-20 NR MIMO enhancement in previous RAN meeting. Considering limited available TUs for 5G-A and MIMO enhancement in Rel-20, only few small items with commercial value for the operators shall be considered. As in previous releases, the leading WG for this item is RAN1 while RAN2 and RAN4 will also be involved.

Potential topics for Rel-20 NR MIMO enhancement

As study on 6G technology as well as 5G-A enhancement will happen in parallel, overall TU allocation for 5G-A enhancement will be limited hence it can be assumed that available TU for Rel-20 NR MIMO will be very limited. Only few precisely defined objectives can be accommodated, below we provide our views on the few topics with the assumption that not all of them can be accommodated in Rel-20 NR MIMO Phase 6 WI.

- Robust uplink transmission – open-loop precoder cycling (highest priority)

To proliferate UL MIMO implementation in real deployment, there should be strong motivation for device vendors to put more components in the device, which definitely increases cost and complexity. Let’s take an example of 2Tx UL MIMO with non-coherent codebook, where there are 2 rank=1 precoders and 1 rank=2 precoder. In the non-coherent codebook subset, the rank=1 precoders are antenna selection precoders, i.e., [1  0]T and [0  1]T, which means only 1 antenna will be used for UL transmission when gNB schedules PUSCH with rank=1. Rank=2 scheduling most likely happens in high SINR/low mobility scenario where the wireless channel is in better condition, however in real network we observed that rank=2 scheduling also happens in moderate/high speed scenario. Rank=2 transmission increases peak data rate, however from overall user experience point of view it is important to boost performance for rank=1 transmission especially in moderate/high speed scenario. For example, for UL intensive services like video conference, live streaming etc., robust transmission is important than peak throughput. In Appendix, relevant simulation results can be found. With the above justification, we have following proposal for Rel-20 NR MIMO enhancement.

Proposed Objective

- Specify necessary enhancement to support open-loop sub-band precoder cycling for PUSCH transmission

- Early SRS/CSI triggering

We see the motivation of early triggering of SRS/CSI to obtain coarse channel information, e.g., for enhancing DL transmission after RRC connection complete and before RRC configuration of CSI related parameters when DCI format 1_0 is used for PDSCH scheduling. We can be supportive to include this item in Rel-20 NR MIMO enhancement with very specific scope.

- CSI-RS overhead reduction

Rel-19 MIMO enhancement supported gNB antenna ports up to 128 ports, we see the motivation of CSI-RS reduction. We can be supportive of this item, we also expect similar discussion will happen for 6G in parallel.

- 2Tx UL full-coherent codebook enhancement

NR Rel-15 supports 2Tx UL MIMO, for the codebook-based UL transmission there are non-coherent codebook and full-coherent codebook specified. However, only non-coherent codebook is implemented in real products. We don’t foresee full-coherent UE will be available in near future, however we can be supportive of this item with limited scope.

- Further enhancement of SRS

NR Rel-15 supports multi-symbol SRS with/without hopping and further enhancement has been supported in later releases. We are open for discussion on SRS related enhancement if there is clear benefit with limited scope.

- PRACH beam indication, SRS beam sweeping in asymmetric DL/UL mTRP

Asymmetric DL/UL multi-TRP is being specified in Rel-19 MIMO. We are open for further discussion, can be supportive if the scope is limited.

- UEI/ED beam switching based on Rel-19

UE-initiated beam reporting is being specified in Rel-19 NR MIMO WI, UE-initiated beam switching was also discussed while drafting the WID. It is not clear what additional benefit can be achieved over Rel-19 UEIBM, however we are open for further discussion.

- IMR measurement report for reciprocity-based operation

NR MIMO has been evolved in successive releases supporting very flexible framework suitable for different deployment scenarios. Given the relatively small scope of the WI, this item has lower priority for us, but we are open for further discussion.

- Dynamic QCL update for common signals (e.g. CSI -RS for tracking)

Considering limited scope and limited available TUs, this item has lower priority for us, but open for further discussion.

- Support of rank>1 for DFT-s-OFDM waveform in UL

This issue has been discussed since Rel-15, in our view it is not suitable for the late stage of NR and as an alternative technology, dynamic waveform switching has been already supported in the specification enabling dynamic rank adaptation when DFT-s-OFDM waveform is configured.

Conclusions

In this contribution, we discussed few potential topics for Rel-20 MIMO enhancement, UL robust transmission in moderate/high scenario is of utmost important. We strongly propose to include following objective in Rel-20 NR MIMO Phase 6 WI.

Proposed Objective

- Specify necessary enhancement to support open-loop sub-band precoder cycling for PUSCH transmission

Reference

[1] RP-243292, “RAN Chair’s Summary of 5G-Advanced in Rel-20”, RAN Chair

[2] RP-242367, “Way Forward on Rel-19 MIMO Scope Expansion”, RAN1 Chair

[3] RP-242630, “Views on MIMO phase 6 in 5G-A Rel-20”, vivo

Appendix

Evaluation on Performance gain

Simulations assumptions:

- UL: 2Tx non-coherent UE

- Open loop precoder cycling for Rank=1 transmission using 4 full-coherent precoders in 38.211

  - Baseline 0: Rel-15 non-coherent codebook, where the max output power is 3dB less than UE declared power class

  - Baseline 1: Rel-16 non-coherent codebook, full power mode 0, where the UE can deliver max output power as declared for given power class

| Parameter | Value |
| --- | --- |
| Frequency Range | 3GHz |
| Antenna setup and port layouts at gNB | (2,8,2,1,1,2,8), (dH, dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE | 2TX: (1,1,2,1,1,1,2), (dH, dV) = (0.5, 0.5)λ |
| Receiver noise figure | 5dB |
| Modulation | Up to 64QAM |
| Numerology | 14 OFDM symbol slot |
|  | SCS 15KHz, 50 PRB |
| MIMO scheme | SU-MIMO, rank=1 |
| SRS | 2-port SRS, 10 and 80ms periodicity |
| PRG size | 2 PRBs |
| UE speed | 30/60/120 Km/h |

Simulation results

Figure 1, Precoder cycling performance gain against baseline 0 (Rel-15 non-coherent codebook)

Precoder cycling gain against baseline 1

Figure 2, Precoder cycling performance gain against baseline 1 (Rel-16 non-coherent codebook with full power mode 0)

Observation 1: from the figures 1 and 2, significant performance gain can be observed with open-loop precoder cycling, especially at 5 percentile UE SE, against closed-loop precoding with non-coherent assumption at the UE in high speed scenario. The performance gain is not sensitive to SRS periodicity; hence SRS overhead can be significantly reduced. Furthermore, SRS and DMRS are used as specified in the current specification without enhancement, MCS is determined based on received SRS and gNB applies precoder cycling for MCS determination.

Evaluation comparison with PUSCH frequency hopping

Simulation assumptions:

| Parameter | Value |
| --- | --- |
| Frequency Range, Waveform, SCS | Fc=4GHz， CP-OFDM, SCS=15KHz |
| Wireless Channel | TDL-C, low correlation, DS=250ns, 30km/h, UE 2/4Tx and gNB 4Rx |
| PUSCH code rate | 32PRB，64QAM and code rate=0.6 (w/o FH), No Retransmission |
| Type A DMRS | [3,11] without Intra-slot Frequency Hopping [2, 6] and [0, 4] with Intra-slot Frequency Hopping |
| Precoding scheme | FullPowerMode0; Small Delay-CDD; Precoding cycle with PRG=4 PRB |
| Receiver | Channel estimation: MMSE filter Equalizer: MMSE-based |

PRG= 4PRBs

slot

32 PRBs

16 PRBs

16 PRBs

Slot，intra-slot FH

Figure 3, DMRS configuration for with and without Frequency Hopping

Figure 3 shows DMRS configuration for both with and without intra-slot frequency hopping PUSCH for high speed scenario where additional DMRS is configured within a slot. For intra-slot frequency hopping PUSCH, there are 2 DMRS symbols on each hop within a slot, hence PUSCH code rate is higher compared to without frequency hopping for the same TB size. In the simulations below, the code rate for PUSCH without frequency hopping is 0.6 with 64QAM modulation.

Simulation results

- UL 2Tx, single layer PUSCH                                            b) UL 4Tx, single layer PUSCH

Figure 4, performance comparison with PUSCH intra-slot FH,

Figure 4 shows the simulation results for different schemes with single layer PUSCH transmission. In the figures, FullPowerMode0 is Rel-16 non-coherent codebook with full power mode 0, where the precoders for rank=1 are [1   0] and [0   1], however each antenna can deliver full power according to power class. For small delay CDD, 2Tx with [0, CP/6] delay and 4Tx with [0, CP/8, CP/4, 3CP/8] delays are applied on different antennas. For precoder cycling, length 4 DFT vectors are applied per PRG, where PRG size is 4 PRBs. For single layer transmission, it can be noticed that performance of precoder cycling and small delay CDD are similar, however to support dynamic rank adaptation there will be additional SRS overhead for small delay CDD. Furthermore, in the case of 4Tx, how to apply small delay CDD for rank=2 is not clear.

Observation 2: Since there is additional DMRS overhead for intra-slot frequency hopping, BLER performance is worst compared to Rel-16 UL full power mode 0, small delay CDD and precoder cycling. To support dynamic rank adaptation there will be additional SRS overhead for small delay CDD.

Evaluation on PAPR

Below we compare PAPR for the case with and without UL precoder cycling.

Figure 5, PAPR comparison between with and without open-loop UL Precoder cycling

Observation 3: figure 5 shows comparison of PAPR between the cases of with and without UL precoder cycling for CP-OFDM waveform, it can be observed that there is no PAPR degradation.

Evaluation on interference fluctuation

Figure 6, comparison of interference across all cells between precoder cycling and wideband scheduling

Observation 4: figure 6 shows comparison of interference fluctuation across all cells between precoder cycling and wideband scheduling, it can be noticed that the CDF curves are similar, hence there is no drastic change in interference due to precoder cycling in UL.

Proposal 1:

- It is proposed to support sub-band precoder with precoder cycling for UL transmission, specify necessary enhanced including definition of PRG (Precoding RB Granularity).