3GPP TSG RAN Meeting #107                                                       RP-250210

Incheon, Korea, March 12-14, 2025

Source:	vivo

Title:	Discussion on IMT-2030 technical performance requirements (TPR)

Agenda Item:	16

Document for:	Discussion and Decision

Introduction

A new study item of Study on 6G Scenarios and requirements has been approved in RAN #106 [1], in which the objectives of the study for ISAC are as follows:

This study item aims to

- investigate a candidate set of items for minimum TPRs based on the Recommendation ITU-R M.2160, and, where applicable, the associated target values and key assumptions for the identified minimum TPRs.

  - The outcome is expected to be shared by LS with ITU-R WP5D, as suitable, and used as a baseline for the subsequent study 6G in RAN

[To fill in additional objectives, on general 6G RAN study, in March’2025]

In RAN #106, 3GPP TSG RAN has made initial considerations of possible TPRs as following, the definitions of these items will be further discussed.

1.	Peak data rate

2.	Peak spectral efficiency

3.	User experienced data rate

4.	5th percentile user spectral efficiency

5.	Average spectral efficiency

6.	Sustainability/Energy efficiency

7.	Area traffic capacity

8.	User plane latency

9.	Control plane latency

10.	Connection density

11.	Reliability

12.	Mobility

13.	Mobility interruption time

14.	Bandwidth

In this contribution, we summarize the status and progress of technical performance requirements (TPR) discussion in ITU-R WP5D and express our views on the TPR.

Current status of IMT-2030 TPRs in ITU-R WP5D

The technical performance requirements (TPR) for IMT-2030 are currently under discussion in ITU-R WP5D SWG radio aspect. According to the ITU-R WP5D timeline, as shown in Figure 1, TPRs have been discussed in the past four meetings and a working document towards a preliminary draft new report ITU-R M.[IMT-2030. TECH PERF REQ] [2] is being developed.

For each TPR, the definition, the requirement values and the applicable usage scenarios are to be studied and determined. Based on the input contributions, the working document and the online/offline discussion in the ITU-R WP5D meetings, the following observations on the progress of each TPR are given:

1) For some traditional TPRs, such as peak data rate, peak spectrum efficiency, the definition and applicable usage scenarios are clear and close to agreement, but the requirement values need more study.

2) For some other traditional TPRs, such as user experienced data rate, average spectrum efficiency etc., their definitions are clear and close to agreement, but their applicable usage scenarios are still under discussion.

3) New TPRs associated with new capabilities, such as sensing-related capability, AI-related capability and sustainability, their definition are still under discussion, including whether these TPRs will be defined as qualitative metric or quantitative metric or mixed metric.

In ITU-R WP5D #48 meeting, AI-related capability and energy efficiency were discussed in DG AI and DG Energy Efficiency, and some agreements were achieved. For AI-related capabilities, majority preferred to define qualitative TPR. A possible quantitative requirement on “Network for AI” which is AI Service Latency is proposed and need further discussed. Possible quantitative requirements on “AI for Network” such as define different target requirement values for avg. spectral efficiency, user experienced data rate, etc., are proposed and need further discussed. For energy efficiency, quantitative TPR for immersive communication and separated requirements for network and UE were agreed. Regarding the calculation of energy efficiency, majority agreed to use the power model (power unit) to calculate the relative power saving/energy consumption. In addition, the impacts on communication performances also need consideration.

4) New TPRs associated with joint capabilities, such as joint requirement on data rate, latency, and reliability, joint requirement on data rate, latency, reliability and capacity, XR connection density/connection capacity, XR area capacity, are proposed in the input contributions and discussed in several meetings.

In ITU-R WP5D #48meeting, joint requirement was discussed in DG Joint Requirements, and some agreements were achieved. This TPR was agreed to be as joint requirement on data rate, latency, reliability and capacity. Regarding the capacity, majority preferred the capacity is number of users, another option is total traffic throughput. Regarding the applicable usage scenarios for the joint requirement, immersive communication was agreed and HRLLC need further discussion. In addition, the final name of the joint requirement is under discussion.

Figure 1 ITU-R WP5D timeline

In the following Table 1, we summarize the discussion status for each TPR.

Table 1  TPR discussion status in ITU-R WP5D

|  | TPR | Definition | Requirement values | Requirements recommendation example in M.2160 | Usage scenarios |
| --- | --- | --- | --- | --- | --- |
| 1 | Peak data rate | Clear Quantitative | TBD | 50, 100, 200 Gbit/s | Immersive Communication |
| 2 | User experienced data rate | Clear Quantitative | TBD | 300 Mbit/s, 500 Mbit/s | TBD Immersive Communication [and Ubiquitous Connectivity] |
| 3.1 | Peak Spectrum Efficiency | Clear Quantitative | TBD | TBD | Immersive communication |
| 3.2 | Average spectrum efficiency | Clear Quantitative | TBD | 1.5 and 3 times greater than that of IMT-2020 | TBD Immersive Communication [and Ubiquitous Connectivity] |
| 3.3 | 5th percentile user spectrum efficiency | Clear Quantitative | TBD | TBD | TBD Immersive Communication [and Ubiquitous Connectivity] |
| 4 | Area traffic capacity | Clear Quantitative | TBD | 30 Mbit/s/m2, 50 Mbit/s/m2 | Immersive Communication |
| 5 | Connection Density | Clear Quantitative | TBD | 106 – 108 devices/km2 | Massive Communication |
| 6 | Mobility | TBD Quantitative | TBD | 500 – 1 000 km/h | TBD Immersive Communication [and/or Massive Communication and/or Ubiquitous Connectivity] |
| 6.1 | Mobility interruption time | TBD Quantitative | TBD | TBD | TBD Immersive Communication [and/or Hyper-Reliable and Low Latency Communication and/or Massive Communication] |
| 7.1 | User plane latency | TBD Quantitative | TBD | 0.1 – 1 ms | Immersive Communication and Hyper-Reliable and Low Latency Communication |
| 7.2 | Control plane latency | TBD Quantitative | TBD | TBD | Immersive Communication and Hyper-Reliable and Low Latency Communication |
| 8 | Reliability | TBD Quantitative | TBD | 1-10−5 to 1-10−7 | Hyper-Reliable and Low Latency Communication |
| 9 | Coverage | TBD | TBD | TBD | TBD [Ubiquitous Connectivity and/or General] |
| 10 | Positioning | TBD Quantitative | TBD | 1 – 10 cm | TBD [Immersive Communication and/or Hyper-Reliable and Low Latency Communication and/or Integrated Sensing and Communication] |
| 11 | Bandwidth | Clear Quantitative | TBD | TBD | TBD [General] |
| 12 | Sensing-related capabilities | TBD Quantitative [Detectability (Detection / False Alarm Probability) Localization Accuracy Velocity Accuracy Sensing Resolution] | TBD | TBD | Integrated Sensing and Communication |
| 13 | AI-related capabilities | TBD Qualitative [and Quantitative] | TBD | TBD | Artificial Intelligence and Communication |
| 14 | Energy efficiency | TBD Quantitative [Relative energy savings between the selected load case(s) and a reference case] | TBD | TBD | Immersive Communication |
| 15 | Security and resilience | TBD | TBD | TBD | TBD |
| 16 | Interoperability | TBD | TBD | TBD | TBD |
|  | Joint requirement of data rate, latency, reliability and capacity | TBD Quantitative [number of users / total traffic throughput of all users] that fulfil the required success probability (reliability) of transmitting a layer 2/3 packet within a required [maximum time / latency] | TBD | TBD | Immersive Communication [and Hyper-Reliable Low Latency Communication] |

Observation 1: For some traditional TPRs, the definitions and applicable usage scenarios are clear, and requirements value(s) needs further study.

Observation 2: For some other traditional TPRs or new TPRs, the definition and applicable usage scenarios as well as requirements value(s) need further study.

According to the workplan for development of the Draft New Report ITU-R M.[IMT-2030.TECH PERF REQ] [3] of ITU-R WP5D, there are still 3 meetings (#49 to #51) to complete the report, the main actions in each meeting are summarized in Table 2.

Table 2  ITU-R WP5D work plan for TPRs

| Meeting number | Time plan | Main workplan |
| --- | --- | --- |
| #49 | 2025-06-24 to 2025-07-03 | - Continue to discuss the detailed definition of each technical performance requirement. - Start to discuss target value(s) for technical performance requirements. |
| #50 | October 2025 | - Agree on the items for technical performance requirements. - Discuss and preliminarily agree on the detailed definition and the target value(s) for technical performance requirements. |
| #51 | February 2026 | - Finalize a document as Draft New Report and send it to ITU-R SG5 for consideration. |

RAN should study ITU IMT-2030 technical performance requirements (TPR), including the TPR definitions, applicable usage scenarios and requirements value(s).

Another aspect related to TPR is evaluation. For some TPRs, the requirements values should be defined in some expected deployment scenarios, and for some other TPRs, the requirements values are different for different deployment scenarios. According to the ITU-R WP5D timeline, as shown in Figure 1, the discussion on evaluation of radio interface technologies for IMT-2030 have been started.

In ITU-R WP5D #48 meeting, the test environments for the six usage scenarios were discussed, which are related to the deployment scenarios in 3GPP. Among them are three usage scenarios, including Immersive Communication, Hyper Reliable & Low-latency Communication and Massive Communication, according to discussion in the meeting, the test environments in IMT-2020 can be taken as a starting point. For those usage scenarios, new test environments are also proposed in some input contributions. Other three new usage scenarios, including Ubiquitous Connectivity, Artificial Intelligence & Communication and Integrated Sensing & Communication, new test environments are also proposed. In the following Table 3, we summarize the status of discussion for test environments of each usage scenarios. For the test environments, the definition, the layout, the detailed configurations (such as frequency, bandwidth, antenna configurations, etc.), as well as whether some of test environment(s) is/are needed or not are to be studied and determined.

Table 3  Test  environments discussion status in ITU-R WP5D

| Usage scenarios | Proposed test environments |
| --- | --- |
| Immersive Communication | Indoor Hotspot-IC Dense Urban-IC [Rural-IC] |
| Hyper-Reliable and Low Latency Communication | [Urban Macro–HRLLC[ [Indoor Factory–HRLLC] |
| Massive Communication | Urban Macro–MC |
| Ubiquitous Connectivity | [Rural-UC] |
| Integrated Sensing and Communication | Agreement in ITU-R WP5D #48 meeting, Maximum number of ISAC test environments is 2. [Indoor Factory–ISAC] [Indoor Hotspot–ISAC] [Dense Urban–ISAC] [Rural–ISAC] [Urban macro – ISAC] |
| Artificial Intelligence and Communication | / |

Observation 3: For the test environment(s) of each usage scenario, the definition, layout and detailed configurations need further study.

RAN should study IMT-2030 deployment scenarios, including the definitions, detailed configurations (e.g. layout, carrier frequency, bandwidth, antenna configuration and etc.).

Initial views on IMT-2030 TPR

In the following sections, our initial views on each IMT-2030 TPR are given.

Peak data rate

TPR definition: Same as in IMT-2020.

Applicable usage scenario: Immersive Communication.

Regarding the requirement value(s), in IMT-2020, the peak data rate is evaluated with the assumption that the maximum available bandwidth is aggregated by 16 component carrier (CC) without considering practical frequency bandwidth situations. According to the evaluation results of 3GPP [4], for FR1, 500~900 MHz (i.e. 5~9 CC) with different frame structure can fulfill the requirements, for FR2, 800~1000 MHz (i.e. 2~6 CC) with different frame structure can fulfill the requirements. Actually, in practical deployments, even though less than 16 CC, availability of these bandwidths are also quite challenging. But considering peak data rate is the highest theoretical data rate and the comparative relationship between IMT-2020 and IMT-2030, it’s better to maintain the same definition and same evaluation methodology for peak data rate. According to the discussion in section 3.3.1, the peak spectral efficiency of IMT-2030 is assumed to be 2.0 times of IMT-2020, and assuming the bandwidth for one CC is at least 2.0 times of IMT-2020 (i.e. 200 MHz per CC), we propose the minimum requirements for peak data rate is 4.0 times of IMT-2020, i.e., 80 Gbps for downlink and 40 Gbps for uplink.

User experienced data rate

TPR definition: Same as in IMT-2020.

Applicable usage scenario: Immersive Communication.

Regarding the requirement value(s), the user experience data rate is given by Ruser = W × SEuser. In IMT-2020, the user experience data rate is evaluated with the assumption that the maximum available bandwidth is aggregated by 16 component carrier (CC) without considering practical available frequency bandwidth situations. Referring the evaluation results of 3GPP [4], for FR1, 200~800 MHz (i.e. 2~8 CC) can fulfill the requirements. According to the discussion in section 3.3.3, the downlink 5th percentile user spectral efficiency of IMT-2030 is assumed to be 2.0~3.0 times of IMT-2020, and assuming the bandwidth for one CC is 2.0 times of IMT-2020 (i.e. 200 MHz per CC), we propose the minimum requirements for downlink user experience data rate is 4.0~6.0 times of IMT-2020, i.e., 400~600 Mbit/s for downlink.

Spectrum efficiency

Peak spectrum efficiency

TPR definition: Same as in IMT-2020.

Applicable usage scenario: Immersive Communication.

Regarding the requirement value(s), in IMT-2020, refer to the ITU M.2410 [5], the requirement values were defined assuming an antenna configuration to enable eight spatial layers (streams) in the downlink and four spatial layers (streams) in the uplink. For IMT-2030, considering the possible and practical antenna configuration for a mobile station, assume an antenna configuration to enable eight spatial layers (streams) in both downlink and uplink. Furthermore, higher modulation (e.g. 4096 QAM) for downlink is possible, which have been considered in WiFi technology. Higher coding rate is also possible depending on the development of channel coding scheme. In addition, lower overhead (i.e., radio resources that are used for physical layer synchronization, reference signals or pilots, control signals, guard bands and guard times) is expected due to technical advancement in 6G timeframe, such as application of AI/ML in PHY. Hence, we propose the minimum requirements for peak spectrum efficiency is 2.0 times of IMT-2020, i.e., 60 bit/s/Hz for downlink and 30 bit/s/Hz for uplink.

Average spectrum efficiency

TPR definition: Same as in IMT-2020.

Applicable usage scenario: Immersive Communication.

Regarding the requirement value(s), some initial simulation evaluation results with extreme antenna array assumption in BS by increasing the antenna elements and TXRU number by 2x/4x/8x/16x from 128 elements 32 TXRU and typical antenna assumptions in UE with 4RX and 2/4TX are provided in the figure 2 below. In the preliminary simulations, ideal PMI measurement are assumed for the transmission port larger than 32. Other potential and possible performance enhancement technologies, such as higher modulation, enhanced channel coding, enhanced CSI-RS/DMRS, enhanced MIMO schemes, AI for CSI enhancement, overhead reduced and so on, are not modeled or assumed.

Figure 2 Initial simulation results of spectral efficiency of Dense Urban

From the preliminary simulation results, it can be observed that for downlink, with higher number of BS antenna elements and TXRU in IMT-2030, spectrum efficiency is significantly increased to 2.0~3.0 times. For uplink with 2 transmission antennas in device, the maximum performance gain is 1.2 times for average spectrum efficiency and 1.7 times for 5th percentile user spectrum efficiency, for uplink with 4 transmission antennas in device, the maximum performance gain is 2.0 times for average spectrum efficiency and 2.1 times for 5th percentile user spectrum efficiency. With the potential enhancement of technologies in IMT-2030, the spectral efficiency may be further enhanced.

Hence, we propose the minimum requirements for average spectrum efficiency is 2.0~3.0 times of IMT-2020 for downlink and 1.5~2.0 times of IMT-2020 for uplink.

5th percentile user spectrum efficiency

TPR definition: Same as in IMT-2020.

Applicable usage scenario: Immersive Communication.

Regarding the requirement value(s), according to the preliminary simulation results and preliminary analysis in section 3.3.2, we propose the minimum requirements for 5th percentile user spectrum efficiency is 2.0~3.0 times of IMT-2020 for downlink and 1.5~2.0 times of IMT-2020 for uplink.

Area traffic capacity

TPR definition: Same as in IMT-2020.

Applicable usage scenario: Immersive Communication.

Regarding the requirement value(s), the area traffic capacity is given by Carea = ρ × W × SEavg. In IMT-2020, the area traffic capacity is evaluated with the assumption that the maximum available bandwidth is aggregated by 16 component carrier (CC) without considering practical available frequency bandwidth situations as the evaluation of user experience data rate and peak data rate. Referring the evaluation results of 3GPP [4], for FR1, 400~800 MHz (i.e. 4~8 CC) can fulfill the requirements, and for FR2, 800~1200 MHz (i.e. 2~4 CC) can fulfill the requirements. According to the discussion in section 3.3.2, the downlink average spectral efficiency of IMT-2030 is assumed to be 2.0 times of IMT-2020, and assuming the bandwidth for one CC is 2.0 times of IMT-2020 (i.e. 200 MHz per CC), we propose the minimum requirements for area traffic capacity is 4.0~6.0 times of IMT-2020, i.e., 40~60 Mbit/s/m2.

Connection Density

TPR definition: Same as in IMT-2020.

Applicable usage scenario: Massive Communication. In IMT-2020, connection density is mainly considered for mMTC usage scenario and the target requirement is developed for the IoT traffic with low data rate and longer latency. For IMT-2030 immersive communication usage scenario, the typical use cases include immersive XR. For XR use cases, system capacity is an important metric for successful commercial deployments, which have been widely discussed and extensively evaluated in the 3GPP XR study item [6]. Although one possible way to quantify system capacity for XR use case is to define connection density requirement, e.g., number of supported XR users per km2, but according to the discussion in ITU, a new TPR as joint requirement on data rate, latency, reliability and capacity is proposed to assess the XR performance, so the typical connection density can only focus on massive communication usage scenarios.

Preliminary requirement value(s): the minimum requirement for connection density is 106~107 devices per km2.

Capacity of traffic with joint requirement on data rate, latency, and reliability

As mentioned in chapter 3.5, immersive XR is important and typical use cases for immersive communication, which there are high requirements on data rate, latency and reliability. Meanwhile, system capacity is an important metric for successful commercial deployments, so the capacity with joint requirement on data rate, latency, and reliability is a suitable KPI to assess the system performance to support XR use cases. Refer to the initial agreement in ITU-R WP5D #48 meeting, we have the following suggestion:

TPR definition:

This requirement is defined as the total number of users per unit area (per km2) that fulfil the required success probability (reliability) of transmitting packets within a required maximum time, which is the time it takes to deliver a data packet for a given data rate and traffic arrive rate from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface.

Applicable usage scenario: Immersive Communication.

Preliminary requirement value(s): the minimum requirement for joint requirement is 103 devices per km2 with [30] Mbps data rate, [10] ms latency and [99%] reliability.

Mobility

In IMT-2020, mobility TPR is defined as the supported maximum mobile station speed (in km/h) at which a defined QoS can be achieved, which are related to extreme high-speed scenarios such as high speed trains. The defined QoS is normalize traffic channel link data rate (bit/s/Hz). However, although the user experience is good in most scenarios, there are still poor user experience in some scenarios although the device moving speed is not high, e.g., when entering or exiting buildings, passing under driveway bridges, being in an underground parking lot, or using an elevator, which are referred to as pain point scenarios. The terrible user experiences in these pain pint scenarios cannot be tolerated, especially when the users are in some important calls or online conference.

The traditional mobility requirements and evaluation methodology cannot reflect the practical user experience, it’s necessary to identify the practical problem in actual deployment scenarios to pursue technology solutions to fundamentally resolve the mobility problem. The mobility capability should be measured in terms of practical typical deployment scenarios and user movements to evaluate radio access technology’s ability to support mobility with minimum service interruption.

Study mobility in terms of practical deployment scenarios and user movements to evaluate technology’s ability to support mobility with minimum service interruption.

Regarding the deployment scenarios, the Urban Grid deployment scenario with buildings modelled around the lanes are used to model the blocking or reflection by obstacles, which is a typical pain point scenario of entering or exiting of street corner, can be used as a starting point for pain point deployment scenarios.

Refer to the definition in IMT-2020, we suggest to add the practical Urban Grid deployment scenario as a test environment to define the mobility classes and minimum requirement.

Mobility classes

|  | Test environments for eMBB |  |  |  |
| --- | --- | --- | --- | --- |
|  | Indoor Hotspot – IC | Dense Urban – IC | Rural – IC | Urban Grid _IC |
| Mobility classes supported | Stationary, Pedestrian | Stationary, Pedestrian, Vehicular (up to 30 km/h) | Pedestrian, Vehicular, High speed vehicular | Pedestrian, Vehicular |

Traffic channel link data rates normalized by bandwidth

| Test environment | Normalized traffic channel link data rate (bit/s/Hz) | Mobility (km/h) |
| --- | --- | --- |
| Indoor Hotspot – IC | TBD | TBD |
| Dense Urban – IC | TBD | TBD |
| Rural – IC | TBD | TBD |
|  | TBD | TBD |
| Urban Grid _IC | TBD | TBD |

Mobility interruption time

According to the discussion in chapter 3.7, the mobility interruption time capability should also be measured in terms of practical typical deployment scenarios and user movements to evaluate radio access technology’s ability to support mobility with minimum service interruption time.

Based on the mobility interruption time definition in M.2410 [5], we have the following proposal:

The mobility interruption time should be evaluated and minimized to achieve consistent user experience especially in practical deployment scenarios, especially in some pain point scenarios, e.g. passing street corners, entering or exiting elevator, etc., where wireless signal quality changes rapidly in a short period.

Latency

User plane latency

TPR definition: Same as in IMT-2020.

Applicable usage scenario: Immersive Communication, Hyper Reliable and Low-Latency Communication.

Preliminary requirement value(s): the minimum requirement for user plane latency is 1ms for Immersive Communication and 0.1ms for Hyper Reliable and Low-Latency Communication.

Control plane latency

In IMT-2020, the definition of control plane latency refers to the time for a device to switch from a battery efficient state (e.g., IDLE/INACTIVE) to start of continuous data transfer (e.g., CONNECTED), including the state transition time from battery efficient state to continuous data transfer state. In 5G NR, the small data packets can already be transmitted in battery efficient state (INACTIVE), which means a device may not always have to switch from INACTIVE to CONNECTED for transmission or reception of small packets. The SDT-like feature with high energy efficiency and low signaling demand for state transition can be a promising technology for IMT-2030 as well. However, the control plane latency in such scenarios would need to be clarified/reconsidered.

For example, the control plane latency can be defined as the latency from a “battery efficient” state to the start of data transfer, including the case with the device state transition (e.g., INACTIVE to CONNECTED) and the case without state transition (e.g., small data transmission during INACTIVE).

Study control plane latency definition to include the cases with and without device state transition.

Applicable usage scenario: Immersive Communication, Hyper Reliable and Low-Latency Communication.

Preliminary requirement value(s): the minimum requirement for control plane latency is 10ms with device state transition and 4ms without device state transition.

Reliability

TPR definition: Same as in IMT-2020.

Applicable usage scenario: Hyper Reliable and Low-Latency Communication.

Preliminary requirement value(s): the minimum requirement for reliability is 1-10-5.

Positioning

Positioning is the ability to calculate the approximate position of connected devices. Positioning accuracy describes the closeness of the measured position of the device to its true position value. The accuracy can describe the accuracy either of an absolute position or of a relative position. It can be further derived into a horizontal position accuracy – referring to the position error in a 2D reference or horizontal plane, and into a vertical position accuracy – referring to the position error on the vertical axis or altitude.

Applicable usage scenario: Immersive Communication, Integrated Sensing and Communication.

The preliminary requirements could be 10 cm. Higher positioning capability is considered for some specific usage scenario, such as XR tracking, the positioning accuracy could be 1cm.

Bandwidth

Preliminary requirement value(s): the maximum bandwidth for single radio frequency (RF) carrier can be 400Mhz, and the maximum aggregated system bandwidth is TBD, which is relevant to the requirement of peak data rate.

Sensing-related capabilities

Sensing-related capabilities refer to the ability to provide functionalities in the radio interface to support sensing tasks, including passive object presence/absence detection, object classification, movement characterization, object localization, etc.

Refer to the discussion in ITU-R WP5D, the following four performance metrics could be used to evaluate the radio access technology’s ability to support sensing related capabilities:

- Detectability (Detection / False Alarm Probability) – Detection probability, PD, is the probability of correctly detecting the presence of the object, and False alarm probability, PFA, is the associated probability that an object is detected given that the object is not present.

- Localization Accuracy - Localization accuracy refers to the closeness of the measured sensing result of position of the target object to its true position value and is the difference between the calculated location and the actual location of an object.

(Ed Note: Whether this accuracy is to be represented in Cartesian or polar coordinate or Euclidian distance is FFS. Components of localization accuracy may include:)

  - Distance accuracy: the difference between the calculated distance and the actual distance of the sensing target

  - Angle accuracy: the difference between the calculated angles and the actual angles of the sensing target

  - Horizontal/Vertical Localization Accuracy (HSA/VSA): It is defined as the difference between the calculated horizontal/vertical position and the actual horizontal/vertical position of an object

- Velocity Accuracy - Velocity accuracy is defined as the difference between the estimated velocity and the actual velocity of the sensing target.

- Sensing Resolution – Is defined as the minimum difference of the measured value (e.g, range, location) of two target objects, in order to distinguish these two target objects. (Ed note: this may be evaluated by an analytical approach).

However, regarding the movement characterization of targets, it is not only linear motion, but also periodic motion, such as drone blade rotation, bird wing flapping, human arm/leg swing, human chest undulation, etc. which are also important movement characterization and need to be identified and measured to support some sensing tasks, so we propose the following sensing-related TPR:

Motion rate accuracy – Motion rate accuracy describes the difference between the measured movement frequency caused by part(s) (e.g. blade, chest) of the target object and the true movement frequency.

Study motion rate accuracy as a sensing-related capability.

AI-related capabilities

This TPR is related to Artificial Intelligence and Communication usage scenario. AI-related capabilities refer to the ability to provide certain functionalities throughout IMT-2030 network to support AI services.

The following qualitative requirements from ITU-R M.2160 “Framework and overall objectives of the future development of IMT for 2030 and beyond” can be considered as a starting point for AI-related TPR:

“Besides communication aspects, this usage scenario is expected to include a set of new capabilities related to the integration of AI and compute functionalities into IMT-2030, including data acquisition, preparation and processing from different sources, distributed AI model training, model sharing and distributed inference across IMT systems, and computing resource orchestration and chaining.”

Energy efficiency

Energy efficiency is a quantifiable metric of sustainability and is important for the success of IMT-2030 technology. The energy efficiency TPR should be defined to ensure that IMT-2030 is designed in an energy efficient way.

Regarding the load cases for energy efficiency, there are two cases to be considered. One is the unloaded case, where no traffic data transmission / reception needs but there are still some signals, such as paging signal, need to be transmitted / received, power saving technologies are very much needed to reduce the power consumption meanwhile maintain the paging performance (i.e. paging latency). The other is loaded case, where there are traffic data for transmission or reception but not always at all the time, power saving technologies to reduce the power consumption meanwhile minimizing the impact on the data transmission performance (e.g. user perceived throughput (UPT), which is widely used as performance metric in the 3GPP study) are necessary to enhance the energy efficiency. Therefore, the energy efficiency KPI should be measured as the relative power saving gain between the evaluated case and reference case at given tolerance of communication performance impact. The tolerance of communication performance impact should be determined for unloaded and loaded cases separately. For example, for device energy efficiency and unloaded case, the tolerable performance impact can be paging latency increase no more than B1%, while for loaded case, the tolerable performance impact can be user perceived throughput (UPT) loss no more than B2%.

Study energy efficiency KPI as the relative power saving gain between the evaluated case and reference case at given tolerance of communication performance impact.

Regarding the applicable usage scenario, extreme low power consumption and ultra long battery life are important for massive communication usage scenarios, such as LPWA, so the energy efficiency should also be considered in massive communication usage scenarios.

Energy efficiency KPI is applicable in both Immersive communication and Massive communication usage scenarios.

With the reference to the initial agreement in ITU-R WP5D #48 meeting, we have the following suggestion:

The energy efficiency is the relative energy saving between the selected load cases (unloaded case and loaded case) and a reference case with tolerable communication performance deterioration.

For unloaded case, the related communication performance can be paging/control signaling latency.

For loaded case, the related communication performance can be user perceived throughput (UPT).

This requirement is defined for the purpose of evaluation in the Immersive communication and Massive communication usage scenarios.

The minimum requirements for Energy efficiency are summarized in Tables below.

Table 4 The minimum requirements for Network Energy Efficiency

| [Test environment(s)] | Selected load case(s) | The reference case | Relative power saving (%) | Maximum communication performance impact (%) |
| --- | --- | --- | --- | --- |
| TBD | Unloaded | Fully loaded IMT-2030 case | [X1%] | [Y1%] |
| TBD | [low load, light load and/or medium load] | Fully loaded IMT-2030 case | [X2%] | [Y2%] |

Table 5 The minimum requirements for Device Energy Efficiency

| [Test environment(s)] | Selected load case(s) | The reference case | Relative power saving (%) | Maximum communication performance impact (%) |
| --- | --- | --- | --- | --- |
| TBD | Unloaded | Fully loaded IMT-2030 case | [A1%] | [B1%] |
| TBD | [low load, light load and/or medium load] | Fully loaded IMT-2030 case | [A2%] | [B2%] |

Initial views on IMT-2030 deployment scenarios

For the three enhanced usage scenarios, Immersive Communication, Hyper Reliable & Low-latency Communication and Massive Communication, which are envisaged to expand from IMT-2020 (i.e., eMBB, URLLC, and mMTC), the deployment scenarios of IMT-2020 in TR 38.913 [7] can be as staring point with some necessary modification (such as carrier frequency, system bandwidth, antenna configuration and so on) to match the IMT-2030 frequency and system design.

For the urban grid deployment scenarios, additional models can be added based on the discussion in RAN1 study item of Study on channel modelling for ISAC for NR. That is, the environment objects, such as buildings of size 413m x 230m x 20m can be modeled in the scenarios to model the actual blocking or reflecting by obstacles.

Deployment scenarios of IMT-2020 in TR 38.913 can be taken as starting point for IMT-2030 immersive communication, hyper reliable and low-latency communication and massive communication usage scenario.

- Some necessary modification (such as carrier frequency, system bandwidth, antenna configuration and so on) can be considered to match the IMT-2030 frequency and system design.

For urban grid scenarios, environment objects, such as buildings of size 413m x 230m x 20m can be modeled in the scenarios to model the actual blocking or reflecting by obstacles.

For the Integrated Sensing and Communication usage scenarios, the agreed output scenarios of the RAN1 study item of Study on channel modelling for ISAC for NR can be taken as staring point.

Deployment scenarios agreed in RAN1 study item of Study on channel modelling for ISAC for NR can be taken as starting point for IMT-2030 integrated sensing and communication usage scenario.

Conclusions

In summary, we summarize the status and progress of technical performance requirements (TPR) related discussion in ITU-R WP5D and express our views on the TPR and deployment scenarios with following observations and proposals:

Observation 1: For some traditional TPRs, the definitions and applicable usage scenarios are clear, and requirements value(s) needs further study.

Observation 2: For some other traditional TPRs or new TPRs, the definition and applicable usage scenarios as well as requirements value(s) need further study.

Observation 3: For the test environment(s) of each usage scenario, the definition, layout and detailed configurations need further study.

- RAN should study ITU IMT-2030 technical performance requirements (TPR), including the TPR definitions, applicable usage scenarios and requirements value(s).

- RAN should study IMT-2030 deployment scenarios, including the definitions, detailed configurations (e.g. layout, carrier frequency, bandwidth, antenna configuration and etc.).

- Study mobility in terms of practical deployment scenarios and user movements to evaluate technology’s ability to support mobility with minimum service interruption.

- Study control plane latency definition to include the cases with and without device state transition.

- Study motion rate accuracy as a sensing-related capability.

- Study energy efficiency KPI as the relative power saving gain between the evaluated case and reference case at given tolerance of communication performance impact.

- Energy efficiency KPI is applicable in both Immersive communication and Massive communication usage scenarios.

- Deployment scenarios of IMT-2020 in TR 38.913 can be taken as starting point for IMT-2030 immersive communication, hyper reliable and low-latency communication and massive communication usage scenario.

- Some necessary modification (such as carrier frequency, system bandwidth, antenna configuration and so on) can be considered to match the IMT-2030 frequency and system design.

- For urban grid scenarios, environment objects, such as buildings of size 413m x 230m x 20m can be modeled in the scenarios to model the actual blocking or reflecting by obstacles.

- Deployment scenarios agreed in RAN1 study item of Study on channel modelling for ISAC for NR can be taken as starting point for IMT-2030 integrated sensing and communication usage scenario.

References

- RP-243327, “New SID: Study on 6G Scenarios and requirements”, CMCC, Verizon, NTT DOCOMO, INC., Deutsche Telekom.

- R23-WP5D-250204-TD-0249!R1!MSW-E

- R23-WP5D-250204-TD-0248!!MSW-E

- TR 37.910, Study on self evaluation towards IMT-2020 submission.

- R-REP-M.2410-2017-MSW-E.

- TR 38.838, Study on XR (Extended Reality) Evaluations for NR.

- TR 38.913, Study on Scenarios and Requirements for Next Generation Access Technologies.