3GPP TSG RAN Meeting #107	RP-250214

# March 12–14, 2024, Incheon, South Korea

# Source:	Ericsson, ZTE, Huawei, HiSilicon

# Title:	Revised WID: Rel-19 NR Carrier Aggregation (CA)/Dual Connectivity (DC) for x bands DL with y bands UL (x<7, y<3) and Supplementary Uplink (SUL) band combinations/CA band combinations with a single SUL or two SUL cells

# Document for:	Approval

Agenda Item:	9.4.2.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Rel-19 NR Carrier Aggregation (CA)/Dual Connectivity (DC) for x bands DL with y bands UL (x<7, y<3) and Supplementary Uplink (SUL) band combinations/CA band combinations with a single SUL or two SUL cells

Acronym: NR_CADC_SUL_R19

Unique identifier:	1040120

NOTE:	For new WIs/SIs leave the Unique identifier empty and make a proposal for an Acronym.

For a revised WI/SI: Take Unique identifier and acronym as shown in 3GPP workplan.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part | X |
| --- | --- |
| This WID includes a Performance part |  |

or:

| This WID includes a Testing part |  |  |
| --- | --- | --- |
| and it addresses the following 3GPP work area: | Radio Access |  |
|  | Core Network |  |
|  | Services |  |

Potential target Release:	Rel-19

{Note that this field above indicates the proposed Release at the time of submission of the WID to TSG approval. It can later be changed without a need to revise the WID. The updated target Release is indicated in the Work Plan}

NOTE: In case of contradiction with the target dates of clause 5, clause 5 determines the target release.

# 1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X |  |  |
| No | X |  |  | X | X |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a …

{Tick one box. "Feature / Building Block / Work Task" form a hierarchical structure. E.g. no Building Block can be proposed without a corresponding parent Feature. The full structure of all existing Work Items is shown in the 3GPP Work Plan in ftp://ftp.3gpp.org/Information/WORK_PLAN }

|  | Feature |
| --- | --- |
| X | Building Block |
|  | Work Task |
|  | Study Item |

NOTE:	Normally, Core/Perf./Testing parts in RAN WIDs are Building Blocks. Only if they are under an SA or CT umbrella, they are defined as work tasks. If you are in doubt, please contact MCC.

### 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A | N/A | N/A | N/A |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. part WI or Testing part WI in one 	WID. Therefore the table above should include the feature WI data (In case the feature covers Core and Perf. 	part, please list under Working Group the leading WG of the Core part).

### 2.3	Other related Work Items and dependencies

| Other related Work/Study Items (if any) |  |  |  |
| --- | --- | --- | --- |
| Acronym | Unique ID | Title | Nature of relationship |
| NR_CA_R18_Intra | 961007 | Rel-18 NR intra band Carrier Aggregation for xCC DL/yCC UL including contiguous and non-contiguous spectrum (x>=y) | REL-18 WI, compare objective 1 of the present REL-19 WI |
| NR_CADC_R18_2BDL_xBUL | 961010 | Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1,2) | REL-18 WI, compare objective 2 of the present REL-19 WI |
| NR_CADC_R18_3BDL_xBUL | 961011 | Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with x bands UL (x=1,2) | REL-18 WI, compare objective 3 of the present REL-19 WI |
| NR_CADC_R18_yBDL_xBUL | 961008 | Rel-18 NR inter-band Carrier Aggregation/Dual Connectivity for y bands DL (y=4, 5, 6) with x bands UL (x=1, 2) | REL-18 WI, compare objective 4 of the present REL-19 WI |
| NR_SUL_combos_R18 | 961009 | Rel-18 single SUL band combinations for SA NR supplementary uplink (SUL), NSA NR SUL, NSA NR SUL with UL sharing from the UE perspective (ULSUP) | REL-18 WI, compare objective 5 of the present REL-19 WI |
| NR_2SUL_cell_combos_R18 | 981047 | Rel-18 NR Carrier Aggregation (CA) band combinations with two SUL cells | REL-18 WI, compare objective 5 of the present REL-19 WI |

NOTE:	Also related or dependent WIs/SIs in other TSGs shall be indicated here.

Dependency on non-3GPP (draft) specification:

# 3	Justification

All new NR Carrier Aggregation (CA), NR Dual Connectivity (DC), with or without Supplemental Uplink (SUL) configurations will be defined under this WI. New configurations still emerge from the existing bands and whenever new band is specified, it will create a potential for several new CA/DC/SUL configurations.

NR_CA_R19_Intra with/without UL-MIMO

NR CA intra-band configurations are fundamental part of all higher order combinations and all new intra-band configurations will be defined under this WI. New configurations still emerge from existing bands and whenever new band is specified, it will create a potential for several new intra-band configurations.

There are a lot of NR FR1 bands supporting intra-band contiguous CA operation, meanwhile UL MIMO feature can also be supported in the same band.

In current TS38.101-1 Rel-18 specification, there are two bands specified with PC2/3 NR intra-band CA configuration with UL MIMO, i.e. CA_n41C, CA_n78C. The general RF requirements have already been defined. In the R19 WID NR_ENDC_RF_Ph4, there are four NR intra-band CA configuration with UL MIMO selected as example band combinations, i.e. CA_n41C, CA_n78C, CA_n77C and CA_n79C.  Moreover, considering the demands from operators, and more NR intra-band UL contiguous CA with UL MIMO will be requested by operators, using Basket WID approach is an efficient and routine way to add PC3 intra-band MIMO.

NR_CADC_R19_2BDL_yBUL (y=1,2), NR_CADC_R19_3BDL_yBUL (y=1,2), and NR_CADC_R19_xBDL_yBUL (3<x<7, y=1,2)

While in REL-18 these combinations were handled in multiple basket WIs (see 2.3), it was decided for REL-19 to merge the handling of these combinations in one basket WI under different objectives with different TRs.NR_SUL_combos_R19NR CA band combinations with SUL band were introduced from Rel-16. In Rel-16, intra-band CA with SUL band combination was supported, and the CA configuration was extended to inter-band CA with SUL band combination in Rel-17.

New configurations still emerge from existing bands and whenever new band is specified, it will create a potential for several new SUL configurations. There is no restriction on supporting UL MIMO in SUL band and/or NR band for this basket WI.

With fast deployment of 5G in China, more spectrum currently utilized by GSM, UMTS and LTE will be evolved into NR deployment recently. As an example, 1.9GHz and 2GHz TDD bands are previously used by 4G and 3G, which are specified as NR TDD bands n39 and n34 in Rel-15 and also specified as SUL bands n98 and n95 in Rel-16 and Rel-17 respectively. The spectrum for these two bands themselves are not very large, thus single SUL band may not fully comply with the fast increased UL usage demanded by the operator. Therefore, NR CA configurations with two SUL band combinations, i.e. two SUL bands in two cells together with other TDD NR band(s), are emerging as a prospective solution for operators. Similarly, 700MHz, 850MHz, 900MHz, 1.8GHz and 2.1GHz bands defined as SUL bands n83, n89, n81, n80 and n84 in which 2 SUL bands in conjunction with NR TDD bands, e.g. n78, n79 are highly interested by operators holding the spectrum to boost the wide band UL performance.

# 4	Objective

### 4.1	Objective of SI or Core part WI or Testing part WI

### 4.1.1	Objective and scope

The core part of this WI includes the objectives for the following Carrier Aggregation (CA)/DDC configurations

- Objective #1: NR_CA_R19_Intra with/without UL-MIMO

- CA/DC configuration with power class 3 (PC3) to be specified

  - NR intra-band CA combinations for m CC DL/n CC UL including contiguous and non-contiguous spectrum (2<=m<=12, n<=8)

  - with or without UL-MIMO

- Specify the configuration specific RF requirements for all listed NR intra-band CA combinations

  - Applicable frequencies

  - Applicable bandwidths and bandwidth sets

- Analyze combinations that have self-desensitization due to following reasons:

  - TX frequency being in close proximity of their own receive bands for FDD bands

  - Any other identified reasons

- For the combination where self-desensitization exists, specify at least needed

  - Reference sensitivity excerptions

  - UL RB restrictions for REFSENS test

- Specify A-MPR for PC3 for NR intra-band CA combinations for m CC DL/n CC UL including contiguous and non-contiguous spectrum. (2<=m<=12, n<=8)
Note: CRs covering AMPR will not be included in the RAN4 block approval process

- Intra-band contiguous UL CA for NR-U
Note: papers and discussions related to NR-U intra-band contiguous UL CA shall not be treated by block approval process

- Specify RF requirements for contiguous UL CA for shared spectrum channel access for both n46 and n96:

  - Including at least the 2 x 20 MHz and 2 x 80 MHz cases

  - Accounting for the regulatory requirements that apply in different regions (including the pending EU regulation for the 6 GHz range).

  - Agree on limit to the number of LBT sub-band case restrictions (Taking into account only contiguous sub-bands supported)

- Specify RF requirements for intra-band UL contiguous CA with UL-MIMO/TxD (Tx diversity) configurations

  - Intra-band contiguous CA configurations to support UL-MIMO/TxD

  - Specify MOP with tolerance for the requested operating bands

  - Specify the band specific A-MPR requirements for the proposed bands, if needed

Note 1: Maximum aggregated BW is limited to 1600 MHz for all combinations

Note 2: CA combinations of this WI are introduced in a REL-independent way starting from REL-15 according to Table 5.2.1-1, Table 5.2.1-2, Table 6.2.1-1 and Table 6.2.1-2 in TS 38.307. However no changes to TS 38.307 are needed.

For the Objective #1, the preconditions to propose NR intra-band CA/DC configurations:

- To propose DL CA for NR Intra-band CA for x CC DL/1 CC UL including contiguous and non-contiguous spectrum in rel-19, the constituent NR band requirements shall be completed and specified in advance.

- To propose UL CA UL CA of x CC DL/y CC UL in rel-19: Constituent x CC DL/ (y-1) CC UL shall be completed and specified in advance.

Example 1: If the following configuration is proposed,

| CA configuration | Uplink CA configuration |
| --- | --- |
| CA_1C | None |

- NR Band 1 requirements shall be completed and specified in advance.

Example 2: If the following configuration is proposed,

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| CA_1E | CA_1D |

- NR CA of DL_CA_1E_UL_1C shall be completed and specified in advance.

The preconditions to propose NR intra-band contiguous CA with UL MIMO in Rel-19 are as follows.

- NR Intra band CA shall be completed and specified in advance.

- NR band supporting UL MIMO shall be completed and specified in advance.

- Objective #2: NR_CADC_R19_2BDL_yBUL (y=1,2)

- CA/DC configurations with power class 3 (PC3)  to be specified

  - NR inter-band CA/DC configurations including inter band CA for 2 different bands DL with up to 2 different bands UL

- Objective #3: NR_CADC_R19_3BDL_yBUL (y=1,2)

- CA/DC configurations with power class 3 (PC3) to be specified

  - NR inter-band CA/DC configurations including inter band CA for 3 different bands DL with y different bands UL(y=1,2)

- Objective #4: NR_CADC_R19_xBDL_yBUL (3<x<7, y=1,2)

- CA/DC configurations with power class 3 (PC3) to be specified

  - NR inter-band CA/DC configurations including inter band CA for x different bands DL with y different bands UL (3<x<7, y=1,2)

- Objective #5: NR_SUL_combos_R19

- SUL configurations and CA band combinations with SUL(s) to be specified with power class 3 (PC3) with a possibility of supporting UL MIMO in SUL:

    - 1 SUL band + (1 NR band or intra-band CA of 1 NR band)

    - 1 SUL band + k NR bands (k=2, 3, 4, 5) inter-band CA

    - 2 SUL bands + intra-band CA of 1 NR band

    - 2 SUL bands + k NR bands (k=2, 3, 4, 5) inter-band CA

NOTE 1:  UL NR bands for CA are a subset of the DL NR bands and for SUL bands the notation rule shall follow the approved WF R4-2306546 and WF R4-2303459.
NOTE 2: The present WID covers SA NR SUL configurations. NSA SUL combinations are covered in REL-19 WI DC_R19_xBLTE_yBNR.
NOTE 3: Simultaneous uplink transmission on band pairs {SUL and another SUL}, {SUL and corresponding NUL} and {SUL and non-corresponding NUL} are not supported in Rel-19 (see also clause 5.5C in TS 38.101-1 REL-18).

For the Objective #2~#4, the preconditions to propose NR inter-band CA/DC configurations are as follows.

- Constituent NR band and NR Intra band CA shall be completed and specified in advance.

- All the fallbacks of the proposed inter-band CA/DC shall be completed and specified in advance

Example 1: If the following configuration is proposed,

| CA configuration | Uplink CA configuration |
| --- | --- |
| CA_n1A-n2A | - |

- NR Band n1 and n2 requirements shall be completed and specified in advanced.

Example 2: If the following configuration is proposed,

| CA configuration | Uplink CA configuration |
| --- | --- |
| CA_n1C-n2C | CA_n1A-n2C |

- NR intra band CA of DL_CA_n2C_UL_CA_n2C requirements shall be completed and specified in advanced.

Example 3: If CA_1A-2A-3A-4A is proposed,

- NR Band 1, 2, 3 ….. Band x requirements shall be completed and specified in advanced.

For the Objective #2~#5, specify the following requirements

- Specify the band-combination specific RF requirements for all listed NR CA/DC configurations including at least

  - Applicable frequencies if necessary

  - Applicable bandwidths and bandwidth sets if necessary

- Analyze combinations that have self-desensitization due to following reasons:

  - TX Harmonic and/or intermodulation overlap of receive band

  - TX signal overlap of receiver harmonic frequency

  - TX frequency being in close proximity of one of the receive bands

  - Any other identified reasons such that insufficient cross band isolation, harmonic mixing

- For the combination where self-desensitization exists, specify at least needed

  - ∆TIB, c and ∆RIB, c

  - Reference sensitivity exceptions including MSD test cases

  - UL RB restrictions for REFSENS test if necessary

  - Exception to the out-of-band blocking requirement

- Specify the release independency if necessary

  - Unless otherwise stated below, CA/DC/SUL band combinations are introduced in release independent manner from Rel-15

  - The NR DC configurations with FR1 and NR CA configurations including NR-U band are introduced in release independent manner from Rel-16.

  - The NR CA and NR DC configurations between FR1 and FR2 where the supported bandwidth classes for the FR2 band are R2, R3, R4, R5, R6, R7, R8, R9, R10, are introduced in a release independent way starting from Rel-17

  - Depending on newly introduced CA/DC/SUL configurations, to check if TS38.307 need to be updated.

- Note: low band-low band combinations should not be under the block approval approaches by default. It is not precluded for RAN4 to discuss the band combination feasibility.

All CA/DC/SUL configurations to be specified in the objectives of this WI are collected in the attached Excel sheet.

### 4.1.2	Way of working

When requesting additional band combinations, proponent should follow the protocols listed below.

- Request for additions of band combinations to this WI shall be provided using an agreed template and sent to the 3GPP_TSG_RAN_WG4_NR_BANDS email reflector before a RAN4 Tdoc submission deadline and no new band combinations are allowed to be requested after the deadline except to correct the missing fallback and add more supporting companies for the proposed band combinations.

- When a proponent requests a new band combination, all the next level fallback configurations shall be listed and recorded in the request template and the status (“New”, “Ongoing”, “Completed”) of all the fallback configurations shall be declared accurately and clearly. For “New” fallback configurations, the proponent shall ensure these fallback configurations are also requested together with the higher order band combination in the same meeting.

- A band combination configuration can only be considered as completed when all of the fallback configurations are completed and specified in advance or at the same meeting. It is the responsibility of the proponent to ensure the status of all of the fallback mode configurations. Rapporteurs and other companies are encouraged to check the status of all of the fallback configurations once the higher order band combinations are declared as completed.

### 4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part.

### 4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed, then leave the field empty otherwise enter a number >0 in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |
| Internal TR | 38.719-01-01 | Rel-19 NR intra-band CA combinations | RAN#108 | RAN#109 | Core part Per Lindell per.lindell@ERICSSON.COM |
| Internal TR | 38.719-02-01 | Rel-19 NR inter-band CA/DC configurations including inter band CA for 2 different bands DL with up to 2 different bands UL | RAN#108 | RAN#109 | Core part Wubin ZHOU, zhou.wubin@ZTE.COM.CN |
| Internal TR | 38.719-03-01 | Rel-19 NR inter-band CA/DC configurations including inter band CA for 3 different bands DL with x different bands UL(x=1,2) | RAN#108 | RAN#109 | Core part Zhifeng MA ma.zhifeng@ZTE.COM.CN |
| Internal TR | 38.719-00-00 | Rel-19 SUL configurations and CA band combinations with SUL(s) | RAN#108 | RAN#109 | Core part Peng ZHANG zhangpeng169@huawei.com |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
By default a new specs can only be new for one of both parts.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 38.101-1 | User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone | RAN#109 | Core part |
| 38.101-2 | User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone | RAN#109 | Core part |
| TS 38.101-3 | User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios | RAN#109 | Core part |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
If an existing spec is affected by both (Core part and Perf. part), then it has to be listed twice with appropriate approval dates.

# 6	Work item Rapporteur(s)

Objective #1: Per Lindell

per.lindell@ERICSSON.COM

Objective #2: Wubin ZHOU,

zhou.wubin@ZTE.COM.CN

Objective #3: Zhifeng MA

ma.zhifeng@ZTE.COM.CN

Objective #4: Reihaneh Malekafzaliardakani

reihaneh.malekafzaliardakani@ericsson.com

Objective #5: Peng ZHANG

zhangpeng169@huawei.com

NOTE:	The first listed Rapporteur has the overall responsibility for this WI (incl all secondary tasks).

# 7	Work item leadership

R4

# 8	Aspects that involve other WGs

None

NOTE:	For RAN WIs: Section 8 applies only toWGs outside of TSG RAN because all RAN WG aspects have to be covered in section 4.

# 9	Supporting Individual Members

{At least 4 supporting Individual Members are needed. There is an expectation that these companies will provide resources to progress the work. Note that having 4 supporting companies is a necessary but not sufficient condition: the usual TSG approval process by consensus is needed for the WID approval.}

Supporting IM name

Ericsson

ZTE

Sanechips

Huawei

HiSilicon

AT&T

BT plc

CBN

China Broadnet

China Telecom

China Unicom

CHTTL

CMCC

LGE

NBN

Nokia

Nokia Shanghai Bell

NTT DOCOMO, INC.

OPPO

Qualcomm

Samsung

Telstra

T-Mobile US

T-Mobile USA

Verizon

Xiaomi