3GPP TSG RAN WG1 #120                                                                               R1-2501637

Athens, Greece, February 17th – 21st, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | 0213 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on the PRACH retransmission indicator field included in the PDCCH order |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (vivo), Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2025-2-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the description of the Cell indicator field and PRACH retransmission field in the TS38.212, when the value of the Cell indicator field is not 0, i.e., the cell indicated by the Cell indicator field is a candidate cell, the PRACH retransmission indicator field is 1 bit. When the value of the Cell indicator field is 0, i.e., the cell indicated by the Cell indicator field is the serving cell where UE detects the PDCCH order, and this serving cell is not a candidate cell, i.e, it is not configured in the LTM-Config, the PRACH retransmission indicator field is reserved. However, when the value of the Cell indicator field is 0 and the serving cell indicated by the Cell indicator field is also configured in the LTM-Config, whether the PRACH retransmission indicator field is 1 bit or reserved is unclear. Once the value of the Cell indicator field is 0, regardless of the configuration of LTM-Config, the indicator cell is the serving cell and the legacy PDCCH-order triggered PRACH transmission should be performed by UE. In this case, the PRACH retransmission field should be reserved. Therefore, the prerequisite of the case where the PRACH retransmission indicator field included in the PDCCH order is reserved should be revised as the cell indicated by the Cell indicator field is the serving cell, i.e., the value of the Cell indicator field is 0. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Revise the prerequisite of the case where the PRACH retransmission indicator field included in the PDCCH order is reserved as when the cell indicated by the Cell indicator field is the serving cell. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE behavior on the PRACH retransmission indicator field is unclear when the cell indicated by the Cell indicator field is the serving cell and is also a candidate cell. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 7.3.1.2.1	Format 1_0

DCI format 1_0 is used for the scheduling of PDSCH in one DL cell.

The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats - 1 bits

-	The value of this bit field is always set to 1, indicating a DL DCI format

-	Frequency domain resource assignment -  bits where  is given by Clause 7.3.1.0

If the CRC of the DCI format 1_0 is scrambled by C-RNTI and the "Frequency domain resource assignment" field are of all ones, the DCI format 1_0 is for random access procedure initiated by a PDCCH order, with all remaining fields set as follows:

-	Random Access Preamble index - 6 bits according to ra-PreambleIndex in Clause 5.1.2 of [8, TS38.321]

-	UL/SUL indicator - 1 bit.

-	If the Cell indicator field is absent or the Cell indicator field indicates serving cell, if the value of the "Random Access Preamble index" is not all zeros and if the UE is configured with supplementaryUplink in ServingCellConfig in the cell, this field indicates which UL carrier in the cell to transmit the PRACH according to Table 7.3.1.1.1-1;

-	If the Cell indicator field indicates a candidate cell, if the value of the "Random Access Preamble index" is not all zeros and if the UE is configured with ltm-EarlyUL-SyncConfigSUL in LTM-Candidate for the candidate cell, this field indicates which UL carrier in the candidate cell to transmit the PRACH according to Table 7.3.1.1.1-1;

-	Otherwise, this field is reserved.

-	SS/PBCH index - 6 bits. If the value of the "Random Access Preamble index" is not all zeros, this field indicates the SS/PBCH that shall be used to determine the RACH occasion for the PRACH transmission; otherwise, this field is reserved.

-	PRACH Mask index - 4 bits. If the value of the "Random Access Preamble index" is not all zeros, this field indicates the RACH occasion associated with the SS/PBCH indicated by "SS/PBCH index" for the PRACH transmission, according to Clause 5.1.1 of [8, TS38.321]; otherwise, this field is reserved

-	Cell indicator - log2C+1 bits indicating the cell for the corresponding PRACH transmission if the UE is configured with higher layer parameter EarlyUL-SyncConfig, where C is the number of candidate cells configured with higher layer parameter EarlyUL-SyncConfig; 0 bit otherwise. The bit field index 0 of the cell indicator field is mapped to the serving cell, and other bit field indexes are mapped to the candidate cells configured with higher layer parameter EarlyUL-SyncConfig according to an ascending order of a candidate identity configured by ltm-CandidateId, with the bit field index 1 mapped to the candidate cell with the smallest candidate identity.

-	PRACH association indicator - 0 or 1 bit

-	1bit if the UE is provided with tag2-Id, and the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for the first CORESETs, and is provided coresetPoolIndex with value 1 for the second CORESETs. This field is reserved if the cell indicated by Cell indicator field is a candidate cell.

-	This field indicates the PCI associated with the PRACH transmission if the UE is provided SSB-MTC-AddtionalPCI. The bit field index 0 of this field is mapped to the PCI of the serving cell, and the bit field index 1 of this field is mapped to the additional PCI associated with active TCI states.

-	This field indicates the PL-RS for the PRACH transmission if the UE is not provided SSB-MTC-AddtionalPCI. The bit field index 0 of this field is mapped to the DL RS that the DM-RS of the PDCCH order is quasi-collocated with, and the bit field index 1 of this field is mapped to the SS/PBCH indicated by the SS/PBCH index field in this DCI format.

-	0 bit otherwise.

-	PRACH retransmission indicator - 0 or 1 bit

-	1bit if the UE is configured with higher layer parameter EarlyUL-SyncConfig. This field indicates initial transmission or retransmission of PRACH according to Table 7.3.1.2.1-3 if the cell indicated by Cell indicator field is a candidate cell, and this field is reserved if the value of  Cell indicator field is  zero.

-	0 bit otherwise.

-	Reserved bits - a number of bits as determined by the following:

-	(12 - Y1 - Y2) bits for operation in a cell with shared spectrum channel access in frequency range 1 or when the DCI format is monitored in common search space for operation in a cell in frequency range 2-2;

-	(10 - Y1 - Y2) bits otherwise;

where,

-	Y1=0 if the UE is not configured with higher layer parameter EarlyUL-SyncConfig; Y1=log2C+1+1 otherwise.

-	Y2=0 if the "PRACH association indicator" field is not present in this DCI format; Y2=1 otherwise.

Otherwise, all remaining fields are set as follows:

-	Time domain resource assignment - 4 bits as defined in Clause 5.1.2.1 of [6, TS 38.214]

-	VRB-to-PRB mapping - 1 bit according to Table 7.3.1.2.2-5

-	Modulation and coding scheme - 5 bits as defined in Clause 5.1.3 of [6, TS 38.214]

-	New data indicator - 1 bit

-	Redundancy version - 2 bits as defined in Table 7.3.1.1.1-2

-	HARQ process number - 4 bits

-	Downlink assignment index - 2 bits as defined in Clause 9.1.3 of [5, TS 38.213], as counter DAI

-	TPC command for scheduled PUCCH - 2 bits as defined in Clause 7.2.1 of [5, TS 38.213]

-	PUCCH resource indicator - 3 bits as defined in Clause 9.2.3 of [5, TS 38.213]

-	PDSCH-to-HARQ_feedback timing indicator - 3 bits as defined in Clause 9.2.3 of [5, TS38.213]

-	ChannelAccess-CPext - 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4, or Table 7.3.1.1.1-4A if channelAccessMode-r16 = "semiStatic" is provided, for operation in a cell with shared spectrum channel access in frequency range 1; 2 bits indicating channel access type as defined in Table 7.3.1.1.1-4B if ChannelAccessMode2-r17 is provided for operation in a cell in frequency range 2-2; 0 bits otherwise

-	Reserved bits - 2 bits when the DCI format is monitored in common search space for operation in a cell in frequency range 2-2 and the number of bits for the field of 'ChannelAccess-CPext' is 0; 0 bits otherwise

The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by P-RNTI:

-	Short Messages Indicator - 2 bits according to Table 7.3.1.2.1-1.

-	Short Messages - 8 bits, according to Clause 6.5 of [9, TS38.331]. If only the scheduling information for Paging, and TRS availability indication if trs-ResourceSetConfig or trs-ResourceSetConfig-r18 is configured, are carried, this bit field is reserved.

-	Frequency domain resource assignment - bits. If only the short message, and TRS availability indication if trs-ResourceSetConfig or trs-ResourceSetConfig-r18 is configured, are carried, this bit field is reserved.

-	 is the size of CORESET 0

-	Time domain resource assignment - 4 bits as defined in Clause 5.1.2.1 of [6, TS38.214]. If only the short message, and TRS availability indication if trs-ResourceSetConfig or trs-ResourceSetConfig-r18 is configured, are carried, this bit field is reserved.

-	VRB-to-PRB mapping - 1 bit according to Table 7.3.1.2.2-5. If only the short message, and TRS availability indication if trs-ResourceSetConfig or trs-ResourceSetConfig-r18 is configured, are carried, this bit field is reserved.

-	Modulation and coding scheme - 5 bits as defined in Clause 5.1.3 of [6, TS38.214], using Table 5.1.3.1-1. If only the short message, and TRS availability indication if trs-ResourceSetConfig or trs-ResourceSetConfig-r18 is configured, are carried, this bit field is reserved.

-	TB scaling - 2 bits as defined in Clause 5.1.3.2 of [6, TS38.214]. If only the short message, and TRS availability indication if trs-ResourceSetConfig or trs-ResourceSetConfig-r18 is configured, are carried, this bit field is reserved.

-	TRS availability indication - 1, 2, 3, 4, 5, or 6 bits, where the number of bits is equal to one plus the highest value of all the indBitID(s) provided by the trs-ResourceSetConfig if configured or the number of bits is equal to one plus the highest value of all the indBitID-r18(s) provided by the trs-ResourceSetConfig-r18 if configured; 0 bits otherwise.

-	Reserved bits - (8 - M) bits for operation in a cell with shared spectrum channel access in frequency range 1 or for operation in a cell in frequency range 2-2; (6 - M) bits for operation in a cell without shared spectrum channel access, where the value of M is the number of bits for the field of 'TRS availability indication' as defined above.

The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by SI-RNTI:

-	Frequency domain resource assignment - bits

-	 is the size of CORESET 0

-	Time domain resource assignment - 4 bits as defined in Clause 5.1.2.1 of [6, TS38.214]

-	VRB-to-PRB mapping - 1 bit according to Table 7.3.1.2.2-5

-	Modulation and coding scheme - 5 bits as defined in Clause 5.1.3 of [6, TS38.214], using Table 5.1.3.1-1

-	Redundancy version - 2 bits as defined in Table 7.3.1.1.1-2

-	System information indicator - 1 bit as defined in Table 7.3.1.2.1-2

-	Reserved bits -  17 bits for operation in a cell with shared spectrum channel access in frequency range 1 or for operation in a cell in frequency range 2-2; otherwise 15 bits

The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by RA-RNTI or MsgB-RNTI:

-	Frequency domain resource assignment - bits

-	 is the size of CORESET 0 if CORESET 0 is configured for the cell and  is the size of initial DL bandwidth part if CORESET 0 is not configured for the cell

-	Time domain resource assignment - 4 bits as defined in Clause 5.1.2.1 of [6, TS38.214]

-	VRB-to-PRB mapping - 1 bit according to Table 7.3.1.2.2-5

-	Modulation and coding scheme - 5 bits as defined in Clause 5.1.3 of [6, TS38.214], using Table 5.1.3.1-1

-	TB scaling - 2 bits as defined in Clause 5.1.3.2 of [6, TS38.214]

-	LSBs of SFN - 2 bits for the DCI format 1_0 with CRC scrambled by MsgB-RNTI as defined in Clause 8.2A of [5, TS 38.213] if msgB-responseWindow is configured to be larger than 10 ms; or 2 bits for the DCI format 1_0 with CRC scrambled by RA-RNTI as defined in Clause 8.2 of [5, TS 38.213] for operation in a cell with shared spectrum channel access if ra-ResponseWindow or ra-ResponseWindow-v1610 is configured to be larger than 10 ms; 0 bit otherwise

-	Reserved bits - (16 - A) bits for operation in a cell without shared spectrum access in frequency range 1 and frequency range 2-1, (18 - A) for operation in a cell with shared spectrum access in frequency range 1 or for operation in a cell in frequency range 2-2, where the value of A is the number of bits for the field of 'LSBs of SFN' as defined above

The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by TC-RNTI:

-	Identifier for DCI formats - 1 bit

-	The value of this bit field is always set to 1, indicating a DL DCI format

-	Frequency domain resource assignment - bits

-	 is the size of CORESET 0

-	Time domain resource assignment - 4 bits as defined in Clause 5.1.2.1 of [6, TS38.214]

-	VRB-to-PRB mapping - 1 bit according to Table 7.3.1.2.2-5

-	Modulation and coding scheme - 5 bits as defined in Clause 5.1.3 of [6, TS38.214], using Table 5.1.3.1-1

-	New data indicator - 1 bit

-	Redundancy version - 2 bits as defined in Table 7.3.1.1.1-2

-	HARQ process number - 4 bits

-	Downlink assignment index - 2 bits

-	2 bits indicating the number of repetitions for PUCCH as defined in clause 9.2.6 of [5, TS38.213] according to Table 7.3.1.2.1-4, if the higher layer parameter numberOfMsg4HARQ-ACK-Repetitions is configured with at least two repetition factors and the UE has indicated capability of PUCCH repetition on common PUCCH resource [8, TS38.321];

-	otherwise, reserved.

-	TPC command for scheduled PUCCH - 2 bits as defined in Clause 7.2.1 of [5, TS38.213]

-	PUCCH resource indicator - 3 bits as defined in Clause 9.2.3 of [5, TS38.213]

-	PDSCH-to-HARQ_feedback timing indicator - 3 bits as defined in Clause 9.2.3 of [5, TS38.213]

-	ChannelAccess-CPext - 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4, or Table 7.3.1.1.1-4A if channelAccessMode-r16 = "semiStatic" is provided, for operation in a cell with shared spectrum channel access in frequency range 1; 2 bits indicating channel access type as defined in Table 7.3.1.1.1-4B if ChannelAccessMode2-r17 is provided for operation in a cell in frequency range 2-2; otherwise 0 bit

-	Reserved bits - 2 bits when the DCI format is monitored in common search space for operation in a cell in frequency range 2-2 and the number of bits for the field of 'ChannelAccess-CPext' is 0; 0 bits otherwise

Table 7.3.1.2.1-1: Short Message indicator

| Bit field | Short Message indicator |
| --- | --- |
| 00 | Reserved |
| 01 | Only scheduling information for Paging, and TRS availability indication if trs-ResourceSetConfig is configured, are present in the DCI |
| 10 | Only short message, and TRS availability indication if trs-ResourceSetConfig is configured, are present in the DCI |
| 11 | Both scheduling information for Paging, TRS availability indication if trs-ResourceSetConfig is configured and short message are present in the DCI |

Table 7.3.1.2.1-2: System information indicator

| Bit field | System information indicator |
| --- | --- |
| 0 | SIB1 [9, TS38.331, Clause 5.2.1] |
| 1 | SI message [9, TS38.331, Clause 5.2.1] |

Table 7.3.1.2.1-3: PRACH retransmission indicator

| Bit field | PRACH retransmission indicator |
| --- | --- |
| 0 | Initial transmission of PRACH |
| 1 | Retransmission of PRACH |

Table 7.3.1.2.1-4: Number of repetitions NPUCCHrepeat as a function of 2 bits of Downlink assignment index field

| Bit field | NPUCCHrepeat |
| --- | --- |
| 00 | First repetition factor configured by numberOfMsg4HARQ-ACK-Repetitions |
| 01 | Second repetition factor configured by numberOfMsg4HARQ-ACK-Repetitions |
| 10 | Third repetition factor configured by numberOfMsg4HARQ-ACK-Repetitions if provided, otherwise reserved |
| 11 | Fourth repetition factor configured by numberOfMsg4HARQ-ACK-Repetitions if provided, otherwise reserved |