3GPP TSG-RAN WG1 Meeting #120	R1-2501572

Athens, Greece, February 17th – 21st, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | 0210 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on determination of DCI format 0_3 in case of dormancy |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Ericsson, Fujitsu, Nokia, Samsung, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MC_enh-Core |  |  |  |  |  | Date: |  |  | 2025-2-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In TDD, DL and UL switch BWP simultaneously since the DL and UL BWPs are linked. If the active DL BWP switches to a DL dormant BWP, the active UL BWP will switch to the UL BWP with the same BWP-Id as the DL dormant BWP. Since there will be no UE behavior on the active UL BWP, this UL BWP may not be configured with some parameters related to uplink scheduling, and this results in ambiguity regarding size of the DCI format 0_3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the corrections for: if the SCell within the cell set is dormant, the UE determines the bitwidth of fields in DCI format 0_3 based on the UL BWP provided by BWP-Id equal to firstWithinActiveTimeBWP-Id if provided; otherwise, based on a UL BWP provided by BWP-Id equal to firstOutsideActiveTimeBWP-Id for the SCell. No differentiation between FDD and TDD as to have a unified UE behavior. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE may fail to detect DCI format 0_3 due to the uncertainty in determining the size of DCI format 0_3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 7.3.1.1.4	Format 0_3

< Unchanged parts are omitted >

If scheduledCellComboListDCI-0-3 for the cell set is configured, zeros shall be appended to DCI format 0_3 if needed until the payload size equals the size of DCI format 0_3 that is determined by the configuration of the corresponding active bandwidth part(s) of the scheduled cells in the entry which results in the largest size among the entries in the higher layer parameter scheduledCellComboListDCI-0-3.

If an SCell within the scheduled cell set is deactivated, the UE determines the bitwidth of the fields in DCI format 0_3 based on a UL BWP provided by firstActiveUplinkBWP-Id for the SCell. If the active DL BWP of an SCell within the scheduled cell set is a dormant DL BWP, the UE determines the bitwidth of the fields in DCI format 0_3 based on the UL BWP provided by BWP-id equal to firstWithinActiveTimeBWP-Id for the SCell if provided; otherwise, based on a UL BWP provided by BWP-id equal to firstOutsideActiveTimeBWP-Id for the SCell.

< Unchanged parts are omitted >