3GPP TSG RAN meeting #107										RP-250241

Incheon, Korea, March 12-14, 2025

Status Report to TSG

Agenda item:			9.2.2

| WI / SI Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: No | Performance part: No |  | Testing part: No |
| Acronym | FS_Sensing_NR |  |  |  |  |
| Unique ID | 1020086 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-242348 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: 06/2025 | Core part: | Performance part: | Testing part: |  |
| Overall Completion level | Study Item: 70% | Core part: | Performance Part: | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Yingyang Li, Jerome Vogedes |
|  | Company | Xiaomi, AT&T |
|  | Email | liyingyang@xiaomi.com, jerome.vogedes@att.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

RAN1 #120, Athens, Greece, Feb 17-21, 2025

ISAC deployment scenarios

Agreement

For ISAC channel modelling calibration, RAN1 considers both large-scale and full-scale calibration to include parameters and values for at least the following:

    - large scale parameters, where fast fading is not included

    - full-scale calibration parameters, which includes fast fading.

- NOTE0: one part of calibration work does not include additional components and does not include spatial consistency

    - FFS: whether spatial consistency is specified as an additional component for ISAC CM

- NOTE1: additional calibrations including spatial consistency can also be considered case by case for different scenarios.

- NOTE2: Inclusion of EO in ISAC CM calibrations can also be considered case by case for different scenarios.

Agreement

Calibration of ISAC CM includes separate calibration of the target channel and of the background channel

- FFS: additional calibration for the combined channel (combination of target and background channel).

Agreement

For the purposes of large scale calibration for UAV sensing targets, the following calibration parameters are proposed below in Table x.

Table x. Simulation assumptions for large scale calibration for UAV sensing targets

| Parameters | Values |
| --- | --- |
| Scenario | UMa-AV |
| Sensing mode | TRP monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-UE bistatic Note: further down-selection of the sensing modes for UAV sensing is not precluded |
| Sectorization | 3 sectors per cell site: 30, 150 and 270 degrees |
| Carrier Frequency | FR1: 6 GHz FR2: 30 GHz |
| BS antenna configurations | Single dual-pol isotropic antenna |
| BS Tx power | FR1: 56dBm FR2: 41dBm |
| Bandwidth | FR1: 100MHz FR2: 400MHz |
| BS noise figure | FR1: 5dB FR2: 7dB |
| UT antenna configurations | (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1) |
| UT noise figure | FR1: 9dB FR2: 10dB |
| Sensing target distribution | 1 target uniformly distributed (across multiple drops) within the center cell. Vertical distribution: Fixed height value of 200 m. |
| Component A of the RCS for each scattering point | a fixed value of A |
| Minimum 3D distances between pairs of Tx/Rx and sensing target | 10 m |
| Wrapping Method | No wrapping method is used if interference is not modelled, otherwise geographical distance based wrapping |
| Metrics | Coupling loss (based on LOS pathloss) - FFS: how to select sensing Tx and Rx FFS: additional metrics, wideband SIR and SINR based on RSRP if interference is modelled. |

ISAC channel modelling

Agreement

For bistatic/monostatic RCS

- RCS values/pattern for a scattering point of a target for bistatic sensing is generated by A*B1*B2 (i.e., Option 3 from the agreement in RAN1 #118bis)

- RCS values/pattern obtained by setting the same incident/scattered angle in the RCS model for bistatic sensing should be aligned with RCS for monostatic sensing

Agreement

RCS model and application in ISAC channel generation

- To define the RCS model (RCS=A*B1*B2) for a scattering point of a target, when the target type is vehicle, large size UAV, human with RCS model 2, AGV

  - The values/pattern of the product of component A and B1, i.e., A*B1 is given per target type, expressed in dBsm scale

  - Component A is expressed in dBsm scale. B1 is dependent on A*B1 and value of component A.

  - A is equal to a single value per target type

    - FFS: this allows different values for the same target type with different size, if needed

    - FFS: this allows different values for monostatic and bistatic sensing, if needed

  - Component B2 follows log-normal distribution. The mean μB2_dB and variance σB2_dB2 used to characterize 10lgB2 satisfied a fixed relation μB2_dB=-ln1020σB2_dB2.

- In the procedure of generating ISAC target channel, B1*B2 is applied after coupling of rays for a STX-SPST link and the corresponding SPST-SRX link before path dropping

- In the procedure of generating ISAC target channel, the following power scaling factor is applied in the last step in target channel generation (i.e., step 14 in the running CR).

LTX-SPST-RX=PLdBd1+PLdBd2+10lgc24πf2-10lgσRCS,A+SFdB,1+SFdB,2

Where,

- PLdBd1 is pathloss between Tx and SPST, where d1 is the distance between Tx and SPST

- PLdBd2 is pathloss between Rx and SPST, where d2 is the distance between SPST and Rx

- σRCS,A is the value of RCS component A

- SFdB,1,SFdB,2 are shadow fading respectively generated for the Tx- SPST link and SPST -Rx link referring to step 4 in section 7.5, TR 38.901

- Note: for monostatic sensing, PLdBd1=PLdBd2,SFdB,1=SFdB,2

Agreement

RCS upper bound: k equals to 3 is adopted to derive the upper bound of RCS component B2, kσ, where σ is the standard deviation of B2 in dB

Agreement

For reducing options for reference TRs: for sensing scenario UMi, UMa, RMa, InH, InF, UMi-AV, UMa-AV, and RMa-AV, the reference TR to generate a TRP-TRP channel is:

- TRP-TRP link of scenario UMi, UMa, InH, and InF following the option based on TR 38.901 defined in section A.3 of TR 38.858

  - For InF, hUE is changed to the same height as the BS

- TRP-UE link of scenario RMa defined in section 7 of TR 38.901 by setting hUE=35m

- FFS: whether to add very low power clusters

R1-2501000	Summary #2 on ISAC channel modelling	Moderator (Xiaomi)

Agreement

For vehicle with single/multiple scattering points:

- For mono-static, the RCS=A*B=A*B1*B2 for a scattering point of a vehicle is generated by

  - The values/pattern A*B1, i.e., rcsdB(θ,φ) is deterministic based on incident/scattered angles

rcsdB(θ,φ)=Gmax-min-σVdBθ+σHdB φ,σmax

Where,

σVdBθ=-min12θ-θcenterθ3dB2, σmax,

σHdB φ=-min12φ-φcenterφ3dB2, σmax,

For example, in case of vehicle with multiple scattering points:

|  | φcenter | φ3dB, n | θcenter | θ3dB,n | Gmax | σmax | Applicable Range of θ | Applicable Range of φ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Left | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] |
| Back | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] |
| Right | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] |
| Front | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] |
| Roof | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] |

  - Note: the applicable angular range is 360 degrees per row in horizontal domain in case of vehicle with multiple scattering points, and the applicable angular range is < 360 degrees per row in horizontal domain in case of vehicle with a single scattering point.

    - FFS: angular continuity

Working assumption

For modelling background channel for monostatic sensing:

Solution A: (previous Option 1)

- drop N reference point(s) following certain distribution, and then the background channel is generated based on the channel generated as in existing TR between the real Tx and the reference point for a scenario.

  - The distance between Tx and the reference point, and the height of the reference point follow Gamma distribution,

drp_2D_n~Γα1, β1+c1

hrp_n~Γα2, β2+c2

  - FFS: value of N (N<4)

- FFS: whether to add additional very low power clusters

- FFS: any update to parameters e.g. angular distribution and delay spread

Agreement

The existing spatial consistency model in TR 38.901 is reused to model correlation of links between one TRP and different STs/UEs.

Agreement

Spatial consistency is not modelled at least for the following links

- Case 1: the links from/to two non-co-located TRPs

  - Link TRP1-X and link TRP2-X, where node X can be a target, a UE or another TRP

  - Link TRP1-X and link TRP2-Y, where node X, Y can be a target, a UE or another TRP

- Case 2: the two links with different LOS/NLOS condition

- Case 3: background channel for monostatic vs. any link (Tx-ST, ST-Rx, background channel) for bistatic

R1-2501001	Summary #3 on ISAC channel modelling	Moderator (Xiaomi)

Agreement

For sensing scenario UMi, UMa, RMa, InH, InF, UMi-AV, UMa-AV, and RMa-AV, the reference TR to generate a UE-UE channel is

- UE-UE link of scenario UMi, UMa, InH, and InF following the option based on TR 38.901 defined in section A.3 of TR 38.858

- TRP-UE link of scenario RMa defined in section 7 of TR 38.901 by setting hBS =1.5m

- FFS: whether to add very low power clusters

Agreement

The reference TR to generate a TRP-UE channel is

| TRP | normal UE | UMi, UMa, RMa, InH, InF, UMi-AV, UMa-AV, and RMa-AV - Option 1: TRP-UE link of scenario UMi, UMa, RMa, InH, and InF in section 7 of TR 38.901 Highway and Urban grid - Option 1: P2B link of scenario Highway and Urban grid in section 6 of TR 37.885 HST - Option 1: TRP-UE link of scenario RMa in section 7 of TR 38.901 for FR1 and TRP-UE link of scenario UMa in section 7 of TR 38.901 for FR2 |
| --- | --- | --- |
| TRP | vehicle UE | Highway and Urban grid - Option 1: V2B link of scenario Highway and Urban grid in section 6 of TR 37.885 UMi, UMa, and RMa - Option 1: TRP-UE link of scenario UMi, UMa, and RMa in section 7 of TR 38.901 |
| TRP | aerial UE | UMa-AV, UMi-AV, and RMa-AV - Option 1: - TRP-aerial UE link of scenario UMa-AV, UMi-AV, and RMa-AV in section Annex A and B of TR 36.777 for FR1 - FFS reuse the channel model of scenario UMa-AV, UMi-AV, and RMa-AV of FR1 for FR2 |

Agreement

For mono-static, the following values of component A, B2 are agreed for UAV of small size

  - Component A: -12.81 dBsm

  - Component B1: 0 dB (already agreed in RAN1#118bis)

  - Component B2:

    - standard deviation: 3.74 dB

Agreement

For mono-static, the following values of component A, B2 are agreed for RCS model 1 of human

  - Component A: -1.37 dBsm

  - Component B1: 0 dB (already agreed in RAN1#118bis)

  - Component B2:

    - standard deviation: 3.94 dB

Working assumption

Absolute delay model (referring to 7.6.9 in TR 38.901 as starting point) is a mandatory feature for both target channel and background channel for ISAC for UMi, UMa, InH, InF

- Related model referring to Δτ values from 7-24GHz study item

Agreement

When absolute delay model Δτ is configured, it applies to all NLOS clusters in each of Tx-target and target-Rx links and background channel.

- For bistatic sensing: Different values of Δτ are separately generated for the Tx-target link, target-Rx link and the background channel

- For monostatic sensing: the same value of Δτ is used for Tx-target link and target-Rx link, and a different value of Δτ is separately generated for the background channel

Agreement

To generate the LOS ray and NLOS clusters for the multiple scattering points, each scattering point is separately handled as if a different target with single scattering point.

Agreement

- The LOS condition between Tx/Rx and each of the multiple scattering points of a same target are individually generated

- The pathloss between Tx/Rx and each of the multiple scattering points of a same target are individually generated

Agreement

For a target with single/multiple scattering points, the 3D location of each scattering point is defined in the evaluation assumptions.

Agreement

Spatial consistency is needed to model correlation of the following links from ST-UT links and UT-UT links

- Case 5: links between same UT and two nodes X/Y, subjected to correlation distance, i.e., link UT1-X and link UT1-Y, where nodes X/Y can be target or UT

- Case 6: links between same target and two nodes X/Y, subjected to correlation distance, i.e., link target1-X and link target1-Y, where nodes X, Y are different UTs

- Case 7: link X1-Y1 and link X2-Y2, subjected to correlation distance, where X1, X2, Y1, Y2 are 4 different nodes

- FFS: Spatial consistency between multiple scattering points of the same target

Agreement

Correlation type is introduced for large scale parameter, cluster specific parameter and ray specific parameter of ST-UT links and UT-UT links

- Definition of link Correlated: parameters for any two links between STs/UTs are correlated, subjected to correlation distance.

Table 4: Correlation type for links between STs/UTs

| Parameters | Correlation type |
| --- | --- |
| Delays | link Correlated |
| Cluster powers | link Correlated |
| AOA/ZOA/AOD/ZOD offset | link Correlated |
| AOA/ZOA/AOD/ZOD sign | link Correlated |
| Random coupling | link Correlated |
| XPR | link Correlated |
| Initial random phase | link Correlated |
| LOS/NLOS states | link Correlated |
| Blockage (Model A) | All-correlated |
| O2I penetration loss | All-correlated |
| Indoor distance | All-correlated |
| Indoor states | All-correlated |

- Note: it is not precluded more parameters for spatial consistency can be discussed and added in the table

Agreement

If a target is modelled with multiple scattering points,

- The number of scattering points of the target is generated in the beginning of the simulation and kept unchanged in the whole simulation

- The number and locations of the scattering points of the target (if it is a vehicle) are common to each pair of sensing Tx/Rx

- RAN1 assumes no ray is scattered from one scattering point to another scattering point of the same target

- RCS values of the multiple scattering points are individually determined

Agreement

To model polarization matrix CPMsp,i=αi,1expjΦsp,iθθβi,1expjΦsp,iθϕβi,2expjΦsp,iϕθαi,2expjΦsp,iϕϕ of a direct/indirect path i of a scattering point of a target

  - in Rel-19 study item (e.g., UAV, human, vehicle, AGV), αi,1=αi,2=1, βi,1=βi,2=κsp,i-1, i.e.,

CPMsp,i=expjΦsp,iθθκsp,i-1expjΦsp,iθϕκsp,i-1expjΦsp,iϕθexpjΦsp,iϕϕ

Where,

    - κsp,i is XPR ratio is randomly generated by log-normal distribution per target type

    - The initial random phase Φsp,iθθ,Φsp,iθϕ,Φsp,iϕθ,Φsp,iϕϕ is uniformly distributed within -π,π

  - FFS: spatial consistency of random phase when a scattering point moves

  - FFS: whether the base station rotation procedure in 38.901 can be reused to support rotation of the target

Agreement

- For target channel

  - The power threshold for removing clusters in step 6 in section 7.5, TR 38.901, i.e., -25 dB is reused to generate Tx-target link and target-Rx link

  - The power threshold for path dropping after concatenation is relaxed to [X=-40] dB

- For background channel

  - The power threshold for removing clusters in step 6 in section 7.5, TR 38.901, i.e., [-25 dB] is reused to generated the background channel

    - FFS: whether to add additional very low power clusters

    - FFS: The reference power for removing cluster is the min (max. Tx-target link cluster power, max. target-Rx link cluster power)

Agreement

If EO type-2 is modelled in an indirect path, only specular reflection is modeled for EO type-2

- polarization of the indirect path is product of polarization matrix of the target-Rx link, the target, and the Tx-target link, i.e., CPMtx,sp,rx= CPMsp,rx . CPMsp . CPMtx,sp

  - For the specular reflected ray generated by a EO type-2 in the Tx-target link or the target-Rx link (i.e., Tx-EO type-2-target, or target-EO type-2-Rx), CPMtx,sp or CPMsp,rx is the polarization matrix of EO type-2

  - To generate polarization matrix of EO type-2, the procedure in [R1-2409394, R1-2410648] is taken as starting point

    - FFS applicability if the surface of EO type-2 is tilted

2.1.2	Remaining Open issues

The following open issues need to be addressed:

- ISAC deployment scenarios

  - Remaining calibration parameters for full calibration, and for calibration of additional features

  - Calibration results collection/analysis

- ISAC channel modelling

  - Physical object model

    - Framework and values for bistatic RCS

    - Remaining issues on values for monostatic RCS

    - XPR values of polarization matrix of target

  - Channel model

    - Remaining issues of basic ISAC channel model, e.g.,

      - Polarization matrix normalization

      - Parameter values for monostatic background channel

      - Details on power normalization of target channel and background channel

    - Remaining issues on channel model for target with multiple scattering points

    - Remaining issues on EO type-2

    - Remaining issues on spatial consistency

      - Correlation among the multiple scattering points

      - New model for spatial consistency for ‘link correlated’

  - Link level channel model

  - Others

    - Angular correlation of RCS

    - Blockage model A/B

    - Micro-Doppler

    - New low power clusters in background channel

2.2	RAN2

2.2.1	Agreements

2.2.2	Remaining Open issues

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

2.4.2	Remaining Open issues

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1 #120

R1-2500998	Draft CR for TR 38.901 to introduce channel model for ISAC	Xiaomi, AT&T

R1-2500059	Email discussion summary on ISAC CM calibration assumptions	Moderator (AT&T)

R1-2500071	Deployment scenarios for ISAC channel model	Huawei, HiSilicon

R1-2500234	Discussion on ISAC deployment scenarios	CATT, CICTCI

R1-2500266	Discussion on ISAC deployment scenarios	China Telecom

R1-2500297	Discussion on ISAC deployment scenarios	CMCC, China Southern Power Grid

R1-2500312	Discussion on ISAC scenario	CALTTA

R1-2500360	Views on Rel-19 ISAC deployment scenarios	vivo

R1-2500413	Discussion on ISAC deployment scenarios and requirements	EURECOM

R1-2500417	Discussion on ISAC deployment scenarios	Tiami Networks

R1-2500462	Discussion on ISAC deployment scenarios and calibration	OPPO

R1-2500482	Discussion on ISAC Deployment Scenarios 	Tejas Network Limited

R1-2500576	Discussion on ISAC deployment scenarios	ZTE Corporation, Sanechips

R1-2500659	Discussion on ISAC deployment scenarios	Sony

R1-2500679	Discussion on ISAC Deployment Scenarios	Nokia, Nokia Shanghai Bell

R1-2500684	Discussion on ISAC deployment scenarios	SK Telecom

R1-2500691	Deployment scenarios for integrated sensing and communication with NR	NVIDIA

R1-2500742	Deployment scenarios and evaluation assumptions for ISAC channel model	Xiaomi

R1-2500747	Discussion on ISAC deployment scenarios	InterDigital, Inc.

R1-2500755	Discussion on ISAC Deployment Scenarios	Ericsson

R1-2500796	Discussion on ISAC deployment scenarios	Apple

R1-2500860	Discussion on ISAC deployment scenarios	Samsung

R1-2500891	Considerations on ISCA deployment scenarios	CAICT

R1-2501011	Discussion on ISAC deployment scenarios for Automotive	Continental Automotive

R1-2501026	Discussion on ISAC deployment scenario	MediaTek Inc.

R1-2501045	Discussion on ISAC deployment scenarios	LG Electronics

R1-2501059	Discussion on ISAC deployment scenarios 	Lenovo

R1-2501081	ISAC channel model calibration and scenario parameters	AT&T, FirstNet

R1-2501135	Discussion on ISAC channel calibration	BUPT, CMCC

R1-2501166	Discussion on ISAC deployment scenarios	Qualcomm Incorporated

R1-2501076	FL Summary #1 on ISAC Scenarios and Calibrations	Moderator (AT&T)

R1-2501077	FL Summary #2 on ISAC Scenarios and Calibrations	Moderator (AT&T)

R1-2501078	FL Summary #3 on ISAC Scenarios and Calibrations	Moderator (AT&T)

R1-2501574	FL Summary #4 on ISAC Scenarios and Calibrations	Moderator (AT&T)

R1-2500072	Channel modelling for ISAC	Huawei, HiSilicon

R1-2500179	Discussion on ISAC channel modeling	Spreadtrum, UNISOC

R1-2500235	Discussion on ISAC channel modelling	CATT, CICTCI

R1-2500267	Discussion on ISAC channel modelling	China Telecom

R1-2500298	Discussion on channel modeling methodology for ISAC	CMCC, BUPT, SEU, PML

R1-2500313	Discussion on ISAC channel modelling	CALTTA

R1-2500361	Views on Rel-19 ISAC channel modelling	vivo, BUPT

R1-2500414	Discussion on ISAC channel modeling	EURECOM

R1-2500418	Discussion on ISAC Channel Modeling	Tiami Networks

R1-2500463	Study on ISAC channel modelling	OPPO

R1-2500483	Discussion on ISAC channel modelling	Tejas Network Limited

R1-2500577	Discussion on channel modelling for ISAC	ZTE Corporation, Sanechips

R1-2500626	ISAC Channel Modeling and Measurement Validation	BUPT, CMCC, VIVO

R1-2500660	Discussion on Channel Modelling for ISAC	Sony

R1-2500680	Discussion on ISAC channel modeling	Nokia, Nokia Shanghai Bell

R1-2500681	Discussion on ISAC Channel Modeling	NIST

R1-2500685	Discussion on ISAC channel modelling	SK Telecom

R1-2500692	Channel modeling for integrated sensing and communication with NR	NVIDIA

R1-2500743	Discussion on ISAC channel model	Xiaomi, BJTU, BUPT

R1-2500748	Discussion on ISAC channel modeling	InterDigital, Inc.

R1-2500756	Discussion on ISAC Channel Modelling	Ericsson

R1-2500797	Discussion on ISAC channel modelling	Apple

R1-2500861	Discussion on ISAC channel modelling	Samsung

R1-2500892	Considerations on ISAC channel modelling	CAICT

R1-2500979	Discussion on ISAC Channel Modelling 	Panasonic

R1-2501027	Discussion on ISAC channel modelling	MediaTek Inc.

R1-2501046	Discussion on ISAC channel modelling	LG Electronics

R1-2501060	Discussion on Channel Modelling for ISAC	Lenovo

R1-2501082	Discussions on ISAC Channel Modeling	AT&T

R1-2501167	Discussion on ISAC channel modelling	Qualcomm Incorporated

R1-2501212	Discussion on ISAC channel modeling	NTT DOCOMO, INC.

R1-2500999	Summary #1 on ISAC channel modelling	Moderator (Xiaomi)

R1-2501000	Summary #2 on ISAC channel modelling	Moderator (Xiaomi)

R1-2501001	Summary #3 on ISAC channel modelling	Moderator (Xiaomi)

R1-2501002	Summary #4 on ISAC channel modelling	Moderator (Xiaomi)

10.11.2023		minor adaptations for RAN #102

02.08.2023		minor adaptations for RAN #101

26.04.2023		minor adaptations for RAN #100

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template