3GPP TSG RAN Meeting #107	                                 RP-250263

Incheon, Korea, March 12-14, 2025

Source: 	CMCC

Title:	Discussion on IMT-2030 TPRs

Agenda item:	16

Document for:	Discussion & Decision

Background

In response to the Liaison Statement (LS) from ITU-R WP5D (Document 5D/TEMP/167) on Minimum requirements related to technical performance (TPR) for IMT-2030 radio interface(s) [1], 3GPP TSG RAN#106 plenary initiated a study on 6G scenarios/use cases and requirements [2]. One important aspect of the study is to investigate a candidate set of items for minimum TPRs for IMT-2030 radio interfaces based on the Recommendation ITU-R M.2160 [3], and the associated target values for the identified minimum TPRs.

During RAN#106 plenary, 3GPP TSG RAN had an initial discussion on the possible TPRs based on Candidate IMT-2030 TPRs under discussion in WP 5D [4] and the inputs from companies. As a result, a reply LS to ITU-R [5] was endorsed and the following items for candidate TPRs for IMT-2030 were proposed for consideration:

1.	Peak data rate

2.	Peak spectral efficiency

3.	User experienced data rate

4.	5th percentile user spectral efficiency

5.	Average spectral efficiency

6.	Sustainability/Energy efficiency

7.	Area traffic capacity

8.	User plane latency

9.	Control plane latency

10.	Connection density

11.	Reliability

12.	Mobility

13.	Mobility interruption time

14.	Bandwidth

In addition, to ensure timely inputs to ITU-R WP5D, the following study plan was also endorsed in RAN#106 plenary  [5]:

- RAN#107 (March 2025):

  - Aim to finalize the candidate set of TPRs

  - Initial discussion on the associated values of the identified TPRs

- RAN#108 (June 2025):

  - Aim to finalize the associated values of the identified TPRs

- After RAN#108:

  - Any remaining issues on 6G focusing on ITU-R may continue, if needed

According to the above study plan, this contribution continues to discuss the candidate set of TPRs, the associated target values and the evaluation methods. On the one hand, for the candidate TPRs agreed in the RAN#106 plenary, this contribution proposes definitions and associated target values, where the definitions of some TPRs keep unchanged and the definitions of TPRs are revised based on the extended usage scenarios described in ITU-R Recommendation M.2160, compared to the definitions in IMT-2020 [6]. Further details on the “unchanged” and “revised” TPRs are given below in Section 2. On the other hand, this contribution also proposes some “new” TPRs defined for extended and new usage scenarios for IMT-2030, which are provided below in Section 3.

TPRs inherited from IMT-2020

As outlined in the preceding section, this section proposes definitions and corresponding target values for the candidate TPRs that were agreed upon during the RAN#106 plenary meeting, which are inherited from IMT-2020. It is noteworthy that compared to the definitions established for IMT-2020, the definitions of most TPRs remain consistent, while one of them is revised to align with the expanded usage scenarios detailed in ITU-R Recommendation M.2160.

Definition-unchanged TPRs

For the following thirteen candidate TPRs inherited from IMT-2020 TPRs shown in Table 1, the definitions captured from IMT-2020 [6] can be reused. For simplicity, these candidate TPRs are called definition-unchanged TPRs in this contribution.

Proposal 1: For the candidate IMT-2030 TPRs endorsed in RAN#106 plenary (except for energy efficiency), the definitions captured from IMT-2020 can be reused, as shown in the following Table 1.

Table 1: Definition-unchanged TPRs inherited from IMT-2020 TPRs

|  | TPR Items | Definition |
| --- | --- | --- |
| 1 | Peak data rate | Peak data rate is the maximum achievable data rate under ideal conditions (in bit/s), which is the received data bits assuming error-free conditions assignable to a single mobile station, when all assignable radio resources for the corresponding link direction are utilized (i.e. excluding radio resources that are used for physical layer synchronization, reference signals or pilots, guard bands and guard times). |
| 2 | Peak spectral efficiency | Peak spectral efficiency is the maximum data rate under ideal conditions normalised by channel bandwidth (in bit/s/Hz), where the maximum data rate is the received data bits assuming error-free conditions assignable to a single mobile station, when all assignable radio resources for the corresponding link direction are utilized (i.e. excluding radio resources that are used for physical layer synchronization, reference signals or pilots, guard bands and guard times). |
| 3 | User experienced data rate | User experienced data rate is the 5% point of the cumulative distribution function (CDF) of the user throughput. User throughput (during active time) is defined as the number of correctly received bits, i.e. the number of bits contained in the service data units (SDUs) delivered to Layer 3, over a certain period of time. |
| 4 | 5th percentile user spectral efficiency | The 5th percentile user spectral efficiency is the 5% point of the CDF of the normalized user throughput. The normalized user throughput is defined as the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time, divided by the channel bandwidth and is measured in bit/s/Hz. |
| 5 | Average spectral efficiency | Average spectral efficiency is the aggregate throughput of all users (the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time) divided by the channel bandwidth of a specific band divided by the number of TRxPs and is measured in bit/s/Hz/TRxP. |
| 6 | Area traffic capacity | Area traffic capacity is the total traffic throughput served per geographic area (in Mbit/s/m2). The throughput is the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time. |
| 7 | User plane latency | User plane latency is the contribution of the radio network to the time from when the source sends a packet to when the destination receives it (in ms). It is defined as the one-way time it takes to successfully deliver an application layer packet/message from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface in either uplink or downlink in the network for a given service in unloaded conditions, assuming the mobile station is in the active state. |
| 8 | Control plane latency | Control plane latency refers to the transition time from a most “battery efficient” state (e.g. Idle state) to the start of continuous data transfer (e.g. Active state). |
| 9 | Connection density | Connection density is the total number of devices fulfilling a specific quality of service (QoS) per unit area (per km2). |
| 10 | Reliability | Reliability relates to the capability of transmitting a given amount of traffic within a predetermined time duration with high success probability. Reliability is the success probability of transmitting a layer 2/3 packet within a required maximum time, which is the time it takes to deliver a small data packet from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface at a certain channel quality. |
| 11 | Mobility | Mobility is the maximum mobile station speed at which a defined QoS can be achieved (in km/h). |
| 12 | Mobility interruption time | Mobility interruption time is the shortest time duration supported by the system during which a user terminal cannot exchange user plane packets with any base station during transitions. |
| 13 | Bandwidth | Bandwidth is the maximum aggregated system bandwidth. The bandwidth may be supported by single or multiple radio frequency (RF) carriers. The bandwidth capability of the RIT/SRIT is defined for the purpose of IMT-2030 evaluation. |

Then, regarding target values associated with the above candidate IMT-2030 TPRs, we have the following aspects of consideration:

- In contrast to those of IMT-2020 TPRs, the values of IMT-2030 TPRs should reflect the advanced capabilities of a new generation of mobile communication technology.

- For the spectral efficiency of IMT-2030, this contribution provides simulation results of massive MIMO systems (64 TXRUs with 128/256/512 antenna elements and 128 TXRUs with 512/1024 antenna elements) based on 6GHz carrier frequency. The detailed evaluation assumptions are given in Table A-1 and the detailed simulation results for spectral efficiency are shown in Table A-2 and Table A-3 under the scenarios of 10 users per TRP and 16 users per TRP, respectively (See Appendix). To be more intuitive, the multiple relationship between the simulated values of spectral efficiency and the corresponding target values in IMT-2020 is depicted in Figure 1 (64 TXRUs) and Figure 2 (128 TXRUs). According to the simulation results, 2-3 x IMT-2020 is recommended for the target values of 5th percentile user spectral efficiency and average spectral efficiency.

- For the user experienced data rate, according to the NR self-evaluation in 3GPP TR 37.910 [7], it can be observed that to satisfy the requirement on DL user experienced data rate defined for IMT-2020 [6], i.e., 100Mbit/s, the assumed DL system bandwidth is at least 160MHz. So, when determining the values of user experienced data rate for IMT-2030, the operators’ spectrum bandwidth should also be taken into account. A similar principle also applies to area traffic capacity. Based on these observations, 1.25-2 x IMT-2020 is recommended for the target values of user experienced data rate and area traffic capacity.

Figure 1: The multiple relationship between the simulated values of spectral efficiency and the corresponding target values in IMT-2020 (64 TXRUs)

Figure 2: The multiple relationship between the simulated values of spectral efficiency and the corresponding target values in IMT-2020 (128 TXRUs)

Based on the above considerations, the recommended target values for the above definition-unchanged TPRs are provided in Table 2.

Proposal 2: For the candidate IMT-2030 TPRs endorsed in RAN#106 plenary (except for energy efficiency), the associated target values in the following Table 2 can be considered as a starting point.

Table 2: Preliminary views on target values for definition-unchanged TPRs for IMT-2030

| TPRs |  | IMT-2020 KPIs (Report ITU-R M.2410) | NR self-evaluation (3GPP TR 37.910) | IMT-2030 Framework (Rec. ITU-R M.2160) | Next: What are IMT-2030 KPIs? |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Peak data rate (Gbps) | DL: 20Gbit/s UL: 10Gbit/s | 171.2Gbit/s @ FR2, DL, TDD(DSUUD), 748MHz BW | e.g. 50, 100, 200Gbit/s | DL: 60-100Gbit/s UL: 30-50Gbit/s | 3-5 x IMT-2020 |
| 2 | Peak spectral efficiency (bps/Hz) | DL: 30bit/s/Hz UL: 15bit/s/Hz | 46.8bit/s/Hz @FR1, DL, TDD(DSUUD), 100MHz BW | SE: e.g. 1.5x, 3x IMT-2020 | DL: 75-120bit/s/Hz UL: 37.5-60bit/s/Hz | 2.5-4 x IMT-2020 |
| 3 | User experienced data rate (Mbps) | DL: 100Mbit/s UL: 50Mbit/s | 104.66 Mbps @ Dense Urban, ISD 200m, 4GHz, DL, 160MHz BW, 32x4 MU-MIMO Type II Codebook; gNB Config = (8,16,2,1,1;1,16) | e.g. 300, 500Mbit/s | DL: 125-200Mbit/s UL: TBD | 1.25-2 x IMT-2020 |
| 4 | 5th percentile user spectral efficiency (bit/s/Hz) | Dense Urban - eMBB: DL: 0.225bit/s/Hz UL: 0.15bit/s/Hz | 0.68bit/s/Hz @ Dense Urban, 4 GHz, TDD, DL 64x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (12,8,2,1,1;4,8) | SE: e.g. 1.5x, 3x IMT-2020 | Dense Urban - Immersive: DL: 0.45-0.675bit/s/Hz UL: TBD | 2-3 x IMT-2020 |
| 5 | Average spectral efficiency (bit/s/Hz) | Dense Urban - eMBB: DL: 7.8bit/s/Hz UL: 5.4bit/s/Hz | 19.91bit/s/Hz @ Dense Urban, 4 GHz, TDD, DL 64x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (12,8,2,1,1;4,8) | SE: e.g. 1.5x, 3x IMT-2020 | Dense Urban - Immersive: DL: 15.6-23.4bit/s/Hz UL: TBD | 2-3 x IMT-2020 |
| 6 | Area traffic capacity (Mbit/s/m2) | Indoor Hotspot 10 Mbit/s/m2 | 10.66 Mbit/s/m2 @Indoor Hotspot, ISD 20m, 4 GHz, DL, 120MHz BW, 36TRxP | e.g. 30, 50 Mbit/s/m2 | Indoor Hotspot 12.5-20 Mbit/s/m2 | 1.25-2 x IMT-2020 |
| 7 | Latency (ms) - user plane | 4ms for eMBB 1ms for URLLC | 0.99ms @DL, TDD(DSUUD), SCS=30kHz, TypeBResource Mapping, UEcapability2 | e.g. 0.1-1ms | 2ms for eMBB 0.5ms for URLLC | 1/2 x IMT-2020 |
| 8 | Latency (ms) - control plane | 20ms | 12.9ms @DL, TDD(DSUUD), SCS=30kHz, TypeBResource Mapping, UEcapability2 | / | 10ms | 1/2 x IMT-2020 |
| 9 | Connection density (devices/km2) | 106 devices per km2 | 36,323,844 devices/km2 @180kHz Bandwidth, Evaluation configuration A (ISD=500 m) | e.g. 106-108 devices per km2 | 107-108 devices per km2 | 10-100 x IMT-2020 |
| 10 | Reliability | 1-10-5 | > 99.9999% | e.g. 1-10-5 -1-10-7 | 1-10-6 - 1-10-7 | 1/10-1/100 x IMT-2020 |
| 11 | Mobility | classes of mobility: Stationary: 0 km/h Pedestrian: 0 km/h to 10 km/h Vehicular: 10 km/h to 120 km/h High speed vehicular: 120 km/h to 500 km/h |  | e.g., 500 - 1000 km/h | ATG: 500 - 1200 km/h | Introduce a new class of mobility for ATG |
| 12 | Mobility interruption time | 0ms | Beam mobility: 0ms CA Mobility: 0ms | / | 0ms | 1 x IMT-2020 |
| 13 | Bandwidth | 100MHz |  | / | 200MHz | 2 x IMT-2020 |

Definition-revised TPRs

Sustainability/Energy efficiency

In IMT-2020 [6], Energy efficiency is defined as an inspection TPR while current ITU-R WP5D is discussing whether it can be a quantitative TPR in IMT-2030. Simultaneously, compare with 5G, 6G:

- Introduces higher frequency range (e.g. 6.425-7.125 GHz), which results in higher requirement on accuracy and power consumption for RF component.

- Introduces larger bandwidth (e.g. up to 200/400 MHz), which results in higher requirement (e.g. FFT size) and power consumption for BB component.

- Introduces larger TRX number (e.g. up to 128/256 TRX), which results more complexity (e.g. precoding in analog/digital domain) and higher power consumption for BB/IF/RF component.

Based on the above analysis, 6G has a higher demand for energy and energy efficiency should be given special attention. Therefore, it is recommended that sustainability/energy efficiency should be considered as a quantitative TPR (i.e. the evaluation method is simulation).

Proposal 3: Support sustainability/energy efficiency is a quantitative TPR (i.e. the evaluation method is simulation).

Furthermore, ITU-WP5D is discussing the evaluation method for sustainability/energy efficiency and is considering to define the relative power saving ratio under different load cases. From our point of view, such mechanism is a good starting point and can be further considered with the following aspects:

- The first point is how to calculate the power consumption for the network and UE. Thanks to the hard work during Rel-18/19 NES and Rel-16/17 UE power saving, the comprehensive power models for BS and UE are provided in TR 38.864 [10] and TR 38.840 [11], respectively. Such mechanism should be reused, or other power model should be clarified/reported by the companies, if any.

- The second point is how to ensure the experience of UE while enhancing the energy efficiency. Regarding such issue, we prefer to introduce the constraint on certain metric while evaluate the network EE or Device EE. To be specific:

  - The relative target of certain metric (e.g. 10% performance loss compared with the benchmark) can be set. Constant like certain percent (e.g. 90%) of the UE fulfils the target should be achieved while evaluating the relative power saving for each load case.

  - In addition, since different service may sensitive to different type of metric, it is recommended that the certain metric can be selected according to the service type, for instance:

    - For Vo6G (i.e. VoNR-like) service, the certain metric is latency

    - For burst-buffer service, the certain metric is UPT

    - For XR service, the certain metric is latency

    - For LPWA service, there is no certain metric (i.e. no constraint)

- The third point is the definition for reference case of relative power saving under UE EE. From our understanding, when NW is fully loaded, a certain UE may not be fully loaded due to there is more than one UE under each BS. Moreover, the load of UE under NW fully loaded case is strongly relevant to the number of UEs under a certain BS, simply reuse the definition of reference cases under NW EE will cause ambiguity. Therefore, it is recommended to clarify that the reference case of UE EE is when UE is fully-loaded/has full-buffer service, and it is also necessary to report the number of UE per BS and the traffic model.

Proposal 4: For sustainability/energy efficiency, support the following definition (changes are marked in red):

- Energy Efficiency Energy efficiency is an important metric of sustainability. Network energy efficiency is the capability of a RIT/SRIT to support radio access network energy saving in relation to the traffic capacity provided. Device energy efficiency is the capability of the RIT/SRIT to support device modem power saving in relation to the traffic characteristics. This requirement is defined for the purpose of evaluation in the Immersive communication usage scenario. The requirement is the relative energy savings between the selected load case(s) and a reference case. For network energy efficiency, the reference case is when network is fully loaded. For UE energy efficiency, the reference case is when UE is fully-loaded/has full-buffer service. Proponents should report the power model used for the evaluation (e.g. BS power model in 3GPP TR 38.864 and UE power model in 3GPP TR 38.840), the number of UE under each BS and the traffic model. The relative target of certain metric (e.g. 10% performance loss compared with the benchmark) can be set. Constant that [90%] percent of UE fulfils the target should be achieved while evaluating the relative power saving for each load case. The certain metric can be selected according to the service type: - For Vo6G (i.e. VoNR-like) service, the certain metric is latency - For burst-buffer service, the certain metric is UPT - For XR service, the certain metric is latency - For LPWA service, there is no certain metric (i.e. no constraint) The minimum requirements for Energy efficiency are summarized in Tables below. Table: The minimum requirements for Network Energy efficiency [Test environment(s)] **; Selected load case(s); The reference case; Relative power saving (%). [Service type, Power model, Device number, Traffic model]; [empty load, low load, light load and/or medium load] *; Fully loaded IMT-2030 case *; [X1%, X2%, ...]. *empty load: L=0, low load: 0 < L≤15, light load: 15 < L≤30, medium load: 30 < L≤50, fully loaded case: L=100 ** needed only in case of simulation Table: The minimum requirements for Device Energy efficiency [Test environment(s)] **; Selected load case(s); The reference case; Relative power saving (%). [Service type, Power model, Device number, Traffic model]; [empty load, low load, light load and/or medium load] *; Device is fully-loaded/has full-buffer service; [A1%, A2%, …].

New TPRs for IMT-2030

Joint requirement

To evaluate the performance of immersive communication services, e.g., XR, a new TPR considering joint requirements of latency, reliability, data rate, and capacity was discussed in the most recent ITU-R 48th meeting of Working Party 5D [9]. The preliminary definition of this TPR is copied as follows:

Copied from [9] This [requirement / title_placeholder] is defined as the [number of users / total traffic throughput of all users] that fulfil the required success probability (reliability) of transmitting a layer 2/3 packet within a required [maximum time / latency], which is the time it takes to deliver a data packet [for a given arrival rate] from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface at a certain channel quality. This requirement is defined for the purpose of evaluation in the Immersive Communication [and Hyper-Reliable Low Latency Communication] usage scenario.

In addition, by reviewing the 3GPP study item (SI) on XR evaluations for NR in Rel-17 (TR 38.838 [8]), it can be observed that the KPI identified for XR capacity study is system capacity, which is defined as the maximum number of users per cell with at least Y% of UEs being satisfied (Y=90 is baseline). A UE is declared as a satisfied UE if all the considered streams meet their own packet error rate (PER) and packet delay budget (PDB) requirements, i.e., more than X% of packets are successfully transmitted within a given air interface PDB. More specifically, PDB indicates the latency requirement of XR traffic in RAN side (i.e., air interface), and for a given packet, it is counted as failure if its delivery delay exceeds the given PDB. The packet success rate requirement for DL single stream is 99% as a baseline.

Considering that XR is a typical service for Immersive Communication usage scenario, the existing XR capacity evaluation can be reused for IMT-2030. Actually, the system capacity identified for Rel-17 XR SI also reveals a joint requirement on latency (PDB), data rate, capacity (number of users), and packet success rate. It should be noted that the packet success rate here is different from the reliability defined in IMT-2020, where the definition of reliability is the success probability of transmitting a packet within a required maximum delivery time, but the packet success rate is the percentage of packets which are successfully transmitted within a required PDB. That is to say, the KPI identified for XR capacity in Rel-17 is not concerned with the success probability of transmitting one packet. For a given packet, no matter whether re-transmission is needed, it is counted as a successfully transmitted packet as long as its delivery time satisfies the PDB requirement.

Based on the above observations, we recommend the following definition of joint requirement instead of the definition given in [9]:

This joint requirement is defined as the total number of users fulfilling a specific ratio of satisfied users. A user is declared as a satisfied user if more than a certain percentage of packets are successfully transmitted within a required maximum latency.

Proposal 5: Define joint requirement as a TPR for immersive communications services for IMT-2030.

- The joint requirement is defined as the total number of users fulfilling a specific ratio of satisfied users. A user is declared as a satisfied user if more than a certain percentage of packets are successfully transmitted within a required maximum latency.

- This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

Sensing-related capabilities

Integrated Sensing and Communication (ISAC) is defined as one of the six usage scenarios of IMT-2030, and will be the enabler for future networks to provide novel sensing services to customers. Representative use cases include unmanned aerial vehicle (UAV) detection, smart transportation, intelligent manufacturing, etc.

Detectability represents the basic performance of a sensing system. And it is represented by receiver operating characteristic (ROC) curve with the curve’s independent variable being false alarm probability (denoted as PFA) and its dependent variable being detection probability (denoted as PD). The PFA is defined as the probability of detecting the existence of a target when no target exists. And the PD is defined as the probability of detecting the existence of a target when a target does exist. However, ROC curve is hard to evaluated especially in condition of coloured noise. To simplified evaluation, one can evaluate the detectability performance by the value of PD with a given value of PFA. Note that either PD without PFA or PFA without PD is meaningless, and two pairs of PD and PFA are comparable if and only if their PD or PFA are equal. Therefore, we propose to evaluate detectability with a given PFA.

Proposal 6: 	Propose to use detection probability with a fixed false alarm probability as a basic TPR to evaluate detectability of the system.

For representative use cases mentioned above, localization accuracy is also fundamental. The sensing localization accuracy is defined as the three-dimensional (3D) distance between the measured 3D coordinate (x,y,z) and the actual coordinate. With this definition, the system performance remains unchanged after coordinate transformations.

Proposal 7: 	Propose to use localization accuracy as a TPR with its definition being the three-dimensional distance between the measured and the actual location of the sensing target with [95%] confidence.

Requirement of velocity measurement are commonly found in above use cases. And velocity accuracy can be similarly defined as localization accuracy.

Proposal 8: 	Propose to use velocity accuracy as a TPR with its definition being the three-dimensional distance between the measured and the actual velocity of the sensing target with [95%] confidence.

Sensing resolution represents the ability to distinguish between two adjacent targets. It is defined as the minimum distance between two distinguishable targets. The resolution usually relates to physical parameters of the system and signal (e.g. bandwidth, array aperture). The achievable resolution with given physical parameters is also affected by sensing algorithms, windowing and even the number of targets (for super resolution algorithms). Thus, the actual sensing resolution of a system may varies a lot.

Proposal 9: Propose to use sensing resolution as a TPR if and only if it is evaluated by nonanalytical approaches.

Sensing TPRs discussed above are defined based on one or two targets. However, for deployment of future sensing functionalities, TPRs for multiple targets must be considered. On the one hand, detectability or accuracy may degrade when multiple targets appear within a serving cell. Echo from different targets may interfere with each other. And those weak targets may be immersed by side-lobe of strong targets. On the other hand, given requirements on detectability and accuracy, sensing consumption may be further decreased by advance algorithms when number of targets within a serving cell decreases, which may affect communication functionalities.

Thus, we considered a system-level TPR named as sensing capacity to reflect multi-target sensing performance. Sensing capacity is defined as the maximum number of targets fulfilling QoS requirements (with [95%] confidence) within a region.

Proposal 10: Propose to use sensing capacity as a system-level TPR with its definition being the maximum number of the targets per unit area when all targets fulfil QoS requirements with [95%] confidence.

The metrics of QoS requirements depend on sensing tasks. For detection and localization purposes, detectability and sensing accuracy are used for QoS requirement metrics. For better understanding, a simplified drop-based detection capacity simulation of 1-D target CA-CFAR detection is presented in Figure 3, with a sensing capacity about 4 per km.

| (a) Range profile of a single drop with 3 targets | (b) Probability of detecting all targets |
| --- | --- |

Figure 3. Result of a simplified drop-based detection capacity simulation

| (a) Definition of latency for tracking |  | (b) Probability of tracking all target |
| --- | --- | --- |

Figure 4. Result of a simplified tracking capacity simulation

For tracking purpose, requirements for accuracy of single observation don't have to be as strict as localization. A few miss-detections or less accurate localization results are tolerable in cases where these outages appear inconsecutively. A well-defined sensing capacity is more useful. Metric of tracking QoS requirement is defined as the maximum tolerable latency (with [95%] confidence) between two consecutive localization results with given accuracy. A simplified simulation result of multi-target tracking in range is presented in Figure 4, with a sensing capacity of 6 per km.

Evaluation methods for sensing capacity are suggested in Table B of the Appendix.

Proposal 11: Proposed to define QoS requirements for sensing capacity as follows.

- QoS requirement of detection purpose is defined by the detection probability with a fixed false alarm probability.

- QoS requirement of localization purpose is defined by the localization accuracy.

- QoS requirement of tracking purpose is defined as the maximum latency between two consecutive measurements with given localization accuracy.

AI-related capabilities

Considering the difficulty and complexity of defining AI-related capabilities as a quantitative TPR, we can accept to define AI-related capabilities as a qualitative TPR.

Proposal 12: Define AI-related capabilities as a qualitative TPR, RIT/SRIT should support mechanisms and/or signaling /related to AI functionalities over the radio interface (between UE and network), including AI data collection and other capabilities.

Considering the following definition for AI-related capabilities:

Applicable AI-related capabilities refer to the ability to provide certain functionalities throughput IMT-2030 to support AI enabled applications. The RIT should support AI-related capabilities, which refer to the ability for the RIT/SRIT to support [mechanisms and/or signalling/related to] AI functionalities over the radio interface (between UE and network). This should include support for one or more of the following: - AI data collection - Distributed data processing - Distributed learning - AI computing - AI model execution - AI model inference - Or other AI capabilities of the RITs/SRITs (to be reported by the proponent)

Conclusions

In this contribution,

Proposal 1: For the candidate IMT-2030 TPRs endorsed in RAN#106 plenary (except for energy efficiency), the definitions captured from IMT-2020 can be reused, as shown in the following Table 1.

Proposal 2: For the candidate IMT-2030 TPRs endorsed in RAN#106 plenary (except for energy efficiency), the associated target values in Table 2 can be considered as a starting point.

Proposal 3: Support sustainability/energy efficiency is a quantitative TPR (i.e. the evaluation method is simulation).

Proposal 4: For sustainability/energy efficiency, support the following definition (changes are marked in red):

- Energy Efficiency Energy efficiency is an important metric of sustainability. Network energy efficiency is the capability of a RIT/SRIT to support radio access network energy saving in relation to the traffic capacity provided. Device energy efficiency is the capability of the RIT/SRIT to support device modem power saving in relation to the traffic characteristics. This requirement is defined for the purpose of evaluation in the Immersive communication usage scenario. The requirement is the relative energy savings between the selected load case(s) and a reference case. For network energy efficiency, the reference case is when network is fully loaded. For UE energy efficiency, the reference case is when UE is fully-loaded/has full-buffer service. Proponents should report the power model used for the evaluation (e.g. BS power model in 3GPP TR 38.864 and UE power model in 3GPP TR 38.840), the number of UE under each BS and the traffic model. The relative target of certain metric (e.g. 10% performance loss compared with the benchmark) can be set. Constant that [90%] percent of UE fulfils the target should be achieved while evaluating the relative power saving for each load case. The certain metric can be selected according to the service type: - For Vo6G (i.e. VoNR-like) service, the certain metric is latency - For burst-buffer service, the certain metric is UPT - For XR service, the certain metric is latency - For LPWA service, there is no certain metric (i.e. no constraint) The minimum requirements for Energy efficiency are summarized in Tables below. Table: The minimum requirements for Network Energy efficiency [Test environment(s)] **; Selected load case(s); The reference case; Relative power saving (%). [Service type, Power model, Device number, Traffic model]; [empty load, low load, light load and/or medium load] *; Fully loaded IMT-2030 case *; [X1%, X2%, ...]. *empty load: L=0, low load: 0 < L≤15, light load: 15 < L≤30, medium load: 30 < L≤50, fully loaded case: L=100 ** needed only in case of simulation Table: The minimum requirements for Device Energy efficiency [Test environment(s)] **; Selected load case(s); The reference case; Relative power saving (%). [Service type, Power model, Device number, Traffic model]; [empty load, low load, light load and/or medium load] *; Device is fully-loaded/has full-buffer service; [A1%, A2%, …].

Proposal 5: Define joint requirement as a TPR for immersive communications services for IMT-2030.

- The joint requirement is defined as the total number of users fulfilling a specific ratio of satisfied users. A user is declared as a satisfied user if more than a certain percentage of packets are successfully transmitted within a required maximum latency.

- This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

Proposal 6: 	Propose to use detection probability with a fixed false alarm probability as a basic TPR to evaluate detectability of the system.

Proposal 7: 	Propose to use localization accuracy as a TPR with its definition being the three-dimensional distance between the measured and the actual location of the sensing target with [95%] confidence.

Proposal 8: 	Propose to use velocity accuracy as a TPR with its definition being the three-dimensional distance between the measured and the actual velocity of the sensing target with [95%] confidence.

Proposal 9: Propose to use sensing resolution as a TPR if and only if it is evaluated by nonanalytical approaches.

Proposal 10: Propose to use sensing capacity as a system-level TPR with its definition being the maximum number of the targets per unit area when all targets fulfil QoS requirements with [95%] confidence.

Proposal 11: Proposed to define QoS requirements for sensing capacity as follows.

- QoS requirement of detection purpose is defined by the detection probability with a fixed false alarm probability.

- QoS requirement of localization purpose is defined by the localization accuracy.

- QoS requirement of tracking purpose is defined as the maximum latency between two consecutive measurements with given localization accuracy.

Proposal 12: Define AI-related capabilities as a qualitative TPR, RIT/SRIT should support mechanisms and/or signaling /related to AI functionalities over the radio interface (between UE and network), including AI data collection and other capabilities.

References

- RP-243202, LS on Minimum requirements related to technical performance for IMT-2030 radio interface(s), RAN#106, Madrid, Spain, December 9-12, 2024.

- RP-243327, New SID: Study on 6G scenarios/use cases and requirements, RAN#106, Madrid, Spain, December 9-12, 2024.

- ITU-R Recommendation M.2160-0 (11/2023), “Framework and overall objectives of the future development of IMT for 2030 and beyond”.

- “Working document towards a Preliminary Draft New Report ITU-R M.[IMT-2030.TECH PERF REQ]: Minimum requirements related to technical performance for IMT-2030 radio interface(s)”, Annex 5.7 to 5D/413 Report on the 47th meeting of Working Party 5D.

- RP-243325, Draft Reply to ITUR_WP5D_TEMP_167 = RP-243202 on Minimum requirements related to technical performance for IMT-2030 radio interface(s), RAN#106, Madrid, Spain, December 9-12, 2024.

- Report ITU-R M.2410, Minimum requirements related to technical performance for IMT-2020 radio interface(s).

- TR 37.910, Study on self evaluation towards IMT-2020 submission.

- TR 38.838, Study on XR (Extended Reality) Evaluations for NR.

- “Working document towards a Preliminary Draft New Report ITU-R M.[IMT-2030.TECH PERF REQ]: Minimum requirements related to technical performance for IMT-2030 radio interface(s)”, Annex 5.7 to 5D/413 Report on the 48th meeting of Working Party 5D.

- 3GPP TR 38.864, Study on network energy savings for NR.

- 3GPP TR 38.840, Study on User Equipment (UE) power saving in NR.

Appendix

Table A-1: Evaluation assumptions for spectral efficiency

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | 6GHz |
| Bandwidth | 100MHz |
| Subcarrier Spacing | 30kHz |
| TDD | DDDDDDDSUU; S(6DL:4GP:4UL) |
| Channel Model | UMa in TR 38.901 |
| Scheduling algorithm | Proportional fair scheduling |
| ISD | 200m |
| BS Tx power | 53dBm |
| UE Tx power | 23dBm |
| BS Antenna Configuration | (Mg, Ng, M, N, P, Mp, Np) = (1, 1, 8, 8, 2, 4, 8) (Mg, Ng, M, N, P, Mp, Np) = (1, 1, 8, 16, 2, 4, 8) (Mg, Ng, M, N, P, Mp, Np) = (1, 1, 16, 16, 2, 4, 8) (Mg, Ng, M, N, P, Mp, Np) = (1, 1, 16, 16, 2, 4, 16) (Mg, Ng, M, N, P, Mp, Np) = (1, 1, 16, 32, 2, 4, 16) (dH, dV) = (0.7, 0.5)λ |
| BS array orientation | azimuth 0 degree; mechanic downtilt: 0 degree; electrical downtilt: 9 degree |
| UE Configuration | (Mg, Ng, M, N, P, Mp, Np) = (1, 1, 2, 2, 2, 2, 1); (dV, dH) = (0.5, 0.5)λ. |
| BS antenna pattern | θ3dB = 65°, φ3dB = 900 with 6 dBi element gain |
| UE antenna pattern | Omni-directional with 0 dBi element gain |
| BS antenna height | 25m |
| UE antenna height | Same as 3D-UMa in TR36.873 |
| Noise figure for BS | 4dB |
| UE receiver noise figure | 7dB |
| UE distribution | 20% Outdoor in cars: 30km/h, 80% Indoor in houses: 3km/h 10 users per TRP; 16 users per TRP |
| Traffic Model | Full buffer |

Table A-2: Evaluation results for spectral efficiency under the scenario of 10 users per TRP

| Number of TXRUs | [Mg, Ng, M, N, P] | 5th percentile user spectral efficiency (bit/s/Hz) | Average spectral efficiency (bit/s/Hz) |
| --- | --- | --- | --- |
| 64 TXRU (Mp, Np) = [4, 8] | [1, 1, 8, 8, 2] (128) | 0.44 | 13.93 |
|  | [1, 1, 8, 16, 2] (256) | 0.57 | 15.48 |
|  | [1, 1, 16, 16, 2] (512) | 0.69 | 16.68 |
| 128 TXRU (Mp, Np) = [4, 16] | [1, 1, 16, 16, 2] (512) | 0.82 | 19.15 |
|  | [1, 1, 16, 32, 2] (1024) | 0.88 | 19.65 |

Table A-3: Evaluation results for spectral efficiency under the scenario of 16 users per TRP

| Number of TXRUs | [Mg, Ng, M, N, P] | 5th percentile user spectral efficiency (bit/s/Hz) | Average spectral efficiency (bit/s/Hz) |
| --- | --- | --- | --- |
| 64 TXRU (Mp, Np) = [4, 8] | [1, 1, 8, 8, 2] (128) | 0.26 | 15.37 |
|  | [1, 1, 8, 16, 2] (256) | 0.34 | 18.21 |
|  | [1, 1, 16, 16, 2] (512) | 0.42 | 20.19 |
| 128 TXRU (Mp, Np) = [4, 16] | [1, 1, 16, 16, 2] (512) | 0.59 | 24.67 |
|  | [1, 1, 16, 32, 2] (1024) | 0.69 | 27.27 |

Table B-1: Evaluation method of sensing capacity for detection and localization purpose

|  | Detail |
| --- | --- |
| Step 1 | Perform system-level simulation and initialize evaluation parameters with N targets dropped. |
| Step 2 | Generate sensing transmitted-signal, obtain sensing received-signal, and perform sensing signal processing to get localization estimation error Δr / detection probability for each sensing target. |
| Step 3 | Update N, which is the number of dropped targets, until at least [95%] of the N’ total targets fulfil localization accuracy / detectability requirement. |
| Step 4 | Calculate the sensing capacity as C = N’/A, where A is the TRxP area |

Table B-2: Evaluation method of sensing capacity for tracking purpose

|  | Detail |  |
| --- | --- | --- |
| Step 1 | Perform system-level simulation and initialize evaluation parameters with N targets dropped. |  |
| Step 2 | Generate sensing transmitted-signal, obtain sensing received-signal, and perform sensing signal processing and tracking processing to get localization estimation error Δr at sampling time t for each sensing target. |  |
| Step 3 | Remove those estimations which do not fulfil localization accuracy requirement. Then obtain maximum latency, which is the time interval between two consecutive estimations, of each sensing target. |  |
| Step 4 | Option-A | Update N, which is the number of dropped targets, until at least [95%] of the cases with N' targets dropped satisfy that all N' targets fulfil maximum latency requirement. |
|  | Option-B | Update N, which is the number of dropped targets, until at least [95%] of the N’ total targets fulfil maximum latency requirement. |
| Step 5 | Calculate the sensing capacity as C = N’/A, where A is the TRxP area. |  |