3GPP TSG RAN Meeting #107							      	       RP-250337

Incheon, Korea, March 12-14, 2025

Source 	: CAICT

Title 	: Proposals for AI/ML Air Interface in 5G-A R20

Agenda Item	: 15.2.1

Document for	: Discussion / Decision

- Introduction

According to the conclusion of last plenary, Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface are proposed by lots of companies to continue in 5G-A R20 [1]. This paper provides some discussions on the justifications and objectives of AI/ML for NR air interface in R20.

- Discussions

2.1 Justifications

The application of AI/ML techniques to NR air interface has been further studied in FS_NR_AIML_air_Ph2, including further study on CSI compression (two-sided model) covering aspects on improved trade-off between performance and complexity/overhead and alleviating/resolving issues related to inter-vendor training collaborations.

During the study, it was observed that more than 20% performance gains can be achieved considering CSI compression in temporal domain and cell/site specific models. For inter-vendor training collaborations, multiple options and sub-options are studied

and it was concluded that specification of model structure and parameters in 3GPP can ensure the minimum performance, while exchanging model parameters/datasets can further improve the performance.

Proposal 1: Based on FS_NR_AIML_air_Ph2 study outcome, it is proposed to specify AIML based CSI compression in Rel-20.

  - Objectives

The following objectives could be considered for the specifications for AI/ML based CSI compression in R20.

- Necessary signalling/mechanism(s) to facilitate model training, inference, performance monitoring, data collection for AI/ML based CSI compression (RAN1/2)

- Signalling/protocol enhancement(s) for Life Cycle Management (LCM) to support two-sided model (RAN2/1)

- Necessary signalling/mechanism to support the option recommended by R19 study (if any) to alleviate inter-vendor collaboration

- Testability and interoperability (RAN4)

  - Define the necessary reference encoder and decoder for CSI reporting performance testing.

  - Specify necessary test cases and performance requirements for LCM procedure, including performance monitoring.

- Conclusion

In summary, the following proposal is provided:

Proposal 1: Based on FS_NR_AIML_air_Ph2 study outcome, it is proposed to specify AIML based CSI compression in Rel-20.

Reference

[1] RP-243292 RAN Chair's summary of 5G Advanced in Rel-20