3GPP TSG RAN Meeting #107							RP-250342

Incheon, Korea, March 12-14, 2025

## Status Report to TSG

Agenda item:			9.3.1.5

| WI / SI Name | Enhancements of network energy savings for NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | Netw_Energy_NR_enh |  |  |  |  |
| Unique ID | 1020095 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-242354 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: n/a | Core part: 09/2025 | Performance part: 03/2026 | Testing part: n/a |  |
| Overall Completion level | Study Item: n/a | Core part: 70% | Performance Part: 0% | Testing part: n/a |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur (primary) | Name | Ajit Nimbalker |
|  | Company | Ericsson |
|  | Email | Ajit.Nimbalker@ericsson.com |
| Rapporteur (RAN2) | Name | Peng Cheng |
|  | Company | Apple |
|  | Email | pcheng24@apple.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

In RAN1#120, the following agreements were made.

Agreement

Regarding the relation in terms of time location between the always-on SSB and on-demand SSB, at least for the case when the center frequency locations of always-on SSB and on-demand SSB are same, down-select one of the following.

- Alt Time-A: It is not allowed that time location of an SSB within on-demand SSB burst overlaps with time location of any SSB within always-on SSB burst.

  - NOTE: It is assumed that the periodicity of always-on SSB is equal to or larger than that of on-demand SSB.

- Alt Time-B: Time-domain locations of always-on SSB are a subset of time-domain locations of on-demand SSB.

  - NOTE: It is assumed that the periodicity of always-on SSB is equal to or larger than that of on-demand SSB.

- Alt Time-C: The specification has no restriction with regards to overlapping

Agreement

- For a cell supporting on-demand SSB SCell operation, to provide time domain location of on-demand SSB by RRC,

  - For Case #2, adopt Alt A in the previous RAN1 agreement, i.e.,

    - Based on two parameters, where one is to indicate SFN offset from a reference point and the other is to indicate half frame index

      - The reference point is SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0

      - If SFN offset parameter is NOT configured, UE assumes SFN offset set to 0.

      - If half frame index parameter is NOT configured, UE assumes half frame index set to 0.

      - The value range of SFN offset is 0 to 15 unless longer periodicity for on-demand SSB than 160 ms is introduced.

      - The value range of half frame index is 0 or 1.

    - FFS: Whether/how to indicate time offset between always-on SSB and on-demand SSB (e.g., similar to ssb-TimeOffset)

      - By defining reference point as SFN where AO-SSB burst exists

Agreement

- For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameter (i.e., od-ssb-config) for on-demand SSB configuration, for both Case #1 and Case #2.

- For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameters to configure each of the followings, for both Case #1 and Case #2.

  - Frequency of the on-demand SSB (i.e., od-ssb-absoluteFrequency)

    - FFS: Whether this parameter can be absent for Case #2

  - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst)

    - FFS: Whether this parameter can be absent for Case #2

  - Periodicity of the on-demand SSB (i.e., od-ssb-Periodicity)

  - Sub-carrier spacing of the on-demand SSB (i.e., od-ssbSubcarrierSpacing)

    - For Case #2, this parameter is absent and sub-carrier spagcing of on-demand SSB is the same as that of always-on SSB.

  - Physical Cell ID of the on-demand SSB (i.e., od-ssb-physCellId)

    - For Case #2, this parameter is absent and physical cell ID of on-demand SSB is the same as that of always-on SSB.

  - Time location of on-demand SSB burst (i.e., od-ssb-sfn-Offset and od-ssb-halfFrameIndex)

  - Downlink transmit power of on-demand SSB (i.e., od-ss-PBCH-BlockPower)

    - For Case #2, this parameter is absent and downlink transmit power of on-demand SSB is the same as that of always-on SSB.

  - Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofTx)

  - FFS: Which of the above parameters are a list

Agreement

For RRC based OD-SSB indication, UE expects on-demand SSB is transmitted from the first on-demand SSB burst after receiving RRC carrying indication of OD-SSB transmission

Conclusion

The following combination of scenarios and cases for indicating OD-SSB are not supported in Rel-19

- Scenario #3A and Case #1

- Scenario #3A and Case #2

Above does not impact discussion on SSB periodicity adaptation in time domain

Agreement

Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB,

- Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB.

  - On-demand SSB is not on sync raster

  - AO-SSB and OD-SSB are located in the same BWP

  - FFS: Additional conditions

  - Subject to separate UE capability

  - Note: UE is not required to measure both AO-SSB and OD-SSB

Agreement

Regarding the relation in terms of time location between the always-on SSB and on-demand SSB,

- For the case when the center frequency locations of always-on SSB and on-demand SSB are same,

  - Alt Time-C: RAN1 specification has no restriction with regards to overlapping

  - From RAN1 perspective,

    - Alt Time-C1: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern is supported (the interval between SSB bursts is even and supported in legacy specification).

    - Alt Time-C2: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern is supported.

  - It is up to RAN4 to define requirements, if any, corresponding to both or either of Alt Time-C1 or Alt Time-C2

  - At least the following is supported: PBCH payload for the same SSB index (other than SFN index, half frame index) is the same for AO-SSB and OD-SSB

    - FFS: Whether half frame index is the same or different for AO-SSB and OD-SSB

- For the case when the center frequency locations of always-on SSB and on-demand SSB are different,

  - Alt Time-C: RAN1 specification has no restriction with regards to overlapping

  - UE assumes that frequency resources of always-on SSB are not overlapped with those of on-demand SSB in frequency domain.

  - AO-SSB and OD-SSB are located in the same BWP

  - FFS: PBCH payload for the same SSB index (other than SFN index, half frame index) should be the same for AO-SSB and OD-SSB

- NOTE: AO-SSB periodicity is not adapted

Send an LS to RAN4 to inform them of the above agreement. Final LS in R1-2501633.

[Active] Proposal #6-3a (SCS of T):

For the case where SCell with on demand SSB transmission and cell with signalling transmission have different numerology, when UE determines time instance A, the SCS to determine the value of T is down selected among the following options

- Option 1: the SCS of the active DL BWP where UE receives MAC CE for on-demand SSB transmission indication

- Option 2: the minimum of “the SCS of the active DL BWP where UE receives MAC CE for on-demand SSB transmission indication” and “the SCS of the active DL BWP where UE receives on-demand SSB”

Option 3: the SCS of the active UL BWP where the UE transmits ACK corresponding to the MAC-CE for on-demand SSB transmission indication

Agreement

The reference time point to determine the window starting time for on-demand SIB1 is based on the starting slot of the RAR window for UL WUS wherein UE successfully received a RAR.

Agreement

The time offset between the reference time point and the starting time for the time window of on-demand SIB1 is indicated in the UL WUS configuration.

- Unit of the time offset is in slot

Conclusion

On how to use PDCCH-ConfigSIB1 for R19 NES-capable UE in MIB of NES Cell when K_SSB = 30 in FR1 or K_SSB = 14 in FR2 on NES cell if SSB of NES cell is on the sync raster, Option 3 is adopted.

- Option 1: Use it to indicate frequency assistance information to search SSB for Cell A

  - FFS: Details on the range/granularity of the frequency assistance information

  - FFS: Potential impact to RAN3

- Option 2: Use it to indicate searchSpaceZero and controlResourceSetZero of OD-SIB1

- Option 3: Above feature is not supported.

Agreement

For type 0 PDCCH monitoring occasions of on demand SIB1, searchSpaceZero and controlResourceSetZero for on-demand SIB1 are provided from UL WUS configuration if SSB on NES cell is on sync raster.

Agreement

The UE assumes that, in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in PDCCH monitoring occasions corresponding to at least the SSB associated with the PRACH for UL-WUS if this is indicated via UL WUS configuration

- FFS: Whether UE expect PDCCH on PDCCH monitoring occasion corresponding to SSB other than the one mentioned above

- FFS: Whether gNB can indicate (in UL-WUS configuration) the SSB the UE can expect PDCCH transmission on

Conclusion

There is no RAN1 consensus on the following proposal:

From RAN1 perspective, OD-SIB1 design does not differentiate depending on frequency location of SSB on NES cell

Conclusion

It’s up to RAN2 to discuss whether the value in OD-SIB1 IE can be different from the value of the corresponding IE in the WUS configuration, and how to define the UE behavior if the values are different.

Conclusion

- The parameter “ssb-PeriodicityServingCell” is not included in the UL-WUS configuration for FDD system.

- RA-RNTI is not included in the UL-WUS configuration for FDD system.

- Cell barring information is not included in the UL-WUS configuration for FDD system.

- Cell selection information is not included in the UL-WUS configuration for FDD system.

- Accessible UE types is not included in the UL-WUS configuration for FDD system.

- CarrierBandwidth (for MPR calculation) is not included in the UL-WUS configuration for FDD system.

- InitialUplinkBWP is not included in the UL-WUS configuration for FDD system.

- TotalNumberOfRA-Preambles is not included in the UL-WUS configuration for FDD system.

- SIB1-RequestResourceRedCap is not included in the UL-WUS configuration for FDD system.

- SupplementaryUL is not included in the UL-WUS configuration

Conclusion

There is no RAN1 consensus to support the following in Rel-19:

Support joint PRACH request to trigger both OD-SIB1 and a subset of OD-OSI, e.g. SIB2/SIB3/SIB4.

- FFS: parameters in UL-WUS config to support joint PRACH request.

FL Proposal 6-1-9 (from vivo2)

If a UE has SIB1 request configuration of a cell and before transmitting UL WUS,

- If the UE detects a SSB where K_SSB>=24 for FR1 or K_SSB>=13 for FR2, down select from the following:

  - Alt. 1: Only if K_SSB=30 for FR1 and K_SSB=14 for FR2, UE monitors Type 0 PDCCH for SIB1 transmission; Otherwise, UE doesn’t monitor Type 0 PDCCH

    - FFS: Whether monitoring time window of Type 0 PDCCH is up to UE implementation or specified.

  - Alt. 2: UE monitors Type 0 PDCCH for SIB1 transmission

    - FFS: Whether monitoring time window of Type 0 PDCCH is up to UE implementation or specified.

  - Alt. 3: It is up to UE implementation on whether to monitor Type 0 PDCCH for SIB1 transmission

  - Alt. 4: UE doesn’t monitor Type 0 PDCCH for SIB1 transmission.

  - Alt. 5: UE monitors Type 0 PDCCH for SIB1 based on some repurposed parameters in MIB or PBCH

Note: If the UE detects a SSB where K_SSB<=23 for FR1 or K_SSB<=12 for FR2, UE monitors Type 0 PDCCH for SIB1 transmission as legacy.

Note: From Ericsson point view, Alt 3 and Alt 4 may be inconsistent with existing RAN2 agreements

Note: The above cases are for SSB on the sync raster.

Agreement

For adaptation of PRACH in time-domain, for determining the additional PRACH resources in time-domain,

- When an additional RO is overlapped with legacy valid RO in both time and frequency domain, the additional RO is invalid before the SSB-RO mapping

  - Note: the overlapped RO for legacy resource is not impacted

  - FFS: Clarification on configuration of legacy ROs

Conclusion

There is no RAN1 consensus to support the following in Rel-19

- New SSB burst periodicity values other than the legacy values (i.e., 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms).

- New UE trigger

- Adapting the transmitted number of SSBs within a SSB burst

- Adaptation of the time domain positions of SSBs within a burst

Conclusion

There is no RAN1 consensus to support time domain adaptation for CD-SSB in Rel-19 for SCell

Agreement

For adaptation of SSB in time-domain, support the following to adapt SSB burst periodicity for an SCell

- UE is configured with SSB burst periodicity using legacy signalling for the SCell

- UE is configured with X additional SSB burst periodicity for the SCell

  - FFS: Value of X

- SSB occasions with larger periodicity are subset of the SSB occasions with shorter periodicity

  - FFS: Whether there is specification impact

  - Note: This does not impact the discussion on OD-SSB

- For switching the periodicity, down-select between

  - (MAC-CE)

    - MAC-CE details for SSB burst periodicity adaptation is up to RAN2.

  - (DCI)

    - Alt 1-3: DCI based signalling is used

Agreement

For adaption of PRACH in time-domain, for a connected mode UE, support a 1-bit field in DCI 1_0 with C-RNTI used to trigger PRACH (i.e. PDCCH order) to indicate whether the additional PRACH resource(s) is available for the triggered PRACH.

- FFS: UE behaviour (e.g. applicable resources for PRACH mask index) when it is indicated of additional PRACH resource(s)

- FFS: Details on how to reuse existing bit for the 1-bit indication

Agreement

For DCI-based adaptation for additional PRACH resources, DCI 1_0 with P-RNTI indicates the availability information for additional PRACH resource from a reference point and for a validity time duration

- FFS: Validity time duration for availability is configured by higher layer signaling or predefined

- FFS: Location of the reference point defined in the specification

- FFS: Value/granularity of the validity time duration.

- FFS: Whether DCI can be used to explicitly deactivate the additional PRACH resources

Agreement

For DCI-based adaptation for additional PRACH resources, support optional semi-static signalling of a single PRACH mask to identify the subset of the additional PRACH resources

- The mask is applicable at unit of

  - Alt 1: PRACH association period

  - Alt 2: PRACH association pattern period

  - Alt 3: SFN level

- The PRACH association period is determined based on valid additional ROs only.

- The mask is applied after valid RO determination and SSB-RO mapping.

- Note: The existing behaviour in TS 38.213 "An association pattern period includes one or more association periods and is determined so that a pattern between PRACH occasions and SS/PBCH block indexes repeats at most every 160 msec." is not impacted due to application of the mask.

- This is applicable at least for adaptation for DCI 1_0 with P-RNTI

- The DCI does not indicate PRACH mask selection

- FFS: how the mask is identified

  - Option 1: The PRACH mask is from a PRACH mask table

    - Pre-defined table with N=[4 or 8 or 16] rows

    - The semi-static signalling indicates a PRACH mask index

  - Option 2: The PRACH mask is based on configuration parameters e.g. bitmap at SFN-level, periodic time domain window, …

Agreement

- Separate configuration of Msg1-FDM for the additional PRACH resources at least for 4-step RACH is supported

  - UE is not expected to be configured such that there are more than 8 FDM-ed valid ROs (legacy + additional ROs)

  - FFS: When there is no configuration of Msg1-FDM

- Separate configuration of number of SSB per RO is supported

Agreement

Study the following options for the reference point (for the availability information of additional PRACH resources indicated by DCI 1_0 with P-RNTI in a PF) for RRC idle/inactive mode UE and RRC connected mode UE,

- Option 1: SFN of the first PF from the next I-DRX cycle

- Option 2: SFN of the first PF from the current I-DRX cycle

- Option 3: From the first frame of the first PRACH association period after UE receives the DCI

- Option 4: From the first frame of the current SI modification period

- Option 5: From the first frame of the next SI modification period

Agreement

For adaptation of SSB in time-domain, for adapting SSB burst periodicity for an SCell

- Support group common DCI signalling for switching the SSB burst periodicity using DCI format 2_9 with [cellDTRX-RNTI]

- FFS: which scenario(s) is this applicable for (e.g. as defined in 9.5.1)

Note: Above does not prevent RAN2 from designing a MAC CE based on OD-SSB feature and also used for SSB burst adaptation

#### 2.1.2	Remaining Open issues

- Details of specifying procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA.

- Details of specifying support for on-demand SIB1 for UEs in idle/inactive mode.

- Details of specifying adaptation of common signal/channel transmissions.

## 2.2	RAN2

#### 2.2.1	Agreements

In RAN2#129, the following agreements were made.

Agreements on OD-SSB

- RAN2 leave it to RAN4 to conclude whether always-on SSB and/or OD-SSB are measured when both are transmitted in OD-SSB case 2.

Agreements on OD-SIB1

- There is no need for additional barring mechanisms (in addition to the k_ssb signaling “no SIB1” indication in MIB) to handle legacy to be able to bar cell using OD-SIB1.

- Specify the following UE behavior to allow the UEs in RRC_CONNECTED state to acquire OD-SIB1 when T311 is running:

- When T311 is running, the UE can trigger the OD-SIB1 acquisition procedure with stored UL WUS configuration in SIB-X, if it is still valid.

- The legacy cell selection criteria are reused as the trigger condition of OD-SIB1 acquisition.

- The OD-SIB1 acquisition behavior is same as that of RRC_IDLE/IANCTIV UEs.

- The UE follows the legacy validity principle of stored SIB.

- For Rel-19 NES UE in RRC_CONNECTED, rely on the NW dedicated RRC for SIB1 delivery if searchSpaceSIB1 is not configured. It is legacy UE behavior and no spec change is expected.

- SIB-x can be cell specific or area specific, as legacy.

- Upon reception of RAR, the UE monitors OD-SIB1 in the window agreed by RAN1.

Agreements on paging adaptation

- For N, values smaller than T/32 are not supported.

- The maximum possible value for Ns is 8.

- Introduce a separate PEI configuration.

- Paging adaptations are configured semi-statically and updated via system information update notification.

- A new UE capability is added for R19 NES paging enhancement, and the new capability is included in UE-RadioPagingInfo. FFS on whether we have a common capability for all NES features.

Agreements on SSB adaptation

- RAN2 confirms SSB adaptation in time domain is not supported for RRC idle/inactive UEs and Rel-19 NES-capable UE’s PCell.

- RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation.

- For L3 measurement, RAN2 assumes the adapted SSB on neighbor cell is measured based on legacy SMTC.

Agreements on RACH adaptation

- Legacy UEs and UEs non-capable of time domain PRACH adaptation are expected to use legacy PRACH resources per legacy configuration and procedures. No barring is needed.

2a.	RAN2 starts CBRA for RACH adaptation.

2b. 	RAN2 starts 4-step RACH adaptation.

3. 	From R2 perspective, not apply time-domain RACH adaptation to RACH resources for MSG1-based SI request.

- From R2 perspective, not apply time-domain RACH adaptation to IAB RACH resources.

5a. 	From R2 perspective, RACH adaptation is not modelled as RA feature(s).

5b.	From R2 perspective, RACH partitioning with all the features, i.e. RedCap, SDT, and Slicing, and feature combinations, are supported for PRACH adaption in time domain.

- Will follow legacy mechanism regarding how to select RACH resource.

#### 2.2.2	Remaining Open issues

- RAN2 specification impacts to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA.

- Details of specifying support for on-demand SIB1 for UEs in idle/inactive mode.

- Details of specifying adaptation of common signal/channel transmissions.

## 2.3	RAN3

#### 2.3.1	Agreements

In RAN3#127, the following agreements were made.

It is agreed that after Cell A gNB has received and accepted the UL WUS Configuration request from the NES Cell gNB, it broadcasts in SIBx at the first opportunity.

It is agreed from the signalling point of view, in UL WUS Configuration Provision Request (naming to be further discussed) from NES Cell gNB to Cell A gNB, one option is “start with UL WUS Configuration”. This codepoint/choice is used to trigger Cell A to broadcast UL WUS configuration in SIBx.

It is agreed that NES Cell gNB can “stop” (naming to be further discussed) Cell A to broadcast UL WUS configuration.

TP R3-250912  Xn impact of On-demand SIB1 for UEs in idle/inactive mode is Agreed.

#### 2.3.2	Remaining Open issues

- Details of specifying procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA.

- Details of specifying support for on-demand SIB1 for UEs in idle/inactive mode.

- Details of specifying adaptation of common signal/channel transmissions.

## 2.4	RAN4

#### 2.4.1	Agreements

In RAN4#114, the following agreements were made.

OD-SSB based deactivated SCell L3 measurement procedure (Case 1)

Agreement:

- The time window is defined with Tuncertain plus actual measurement duration.

  - Tuncertain equals Tmin(RAN1) plus additional processing/preparation time, if it allows.

  - The actual measurement duration equals min(Treport , Tidentify) from the end of uncertain time.

    - Treport is the first OD-SSB based L3 measurement reporting

    - Tidentify equals the minimum RAN4 OD-SSB meas. period requirement as follow.

      - A detected cell: OD-SSB based measurement period

      - A newly detectable cell: OD-SSB based cell identification period

- No OD-SSB measurement requirement applies in fast window if NW deactivates the OD-SSB earlier than the end of min(Treport, Tidentify).

Agreement:

RAN4 to define measurement requirement based on T2.

- When NW re-activates the OD-SSB within the T2,

  - Option 1: UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement.

  - Option 2: UE is required to perform “fast measurement” without cell search.

    - The UE is assumed to skip the PSS/SSS detection and/or SSB index acquisition.

Option 3: UE is required to perform FMW after T2.

- When NW re-activates/re-configures the OD-SSB transmission after T2, UE performs the OD-SSB based fast L3 measurement within FMW

  - The UE is assumed to meet fast cell identification and measurement.

- The start point of T2 is based on

  - Option 1: the time point Tend

  - Option 2: the latter point of L3 measurement reporting for FMW and Tend

  - Option 3: the latter point of latest L3 measurement reporting before NW re-indicating OD-SSB transmission and Tend

  - Option 4: the time point of OD-SSB deactivation

  - Other options are not precluded

Where, Tend equals the time point of OD-SSB indication plus Tuncertain +Tidentify applies to option 1,2,3.

- The duration is

  - Option 1: 5*max(measCycleSCell,  DRX cycles) [– OD-SSB periodicity]

  - Option 2: 5* measCycle when measCycle =160 ms

  - Other options are not precluded

- The activation and deactivation of OD-SSB is fully up to NW.

OD-SSB based deactivated SCell L3 measurement requirement in FMW (Case 1)

Agreement:

When NW configures single OD-SSB carrier with multiple legacy SSB carriers, in FMW

- Option 1: The CSSF only counts OD-SSB layers

- Option 2: The CSSF of OD-SSB carrier counts OD-SSB layers and other layers

Agreement:

RAN4 to prioritize single OD-SSB carrier with multiple legacy SSB carriers in deactivated SCell measurement.

- FFS multiple OD-SSB carriers

OD-SSB based deactivated SCell L3 measurement (Case 2)

Agreement:

When NW indicates the OD-SSB transmission for deactivated SCell measurement(Case 2 Scenario 2),

- Option 1: The same approach in Case 1 can be reused.

  - UE performs the OD-SSB based fast L3 measurement following OD-SSB periodicity for deactivated SCell within a time window. After the time window, UE fallbacks to a slow mode measurement(with DRX and measCycleSCell).

- Option 2:

  - If the serving cell is unknown, the same approach in Case 1 can be reused.

  - If the serving cell is known, UE follows legacy requirements based on measCycleSCell.

    - The known condition reuses the principle of legacy known condition.

    - This scenario applies at least when UE sends the meas. reporting.

  - FFS the case if the L3 meas. report cannot be triggered.

- The above requirement is applied at least when the OD-SSB and AO-SSB are on the same frequency and spatial relation.

Agreement:

RAN4 to start from the following scenarios:

- Scenario 1: OD-SSB and always-on SSB are with same frequency, same offset but different periodicities.

- Scenario 4: OD-SSB and always-on SSB are with same frequency, different offset but same periodicities.

  - At least the offset between OD-SSB and AO-SSB equals the half of AO-SSB periodicity

- The scenarios can be updated based on other WGs’ progress.

Agreement:

- In scenario 4, the measurement requirement is defined based on the effective measurement periodicity.

  - The requirement is only applied in the FMW which includes both OD-SSB and AO-SSB, and

  - The effective measurement periodicity is half of OD-SSB periodicity

  - AO-SSB and OD-SSB are on the same frequency with the same spatial relation

OD-SSB general issues

Agreement:

- RAN4 not to support OD-SSB based L1 measurements for deactivated Scell.

Agreement:

- RAN4 to support OD-SSB L1 measurement during and after SCell activation.

  - When non-DRX is configured, the current requirement can be reused with the clarification of TSSB in TL1-RSRP_Measurement_Period_SSB is the SSB periodicity of OD-SSB

- RAN4 to support OD-SSB L1 measurement during SCell activation.

  - The requirement when DRX is configured.

    - Based on OD-SSB periodicity

Agreement:

- The legacy measurement accuracy is applied for OD-SSB based measurement.

OD-SSB SCell activation scenarios

Agreement:

- RAN4 to define OD-SSB based direct SCell activation at SCell addition requirements.

  - The details can be discussed in the CR directly.

  - It doesn’t impact the WI’s completion.

OD-SSB based Neighbour cell measurement

Scenario 1a: There is OD-SSB neighbor cell in the same MO with legacy serving cell

Agreement:

- For Scenario 1a: No RAN4 spec. change.

Scenario 1b: Neighbour cell with legacy SSB in the same MO with OD-SSB serving cell

Agreement:

- For FR1, follow legacy requirement for neighbour cell measurement with legacy SSB.

  - The agreement can be revisited if not aligned with the further progress in other WGs.

- Further discuss for FR2.

Scenario 1c: There is OD-SSB neighbor cell in the same MO with OD-SSB serving cell

Agreement:

- For FR1, follow legacy requirement for neighbour cell measurement.

  - The agreement can be revisited if not aligned with the further progress in other WGs.

- Further discuss for FR2.

SSB adaptation – Scenario

Agreement:

- Further clarification on the scope of RRM impact for SSB adaption:

  - No RRM impacts for SSB adaptation requirement for PCell:

  - Frequency location is same before and after SSB adaptation.

  - No change to spatial relation (in terms of QCL assumption) for the same SSB index before and after SSB adaptation

- Note: Above could be further updated based on further RAN1 progress if any.

Applicable L1 requirements for SSB adaptation

Agreement:

- No impact of SSB adaptation:

  - RLM

  - BFD

- Define requirements if SSB adaption happens:

  - L1-RSRP for serving cell

  - L1-SINR

  - CBD

- Whether and how to define requirements at transition is discussed in other issues.

Transition requirements applicability & Issue 1-2-3: Transition requirements – details

Agreement:

- For L1-RSRP/L1-SINR/CBD

  - The measurement/evaluation period based on SSB periodicity after adaptation apply from Tfirst_SSB.

  - Tfirst_SSB is the uncertainty time from the timepoint when SSB adaptation occurs till the first SSB after SSB adaptation.

L3 requirements for SSB adaptation – general

Agreement:

Deactivated SCell measurement:

- Option 1: For SSB adaption, reuse the same conclusion from OD-SSB case#2 (scenario#4) discussion to define RRM requirements.

  - At least for the case when the SSB periodicity is adapted to be shorter than before

- Option 2: Same as legacy

- Other options are not precluded.

The requirement can be updated based on further progress in other WG or other aspects are identified that the conclusion can not be directly reused.

SCell activation requirement for SSB adaptation

Agreement:

For SCell activation:

- Define the SCell activation requirements based on the SSB periodicity after SSB adaptation.

  - For the case SSB adaptation command comes together with SCell activation command.

The requirement can be updated based on further progress in other WG

TCI state switching requirements for SSB adaptation

Agreement:

- No requirements when SSB adaptation happens during TCI state switching.

RRM requirements impacts for OD-SIB1

Agreement:

RAN4 to discuss the potential RRM impact for OD-SIB1 from the next meeting.

#### 2.4.2	Remaining Open issues

- Details of specifying procedures and core requirement(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA.

- Details of specifying core requirement(s) of adaptation of common signal/channel transmissions.

- Details of specifying possible core requirement(s) for on-demand SIB1 for UEs in idle/inactive mode.

- Performance requirements and test cases for the network energy saving techniques.

## 2.5	RAN5

n/a

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

n/a

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

n/a

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

#### 4.1	List of RAN WG1 contributions

#### 4.1.1	RAN1#120

R1-2500053	Discussion of on-demand SSB Scell operation	FUTUREWEI

R1-2500054	Discussion of on-demand SIB1 for idle/inactive mode UEs	FUTUREWEI

R1-2500055	Discussion of the adaptation of common signal/channel transmissions	FUTUREWEI

R1-2500077	On-demand SSB SCell operation for eNES	Huawei, HiSilicon

R1-2500078	Discussion on on-demand SIB1 for eNES	Huawei, HiSilicon

R1-2500079	On common channel/signal adaptation for eNES	Huawei, HiSilicon

R1-2500128	Discussion on on-demand SSB for NES	ZTE Corporation, Sanechips

R1-2500129	Discussion on on-demand SIB1 for NES	ZTE Corporation, Sanechips

R1-2500130	Discussion on common signal channel for NES	ZTE Corporation, Sanechips

R1-2500173	Discussion on on-demand SSB SCell operation	Spreadtrum, UNISOC

R1-2500174	Discussion on on-demand SIB1 for idle/inactive mode UEs	Spreadtrum, UNISOC

R1-2500175	Discussion on adaptation of common signal/channel transmissions	Spreadtrum, UNISOC

R1-2500228	Discussion on on-demand SSB SCell operation	CATT

R1-2500229	Discussion on on-demand SIB1	CATT

R1-2500230	Discussion on adaptation of common signal/channel transmissions	CATT

R1-2500263	Discussion on on-demand SSB operation for SCell	China Telecom

R1-2500264	Discussion on on-demand SIB1 for idle/inactive mode UEs	China Telecom

R1-2500265	Discussion on common signal/channel adaptation	China Telecom

R1-2500291	Discussion on on-demand SSB SCell operation	CMCC

R1-2500292	Discussion on on-demand SIB1 for UEs in idle/inactive mode	CMCC

R1-2500293	Discussion on adaptation of common signal/channel transmissions	CMCC

R1-2500353	Discussion on on-demand SSB Scell operation	vivo

R1-2500354	Discussion on on-demand SIB1 for idle/inactive mode UEs	vivo

R1-2500355	Discussion on adaptation of common signal/channel transmissions	vivo

R1-2500398	On demand SIB1 for idle/inactive UEs	Tejas Networks Limited

R1-2500399	Adaptation of common signals/channels	Tejas Networks Limited

R1-2500400	On demand SSB for connected UEs	Tejas Networks Limited

R1-2500440	Discussion on the enhancement to support on demand SSB SCell operation	OPPO

R1-2500441	Discussion on the enhancement to support on demand SIB1 for idle/inactive mode UE	OPPO

R1-2500442	Discussion on adaptation of common signal/channel transmission	OPPO

R1-2500476	On-demand SSB SCell Operation	Nokia, Nokia Shanghai Bell

R1-2500477	On-demand SIB1 for Idle/Inactive mode UEs	Nokia, Nokia Shanghai Bell

R1-2500478	Adaptation of common signal/channel transmissions	Nokia, Nokia Shanghai Bell

R1-2500489	Discussion on on-demand SSB SCell operation	Panasonic

R1-2500490	Discussion on on-demand SIB1 for idle/inactive mode UEs	Panasonic

R1-2500525	Discussion on on-demand SSB SCell operation	InterDigital, Inc.

R1-2500526	Discussion on on-demand SIB1 for idle/inactive mode UEs	InterDigital, Inc.

R1-2500527	Discussion on adaptation of common signal/channel transmissions	InterDigital, Inc.

R1-2500552	On-demand SSB SCell Operation	Google

R1-2500553	On-demand SIB1 for Idle/Inactive Mode UE	Google

R1-2500554	Adaptation of Common Signals	Google

R1-2500621	Discussion on on-demand SSB for SCell operation	NEC

R1-2500622	Discussion on on-demand SIB1 for UEs in idle/inactive mode	NEC

R1-2500623	Discussion on adaptation of common signal/channel transmissions	NEC

R1-2500629	Discussion on on-demand SIB1 transmission for network energy savings	Fujitsu

R1-2500630	Discussion on adaptation of common signal / channel transmissions	Fujitsu

R1-2500654	On-demand SIB1 for idle/inactive mode UEs	Sony

R1-2500655	Adaptation of common signal/channel transmissions	Sony

R1-2500682	Discussion on On-demand SSB SCell operation	KT Corp.

R1-2500683	Discussion on Adaptation of common signal/channel transmissions	KT Corp.

R1-2500736	Discussion on on-demand SSB SCell operation	Xiaomi

R1-2500737	Discussion on on-demand SIB1 for idle/inactive mode UEs	Xiaomi

R1-2500738	Discussion on adaptation of common signal and channel transmissions	Xiaomi

R1-2500785	On-demand SSB SCell Operation	Apple

R1-2500786	On On-demand SIB1 for IDLE/INACTIVE mode UEs	Apple

R1-2500787	On adaptation of common signal/channel for NES enhancements	Apple

R1-2500854	On-demand SSB SCell operation	Samsung

R1-2500855	On-demand SIB1 for idle/inactive mode UEs	Samsung

R1-2500856	Adaptation of common signal/channel transmissions	Samsung

R1-2500884	On-demand SSB SCell operation	Lenovo

R1-2500885	On-demand SIB1 for idle/inactive mode UEs	Lenovo

R1-2500913	Discussion on On-demand SSB SCell operation	ETRI

R1-2500914	On-demand SIB1 for idle/inactive mode UEs	ETRI

R1-2500915	Adaptation of common signal/channel transmissions	ETRI

R1-2500940	Discussion on on-demand SSB SCell operation	Fujitsu

R1-2500946	Discussion on adaptation of common signal/channel transmission	Panasonic

R1-2500953	On-demand SSB SCell operation	LG Electronics

R1-2500954	On-demand SIB1 for idle/inactive mode UEs	LG Electronics

R1-2500955	Adaptation of common signal/channel transmissions	LG Electronics

R1-2500967	Discussion on On-Demand SSB SCell operation	Transsion Holdings

R1-2500968	Discussion on on-demand SIB1 transmission for idle/inactive mode UEs	Transsion Holdings

R1-2500969	Discussion on adaptive transmission of common signal or common channel	Transsion Holdings

R1-2501020	On-demand SSB SCell operation	MediaTek Inc.

R1-2501021	On-demand SIB1 for idle or inactive mode UEs	MediaTek Inc.

R1-2501022	Adaptation of common signal/channel transmissions	MediaTek Inc.

R1-2501049	Views on On-demand SIB1 for idle-inactive UEs	Vodafone

R1-2501053	Adaptation of common signals and channels	Lenovo

R1-2501089	Discussion on on-demand SIB1 for idle/inactive mode UEs	DENSO CORPORATION

R1-2501112	Discussion on adaptation of common signal channel transmissions	HONOR

R1-2501123	Discussion of On-demand SSB SCell operation	Mavenir

R1-2501133	Discussion on on-demand SIB1 transmission for idle UEs	Sharp

R1-2501134	Discussion on adaptation of common signal/channel transmissions	Sharp

R1-2501160	On-demand SSB operation for Scell	Qualcomm Incorporated

R1-2501161	On-demand SIB1 procedure	Qualcomm Incorporated

R1-2501162	Adaptation of common channel transmissions	Qualcomm Incorporated

R1-2501186	Discussion on remaining details of on-demand SSB operation on SCell	Sharp

R1-2501206	Discussion on on-demand SSB SCell operation	NTT DOCOMO, INC.

R1-2501207	Discussion on on-demand SIB1 for idle/inactive mode UEs	NTT DOCOMO, INC.

R1-2501208	Discussion on adaptation of common signal/channel transmissions	NTT DOCOMO, INC.

R1-2501233	Discussion on on-demand SSB SCell operation	ITRI

R1-2501238	Discussion on on-demand SSB Scell operation for NES	TCL

R1-2501241	DCI based signaling for on-demand SSB	ASUSTeK

R1-2501242	DCI based signaling for on-demand SSB	ASUSTeK

R1-2501248	List of RAN1 agreements for R19 NES WI until RAN1#119	Rapporteur(Ericsson)

R1-2501249	Initial list of RRC parameters for R19 NES WI	Rapporteur(Ericsson)

R1-2501251	On-demand SSB SCell operation	Ericsson

R1-2501252	On-demand SIB1 for UEs in idle/inactive mode for NES	Ericsson

R1-2501253	Adaptation of common signal/channel transmissions for NES	Ericsson

R1-2501263	Discussion on on-demand SIB1 in idle/inactive mode	CAICT

R1-2501277	Discussion on on-demand SSB Scell operation	CEWiT

R1-2501278	Discussion on  on-demand SIB1	CEWiT

R1-2501279	Discussion on adaptation of common signal and channel transmissions	CEWiT

R1-2501299	On-demand SIB1 for NES	Fraunhofer IIS, Fraunhofer HHI

R1-2501378	FL summary 1 for on-demand SIB1 in idle/inactive mode	Moderator (MediaTek)

R1-2501379	FL summary 2 for on-demand SIB1 in idle/inactive mode	Moderator (MediaTek)

R1-2501380	FL summary 3 for on-demand SIB1 in idle/inactive mode	Moderator (MediaTek)

R1-2501381	FL summary 4 for on-demand SIB1 in idle/inactive mode	Moderator (MediaTek)

R1-2501382	FL summary 5 for on-demand SIB1 in idle/inactive mode	Moderator (MediaTek)

R1-2501451	Summary#1 of AI 9.5.3 for R19 NES	Moderator (Ericsson)

R1-2501452	Summary#2 of AI 9.5.3 for R19 NES	Moderator (Ericsson)

R1-2501453	Summary#3 of AI 9.5.3 for R19 NES	Moderator (Ericsson)

R1-2501493	Summary #1 of on-demand SSB for NES	Moderator (LG Electronics)

R1-2501494	Summary #2 of on-demand SSB for NES	Moderator (LG Electronics)

R1-2501495	Summary #3 of on-demand SSB for NES	Moderator (LG Electronics)

R1-2501496	Summary #4 of on-demand SSB for NES	Moderator (LG Electronics)

R1-2501599	Summary#4 of AI 9.5.3 for R19 NES	Moderator (Ericsson)

R1-2501600	Final Summary of AI 9.5.3 for R19 NES	Moderator (Ericsson)

R1-2501632	Draft LS on time location of on-demand SSB for Scell	LG Electronics

R1-2501633	LS on time location of on-demand SSB for Scell	RAN1, LG Electronics

R1-2501635	Summary #5 of on-demand SSB for NES	Moderator (LG Electronics)

R1-2501647	Updated list of RAN1 agreements for Rel-19 NES WI	Rapporteur (Ericsson)

#### 4.2	List of RAN WG2 contributions

#### 4.2.1	RAN2#129

R2-2500014        Reply LS on SSB relation in On-demand SSB and SSB adaptation on Scell (R1-2410917; contact: Ericsson)

R2-2501245        Running RRC CR for enhancements for network energy efficiency	Ericsson	draftCR	Rel-19	38.331.

R2-2500075	Discussion on on-demand SSB	Xiaomi

R2-2500115	Discussion on On-Demand SSB	OPPO

R2-2500171	On-demand SSB SCell Operation	Samsung

R2-2500211	Discussion on OD-SSB	Rakuten Mobile, Inc

R2-2500225	Consideration on on-demand SSB SCell operation	CATT

R2-2500256	Discussion on on-demand SSB for Scell	Apple

R2-2500334	Remaining issues of on-demand SSB SCell operation	vivo

R2-2500398	Discussion on on-demand SSB SCell operation	Sharp

R2-2500502	Further discussion on On-demand SSB for SCell	NEC

R2-2500518	Discussion on on-demand SSB SCell operation	Fujitsu

R2-2500551	On demand SSB handling	Nokia, Nokia Shanghai Bell

R2-2500626	Issues on the procedure of on-demand SSB SCell operation	Lenovo

R2-2500646	Discussion on on-demand SSB configuration	HONOR

R2-2500670	On-demand SSB SCell operation	Panasonic

R2-2500691	Discussion on on-demand SSB SCell operation for NES	Huawei, HiSilicon

R2-2500731	Discussion on on-demand SSB for NES	Ericsson

R2-2500742	On-demand SSB Scell operation discussion	Sony

R2-2500791	On demand SSB transmission for SCell	InterDigital

R2-2500870	On-demand SSB SCell operation	LG Electronics Inc.

R2-2501019	Discussion on On-demand SSB for Scell	CMCC

R2-2501059	Discussion on on-demand SSB SCell operation	NTT DOCOMO, INC.

R2-2501086	Remaining issues of on demand SSB Scell operation	ZTE Corporation, Sanechips

R2-2501317	Discussion on On-demand SSB SCell Operation	Qualcomm Incorporated

R2-2500170	On-demand SIB1	Samsung

R2-2500190	Discussion on on-demand SIB1	Xiaomi

R2-2500226	Consideration on on-demand SIB1	CATT

R2-2500257	Remaining issues on on-demand SIB1	Apple

R2-2500280	Discussion on on-demand SIB1 for NES	Rakuten Mobile, Inc

R2-2500335	Discussion on OD-SIB1 for RRC IDLE and INACTIVE UE	vivo

R2-2500341	Discussion on on-demand SIB1 operation for NES	Huawei, HiSilicon

R2-2500396	On-demand SIB1 WUS provisioning and UE behaviour	NEC.

R2-2500449	Discussion on Ondemand-SIB1	KDDI Corporation

R2-2500463	Discussion on the support for OD-SIB1	Google

R2-2500519	Discussion on on-demand SIB1 procedure for NES	Fujitsu

R2-2500552	On demand SIB1 handling	Nokia, Nokia Shanghai Bell

R2-2500647	Discussion on on-demand SIB1	HONOR	discussion

R2-2500671	RAN Plenary outcome and other remaining issues	Lenovo

R2-2500729	Discussion on on-demand SIB1 for NES	Ericsson

R2-2500735	Consideration on on-demand SIB1	OPPO

R2-2500743	Remaining details for on-demand SIB1	Sony

R2-2500789	On-demand SIB1 request and reception	InterDigital

R2-2500871	On-demand transmission of SIB1	LG Electronics Inc.

R2-2500921	Views on On-demand SIB1 operation for idle/inactive UEs	Vodafone

R2-2501008	Discussion on handling RLF in a NES cell	ITRI

R2-2501020	Discussion on On-demand SIB1	CMCC

R2-2501061	Discussion on on-demand SIB1	NTT DOCOMO, INC.

R2-2501087	Remaining issues of on demand SIB1	ZTE Corporation, Sanechips

R2-2501157	Discussion on on-demand SIB1	Jio

R2-2501163	Discussion on On-demand SIB1	Qualcomm Incorporated

R2-2501192	Discussion on remaining details of on-demand SIB1	SHARP Corporation

R2-2501216	On-demand SIB1 for Idle/Inactive mode UEs	III

R2-2501270	Discussion on On-demand SIB1 for NES	Fraunhofer IIS, Fraunhofer HHI

R2-2500076	Discussion on common signal adaptation	Xiaomi

R2-2500149	Discussion on common signal and channel adaptation	LG Electronics Inc.

R2-2500172	Adaptation of common signal channel transmissions	Samsung

R2-2500199	Discussion on adaptation of common signal channel transmission	OPPO

R2-2500227	Adaptation of Common signal channel transmissions	CATT

R2-2500258	Discussion on common signal transmission adaptation	Apple

R2-2500336	Discussion on adaptation of common signal transmissions	vivo

R2-2500397	PRACH and paging adaptation for NES	NEC

R2-2500416	Discussion on PRACH adaptation	ASUSTeK

R2-2500475	Adaptation of common signal or channel	Fujitsu

R2-2500553	Adaptation of common signals	Nokia, Nokia Shanghai Bell

R2-2500627	Paging and PRACH adaptation	Lenovo

R2-2500692	Discussion on adaptation of common signals/channels transmissions	Huawei, HiSilicon

R2-2500744	RAN2 impacts on SSB and RACH adaptations	Sony

R2-2500790	Time domain adaptation of common signalling and channels	InterDigital

R2-2501021	Discussion on common signalling adaptation	   CMCC

R2-2501062	Discussion on adaptation of common signal and channel	NTT DOCOMO, INC.

R2-2501065	Adaptation of common signal/channel transmissions for NES	Ericsson

R2-2501088	Common signal/channel adaption	ZTE Corporation, Sanechips

R2-2501165	Discussion on Adaptation of Common Signal/Channel Transmissions	Qualcomm Incorporated

R2-2501231	Discussion on RACH adaptation 	SHARP Corporation

R2-2501465	Summary report of [AT129][105][NES] (Apple)	Apple

#### 4.3	List of RAN WG3 contributions

#### 4.3.1	RAN3#127

R3-250024	LS on energy saving indication from CN to RAN	SA2(Ericsson)

R3-250055	(BL CR to 38.423) Introduction of Network Energy Saving Enhancement	Ericsson, Huawei, CMCC, Samsung, Qualcomm, Nokia, ZTE, Lenovo, Deutsche Telekom, NEC

R3-250056	(BL CR to 38.473) Introduction of Network Energy Saving Enhancement	Huawei, Ericsson, CMCC, Samsung, ZTE, Nokia, Deutsche Telekom, Lenovo, NEC, Jio

R3-250104	Aspects of on-demand SIB1 for NES enhancements	Qualcomm Inc.

R3-250129	(TP to BLCR for TS 38.473) Discussion on on-demand SSB SCell operation	Huawei

R3-250130	(TP to BLCR for TS 38.473 and TS 38.423) Discussion on on-demand SIB1 for UEs in idle or inactive mode	Huawei

R3-250175	Support on-demand SSB operation	NEC

R3-250176	Support on-demand SIB1	NEC

R3-250202	On-demand SIB1 for UEs in idle/inactive mode	Fujitsu

R3-250289	Discussion on On-demand SSB for SCell	Lenovo

R3-250290	Discussion on On-demand SIB1 for Idle/Inactive UE	Lenovo

R3-250337	(TP to 38.473) On support on-demand SSB SCell operation	China Telecom

R3-250338	On support on-demand SIB1	China Telecom

R3-250360	(TP to BLCR for 38.300) Introduction of On-demand SIB1	CATT, Ericsson, Huawei, Samsung, China Telecomm, Rakuten

R3-250361	(TP to BLCR for 38.473) On support on-demand SSB SCell operation	CATT

R3-250362	(TP to BLCR for XnAP/F1AP) On support on-demand SIB1 for idle UE	CATT, Fujitsu

R3-250387	Reply LS on energy saving indication from CN to RAN	Ericsson Telecomunicazioni SpA

R3-250506	Further discussion on on-demand SSB Scell operation	ZTE Corporation

R3-250507	Further discussion on on-demand SIB1	ZTE Corporation

R3-250522	TP to BL CR for TS 38.473) Discussion on On-Demand SSB SCell Operation	CMCC

R3-250523	Support On-Demand SIB1 for UEs	CMCC

R3-250534	Xn and F1 signalling for On-Demand SIB1	Nokia

R3-250535	Energy saving indicator from AMF and adaptation of common signals/channels	Nokia

R3-250548	WI Work plan for R19 Network Energy Savings	Ericsson

R3-250549	Xn impact of On-demand SIB1 for UEs in idle/inactive mode	Ericsson, Deutsche Telekom, China Unicom

R3-250550	F1 impact of On-demand SIB1 for UEs in idle/inactive mode	Ericsson, Deutsche Telekom, China Unicom, Rakuten

R3-250551	On-demand SSB SCell operation and other Energy Saving topics	Ericsson

R3-250552	Adaptation of common signal/channel transmissions	Ericsson

R3-250586	Discussion on energy saving indication from CN to RAN	Huawei, CATT, NEC

R3-250587	[draft] Reply LS on energy saving indication from CN to RAN	Huawei, CATT, NEC

R3-250607	Discussion on on-demand SSB SCell operation in low-carbon green network	Samsung

R3-250608	Discussion on on-demand SIB1 in low-carbon green network	Samsung

R3-250609	Introduction of Network Energy Saving Enhancement	Samsung, Huawei, NEC, CATT, ZTE, Ericsson

R3-250696	Support for OD-SIB1 mechanism at Cell A and NES Cell	Rakuten Mobile, Inc

R3-250697	Discussion on OD-SSB based NES from neighbour gNB’s perspective	Rakuten Mobile, Inc

R3-250800	CB:#ES1	Huawei

R3-250801	CB:#ES2	Ericsson

R3-250812	Reply LS on energy saving indication from CN to RAN	RAN3(Huawei)

R3-250845	Xn impact of On-demand SIB1 for UEs in idle/inactive mode	Ericsson, Deutsche Telekom, China Unicom, CMCC, Lenovo, NEC, Samsung, CATT, Huawei, Rakuten

R3-250912	(TP for BL CR for TS 38.423) Xn impact of On-demand SIB1 for Ues in idle/inactive mode	Ericsson, Deutsche Telekom, China Unicom, CMCC, Lenovo, NEC, Samsung, CATT, Huawei, Rakuten, JIO, Qualcomm Inc, ZTE, Nokia

R3-250923	(BL CR to 38.423) Introduction of Network Energy Saving Enhancement	Ericsson, Huawei, CMCC, Samsung, Qualcomm, Nokia, ZTE, Lenovo, Deutsche Telekom, NEC

#### 4.4	List of RAN WG4 contributions

#### 4.4.1	RAN4#114

R4-2500056	Further considerations for on-demand SSB SCell operation	Qualcomm Technologies Ireland

R4-2500057	On RRM requirements for adaptation of common signal/channel transmission	Qualcomm Technologies Ireland

R4-2500388	Discussion on RRM requirements for OD-SSB for network energy saving	MediaTek inc.

R4-2500389	Discussion on RRM requirements for adaptation of common signal/channel transmission	MediaTek inc.

R4-2500456	Discussion on on-demand SSB SCell operation	Apple

R4-2500457	Discussion on adaptation of common signal/channel transmission	Apple

R4-2500465	RRM requirements for OD-SSB SCell operation	Nokia, Nokia Shanghai Bell

R4-2500537	Topic summary for [114][223] Netw_Energy_NR_enh_Part1	Moderator (Ericsson)

R4-2500538	Topic summary for [114][224] Netw_Energy_NR_enh_Part2	Moderator (Huawei)

R4-2500635	Discussion on OD-SSB requirements	Xiaomi

R4-2500636	Discussion on RRM requirements for adaptation of common signal/channel in NES	Xiaomi

R4-2500772	Discussion on RRM requirements of on-demand SSB Scell operation for Rel-19 NES	LG Electronics Inc.

R4-2500773	Discussion on RRM requirements of SSB adaptation for Rel-19 NES	LG Electronics Inc.

R4-2500848	Discussion on RRM requirement of OD-SSB for NES enhancement	CMCC

R4-2500849	Discussion on RRM requirement of OD-SIB1 and signal adapatation for NES enhancement	CMCC

R4-2500907	Discussion on On-demand SSB SCell operation of R19 NES	ZTE Corporation, Sanechips

R4-2500908	Discussion on adaptation of common signal and channel of R19 NES	ZTE Corporation, Sanechips

R4-2501017	Discussion on RRM requirements for on-demand SSB SCell operation	CATT

R4-2501018	Discussion on RRM requirements for adaptation of common signalchannel transmission	CATT

R4-2501126	On RRM requirements of on-demand SSB SCell operation	OPPO

R4-2501127	On RRM requirements of SSB adaptation	OPPO

R4-2501146	Discussion on RRM requirements for On-demand SSB SCell operations	Intel Corporation

R4-2501220	Discussion on RRM impact for Rel-19 NES OD-SSB	Huawei, HiSilicon

R4-2501221	Discussion on RRM impact for Rel-19 NES on adaptation	Huawei, HiSilicon

R4-2501304	Discussion on OD-SSB in Rel-19 NES	Ericsson

R4-2501305	Discussion on RS and channel adaptation in Rel-19 NES	Ericsson

R4-2501335	Discussion on On-demand SSB SCell operation for enhancements of network energy savings for NR	China Telecom

R4-2501336	Discussion on adaptation of common signal/channel transmission for enhancements of network energy savings for NR	China Telecom

R4-2501600	Discussion on RRM requirement impacts from on-demand SIB1 in R19 NES	vivo

R4-2501601	Discussion on RRM requirements for on-demand SSB SCell operation in R19 NES	vivo

R4-2501602	Discussion on RRM requirements for common signal or channel adaptation in R19 NES	vivo

R4-2501676	General discussion on the NES common signal adaptation	Samsung

R4-2502240	RRM requirements for SSB adaptation	Nokia, Nokia Shanghai Bell

R4-2502578	Ad-hoc minutes for Netw_Energy_NR_enh WI	Ericsson

R4-2502598	WF on RRM requirements for Netw_Energy_NR_enh_Part1	Ericsson

R4-2502599	WF on RRM requirements for Netw_Energy_NR_enh_Part2	Huawei

R4-2502689	WF on RRM requirements for Netw_Energy_NR_enh_Part2	Huawei