3GPP TSG-RAN Meeting #107 		                                    		   RP-250344

Incheon, Korea, March 12th – 14th, 2025

Agenda Item:	9.3.1.3

Source:	InterDigital, Inc.

Title:	On the Scope of Rel-19 AI/ML Positioning

Document for:	Discussion and Decision

# Introduction

In this contribution, scope of Rel 19 AIML positioning for Q2 and Q3 for 2025 is discussed and we make a proposal to remove 2nd priority issues, namely Case 2a and 2b, from the WID.

# Progress in RAN WGs

As indicated in the WID, Case 2a and 2b are in the 2nd priority. In this contribution, we discuss why we support to remove Case 2a and 2b from the WID.

The following key issues are discussed in RAN1 for Rel-19;

- Case 1, details related to consistency between training phase and inference phase

- Case 1, details related performance monitoring in LCM (Life Cycle Management)

- Case 1, contents of assistance data provided by the LMF to the UE

The following agreement, related to the LCM procedure, was agreed in RAN2#127b [1]. It is clear from the agreement that RAN1 inputs related to details in the procedure are needed to make progress in RAN2 discussion. For example, RAN2 needs to know whether how the LMF can signal network side additional conditions to the UE.

Agreements: 1: The following procedures for LCM for UE sided model for AI positioning case 1 is the baseline: Step 1: LMF may request the UE to report the supported functionalities at the UE side by LPP request capabilities message. Step 2: UE sends LPP provide capabilities message to LMF with the supported functionalities at the UE side. Step 3: LMF sends the LPP provide assistance data message (which may contain network side additional condition). Step 4: UE reports the applicable functionality to the LMF by the LPP provide capabilities message. Step 5: The LMF requests the inferred location information using the LPP request location information message. Step 6: UE reports the inferred location using LPP provide location information message. 2: Whether the inference configuration is provided in step 3 or/and step 5 is FFS (to be revised based on RAN1 progress). 3: Whether network side additional condition is needed and what it contains is FFS (to be revised based on RAN1 progress). 4: FFS whether LMF controls the UE sending unsolicited LPP provide capabilities (i.e. whether step4 is sent reactively or proactively). FFS the signalling details. 5: RAN2 will decide whether AI positioning will be a new method after further details from RAN1 are received.

In RAN1#119, RAN1 agreed to study the following options and down-select one of the options for determination of consistency for Case 1 [2]. It should be noted that the aforementioned options for a study in RAN1 only focus on explicit or implicit indication for TRP location and it is not clear whether other network side additional conditions need to be signalled explicitly or implicitly to the UE.

Agreement For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA, other than info #7, can be provided from LMF to UE. For info #7, RAN1 study, if necessary, choose one alternative from the following: - Alternative 1. Info #7 is provided implicitly via associated ID. - Associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference. - Alternative 2. Info #7 can be provided either implicitly or explicitly by LMF. Note: no UE capability is introduced on whether info #7 is provided implicitly or explicitly, and the UE can request info #7 to be provided explicitly or implicitly. - If provided implicitly, associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference. - Alternative 3. Info #7 is not be provided from LMF to UE. - If info #7 is not provided, UE may assume info #7 is consistent between training and inference. - Alternative 4. Info #7 is provided explicitly from LMF to UE. 7; Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs).

In addition, RAN2 needs feedback from RAN1 related to details of inference configurations and when they, in the agreed procedure, can be provided to the UE. Finally, a definition of a “functionality” also needs to be discussed in RAN1 such that RAN2 can work on activation or deactivation mechanism for the function. Overall, RAN1 and RAN2 need a tight coordination to finalize the details of the LCM procedure and achieve successful completion of the work item. We make the following observation

Observation 1: For Case 1, RAN1 needs to discuss details related to determination of consistency, inference configurations and definition of a “functionality” and coordinate with RAN2 to finalize the details of LCM

In RAN1#116b, RAN1 agreed on the following options for entities which can perform performance monitoring [3]. In Option A, the UE performs performance monitoring. In Option B, the LMF performs performance monitoring. RAN1 is currently evaluating which options to support. Depending on the options, new signaling between UE and LMF may need to be introduced, impacting RAN2 work.

Agreement For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded.

For Case 1, for AIML based positioning, whether an associated ID will be used or not for indicating consistency in network side additional conditions will have an impact on signalling. In addition, the structure of assistance data will also depend on whether an associated ID is present or not. Discussions related to performance monitoring may also have an impact on signalling.

Observation 2 : RAN1 still needs to finalize details for performance monitoring, contents of assistance data and determination of consistency for Case 1

Based on the observations, we have a concern on starting discussion on Case 2a and Case 2b which may significantly increase the risk of finalizing details related to Case 1, Case 3a and Case 3b given the current workload in RAN1.

Finally, in RAN2#129 and RAN1#120, each working group received an LS from SA2 in [4] and [5], respectively where SA2 asked both RAN1 and RAN2 for the progress for Case 2b and future plan to complete Case 2b. As indicated in [6], RAN2 made the decision to drop Case 2a and Case 2b in Rel-19. Therefore, it is clear that the second priority topics (Case 2a/2b) will not be completed in Rel-19 irrespective of the progress in RAN1.

Observation 3: Given the current workload and no likelihood to complete details of LPP signalling to support Case 2a and Case 2b in both RAN1 and RAN2, completion of Case 2a and Case 2b during Release 19 is highly unlikely.

Therefore, based on the observations, we make the following proposal.

Proposal 1: Remove 2nd priority issues from the WID

# Conclusion

Observation 1: For Case 1, RAN1 needs to discuss details related to determination of consistency, inference configurations and definition of a “functionality” and coordinate with RAN2 to finalize the details of LCM

Observation 2 : RAN1 still needs to finalize details for performance monitoring, contents of assistance data and determination of consistency for Case 1

Observation 3: Given the current workload and no likelihood to complete details of LPP signalling to support Case 2a and Case 2b in both RAN1 and RAN2, completion of Case 2a and Case 2b during Release 19 is highly unlikely.

Proposal 1: Remove 2nd priority issues from the WID.

# Reference

[1] RAN2 chair notes, RAN2#127b, Hefei, China, Oct., 2024.

[2] RAN1 chair notes, RAN1#119, Orland, USA, Nov., 2024.

[3] RAN1 chair notes, RAN1#116b, Changsha, China, Apr., 2024.

[4] R2-2500062, “LS on LMF-based AI/ML Positioning for Case 2b (S2-2501133; contact: vivo), ” vivo, RAN2#129, Athens, Greece, Feb., 2025.

[5] R1-2500388, “LS on LMF-based AI/ML Positioning for Case 2b, ” vivo, RAN1#120, Athens, Greece, Feb., 2025.

[6] R2-2501505, “[DRAFT] Reply LS to SA2, ” vivo, RAN2#129, Athens, Greece, Feb., 2025.