3GPP TSG RAN Meeting #107	                                          RP- 250346

Incheon, Korea, March 12-14, 2025

Title: 	NES

Source: 	Tejas Networks

Agenda item:	15.2.7.2

Document for:	Discussion and decision

# Introduction

Network energy saving (NES) was introduced in 3GPP release 18. Release 18 of NES dealt with SSB-less SCell operation for inter-band CA for FR1 and co-located cells, enhancement on cell DTX/DRX mechanism and Spatial and Power domain adaptations. Whereas release 19 of NES dealt with OD-SSB on SCells for connected UE, OD-SIB1 for idle/inactive UEs and adaptation of common signals or channels. In this contribution we will be covering the potential enhancements that can be considered for further improving the NES gain.

# On-Demand SSB for Idle/Inactive UE Operation

In release 19, OD-SSB for SCELL is limited to the NCD-SSB not on synchronisation raster and adaptation of SSB is limited to SCELL. OD-SSB adaptation is agreed for very specific scenarios such as, scenario #2(SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321)) and scenario #2A (When UE receives SCell activation command (e.g., as defined in TS 38.321)). This will provide very less energy saving gain due to restriction on the scenario and the type of SSB. And majority of network energy consumption occur due to CD-SSB on the synchronisation raster. But CD-SSB adaptation on the synchronisation raster was not agreed either for PCELL/SCELL in release 19. Hence there is a scope for improved NES gain by adapting the CD-SSB on synchronisation raster.

OD-SSB for idle or inactive UEs will improve network energy saving. When there is no requirement of SSB by UE, base station will not transmit SSBs at all. When the UE want to do measurements or when UE want to connect to the NES CELL, UE can request the SSBs by sending a WUS to the base station, this WUS configuration can be provided by the anchor CELL. This will improve the NES gain, as base station will only send the SSBs only when there is a requirement from the UE. This can be deployed for co-located CELLS.

A base station can also transmit long periodicity SSBs, and when the UE require denser periodicity SSBs, UE can send a WUS to the base station requesting SSBs. Using long periodicity SSBs, standalone deployment of NES CELL is possible, as NES CELL can send its own WUS configuration to the UE.

Support on-demand SSB operation for UEs in idle/inactive mode.

  - Standalone NES CELL supporting long period always on SSB on top of OD-SSB.

  - NES CELL transmitting only OD-SSB with no always on SSB, deployed along with co-located anchor CELL.

# On-Demand SIB1 using standalone NES CELL

In the previous release 19 of 5G NR for NES, OD-SIB1 for case 2 is agreed. In case 2, UE obtains the UL WUS configuration from Cell A, UE transmits UL WUS to NES Cell, UE receives on-demand SIB1 from NES Cell. UE has to depend on the CELL-A to get the UL-WUS configuration and CELL-A has to coordinate with all other CELLS in the backhaul to get the updated UL-WUS configuration. Standalone deployment of OD-SIB1 CELL will reduce the backhaul communication between NES CELLS and CELL-A. Standalone case will also reduce the UE initial access latency. Hence standalone case for OD-SIB1 should be supported where a UE does not need to connect to CELL-A and get UL-WUS configuration for accessing the NES CELL. Standalone case can be deployed in the new frequency bands to avoid the legacy issue problems.

Support standalone deployment of on-demand SIB1 operation for UEs in idle/inactive mode.

  - NES CELL will transmit the WUS configuration to the UE

  - UE will transmit the UL-WUS to the NES and receive OD-SIB1 from the NES CELL.

# Conclusion

Based on the discussion above, we have the following proposals.

Support on-demand SSB operation for UEs in idle/inactive mode.

Standalone NES CELL supporting long period always-on SSB on top of OD-SSB.

NES CELL transmitting only OD-SSB with no always-on SSB, deployed along with co-located anchor CELL.

Support standalone deployment of on-demand SIB1 CELL, for UEs in idle/inactive mode.

  - NES CELL will transmit the WUS configuration to the UE.

  - UE will transmit the UL-WUS to the NES and receive OD-SIB1 from the NES CELL.

# References

1. RP-223540, New WID: Network energy savings for NR

2. RP-242354, Revised WID: Enhancements of network energy savings for NR