3GPP TSG RAN Meeting #107																 RP-250348

Incheon, Korea, March 12-14, 2025

Agenda Item:		15.2.2

Source:				InterDigital, Inc.

Title:						Scope of Rel-20 NR MIMO 5G-Advanced

Document for:		Discussion and Decision

# Background

3GPP has paved the way for deployment of advanced MIMO technology by providing related specifications and procedures for UE vendors, network equipment suppliers and operators. MIMO is the backbone of a cellular network and plays a key role for achieving high spectrum efficiency, coverage and reliability. Therefore, in each release, one or more of the above objectives for uplink or downlink transmission have been the focus of the developed specifications. In this contribution, we provide our views regarding the general scope of NR MIMO in Rel-20 5G-Advanced.

# Discussion

For Rel-20, MIMO standardization activities is divided into two separate tracks, namely, NR MIMO 5G-A and MIMO 6G. Therefore, due to limited TUs, the objectives for the former needs to be carefully scoped. As such, in our assessment of topics for Rel-20 NR MIMO 5G-A, the following aspects have been considered,

- Addressing market-urgent issues of 5G with limited specifications impacts,

- Emphasizing on UL-related enhancements,

- Having a manageable workload (i.e., study/evaluate/specify) given the limited allocated TU(s).

Observation 1:  For Rel-20, MIMO standardization activities is divided into two separate tracks, where there will be a very limited TU available for NR MIMO 5G-A.

Proposal 1: For Rel-20 NR MIMO 5G-A, market-urgent issues of 5G with limited specifications impact and UL-focused enhancements with manageable corresponding workloads are prioritized.

Table 1 shows a summary of potential topics for Rel-20 NR MIMO 5G-A that is compiled based on an initial consultation among companies. The topics are structured in three main categories of enhancements for UL capacity/coverage, DL CSI acquisition and Rel-19 left-over topics. In the following, we provide our reflection for each topic.

Table 1 - Summary of potential scope for Rel-20 NR MIMO 5G-A

| - UL capacity/coverage | (a) UL frequency-selective precoding using legacy NR UL codebooks | TPMI-based (closed-loop), mainly 4Tx/[8Tx] |
| --- | --- | --- |
|  |  | Precoder cycling (open-loop), mainly 2Tx |
|  | (b) DFT-S-OFDM 2 layers with LTE UL codebooks |  |
|  | (c) SRS capacity/coverage: cross-slot repetition (U and S slots), hopping for partial sounding |  |
|  | (d)Additional precoders for 2Tx full-coherent UL codebook with 8PSK/16PSK alphabet |  |
| (2) DL CSI acquisition | (a) Early SRS (targeting reciprocity, i.e. SRS AS), CSI (for legacy CSI content(s), e.g. with smaller overhead), TRS triggering for IDLE/INACTIVE to CONNECTED UE with MSG2/4 |  |
|  | (b) CSI -RS overhead reduction in frequency-domain (to 1/4 and 1/8 RE/RB/port) and possibly port-domain (targeting large # ports e.g. 128) |  |
|  | (c) Sub-band scalar IMR measurement report (targeting reciprocity-based operation) with larger dynamic range than the current 4-bit CQI |  |
| (3) Left-over FR2 | (a) beam switching based on Rel-19 UEI /ED beam reporting using gNB confirmation (i.e. NW-controlled), focusing on beam switching delay reduction |  |
|  | (b) Asymmetric DL sTRP/UL mTRP: PDCCH order PRACH beam indication, same Tx beam constraint across SRS occasions (per given SRS resource(s)) for beam sweeping (similar to CSI-RS) |  |
|  | (c) Dynamic QCL update via MAC CE for periodic TRS |  |

(1) UL capacity/coverage

1-a) UL frequency-selective precoding: UL frequency-selective precoding can be realized in a closed-loop (TPMI-based) or an open-loop (precoder cycling) mechanism. In our view, frequency-selective precoding will be beneficial for UL robustness. However, to manage the workload, we should only consider open-loop (precoder cycling) for Rel-20 5G-A which requires a very small specification effort. Closed-loop mechanisms could be discussed as part of 6G standardization.

1-b) DFT-S-OFDM 2 layers: Since we are in the very last stage NR 5G standardization, introduction of this feature does not seem to be reasonable. Before introduction of this enhancement, its benefits and the required specification support for peripheral procedures (e.g., impact of DMRS, PAPR, etc.) should be assessed. Given the limited available TU, DFT-S-OFDM-based MIMO should be discussed as part of 6G standardization.

1-c) SRS capacity/coverage: The proposed specification support for cross-slot repetition (U and S slots) and hopping for partial sounding seem relatively small enhancements. In our view, the expected benefits, especially for the hopping-related enhancement, seem marginal. However, given its limited scope, we would be open for further discussion.

1-d) Additional precoders for 2Tx full-coherent UL codebook: In general, we believe there are some benefits in enhancing the resolution of the codebook. However, for simplicity, our preference is to not to extend the TPMI indexing, and instead, re-interpret the current range of TPMI when a precoder from the new set of precoders are preferred.

Observation 2:  Since we are in the very last stage of NR 5G standardization, objective 1-b that involves introduction of DFT-S-OFDM-based MIMO does not seem to be reasonable.

Proposal 2: For Rel-20 NR MIMO 5G-A, consider support of objectives 1-a and 1-d, and further discuss objective 1-c.

- DL CSI acquisition

2-a) Early SRS/CSI-RS/TRS triggering for UE transitioning from IDLE/INACTIVE to CONNECTED mode: An early DL CSI acquisition is an effective way to enhance system operational efficiency. As such, instead of waiting for completion of transitioning to the connected mode and then receive an RRC configuration for CSI measurement and report, by support of this enhancement, a UE can enable a gNB to acquire DL CSI in an accelerated manner. We believe there are some benefits and expect a reasonable workload for support of this enhancement.

2-b) CSI-RS overhead reduction in frequency-domain and possibly port-domain: Considering recent Rel-19 CSI enhancements for support of up to 128 CSI ports, we think this is an important objective that could further improve the spectrum efficiency of NR 5G system. Since we are in the last phase of 5G standardization, it is not reasonable to revisit CSI-RS design, however for now, frequency-domain or port-domain down-sampling could be considered for use in NR 5G systems. We consider this objective a highly important enhancement with clear benefit for operation of NR 5G. Further, the required specification work is very limited.

2-c) Sub-band IMR measurement report (targeting reciprocity-based operation): We understand the motivation of the enhancement, however we believe that it requires more investigation and a mere increase in dynamic range may not be sufficient. Since we are in the very last stage of NR 5G standardization, our preference is to visit this enhancement as part of 6G studies.

Observation 3:  To support objective 2-c, more investigation and evaluation is needed, hence our preference is to visit this enhancement as part of 6G studies.

Proposal 3: For Rel-20 NR MIMO 5G-A, support objectives 2-a and 2-b.

- Left-over FR2 topics:

3-a) UEI/ED beam switching based on Rel-19 UEI/ED beam reporting: As this is currently under the discussion in Rel-19, our preference is to discuss and enable this enhancement as part of ongoing work in Rel-19. In case, a resolution cannot be achieved in Rel-19, given that there is no urgency for the issue, we believe that the objective could be discussed as part of 6G studies.

3-b) Asymmetric DL sTRP/UL mTRP: PDCCH order PRACH beam indication and SRS beam sweeping: We believe that there is a clear motivation for the suggested enhancement related to PRACH beam indication. However, we are not sure if SRS beam repetition would be the right mechanism to enable PDCCH order PRACH indication. In our view, an SRS beam sweeping without the constraint of maintaining a same beam over SRS occasions should be supported, such that gNB could determine the best UL beam, so that it could be indicated by a PDCCH order for PRACH transmission. We are open to support SRS beam sweep with/without the proposed beam constraint.

3-c) Dynamic QCL update for common signals (e.g. CSI-RS for tracking): In our view, this topic needs more discussion to examine its potential benefit and impact on other system aspects. Hence, the objective may not be a right fit in the limited scope of the Rel-20 NR MIMO 5G-A. This topic should be discussed as part of 6G studies.

Observation 4:  There does not seem to be sufficient urgency to support objective 3-a and 3-c, hence our preference is to visit these enhancements as part of 6G studies.

Proposal 4: For Rel-20 NR MIMO 5G-A, support objectives 3-b, without the constraint of maintaining a same beam over SRS occasions, such that gNB could determine the best UL beam for indication by a PDCCH order for PRACH transmission.

While none of the individual proposed topics requires a significant workload, its numerosity represents a considerable effort that should be avoided in Rel-20. Table 2, captures our recommendations for the scope of Rel-20 NR MIMO 5G-A.

Observation 5: None of the individual proposed topics requires a significant workload, however its numerosity represent a considerable effort that should be avoided.

Proposal 5: For Rel-20 NR MIMO 5G-A,

- Support objectives 2-b,

- Support most/all of objectives: 1-a (open-loop), 1-c, 1-d, 2-a and 3-b,

- If sufficient TU is allocated, support at most one of 2-c, 3-a and 3-c.

Table 2 - Summary of potential scope for Rel-20 NR MIMO 5G-A

| - UL cap./cov. | (a) UL frequency-selective precoding using legacy NR UL codebooks | TPMI-based (closed-loop), mainly 4Tx/[8Tx] | Not Support |
| --- | --- | --- | --- |
|  |  | Precoder cycling (open-loop), mainly 2Tx | Medium Priority |
|  | (b) DFT-S-OFDM 2 layers with LTE UL codebooks |  | Not Support |
|  | (c) SRS capacity/coverage: cross-slot repetition (U and S slots), hopping for partial sounding |  | Medium Priority |
|  | (d)Additional precoders for 2Tx full-coherent UL codebook with 8PSK/16PSK alphabet |  | Medium Priority |
| (2) DL CSI acquisition | (a) Early SRS (targeting reciprocity, i.e. SRS AS), CSI (for legacy CSI content(s), e.g. with smaller overhead), TRS triggering for IDLE/INACTIVE to CONNECTED UE with MSG2/4 |  | Medium Priority |
|  | (b) CSI -RS overhead reduction in frequency-domain (to 1/4 and 1/8 RE/RB/port) and possibly port-domain (targeting large # ports e.g. 128) |  | High Priority |
|  | (c) Sub-band scalar IMR measurement report (targeting reciprocity-based operation) with larger dynamic range than the current 4-bit CQI |  | Low Priority |
| (3) Left-over FR2 | (a) beam switching based on Rel-19 UEI /ED beam reporting using gNB confirmation (i.e. NW-controlled), focusing on beam switching delay reduction |  | Low Priority |
|  | (b) Asymmetric DL sTRP/UL mTRP: PDCCH order PRACH beam indication, same Tx beam constraint across SRS occasions (per given SRS resource(s)) for beam sweeping (similar to CSI-RS) |  | Medium Priority |
|  | (c) Dynamic QCL update via MAC CE for periodic TRS |  | Low Priority |

# Conclusion

In this contribution, as captured in Table 1, we discussed our views with respect to the scope of Rel-20 NR MIMO 5G-A. Based on the presented discussion, following observations and proposals are made,

Observation 1:  For Rel-20, MIMO standardization activities is divided into two separate tracks, where there will be a very limited TU available for NR MIMO 5G-A.

Proposal 1: For Rel-20 NR MIMO 5G-A, market-urgent issues of 5G with limited specifications impact and UL-focused enhancements with manageable corresponding workloads are prioritized.

Observation 2:  Since we are in the very last stage of NR 5G standardization, objective 1-b that involves introduction of DFT-S-OFDM-based MIMO does not seem to be reasonable.

Proposal 2: For Rel-20 NR MIMO 5G-A, support objectives 1-a and 1-d, and further discuss objective 1-c.

Observation 3:  To support objective 2-c, more investigation and evaluation is needed, hence our preference is to visit this enhancement as part of 6G studies.

Proposal 3: For Rel-20 NR MIMO 5G-A, support objectives 2-a and 2-b.

Observation 4:  There does not seem to be sufficient urgency to support objective 3-a and 3-c, hence our preference is to visit these enhancements as part of 6G studies.

Proposal 4: For Rel-20 NR MIMO 5G-A, support objectives 3-b, without the constraint of maintaining a same beam over SRS occasions, such that gNB could determine the best UL beam for indication by a PDCCH order for PRACH transmission.

Observation 5: None of the individual proposed topics requires a significant workload, however its numerosity represent a considerable effort that should be avoided.

Proposal 5: For Rel-20 NR MIMO 5G-A,

- Support objectives 2-b,

- Support most/all of objectives 1-a (open-loop), 1-c, 1-d, 2-a and 3-b,

- If sufficient TU is allocated, support at most one of 2-c, 3-a and 3-c.