3GPP TSG-RAN Meeting #107	RP-250391

Incheon, Republic of Korea, March 12th – 14th, 2025

Agenda Item:	15.2.5

Source:	Ericsson

Title:	Scope for Rel-20 IoT NTN

Document for:	Discussion

# 1	Introduction

The support of voice-over-GEO for NB-IoT NTN is currently under discussion and is being envisioned to be part of the Rel-20 evolution of “Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 4”. To our best knowledge, the following contributions were submitted to the previous RAN Plenary# 106 to discuss the potential support of voice-over-GEO in Rel-20 [1-9].

Table I: Overview of T-docs under agenda item 15.2 “Specific RAN1/2/3-led Rel-20 topics” discussing voice-over-GEO.

|  | Documents submitted to RAN Plenary # 106 discussing the potential support of voice-over-GEO for NB-IoT |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T-doc number | RP-242550 | RP-242627 | RP-242893 | RP-243161 | RP-242514 | RP-242664 | RP-242876 | RP-242968 | RP-243127 |
| Source | 1 Company | 8 Companies Co-sourcing | 1 Company | 2 Companies Co-sourcing | 1 Company | 8 Companies Co-sourcing | 1 Company | 1 Company | 1 Company |

In this contribution we provide our initial views on the support of voice-over-GEO for NB-IoT NTN in Rel-20, including aspects that in our view should be part of the scope of this potential Rel-20 objective.

# 2	Discussion

## 2.1 	Initial views on potential impacts

### 2.1.1	RAN impacts

One important aspect to consider is that the RAN technical specifications currently exclude support for real-time services such as voice and emergency services for terrestrial NB-IoT and, consequently, for IoT-NTN (See clause 4.1 in TS 36.331 [10]). Thus, supporting voice in the RAN will require some changes to the specifications including initial access and RRC connection procedures, as well as for scheduling. And RAN WGs should can also consider addressing potential overload situations.

For example, RRC connection establishment in NB-IoT does not include voice nor emergency as shown in Table II.

Table II: Comparison between the “establishment causes” for LTE and NB-IoT, the latter not including voice nor emergency [10].

| Establishment Cause for NB-IoT (Not including voice nor emergency) | Establishment Cause for LTE (Including voice and emergency) |
| --- | --- |
| -- ASN1START EstablishmentCause-NB-r13 ::= ENUMERATED { mt-Access, mo-Signalling, mo-Data, mo-ExceptionData, delayTolerantAccess-v1330, mt-EDT-v1610, spare2, spare1} -- ASN1STOP | EstablishmentCause ::= ENUMERATED { emergency, highPriorityAccess, mt-Access, mo-Signalling, mo-Data, delayTolerantAccess-v1020, mo-VoiceCall-v1280, spare1} |

Similarly, access control for LTE differentiates between establishment of RRC connection for emergency calls and other cases, whereas NB-IoT does not [10]:

RAN specifications currently exclude support for real-time services such as voice and emergency services for terrestrial NB-IoT and, consequently, for IoT-NTN.

Supporting voice in the RAN will require changes to the specifications in the access and connection procedures and for scheduling.

### 2.1.2	Transfer of user plane data

For NB-IoT, the technical specifications support transfer of user plane data in two different modes:

- Over CP, using NAS signalling between the UE and the MME (Control Plane CIoT EPS optimisation or DoNAS). Note that e.g., there are no RRC reconfigurations for DoNAS, thus the feasibility of using DoNAS for the support of voice-over-GEO should be studied.

- Over UP (User Plane CIoT EPS optimisation).

In our view, RAN in coordination with SA2 should study the feasibility and specification impact of supporting data over UP and data over CP. Yet, another aspect to consider is that the impact of supporting IMS voice on GEO IoT NTN products should be minimized. This would simplify the introduction of the feature.

RAN in coordination with SA2 should study the feasibility and specification impact of supporting data over UP and data over CP. It is desirable to limit the amount of specification work required to support IMS voice via NB-IoT.

### 2.1.3	PHY-layer aspects

In our understanding, a voice frame is typically delivered every 20 ms in each link direction (note that for reducing the protocol overhead several voice frames can be bundled e.g., 2 or 4 voice frames leading to deliveries every 40 or 80 ms), therefore rather than using dynamic scheduling for handling those speech frames, a semi-persistent scheduling (SPS) is expected to be a more suitable scheme to cope with such transmissions.

Overall, an SPS-based transmission does not require an NPDCCH to provide the characteristics of the transmission/reception of the data nor for pointing to the granted time-frequency resources, since instead the transmission/reception characteristics will be pre-established whereas the time-frequency resources will be granted in advance which will become available persistently with certain periodicity. To our knowledge, SPS is supported for LTE-MTC in coverage enhancement mode A (CE Mode A), but it is not supported for LTE-MTC in CE Mode B nor for NB-IoT.

For the support of voice-over-GEO, a semi-persistent scheduling (SPS) scheme is foreseen to be suitable to deal with voice frames which are typically delivered every 20 ms in each link direction (bundled voice frames for reducing the protocol overhead can lead to longer deliveries, e.g., every 40 or 80 ms).

Whereas the design details of the SPS scheme should be discussed in RAN1, it would be beneficial defining the scope and objectives in RAN Plenary which will facilitate the technical discussions. In our view, and as it was done for some Rel-19 objectives, there should be at least two meetings dedicated to study voice-over-GEO scenarios within a NB-IoT NTN context, and thereafter RAN1 can proceed to discuss the required solutions to specify the support of voice-over-GEO for NB-IoT NTN.

In our view, as part of the Rel-20 for the support of voice-over-GEO using NB-IoT NTN, there should be a study phase preceding the discussions on what to specify.

As part of the study phase, RAN1 should:

- Reach consensus on the simulation assumptions to evaluate a SPS-based scheme for voice-over-GEO scenarios in NB-IoT NTN.

- Perform an analysis including both voice frames (delivered every 20 ms in each link direction) and bundled voice frames (e.g., 2 or 4 bundled voice frames leading deliveries every 40 or 80 ms).

- Perform UL evaluations:

  - NPUSCH Format 1 with 15 kHz SCS using single-tone and/or multi-tone.

  - NPUSCH Format 1 with 3.75 kHz SCS using single-tone.

  - Assuming one or more than one Resource Unit (RU) depending on Transport Block size deemed to be suitable for transmitting a voice packet.

  - Assuming the use of repetitions (e.g., the number of required repetitions can be based on a link-budget analysis).

  - Achievable data rate.

- Perform DL evaluations:

  - NPDSCH with 15 kHz SCS.

  - Assuming one or more than one NPDSCH subframes depending on Transport Block size deemed to be suitable for transmitting a voice packet.

  - Assuming the use of repetitions (e.g., the number of required repetitions can be based on a link-budget analysis).

  - Achievable data rate.

  - Any NPDCCH/NPDSCH scheduling required e.g., for IMS.

Then, based on the observations and conclusions reached during the study phase of this objective, RAN1 can proceed to discuss the design details and components of the SPS scheme intended to be used for the support of voice-over-GEO for NB-IoT NTN.

### 2.1.4	Other impacts

From a radio point of view, connectivity with satellites in the GEO orbits presents challenges due to the low supported bitrates and the long propagation delays. To address this, the following changes to the 3GPP specifications have been discussed [11]:

- Supporting a new voice codec with low rate, adapted to the typical bitrates supported by a GEO IoT NTN connection. This objective has no impact on RAN working groups.

- Potentially, CN signalling optimizations to minimize the overhead. The need for this objective should be determined by SA2. It has no impact on RAN working groups.

Moreover, NB-IoT NTN is built on terrestrial NB-IoT, which has a reduced set of features compared to LTE or even LTE-M. For example, it lacks mobility support in RRC Connected. Rather than attempting to address these limitations, the work should be restricted to supporting stationary UEs communicating with GEO satellites.

In our view, the specification work in RAN WGs should be limited to supporting stationary UEs communicating over GEO satellites.

## 2.2	Summary

Based on the information provided in previous sections, we have the following proposal for defining the scope of the objective on voice-over-GEO for NB-IoT NTN:

Study and then specify, support of voice-over-GEO for NB-IoT NTN including:

- Impact on initial access and RRC connection procedures required updates (if any), from supporting voice and emergency (the latter if support by SA2).

- Feasibility and specification impact of supporting data over UP and data over CP, in coordination with SA2.

- Semi-persistent scheduling (SPS) for voice frames (e.g., 20 ms) and bundled voice frames (e.g., 40 ms, 80 ms).

- Simulation assumptions and evaluations.

  - NPUSCH Format 1 with 3.75 kHz and/or 15 kHz SCS using single-tone and/or multi-tone/NPDSCH with 15 kHz SCS.

  - Assuming one or more than one Resource Unit (RU)/NPDSCH subframes depending on Transport Block size deemed to be suitable for transmitting a voice packet.

  - Assuming the use of repetitions (e.g., the number of required repetitions can be based on a link-budget analysis).

  - Any NPDCCH/NPDSCH scheduling required e.g., for IMS

- Note: Based on the feasibility identified from the study, the options in the bullets above may be subject to down-selection.

# 3	Conclusion

Based on the discussion in the previous sections we made the following observations:

Observation 1	RAN specifications currently exclude support for real-time services such as voice and emergency services for terrestrial NB-IoT and, consequently, for IoT-NTN.

Observation 2	Supporting voice in the RAN will require changes to the specifications in the access and connection procedures and for scheduling.

Observation 3	RAN in coordination with SA2 should study the feasibility and specification impact of supporting data over UP and data over CP. It is desirable to limit the amount of specification work required to support IMS voice via NB-IoT.

Observation 4	For the support of voice-over-GEO, a semi-persistent scheduling (SPS) scheme is foreseen to be suitable to deal with voice frames which are typically delivered every 20 ms in each link direction (bundled voice frames for reducing the protocol overhead can lead to longer deliveries, e.g., every 40 or 80 ms).

Observation 5	In our view, as part of the Rel-20 for the support of voice-over-GEO using NB-IoT NTN, there should be a study phase preceding the discussions on what to specify.

Observation 6	In our view, the specification work in RAN WGs should be limited to supporting stationary UEs communicating over GEO satellites.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Study and then specify, support of voice-over-GEO for NB-IoT NTN including:

	Impact on initial access and RRC connection procedures required updates (if any), from supporting voice and emergency (the latter if support by SA2).

	Feasibility and specification impact of supporting data over UP and data over CP, in coordination with SA2.

	Semi-persistent scheduling (SPS) for voice frames (e.g., 20 ms) and bundled voice frames (e.g., 40 ms, 80 ms).

	Simulation assumptions and evaluations.

o	NPUSCH Format 1 with 3.75 kHz and/or 15 kHz SCS using single-tone and/or multi-tone/NPDSCH with 15 kHz SCS.

o	Assuming one or more than one Resource Unit (RU)/NPDSCH subframes depending on Transport Block size deemed to be suitable for transmitting a voice packet.

o	Assuming the use of repetitions (e.g., the number of required repetitions can be based on a link-budget analysis).

o	Any NPDCCH/NPDSCH scheduling required e.g., for IMS

Note: Based on the feasibility identified from the study, the options in the bullets above may be subject to down-selection.

# 4 References

RP-242550, “Views on NTN for IOT WI in Rel-20”, Qualcomm, Madrid, Spain, Dec 9-12, 2024.

RP-242627, “Support of voice call over GEO satellite based on NB-IOT in 5G-A Rel-20”, vivo, CMCC, KT Corp., Spreadtrum, Viasat, Inmarsat, Verizon, NTT DOCOMO, Madrid, Spain, Dec 9-12, 2024.

RP-242893, “IMS voice support over GEO IoT NTN”, Ericsson, Madrid, Spain, Dec 9-12, 2024.

RP-243161, “Views on NR- and IoT-NTN in 5G-A Rel-20”, ZTE, Sanechips, Madrid, Spain, Dec 9-12, 2024.

RP-242514, “NTN enhancements phase 4”, NEC, Madrid, Spain, Dec 9-12, 2024.

RP-242664, “RAN123 NTN topics for Rel-20 5G Advanced - Overview”, Thales, Novamint, TNO, Echostar, ESA, Eutelsat, Airbus, Sateliot, Madrid, Spain, Dec 9-12, 2024.

RP-242876, “Views on Rel-20 NTN Evolution,” Lenovo, Madrid, Spain, Dec 9-12, 2024.

RP-242968, “Support of GEO satellite voice call,” CMCC, Madrid, Spain, Dec 9-12, 2024.

RP-243127, “Consideration on NTN in 5G-A Rel-20,” CATT, Madrid, Spain, Dec 9-12, 2024.

3GPP TS 36.331, “Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification,” version 18.4.0.

S1-244889, “Consolidation proposal on single use case IMS Voice Call using GEO satellite access”, Vivo et al, 3GPP TSG-SA WG1 Meeting #108, Nov. 18 – 22, 2024, Orlando, USA.