3GPP TSG-RAN WG1 Meeting #107	RP-250411

Incheon, Republic of Korea, March 12th – 14th, 2025

Agenda Item:	15.2.8.3

Source:	Ericsson

Title:	Rel-20 UAV

Document for:	Discussion

# 1	Introduction

3GPP specified RAN features targeting UAVs in NR Rel-18. There was some interest in further enhancements for UAV and UAM in Rel-19, but no SI/WI was approved. In the Rel-20 scoping discussions, there have been a few proposals to include UAV/UAM.

In this contribution we present Ericsson’s view on this topic.

# 2	Discussion

Rel-18 introduced NR RAN features to support UAV operations. This was done by porting Rel-15 LTE features to NR and adding some new features targeting spectrum regulation (i.e., new NS values to limit out-of-band emissions by the aerial UEs) and aviation regulation (i.e., support for remote ID broadcasting over the PC5 interface.)

In Rel-19 scoping discussions, the following topics were identified as potential enhancements [1]:

- Mobility enhancements

  - Specifying new CHO triggering events

  - Studying/specifying LTM enhancements for UAV

- No transmit zone (if SA2 will also do work on this)

- Enhancements for RRC_IDLE/RRC_INACTIVE

In our view, among these topics, the most meaningful enhancements would be those to comply with no-transmit zone (NTZ) regulations, if deemed necessary. Indeed, the NTZ topic was heavily discussed in SA2 and in RAN plenary. The conclusion that came out was that NTZ compliance shall have no RAN impacts.

In the Rel-20 NR scoping discussion, particularly in RAN#106, there were a few proposals focusing on the remaining two topics, namely mobility enhancements (CHO/LTM) and enhancements for RRC_IDLE/RRC_INACTIVE targeting UAV/UAM. Most of the proposals concerned small enhancements such as adding height-based and flight-path-based conditions to the existing idle-mode or connected-mode mobility procedures. In our view, those enhancements are neither essential nor provably beneficial for UAV/UAM operations. We believe that the most beneficial enhancements (which included several height-based enhancements) have already been done in Rel-18. Furthermore, given very tight TU budget in Rel-20, due to the parallel running of 6G SI and 5G-advance SI/WI's, there is no reason for RAN to prioritize optimization for UAV/UAM.

No enhancements to NR for UAV/UAM in Rel-20.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	No enhancements to NR for UAV/UAM in Rel-20.

# References

RP-232627, Moderator’s summary for REL-19 RAN2 additional topics, 3GPP TSG RAN Meeting #101, Bangalore, India, September 11-15, 2023.