3GPP TSG RAN meeting #107										       RP-250413

Incheon, Korea, March 12-14, 2025

## Status Report to TSG

Agenda item:			9.3.1.6

| WI / SI Name | Multi-carrier enhancements for NR Phase 3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: No |  | Testing part: No |
| Acronym | NR_MC_enh2 |  |  |  |  |
| Unique ID | 1050120 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-242904 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 09/2025 | Performance part: N/A | Testing part: N/A |  |
| Overall Completion level | Study Item: N/A | Core part: 80% | Performance Part: N/A | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Haipeng Lei |
|  | Company | Lenovo |
|  | Email | leihp1@lenovo.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

RAN1#120:

Agreement

- For determining the timing of a PUCCH carrying HARQ-ACK information corresponding to a set of co-scheduled PDSCHs by a DCI format 1_3, follow Rel-18 operation.

- If the UE is not provided subslotLengthForPUCCH, the DL slot 𝑛𝐷 is the DL slot ending last, amongst the DL slots where the more than one PDSCH are scheduled by the DCI format 1_3.

  - FFS: RAN1 spec impact till RAN1#120-bis

- If the UE is provided subslotLengthForPUCCH, no spec change is necessary.

- Note: Specification of this feature shall not impact the existing UE processing PDSCH timeline requirement for any individual PDSCH, as specified in 5.3.1 of TS38.214.

Agreement

- For the second sub-codebook, the number of HARQ-ACK information bits for each DCI format 1_3 is equal to M.

- M is the maximum number of HARQ-ACK information bits which can be generated for a DCI format 1_3 across all the configured cell set(s) in the PUCCH group for the UE. M is implicitly derived based on RRC configuration.

Agreement

- For NDI indication in DCI format 0_3/1_3 for a TB,

  - Option 1 is adopted.

Agreement

- For RV indication in DCI format 0_3/1_3 for a TB,

  - Option 1 is adopted.

Agreement

- For multi-PUSCH/PDSCH scheduling using a DCI format 0_3/1_3, 1 bit RV indication is determined according to Table 7.3.1.2.3-1 of TS 38.212.

  - Note: This is aligned with Rel-18 DCI format 0_3/1_3 for cells configured with 1 bit RV by numberOfBitsForRV-DCI-0-3/1-3.

Agreement

- For the second sub-codebook, the HARQ-ACK information bits for a DCI format 1_3 are ordered firstly according to same ordering as in Rel-17 multi-PDSCHs scheduling for PDSCH receptions on a same serving cell, then according to ascending order of associated serving cell indexes.

Agreement

- For Type-1C fields in DCI format 0_3, when the TDRA field indicates more than one scheduled PUSCHs on the scheduled cell with the smallest cell index:

  - The CSI request field applies to the PUSCH determined based on Rel-17 multi-PUSCH scheduling on same serving cell.

  - Note for background: When the TDRA field of DCI format 0_3 indicates only one scheduled PUSCH on the scheduled cell with the smallest cell index, DCI interpretation and UE procedure is same as in Rel-18.

Working Assumption

For a DCI format 1_3 with fields repurposed for SCell dormancy indication and scheduling one or more PDSCHs, if TDRA field indicates multiple SLIVs for the serving cell with smallest serving cell index with invalid FDRA, the HARQ-ACK information bit for SCell dormancy indication is ACK for the first SLIV and followed by NACK bits for the remaining SLIVs.

Agreement

- Regarding presence of UL-SCH field,

  - 1 bit UL-SCH field is always present in DCI format 0_3.

- UL-SCH field and CSI request field in a DCI format 0_3 are applied to the same PUSCH.

Agreement

- For NDI/RV indication in DCI format 0_3/1_3 for a TB,

  - Within each block of NDI field, the NDI bits are placed in the LSBs based on the SLIV position in the indicated TDRA row. Padding bits, if any, are placed in the MSBs within the block.

  - Within each block of RV field, the RV bits are placed in the LSBs based on the SLIV position in the indicated TDRA row. Padding bits, if any, are placed in the MSBs within the block.

#### 2.1.2	Remaining Open issues

Discussion on RRC parameter design,

–	e.g., TDRA table

Discussion on spec impact on HARQ-ACK timing determination in case of slot-based PUCCH

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

#### 2.4.2	Remaining Open issues

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1#120

- R1-2501377	Feature lead summary #4 on multi-cell scheduling with a single DCI Moderator (Lenovo)

- R1-2501376	Feature lead summary #3 on multi-cell scheduling with a single DCI Moderator (Lenovo)

- R1-2501375	Feature lead summary #2 on multi-cell scheduling with a single DCI Moderator (Lenovo)

- R1-2501374	Feature lead summary #1 on multi-cell scheduling with a single DCI Moderator (Lenovo)

- R1-2500065	On Rel-19 Multi-carrier enhancements	Nokia

- R1-2500092	Discussion on Rel-19 Multi-carrier enhancements	Huawei, HiSilicon

- R1-2500139	Discussion on multi-cell scheduling with a single DCI	Lenovo

- R1-2500147	Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI	ZTE Corporation, Sanechips

- R1-2500187	Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI	Spreadtrum, UNISOC

- R1-2500212	Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI	CATT

- R1-2500271	Discussion on multi-carrier enhancements for NR phase 3	China Telecom

- R1-2500306	Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI	CMCC

- R1-2500371	Discussion on enhancement of multi-cell PUSCH/PDSCH scheduling with a single DCI	vivo

- R1-2500461	Discussion of multi-cell scheduling with a single DCI	OPPO

- R1-2500808	On multi-cell PUSCH/PDSCH scheduling with single DCI	Apple

- R1-2500818	Discussion on multi-carrier enhancements for NR Phase 3	Panasonic

- R1-2500871	Enhancements for multi-cell PUSCH/PDSCH scheduling	Samsung

- R1-2500923	Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI	ETRI

- R1-2500960	Discussion on single DCI based multi-cell scheduling for Rel-19	LG Electronics

- R1-2500985	Discussion on Multi-cell PUSCH/PDSCH scheduling with a single DCI	TCL

- R1-2501033	Multi-cell PUSCH/PDSCH scheduling with a single DCI	MediaTek Inc.

- R1-2501178	Multi-cell PUSCH/PDSCH scheduling with a single DCI	Qualcomm Incorporated

- R1-2501220	Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI	NTT DOCOMO, INC.

- R1-2501345	Multi-cell PxSCH scheduling with a single DCI	Ericsson

10.11.2023		minor adaptations for RAN #102

02.08.2023		minor adaptations for RAN #101

26.04.2023		minor adaptations for RAN #100

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template