3GPP TSG RAN Meeting #107						RP-250431

Incheon, Korea, March 12-14, 2025

Agenda item:	16

Source:	Huawei, HiSilicon

Title:	Further considerations on IMT-2030 TPRs

Document for:	Discussion

Background

According to the endorsed 3GPP 6G timeline [1], RAN#106 approved the new Study Item of “Study on 6G scenarios/use cases and requirements” (ITU focused part) [2]. A reply LS was sent to ITU-R WP5D including initial considerations of possible TPRs, evaluation configurations of carrier frequency and bandwidth, as well as the workplan [4]. According to the workplan, RAN#107 aims to finalize the candidate set of TPRs, initially discuss the associated values, and send further LS to ITU-R as necessary [5]. In addition, RAN#107 meeting will revise the SID to include part 2 of general 6G RAN study. To facilitate the meeting discussions, this document provides our suggestions on these planned topics for RAN#107.

Candidate set of TPRs

Table 1: Proposal on candidate set of minimum Technical Performance Requirements for IMT-2030 radio interface(s) in ITU-R WP 5D

|  | Candidate IMT-2030 TPRs under discussion in WP 5D [6] |  | TPRs in ITU IMT-2020 [7] | Proposed to be defined as ITU IMT-2030 TPR |
| --- | --- | --- | --- | --- |
| 1 | Peak data rate |  | Legacy TPRs of IMT-2020 (At RAN#106, these 14 items were agreed as the first set of 3GPP’s proposal for IMT-2030 TPRs) |  |
| 2 | Peak spectral efficiency |  |  |  |
| 3 | User experienced data rate |  |  |  |
| 4 | 5th percentile user spectral efficiency |  |  |  |
| 5 | Average spectral efficiency |  |  |  |
| 6 | Area traffic capacity |  |  |  |
| 7 | User plane latency |  |  |  |
| 8 | Control plane latency |  |  |  |
| 9 | Connection density |  |  |  |
| 10 | Energy efficiency |  |  |  |
| 11 | Reliability |  |  |  |
| 12 | Mobility |  |  |  |
| 13 | Mobility interruption time |  |  |  |
| 14 | Bandwidth |  |  |  |
| 15 | Positioning |  | New TPRs | (1) |
| 16 | Sensing-related capabilities | [Detection/False alarm probability] (2) |  |  |
|  |  | [Localization accuracy] |  |  |
|  |  | [Velocity accuracy] |  |  |
|  |  | [Sensing resolution] |  |  |
| 17 | AI-related capabilities |  |  |  |
| 18 | [“Joint / Composite requirement [on data rate, …]”] |  |  |  |
| 19 | [Coverage] |  |  |  |
| 20 | [Security] (3) |  |  |  |
| 21 | [Resilience] |  |  |  |
| 22 | [Interoperability] (3) |  |  |  |
| 23 | [Sustainability] |  |  |  |

(1) Note: It is not intended to restrict the implementation details of IMT-2030 technology on how to meet these requirements. For example, setting “positioning accuracy” as performance requirements for IMT-2030 radio interface technology doesn’t imply that LMF (Location management function) is required in IMT-2030 RAN. LMF is located in core network of NR technology. This is up to implementation details of IMT-2030 technology.

(2) Note: Items in square brackets haven’t reached consensus in ITU discussion.

(3) Note: Security and Interoperability are important design principle for communication system. However, they are not appropriate to be the technical performance requirement of radio interface technology.

Proposal 1: To adopt the candidate items in above table 1 for minimum technical performance requirements for IMT-2030 radio interface(s), and provide feedback to ITU-R WP 5D#49 meeting in June 2025.

Preliminary views on target values

Table 2: Preliminary views on target values for minimum technical performance requirements for IMT-2030 radio interface(s) in ITU-R WP 5D

| ITU IMT-2030 TPR |  |  | Preliminary views on target value | Comments |
| --- | --- | --- | --- | --- |
| Peak data rate |  |  | DL: 100~200, UL: 50~100Gbps | 5~10 x IMT-2020 |
| Peak spectral efficiency |  |  | DL: 60~90, UL: 30~45bps/Hz | 2~3 x IMT-2020 |
| User experienced data rate |  |  | DL: 500~1000, UL: 250~500Mbps | 5~10 x IMT-2020 |
| 5th percentile user spectral efficiency |  |  | Indoor Hotspot 0.9-1.5 (DL), 0.63-1.05 (UL) Dense Urban 0.675-1.125, 0.45-0.75 Rural 0.36-0.6, 0.135-0.225 | 3~5 x IMT-2020, see annex for simulations |
| Average spectral efficiency |  |  | Indoor Hotspot 27-45 (DL), 20.25-33.75 (UL) Dense Urban 23.4-39, 16.2-27 Rural 9.9-16.5, 4.8-8 bps/Hz |  |
| Area traffic capacity |  |  | 50~100 Mbit/s/m2 @ Indoor Hotspot | 5~10 x IMT-2020 |
| latency |  | User plane latency | 0.5-1ms for HRLLC 1ms for Immersive communication | 1/2-1 x IMT-2020 |
|  |  | Control plane latency | 10ms | 1/2 x IMT-2020 |
| Connection density |  |  | 107 - 108 devices/km2 for IoT traffic | 10-100 x IMT-2020 |
| Energy efficiency |  |  | Target value TBD |  |
| Reliability |  |  | 1-10-6 - 1-10-7 for HRLLC | 10-100 x IMT-2020 |
| Mobility |  |  | Target value TBD |  |
| Mobility interruption time |  |  | 0ms |  |
| Bandwidth |  |  | At least 200MHz | 2 x IMT-2020 |
| Positioning |  |  | Horizontal accuracy: [20cm] @90% for indoor, [3-5m] @90% for outdoor [Vertical accuracy]: [1-3m] @90% for indoor | Assume 400MHz bandwidth, the necessity of vertical accuracy is FFS |
| Sensing-related capabilities | Detection/False alarm probability |  | Target value TBD | Awaiting the outcome of channel model study |
|  | Localization accuracy |  |  |  |
|  | Velocity accuracy |  |  |  |
|  | Sensing resolution |  |  |  |
| AI-related capabilities |  |  | N. A | Evaluated by inspection |
| “Joint / Composite requirement [on data rate, …]” |  |  | 500-1000 users/km2 for example XR use case of TR38.838 | Corresponding to 6-12 users per cell of ISD=200m, see annex for simulations |

In previous IMT-2020 discussion the values adopted are several times higher than IMT-Advanced, e.g. requirements on cell average/cell edge spectral efficiency are 3 x IMT-A, which is to show the advanced capabilities for a new generation compared with the old ones. The values in table 2 are suggested by the same principle.

In 2019, 3GPP submitted IMT-2020 self-evaluation report to ITU-R [8]. The evaluation results of NR Release 16 significantly exceeded ITU-R requirements, for example 2~3 times of ITU IMT-2020 requirements in case of cell average and cell edge spectrum efficiency in Dense Urban environment as shown in table below from 3GPP TR 37.910.

Table 5.4.1.2.1-1 from 3GPP TR 37.910, DL spectral efficiency for NR in Dense Urban – eMBB 
(Evaluation configuration A, CF=4 GHz)

(b) NR TDD

| Scheme and antenna configuration | ITU Requirement |  | Channel model A |  |  |  | Channel model B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Number of samples | BW= 20MHz | BW= 40MHz | BW= 100MHz | Number of samples | BW= 20MHz | BW= 40MHz | BW= 100MHz |
| 32x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (8,8,2,1,1;2,8) | Average [bit/s/Hz/TRxP] | 7.8 | 4 | 13.03 | 15.26 | 16.77 | 4 | 13.07 | 15.20 | 16.60 |
|  | 5th-tile [bit/s/Hz] | 0.225 |  | 0.47 | 0.55 | 0.61 |  | 0.45 | 0.53 | 0.57 |
| … |  |  |  |  |  |  |  |  |  |  |
| 64x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (12,8,2,1,1;4,8) | Average [bit/s/Hz/TRxP] | 7.8 | 1 | 16.73 | 20.08 | 22.33 | 1 | 16.59 | 19.91 | 22.14 |
|  | 5th-tile [bit/s/Hz] | 0.225 |  | 0.61 | 0.73 | 0.81 |  | 0.57 | 0.68 | 0.76 |

Observation 1: ITU-R requirements of IMT-2020 are several times higher than IMT-Advanced, e.g. 3 x IMT-A in case of cell average/cell edge spectral efficiency.

Observation 2: Evaluation result of 3GPP NR R16 in 2019 significantly exceeded ITU-R IMT-2020 requirements, e.g. 2~3 times of ITU IMT-2020 requirements in case of cell average/cell edge spectrum efficiency in Dense Urban environment.

Furthermore, the relation between these target values and the real network performance was mentioned in previous discussion, e.g. due to the limitation of operating conditions (availability of cell sites, spectrum etc.), it may be difficult for real network to achieve downlink user experience data rate > 100Mbps as required by ITU IMT-2020 TPRs. This is true and has been clarified in ITU document with following text in section 4 of IMT-2020 TPR [6].

“These requirements are not intended to restrict the full range of capabilities or performance that candidate RITs/SRITs for IMT-2020 might achieve, nor are they intended to describe how the RITs/SRITs might perform in actual deployments under operating conditions that could be different from those presented in other ITU-R Recommendations and Reports on IMT-2020.”

According to the working method in ITU-R, TPRs are intended for the evaluation of candidate technology proposals (RITs/SRITs: Radio interface technologies/Set of radio interface technologies) on the fulfilment of ITU IMT-2030 requirements. It is conducted with associated evaluation methodology of simulation which may not be reflecting the real network conditions.

Observation 3: TPRs are intended for the simulation evaluation of candidate technology proposals on the fulfilment of ITU requirements. They are not intended to describe how the RITs/SRITs might perform in actual deployments.

Proposal 2: To adopt the target values in above table 2 for IMT-2030.

Conclusion

Proposal 1: To adopt the candidate items in above table 1 for minimum technical performance requirements for IMT-2030 radio interface(s), and provide feedback to ITU-R WP 5D#49 meeting in June 2025.

Proposal 2: To adopt the target values in above table 2 for IMT-2030.

- Observation 1: ITU-R requirements of IMT-2020 are several times higher than IMT-Advanced, e.g. 3 x IMT-A in case of cell average/cell edge spectral efficiency.

- Observation 2: Evaluation result of 3GPP NR R16 in 2019 significantly exceeded ITU-R IMT-2020 requirements, e.g. 2~3 times of ITU IMT-2020 requirements in case of cell average/cell edge spectrum efficiency in Dense Urban environment.

- Observation 3: TPRs are intended for the simulation evaluation of candidate technology proposals on the fulfilment of ITU requirements. They are not intended to describe how the RITs/SRITs might perform in actual deployments.

References

- RP-240823, Additional Considerations for 6G Timeline, TSG Chairs, 3GPP TSG#103 Joint Session, March 2024

- RP-243327 New SID: Study on 6G scenarios/use cases and requirements, CMCC, Verizon, NTT DOCOMO, INC., Deutsche Telekom, 3GPP RAN#106, December 2024

- RP-243202 Liaison Statement to External Organizations Minimum requirements related to technical performance for IMT-2030 radio interface(s), source ITU-R WP5D, 3GPP RAN#106, December 2024

- RP-243325	Draft Reply to ITUR_WP5D_TEMP_167 = RP-243202 on Minimum requirements related to technical performance for IMT-2030 radio interface(s) (to: SA; cc: -; contact: CMCC), 3GPP RAN#106, December 2024

- RP-243276	RAN Chair’s Guidance for 6G Study: ITU Focused, 3GPP RAN#106, December 2024

- “Working document towards a Preliminary Draft New Report ITU-R M.[IMT-2030.TECH PERF REQ]: Minimum requirements related to technical performance for IMT-2030 radio interface(s)”, Annex 5.7 to Working Party 5D Chairman’s Report.

- Report ITU-R M.2410-0 (11/2017), “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”.

- 3GPP TR 37.910 Study on self evaluation towards IMT-2020 submission (Release 16)

Annex 1: proposed new TPR definitions for IMT-2030

Note: Following texts are cited from latest TPR working document of ITU-R WP5D. Tracked changes (highlighted in turquois colour) are applied in case of proposing modifications.

Energy efficiency

Energy efficiency is an important metric of sustainability. Network energy efficiency is the capability of a RIT/SRIT to support radio access network energy saving in relation to the traffic capacity provided. Device energy efficiency is the capability of the RIT/SRIT to support device modem power saving in relation to the traffic characteristics.

This requirement is defined for the purpose of evaluation in the Immersive communication usage scenario.

The requirement is the relative energy savings between the selected load case(s) and a reference case. Proponents should report the power model used for the evaluation, as well as impacts on communication performance associated with the evaluated power saving technologies, see [IMT-2030 EVAL] for details.

The minimum requirements for Energy efficiency are summarized in Tables below.

[Editor’s note: The following table needs further discussion, including detailed items and the evaluation methodology such as analytical and/or simulation]

Table: The minimum requirements for Network Energy efficiency

| [Test environment(s)] ** | Selected load case(s) | The reference case | Relative power saving (%) |
| --- | --- | --- | --- |
| TBD | [empty load, low load, light load and/or medium load] * | Fully loaded IMT-2030 case * | [X1%, X2%, ...] |

*empty load: L=0, low load: 0 < L≤15, light load: 15 < L≤30, medium load: 30 < L≤50, fully loaded case: L=100

** needed only in case of simulation

Table: The minimum requirements for Device Energy efficiency

| [Test environment(s)] ** | Selected load case(s) | The reference case | Relative power saving (%) |
| --- | --- | --- | --- |
| TBD | [empty load, low load, light load and/or medium load] * | Fully loaded IMT-2030 case * | [A1%, A2%, …] |

Sensing-related capabilities

- Detectability (Detection / False Alarm Probability) – Detection probability, PD, is the probability of correctly detecting the presence of the object, and False alarm probability, PFA, is the associated probability that an object is detected given that the object is not present.

- Localization Accuracy - Localization accuracy refers to the closeness of the measured sensing result of position of the target object to its true position value and is the difference between the calculated location and the actual location of an object.

- Velocity Accuracy - Velocity accuracy is defined as the difference between the estimated velocity and the actual velocity of the sensing target.

- Sensing Resolution – Is defined as the minimum difference of the measured value (e.g, range, location) of two target objects, in order to distinguish these two target objects. (Ed note: this may be evaluated by an analytical approach).

AI-related capabilities

AI-related capabilities refer to the ability to provide certain functionalities throughout IMT-2030 to support AI enabled applications.

The RIT should support AI-related capabilities, which refer to the ability for the RIT/SRIT to support mechanisms and/or signalling/ related to AI functionalities over the radio interface (between UE and Network). This should include support for one or more of the following for IMT-2030 assisting AI applications i.e. “Network for AI” and/or using AI to improve IMT-2030 performance, i.e. “AI for network”

-	Distributed data processing

- 	Distributed learning

-	AI computing

-	AI model execution

-	AI model inference

-	or other AI capabilities of the RITs/SRITs (to be reported by the proponent)

Composite requirement

[Editor’s note: Name of this TPR will be discussed further. Currently this is a placeholder.]

This composite requirement is defined as the number of users  that fulfil the required success probability (reliability) of transmitting a layer 2/3 packet within a required [maximum time / latency], which is the time it takes to deliver a data packet [for a given arrival rate] from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface at a certain channel quality.

This requirement is defined for the purpose of evaluation in the Immersive Communication [and Hyper-Reliable Low Latency Communication] usage scenario.

The minimum performance requirement for various test environments is summarized in Table X.

TABLE X

Composite requirement (title_placeholder)

[editor’s note: This table needs further discussions.]

| Test environment | Data rate (Mbits/s) | Latency (ms) | Reliability | Number of satisfied users (/km2) |
| --- | --- | --- | --- | --- |
| Dense Urban - Immersive Communication | DL = 30Mbps/UL=10Mbps | DL=10ms/UL=30ms | 99% | [500-1000] devices per km2 |  |
| [Indoor – HRLLC] |  |  |  |  |  |

Annex 2: preliminary analysis and simulation results

Spectral efficiency

Based on extension of ITU IMT-2020 evaluation methodologies [6], spectral efficiency is simulated for three eMBB test environments of Dense urban, Rural and Indoor. The evaluated carrier frequency is replaced from 4GHz to 7 GHz. The extended MIMO configuration is BS up to 1536 Tx/Rx antenna elements, UE up to 16 Tx/Rx antenna elements. ITU IMT-2020’s configuration is BS up to 256 Tx/Rx antenna elements, UE up to 8 Tx/Rx antenna elements (AE) for 4GHz carrier frequency [6].

The simulation results are in figures and summarized in table below. The total overhead (control channel, reference signal etc.) is counted as 20%. The effect of real channel estimation is considered. Compared with ITU IMT-2020 requirements, 4 to 35 times improvement of spectral efficiency are observed with different extended MIMO configurations at 7 GHz.

Table 8.1: Spectral efficiency of different MIMO configurations@7GHz

| eMBB test environments |  | ITU IMT-2020 requirements of Spectral efficiency (Average/Cell edge, bps/Hz) | Spectral efficiency of different MIMO configurations@7GHz (Average/Cell edge, N x ITU IMT-2020 requirements) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 256x8* | 256x16* | 512x8* | 512x16* |
| DL | Dense urban | 7.8/0.225 | 4.8x/5.6x | 7.2x/9.4x | 6.4x/7.8x | 9x/13.4x |
|  | Rural | 3.3/0.12 | 11.7x/8.4x | 17.7x/14.8x | 16x/12.3x | 22.5x/22.9x |
|  | Indoor | 9/0.3 | 4.1x/6.3x | 6.2x/8.7x | 6.1x/8.9x | 8x/13x |
| UL | Dense urban | 5.4/0.15 | 10.9x/4.2x | 12.4x/5.2x | 13.2x/5.7x | 14.6x/7.4x |
|  | Rural | 1.6/0.05 | 26.7x/8.9x | 29.8x/11.6x | 32.5x/13.6x | 35.4x/17.8x |
|  | Indoor | 6.75/0.21 | 8.3x/16.8x | 10x/21.2x | 11.2x/27x | 12.9x/33.4x |

* for outdoor (Dense urban and Rural)

- BS 256: (M, N, P, Mg, Ng; Mp, Np) = (24,16,2,1,1,8,16), 768 antenna elements

- BS 512 = (24,32,2,1,1,8,32), 1536 antenna elements

For outdoor hotspot

- BS 256 = (8,16,2,1,1,8,16), 256 antenna elements

- BS 512 = (8,32,2,1,1,8,32), 512 antenna elements

For outdoor and indoor, UE 8 = (2,2,2,1,1,2,2), UE 16 = (2,4,2,1,1,2,4)

Composite requirements (for XR use case)

Using the evaluation methodology of 3GPP study on XR (Extended Reality) evaluations for NR in TR38.838, XR capacity is evaluated for eMBB dense urban test environment with similar extended MIMO configurations as above section 7.1 at 7 GHz.

The simulation results are in figures below for DL and UL respectively. It can be seen that, under the simulated cell configurations,

- For downlink, 256x8 MIMO configuration supports about 54 user per cell of ISD=200m (corresponding to 4676 users/km2), with 90% of UEs being satisfied.

- For uplink, 256x8 MIMO configuration supports about 12.5 user per cell of ISD=200m (corresponding to 1082 users/km2), with 90% of UEs being satisfied.

Table 8.2: Traffic model: GOP (Group-Of-Picture) based I/P-Stream traffic model from 3GPP TR38.838

| Application |  | DL I/P-stream | UL pose/control | UL I/P-stream |
| --- | --- | --- | --- | --- |
| Frame per second/Sampling Frequency |  | 60 FPS | 250 Hz | 60 FPS |
| Average data rate |  | R = 30 Mbps α = 2 | 0.2 Mbps | R = 10 Mbps α = 2 |
| Packet delay budget |  | 10ms | 10ms | 30ms |
| Packet success rate |  | 99% | 99% | 99% |
| GOP size |  | K = 8 |  | K = 8 |
| Packet size | Distribution | Truncated Gaussian Distribution | Fixed Size | Truncated Gaussian Distribution |
|  | I-stream Mean | 111112 Bytes | - | 37037 Bytes |
|  | P-stream Mean | 55556 Bytes | - | 18519 Bytes |
|  | STD | 0.105*mean | - | 0.105*mean |
|  | Max | 1.5*mean | - | 1.5*mean |
|  | Min | 0.5*mean | - | 0.5*mean |
| Jitter | Distribution | Truncated Gaussian Distribution |  | Truncated Gaussian Distribution |
|  | Mean | 0 ms |  | 0 ms |
|  | STD | 2 ms |  | 2 ms |
|  | Range | [-4, 4] ms |  | [-4, 4] ms |