3GPP TSG RAN Meeting #107	RP-250434

Incheon, Korea, March 12-14, 2025

Agenda Item:	16

Source:	Apple

Title:	On IMT-2030 Technical Performance Requirements

Document for:	Discussion

# Introduction

In this contribution we provide our views on the IMT-2030 Technical Performance Requirements (TPRs), which are under discussion in ITU-R WP5D.

# Discussion

RAN#106 approved the SI [1], to investigate the IMT-2030 Technical Performance Requirements, which are being discussed in ITU-R WP5D.

Furthermore, RAN#106 approved the LS [2] where RAN provided our (very preliminary) view on the list of TPRs. It must be noted that the LS [2] approved by RAN#106 only covered the “legacy” TPRs (i.e. those common to IMT-2020 and IMT-2030) and did not address the new ones (e.g. Sensing). Furthermore, the LS [2] did not provide 3GPP recommendation about the TPR values.

Observation 1: the LS to ITU-R WP5D on IMT-2030 TPRs approved by RAN#106 did not address neither the new (e.g. Sensing) TPRs nor provided a 3GPP recommendation on the TPR values.

In the present contribution we address these open questions.

In the last ITU-R WP5D meeting, Sub-Working Group (SWG) Technology [3] sent an LS to SWG Evaluation with the list of TPRs currently under discussion. The list is provided below to facilitate the discussion:

Preliminary list of Minimum Technical Performance Requirements

(Note: Square brackets indicate further discussion required)

| Minimum Technical Performance Requirement | Method |
| --- | --- |
| AI-related capabilities | (to be determined) |
| Area traffic capacity | (Quantitative) (Note 1) |
| Average Spectral efficiency | (Quantitative) |
| Peak spectral efficiency | (Quantitative) |
| 5th percentile user Spectral efficiency | (Quantitative) |
| Bandwidth | (Quantitative) |
| Connection density | (Quantitative) |
| Control plane latency | (Quantitative) |
| User plane latency | (Quantitative) |
| Energy Efficiency | (Quantitative) |
| Mobility | (Quantitative) |
| Mobility interruption time | (Quantitative) |
| Peak data rate | (Quantitative) |
| User Experienced Data Rate | (Quantitative) |
| Sensing-related capabilities | (Quantitative) |
| Positioning | (Quantitative) |
| Reliability | (Quantitative) (Note 2) |
| [Joint requirement on data rate, latency, reliability and capacity] | To be determined |
| [Coverage] | To be determined |
| [Security] | To be determined |
| [Resilience] | To be determined |
| [Sustainability] | To be determined |
| [Interoperability] | To be determined |

Table 1: TPR list from ITU-R WP5D

Note 1: The evaluation technique for quantitative requirements is determined by SWG Eval (e.g., analysis, simulation, etc.).

Note 2: The reliability requirement may be replaced by the “Joint Requirement” (if agreed).

An immediate observation one can make is that while for many TPRs, WP5D have already agreed whether they are to be qualitative or quantitative, for some this is still under discussion. This topic must be addressed first, as there is no point in discussing a TPR value while not knowing whether the said TPR is to be evaluated quantitatively or qualitatively.

Observation 2: for some TPRs, ITU-R WP5D is yet to decide whether they are quantitative or qualitative, which in our view requires 3GPP feedback.

## Legacy TPRs

We start the discussion with “legacy” TPRs, i.e. these IMT-2030 “inherits” from IMT-2020. For these, we have a pretty good understanding of what they are and how they should be evaluated and so the main topic for discussion is the actual TPR values. So we omit the discussion on the TPR description (which is well known to all 3GPP members) and go straight to discuss the values.

Arguably, the most visible TPRs are the ones related to data rate, i.e. Peak Data Rate, User Experienced Data Rate and Spectrum Efficiency. We observe that in real deployments in the field the gap between what a 3GPP technology can deliver (on paper) and what is actually being used has grown larger and larger. This is an indication that market demand has not caught up with IMT requirements and 3GPP technologies capabilities. We believe that driving the requirements much further will widen the gap and may have negative consequences.

It is perhaps time to let market demand catch up and therefore we think that substantial increase of the throughput related TPRs is not needed at this time. Naturally, IMT-2030 data rate TPRs should be an improvement compared to IMT-2020, but such improvement should acknowledge the real market needs. We believe that 6G/IMT-2030 should focus on new services and new markets (e.g. Sensing, AI/ML) rather than pushing the theoretical data rates higher (in disconnect with real deployments).

Observation 3: the gap between what 5G/IMT-2020 can deliver “on paper” and what is actually being deployed is significant.

Proposal 1: 6G/IMT-2030 should address real market needs and therefore should focus on new services and new markets (e.g. Sensing, AI/ML) rather than pushing the theoretical data rates higher (in disconnect with real deployments).

With this in mind, we provide our view on the “legacy” TPR values in the Table 2 below. We are generally open to a discussion with regards to the actual numbers, but the overall “message” should be clear – 6G/IMT-2030 requirements should not be theoretical but rather address actual market needs.

| TPR | Value |
| --- | --- |
| Peak data rate | – Downlink peak data rate is TBD 50 Gbit/s. – Uplink peak data rate is TBD 20 Gbit/s. |
| Peak spectral efficiency | – Downlink peak spectral efficiency is 45 bit/s/Hz. – Uplink peak spectral efficiency is 22 bit/s/Hz. |
| User experienced data rate | – Downlink user experienced data rate is 300 Mbit/s. – Uplink user experienced data rate is 200 Mbit/s. |
| 5th percentile user spectral efficiency | 1.5 times that of IMT-2020, value depends on the Test Environment |
| Average spectral efficiency | 1.5 times that of IMT-2020, value depends on the Test Environment |
| Energy efficiency | TBD |
| Area traffic capacity | Area traffic capacity in downlink is 10 Mbit/s/m |
| User plane latency | 1ms |
| Control plane latency | 15ms |
| Connection density | 10 000 000 devices per km2. |
| Reliability | 1-10−6 success probability of transmitting a layer 2 PDU |
| Mobility | The following classes of mobility are defined: – Stationary: 0 km/h – Pedestrian: 0 km/h to 10 km/h – Vehicular: 10 km/h to 120 km/h – High speed vehicular: 120 km/h to 500 km/h. |
| Mobility interruption time | 0 ms |
| Bandwidth | At least 400MHz system bandwidth |

Table 2: Proposed values for "legacy" TPRs

NOTEs:

- Some values, e.g. peak data rate, are to be evaluated by inspection and therefore do not depend on a Test Environment (TE), yet to be defined by ITU-R WP5D. Some others, though, specifically those to be evaluated by simulations in a yet-to-be-defined TE, naturally depend on the simulation assumptions in the TE. Therefore, for those TPRs where the value depends on the TE, instead of providing a number (which is not possible without knowing the details of a TE), we provide a relative value (relative to the corresponding IMT-2020 values).

- In IMT-2020, Energy Efficiency was a “qualitative” TPR with the following requirement: “The RIT/SRIT shall have the capability to support a high sleep ratio and long sleep duration.”. Since in IMT-2030 WP5D decided to define a quantitative TPR for energy efficiency, but is yet to provide a definition, it is hard for 3GPP to discuss a value. Therefore, we propose to come back to discussing this TPR value in the next meeting.

- Even though IMT TPR document don’t normally refer to frequencies, 400MHz system bandwidth requirement assumes FR2.

Proposal 2: to include the values values provided in Table 2 in the TR and to communicate them to ITU-R WP5D.

## New TPRs

In this section we discuss the new IMT-2030 TPRs, i.e. those not evaluated in IMT-2020, specifically:

- AI-related capabilities

- Positioning

- Sensing-related capabilities

- [Joint requirement on data rate, latency, reliability and capacity]

- [Coverage]

- [Security]

- [Resilience]

- [Sustainability]

- [Interoperability]

Note: square brackets [] indicate that as of now there is no consensus in ITU-R to introduce the TPR.

### AI-related capabilities

AI/ML is an important aspect of 5G/IMT-2030. Notwithstanding that, it is not quite clear how to define a numerical TPRs for these since:

- There are numerous potential AI/ML capabilities and even if we manage to list them all (which seems unlikely), it would be very much unclear how to select the ones to evaluate.

- Even more importantly, AI/ML is perhaps the most forward-looking 6G feature and as such we must design 6G to be open to a wide range of AI/ML capabilities, not limited to the few we can think of at the moment.

- Finally, even if we do manage to select a few AI/ML capabilities, it does not appear to be technically feasible to devise a numerical TPR for those, as such capabilities would greatly depend on implementation and compute (not communications) capabilities – present and future.

Therefore, our proposal is that AI/ML capabilities should be a qualitative TPR evaluated by inspection.

Proposal 3: AI/ML capabilities should be a qualitative TPR evaluated by inspection.

### Positioning

Positioning is a new TPR as far as ITU-R evaluation is concerned, but it’s definitely not new to 3GPP technologies. This makes the discussion easier, as we can refer to the positioning capabilities of 5G,. Going beyond the 5G capabilities for positioning should be justified with clear market use cases and requirements.

Observation 4: IMT-2030 positioning accuracy requirements can be based on 5G positioning requirements; going beyond those should be justified with clear market use cases and requirements.

The bulk of the work on positioning was carried out in 3GPP Release-16 and Release-17. According to the Release-17 TR 38.857 [5], the target positioning accuracy requirements were:

- For commercial use cases

  - Horizontal position accuracy (< 1 m) for 90% of UEs

  - Vertical position accuracy (< 3 m) for 90% of Ues

- For Industrial-IoT use cases

- Horizontal position accuracy (< 0.2 m) for 90% of UEs

- Vertical position accuracy (< 1 m) for 90% of UEs

In Release-18, according to TR 38.859 [6], a number of positioning methods (such as NR carrier phase positioning) have been added, which can significantly improve positioning accuracy beyond the Release-17 numbers. It is important to note, though, that such high accuracy is not intended for commercial (and neither public safety) use cases.

The Release-18 positioning accuracy requirements can be found in TS 22.104 [7]. The most extreme requirements are for the “Inbound logistics for manufacturing (for storage of goods):

- Horizontal accuracy: < 20cm

- Vertical accuracy: < 20cm

Therefore, in our view, the recommended positioning accuracy numbers depend on the Usage Scenario. If they are to be evaluated in a Test Environment (TE) for a HRLLC, it is reasonable to target the numbers achievable with Release-18 functionality. On the other hand, if IMT-2030 positioning is to be evaluated in a TE environment targeting commercial use cases (such as IC), it would be reasonable to target Release-17 commercial use cases numbers (with potentially a modest improvement).

Observation 5: positioning accuracy numbers depend on whether they are to be evaluated in a commercial (e.g. IC) or industrial (e.g. HRRLC) TE.

Proposal 4:  positioning accuracy requirements are based on Release-17 numbers (when evaluated in an IC TE) and on Release-18 numbers when evaluated in a MC or HRLLC TE.

### Sensing-related capabilities

A number of sensing-related capabilities are under discussion in WP5D and the group is yet to down-select them. Therefore, we think it would be beneficial if 3GPP could:

- Provide a recommendation of which capabilities to define, and

- Provide a recommendation on the TPR values for those

While sensing is a new area for IMT (and 3GPP) with potentially wide-ranging capabilities, it is important to select a few representative ones for the purpose of evaluation (which in no way should limit future sensing usages and applications).

With this in mind, out of the multitude of different TPRs which are in the current WP5D draft report [4], we propose to recommend the following:

- Localization accuracy

- Resolution

In 3GPP, we can refer to TS 22.137 [8], which includes ISAC performance requirements. The TS [8] provides a range of numbers for different Sensing Service Categories. In our view, category 3 is the most applicable for IMT evaluation, as IMT requirements are minimum requirements and 3GPP technologies often exceed the minimum IMT requirements. Therefore, we propose the following numbers for ISAC TPRs:

- Localization Accuracy

  - Horizontal: 1m

  - Vertical: 1m

- Range resolution: 1m

Notes:

- Velocity accuracy can also be considered; however it would be good to keep the number of TPRs limited; if velocity accuracy is included, similarly to localization accuracy, service category 3 from TS 22.137 should be used.

- Maximum Sensing Range (which is also being discussed in WP5D) should be part of a TE definition, not a TPR.

- Localization is to be evaluated by simulations, resolution – by analysis.

- The numbers might depend on TEs, which are not yet defined. Therefore, we should be open to revising the numbers (if needed), when TE definitions are available.

Proposal 5: For ISAC, the requirements are: localization accuracy of 1m and range resolution of 1m.

Furthermore, sensing functionality (and the evaluation) would differ depending on:

- Mono-static vs. bi-static vs. multi-static mode of operation, and

- UE-based vs. network-based sensing

On the former, we suggest to focus (for the purpose of evaluation) on mono-static, as the other variants are more dependent on implementation details and harder to evaluate. On the latter, it is important to cover both UE-based and network-based sensing in the evaluation.

Proposal 6: ISAC evaluation should focus on mono-static sensing and should cover both UE-based and network-based sensing (e.g. UE-based can be evaluated in an Indoor TE while network-based evaluated in an Outdoor TE).

### Others

With regards to the remaining TPRs, specifically:

- [Joint requirement on data rate, latency, reliability and capacity]

- [Coverage]

- [Security]

- [Resilience]

- [Sustainability]

- [Interoperability]

Considering the lack of clarity about their definitions in WP5D, we suggest to discuss in the next meeting.

# Conclusion and Proposals

TBD

# References

[1]	RP-243327	New SID: Study on 6G Scenarios and requirements

[2]	RP-243325	Response to LS on Minimum requirements related to technical performance for IMT-2030 radio interface(s)

[3]	5D/563	Annex 5.8 to Working Party 5D Chair’s Report, MEETING REPORT OF SWG RADIO ASPECTS

[4]	5D/563	Annex 5.9 to Working Party 5D Chair’s Report, WORKING DOCUMENT TOWARDS A PRELIMINARY DRAFT NEW REPORT ITU-R M.[IMT-2030.TECH PERF REQ]

[5]	TR 38.857	Study on NR Positioning Enhancements; (Release 17)

[6]	TR 38.859	Study on expanded and improved NR positioning; (Release 18)

[7]	TS 22.104	Service requirements for cyber-physical control applications in vertical domains

[8]	TS 22.137	Service requirements for Integrated Sensing and Communication