3GPP TSG RAN Meeting #107                                                                         RP-250435

Incheon, Korea, March 12-14, 2025

Agenda Item:	15.2.8.4

Source:	CSCN

Title:	Scope for Rel-20 NR NTN in 5G-A

Document for:	Discussion, Decision

Introduction

In Release 19, a work item was introduced to enhance NG-RAN based Non-Terrestrial Networks (NTN). Considering the most critical commercial deployment need of NR NTN, further DL coverage enhancement for NR NTN should be studied in order to:

- Offer optimized performance especially when LEO satellite deployment is carried out based on the satellite capability, such as satellite parameter Set 1-2, addressing the limited number of beams for a satellite that can be simultaneously activated with the nominal EIRP density per beam. Moreover, since the NTN deployment is both time consuming and costly, operators can start the commercialization even the number of the satellites is insufficient at the early deployment stage, which means the sparse LEO satellite deployment in real-world scenario. Continued DL coverage enhancements are strongly needed to support the commercial demands and interests through capability-limited satellites and/or un-completely deployed LEO constellation. At least, larger coverage of each satellite should be provided compared with the case of dense satellite deployment, and then pursue global coverage in the maximum extent and continuous service for the users of target service area.

Further DL coverage enhancements can be considered at both

- System level to support an efficient beam hopping across the satellite foot prints for FR1-NTN and FR2-NTN.

- Link level to further improve the link margin of selected physical channels in order to accommodate the EIRP reduction and/or sparse deployment in FR1-NTN.

- Enhance the efficient utilization of air interface resources for UE mobility, motivated by:

- In the current NTN mechanism, the network transmits configuration at the cell level. The cell-level broadcast efficiency is relatively low under the wide coverage of satellite with beam hopping, leading to discrepancies between the neighbor cell configurations (e.g., SMTC, satellite ephemeris, etc.) and the actual requirements of UEs.

- In the current paging mechanism, gNB is required to send paging messages in the entire cell to reach UEs in RRC_IDLE and in RRC_INACTIVE state, resulting in low paging efficiency in the case of beam hopping. The network can configure separate paging configurations on distinct SSB beams or footprints, to enhance the paging efficiency of NTN and mitigate air interface overhead.

Objectives

- Support effective DL coverage of LEO satellite deployment based on limited satellite capability and/or sparse constellation operating in FR1-NTN or FR2-NTN. [RAN1, RAN2, RAN4]

- Based on existed evaluation methodology and relevant KPIs for coverage evaluation, identify the physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements.

- Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g., R19 leftovers) and/or system level enhancements for FR1-NTN and FR2-NTN.

  - further system level enhancements based on default SSB periodicity extending [RAN1, RAN2]

- Support of dynamic and flexible beam-sharing mechanisms (e.g., beam-hopping)

- Notes for this objective:

  - LEO Set-1 @ 600 km to be considered in priority

- Support mobility in IDLE/INACTIVE mode within the enhanced downlink coverage of regenerative LEO constellation. [RAN1, RAN2, RAN4]

- Specify necessary signalling and procedure for SSB specific system information broadcasting for measurement and paging. [RAN1, RAN2, RAN4]

2.1 Overall TU table

Considering the core and performance parts related to WGs affected potentially, an overall TU estimation is given as following.

| WG | RAN1 | RAN2 | RAN4 |
| --- | --- | --- | --- |
| TU | 3 | 3.5 | 2 |

References

RP-241789 – NR NTN (Non-Terrestrial Networks) enhancements – Thales (WI Rapporteur)