3GPP TSG RAN Meeting #107												RP-250455

Incheon, South Korea, March 10-12, 2025

Agenda Item:	9.3.1.7

Source:	Huawei, HiSilicon

Title:	Rel-19 Ambient IoT WI specifications and other issues

Document for:	Discussion and decision

# Introduction

The Rel-19 Ambient IoT WI approved in RAN#106 leaves a task for RAN#107 to decide in certain cases whether new or existing 38-series specifications will be used [1]. Section 2 provides background and guidelines on how to decide the specification method for the principal RAN1 specifications, and MAC.

In RAN1#120, it was indicated that RAN could discuss whether non-use of the agreed forward error-correcting code (FEC) in D2R was permitted by the WID. Section 3 explains that it is permitted.

Other debates in RAN WGs are normal technical consensus-finding, and do not require intervention in RAN#107.

# Specifications for A-IoT

The WID already creates new specifications in RAN4 for A-IoT BS and device requirements for the core and performance part, and for A-IoT BS conformance. See Section 2.3 for CW-node RF requirements.

For others, the current status is [1]:

| Impacted existing TS/TR |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 38.201 | NR; Physical layer; General description | RAN#108 | Core part |
| TS 38.202 | NR; Services provided by the physical layer | RAN#108 | Core part |
| TS 38.211 | NR; Physical channels and modulation | RAN#108 | Core part. Note: whether a new specification in 38 series will be used instead will be decided at RAN#107 |
| TS 38.212 | NR; Multiplexing and channel coding | RAN#108 | Core part Note: whether a new specification in 38 series will be used instead will be decided at RAN#107 |
| TS 38.213 | NR; Physical layer procedures for control | RAN#108 | Core part Note: whether a new specification in 38 series will be used instead will be decided at RAN#107 |
| TS 38.214 | NR; Physical layer procedures for data | RAN#108 | Core part Note: whether a new specification in 38 series will be used instead will be decided at RAN#107 |
| TS 38.300 | NR; NR and NG-RAN Overall description; Stage-2 | RAN#109 | Core part |
| TS 38.321 | NR; Medium Access Control (MAC) protocol specification | RAN#109 | Core part Note: whether a new specification in 38 series will be used instead will be decided at RAN#107 |
| TS 38.401 | NG-RAN; Architecture description | RAN#109 | Core part |
| TS 38.413 | NG-RAN; NG Application Protocol (NGAP) | RAN#109 | Core part |
| TS 38.410 | NG-RAN; NG general aspects and principles | RAN#109 | Core part |
| TR 38.870 | OTA test method | RAN#109 | Core part |
| TR 38.870 | OTA method for performance requirements | RAN#111 | Performance part |

## RAN1

Radios with differentiation from the fundamental LTE or NR air interfaces have previously been captured in existing specifications: most notably NB-IoT, sidelink for LTE and NR, and LTE-MTC. For NB-IoT and SL, in RAN1, they are held in separated clauses of the physical layer specifications. LTE-MTC is variably embedded at sentence-level (mainly in TS 36.213) or clause-level (mainly in TS 36.211 and 36.212).

Implementation and maintenance of all those differentiated radios from within LTE and NR specifications has proven to be feasible. However, maintenance and enhancements of LTE-MTC in RAN1 was a more complicated operation due to the sentence-level changes that were needed often having potential unintended impacts to legacy text and non-MTC UEs.

We assume that, if existing specifications are used for A-IoT, it should be held primarily in separated sections of 38-series NR specs. This would be a matter for specification editors to decide on in the first place. There would be some cross-linking to existing sections, especially in R2D, due to the dependency on OFDM symbol alignment, CP handling, etc.

Maintenance and enhancement of A-IoT in existing specifications would be per the usual procedures, and would sometimes need agreement on which specification should handle/change for a particular issue. Views on this can be controversial and time-consuming in some cases. In the initial stages of each Release, drafting would benefit from the experience accumulated in the NR specification editors.

Alternatively, new specification(s) could be created to hold only A-IoT. It might make lighter work for an implementer only interested in these A-IoT devices or readers to not have to extract the information from NR specifications. There would still be a need to understand certain elements of the NR specifications due to the related quantities and concepts, at least in R2D, where cross-references or copy-pastes would be needed, and already some of the existing specs (38.201, 38.202) will be used.

Maintenance and enhancement in RAN1 might be easier, if a single new specification is introduced, as it would avoid overlapping or conflicting changes and there would be no debate about the proper place to capture a given aspect. Just one specification editor would be able to provide an overall consistent approach to expression, maintenance and enhancement, within and across Releases. If those simplifications are beneficial, they would not apply if multiple new RAN1 specifications are created. The overhead of that route seems rather high, and although it is feasible in principle, its benefit over re-using existing specifications is diminished.

Whatever is chosen in Rel-19 should be continued for Rel-20, which will introduce new devices, new use cases, and Topology 2 with an intermediate UE as reader. Since the intermediate UE is also an A-IoT reader like the Rel-19 BS, if there are new RAN1 specifications from Rel-19, it should be possible to use them for the Rel-20 UE also, although the amount of dependency on NR specifications might be higher. The Uu link from intermediate UE to BS we assume would still be in NR specifications for convenience.

Observation 1: On using existing NR specifications for RAN1:

- Other differentiated radios, such as NB-IoT, LTE-MTC, and sidelink have seen successful specification, implementation, maintenance, and enhancement in LTE/NR specifications.

- We assume that, if existing RAN1 NR specifications are used for A-IoT, it should be held primarily in separated sections of those specifications, subject to editorial discretion.

- Maintenance and enhancement work might, as usual, sometimes experience controversy over which specification should handle/change for a particular issue.

Observation 2: On using new 38-series specifications for RAN1:

- It may provide faster understanding if an implementer is interested only in A-IoT.

- To external organizations, the visibility of A-IoT technology could be higher.

- Initial specification, maintenance and future enhancement in RAN1 might be easier, if a single new specification is introduced.

- However, if multiple new specifications are introduced, any benefit of simplicity or separation from NR specifications seems to be diminished.

Observation 3: There will be some dependency on existing NR specifications regardless of the overall arrangement, particularly in R2D.

Observation 4: The specification method chosen in Rel-19 would be continued in Rel-20. Uu aspects of the intermediate UE towards the BS would likely be contained in NR specifications regardless of the A-IoT interface’s arrangement.

Proposal 1: Either existing or new 38-series RAN1 specifications are deemed suitable for A-IoT. If existing specifications are not used, there should preferably be just one single new TS in RAN1.

- If existing NR specifications are used, it is preferred that editors primarily arrange A-IoT in separated sections, similar to NB-IoT and sidelink.

## MAC

Similar considerations exist for RAN2 as RAN1, with the simplification that there is only one specification needed (apart from TS 38.300) in Rel-19. Traditionally, all technologies have been contained in a single MAC specification, including in the cases where other specifications have separated them, e.g. NB-IoT, LTE-MTC, and sidelink are all in TS 36.321 or 38.321, arranged mainly by if-conditions or clause-level separation, with some sentence-level containment also used. At times, this has produced some hard-to-manage specification, needing more maintenance than might be desirable. With experience, the specification eventually settles down.

Proposal 2: Either existing TS 38.321 or a new 38-series MAC specification is deemed suitable for A-IoT. Experience suggests spec management may be slightly lighter if a new TS is used for the A-IoT MAC.

## RAN4

For carrier-wave RF requirement, the WID contains:

“Note: which specification will contain the RF requirements for CW is to be further discussed at RAN#107 based on RAN4 feedback.”

RAN4#114 decided as follows:

Issue 2-2-1: CW spec Agreement: - Add CW transmission in the new specification title: “TS 38.xxx Ambient IoT Base Station (BS) and Carrier-Wave (CW) node radio transmission and reception”

On OTA test methods, RAN4#114 decided as follows, and hence the performance part impact to TR 38.870 needs to be removed.

Issue 4-1-2: Measurement campaign Measurement campaign is not possible in this work item. Refinement in future work items based on measurement campaign is not precluded.

For the core part, per WID, the discussion is focusing on OTA test methodology instead of specific TRP/TRS requirements enhancement. It is also better that the output of OTA test method for A-IoT device is captured in the new A-IoT device requirements TS.

Proposal 3: For RAN4, update the specifications tables in the WID:

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |
| New TS | TS 38.xxx | Requirements for Ambient IoT device |  | RAN#109 | RAN4 Core part Note: including RF requirements and RRM requirements (if any) and device OTA test methods |
| New TS | TS 38.xxx | Ambient IoT Base Station (BS) and Carrier-Wave (CW) node radio transmission and reception |  | RAN#109 | RAN4 Core part |
| New TS | TS 38.xxx | Requirements for Ambient IoT device |  | RAN#111 | RAN4 Performance part Note: including RF requirements, demodulation and RRM requirements (if any) |
| New TS | TS 38.xxx | Ambient IoT Base Station (BS) and Carrier-Wave (CW) node radio transmission and reception |  | RAN#111 | RAN4 Performance part |
| New TS | TS 38.xxx | Ambient IoT Base Station (BS) and Carrier-Wave (CW) node conformance testing |  | RAN#111 | RAN4 Performance part |

| Impacted existing TS/TR |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| … | … | … | … |
| TR 38.870 | OTA test method | RAN#109 | Core part |
| TR 38.870 | OTA method for performance requirements | RAN#111 | Performance part |

# FEC

Regarding forward error-correcting codes (FECs), the WID says [1]:

The following objectives are set, within the General Scope: - RAN1 scope: … - R2D does not support FEC. D2R supports only convolutional code with generator polynomials as per TS 36.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis).

It was asserted by a small number of companies that not applying D2R FEC in some circumstances is not permitted by the WID. We regard this assertion as obviously unrealistic, due for example to cases where a deployment can know by design that the reader is always close to the devices, and the devices need to be rapidly inventoried, such as when passing through a factory gate. Peak data rate and hence inventory rate are important metrics to consider in any system design.

Hence, we support and co-source the discussion and proposals in [3], and refer to that paper for details.

Proposal 4: Confirm that D2R is not required to always use a FEC.

Figure 1: Example of the device located near the reader corresponds to high data rate, in a fixed deployment.

# Summary

We have discussed points to consider in deciding the remaining questions on new or existing RAN specifications for A-IoT in Rel-19, summarized as follows, together with some administrative updates for RAN4 specifications.

We have also co-sourced [3, RP-250269] to confirm that D2R is not required to always use a FEC, in order to access high peak inventory rates in applicable circumstances.

Proposal 1: Either existing or new 38-series RAN1 specifications are deemed suitable for A-IoT. If existing specifications are not used, there should preferably be just one single new TS in RAN1.

- If existing NR specifications are used, it is preferred that editors primarily arrange A-IoT in separated sections, similar to NB-IoT and sidelink.

Proposal 2: Either existing TS 38.321 or a new 38-series MAC specification is deemed suitable for A-IoT. Experience suggests spec management may be slightly lighter if a new TS is used for the A-IoT MAC.

Proposal 3: For RAN4, update the specifications tables in the WID:

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |
| New TS | TS 38.xxx | Requirements for Ambient IoT device |  | RAN#109 | RAN4 Core part Note: including RF requirements and RRM requirements (if any) and device OTA test methods |
| New TS | TS 38.xxx | Ambient IoT Base Station (BS) and Carrier-Wave (CW) node radio transmission and reception |  | RAN#109 | RAN4 Core part |
| New TS | TS 38.xxx | Requirements for Ambient IoT device |  | RAN#111 | RAN4 Performance part Note: including RF requirements, demodulation and RRM requirements (if any) |
| New TS | TS 38.xxx | Ambient IoT Base Station (BS) and Carrier-Wave (CW) node radio transmission and reception |  | RAN#111 | RAN4 Performance part |
| New TS | TS 38.xxx | Ambient IoT Base Station (BS) and Carrier-Wave (CW) node conformance testing |  | RAN#111 | RAN4 Performance part |

| Impacted existing TS/TR |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| … | … | … | … |
| TR 38.870 | OTA test method | RAN#109 | Core part |
| TR 38.870 | OTA method for performance requirements | RAN#111 | Performance part |

Proposal 4: Confirm that D2R is not required to always use a FEC.

Observation 1: On using existing NR specifications for RAN1:

- Other differentiated radios, such as NB-IoT, LTE-MTC, and sidelink have seen successful specification, implementation, maintenance, and enhancement in LTE/NR specifications.

- We assume that, if existing RAN1 NR specifications are used for A-IoT, it should be held primarily in separated sections of those specifications, subject to editorial discretion.

- Maintenance and enhancement work might, as usual, sometimes experience controversy over which specification should handle/change for a particular issue.

Observation 2: On using new 38-series specifications for RAN1:

- It may provide faster understanding if an implementer is interested only in A-IoT.

- To external organizations, the visibility of A-IoT technology could be higher.

- Initial specification, maintenance and future enhancement in RAN1 might be easier, if a single new specification is introduced.

- However, if multiple new specifications are introduced, any benefit of simplicity or separation from NR specifications seems to be diminished.

Observation 3: There will be some dependency on existing NR specifications regardless of the overall arrangement, particularly in R2D.

Observation 4: The specification method chosen in Rel-19 would be continued in Rel-20. Uu aspects of the intermediate UE towards the BS would likely be contained in NR specifications regardless of the A-IoT interface’s arrangement.

# References

- RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN#106, December 2024.

- v08, Chair notes RAN#120 (9.4 Ambient IoT). https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_120/Inbox/David_sessions/R1-250xxxx%20Chair%20notes%20RAN1%23120%20(9.4%20R19%20Ambient%20IoT)%20v08.zip.

- RP-250269, “Clarification on the scope of Ambient loT WID”, CMCC, Huawei, Spreadtrum, RAN#107, March 2025.