3GPP TSG RAN Meeting #107								   RP-250478

Incheon, Korea, March 12-14, 2025

Agenda Item:		9.2.4

Source:		         Mediatek Inc.

Title:				On AI/ML Mobility: RLF prediction

Document for:		Discussion

Introduction

The following RAN4 related objective is captured in the SID on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR [1]:

The study will focus on mobility enhancement in RRC_CONNECTED mode over air interface by following existing mobility framework, i.e., handover decision is always made in network side. Mobility use cases focus on standalone NR PCell change. UE-side and network-side AI/ML model can be both considered, respectively. Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects: - AI/ML based RRM measurement and event prediction, - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2] - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2] - HO failure/RLF prediction (UE sided model) [RAN2] - Measurement events prediction (UE sided model) [RAN2] - Study the need/benefits of any other UE assistance information for the network side model [RAN2] - The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2] - NOTE: Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2 - Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2] - NOTE: This would only be treated after sufficient progress is made in the Rel-19 AI/ML air interface WID - Potential specification impacts of AI/ML aided mobility [RAN2] - Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation - NOTE 1: RAN1/3 work can be triggered via LS NOTE 2: To avoid duplicate study with “AI/ML for NG-RAN” led by RAN3 NOTE 3: Two-sided model is not included

In this paper, we consider progress and status of the study and discuss whether to remove the objective related to HO failure/RLF prediction (UE-side model).

Discussion

During the RAN2#126 meeting[2], it was agreed by RAN2 to deprioritize the use case of HOF. As a result, there will be no further simulations, evaluations, or discussions on this topic.

Agreements

1	Study Indirect: RLF prediction based on the temporal domain serving cell measurement predictions (e.g. SINR).

2	Study Direct: Directly RLF prediction by AI/ML models.

3	FR2 study will be prioritized for RLF prediction.

4	The study should focus on RLF due to T310 expiry (i.e. in-synch/out-of-synch case) as the representative RLF case for direct and indirect prediction.

5	HOF prediction is downprioritized in our study.  NO simulations/evaluations should be done/submitted

6	RLF prediction result is the RLF probability within a time window or at time instance, at least for direct case.  FFS on expected RLF time and indirect case.

7	No evaluation/simulations are expected for August meeting for RLF

8	Simulation assumption specific to RLF will be discussed in August. The assumption is that we will reuse RRM simulation assumptions (where possible).

Observation 1: Since the RAN2#126 meeting, RAN2 has deprioritized the HOF use case, with no simulations or evaluations being conducted on it.

During the RAN2#129 meeting[3], it was agreed by RAN2 that RLF prediction will not be studied in Rel-19.

- Agreements on priority RLF prediction will not be studied in Rel-19 Further RRM prediction simulations are not expected. For next specification general impact study phase we will consider spatial, temporal, and frenquency domain cell level prediction. FFS if L3 beam level needs to be considered. For now, temporal and frequency domain prediction will be considered with higher priority for detailed study (if needed). Continue with generalization across cell configurations for RRM prediction Continue with measurement events

The primary reason for discontinuing further research on RLF prediction is the difficulty in evaluating the benefits of AI under the current simulation assumptions. In the existing simulator, which is based on a homogeneous network deployment, the main cause of RLF is interference from neighboring cells. In conventional system-level simulations (SLS) without AI, it is feasible to model this interference and assess system performance. However, justifying the benefits of AI is challenging because the UE side cannot accurately predict RLF due to interference. This is because interference from neighboring cells is heavily dependent on network scheduling and resource allocation, which are unknown to the UE.

Moreover, implementing realistic interference in the simulator is both time-consuming and difficult to calibrate. Running the interference model imposes a significant burden on the simulator and consumes a substantial amount of simulation time. Given the remaining time in Rel-19, it is impractical to conduct and finalize the evaluation of RLF prediction.

Observation 2: RAN2 agreed to cease the study the use case of RLF in Rel-19 due to the challenge to evaluate the RLF prediction performance and time limitation.

In real networks, the primary causes of RLF are sudden channel degradation or the UE moving into areas with deep shadowing. However, replicating such abnormal cases in current simulations, which use stochastic channel models, is challenging. Consequently, models trained for RLF prediction based on the current simulator are unlikely to provide accurate predictions due to the inherent randomness of stochastic channel models. To replicate failure cases in the simulator more accurately, one potential approach is to use deterministic channel models (e.g., ray-traced model) or field data for model training and inference.

Some companies have attempted to use field data to evaluate RLF prediction and have demonstrated its benefits. However, this approach is not widely feasible because field data is often proprietary and cannot be shared among companies now.

Observation 3: A viable method to evaluate RLF prediction performance is to utilize deterministic channel models (e.g., ray-traced models) or field data.

Considering current situation and difficulties to evaluate the performance of RLF prediction, it would be beneficial to remove this objective from the SID with following reasons:

- Improve the accuracy of project completion assessment.

- Reduce the pressure from incomplete tasks: allow the RAN2 to focus more on core tasks.

- Avoiding confusion on RAN4 work: RAN4 will also not study it in Rel-19.

There already has been some description of RLF prediction in current draft TR. We keep either keep the content or remove it. If we keep the content, we can capture in the TR that the study and evaluation on RLF prediction is not completed due to time limitation in Rel-19.

Proposal: Remove the objective of HO failure/RLF prediction (UE sided model) [RAN2] from Rel-19 Study on AI/ML for Mobility.

Conclusion

Observations:

Observation 1: Since the RAN2#126 meeting, RAN2 has deprioritized the HOF use case, with no simulations or evaluations being conducted on it.

Observation 2: RAN2 agreed to cease the study the use case of RLF in Rel-19 due to the challenge to evaluate the RLF prediction performance and time limitation.

Observation 3: A viable method to evaluate RLF prediction performance is to utilize deterministic channel models (e.g., ray-traced models) or field data.

Proposal: Remove the objective of HO failure/RLF prediction (UE sided model) [RAN2] from Rel-19 Study on AI/ML for Mobility.

Reference

[1] RP-242393, Revised SID on AIML for mobility in NR, OPPO

[2] R2-2406202, Report of 3GPP TSG RAN WG2 meeting #126, Fukuoka, Japan

[3] R2_129_ChairNotes_25-02-21_13-00