3GPP TSG RAN Meeting #107						                                         RP-250487

Incheon, Korea, March 12-14, 2025

## Status Report to TSG

Agenda item:			9.4.1.4

| WI / SI Name | # Introduction of another NR-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_NTN_Sband |  |  |  |  |
| Unique ID | 1040129 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-241885 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: NA | Core part: 6/2025 | Performance part: 6/2025 | Testing part: NA |  |
| Overall Completion level | Study Item: NA | Core part: 70% | Performance Part: 0% | Testing part: NA |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Laehteensuo, Toni |
|  | Company | Qualcomm Incorporated |
|  | Email | tlaehtee@qti.qualcomm.com |

# 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

# 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

## 2.1	RAN1

### 2.1.1	Agreements

### 2.1.2	Remaining Open issues

## 2.2	RAN2

### 2.2.1	Agreements

### 2.2.2	Remaining Open issues

## 2.3	RAN3

### 2.3.1	Agreements

### 2.3.2	Remaining Open issues

## 2.4	RAN4

### 2.4.1	Agreements

#### 2.4.1.1	RAN4 #114

#### UE to UE Coexistence with band n2/n25

Agreement:

- Agree to two different additional spurious emission requirements, each associated to an NS value, and corresponding A-MPR, for protection of band b25/n25/b2/n2

  - -30 dBm/MHz (NS_XX)

  - -40 dBm/MHz (NS_YY)

- For the above requirements, the following statement will be captured in a normative way:

  - “The requirement applies based on coordination between operators and subject to regional/national regulation”

    - Apply the requirements to UE based on NS signal

- Agree to specify asymmetric channel bandwidths with 5, 10 and 15 MHz UL CHBW for band n252, with BCS0 including all permutation based on UL channel bandwidth

Table 5.3.6-1: FDD asymmetric UL and DL channel bandwidth combinations

| NR Band | Channel bandwidths for UL (MHz) | Channel bandwidths for DL (MHz) | Asymmetric channel bandwidth combination set |
| --- | --- | --- | --- |
| n252 | 5 | 10,15, 20 | 0 |
|  | 10 | 15, 20 |  |
|  | 15 | 20 |  |

#### UE NS For Additional Spurious emissions

6.5.3.3.X	Requirement for network signalling value "NS_XX"

When "NS_XX" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.2-X. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.2-X: Additional requirements for "NS_XX"

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz, 20MHz |  |  |
| 1930 ≤ f <1995 | -30 | 1MHz | XX |

Agreement:

- the note will be moved to NS mapping table, including the contents below

  - “The requirement applies based on coordination between operators and subject to regional/national regulation”

  - Apply the requirements to UE based on NS signal

6.5.3.3.Y	Requirement for network signalling value "NS_YY"

When "NS_YY" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.2-Y. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.2-Y: Additional requirements for "NS_YY"

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz, 20MHz |  |  |
| 1930 ≤ f < 1995 | -40 | 1MHz | YY |

Agreement:

- the note will be moved to NS mapping table, including the contents below

  - “The requirement applies based on coordination between operators and subject to regional/national regulation”

  - Apply the requirements to UE based on NS signal

#### UE A-MPR for corresponding NS values

- Agreement:

  - Specify different A-MPR values for each NS depending on the UL CHBW, RB start and LcRB.

    - FFS on exact values

  - Adopt the following table skeleton

Table 6.2.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR satellite Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_XX | 6.5.3.3.X | n252 | 5, 10, 15, 20 |  | Clause 6.2.3.X |
| NS_YY | 6.5.3.3.Y | n252 | 5, 10, 15, 20 |  | Clause 6.2.3.Y |
| NOTE 4: The NS is signalled based on coordination between operators and subject to regional/national regulation |  |  |  |  |  |

6.2.3.2	A-MPR for NS_XX

Table 6.2.3.X-X: A-MPR regions for NS_XX

| UL Channel BW | Carrier Center Frequency | RB_start*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5MHz |  |  |  |  |
| 10MHz |  |  |  |  |
| 15MHz |  |  |  |  |
| 20MHz |  |  |  |  |

Table 6.2.3.X-X: A-MPR for NS_XX

|  | Modulation | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK |  |  |  |  |  |  |
|  | QPSK |  |  |  |  |  |  |
|  | 16QAM |  |  |  |  |  |  |
|  | 64QAM |  |  |  |  |  |  |
|  | 256QAM |  |  |  |  |  |  |
| CP-OFDM | QPSK |  |  |  |  |  |  |
|  | 16QAM |  |  |  |  |  |  |
|  | 64QAM |  |  |  |  |  |  |
|  | 256QAM |  |  |  |  |  |  |

#### Protection of aeronautical radionavigation-satellite service (i.e. GNSS)

- Agreement:

  - Specify the following additional spurious requirements for protection of the aeronautical radionavigation satellite service:

Additional spurious emission requirements for band n252

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz, 20 MHz |  |  |
| 1559≤ f < 1610 | -50 | 700 Hz | Averaged over any 2 millisecond active transmission interval |
| 1559 ≤ f < 1610 | -40 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0 dBi antenna. |  |  |  |

#### UE Asymmetric Channel Bandwidths

- Agreement:

  - Specify Asymmetric channel bandwidths as follows (see Draft CR)

Table 5.3.6-1: FDD asymmetric UL and DL channel bandwidth combinations

| NR Band | Channel bandwidths for UL (MHz) | Channel bandwidths for DL (MHz) | Asymmetric channel bandwidth combination set |
| --- | --- | --- | --- |
| n252 | 5 | 10,15, 20 | 0 |
|  | 10 | 15, 20 |  |
|  | 15 | 20 |  |
| NOTE 1: The assignment of the paired UL and DL channels are subject to a TX-RX separation as specified in clause 5.4.4. NOTE 2: As indicated in TS38.306 [11], it is mandatory for UEs to support asymmetric channel BCS0 if there is an asymmetric BCS0 defined for the band. |  |  |  |

#### UE REFSENS

- Agreement:

  - Specify n252 UE REFSENS based on

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
|  | 15 | [-99.5] | [-96.3] | [-94.5] | [-93.3] |
| n252 | 30 |  | [-96.6] | [-94.6] | [-93.5] |
|  | 60 |  | [-97.0] | [-94.9] | [-93.7] |

#### UE Out-of-band Blocking

- Agreement:

  - Further discuss the Out-of-Band Blocking requirements for band n252 based on the options following

    - Option 1, with no co-banding assumption with n256 and n65

Table 2.2-1: Out of-band blocking for NR satellite bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| n252 | Finterferer (CW) | MHz | -70 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -95 < f – FDL_low ≤ -70 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 95 or FDL_high + 85 ≤ f ≤ 12750 |
| n2542 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| n255 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| n2561 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band n256 lower frequency ranges are modified to enable specific implementations NOTE 2: Band n254 power level of the interferer (Pinterferer) for Range 3 shall be modified to -20 dBm for Finterferer > 2585 MHz and FInterferer < 2775 MHz. NOTE 3: void NOTE 4: void |  |  |  |  |  |

  - Option 2 with assumption of n252 co-banding with n256 and n65

Table 1: Out of-band blocking for n252

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| n2521 | Finterferer (CW) | MHz | -110 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -155 < f – FDL_low ≤ -110 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 155 or FDL_high + 85 ≤ f ≤ 12750 |
| n2561 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band n256 and n252 lower frequency ranges are modified to enable specific implementations |  |  |  |  |  |

  - Option3: reuse the OOB requirements of band B23

## 2.5	RAN5

### 2.5.1	Agreements

### 2.5.2	Remaining Open issues

### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

### 2.6.1	Agreements

### 2.6.2	Remaining Open issues

# 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

## 3.1	SAx/CTs

### 3.1.1	Agreements with cross-TSG impacts

### 3.1.2	Remaining Open issues with cross-TSG impacts

# 4.	References

## RAN4#114

- R4-2500291 	Further input on UE power back-off while operating in 2000-2020 MHz UL range of the NTN band n252	Apple

- R4-2500430	On NTN S-Band UE Coexistence Requirement, Verizon, T-Mobile USA	AT&T

- R4-2500602	Discussion on system parameters and UE RF requirements for NR-NTN S-band	ZTE Corporation, Sanechips

- R4-2501001	Discussion on SAN RF requirements for the NTN new S-band	CATT

- R4-2501002	Discussion on UE RF requirements for the NTN new S-band	CATT

- R4-2501118	Way Forward on n252/band 252 co-existence	EchoStar, Boost Mobile Network

- R4-2501179	Draft CR to TS 38.133 Introduction of a new NTN S-band	ZTECorporation,Sanechips

- R4-2501321	Discussion on NR-NTN n252 UE RF requirements	MediaTek (Hefei) Inc.

- R4-2501330	Discussion on UE-to-UE coexistence of n252 UL interference with n2 DL and n25 DL	MediaTek (Hefei) Inc.

- R4-2501455	Discussion on the NTN S band definition	Huawei, HiSilicon

- R4-2501706	Discussion on NR-NTN n252 UE RF requirements	Tejas Network Limited

- R4-2502005	UE-to-UE coexistence and OOB blocking for n252	Qualcomm Inc.

- R4-2502162	Draft CR to TR 38.863 to capture agreement for n252 WI 	EchoStar, Boost Mobile Network

- R4-2502202	NTN S-Band UE-UE coexistence	T-Mobile USA, Verizon

- R4-2503078	Discussion on NR-NTN n252 UE RF requirements	Tejas Network Limited