3GPP TSG RAN meeting #107										RP-250490

Incheon, Korea, March 12 – 14, 2025

## Status Report to TSG

Agenda item:			9.3.1.2

| WI / SI Name | Evolution of NR duplex operation: Sub-band full duplex (SBFD) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_duplex_evo |  |  |  |  |
| Unique ID | 1020090 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-241614 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: n/a | Core part: 09/2025 | Performance part: 03/2026 | Testing part: n/a |  |
| Overall Completion level | Study Item: n/a | Core part: 68% | Performance Part: 0% | Testing part: n/a |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Xinghua Song, He (Jackson) Wang |
|  | Company | Huawei, Samsung |
|  | Email | songxinghua@huawei.com, h0809.wang@samsung.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

In RAN1#118bis, the following agreements were made.

SBFD TX/RX/measurement procedures

Agreement

For aperiodic SRS with available slot counting,

- For SRS-ResourceSet configured for SBFD symbol, an available slot is a slot satisfying there are SBFD symbol(s) for the time-domain location(s) for all the SRS resources in the resource set and it satisfies UE capability on the minimum timing requirement between triggering PDCCH and all the SRS resources in the resource set.

- For SRS-ResourceSet configured for non-SBFD symbol, an available slot is a slot satisfying there are UL or flexible symbol(s) not configured as SBFD symbols for the time-domain location(s) for all the SRS resources in the resource set and it satisfies UE capability on the minimum timing requirement between triggering PDCCH and all the SRS resources in the resource set.

Agreement

The configuration of Configuration 1 or 2 for DL BWP only applies to PDSCH receptions within the DL BWP.

Agreement

For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation is configured in CSI-ReportConfig.

Agreement

For Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only,

- The valid symbol type for PUSCH carrying SP-CSI is determined based on the symbol type of the first PUSCH occasion associated with activation DCI.

- For SP-CSI on PUCCH:

  - Option 2: the valid symbol type for PUCCH carrying SP-CSI is configured in semiPersistentOnPUCCH in CSI-ReportConfig.

Conclusion

- There is no RAN1 consensus to support semi-static link direction indication for SBFD aware UEs.

- For a serving cell configured with SBFD subband time and frequency location, for an SBFD aware UE, TDD-UL-DL-ConfigDedicated is only applicable to non-SBFD symbols but not applicable to SBFD symbols

Agreement

For determining starting PRB for PUSCH transmissions in SBFD symbols for Configuration 2,

- For type 1 CG PUSCH, a RBoffsetSBFD is configured in rrc-ConfiguredUplinkGrant in ConfiguredGrantConfig.

- For type 2 CG PUSCH and DG PUSCH, a RBoffsetSBFD is configured per UL BWP in PUSCH-Config.

  - The RBoffsetSBFD is commonly applicable to all DCI formats which schedule PUSCH

- If RBoffsetSBFD is not configured, it is assumed to be zero

Agreement

When higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2', the numbers of SRS ports should be the same for SRS resources with the same corresponding SRI values in the two SRS resource sets associated with SBFD and non-SBFD symbols.

Agreement

For Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only,

- The valid symbol type for Type 1 CG PUSCH is configured in rrc-ConfiguredUplinkGrant in ConfiguredGrantConfig.

- The valid symbol type for PUCCH configured for SR is configured in SchedulingRequestResourceConfig.

- The valid symbol type for PUCCH carrying P-CSI is configured in periodic in CSI-ReportConfig.

Agreement

For a type 1 CG PUSCH with Configuration 2, if FH offset for SBFD symbols is not configured for the type 1 CG PUSCH, frequency hopping is disabled for the type 1 CG PUSCH transmissions in SBFD symbols.

For a type 2 CG PUSCH/PUSCH scheduled by DCI, if FH offset lists for SBFD symbols are not configured for the type 2 CG PUSCH/PUSCH scheduled by DCI, frequency hopping is disabled for the transmission of the type 2 CG PUSCH/PUSCH scheduled by DCI in SBFD symbols.

Agreement

For SPS PDSCH with Configuration 2, for TBS determination,

- For SPS PDSCH without repetition,

  - The number of PRBs for TBS determination for an SPS PDSCH in SBFD symbols is based on assigned PRBs within DL usable PRBs only.

  - The number of PRBs for TBS determination for an SPS PDSCH in non-SBFD symbols is based on assigned PRBs.

- For SPS PDSCH with repetition, TBS is determined based on the first repetition occasion.

  - If the first repetition occasion of a SPS PDSCH is in SBFD symbols, the number of PRBs for TBS determination for the SPS PDSCH is based on assigned PRBs within DL usable PRBs only.

  - If the first repetition occasion of a SPS PDSCH is in non-SBFD symbols, the number of PRBs for TBS determination for the SPS PDSCH is based on assigned PRBs.

Agreement

If starting PRB is not configured for SBFD symbols for a PUCCH-Resource, starting PRB configured for non-SBFD symbols for the PUCCH-Resource is used for PUCCH transmissions in SBFD symbols associated with this pucch-ResourceId.

Agreement

The one configuration of intraSlotFrequencyHopping for a PUCCH-Resource is applied to both PUCCH transmissions in SBFD symbols and in non-SBFD symbols associated with this pucch-ResourceId.

Agreement

Support SBFD operation on one TDD carrier in multi-carrier scenario.

- Note: If the TDD carrier for SBFD operation is an Scell, support UE dedicated signaling for cell-specific configuration of time and frequency location of SBFD subbands for the Scell.

- FFS whether/how to handle half-duplex CA case

SBFD random access operation

Conclusion

There is no RAN1 consensus on the following proposal:

For RACH configuration Option 1, for support of separate power control parameters for PRACH transmission in additional-ROs and legacy-ROs,

- also support separate powerRampingStep and powerRampingStepHighPriority for additional-ROs.

Conclusion

For RACH configuration Option 2, for the case that additional ROs overlap with legacy ROs, support Alt-1:

- Alt-1: do not optimize for this case

Conclusion

For RACH configuration Option 2, and for interpretation of the parameter prach-ConfigurationIndex provided by the additional RACH configuration:

- For FR1 support Alt-4: no enhancement

- For FR2 support Alt-4: no enhancement

Agreement

For determination of the Msg3 PUSCH transmission power and when separate preambleReceivedTargetPower for additional-ROs is configured for RACH configuration Option 1

- preambleReceivedTargetPower configured for legacy-RO is used if Msg3 PUSCH is transmitted in non-SBFD symbols;

- preambleReceivedTargetPower configured for additional-RO is used if Msg3 PUSCH is transmitted in SBFD symbols.

- FFS: Option 2

Conclusion

For SBFD-aware UEs, there is no restriction on the combination of RO type of the latest PRACH transmission and the symbol type of following Msg3 PUSCH transmission or PUCCH carrying HARQ ACK for Msg4.

Agreement

Consider the following alternatives about power ramping when a SBFD-aware UE switches from additional-ROs to legacy-ROs and from legacy-ROs to additional-ROs (if supported) in PRACH transmission re-attempt in one RACH procedure for RACH configuration Option 1 and Option 2.

- Alt-1: Increase the power ramping counter.

  - FFS whether to introduce a power offset to compensate the power ramping difference for RACH configuration Option 2.

- Alt-2: Reset the power ramping.

- Note: different alternatives can be applied for different RACH configuration options.

Agreement

For an SBFD aware UE, if additional-RO is selected for PRACH transmission, for Msg3 PUSCH repetition Configuration 1,

- In case the valid symbol type is SBFD symbol, a slot is determined as available if the symbols allocated for Msg3 PUSCH are all SBFD symbols and not include a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

- In case the valid symbol type is non-SBFD symbol, a slot is determined as available if the symbols allocated for Msg3 PUSCH are all non-SBFD symbols and not include a DL symbol indicated by tdd-UL-DL-ConfigurationCommon if provided or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

Agreement

For RACH configuration Option 1, when separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 for PRACH transmission with preamble repetitions within additional-ROs is not configured, the rsrp-ThresholdMsg1-RepetitionNum2/4/8 configured for PRACH transmission with preamble repetitions within legacy-ROs is reused for additional-ROs.

Conclusion

There is no consensus in RAN1 to support Type-2 random access procedure (2-step RACH) in SBFD symbols for SBFD-aware UEs.

Agreement

For RACH configuration Option 1, down-select from the following two alternatives:

- Alt-1: Shared ssb-SharedRO-MaskIndex configuration for Additional-ROs and Legacy-ROs.

- Alt-2: Separate ssb-SharedRO-MaskIndex configurations for Additional-ROs and Legacy-ROs.

CLI handling

Agreement

For the semi-statically configured time location and frequency location of UL muting symbol(s) for PUSCH,

- Separate configurations of up to two UL muting symbols can be provided per UE for DG PUSCH/Type 2 CG PUSCH and Type 1 CG PUSCH

  - Separate configuration for each of multiple Type 1 CG PUSCH configurations is supported

Agreement

For PUSCH that is scheduled by DCI format 0_0 or Type 2 CG PUSCH activated by DCI format 0_0, UL resource muting is not applied.

Agreement

Confirm the following working assumption

Working assumption

For the frequency location configuration of UL resource muting for a PUSCH, for DFT-S-OFDM

- A comb offset {0, 1} is configured for the up to two UL muting symbols

Agreement

For the configuration of UL resource muting

- Alt.1: A new IE PUSCH-MutingResources is introduced to define the time location and frequency location of UL muting resources. The UL muting pattern configures up to two symbols within a slot and a configurable comb offset.

Agreement

For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to ‘cli-RSSI’ or ‘cli-SRS-RSRP’

- carrier in CSI-ReportConfig is used to indicate in which serving cell the CLI-RSSI measurement resources or SRS-RSRP measurement resources in CSI-ResourceConfig are to be found. If the field is absent, the resources are on the same serving cell as this report configuration.

- bwp-Id in the associated CSI-ResourceConfig is used to indicate the DL BWP where the CLI-RSSI measurement resources or SRS-RSRP measurement resources are located in.

Note: No RAN1 specification impact

Agreement

A new parameter ul-MutingIndicator is introduced to PUSCH-Allocation-r16 to indicate whether the semi-statically configured UL muting symbol(s) is enabled.

Agreement

The following agreement replaces the earlier agreement from RAN1#119

If transform precoding is enabled for a PUSCH, the following options are provided for the purpose of specification drafting (how UE implementation is done is left to each company).

- Option 1: The DFT size is modified as MscPUSCH2 for symbol(s) with UL resource muting

  - The MscPUSCH2 modulated symbols are transformed to the frequency domain with a DFT of size MscPUSCH2, and the result is mapped to the MscPUSCH2 unmuted scheduled subcarriers.

- Option 2: The DFT size is MscPUSCH for symbol(s) with UL resource muting

  - Option 2A: The block of MscPUSCH2 modulated symbols is repeated before DFT. After DFT, the MscPUSCH2 non-zero even-indexed subcarriers are extracted and then mapped to the unmuted scheduled subcarriers.

  - Option 2B: The block of MscPUSCH2 modulated symbols is extended to  MscPUSCH modulated symbols by inserting ‘0’ after every modulated symbol. After DFT, the first MscPUSCH2 subcarriers are extracted and then mapped to the unmuted scheduled subcarriers.

Note: MscPUSCH is the total number of subcarriers of the PUSCH.

Note: Above options are mathematically equivalent.

Note: It is up to the spec editor how to capture the above as long as the specification captures the above in a mathematically identical manner.

Agreement

For the PUSCH symbol containing PT-RS with UL resource muting when transform precoding is not enabled, 3 dB power boosting for both PUSCH and PTRS REs if present, regardless of the PTRS density K and number of PUSCH layers L.

Agreement

- UE determines the subcarrier spacing of the CLI-RSSI measurement resources configured in a CSI-ResourceConfig based on the subcarrier spacing of the DL BWP identified by the higher-layer parameter bwp-Id in the CSI-ResourceConfig.

- UE determines the subcarrier spacing of the SRS-RSRP measurement resources configured in a CSI-ResourceConfig based on the subcarrier spacing of the DL BWP identified by the higher-layer parameter bwp-Id in the CSI-ResourceConfig.

- Note: No RAN1 spec impact is expected.

Agreement

Update the following agreement in RAN1#118 (in red)

For frequency resource allocation of a CLI-RSSI measurement resource, the following are supported

- Measurement resource type#1: OneThe CLI-RSSI measurement resource is configured within a DL subband or a UL subband

- Measurement resource type#2: OneThe CLI-RSSI measurement resource is configured across two DL subbands

- FFS: Number of resources that can be configured

FFS: UE behavior for measurement

Working assumption

For L1 based CLI measurement and reporting, the following is adopted

- For L1 SRS-RSRP, a 7-bit value in the quantization range [-140, -44] dBm with 1dB step size, and the differential L1 SRS-RSRP is quantized to a 4-bit value. The differential L1 SRS-RSRP value is computed with 2 dB step size with a reference to the largest measured L1 SRS-RSRP value.

- For L1 CLI-RSSI, a 7-bit value in the quantization range [-100, -25] dBm with 1dB step size, and the differential L1 CLI RSSI is quantized to a 4-bit value. The differential L1 CLI-RSSI value is computed with 2 dB step size with a reference to the largest measured L1 CLI-RSSI value.

Send an LS to RAN4 about the above working assumption. LS on L1 based UE-to-UE CLI measurement and reporting to RAN4 is agreed. Final LS in R1-2501620.

Agreement

For discussion purpose, SRS-RSRP measurement resource index is referred to as SRS-RSRP-MRI and CLI-RSSI measurement resource index is referred to as SRS-RSRP-MRI.

The mapping order of CSI fields of one report for SRS-RSRP-MRI/SRS-RSRP or CLI-RSSI-MRI/CLI-RSSI reporting is provided in Table 1 with following modifications based on Table 6.3.1.1.2-8 defined in TS38.212

- CRI and SSBRI is replaced with SRS-RSRP-MRI or CLI-RSSI-MRI.

- Capability index is crossed out in the table.

- RSRP is replaced with L1-SRS-RSRP

- L1-CLI-RSSI is added in the table if the CSI report is associated with CLI-RSSI measurement resource.

Table 1: Mapping order of CSI fields of one report for SRS-RSRP-MRI/SRS-RSRP or CLI-RSSI-MRI/CLI-RSSI

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n | SRS-RSRP-MRI or CLI-RSSI-MRI #1 as in Table 2, if reported |
|  | SRS-RSRP-MRI or CLI-RSSI-MRI #2 as in Table 2, if reported |
|  | … |
|  | SRS-RSRP-MRI or CLI-RSSI-MRI #4 as in Table 2, if reported |
|  | L1-SRS-RSRP or L1-CLI-RSSI #1 as in Table 2, if reported |
|  | Differential L1-SRS-RSRP or L1-CLI-RSSI #2 as in Table 2, if reported |
|  | … |
|  | Differential L1-SRS-RSRP or L1-CLI-RSSI #4 as in Table 2, if reported |

Table 2. Bitwidth for SRS-RSRP-MRI, SRS-RSRP, CLI-RSSI-MRI and CLI-RSSI

| Field | Bitwidth |
| --- | --- |
| SRS-RSRP-MRI | ⌈log2KsSRS-RSRP⌉ |
| CLI-RSSI-MRI | ⌈log2KsCLI-RSSI⌉ |
| L1-SRS-RSRP | 7 |
| L1-CLI-RSSI | 7 |
| Differential RSRP | 4 |
| Differential RSSI | 4 |

KsSRS-RSRP: Number of SRS-RSRP measurement resources in the corresponding resource set

KsCLI-RSSI: Number of CLI-RSSI measurement resources in the corresponding resource set

Agreement

For PUSCH repetition Type A in SBFD symbols with Configuration 2, the same time location of none, one or two UL muting symbol(s) is applied for all PUSCH repetitions in SBFD symbols.

For PUSCH repetition Type B in SBFD symbols with Configuration 2, the same time location of none, one or two UL muting symbol(s) per slot is applied for all the actual PUSCH repetitions in SBFD symbols.

Note: whether UL resource muting is also applied in non-SBFD symbols is discussed separately

Agreement

For UL muting applied for UL TBoMS, the index of the starting coded bit in a slot, except the first slot, within the 𝑁𝑠 slots allocated for the transmission of TB processing over multiple slots is determined according to Section 6.2.5, TS38.212 as follows

k0=k0'+H+τmodNcb

and H is down-selected from one of the following options

  - Option 1: 𝐻 is the total number of coded bits available for transmission of the transport block in the previous slot within the 𝑁𝑠 slots assuming no UCI multiplexing and assuming no muted REs.

  - Option 2: 𝐻 is the total number of coded bits available for transmission of the transport block in the previous slot within the 𝑁𝑠 slots assuming no UCI multiplexing and considering the muted REs

Note: No impact on TBS determination in Option 1 or Option 2.

Agreement

- Periodic L1 CLI-RSSI reporting for L1 based UE-to-UE CLI measurement and reporting is supported

- Semi-persistent L1 CLI-RSSI reporting for L1 based UE-to-UE CLI measurement and reporting is NOT supported

- Periodic/Semi-persistent L1 SRS-RSRP reporting for L1 based UE-to-UE CLI measurement and reporting is NOT supported

Conclusion

For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to ‘cli-RSSI’ or ‘cli-SRS-RSRP’, new reporting criteria considering the least interfered measured L1-CLI-RSSI or L1-SRS-RSRP are not supported.

Agreement

For L1 CLI measurement and reporting, the number of occupied CPU(s) is determined as follows

- OCPU=1 for a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to ‘cli-RSSI’, ‘cli-SRS-RSRP’.

Agreement

If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI for aperiodic CLI reporting and the first symbol of the aperiodic SRS-RSRP measurement resource or CLI-RSSI measurement resources is smaller than the UE reported threshold beamSwitchTiming, down-select from the following alternatives:

- Alt.1: UE does not expect to be configured with a scheduling offset between the last symbol of the PDCCH carrying the triggering DCI for AP CLI and the first symbol of the aperiodic CLI SRS-RSRP or CLI-RSSI resources is smaller than the UE reported threshold beamSwitchTiming.

- Alt.2: Same default beam rules as AP CSI-RS resource defined in 38.214 can be applied to AP CLI SRS-RSRP or CLI-RSSI resource.

FFS: whether Alt.1 or Alt.2 has any specification impact.

#### 2.1.2	Remaining Open issues

SBFD TX/RX/measurement procedures

  - Remaining details on semi-static indication of frequency domain location of SBFD subbands to UEs in RRC_CONNECTED mode

  - Remaining details on UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE

SBFD random access operation

  - Remaining details on SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode

CLI handling

  - Remaining details on UL resource muting for PUSCH, including

  - Remaining details on L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework

## 2.2	RAN2

#### 2.2.1	Agreements

When a SBFD aware UE supporting one or both SBFD RACH configuration options accesses a cell, the UE can apply the supported SBFD RACH configuration option in the cell.

When a SBFD aware UE supporting a SBFD RACH configuration option accesses a cell configured with a different SBFD RACH configuration option, the UE applies the legacy RA operation, and does not apply the SBFD RACH configuration.

For initial RA transmission, the network can indicate the RO type (legacy RO or additional RO) to the SBFD-aware UE for the case of CBRA. Detailed signalling is FFS.

If no RO type indication is provided by the NW, a UE selects RO type based on a SSB RSRP threshold. FFS whether NW can further indicate whether to select the additional RO type below or above this SSB RSRP threshold.

FFS whether RO type selection is performed before or after the RA type selection.

FFS if switching from the PRACH resources in non-SBFD symbols to the PRACH resources in SBFD symbols is supported.

A new SP CLI measurement resource set activation/deactivation MAC CE is introduced to activate/deactivate the SP CLI measurement resource.

#### 2.2.2	Remaining Open issues

  - Remaining details on SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode

  - Other remaining details that have RAN2 impact such as SBFD time/frequency subbands configuration

## 2.3	RAN3

#### 2.3.1	Agreements

LS to RAN2 on SBFD information exchange among gNBs for CLI mitigation

Rev in R3-250888  Agreed unseen

Exchange of SBFD configuration:

RAN3 to confirm that SSB resource configuration has already been supported to be exchanged between gNBs in Served Cell Information NR IE.

gNB-DU provides the SBFD time and frequency configuration to gNB-CU in Served Cell Information IE, and gNB-CU provides the SBFD time and frequency configuration info received over Xn interface from gNB-CU to gNB-DU, in Served Cell Information IE in Served Cell Information IE in gNB-CU Configuration Update message.

The SBFD time and frequency configuration is encoded as container defined by RAN2, an LS was sent to RAN2 and RAN1 in CC informing the agreement in RAN3 and asking RAN2 to work on the IE design.

Exchange of measurement configuration:

For exchange of SSB info over Xn, nothing to be done, already supported in RAN3 spec.

Exchange of measurement result:

For exchange of strongest DL beam information over Xn, SSB info is defined as SSB-Index INTEGER (0..63), NZP CSI-RS info is defined as NZP CSI-RS Resource Indicator INTEGER (1..64).

Exchange of CLI-mitigation request:

To define CLI-mitigation request as an optional IE.

#### 2.3.2	Remaining Open issues

  - Remaining details on information exchange among gNBs, including

      - Semi-static cell-specific SBFD time and frequency location configuration, which is now mainly pending on RAN2 design

      - Measurement resource configuration, i.e., periodic NZP CSI-RS

      - How to exchange strongest DL beam information

      - How to exchange CLI-mitigation request

## 2.4	RAN4

#### 2.4.1	Agreements

In RAN4#114, the following agreements were made.

The agreements were captured in following T-docs:

| Tdoc number | Title | Source | Status |
| --- | --- | --- | --- |
| R4-2502310 | Way Forward for [114][306] NR_duplex_evo_General | Samsung | Approved |
| R4-2502380 | Way Forward for [114][307] NR_duplex_evo_BSRF | Huawei | Approved |
| R4-2502699 | WF on RRM requirements for NR_duplex_evo | Huawei | Approved |
| R4-2502632 | LS on L1 CLI measurement | Huawei | Approved |

General aspects (including RAN4 aspects for SBFD system parameters)

Issue 1-1-1: FR2-1 CBW and SCS support for SBFD

Agreement:

- In FR2-1 bands, the BS channel bandwidth for SBFD operation shall be no less than 100MHz.

  - RAN4 to further study on 100MHz for DUD case to identify if any issue.

Issue 1-1-2: Specification implementation of operating bands support for SBFD

Agreement:

- Text proposal below is endorsed:

=================== Start of Text Proposal ===================

5.2	Operating bands

NR is designed to operate in the operating bands defined in table 5.2-1 and 5.2-2.

NR operating band n1, which is defined in Table 5.2-1, can be applied for HAPS operation.

NR operating bands n1, n3, n34, n39, n41, n78, n79, which are defined in Table 5.2-1, can be applied for ATG operation.

NB-IoT is designed to operate in the NR operating bands n1, n2, n3, n5, n7, n8, n12, n13, n14, n18, n20, n25, n26, n28, n31,  n41, n65, n66, n70, n71, n72, n74, n85, n90, n106 which are defined in Table 5.2-1.

Subband full duplex (SBFD) is designed to operate in NR TDD operating bands in FR1 which are defined in Table 5.2-1 except bands n34, n46, n51, n53, n54, n96, n101 and n102, and NR operating bands in FR2-1 which are defined in Table 5.2-2.

Table 5.2-1: NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high (MHz) | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high (MHz) | Duplex mode | Notes |
| --- | --- | --- | --- | --- |
| n1 | 1920 – 1980 | 2110 – 2170 | FDD |  |
| n2 | 1850 – 1910 | 1930 – 1990 | FDD |  |
| n3 | 1710 – 1785 | 1805 – 1880 | FDD |  |
| n5 | 824 – 849 | 869 – 894 | FDD |  |
| n7 | 2500 – 2570 | 2620 – 2690 | FDD |  |
| n8 | 880 – 915 | 925 – 960 | FDD |  |
| n12 | 699 – 716 | 729 – 746 | FDD |  |
| n13 | 777 – 787 | 746 – 756 | FDD |  |
| n14 | 788 – 798 | 758 – 768 | FDD |  |
| n18 | 815 – 830 | 860 – 875 | FDD |  |
| n20 | 832 – 862 | 791 – 821 | FDD |  |
| n24 | 1626.5 – 1660.5 | 1525 – 1559 | FDD | Note 7 |
| n25 | 1850 – 1915 | 1930 – 1995 | FDD |  |
| n26 | 814 – 849 | 859 – 894 | FDD |  |
| n28 | 703 – 748 | 758 – 803 | FDD |  |
| n29 | N/A | 717 – 728 | SDL |  |
| n30 | 2305 – 2315 | 2350 – 2360 | FDD |  |
| n31 | 452.5 – 457.5 | 462.5 – 467.5 | FDD |  |
| n34 | 2010 – 2025 | 2010 – 2025 | TDD | Note 12 |
| n38 | 2570 – 2620 | 2570 – 2620 | TDD |  |
| n39 | 1880 – 1920 | 1880 – 1920 | TDD |  |
| n40 | 2300 – 2400 | 2300 – 2400 | TDD |  |
| n41 | 2496 – 2690 | 2496 – 2690 | TDD |  |
| n46 | 5150 – 5925 | 5150 – 5925 | TDD | Note 3, Note 12 |
| n48 | 3550 – 3700 | 3550 – 3700 | TDD |  |
| n50 | 1432 – 1517 | 1432 – 1517 | TDD |  |
| n51 | 1427 – 1432 | 1427 – 1432 | TDD | Note 12 |
| n53 | 2483.5 – 2495 | 2483.5 – 2495 | TDD | Note 12 |
| n54 | 1670 – 1675 | 1670 – 1675 | TDD | Note 12 |
| n65 | 1920 – 2010 | 2110 – 2200 | FDD |  |
| n66 | 1710 – 1780 | 2110 – 2200 | FDD |  |
| n67 | N/A | 738 – 758 | SDL |  |
| n70 | 1695 – 1710 | 1995 – 2020 | FDD |  |
| n71 | 663 – 698 | 617 – 652 | FDD |  |
| n72 | 451 – 456 | 461 – 466 | FDD |  |
| n74 | 1427 – 1470 | 1475 – 1518 | FDD |  |
| n75 | N/A | 1432 – 1517 | SDL |  |
| n76 | N/A | 1427 – 1432 | SDL |  |
| n77 | 3300 – 4200 | 3300 – 4200 | TDD | Note 11 |
| n78 | 3300 – 3800 | 3300 – 3800 | TDD |  |
| n79 | 4400 – 5000 | 4400 – 5000 | TDD |  |
| n80 | 1710 – 1785 | N/A | SUL |  |
| n81 | 880 – 915 | N/A | SUL |  |
| n82 | 832 – 862 | N/A | SUL |  |
| n83 | 703 – 748 | N/A | SUL |  |
| n84 | 1920 – 1980 | N/A | SUL |  |
| n85 | 698 – 716 | 728 – 746 | FDD |  |
| n86 | 1710 – 1780 | N/A | SUL |  |
| n89 | 824 – 849 | N/A | SUL |  |
| n90 | 2496 – 2690 | 2496 – 2690 | TDD |  |
| n91 | 832 – 862 | 1427 – 1432 | FDD | Note 2 |
| n92 | 832 – 862 | 1432 – 1517 | FDD | Note 2 |
| n93 | 880 – 915 | 1427 – 1432 | FDD | Note 2 |
| n94 | 880 – 915 | 1432 – 1517 | FDD | Note 2 |
| n95 | 2010 – 2025 | N/A | SUL | Note 1 |
| n96 | 5925 – 7125 | 5925 – 7125 | TDD | Note 3, Note 4, Note 12 |
| n97 | 2300 – 2400 | N/A | SUL | Note 5 |
| n98 | 1880 – 1920 | N/A | SUL | Note 5 |
| n99 | 1626.5 – 1660.5 | N/A | SUL | Note 6 |
| n100 | 874.4 – 880 | 919.4 – 925 | FDD | Note 10 |
| n101 | 1900 – 1910 | 1900 – 1910 | TDD | Note 10, Note 12 |
| n102 | 5925 – 6425 | 5925 – 6425 | TDD | Note 3, Note 4, Note 12 |
| n104 | 6425 – 7125 | 6425 – 7125 | TDD | Note 8 |
| n105 | 663 – 703 | 612 – 652 | FDD |  |
| n106 | 896 – 901 | 935 – 940 | FDD |  |
| n109 | 703 – 733 | 1432 – 1517 | FDD | Note 2 |
| NOTE 1: This band is applicable in China only. NOTE 2: Variable duplex operation does not enable dynamic variable duplex configuration by the network, and is used such that DL and UL frequency ranges are supported independently in any valid frequency range for the band. NOTE 3: This band is restricted to operation with shared spectrum channel access as defined in TS 37.213 [20]. NOTE 4: This band is applicable only in countries/regions designating this band for shared-spectrum access use subject to country-specific conditions. NOTE 5: The requirements for this band are applicable only where no other NR or E-UTRA TDD operating band(s) are used within the frequency range of this band in the same geographical area. For scenarios where other NR or E-UTRA TDD operating band(s) are used within the frequency range of this band in the same geographical area, special co-existence requirements may apply that are not covered by the 3GPP specifications. NOTE 6: UL operation is restricted to 1627.5 – 1637.5 MHz and 1646.5 – 1656.5 MHz per FCC Order DA 20-48 [25]. NOTE 7: DL operation is restricted to 1526-1536 MHz frequency range. UL operation is restricted to 1627.5 – 1637.5 MHz and 1646.5 – 1656.5 MHz per FCC Order 20-51 [24]. NOTE 8: This band is applicable only in countries/regions designating this band for IMT licensed operation subject to country-specific conditions. NOTE 9: Operating band n200 is a reserved value. NOTE 10: This band is applicable to CEPT countries subject to ECC Decision (20)02 [21], for the RMR application. NOTE 11: In the USA this band is restricted to 3450 – 3550 MHz and 3700 – 3980 MHz. In Canada this band is restricted to 3450 – 3650 MHz and 3650 – 3980 MHz. NOTE 12: This TDD band is not applicable to SBFD operation. |  |  |  |  |

Table 5.2-2: NR operating bands in FR2

| NR operating band | Uplink (UL) and Downlink (DL) operating band BS transmit/receive UE transmit/receive FUL,low – FUL,high FDL,low – FDL,high (MHz) | Duplex mode |
| --- | --- | --- |
| n257 | 26500 – 29500 | TDD |
| n258 | 24250 – 27500 | TDD |
| n259 | 39500 – 43500 | TDD |
| n260 | 37000 – 40000 | TDD |
| n261 | 27500 – 28350 | TDD |
| n262 | 47200 – 48200 | TDD |
| n263 | 57000 – 71000 | TDD |

=================== End of Text Proposal ===================

Issue 1-1-3: Specification implementation of BS CBW support for SBFD

Agreement:

- Text proposal to Table 5.3.2-1 and Table 5.3.2-2 is endorsed:

=================== Start of Text Proposal ===================

Table 5.3.2-1: Transmission bandwidth configuration NRB for FR1

| SCS (kHz) | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 15 | 25 | 52 | 79 | 106 | 133 | 160 | 188 | 216 | 242 | 270 | N/A | N/A | N/A | N/A | N/A |
| 30 | N/A | 11 | 24 | 38 | 51 | 65 | 78 | 92 | 106 | 119 | 133 | 162 | 189 | 217 | 245 | 273 |
| 60 | N/A | N/A | 11 | 18 | 24 | 31 | 38 | 44 | 51 | 58 | 65 | 79 | 93 | 107 | 121 | 135 |
| Note: In FR1 bands applicable to SBFD operation, the BS channel bandwidth for SBFD operation shall be no less than 40MHz and no less than 20MHz for band n39. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5.3.2-2: Transmission bandwidth configuration NRB for FR2-1

| SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB |
| 60 | 66 | 132 | 264 | N/A |
| 120 | 32 | 66 | 132 | 264 |
| Note: The BS channel bandwidth for SBFD operation shall be no less than 100MHz. |  |  |  |  |

=================== End of Text Proposal ===================

Agreement:

- RAN4 shall aim to adopt the same Notes for Table 5.3.5-1 and Table 5.3.5-2 respectively. Take the table for FR2-1 as example:

=================== Start of Text Proposal ===================

Table 5.3.5-2: BS channel bandwidths and SCS per operating band in FR2-1

| NR Band | SCS (kHz) | BS channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 | 100 | 200 | 400 |
| n257 | 60 | 501 | 100 | 200 |  |
|  | 120 | 501 | 100 | 200 | 400 |
| n258 | 60 | 501 | 100 | 200 |  |
|  | 120 | 501 | 100 | 200 | 400 |
| n259 | 60 | 501 | 100 | 200 |  |
|  | 120 | 501 | 100 | 200 | 400 |
| n260 | 60 | 501 | 100 | 200 |  |
|  | 120 | 501 | 100 | 200 | 400 |
| n261 | 60 | 501 | 100 | 200 |  |
|  | 120 | 501 | 100 | 200 | 400 |
| n262 | 60 | 501 | 100 | 200 |  |
|  | 120 | 501 | 100 | 200 | 400 |
| Note 1: For this bandwidth, it is not applicable for SBFD operation. |  |  |  |  |  |

=================== End of Text Proposal ===================

Issue 1-1-4: Declaration and flexibility for SBFD operation

Agreement:

- The SBFD capable BS product shall not operate any declared SBFD configuration (DL/UL subband) other than those that have been tested.

  - This restriction shall not be applied to FR1 LA BS

  - FFS FR1 MR BS

Issue 1-3-1: Requirement set applicability table for SBFD-capable BS

Way Forward:

- RAN4 further study the applicability of RAN4 requirement for SBFD capable BS, by taking the table provided in R4-2501967 as example to be further study on. NOTE: The table does not imply that the BS can be declared 1-H for non-SBFD and 1-O for SBFD, or vice versa.

Issue 1-4-1: Limiting Maximum UE Transmit power in SBFD symbols/slots.

Way Forward:

- No further RAN4 discussion is expected until RAN1 confirm the necessity of introducing the mechanism of limiting Maximum UE Transmit power in SBFD symbols/slots

Issue 1-4-2: Using threshold(s) to limit the scheduling of cell-edge UEs in SBFD symbols/slots

Way Forward:

- No further RAN4 discussion is expected until RAN1 confirm the necessity of introducing the mechanism of using threshold(s) to limit the scheduling of cell-edge UEs in SBFD symbols/slots

Issue 1-5-1: BS RF Specification structure for SBFD requirements

Agreement:

- RAN4 shall capture RF requirements in SBFD-capable BS in TS 38.104. To use a separate chapter for requirements for SBFD-capable BS in TS 38.104 is the baseline, while details can be FFS by taking below structure as the example.

Foreword

1	Scope

2	References

3	Definitions, symbols and abbreviations

4	General

5	Operating bands and channel arrangement

6	Conducted transmitter characteristics

7	Conducted receiver characteristics

8	Conducted performance requirements

9	Radiated transmitter characteristics

10	Radiated receiver characteristics

11	Radiated performance requirements

12	Requirements for SBFD-capable BS

Issue 2-1-1: RF requirement impact for PUSCH with resource muting

Way Forward:

- RAN4 continue to study RF requirement impact for PUSCH with resource muting.

Potentially new requirements for SBFD

Issue 3-1-1: Location of transient period between SBFD and non-SBFD

Agreement:

- RAN4 reach the conclusion of the location of transient periods:

  - Case-A: in TA_offset in SBFD symbol(s);

  - Case-B: in guard symbol(s) reserved before UL non-SBFD symbols

    - FFS guard symbol(s) in this case is SBFD symbol or not.

  - Case-C: FFS in guard symbol(s) reserved in non-SBFD DL symbol(s) or blanked SBFD symbols in the beginning of SBFD UL transmission:

    - RAN4 recognize that it can be dependent on RAN1 discussion for guard period design.

    - If still no RAN4 agreement can be achieved in RAN4#114bis, RAN4 shall

      - either send LS to RAN1 to ask for clarification,

      - or conclude the location of transient period between SBFD and non-SBFD as gNB implementation issue without RAN4 requirement impact.

  - Case-D: in TA_offset in non-SBFD UL symbol(s).

  - Note: The illustrative figure for Case A, B, C and D is provided below:

Issue 3-1-3: Transient period requirement applicability on SBFD subbands

Way Forward:

- Text proposal below is taken as the baseline for further discussion, with the understanding that the content can be further updated based on the future agreement on the location of transient period between SBFD and non-SBFD:

=================== Start of Text Proposal ===================

6.4.2B	Transmitter transient period for SBFD

6.4.2.1B	General

Transmitter transient period for SBFD requirements apply only to SBFD operation of the BS.

The transmitter transient period for SBFD is the time period during which the transmitter is changing from the transmitter OFF period to the transmitter ON period or vice versa. The transmitter transient period is illustrated in figure 6.4.2.1-1.

The transmitter transient period for SBFD shall be measured in the transmission bandwidth configuration for SBFD DL subband, UL subband and guardband, depending on different SBFD cases:

Table 6.4.2.1B-1: Transmitter transient period for SBFD

|  | Frequency regions for transition | BS transmitter behavior during transition |
| --- | --- | --- |
| Case-A (SBFD symbol to DL symbol) | SBFD UL subband and guardband(s) | BS transmitter OFF-to-ON |
| Case-B (SBFD symbol to UL symbol) | SBFD DL subband | BS transmitter ON-to-OFF |
| Case-C (DL symbol to SBFD symbol) | SBFD UL subband and guardband(s) | BS transmitter ON-to-OFF |
| Case-D (UL symbol to SBFD symbol) | SBFD DL subband | BS transmitter OFF-to-ON |

For BS type 1-H this requirement shall be applied at each TAB connector supporting transmission in the operating band.

=================== End of Text Proposal ===================

Issue 3-2-1: In-channel adjacent subband leakage ratio: General

Agreement:

- New requirement for In-channel adjacent subband leakage ratio:

  - In-channel adjacent subband leakage ratio requirement is defined with the existing ACLR requirement in absolute PSD limit value as baseline (i.e., Table 6.6.3.2-2 in TS 38.104 for FR1 and Table 9.7.3.3-2 for FR2 in TS 38.104)

    - FFS the value for absolute PSD limit

    - FFS the naming of this requirement, by considering the absolute PSD limit used instead of the ratio. Note that the current naming in TS 38.104 is ACLR absolute limit.

Issue 3-2-2: In-channel adjacent subband leakage ratio: Configuration

Way Forward:

- Further study the configuration of the in-channel adjacent subband leakage ratio requirement, by taking the following table as the baseline:

| BS DL subband bandwidth BWSBFD,DL (MHz) | BS UL subband bandwidth BWSBFD,UL (UL) | Filter on UL subband and corresponding filter bandwidth | In-Channel Adjacent Subband Leakage Ratio limit |
| --- | --- | --- | --- |
| BWSBFD,DL as configured by gNB | BWSBFD,UL as configured by gNB | Square (BWSBFD,UL) | TBD |

Issue 3-2-4: In-channel adjacent subband blocking requirement

Way Forward:

- RAN4 continue to study In-channel adjacent subband blocking requirement, by following the agreement achieved in RAN4#113.

Modification of existing Tx requirements for FR1 and FR2-1

Issue 1: Joint measurement for normal DL symbols/slots and SBFD symbols/slots

Way Forward:

- Separate testing for SBFD slot and non-SBFD slot, with the consideration to try to adopt slot-level for the test granularity.

Issue 2:  In-band blocking and additional emission

Way Forward:

- For FR2-1 WA SBFD BS, FFS re-use the current FR2-1 WA BS in-band blocking requirements.

- For FR1 WA BS, define the in-band blocking requirement, FFS on following options. Other options are not precluded.

  - Option 1: Define the requirement by selecting one of the following grid-shift values. Additional emission issue is not included in this release. If regulators make a decision to require SBFD on adjacent operators in a band, a future WI would address necessary emissions and blocking requirements for the SBFD-SBFD adjacent operator scenario.

    - Option A: 20%

    - Option B: 50%

  - Option 2: Define general requirements for SBFD-TDD adjacent operator scenario. Define an additional in-band blocking requirement and additional emission to address SBFD-SBFD adjacent operator scenario.

  - Option 3: Define in-band blocking requirements based on co-existence simulation results with grid-shift 5%.

  - Option 4: Define in-band blocking requirements based on co-existence simulation results with grid-shift 100%.

- For FR1 MR SBFD BS, FFS.

Issue 3:  Dynamic range requirement

Way Forward:

- The dynamic range requirements of FR1 can reuse existing requirement.

- For FR2, no dynamic range requirement is needed.

Issue 4:  ACS requirement

Way Forward:

- FFS on the following proposals.

  - Proposal 1: Prioritize discussing the scenarios under non-colocation case first.

  - Proposal 2: The same definition of ACS could be reused for SBFD, and the interference level of [9] dB offset should be further checked.

  - Proposal 3: The definition of ACS is reused for SBFD. The interference level keeps 9 dB offset to the general in-band blocking interference level defined for SBFD.

  - Proposal 4: It is needed to determine whether interference signal level of BS to BS can be defined as ACS interference signal level.

  - Proposal 5: The requirement level of ACS should be derived from the BS-BS co-existence simulation results obtained in SI, instead of keeping 9 dB offset to the general in-band blocking interference level defined for SBFD.

  - Proposal 6: Derive ACS requirement level based on the simulation results using 5% GS in SI.

  - Proposal 7: The OTA sensitivity degradation should be taken into account for the baseline REFSENS for OTA ACS requirement and should not be taken into account when testing conducted ACS.

Issue 5:  Necessity of New RX intermodulation requirement with 1 interfering signal

Way Forward:

- FFS on the following options.

  - Option 1: IMD requirement for single interfering signal scenario is not needed. Such a requirement is implicitly captured by the SBFD RX blocking requirement.

  - Option 2: RAN4 to consider adjusting current RX IMD requirement with interfering signals mean power levels and frequencies.

Issue 6:  Impact on existing RX intermodulation requirement with 2 interfering signals

Way Forward:

- FFS on the following options.

  - Option 1:

    - for the receiver intermodulation requirements, BS2BS CLI should be taken into account for power level for interference signal.

    - for receiver intermodulation requirements, consider IMD between CW/NBB/general intermodulation interfering signal intermodulate with SBFD DL transmission with some performance degradation on SBFD receiver.

  - Option 2: new IMD requirement with 2 interfering signal scenario is not needed.

  - Option 3: RAN4 to consider new RX intermodulation requirement with 2 interfering signals. RAN4 to consider the three possibilities for RX IMD and choose the most challenging one to cover others.

  - Option 4: The scenario for 2 input signals can be regarded as the worst case when setting requirement on receiver linearity. For RX IMD, the mean power of both CW and modulated interfering signals is [the final agreed in-band blocking level] – 9 dB.

RRM impacts of UE-to-UE CLI handling

Issue 1-1-1: Measurement methods

Agreements (from Mon AH session)

- RAN4 to define the CLI measurement requirements for Method #3 (UE measures RSSI within UL subband)

Issue 1-1-3: Measurement resources

Agreements

- Define measurement requirements assuming wideband reporting for L1-SRS-RSRP and L1-CLI-RSSI, and do not need to consider subband reporting. FFS if UE is able to measure both DL subbands simultaneously in case of DUD and how such measurement capability affects measurements period and accuracy requirements.

- RAN4 to define requirements for L1-CLI-RSRI and L1-SRS-RSRP measurement for semi-static, aperiodic and periodic resource types.

Issue 1-1-4: Measurement reporting

Agreements

- Focus on aperiodic reporting mode for Rel-19 CLI measurement, for other reporting modes hold on the discussion pending on further progress in RAN1.

- L1-RSRP measurement reporting requirement for aperiodic reporting can be reused for the aperiodic L1 based CLI measurement reporting.

Issue 1-1-5: Scheduling restriction

Agreements (from Thu AH session)

- For L1-SRS-RSRP,

  - For UL, scheduling restrictions always apply, for both FR1 and FR2.

  - For DL, scheduling restriction does not apply to the symbols with SRS if UE is able to support FDM-ed DL reception and SRS measurement if RAN1 will define optional UE capability for FDMed DL reception and SRS measurement; otherwise (if UE does not support the capability or RAN1 does not define the capability), scheduling restriction applies.

  - The restricted symbols at least include symbols for measurement resource and Y symbols before symbols for measurement resource and Y is re-using same number as R16 requirements,

    - Note: this time has accounted for the UL/DL switching time.

  - FFS whether there is restriction on symbols after the symbols for measurement resource considering UL/DL transition.

- For L1-CLI-RSSI Methods #1,

  - For UL, scheduling restrictions always apply, for both FR1 and FR2.

  - For DL, scheduling restrictions do not apply except when CLI resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2.

  - The restricted symbols at least include symbols for measurement resource, FFS whether restriction on additional symbols is needed.

- For L1-CLI-RSSI Method #3, scheduling restriction requirements are FFS

Issue 1-2-1: Side conditions

Agreements (from online session)

- For SRS configuration, reuse R16 assumptions on SRS configuration for L1 SRS-RSRP measurements

Agreements

- Further discuss timing offset side condition for accuracy requirements in the performance part of the WI, including whether to consider intra-cell scenario only or both intra- and inter-cell scenarios for defining the side condition.

- For SRS Es/Iot, further discuss the following options

  - Option 1: 1dB

  - Option 2: > 1dB

- For SRS BW, further discuss the following options

  - Option 1: 24RB

  - Option 2: 48RB

Issue 1-2-2: Measurement period

Agreements (from online session)

- RAN4 to define one set of accuracy requirements based 1 sample.

- Send LS to RAN1/2 on whether the NW configuration of timeRestrictionForChannelMeasurement or a similar configuration will apply for L1 SRS-RSRP and/or CLI-RSSI measurement.

  - From RAN4 requirement perspective: if it applies, RAN4 to define two sets of requirements for L1 SRS-RSRP measurement delay.

Agreements

- For aperiodic resources, measurement period is based on 1 sample.

Issue 1-3-1: Measurement timing for Method #3

Agreements (from online session)

- Base on DL reference timing

RRM impacts of SBFD operation

Issue 2-1-1b: CSI-RS L1 measurement – puncturing in frequency domain

Agreements

- Further discuss measurement period requirements for CSI-RS based L1 measurement in SBFD symbols with DUD case

  - Option 1: existing measurement period for CSI-RS based L1 measurement apply, with the assumption that UE measures both DL subbands on each CSI-RS resource occasion.

  - Option 2: doubling existing measurement period, with the assumption that UE measures one DL subband on each CSI-RS resource occasion.

  - Option 3: Consider the UE measurement capability

- FFS whether and how to account for the UE capability on CSI processing time.

- Further discuss the accuracy requirements for the following cases

  - Case 1: X1 >= X3 and X2 >= X3

  - Case 2: X1 >= X3 and X2 < X3

  - Case 3: X1 < X3 and X2 < X3, (X1 + X2) >= X3

where X1 and X2 are CSI-RS BW in DL sub-band #1 and sub-band #2, and X3 is the min BW with which the existing accuracy requirements apply (X3=48 for L1-RSRP/L1-SINR and X3=24 for RLM/BFD/CBD).

Issue 2-1-2: Requirements for CSI-RS based L3 measurement

Agreements (from Wed AH session)

- For DU case:

  - UE measurement requirements would be same as in legacy with the condition that all CSI-RS resources configured for measurement are available in DL subband of SBFD symbols and/or on non-SBFD DL symbols.

- FFS on DUD case

#### 2.4.2	Remaining Open issues

  - Remaining details on RAN4 aspects for SBFD system parameters, including:

    - Declaration and flexibility for SBFD operation for FR1 MR BS;

    - Remaining issues for DL/UL subband configurations support for SBFD;

    - Requirement applicability for SBFD

    - RAN4 impact of the mechanisms of limiting Maximum UE Transmit power and using threshold(s) to limit the scheduling of cell-edge UEs in SBFD symbols/slots, if RAN1 confirm the necessity of introducing those mechanisms.

    - RF requirement impact for PUSCH with resource muting.

  - Remaining details on BS RF requirements for SBFD operation at gNB:

    - Location of transient period between SBFD and non-SBFD

    - Transient period requirement applicability on SBFD subbands

    - In-channel adjacent subband requirements for TX leakage and RX blocking

    - In-band blocking requirement and additional emission requirement if the necessity confirmed

    - ACS requirement

    - RX intermodulation requirement with 1 interfering signal, and 2 interfering signals if the necessity confirmed.

  - Remaining details on applicable RRM core requirements for UE-to-UE CLI handling mechanisms:

    - Scheduling restriction for L1-SRS-RSRP and L1-CLI-RSSI measurements

    - Measurement period for L1-SRS-RSRP and L1-CLI-RSSI measurement

  - Remaining details on other RRM core requirements for SBFD operation, if identified.

    - CSI-RS L1 measurement requirement for SBFD operation

    - CSI-RS L3 measurement requirement for DUD case.

  - CR drafting to implement the agreements for BS RF requirements in TS 38.104 and RRM requirement in TS38.133.

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1#120

R1-2500061	Discussion on SBFD TX/RX/measurement procedures	TCL

R1-2500062	Discussion on SBFD random access operation	TCL

R1-2500093	On subband full duplex design	Huawei, HiSilicon

R1-2500094	On subband full duplex random access operation	Huawei, HiSilicon

R1-2500095	On cross-link interference handling for subband full duplex	Huawei, HiSilicon

R1-2500144	Discussion on transmission, reception and measurement procedures for SBFD operation	ZTE Corporation, Sanechips

R1-2500145	Discussion on SBFD random access operation	ZTE Corporation, Sanechips

R1-2500146	Discussion on CLI handling for Rel-19 duplex operation	ZTE Corporation, Sanechips

R1-2500157	Higher layer parameters for Rel-19 SBFD	Huawei, HiSilicon

R1-2500166	Discussion on SBFD TX/RX/measurement procedures	Spreadtrum, UNISOC

R1-2500167	Discussion on SBFD random access operation	Spreadtrum, UNISOC

R1-2500168	Discussion on CLI handling	Spreadtrum, UNISOC

R1-2500221	Discussion on SBFD TX/RX/measurement procedures	CATT

R1-2500222	Discussion on SBFD random access operation	CATT

R1-2500223	Discussion on CLI handling for NR duplex evolution	CATT

R1-2500249	Discussion on SBFD TX/RX/measurement procedures	LG Electronics

R1-2500250	Discussion on SBFD random access operation	LG Electronics

R1-2500251	Discussion on CLI handling	LG Electronics

R1-2500257	Discussion on SBFD TX/RX/measurement procedures	China Telecom

R1-2500258	Discussion on SBFD random access operation	China Telecom

R1-2500283	Discussion on SBFD TX/RX/measurement procedures	CMCC

R1-2500284	Discussion on SBFD random access operation	CMCC

R1-2500285	Discussion on CLI handling	CMCC

R1-2500314	Discussion on SBFD TX/RX/measurement procedures	MediaTek Inc.

R1-2500315	Discussion on SBFD Random Access Operation	MediaTek Inc.

R1-2500316	Discussion on CLI Handling in SBFD system	MediaTek Inc.

R1-2500346	Discussion on Rel-19 SBFD operation	vivo

R1-2500347	Discussion on random access for Rel-19 SBFD	vivo

R1-2500348	Discussion on CLI handling for Rel-19 NR duplex	vivo

R1-2500401	Discussion for SBFD TX/RX/ measurement procedures	Tejas Networks Limited

R1-2500402	Discussion on SBFD Random Access Operation	Tejas Networks Limited

R1-2500403	Discussion on gNB-gNB CLI handling	Tejas Networks Limited

R1-2500454	Discussion on SBFD Tx/Rx/measurement procedures	OPPO

R1-2500455	Discussion on SBFD random access operation	OPPO

R1-2500456	CLI handling in NR duplex operation	OPPO

R1-2500500	Discussion on SBFD Tx/Rx/measurement procedures	Fraunhofer HHI, Fraunhofer IIS

R1-2500515	Discussion on SBFD TX/RX/measurement procedures	Ofinno

R1-2500516	Discussion on SBFD for random-access procedure	Ofinno

R1-2500520	Views on SBFD TX_RX_measurement procedures	InterDigital, Inc.

R1-2500521	Discussion on SBFD random access operation	InterDigital, Inc.

R1-2500522	Discussion on CLI handling for SBFD operation	InterDigital, Inc.

R1-2500587	Discussion on random access for SBFD	NEC

R1-2500588	CLI handling for NR duplex operations	NEC

R1-2500620	Discussion on subband non-overlapping full duplex Tx-Rx and measurement operations	NEC

R1-2500648	SBFD procedures	Sony

R1-2500649	SBFD RACH operations	Sony

R1-2500650	SBFD CLI handling	Sony

R1-2500697	Discussion on SBFD TX/RX/measurement procedures	Panasonic

R1-2500698	Discussion on SBFD random access operation	Panasonic

R1-2500707	Discussion on CLI handling	Panasonic

R1-2500729	Discussion on reception and transmission procedure for SBFD operation	Xiaomi

R1-2500730	Discussion on SBFD random access operation	Xiaomi

R1-2500731	Discussion on CLI handling for SBFD operation	Xiaomi

R1-2500749	SBFD TX/RX/measurement procedures	Ericsson

R1-2500750	SBFD Random access operation	Ericsson

R1-2500751	CLI handling	Ericsson

R1-2500775	Discussion on SBFD TX/RX/measurement procedures	Apple

R1-2500776	Views on SBFD random access operation	Apple

R1-2500777	Discussion on CLI handling	Apple

R1-2500847	SBFD operation and procedures	Samsung

R1-2500848	Random access on SBFD resources	Samsung

R1-2500849	Cross-link interference handling for SBFD	Samsung

R1-2500880	Discussion on SBFD TX/RX/ measurement procedures	CEWiT

R1-2500881	Discussion on SBFD random access operation	CEWiT

R1-2500882	Discussion on CLI handling	CEWiT

R1-2500883	SBFD random access operation	Lenovo

R1-2500908	Discussion on SBFD TX/RX/measurement procedures	ETRI

R1-2500909	SBFD random access operation	ETRI

R1-2500910	Discussion on CLI handling for SBFD operation	ETRI

R1-2500934	Discussion on SBFD Tx/Rx/measurement procedures	Fujitsu

R1-2500935	Discussion on SBFD random access operation	Fujitsu

R1-2500965	Discussion on SBFD operation	Transsion Holdings

R1-2500966	Discussion on SBFD random access operation	Transsion Holdings

R1-2500988	Discussion on SBFD TX/RX/measurement procedures	Kookmin University

R1-2500989	Discussion on SBFD random access operation	Kookmin University

R1-2501003	On SBFD TX/RX/measurement procedures	Nokia, Nokia Shanghai Bell

R1-2501004	SBFD random access operation	Nokia, Nokia Shanghai Bell

R1-2501005	Cross-link interference handling for duplex evolution	Nokia, Nokia Shanghai Bell

R1-2501010	Views on SBFD TX/RX Measurement Procedures	Lenovo

R1-2501052	SBFD Tx/Rx/measurement procedures	Charter Communications, Inc

R1-2501055	SBFD Tx/Rx/measurement aspects	Sharp

R1-2501056	SBFD random access aspects	Sharp

R1-2501061	Cross-link interference (CLI) handling	Sharp

R1-2501088	Discussion on SBFD TX/RX/measurement procedures	Hyundai Motor Company

R1-2501107	Discussion on SBFD TX/RX/measurement procedures	HONOR

R1-2501153	SBFD Transmission, Reception and Measurement Procedures	Qualcomm Incorporated

R1-2501154	SBFD Random Access Operation	Qualcomm Incorporated

R1-2501155	Enhancements for CLI handling	Qualcomm Incorporated

R1-2501199	Discussion on SBFD TX/RX/measurement procedures	NTT DOCOMO, INC.

R1-2501200	Discussion on SBFD random access operation	NTT DOCOMO, INC.

R1-2501201	Discussion on CLI handling	NTT DOCOMO, INC.

R1-2501227	Discussion on SBFD Random Access operation	KT Corp.

R1-2501228	Discussion on SBFD TX/RX/measurement procedures	Google Korea LLC

R1-2501231	Discussion on SBFD TX/RX/measurement procedures	ITRI

R1-2501232	Discussion on SBFD random access operation for SBFD aware UEs	ITRI

R1-2501240	Partial PRG handling for SBFD	ASUSTeK

R1-2501285	Discussion on SBFD TX/RX/measurement procedures	WILUS Inc.

R1-2501286	Discussion on SBFD random access operation	WILUS Inc.

R1-2501291	On SBFD random access operation	Google Ireland Limited

R1-2501292	On the gNB-to-gNB CLI and the UE-to-UE CLI handling	Google Ireland Limited

R1-2501338	Summary #1 of SBFD TX/RX/measurement procedures	Moderator (Xiaomi)

R1-2501339	Summary #2 of SBFD TX/RX/measurement procedures	Moderator (Xiaomi)

R1-2501340	Summary #3 of SBFD TX/RX/measurement procedures	Moderator (Xiaomi)

R1-2501353	Discussion on CLI handling for Rel-19 NR duplex	vivo

R1-2501415	Summary#1 on SBFD random access operation	Moderator (CMCC)

R1-2501416	Summary#2 on SBFD random access operation	Moderator (CMCC)

R1-2501417	Summary#3 on SBFD random access operation	Moderator (CMCC)

R1-2501455	Summary #1 of CLI handling	Moderator (Huawei)

R1-2501456	Summary #2 of CLI handling	Moderator (Huawei)

R1-2501457	Summary #3 of CLI handling	Moderator (Huawei)

R1-2501458	Summary #1 of discussion on RRC parameters for 9.3.3	Moderator (Huawei)

R1-2501536	Summary of Discussion on LS on CSI-RS measurement with SBFD operation	Moderator (MediaTek)

R1-2501537	Draft Reply LS on CSI-RS measurement with SBFD operation	Moderator (MediaTek)

R1-2501560	Reply LS on CSI-RS measurement with SBFD operation	RAN1, MediaTek

R1-2501619	[Draft] LS on L1 based UE-to-UE CLI measurement and reporting	Moderator (Huawei)

R1-2501620	LS on L1 based UE-to-UE CLI measurement and reporting	RAN1, Huawei

RAN2#129

R2-2500036	LS on CSI-RS measurement with SBFD operation (R4-2420165; contact: MediaTek)	RAN4

R2-2500098	Impacts on the random access by the evolution of duplex operation	Huawei, HiSilicon

R2-2500105	Random Access Operation of SBFD	Nokia Corporation

R2-2500109	SBFD Configuration for initial random access and operations	Charter Communications, Inc

R2-2500110	Cross link interference handling in SBFD networks	Charter Communications, Inc

R2-2500191	Discussion on RACH in SBFD	Xiaomi

R2-2500273	38300 Running CR for SBFD	CATT

R2-2500274	Discussion on Random Access in SBFD symbols	CATT

R2-2500275	Discussion on other aspects of SBFD	CATT

R2-2500298	Random Access for SBFD Operation	NEC

R2-2500337	Discussion on random access procedure in SBFD	vivo

R2-2500338	SBFD other aspects	vivo

R2-2500339	Discussion on Random Access operation in SBFD	InterDigital, Inc.

R2-2500340	Discussion on resource configuration in SBFD	InterDigital, Inc.

R2-2500376	Random Access Issues for SBFD	Sharp

R2-2500403	Other aspects of SBFD	Xiaomi

R2-2500554	Other aspects of SBFD	Nokia

R2-2500588	Remaining issues for RACH in SBFD	Apple

R2-2500606	Discussion on random access procedure in SBFD	ZTE Corporation

R2-2500607	Discussion on L3 and L1 measurement in SBFD	ZTE Corporation

R2-2500637	Other impacts by the evolution of duplex operation	Huawei, HiSilicon

R2-2500751	Random access for SBFD Operation	Sony

R2-2500884	SBFD RA aspects	Ericsson

R2-2500885	CSI-RS measurements for RLM/BFD/CBD in SBFD	Ericsson

R2-2500886	SBFD UE capabilities running CR	Ericsson

R2-2501029	Discussion on random access in SBFD	CMCC

R2-2501129	Views on random access for SBFD	Qualcomm Incorporated

R2-2501130	Other aspects of SBFD	Qualcomm Incorporated

R2-2501135	Random access in SBFD	Samsung

R2-2501170	Other Aspects of SBFD	Samsung

R2-2501244	Discussion on Random Access in SBFD	LG Electronics Inc.

RAN3#127

R3-250163	Discussion on CLI handling support	NEC

R3-250193	Signaling design to enable NR SBFD operation	Qualcomm Incorporated

R3-250194	CLI Mitigation	Charter Communications, Inc

R3-250329	Discussion on Evolution of Duplex Operation	CATT

R3-250330	(TP for 38.420, 38.423, 37.340 and 38.473) Support SBFD Operation	CATT

R3-250382	Discussion on issues of CLI handling in SBFD	LG Electronics Inc.

R3-250431	Discussion on Information exchange for SBFD	ZTE corporation

R3-250432	(TP for BLCR 38.300&401&420&423&473) SBFD	ZTE corporation

R3-250469	Discussion on RAN3 impacts to support enhancements for CLI handling for SBFD	Ericsson

R3-250470	TP to TS38.423 on enhancements for CLI handling for SBFD	Ericsson

R3-250537	Further discussion on network support for SBFD operation and CLI management	Nokia

R3-250622	Discussion on information exchanges among gNBs for CLI handling	Samsung

R3-250623	TPs to BLCRs of TS38.423 and TS38.473 on CLI handling	Samsung

R3-250651	Further discussions on SBFD related information exchange	Huawei, China Telecom,China Unicom

R3-250652	Stage 2&3 CR on the introduction of Evolution of NR duplex operation: Sub-band full duplex (SBFD)	Huawei

R3-250653	Stage 2&3 CR on the introduction of Evolution of NR duplex operation: Sub-band full duplex (SBFD)	Huawei

R3-250662	Discussion on supporting CLI handling in SBFD	CMCC

R3-250663	(TP for TS 38.423) Support CLI handling	CMCC

R3-250803	CB:#SBFD	Huawei

R3-250843	LS on SBFD information exchange among gNBs for CLI mitigation	RAN3(Huawei)

R3-250888	SBFD information exchange among gNBs for CLI mitigation	RAN3(Huawei)

RAN4#114

R4-2500169	Discussion on RRM impacts of UE-to-UE CLI handling	LG Electronics Inc.

R4-2500171	Discussion on RRM impacts of SBFD operation	LG Electronics Inc.

R4-2500245	On UE RRM requirements for UE-to-UE CLI handling	Apple

R4-2500342	Discussion on SBFD General aspects	Tejas Network Limited

R4-2500343	Discussion on Potentially new requirements for SBFD operation	Tejas Network Limited

R4-2500345	Discussion on existing Tx requirements for SBFD operation	Tejas Network Limited

R4-2500346	Views on modification of existing Rx requirements for SBFD	Tejas Network Limited

R4-2500466	RRM requirements for UE-to-UE CLI handling	Nokia, Nokia Shanghai Bell

R4-2500467	RRM requirements for SBFD operation	Nokia, Nokia Shanghai Bell

R4-2500468	Simulation result for L1-CLI-SRS-RSRP measurement	Nokia, Nokia Shanghai Bell

R4-2500474	Fair coexistence between SBFD and TDD networks	Charter Communications, Inc

R4-2500536	Topic summary for [114][222] NR_duplex_evo	Moderator (Huawei)

R4-2500567	Topic summary for [114][306] NR_duplex_evo_General	Moderator (Samsung)

R4-2500568	Topic summary for [114][307] NR_duplex_evo_BSRF	Moderator (Huawei)

R4-2500737	Discussion on the subband configurations and guardbands for gNB SBFD	vivo

R4-2500865	Discussion on SBFD general part	CMCC

R4-2500866	Discussion on new RF requirement for SBFD	CMCC

R4-2500867	Discussion on existing Tx requirements for SBFD	CMCC

R4-2500868	Discussion on existing Rx requirements for SBFD	CMCC

R4-2500911	Discussion on UE-to-UE CLI of R19 SBFD	ZTE Corporation, Sanechips

R4-2500912	Discussion on other RRM impacts of R19 SBFD	ZTE Corporation, Sanechips

R4-2501025	Discussion on SBFD general issues	CATT

R4-2501026	Discussion on modification of existing Tx requirements for SBFD operation	CATT

R4-2501027	Discussion on modification of existing Rx requirements for SBFD operation	CATT

R4-2501028	Discussion on potentially new requirements for SBFD operation	CATT

R4-2501042	Discussion on UE to UE CLI handling	CATT

R4-2501043	Discussion on impacts of SBFD on RRM requirements	CATT

R4-2501087	Discussion on RRM impacts for SBFD operation	CMCC

R4-2501088	Discussion on RRM requirements for UE-to-UE CLI handling	CMCC

R4-2501113	Discussion for RRM requirements for UE-to-UE CLI handling	Ericsson

R4-2501114	Simulation results for L1-SRS-RSRP	Ericsson

R4-2501115	Discussion for RRM requirements for SBFD operations	Ericsson

R4-2501266	Discussion on UE CLI measurement for SBFD	Huawei, HiSilicon

R4-2501267	Additional simulation results for UE CLI measurement	Huawei, HiSilicon

R4-2501268	Discussion on RRM impacts for SBFD operation	Huawei, HiSilicon

R4-2501324	On existing BS RF RX requirements for SBFD operation	Nokia

R4-2501325	On existing BS RF TX requirements for SBFD operation	Nokia

R4-2501326	On potentially new BS RF requirements for SBFD operation	Nokia

R4-2501327	On SBFD system parameters	Nokia

R4-2501333	Discussion on RRM requirements for UE-to-UE CLI handling for evolution of NR duplex operation	China Telecom

R4-2501334	Discussion on RRM impacts for SBFD operation for evolution of NR duplex operation	China Telecom

R4-2501415	Further consideration on RRM requirements for UE-to-UE CLI handling	vivo

R4-2501416	Further consideration on RRM impacts for SBFD operation	vivo

R4-2501464	Views on L1 CLI measurement requirements	Samsung

R4-2501465	Views on RRM requirements for Rel-19 SBFD operation	Samsung

R4-2501696	Views on general aspects for SBFD	Qualcomm Germany

R4-2501697	Views on existing BS RF requirements for SBFD	Qualcomm Germany

R4-2501708	views on new BS RF requirements for SBFD	Qualcomm Germany

R4-2501840	On general aspects for SBFD	Huawei, HiSilicon

R4-2501841	On potentially new requirements for SBFD	Huawei, HiSilicon

R4-2501855	On modification of existing TX RF requirements for SBFD	Huawei, HiSilicon

R4-2501856	On modification of existing RX RF requirements for SBFD	Huawei, HiSilicon

R4-2501883	Discussion on system parameters for SBFD BS	ZTE Corporation, Sanechips

R4-2501884	Discussion on potentially new requirements for SBFD operation	ZTE Corporation, Sanechips

R4-2501885	Discussion on modification of existing Tx requirements for FR1 and FR2-1 for SBFD BS	ZTE Corporation, Sanechips

R4-2501886	Discussion on modification of existing Rx requirements for FR1 and FR2-1 for SBFD BS	ZTE Corporation, Sanechips

R4-2501943	Discussion on MPR impact on SBFD UL resource muting	Ericsson

R4-2501967	Further discussion on general aspects for SBFD system	Samsung

R4-2501968	On the potentially new requirements for SBFD operation for FR1 and FR2-1	Samsung

R4-2501969	On the modification of existing TX requirements for SBFD-capable BS	Samsung

R4-2501970	On the modification of existing RX requirements for SBFD-capable BS	Samsung

R4-2502082	Discussion on SBFD UE-to-UE CLI	MediaTek inc.

R4-2502083	Discussion on SBFD simulation performance	MediaTek inc.

R4-2502084	Discussion on SBFD Operation	MediaTek inc.

R4-2502155	SBFD general aspects	Ericsson

R4-2502156	Potentially new SBFD BS RF requirements	Ericsson

R4-2502157	Impact on SBFD BS RF TX requirements	Ericsson

R4-2502158	Impact on SBFD BS RF RX requirements	Ericsson

R4-2502230	RRM requirements for SBFD operation	Qualcomm Incorporated

R4-2502231	RRM requirements for UE-to-UE CLI handling	Qualcomm Incorporated

R4-2502250	Cross Link Interference (CLI) handling impact on MPR for SBFD operation	Qualcomm Incorporated

R4-2502297	Ad-hoc meeting minutes for [114][307] NR_duplex_evo_BSRF	Huawei

R4-2502307	Ad-hoc meeting minutes for [114][306] NR_duplex_evo_General	Samsung

R4-2502310	Way Forward for [114][306] NR_duplex_evo_General	Samsung

R4-2502311	Way Forward for [114][307] NR_duplex_evo_BSRF	Huawei

R4-2502380	Way Forward for [114][307] NR_duplex_evo_BSRF	Huawei

R4-2502573	Ad-hoc minutes for NR_duplex_evo WI	Samsung

R4-2502597	WF on RRM requirements for NR_duplex_evo	Samsung

R4-2502632	LS on L1 CLI measurement 	Huawei

R4-2502633	Updated summary of simulation results for L1-SRS-RSRP	Huawei

R4-2502668	Ad-hoc #2 minutes for NR_duplex_evo WI	Samsung

R4-2502699	WF on RRM requirements for NR_duplex_evo	Samsung

17.10.2024		minor adaptations for RAN #106 and 2.6 for RAN6 removed

12.08.2024		minor adaptations for RAN #105

30.04.2024		minor adaptations for RAN #104

16.02.2024		minor adaptations for RAN #103

10.11.2023		minor adaptations for RAN #102

02.08.2023		minor adaptations for RAN #101

26.04.2023		minor adaptations for RAN #100

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template