3GPP TSG RAN Meeting #1	07                                                RP-250544

Incheon, Korea, March 12-14, 2025

Title: 	Discussion on A-IoT in Rel-19

Source: 	ZTE Corporation, Sanechips

Agenda item:	9.3.1.7

Document for:	Discussion and decision

Introduction

In RAN#106 meeting, the work item [1] for Ambient IoT was approved. RAN WGs have started the normative work since 2025 Q1. In this contribution, the Rel-19 A-IoT issues are discussed.

Discussion

FEC in D2R

- RAN1 scope: ...............................................................................omitted........................................................................................... - R2D does not support FEC. D2R supports only convolutional code with generator polynomials as per TS 36.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis). - PRDCH and PDRCH both support transmission without CRC, and with CRC as per the generator polynomials in TS 38.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis) for 6-bit CRC and 16-bit CRC. Cases to use which length of CRC, or no CRC, to be decided in RAN1. - D2R supports physical layer repetition transmission. R2D does not support physical-layer repetition transmission. ..............................................................................omitted...........................................................................................

- Based on the RAN1 WI scope highlighted above, D2R supports FEC. However, whether the FEC is optional or mandatory in D2R has been discussed in RAN1 without consensus and hence needs further guidance from plenary.

- In our view, it is clear that WID doesn’t mandate to always enable FEC in D2R. Based on TR38.769, the following can be observed.

- Whether to apply FEC in the D2R coverage evaluation is up to companies’ preference.

- According to the observation in subclause 7.1.1.1.2, the coverage of device 1 can meet the design target evaluation with or without D2R FEC.

- The block diagram of PDRCH in subclause 6.1.2.5 is applicable to the case when FEC is present as it is highlighted.

- Therefore, FEC is not mandatory in D2R according to the study during the SI phase.

WID/TR38.679 doesn’t mandate to always enable FEC in D2R.

- 4.2.2 Evaluation assumptions ...............................................................................omitted........................................................................................... Table 4.2.2-1: Coverage evaluation assumptions for link-level simulation D2R specific parameters. [2g]; FEC; Companies to report, e.g., CC, No FEC. ...............................................................................omitted........................................................................................... - 6.1.2.5 PDRCH For D2R, a physical channel PDRCH carries any higher-layer payload, the response transmitted from device to reader during the contention-based access procedure, and L1 D2R control information, if defined. PDRCH is studied via the blocks shown in Figures 6.1.2.5-1 and 6.1.2.5-2, where other clauses give their detailed descriptions and the location of the "Repetition" block depends on the repetition types described in Clause 6.1.0.1. Figure 6.1.2.5-1: Single PDRCH generation at the device with line coding, for the case where FEC and repetition are present NOTE: Presence of the block "small frequency shift" depends on the options for small frequency shift described in clause 6.1.2.7.1. Figure 6.1.2.5-2: Single PDRCH generation at the device without line coding, for the case where FEC and repetition are present. ...............................................................................omitted........................................................................................... - 7.1.1 Link budget for D1T1 - 7.1.1.1 Device 1 - 7.1.1.1.2 Observations ...............................................................................omitted........................................................................................... Table 7.1.1.1.2 Device 1; D1T1-B. FEC code (1/2, 1/3, 1/4); [6.9 ~ 36.59] meters. (11: [193]:36.59, [207]:16.11, [192]:24.4, [203]:23.52, [205]:36.59, [210]:17.8, [190]:16.03, [202]:14.43, [209]:27.11, [194]:30.11, [213]:6.9). without FEC; [3.3 ~ 41.28] meters. (12: [199]:41.28, [215]:19.77, [200]:15.17, [191]:17.22, [201]:25.66, [212]:19.57, [211]:3.3, [190]:16.03, [214]:11.69, [196]:21.7, [198]:16.03, [213]:6.89). ...............................................................................omitted...........................................................................................

- Moreover, based on the relationship between transmission bandwidth and data rate with different coding schemes in Table 1, it can be observed that when FEC is disabled, the A-IoT D2R can achieve a data rate of 645kHz at device clock frequency of 2.58MHz, which are comparable to other techniques. However, if convolutional code is always mandated, the A-IoT device needs to be equipped with much higher clock frequency, i.e., 5~7MHz, as it is highlighted in Table 1. Thus, the device complexity, cost, and power consumption would increase accordingly, which is beyond target device type in Rel-19 WI. Hence, FEC should be disabled when a peak data rate comparable with other technologies is pursued. And the peak/higher data rate can be used in the scenarios with sufficiently good coverage, which can be based on R2D  indication by reader.

If convolutional code is always mandatory, the A-IoT device needs to be equipped with much higher clock frequency, i.e., 5~7MHz, which would increase device complexity, cost, and power consumption.

- Table 1 D2R transmission bandwidth and data rate

| D2R transmission bandwidth (double sideband, kHz) | Chip duration (without small frequency shift,µs) | Data rate (kbps) with Manchester code | Data rate (kbps) with Manchester code +1/2 CC | Data rate (kbps) with Manchester code+1/3 CC |
| --- | --- | --- | --- | --- |
| 15 | 133.33 | 3.75 | 1.875 | 1.25 |
| 1920 | 1.04 | 480 | 240 | 160 |
| 2580 | 0.78 | 645 | 322.5 | 215 |
| 3840 | 0.52 | 960 | 480 | 320 |
| 4500 | 0.44 | 1125 | 562.5 | 375 |
| 5250 | 0.38 | 1312.5 | 656.25 | 437.5 |
| 6000 | 0.33 | 1500 | 750 | 500 |
| 7680 | 0.26 | 1920 | 960 | 640 |

FEC in D2R can be disabled by reader.

Another issue is whether to support flexible FEC coding rate than the 1/3 mother code rate of convolutional code. In our view, supporting other code rate, such as 1/2, can offer higher resource efficiency when the coverage allows. Meanwhile, it offers coding gain than simple repetition without FEC.

Supporting other FEC code rate, such as 1/2, can offer higher resource efficiency when the coverage allows. Meanwhile, it offers coding gain than simple repetition without FEC.

Support FEC coding rate of 1/2 by puncturing.

Temporary identifier for A-IoT paging

- In the A-IoT WID, it is assumed that “Temporary identifier is not supported, unless required by SA WGs.”. Subsequent to this, SA WGs discussed this issue and informed RAN2 that temporary identifier is supported as follows:

The following aspects and principles are agreed for the conclusion on KI#3 - A mechanism to protect AIoT device ID based on the use of temporary ID shall be supported. - Mechanism shall allow unambiguous identification of the AIoT device - A mechanism to re-synchronize de-synchronized temporary IDs shall be supported.

- Thus, it is clear that use of temporary ID for the device will be supported from upper layer perspective. However, from RAN perspective, this should have no impact to the general design. RAN2 procedures support only inventory and inventory+command use cases. So, the mechanisms to use and update the temporary ID at the UE from NAS perspective will have to use the command procedure. Thus, the allocation and maintenance of temporary ID in NAS should be transparent to AS procedures. Thus, the sentence in A-IoT WID about temporary ID can simply be removed.

Remove the sentence in WID that says “Temporary identifier is not supported, unless required by SA WGs.” - with the understanding that the NAS procedures for temporary ID management are transparent to AS and can be implemented using command procedure in AS.

A-IoT specification

- Discussion about how to capture A-IoT specification in RAN1, RAN2 and RAN3 is needed according to the following note in WID.

- Note: whether a new specification in 38 series will be used instead will be decided at RAN#107

- In general, it should be possible for A-IoT technology to evolve independently from other NR features. For topology 1, A-IoT operation is totally independent to other NR features. However, from Rel-20, with topology 2, there will be some inter-dependencies with RRC especially for Uu operation to control the UE based reader. Given this situation, it is possible either to have separate specs for A-IoT or to reuse the existing specs. However, if existing specs are used for A-IoT, it should be captured in such a way that the NR procedures are not applicable to A-IoT. For instance in MAC spec, when the MAC entity is referenced, it should be clear that this is not referring to an A-IoT MAC entity, Similarly in case of NGAP specs, RAN3 has achieved a working assumption to include AIoTF information in NGAP. Similar approach to include A-IoT functionality can also be adopted for other specs too.

If existing specs are used for A-IoT, the A-IoT procedures should be captured in a separate section such that the existing NR procedures are not applicable or impacted.

Conclusion

Based on the above discussion about Rel-19 A-IoT, we have the following observations and proposals:

- WID/TR38.679 doesn’t mandate to always enable FEC in D2R.

If convolutional code is always mandatory, the A-IoT device needs to be equipped with much higher clock frequency, i.e., 5~7MHz, which would increase device complexity, cost, and power consumption.

Supporting other FEC code rate, such as 1/2, can offer higher resource efficiency when the coverage allows. Meanwhile, it offers coding gain than simple repetition without FEC.

- FEC in D2R can be disabled by reader.

Support FEC coding rate of 1/2 by puncturing.

Remove the sentence in WID that says “Temporary identifier is not supported, unless required by SA WGs.” - with the understanding that the NAS procedures for temporary ID management are transparent to AS and can be implemented using command procedure in AS.

- If existing specs are used for A-IoT, the A-IoT procedures should be captured in a separate section such that the existing NR procedures are not applicable or impacted.

References

- RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, RAN#106

- TR38.769 V19.0.0. Study on solutions for Ambient IoT (Internet of Things) (Release 19)