3GPP TSG RAN WG meeting #107	                                          RP-250545

Incheon, Korea, March 12-14, 2025

# Title: 	Discussion on XR enhancements

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	9.3.2.3

# Document for:	Discussion and decision

# Introduction

In this paper, we discuss the conclusions made in RAN2/RAN for the multi-modality related objectives and propose the updated objective for the WI. Further, during the discussion in RAN2 for RAN controlled rate adaptation for XR, it was agreed that the XR rate control is possible on a per QoS flow basis. RAN3 work is needed to support this work too and hence the WI objectives need to reflect this.

# Multi-modality

The WI for XR enhancements in Rel-19 has a check-point in RAN#107 to check the progress of multi-modality related objective in the WID:

RP-243318: - Specify support for multimodality in RAN for UL and DL [RAN3]: NOTE: This is subject to alignment with SA2, e.g., if MMSID is not available from CN, then UE assistance information-based approach as an alternative. RAN#107 to check handling of uplink discard based on SA2/SA4 outputs on whether the corresponding information is available at the UE. NOTE: Review in RAN#107 the conclusions in SA2 side (including MMSID delivery to RAN) to allocation for further TUs in RAN3 from Q2/25 onwards.

At SA2#167, it was agreed that the CN can provide the MMSID to RAN and the corresponding CRs were agreed in SA2 – see S2-2502465.

Thus, RAN3 can now go ahead and agree the necessary changes for the RAN awareness of multi-modality. So, the above notes can be deleted from the XR WID and the necessary TUs can be added for RAN3.

Proposal 1: Based on the agreements in SA2 and RAN2, the work on multi-modality awareness in RAN can start with TUs allocated (i.e., 0.5 TU per RAN3 meeting) for the corresponding work in RAN3 from Q2, 2025.

# RAN exposure/RAN involved rate control for XR

At RAN2#128, The following agreements were reached for XR rate control:

Agreements on XR rate control - RAN2 confirms it is feasible for RAN to estimate the congestion information at both per-DRB and per-QoS flow level. - gNB can be indicated which QoS flows can be throttled. FFS whether this is indicated from UE/CN - Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB.

Further to the above agreements, RAN3 discussed RAN based XR rate control and confirmed that the per QoS flow level rate control mechanism agreed by RAN2 using either the per QoS flow or per DRB MAC CE has an impact on the F1 interface– see the RAN3 LS in R2-2501508. Thus, objective for XR rate awareness should also include RAN3 work.

Proposal 2: Modify the existing objective for RAN based XR rate control to support RAN3 work.

# SA2 progress

Based on the current SA2 progress, normalization of the following aspects may need further discussion in RAN WGs.

- Key issue#1

The following content has been agreed in SA2 and captured in the updated TR 23.700-70.

1.	In the case of no DL PDU Set QoS parameters provided by the AF, according to the Protocol Description and PSA UPF supports of PDU Set Information marking, the SMF sends NG-RAN a new indicator indicating the DL PDU Set Information Marking is supported. The SMF activates the PDU Set Information marking based on information from NG-RAN.

It is clear that SMF may send the DL PDU Set Information information to NG-RAN node to show this function is supported by SMF.

In addition, based on the received LS(R3-247113) from SA2, RAN3 further discussed how to transmit the PDU Set based Handling information between SMF and NG-RAN. No consensus has been made in RAN3 on how does SMF and NG-RAN send the PDU Set handling information. Hence, it is clear that at least there is stage 2 impact in RAN3 to capture the SA2 decision in the other PDU Set Handling. Whether any stage 3 enhancement on this aspect depends on SA2 further progress.

- Key issue#5

Time to Next Burst information has been defined by SA2 which is used to provide the time interval between the current burst and the next burst in the 1st packet of the burst via N6. SA2 also sent LS(S2-2407351) to SA4 and asked whether it is feasible for the server to provide such info. Besides, the Burst Size has also been defined by SA2 and introduced in SA2 TR. If this is to be supported in RAN, then signalling needs to be enabled by RAN3. Detailed impact at RAN interfaces may be further discussed in related WGs (e.g., RAN3, SA2, SA4).

- Key issue#9

In the reply LS(S2-2412720), SA2 answers the RAN3 question on the Available Data Rate and further clarify SA2 understanding on how to use this information for XR data transmission. In addition, the following content has been captured in the updated TR 23.700-70.

-	The NG-RAN reports the available data rate to the UPF via the GTP-U header of UL packets and UPF further exposes, based on mechanisms defined in clause 5.8.2.17 of TS 23.501 [2], and the available data rate is reported to the AF directly or via the local NEF.

-	Available data rate refers to the data rate that RAN is able to provide within a duration. Such duration is based on RAN implementation.

NOTE 1:	It is recommended that the available data rate exposure is not reported very frequently.

NOTE 2:	How the RAN measures the available UL and/or DL data rate provided by the RAN node is to be refined based on the potential feedback from RAN WG3.

In RAN3#127 meeting, RAN3 responded to the SA2 LS in R3-250806. RAN3 discussed the usage of the available data rate and agreed that this information, reported to the CN, refers to the data rate that the gNB is currently able to provide for a QoS flow with GFBR. The derivation of the available data rate value is dependent on the gNB's implementation. Additionally, RAN3 understands that it is up to the CN configuration whether the gNB uses the available data rate in conjunction with the Alternative QoS Profiles mechanism. Therefore, RAN3 may further discuss the details and introduce the Available Data Rate information in future RAN3 meetings based on the progress of SA2.

Proposal 3: Add the following objective for normative work on XR in Rel-19 [RAN3]:

- Support PDU Set Information Marking without QoS parameters;

- Identify signalling impacts to support of time to next burst/burst size information and exposure of available data rate to CN, if any

- Signaling support towards multi-modality awareness.

# Conclusion

Proposal 1: Based on the agreements in SA2 and RAN2, the work on multi-modality awareness in RAN can start with TUs allocated (i.e., 0.5 TU per RAN3 meeting) for the corresponding work in RAN3 from Q2, 2025.

Proposal 2: Modify the existing objective for RAN based XR rate control to support RAN3 work.

Proposal 3: Add the following objective for normative work on XR in Rel-19 [RAN3]:

- Support PDU Set Information Marking without QoS parameters;

- Identify signalling impacts to support of time to next burst/burst size information and exposure of available data rate to CN, if any.

- Signaling support towards multi-modality awareness.

The following updates to the objectives are proposed as a result:

The Rel-19 XR Phase 3 objectives are as follows: - Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4] - Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4] - Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]: - Specify additional Logical Channel priority handling using delay/deadline information of packets; - Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG. - Specify the following user plane enhancements [RAN2]: - RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget. - Extend Release 18 standalone mechanism to support NR-NR dual connectivity as follows [RAN3]: - PDU set based handling; - ECN marking; - Burst Arrival Time reporting, if needed; - PSI Discard coordination, if needed; NOTE: No RAN2 impact from above items. - Specify uplink congestion signalling [RAN2, RAN3]: - Specify in MAC layer XR rate control signalling over downlink per QoS flow and the associated F1 signalling to enable faster source rate adaption to uplink congestion. - Specify Core requirements related to the above objectives as necessary [RAN4]. - Support the following objectives as concluded by SA2 [RAN3]: - Support PDU Set Information Marking without QoS parameters. - Identify signalling impacts to support time to next burst/burst size information and exposure of available data rate to CN, if any. - Signalling support for multi-modality awareness

# References

- RP-240791 Revised WID on XR (eXtended Reality) for NR Phase 3	RAN#103