3GPP TSG RAN Meeting #107						                                                             RP-250546

Incheon, Korea, March 12-14, 2025

Source:	ZTE

Title:	Discussion on PWS Support for NB-IoT NTN

Agenda Item:	10.3.1

Document for: Discussion

- Introduction

In the WID of Rel-19 [1], the broadcast of PWS messages was agreed to be supported for NB-IoT NTN and specified by RAN2.

- Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2] Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI. Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome.

In this contribution, the views on the above objective are elaborated.

- Discussion on PWS support in RRC_CONNECTED for NB-IoT NTN

In RAN#105 [1], the updated WID is endorsed to include support of PWS messages for NB-IoT as part of scope in RAN2. In RAN2#128, no consensus was reached in the corresponding topic, i.e., whether to introduce support for reception of PWS (ETWS and CMAS) notifications in RRC_CONNECTED for NB-IoT. Then RAN2 sent a LS [2] to RAN1 asking whether it is feasible from RAN1 perspective to introduce support in Rel-19 for NB-IoT UEs reception of PWS notifications (in paging, direct indication, and system information) while in RRC_CONNECTED.

In RAN1#120, as reflected in the reply LS [3],  the following content is provided:

From RAN1 perspective, it may be possible to find solutions with specification impact for supporting NB-IoT UEs in RRC connected mode monitor PWS notifications, while some companies expressed concerns due to increased implementation complexity, power consumption at UE side and timely delivery of PWS notifications in NTN context. However, the amount of RAN1 workload for specifying this may not fit in the Rel-19 TU allocation for NTN. RAN1 would like to ask RAN plenary to decide whether RAN1 should work on specifying support of NB-IoT UEs in RRC connected mode monitor PWS notifications in Rel-19.

Based on the feedback from RAN1, it’s clear that the main concern is related to the “workload vs. limtied TU” in Rel-19, and the RAN guidance is definitely needed.

Observation 1: To address the RAN1’s concern on the workload and TU, RAN-level guidance is needed.

As justified in the WID [1], satellite is by nature a broadcast medium and can quickly reach very large coverage areas when TN is down:

Support emergency broadcast messaging: Satellite is by nature a broadcast medium, that can quickly reach very large coverage areas even when terrestrial networks are down. Delivery of same data to a very large UE population in the same area via Unicast is not scalable. Therefore, supporting emergency broadcast messaging is beneficial and should be supported.

Therefore, supporting the reception of PWS is very beneficial and suitable in NTN system. Considering the typical use case of the NB-IoT device, especially future-proof case (e.g., voice), the UE in may keep in RRC_CONNECTED state for a long time. If only PWS reception in RRC_IDLE state is supported for NB-IoT NTN, additional delay of PWS notification will be introduced due to the state change before reception of PWS message. For emergency broadcast messaging, the reduction of PWS reception delay is really important especially for the earthquake and tsunami. Considering longer propagation delay in NTN, reduction of PWS reception delay caused by other procedures will be even more important. Hence, it is necessary to support PWS in RRC_CONNECTED for NB-IoT NTN to enable a timely reception of PWS notification.

Observation 2: Supporting PWS is beneficial and is justified in WID of Rel-19 IoT-NTN.

Observation 3: Reduction of notification delay is important for PWS in NTN especially for earthquake and tsunami.

Observation 4: If PWS reception is not supported for RRC_CONNECTED state, PWS notification delay for UE in RRC_CONNECTED state will increase significantly due to additional state change procedure.

Proposal 1: Reception of PWS notifications in RRC_CONNECTED for NB-IoT NTN should be supported to enable a timely reception of emergency message.

Regarding the detailed solution, as shown in WID above, PWS messages for NB-IoT is supported by reusing the LTE mechanisms, i.e., PWS notification is via paging, direct indication, and system information. However, in current NB-IoT specification, UE is not required to monitor paging or direct indication during RRC_CONNECTED state. In order to support reception of PWS notification in RRC_CONNECTED, UE should be able to monitor NPDCCH for paging during RRC_CONNECTED state, which will have RAN1 spec impact. However, it can be handled with limited efforts, e.g., new UE capabilities.

To address this issue, the following RAN level guidance can be considered:

- Option-1: Update the WID by including RAN1 as the secondary working group;

- Option-2: Keeping the existing WID unchanged and RAN1 will handle the corresponding works triggered by RAN2 LS.

Proposal 2: The following RAN-level guidance can be considered:

- Option-1: Update the WID by including RAN1 as the secondary working group;

- Option-2: Keeping the existing WID unchanged and RAN1 will handle the corresponding works triggered by RAN2 LS.

- Conclusion

- In this contribution, the detailed views on PWS Support for NB-IoT are provided with the following observations and proposals:

Observation 1: To address the RAN1’s concern on the workload and TU, RAN-level guidance is needed.

Observation 2: Supporting PWS is beneficial and is justified in WID of Rel-19 IoT-NTN.

Observation 3: Reduction of notification delay is important for PWS in NTN especially for earthquake and tsunami.

Observation 4: If PWS reception is not supported for RRC_CONNECTED state, PWS notification delay for UE in RRC_CONNECTED state will increase significantly due to additional state change procedure.

Proposal 1: Reception of PWS notifications in RRC_CONNECTED for NB-IoT NTN should be supported to enable a timely reception of emergency message.

Proposal 2: The following RAN-level guidance can be considered:

- Option-1: Update the WID by including RAN1 as the secondary working group;

- Option-2: Keeping the existing WID unchanged and RAN1 will handle the corresponding works triggered by RAN2 LS.

- References

- RP-242397, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, RAN#105.

- R2-2411194, LS on PWS support in RRC_CONNECTED for NB-IoT NTN, RAN2#128.

- R1-2501613, Reply LS on PWS support in RRC_CONNECTED for NB-IoT NTN, RAN1#120.