3GPP TSG RAN Meeting #1	07                                                RP-250550

Incheon, Korea, March 12-14, 2025

Title: 	Views on Ambient IoT enhancement in 5G-A Rel-20

Source: 	ZTE Corporation, Sanechips

Agenda item:	15.2.3

Document for:	Discussion and decision

Introduction

In RAN#106 meeting, the way forward [1] and work item [2] for Ambient IoT were approved. The general scope for Rel-19 work item and Rel-20 work plan are provided as below:

General Scope for Rel-19 WI The definitions provided in TR 38.848, TR 38.769, and decisions, etc. made during the Rel-19 SI in RAN WGs are taken into this WI, and the following is the exclusive general scope: - The overall objective shall be to standardize the following A-IoT device: Device 1: ~1 µW peak power consumption, has energy storage, RF envelope detector receiver, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally. - Deployment scenario 1 with Topology 1, according to D1T1-B. - FR1 licensed spectrum in FDD, with R2D in DL spectrum and D2R and CW in UL spectrum. - Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor. - Traffic types DO-DTT, DT, for rUC1 (indoor inventory) and rUC4 (indoor command). - Carrier wave transmission for waveform 1 only, without hopping, per the following cases in TR 38.769: - Case 1-4 for D1T1-B - Proximity determination via Solution 1 in TR 38.769 only. - Device (un)availability via Direction 1 in TR 38.769 only. Work plan for Rel-20 - Includes D2T2 - Includes Device 2 - A limited number of additional use cases, deployments, connectivity topologies, devices, traffic types in TR38.848 as a starting point - Note: work on D2T2, device 2 shall follow the Rel-19 study conclusions, and further down-selection of architecture options for topology 2 may be needed. For indoor use cases, deviations from Rel-19 study conclusion shall be well justified and agreed. - Details to be discussed at a later RAN plenary meeting, including the need for study phase.

In this contribution, the potential enhancements for Rel-20 A-IoT are discussed.

Discussion

Use cases

- According to RAN studies, eight representative use cases (rUCs) have been formulated for A-IoT. The mapping from SA1 use cases and traffic scenarios onto these RAN rUCs is captured in Table 4.1.1-1 of TR38.848 [3]. The eight rUCs inculdes:

- rUC1: Indoor inventory

- rUC2: Indoor sensors

- rUC3: Indoor positioning

- rUC4: Indoor command

- rUC5: Outdoor inventory

- rUC6: Outdoor sensors

- rUC7: Outdoor positioning

- rUC8: Outdoor command

- Wherein, the solutions to supporting rUC1 (indoor inventory), rUC4 (indoor command) and rUC3 (indoor positioning) were studied and specified in Rel-19 .

- It is evident that Rel-19 A-IoT focuses on indoor scenarios and a significant number of representative use cases were not considered. Extending A-IoT from indoor to outdoor scenarios is crucial for optimizing efficiency and reducing maintenance costs in applications such as automated supply chain management, logistics, and smart grids, etc. Moreover, the combination of A-IoT with extremely low cost/complexity with wide-area coverage will provide a compelling advantage over other technologies with limited coverage and narrower use cases.

- The A-IoT sensors can be used to report real-time monitoring data, such as temperature, humidity, which requires devices to autonomously initiate D2R transmission (i.e.,DO-A traffic) with larger message sizes. However, due to the limited TUs, DO-A traffic was deprioritized in Rel-19.

- Additionally, the design target of positioning has been provided in TR38.848, but the locating introduced in Rel-19 is limited to reader ID granularity. Therefore, A-IoT positioning solutions with higher accuracy need to be studied to fulfill the RAN design target.

- In summary, inventory, sensors, positioning and command should be considered for outdoor scenarios in Rel-20. Meanwhile, the indoor inventory/command can be further enhanced for D2T2 according to the approved WF in RAN#106.

- Support the following use cases in Rel-20：

- Outdoor inventory/command,

- Outdoor/indoor sensors/positioning,

- Indoor inventory/command with enhancement for D2T2.

Device types

- In Rel-19 RAN1 SI, three device types, device 1, 2a and 2b, were studied for A-IoT. The basic blocks of device architectures based on RF-ED receivers for devices 1, 2a, and 2b have been defined, as well as those for receiver architectures based on IF-ED or ZIF for device 2b. Additionally, the purposes and characteristics (including clock speed, power consumption and accuracy, etc.) of the clocks used by different A-IoT devices have been identified.

- Co-existence with NR and providing similar coverage as NR are critical to extend the A-IoT use cases to outdoor. Device 1, 2a, and 2b have different peak power consumption, complexities and capabilities, which allows them to meet various application requirements. Therefore, the applicable device types should be identified.

Co-existence with NR and providing similar coverage as NR are critical to extend the A-IoT use cases to outdoor.

- Based on the evaluation of indoor scenarios in Rel-19, co-existence between A-IoT and NR is feasible when NR UEs are not deployed indoors alongside A-IoT devices. But if NR UEs are present indoors along with A-IoT devices, the interference caused by their co-existence will result in obvious NR capacity reduction and/or A-IoT SINR performance degradation. Therefore, A-IoT and NR systems should be operated with indoor/outdoor separation or to adopt other techniques to mitigate interference. For outdoor scenarios, the interference between macro cells and devices becomes more serious, leading to significant performance degradation in R2D link or NR. Solutions to enable better co-existence between A-IoT and NR are needed. For example, narrower RF filters can help A-IoT devices filter out unwanted frequency-domain signals, device without CW can resolve the CW interference issues, thereby improving overall performance.

- Solutions to enable better co-existence between A-IoT and NR need to be considered for outdoor scenarios.

- In Rel-19 WI, only device 1 was supported for A-IoT. The maximum coverage of device 1 is up to tens of meters, which doesn’t meet outdoor coverage requirements. Thus, device 1 cannot be supported for outdoor deployment.

- For device 2a, high CW transmission power is required to improve the coverage. However, in outdoor scenarios, this will cause significant interference to the NR system. Additionally, since device 2a may have a large receiving bandwidth of 20MHz, NR signals from neighboring frequency could also interfere with R2D reception of device 2a. In other words, supporting device 2a will face larger coexistence challenges with NR in outdoor scenarios. Therefore, device 2a should be deprioritized in Rel-20.

- For device 2b, since CW is not needed and an RF BPF can be implemented, the coexistence impact on NR is minimal. Moreover, considering that device 2b can support larger coverage distance, it should be introduced in Rel-20. In addition, a device type (device C) with higher power consumption (several mWs) described in TR38.848 could also be considered.

- Device 2b/C is introduced to support outdoor scenarios in Rel-20 A-IoT.

Deployment scenarios and Connectivity topologies

- In TR38.848, the following deployment scenarios and connectivity topologies are defined for A-IoT networks.

- Deployment scenario

- Deployment scenario 1: Device indoors, basestation indoors

- Deployment scenario 2: Device indoors, basestation outdoors

- Deployment scenario 3: Device indoors, UE-based reader

- Deployment scenario 4: Device outdoors, basestation outdoors

- Deployment scenario 5: Device outdoors, UE-based reader

- Conectivity topologies

- Topology 1: BS ↔ Ambient IoT device

- Topology 2: BS ↔ intermediate node ↔ Ambient IoT device

- Topology 3: BS ↔ assisting node ↔ Ambient IoT device ↔ BS

- Topology 4: UE ↔ Ambient IoT device

- In Rel-19 SI, Deployment scenario 1 with Topology 1 (D1T1) and Deployment scenario 2 with Topology 2 (D2T2) were studied and evaluated for A-IoT. Specifically, D1T1 and D2T2 are further categorized into several scenarios in Table 1 considering different CW deployments:

- Table 1: Topologies for D1T1 and D2T2

|  | - D1T1 | - D2T2 |
| --- | --- | --- |
| - CW inside topology | - D1T1-A1, D1T1-A2 | - D2T2-A1, D2T2-A2 |
| - CW outside topology | - D1T1-B | - D2T2-B |
| - No CW | - D1T1-C | - D2T2-C |

- Among the scenarios listed in Table 1, only D1T1-B is supported in Rel-19 WI. Given that the CW interference capability of UE readers is lower than of BS reader due to the limited form factors of UEs, D2T2-B/C without the need of self-interference cancellation can be considered. However, D2T2-B requires separate deployment of a CW node, which complicates the coordination among gNB, UE reader and CW node. Thus, D2T2-C should be prioritized for indoor scenario.

- Furthermore, as discussed above, outdoors scenarios are essential for supporting more use cases and traffic. Therefore, some deployment scenarios and topologies for outdoor services should be considered in Rel-20 A-IoT. Specifically, D4T1 should be supported, as it is designed for large coverage. And D4T2 can be considered for relatively smaller coverage, which can reuse the physical design of D4T1 and higher layer protocol of D2T2.

- Additionally, when D4T2, D2T2-C and D1T1-B are supported, other scenarios, such as D1T1-C and D1T1/D2T2-A1/A2, can be implemented with minimalor even no specification impact.

D4T2 can be implemented by reusing the physical design of D4T1 and higher layer protocol of D2T2.

When D4T2, D2T2-C and D1T1-B are supported, D1T1-C and D1T1/D2T2-A1/A2 can be implemented with minimal or even no specification impact.

- The following connectivity topologies and device types can be considered for Rel-20 A-IoT.

- D4T1, D4T2 for device 2b/C outdoors

- D1T1-C, D2T2-C for device 2b indoors

Traffic type

- In Rel-19 A-IoT SI, three traffic types were considered: Device-terminated (DT), Device-originated device-terminated triggered (DO-DTT) and Device-originated autonomous (DO-A). The harmonized air interface design focused on DT and DO-DTT traffic types. Additionally, whether this design can address the DO-A traffic was also assessed. The conclusions from TR38.769 are shown as follows:

- From the RAN1 perspective, at least the following aspect of the air interface for DO-DTT and DT traffic types is not sufficient for the DO-A traffic type: For DO-DTT and DT traffic types, the D2R resource(s) for D2R transmission is/are indicated in a R2D transmission, but this is not applicable at least for the first D2R transmission for DO-A traffic. - From RAN2 perspective, the DO-A traffic type/use case cannot be supported with the current design in the study item. It is assessed that, from RAN2 perspective, at least the A-IoT paging is an aspect/part of the current design which is not sufficient for the DO-A use case.

- Thus, it is concluded that the current design is not applicable for DO-A traffic type. While the DO-A use case has been widely acknowledged for device to autonomously report environment monitoring information such as temperature, humidity, etc. Therefore, we propose to support the DO-A traffic type in Rel-20 A-IoT. The corresponding configurations and procedures for physical-layer and higher-layer needs to be specified.

- Support DO-A traffic type and specify the corresponding configuration and procedures in Rel-20.

Enhancements for Rel-20

Coverage/spectral efficiency enhancements

In Rel-19 SI, it was agreed that the maximum coverage target of device 1 is 15m for indoor scenarios (D1T1). To improve coverage performance and better utilize the higher capabilities of device 2b/C for outdoor cases, some potential enhancements are needed.

In Rel-19, given the limited capabilities of device 1, only binary modulation, i.e., OOK for the R2D link and OOK/BPSK for the D2R link, is utilized. Furthermore, FEC coding is not applied in the R2D link. In Rel-20, due to the inclusion of higher traffic load cases such as sensors, spectral efficiency enhancement is necessary to reduce latency. Additionally, considering the coexistence with NR systems, improving spectral efficiency can minimize the impact on existing NR systems. Therefore, in Rel-20, the enhancements like FEC for R2D transmission and higher modulation order such as QPSK for D2R transmission, can be considered to improve the spectral efficiency.

Therefore, the solutions to coverage and spectral efficiency enhancements are summarized as below.

- R2D link:FEC coding, repetition, enhanced device receiver, frequency hopping, etc.

- D2R link:higher modulation order, spreading code in time or frequency domain, frequency hopping, etc.

- Techniques to increase coverage and spectral efficiency should be considered in Rel-20 A-IoT.

Random access enhancement

In the evaluation of inventory completion time for multiple devices during SI phase, it is assumed that there are 600 A-IoT devices in indoor scenarios. The inventory completion time captured in TR 38.769  ranges from several seconds to tens of seconds based on varied assumptions. For outdoor cases with larger coverage, more devices are reachable within a reader's range. In R19 WI, FDMA is considered, wherein increasing the number of access devices is achieved by increasing the frequency domain resources.It is necessary to coexist with NR in outdoor scenario, the increase in AIoT frequency domain resources will affect NR capacity. Therefore, solutions to further increase network capacity and improve access efficiency should be considered, for example, CDMA based random access.

Moreover, for contention-based random access, the slotted-ALOHA algorithm was considered in Rel-19 to reduce collision. Under this approach, the device randomly selects one access occasion for the A-IoT Msg1 within the range of access occasions assigned by the reader. This method may still lead to conflicts, requiring re-access or multiple rounds of inventory. To address this issue, several enhanced anti-collision algorithms can be considered for Rel-20, e.g., combined algorithm of Q-selection and BTree, combined algorithm of Q-selection and ALOHA [6].

Alternatively, we understand deterministic access occasion selection can be considered to reduce access collision in multiple devices paging or group paging cases, e.g., access occasion selection based on device identity.

Furthermore, in Rel-19, RAN2 has studied and evaluated 2-step RA procedure. It is seen that 2-step RA provides significant gains at low and medium collision rates (see R2-2500778). 2-step RA procedure will also improve the latency of the overall RA procedure. Unified RA which includes both RN16 and the device ID in MSG1 was also considered as a compromise between the device complexity and the overall gains. However, given that new device types will be introduced in Rel-20 and low complexity devices can use the Rel-19 baseline, it is possible to directly consider 2-step RA as a possible candidate for A-IoT RACH in Rel-20.

- Solutions to increase network capacity and improve access efficiency should be considered for outdoor scenario, e.g., CDMA based random access, enhanced anti-collision algorithms, deterministic access occasion selection.

- 2-step RA procedure should be considered for Rel-20 A-IoT.

Regarding paging, the following scenarios have been acknowledged in the SI:

- Scenario 1: The reader may send multiple (subsequent) A-IoT paging messages that are associated with the same service request from the CN. The duplicated response from devices for the same service request should be avoided.

- Scenario 2: Different readers may send A-IoT paging messages, which are associated with the same service request from the CN, to the same device for response.

For the above scenarios, if it is possible to avoid duplicated response from the same device for the same service request, the inventory and command efficiency can be improved while access collision can be alleviated a little bit. There has been discussion in SI phase for these while further design and detailed discussion is still needed. Although Rel-19 may support some mechanisms to reduce the duplicate response from the same device for the same service, the Rel-19 procedures largely leave the multi-reader scenarios to implementation. Hence, coordinating duplicated responses across multiple readers may not be optimal in Rel-19. Thus enhanced mechanisms to avoid duplicate responses for same service can be considered in Rel-20.

- Enhanced mechanisms to reduce duplicated response from the same device for the same service request should be considered in Rel-20.

- Device (un)availability

Due to limited energy storage and energy harvesting capabilities, the device availability time may be shorter than its unavailability time.

According to the current slotted-ALOHA procedure, the device is required to randomly select a counter from the access occasions provided by reader and continuously monitor each R2D QueryRep-like signaling to decrease the counter until it equals to 0. This process significantly consumes device energy, especially for device type 2 with relatively high power consumption. In scenarios where involves large number of devices, the prolonged R2D detection time may cause many devices to deplete their charge before completing the access procedure. Therefore, device energy conservation becomes particularly important.

The following directions are studied to better utilize the limited device availability time due to energy harvesting, and solutions are captured in TR38.769.

- Direction 1: Reader does not provide information to a device regarding when the device may become available/unavailable.

- Direction 2: Reader can provide information to a device based on which the device may become available /unavailable.

However, only direction 1 is considered in Rel-19 WI. Given the challenges described above for device 2, Direction 2 should be further considered for device (un)availability in Rel-20.

- Direction 2 should be used for device (un)availability in Rel-20.

- Device synchronization

- According to the Rel-19 SI, the clock characteristics of device 2b have been captured as below in TR38.769. Therefore, improving the device clock accuracy is needed to enhance the spectrum efficiency and enable the D2R coherent detection.

| Clock purpose # | Description | Clock speed | Applicable devices | Power consumption | Initial clock Accuracy [ppm] | Accuracy after clock sync / calibration at device side [ppm] |
| --- | --- | --- | --- | --- | --- | --- |
| 5 | LO for carrier frequency (for up/down conversion) | According to carrier frequency e.g., 900 MHz | 2b | 10s - 100s of µW | 103 - 104 | Within 10s – 200 (Note 2), there is no need to determine a single value for the study |

- Improving the device clock accuracy is needed to enhance the spectrum efficiency and enable the D2R coherent detection.

Topology 2

According to the work plan for Rel-20 A-IoT, Topology 2 (TP#2) will be supported at least for D2T2. Thus, we have proposed to specify the following aspects for TP#2 [4]:

- RRC signaling to provision radio resources at the UE reader for A-IoT operation

- Dedicated signaling in RRC_CONNECTED state for configuring A-IoT radio resources

- Procedures to enable the UE to release the A-IoT resources upon cell change or moving to out of coverage

- UE assistance information from the reader to gNB for configuration of A-IoT radio resources

- As a second priority, support operation of A-IoT whilst the UE reader is in one of RRC_INACTIVE state or RRC_IDLE state (RAN2 to down select between INACTIVE and IDLE states)

- For TP#2, specify RRC signaling to provision radio resources at the UE reader for A-IoT operation.

- Dedicated signaling in RRC_CONNECTED state for configuring A-IoT radio resources

- Procedures to enable the UE to release the A-IoT resources upon cell change or moving to out of coverage

- UE assistance information from the reader to gNB for configuration of A-IoT radio resources

- As a second priority, support operation of A-IoT whilst the UE reader is in one of RRC_INACTIVE state or RRC_IDLE state (RAN2 to down select between INACTIVE and IDLE states)

Work scope for WGs

Based on the discussion above, the key aspects for Rel-20 A-IoT can be summarized as below:

- outdoor scenarios with device 2b/C at least in Topology 1,

- DO-A traffic,

- Positioning,

- Topology 2 in higher layer,

- and relevant enhancements.

Furthermore, as analyzed earlier, supporting Topology 2 with device 2b/C can reuse the designs of other scenarios. Considering the TU budget, the following scope for each WG is suggested.

- Consider the following scope for each WG in Rel-20.

RAN1

- Outdoor scenarios with device 2b/C in Topology 1, considering device synchronization

- Enhancements to improve coverage, spectral efficiency, and random access efficiency

- Enhancements to support DO-A, A-IoT positioning , and extend device availability

RAN2

- Protocol enhancements to support Topology 2

- Enhancements to the random access procedure including support of 2-step RACH

- Enhancements to avoid duplicate responses to the same service request

- MAC enhancement for DO-A and A-IoT positioning enhancement

RAN3

- Protocol stack and signalling support for Topology 2

- Support A-IoT positioning enhancement

RAN4

- RF/RRM/de-modulation requirements

Conclusion

Based on the above discussion about Rel-20 A-IoT scope, we have the following observations and proposals:

- Co-existence with NR and providing similar coverage as NR are critical to extend the A-IoT use cases to outdoor.

- D4T2 can be implemented by reusing the physical design of D4T1 and higher layer protocol of D2T2.

- When D4T2, D2T2-C and D1T1-B are supported, D1T1-C and D1T1/D2T2-A1/A2 can be implemented with minimal or even no specification impact.

- Support the following use cases in Rel-20：

- Outdoor inventory/command,

- Outdoor/indoor sensors/positioning,

- Indoor inventory/command with enhancement for D2T2.

- Solutions to enable better co-existence between A-IoT and NR need to be considered for outdoor scenarios.

- Device 2b/C is introduced to support outdoor scenarios in Rel-20 A-IoT.

- The following connectivity topologies and device types can be considered for Rel-20 A-IoT.

- D4T1, D4T2 for device 2b/C outdoors

- D1T1-C, D2T2-C for device 2b indoors

- Support DO-A traffic type and specify the corresponding configuration and procedures in Rel-20.

- Techniques to increase coverage and spectral efficiency should be considered in Rel-20 A-IoT.

- Solutions to increase network capacity and improve access efficiency should be considered for outdoor scenario, e.g., CDMA based random access, enhanced anti-collision algorithms, deterministic access occasion selection.

- 2-step RA procedure should be considered for Rel-20 A-IoT.

- Enhanced mechanisms to reduce duplicated response from the same device for the same service request should be considered in Rel-20.

- Direction 2 should be used for device (un)availability in Rel-20.

- Improving the device clock accuracy is needed to enhance the spectrum efficiency and enable the D2R coherent detection.

- For TP#2, specify RRC signaling to provision radio resources at the UE reader for A-IoT operation.

- Dedicated signaling in RRC_CONNECTED state for configuring A-IoT radio resources

- Procedures to enable the UE to release the A-IoT resources upon cell change or moving to out of coverage

- UE assistance information from the reader to gNB for configuration of A-IoT radio resources

- As a second priority, support operation of A-IoT whilst the UE reader is in one of RRC_INACTIVE state or RRC_IDLE state (RAN2 to down select between INACTIVE and IDLE states)

- Consider the following scope for each WG in Rel-20.

RAN1

- Outdoor scenarios with device 2b/C in Topology 1, considering device synchronization

- Enhancements to improve coverage, spectral efficiency, and random access efficiency

- Enhancements to support DO-A, A-IoT positioning , and extend device availability

RAN2

- Protocol enhancements to support Topology 2

- Enhancements to the random access procedure including support of 2-step RACH

- Enhancements to avoid duplicate responses to the same service request

- MAC enhancement for DO-A and A-IoT positioning enhancement

RAN3

- Protocol stack and signalling support for Topology 2

- Support A-IoT positioning enhancement

RAN4

- RF/RRM/de-modulation requirements

References

- RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, Huawei, RAN#106

- RP-243280, Way forward for Ambient IoT for REL-19 and REL-20, Huawei, RAN#106

- TR38.848 V18.0.0. Study on Ambient IoT (Internet of Things) in RAN (Release 18)

- TR38.769 V19.0.0. Study on solutions for Ambient IoT (Internet of Things) (Release 19)

- Draft Report of 3GPP TSG RAN WG1 #119 v0.3.0, RAN1#119

- R1-2409553, Discussion on frame structure and physical layer procedure for Ambient IoT, ZTE Corporation, Sanechips, RAN1#119