3GPP TSG RAN Meeting #107																		RP-250553

Incheon, Korea, March 12-14, 2025

Agenda Item:	15.2.6

Source:	ZTE Corporation

Title:	Views on AI for NG-RAN in 5G-A Rel-20

Document for:		Discussion

- Introduction

The Work Item for enhancements on AI/ML for NG-RAN is currently under discussion in RAN3 during Rel-19 stage. During the Rel-19 study item phase, two new use cases were identified for study: AI/ML-assisted Network Slicing and AI/ML-assisted Coverage and Capacity Optimization. Additionally, a down-selection was conducted for the Rel-18 leftovers. As a result, mobility optimization for NR-DC, support for split architecture and continuous MDT collection have been retained for further normative work during the WI phase.

It is acknowledged that AI/ML plays a pivotal role in the commercial deployment of 5G Advanced (5G-A), enhancing network capabilities and enabling innovative services and applications. AI/ML NG-RAN helps to optimize performance, reduce capital expenditure, and build new revenue streams.

In this contribution, we share our insights on AI/ML for NG-RAN in 5G-A Rel-20. Given that the numerous traditional features addressing optimization issues in the network, the potential enhancement of new use cases using AI/ML warrants further study in Rel-20.

- Justification of Rel-20 AI RAN

AI/ML-assisted Load Balancing, Network Energy Saving, and Mobility Optimization were specified in Rel-18. AI/ML-assisted Network Slicing and Coverage and Capacity Optimization are currently under discussion in Rel-19. In addition to the use cases in RAN3, various AI/ML-assisted use cases are being studied and specified across RAN2 and RAN3, including AI/ML-assisted positioning and CSI prediction.

  - New AI/ML based use cases

In the operation of 5G-A networks, leveraging AI is crucial for enhancing user experience and improving network energy efficiency. AI can be utilized to optimize network resource allocation dynamically, predict and address potential service quality issues before they impact users, and provide personalized services based on real-time data analytics. This proactive approach helps to ensure seamless connectivity and high-quality user experiences. Therefore, in Rel-20, new AI assisted use cases can be considered in order to fulfill the requirements on ensuring customer experience and lowering operational costs.

2.1.1 AI/ML assisted Energy Saving Enhancements

It is acknowledged that network operators have significant interest in AI/ML assisted Energy Saving use case. During Rel-18, it was specified that the measured energy cost can be transferred between NG-RAN nodes upon request, enabling the requesting NG-RAN node to generate appropriate energy-saving actions. However, consensus could not be reached on how to reflect the predicted energy cost with or without the additional load. The following approaches to improve the EC granularity were initially discussed:

- EC per group of Cells based on energy consumption measurements associated to the hardware serving the group of cells

- EC per Cell based on energy consumption estimations

- EC per one or more HO event based on energy consumption estimations

- EC per gNB-DU based on energy consumption measurements associated to the hardware serving the gNB-DU

At last, Energy-saving enhancements were excluded from the scope of the Rel-19 WI due to the lack of an appropriate solution.

- Network operators have significant interests in AI/ML assisted Energy Saving use case.

Given the popularity of energy-saving enhancements and the potential for further work and study, AI/ML-assisted energy-saving enhancements can be considered as a new use case for further exploration in Rel-20. Following is the potential key aspects of AI/ML based energy saving enhancements:

  - Predicted Energy Cost over Xn interface

  - Estimated Energy Saving Strategy

  - Green/Renewable Energy Consumption Prediction

- Further study the benefits and scope of AI/ML assisted Energy Saving Enhancements, and discuss the normative work during Rel-20 in RAN3.

2.1.2 AI/ML assisted QoE Optimization

AI/ML-assisted QoE Optimization was another use case that received significant support from companies. However, due to workload considerations, it was temporarily excluded from Rel-19. Further studying the standard impacts of AI/ML-assisted QoE Optimization would be a next step.

QoE refers to the overall satisfaction or perceived quality experienced by users when interacting with a specific service or application, such as video streaming, online gaming, or voice communication. AI/ML techniques can be employed to enhance and optimize QoE in various ways. For example, utilizing QoE prediction can effectively address uncertainties and improve the efficiency of radio resource management. The following are the key aspects of AI/ML-based QoE optimization to be studied:

- QoE Estimation/Prediction: AI/ML models can be trained to predict QoE metrics by analyzing various factors, such as network conditions, device characteristics, and user behavior. Using historical data and patterns, these models can estimate expected QoE metrics (e.g., buffer levels, UL/DL throughput, etc.) for a given service or application in real-time. This information can be leveraged to make decisions on adaptive bitrate streaming or network resource allocation, ultimately enhancing user satisfaction.

- QoE Optimization Feedback: Comparing historical QoE metrics (e.g., buffer levels, playout delay, giitter, UL/DL throughput) with predicted QoE metrics can help evaluate the performance of QoE prediction. By collecting and analyzing both historical and current QoE data, the NG-RAN node generating QoE prediction can assess the accuracy of its predictions against the actual QoE experienced by users. This feedback provides valuable insights into the reliability and effectiveness of the prediction model, enabling continuous improvement.

Given the benefits of AI/ML assisted QoE optimization, it is crucial to study the solutions, measurements, signaling, and procedures for AI/ML assisted QoE optimization in Rel-20.

- AI/ML assisted QoE optimization enhances user satisfaction by dynamically adapting and optimizing network parameters, resulting in an improved quality of experience.

- Study the benefits and scope of AI/ML assisted QoE Optimization, and discuss the normative work of input/output/feedback and the normative work during Rel-20 in RAN3.

  - Potential Leftovers

Some essential leftovers, which has heated discussion during Rel-18 & Rel-19 phase, can be studied and specified in Rel-20:

-	Multiple-hop target node UE trajectory: In Rel-18, UE trajectory prediction was limited to the next one-hop target NG-RAN node, and RAN3 decided not to pursue enhancements allowing a gNB to request UE trajectory information from nodes beyond one hop. However, this limitation restricts the utility of predicted UE trajectories and the capabilities of AI/ML algorithms. Predicting multiple-hop UE trajectories could provide valuable information for subsequent mobility decisions. A key topic for discussion is how to retrieve measured UE trajectory data from nodes beyond one hop.

-	Finer granularity level UE trajectory: In Rel-18, only cell-level UE trajectory was considered. However, geographic location-level UE trajectory can offer more detailed information, which is valuable for mobility optimization in NG-RAN nodes.

- Multiple-hop target node UE trajectory and finer granularity level UE trajectory can be further studied and specified in Rel-20.

2.3 Data Collection across different entities

In Rel-18, coordination between CN and RAN for support AI/ML function is not considered. Upon checking the progress of AI/ML in other groups, SA2 has specified NWDAF in 5GC to support AI/ML functionality, and RAN3 has also specified support of AI/ML NG-RAN function based on following framework:

Figure 1. Functional Framework for RAN Intelligence

In RAN3, it was agreed that model training and model inference were considered to be deployed either in the OAM or NG-RAN. Additionally, in the RAN1 discussion, data collection has been explored to determine whether it can be gathered from the UE, NG-RAN nodes, or the CN. For instance, in the case of AI/ML-assisted positioning, the NG-RAN node requires label information from the LMF for model training. This highlights the potential for further consideration of data collection between the CN and RAN for other AI/ML-assisted use cases. In Rel-20, AI/ML information coordination between different network entities, such as the CN and RAN, can be included as one of the key objectives.

- For AI/ML assisted positioning, coordination between CN and RAN to support AI/ML function is under discussion in Rel-19 in RAN3.

The benefits of AI/ML interaction between the CN and RAN are evident. Taking mobility as an example, the CN can provide UE trajectory analysis reports and/or predictive information to the RAN as assistance for improved training and prediction on the RAN side. Conversely, the RAN can share its own AI/ML-predicted information with the CN. Currently, expected UE behavior, including predictable activity and/or mobility patterns potentially generated by NWDAF, is already being optionally conveyed from the CN to the RAN.

- Study the benefits and potential coordination/interaction between CN and NG-RAN for support of AI/ML function in Rel-20.

The detailed objectives of the Rel-20 SI are listed as follows:

- Study the following new AI/ML based use cases, i.e., Energy Saving Enhancements and QoE Optimization, and the corresponding solution for the identified use cases within the existing architecture. [RAN3]

- Study the benefits and potential coordination/interaction between CN and NG-RAN for support of identified AI/ML assisted use cases. [RAN3]

- Continue to work on the following Rel-18 leftovers:

-   Multiple-hop target node UE trajectory

-   Finer granularity level UE trajectory

Note: Coordination based on LSs with other groups, if needed, e.g. SA2, SA5.

- Conclusion

Following is our observations and proposals:

- Network operators have significant interests in AI/ML assisted Energy Saving use case.

- Further study the benefits and scope of AI/ML assisted Energy Saving Enhancements, and discuss the normative work during Rel-20 in RAN3.

- AI/ML assisted QoE optimization enhances user satisfaction by dynamically adapting and optimizing network parameters, resulting in an improved quality of experience.

- Study the benefits and scope of AI/ML assisted QoE Optimization, and discuss the normative work during Rel-20 in RAN3.

- Multiple-hop target node UE trajectory and finer granularity level UE trajectory can be further studied and specified in Rel-20.

Observation 3 	For AI/ML assisted positioning, coordination between CN and RAN to support AI/ML function is under discussion in Rel-19 in RAN3.

- Study the benefits and potential coordination/interaction between CN and NG-RAN for support of AI/ML function in Rel-20.

- Following is the detailed objectives of the Rel-20 SI:

- Study the following new AI/ML based use cases, i.e., Energy Saving Enhancements and QoE Optimization, and the corresponding solution for the identified use cases within the existing architecture. [RAN3]

- Study the benefits and potential coordination/interaction between CN and NG-RAN for support of identified AI/ML assisted use cases. [RAN3]

- Continue to work on the following Rel-18 leftovers:

-   Multiple-hop target node UE trajectory

-   Finer granularity level UE trajectory

Note: Coordination based on LSs with other groups, if needed, e.g. SA2, SA5.