3GPP TSG-RAN4 Meeting #114	R4-2500052

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0705 | rev | - | Current version: | 17.15.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_demod_enh2-Perf) CR for TS38.101-4, corrections on configurations for CQI reporting with inter-cell interference |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek inc., Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_demod_enh2-Perf |  |  |  |  |  | Date: |  |  | 2025-01-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - It wsas agreed in R4-2120708 that CSI-IM on target cell overlaps with PDSCH from interference cell. Issue 2-2-1: CSI-IM on target cell - Agreement: - Only consider overlapping with PDSCH from interference. However, for FDD requirements in Clauses 6.2.2.1.2.3 and 6.2.3.1.2.3, the CSI-IM on target cell, (kCSI-IM,lCSI-IM) = (4, 9), overlaps with ZP CSI-RS (k0=Row5,(4) and I0 = 9) from interference cell, not the PDSCH from interference cell. Therefore, for FDD requirements in Clauses 6.2.2.1.2.3 and 6.2.3.1.2.3, we modify the configuration of ZP CSI-RS from interference cell as k0 = Row5,(6) to make CSI-IM on target cell overlaps with PDSCH from interference cell. - It was agreed in R4-2115732 to reuse test configutaions from Clauses 6.2.2.1.2.1, 6.2.2.2.2.1, 6.2.2.2.2.1, 6.2.3.2.2.1 for serving cell. Issue 3-1: Test configuration - Agreement: - Serving cell - Reuse assumptions from Sections 6.2.2.1.2.1, 6.2.2.2.2.1, 6.2.2.2.2.1, 6.2.3.2.2.1 in TS 38-101-4.. However, for TDD requirements in Clause 6.2.2.2.2.3 and 6.2.3.2.2.3, the ZP CSI-RS configuration, k0 = Row3,(8) and I0 = 9, is different from that in Clauses 6.2.2.2.2.1 and 6.2.3.2.2.1, where k0 = Row5,(4) and I0 = 9. Therefore, for TDD requirements in Clause 6.2.2.2.2.3 and 6.2.3.2.2.3, we modify the configuration of ZP CSI-RS in serving cell as k0 = Row5,(4). Also, we modify the configuration of ZP CSI-RS for interference cell as k0 = Row5,(6), which align the setting in FDD. - Missing “:” in the caption of Table 6.2.2.1.2.3-1 - Taking the following table as an exampe, in TS38.101-4, the expression for configuraitons of ZP CSI-RS and NZP CSI-RS are differnet in CQI, PMI and RI requirements. Also, according to “Table 7.4.1.5.3-1: CSI-RS locations within a slot.” in TS38.211, there is no k1 or l1 from some CSI-RS resource configutation. Table 6.2.2.1.1.3-1 (CQI requirement); ZP CSI-RS configuration; First subcarrier index in the PRB used for CSI-RS (k0); Row 5,4. First OFDM symbol in the PRB used for CSI-RS (l0); 9. NZP CSI-RS for CSI acquisition; First subcarrier index in the PRB used for CSI-RS (k0); Row 3,(6). First OFDM symbol in the PRB used for CSI-RS (l0); 13. Table 6.3.2.1.3-1 (PMI requirement); ZP CSI-RS configuration; First subcarrier index in the PRB used for CSI-RS (k0, k1); Row 5, (4,-). First OFDM symbol in the PRB used for CSI-RS (l0, l1); (9,-). NZP CSI-RS for CSI acquisition; First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3); Row 12, (2, 4, 6, 8). First OFDM symbol in the PRB used for CSI-RS (l0, l1); (5, -). Table 6.4.2.1-1 (RI requirement); ZP CSI-RS configuration; First subcarrier index in the PRB used for CSI-RS (k0); Row 5,(4). First OFDM symbol in the PRB used for CSI-RS (l0); (9). NZP CSI-RS for CSI acquisition; First subcarrier index in the PRB used for CSI-RS (k0); Row 3 (6). First OFDM symbol in the PRB used for CSI-RS (l0); (13). |
| Summary of change: |  | - For FDD requirements in Clause 6.2.2.1.2.3 and 6.2.3.1.2.3, - modify the configuration of ZP CSI-RS from interference cell to k0 = Row5,(6) - For TDD requirements in Clause 6.2.2.2.2.3 and 6.2.3.2.2.3, - modify the configuration of ZP CSI-RS in serving cell to k0 = Row5,(4) - modify the configuration of ZP CSI-RS for interference cell to k0 = Row5,(6) - modfify the number of CSI-RS ports to 4 - modfify the CRI-RS resource type of interfrence cell from noCDM to FD-CDM2 - Add “:” in the caption of Table 6.2.2.1.2.3-1 - Remove k1 or l1 from ZP and NZP CSI-RS configuration when there is no k1 or l1 in CSI-RS resource configutation. - Align the expression for configuraitons of ZP CSI-RS and NZP CSI-RS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be inconsist between specification and RAN4 agreements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clauses 6.2.2.1.2.3, 6.2.3.1.2.3, 6.2.2.2.2.3 and 6.2.3.2.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE 1

###### 6.2.2.1.2.3	Minimum requirement for wideband CQI reporting with inter-cell interference

The purpose of the requirements is to verify that the UE is tracking the channel variations and selecting the largest transport format possible based on inter-cell interference mitigation receiver.

For the parameters specified in Table 6.2.2.1.2.3-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following,

a)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥ g  where g  is specified in Table 6.2.2.1.2.3-2;

b)	when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR, the average BLER for the indicated transport formats shall be greater than or equal to 0.02.

Table 6.2.2.1.2.3-1: Wideband CQI reporting test with inter-cell interference

| Parameter |  |  | Unit | Test1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 |
| Bandwidth |  |  | MHz | 10 | 10 |
| Duplex Mode |  |  |  | FDD | FDD |
| Subcarrier spacing |  |  | kHz | 15 | 15 |
| SINR |  |  | dB | -2 | - |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |
| Interference Model |  |  |  |  | As specified in B.6.2 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 4 | 4 |
|  | CDM Type |  |  | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,(4) | Row 5,(6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | Row 5,(9) | Row 5,(9) |
|  | CSI-RS periodicity and offset |  | slot | 5/1 | Same as serving cell |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 2 | 1 |
|  | CDM Type |  |  | FD-CDM2 | noCDM |
|  | Density (ρ) |  |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 3,(6) | Row 2,(6,) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | Row 3,(13) | Row 2,(13) |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 5/1 | Same as serving cell |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic | Periodic |
|  | CSI-IM RE pattern |  |  | 0 | 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) | (6, 9) |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 5/1 | Same as serving cell |
| ReportConfigType |  |  |  | Periodic | Not configured |
| CQI-table |  |  |  | Table 2 | Table 2 |
| reportQuantity |  |  |  | cri-RI-PMI-CQI | Not configured |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured | Not configured |
| cqi-FormatIndicator |  |  |  | Wideband | Wideband |
| pmi-FormatIndicator |  |  |  | Wideband | Wideband |
| Sub-band Size |  |  | RB | 8 | - |
| Csi-ReportingBand |  |  |  | 1111111 | Not configured |
| CSI-Report periodicity and offset |  |  | slot | 5/0 | Not configured |
| aperiodicTriggeringOffset |  |  |  | Not configured | Not configured |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel | typeI-SinglePanel |
|  |  | Codebook Mode |  | 1 | 1 |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured | Not configured |
|  |  | CodebookSubsetRestriction |  | 000001 | Not configured |
|  |  | RI Restriction |  | N/A | Not configured |
| Physical channel for CSI report |  |  |  | PUCCH | Not configured |
| CQI/RI/PMI delay |  |  | ms | 8 | Not configured |
| Maximum number of HARQ transmission |  |  |  | 1 | Not configured |
| Measurement channel |  |  |  | As specified in Table A.4-2, TBS.2-1 | - |
| INR (Note 6) |  |  | dB | N/A | 10.04 |
| Propagation condition |  |  |  | TDLA30-5 | AWGN |
| Antenna configuration |  |  |  | 2×2 | 1×2 |
| Correlation configuration |  |  |  | ULA Low | N/A |
| Note 1: The respective received power spectral density of each interfering cell relative to is defined by its associated INR value as specified in clause B.6.1. Note 2: Two cells are considered in which Cell 1 is the serving cell and Cell 2 is the interfering cell. Interfering cell is fully loaded. Note 3: Both cells are time-synchronous. Note 4: Static channel is used for the interference model. In case for white Gaussian noise model Cell 2 is not present. Note 5: SINR corresponds to of Cell 1 as defined in clause 4.4.5. Note 6: INR is defined in clause B.6.1. |  |  |  |  |  |

Table 6.2.2.1.2.3-2 Minimum requirements

| Parameters | Test 1 |
| --- | --- |
| g | 1.9 |

END OF CHANGE 1

START OF CHANGE 2

###### 6.2.2.2.2.3	Minimum requirement for wideband CQI reporting with inter-cell interference

The purpose of the requirements is to verify that the UE is tracking the channel variations and selecting the largest transport format possible based on inter-cell interference mitigation receiver.

For the parameters specified in Table 6.2.2.2.2.3-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following,

a)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥ g  where g  is specified in Table 6.2.2.2.2.3-2;

b)	when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR, the average BLER for the indicated transport formats shall be greater than or equal to 0.02.

Table 6.2.2.2.2.3-1: Wideband CQI reporting test with inter-cell interference (TDD)

| Parameter |  |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 |
| Bandwidth |  |  | MHz | 40 | 40 |
| Duplex Mode |  |  |  | TDD | TDD |
| Subcarrier spacing |  |  | kHz | 30 | 30 |
| TDD UL-DL pattern |  |  |  | FR1.30-1 | FR1.30-1 |
| SINR |  |  | dB | -2 | - |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |
| Interference Model |  |  |  |  | As specified in B.6.2 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 4 | 4 |
|  | CDM Type |  |  | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,(4) | Row 5,(6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | Row 5,(9) | Row 5,(9) |
|  | CSI-RS periodicity and offset |  | slot | 10/1 | Same as serving cell |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 2 | 1 |
|  | CDM Type |  |  | FD-CDM2 | noCDM |
|  | Density (ρ) |  |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 3,(6) | Row 2,(6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | Row 3,(13) | Row 2,(13) |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 10/1 | Same as serving cell |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic | Periodic |
|  | CSI-IM RE pattern |  |  | 0 | 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) | (6,9) |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 10/1 | Same as serving cell |
| ReportConfigType |  |  |  | Periodic | Not configured |
| CQI-table |  |  |  | Table 2 | Table 2 |
| reportQuantity |  |  |  | cri-RI-PMI-CQI | Not configured |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured | Not configured |
| cqi-FormatIndicator |  |  |  | Wideband | Wideband |
| pmi-FormatIndicator |  |  |  | Wideband | Wideband |
| Sub-band Size |  |  | RB | 16 |  |
| Csi-ReportingBand |  |  |  | 1111111 | Not configured |
| CSI-Report periodicity and offset |  |  | slot | 10/9 | Not configured |
| aperiodicTriggeringOffset |  |  |  | Not configured | Not configured |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel | typeI-SinglePanel |
|  |  | Codebook Mode |  | 1 | 1 |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured | Not configured |
|  |  | CodebookSubsetRestriction |  | 000001 | Not configured |
|  |  | RI Restriction |  | N/A | Not configured |
| Physical channel for CSI report |  |  |  | PUCCH | Not configured |
| CQI/RI/PMI delay |  |  | ms | 9.5 | Not configured |
| Maximum number of HARQ transmission |  |  |  | 1 | Not configured |
| Measurement channel |  |  |  | As specified in Table A.4-2, TBS.2-3 |  |
| INR |  |  | dB | N/A | 10.04 |
| Propagation condition |  |  |  | TDLA30-5 | AWGN |
| Antenna configuration |  |  |  | 2×2 | 1×2 |
| Correlation configuration |  |  |  | ULA Low | N/A |
| Note 1: The respective received power spectral density of each interfering cell relative to is defined by its associated INR value as specified in clause B.6.1. Note 2: Two cells are considered in which Cell 1 is the serving cell and Cell 2 is the interfering cell. Interfering cell is fully loaded. Note 3: Both cells are time-synchronous. Note 4: Static channel is used for the interference model. In case for white Gaussian noise model Cell 2 is not present. Note 5: SINR corresponds to of Cell 1 as defined in clause 4.4.5. Note 6: INR corresponds to Cell 2 is defined in clause B.6.1 |  |  |  |  |  |

Table 6.2.2.2.2.3-2: Minimum requirement (TDD)

| Parameters | Test 1 |
| --- | --- |
| g | 1.9 |

END OF CHANGE 2

START OF CHANGE 3

###### 6.2.3.1.2.3	Minimum requirement for wideband CQI reporting with inter-cell interference

The purpose of the requirements is to verify that the UE is tracking the channel variations and selecting the largest transport format possible based on inter-cell interference mitigation receiver.

For the parameters specified in Table 6.2.3.1.2.3-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following,

a)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥ g  where g  is specified in Table 6.2.3.1.2.3-2;

b)	when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR, the average BLER for the indicated transport formats shall be greater than or equal to 0.02.

Table 6.2.3.1.2.3-1 Wideband CQI reporting test with inter-cell interference

| Parameter |  |  | Unit | Test1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 |
| Bandwidth |  |  | MHz | 10 | 10 |
| Duplex Mode |  |  |  | FDD | FDD |
| Subcarrier spacing |  |  | kHz | 15 | 15 |
| SINR |  |  | dB | -2 | - |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |
| Interference Model |  |  |  |  | As specified in B.6.2 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 4 | 4 |
|  | CDM Type |  |  | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,(4) | Row 5,(6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | Row 5,(9) | Row 5,(9) |
|  | CSI-RS periodicity and offset |  | slot | 5/1 | Same as serving cell |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 2 | 1 |
|  | CDM Type |  |  | FD-CDM2 | noCDM |
|  | Density (ρ) |  |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 3,(6) | Row 2,(6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | Row 3,(13) | Row 2,(13) |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 5/1 | Same as serving cell |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic | Periodic |
|  | CSI-IM RE pattern |  |  | 0 | 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) | (6, 9) |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 5/1 | Same as serving cell |
| ReportConfigType |  |  |  | Periodic | Not configured |
| CQI-table |  |  |  | Table 2 | Table 2 |
| reportQuantity |  |  |  | cri-RI-PMI-CQI | Not configured |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured | Not configured |
| cqi-FormatIndicator |  |  |  | Wideband | Wideband |
| pmi-FormatIndicator |  |  |  | Wideband | Wideband |
| Sub-band Size |  |  | RB | 8 | - |
| Csi-ReportingBand |  |  |  | 1111111 | Not configured |
| CSI-Report periodicity and offset |  |  | slot | 5/0 | Not configured |
| aperiodicTriggeringOffset |  |  |  | Not configured | Not configured |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel | typeI-SinglePanel |
|  |  | Codebook Mode |  | 1 | 1 |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured | Not configured |
|  |  | CodebookSubsetRestriction |  | 000001 | Not configured |
|  |  | RI Restriction |  | N/A | Not configured |
| Physical channel for CSI report |  |  |  | PUCCH | Not configured |
| CQI/RI/PMI delay |  |  | ms | 8 | Not configured |
| Maximum number of HARQ transmission |  |  |  | 1 | Not configured |
| Measurement channel |  |  |  | As specified in Table A.4-2, TBS.2-1 | - |
| INR (Note 6) |  |  | dB | N/A | 10.04 |
| Propagation condition |  |  |  | TDLA30-5 | AWGN |
| Antenna configuration |  |  |  | 2×4 | 1×4 |
| Correlation configuration |  |  |  | ULA Low | N/A |
| Note 1: The respective received power spectral density of each interfering cell relative to is defined by its associated INR value as specified in clause B.6.1. Note 2: Two cells are considered in which Cell 1 is the serving cell and Cell 2 is the interfering cell. Interfering cell is fully loaded. Note 3: Both cells are time-synchronous. Note 4: Static channel is used for the interference model. In case for white Gaussian noise model Cell 2 is not present. Note 5: SINR corresponds to of Cell 1 as defined in clause 4.4.5. Note 6: INR is defined in clause B.6.1. |  |  |  |  |  |

Table 6.2.3.1.2.3-2: Minimum requirements

| Parameters | Test 1 |
| --- | --- |
| g | 2.0 |

END OF CHANGE 3

START OF CHANGE 4

###### 6.2.3.2.2.3	Minimum requirement for wideband CQI reporting with inter-cell interference

The purpose of the requirements is to verify that the UE is tracking the channel variations and selecting the largest transport format possible based on inter-cell interference mitigation receiver.

For the parameters specified in Table 6.2.3.2.2.3-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following,

a)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥ g  where g  is specified in Table 6.2.3.2.2.3-2;

b)	when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR, the average BLER for the indicated transport formats shall be greater than or equal to 0.02.

Table 6.2.3.2.2.3-1: Wideband CQI reporting test with inter-cell interference (TDD)

| Parameter |  |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 |
| Bandwidth |  |  | MHz | 40 | 40 |
| Duplex Mode |  |  |  | TDD | TDD |
| Subcarrier spacing |  |  | kHz | 30 | 30 |
| TDD UL-DL pattern |  |  |  | FR1.30-1 | FR1.30-1 |
| SINR |  |  | dB | -2 | - |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |
| Interference Model |  |  |  |  | As specified in B.6.2 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 4 | 4 |
|  | CDM Type |  |  | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,(4) | Row 5,(6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | Row 5,(9) | Row 5,(9) |
|  | CSI-RS periodicity and offset |  | slot | 10/1 | Same as serving cell |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 2 | 1 |
|  | CDM Type |  |  | FD-CDM2 | noCDM |
|  | Density (ρ) |  |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 3,(6) | Row 2,(6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | Row 3,(13) | Row 2,(13) |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 10/1 | Same as serving cell |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic | Periodic |
|  | CSI-IM RE pattern |  |  | 0 | 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) | (6,9) |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 10/1 | Same as serving cell |
| ReportConfigType |  |  |  | Periodic | Not configured |
| CQI-table |  |  |  | Table 2 | Table 2 |
| reportQuantity |  |  |  | cri-RI-PMI-CQI | Not configured |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured | Not configured |
| cqi-FormatIndicator |  |  |  | Wideband | Wideband |
| pmi-FormatIndicator |  |  |  | Wideband | Wideband |
| Sub-band Size |  |  | RB | 16 |  |
| Csi-ReportingBand |  |  |  | 1111111 | Not configured |
| CSI-Report periodicity and offset |  |  | slot | 10/9 | Not configured |
| aperiodicTriggeringOffset |  |  |  | Not configured | Not configured |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel | typeI-SinglePanel |
|  |  | Codebook Mode |  | 1 | 1 |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured | Not configured |
|  |  | CodebookSubsetRestriction |  | 000001 | Not configured |
|  |  | RI Restriction |  | N/A | Not configured |
| Physical channel for CSI report |  |  |  | PUCCH | Not configured |
| CQI/RI/PMI delay |  |  | ms | 9.5 | Not configured |
| Maximum number of HARQ transmission |  |  |  | 1 | Not configured |
| Measurement channel |  |  |  | As specified in Table A.4-2, TBS.2-3 |  |
| INR |  |  | dB | N/A | 10.04 |
| Propagation condition |  |  |  | TDLA30-5 | AWGN |
| Antenna configuration |  |  |  | 2×4 | 1×4 |
| Correlation configuration |  |  |  | ULA Low | N/A |
| Note 1: The respective received power spectral density of each interfering cell relative to is defined by its associated INR value as specified in clause B.6.1. Note 2: Two cells are considered in which Cell 1 is the serving cell and Cell 2 is the interfering cell. Interfering cell is fully loaded. Note 3: Both cells are time-synchronous. Note 4: Static channel is used for the interference model. In case for white Gaussian noise model Cell 2 is not present. Note 5: SINR corresponds to of Cell 1 as defined in clause 4.4.5. Note 6: NR corresponds to Cell 2 is defined in clause B.6.1. |  |  |  |  |  |

Table 6.2.3.2.2.3-2: Minimum requirement (TDD)

| Parameters | Test 1 |
| --- | --- |
| g | 2.0 |

END OF CHANGE 4