3GPP TSG-RAN WG4 Meeting #114	R4- 2502990

# Athens, GR, 17th – 21st February, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.102 | CR | 0059 | rev | 1 | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (LTE_NBIOT_eMTC_NTN_req-Core) CR to TS 36.102 on variable TX-RX frequency separation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-Core |  |  |  |  |  | Date: |  |  | 2025-02-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN2 specifications already allow NB-IoT to configure SIB2-UL and SIB1-DL center frequencies independently. Normally RAN4 defines minimum requirements with a default TX-RX frequency separation. Satellite industry has a strong need to use different TX-RX spacing in different deployments. Therefore, this CR adds requirements to validate that different TX-RX spacing is supported. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Addition of new TX-RX frequency separations for B256, B255, B254, and B253. - Addition of reference sensitivity for these TX-RX frequency separations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RAN2 specifications already allow LTE NB-IoT to configure SIB2-UL and SIB1-DL center frequencies independently. RF requirements to verify this functionality for NB-IoT NTN are absent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4A.3, 5.4B.3, 7.1, 7.3A, 7.3B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.521-5 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This’CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change 1>

5.4A.3	TX–RX frequency separation

The default E-UTRA TX channel (carrier centre frequency) to RX channel (carrier centre frequency) separation is specified in Table 5.4A.3-1 for the TX and RX channel bandwidth defined in Table 5.3A-1.

Table 5.4A.3-1: Default UE TX-RX frequency separation

| E-UTRA Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| 256 | 190 MHz1 161.4 to 218.6 MHz2 |
| 255 | -101.5 MHz1 -68.9 to -134.1 MHz2 |
| 254 | 873.5 MHz1 858.4 to 888.6 MHz2 |
| 253 | -150 MHz1 -144.4 to -155.6 MHz2 |
| NOTE 1: Default TX-RX separation. NOTE 2: The verification of flexible TX-RX frequency separation within this range is limited to reference sensitivity. Further details are specified in clause 7.3A. |  |

<End of change 1>

<Start of change 2>

5.4B.3	TX–RX frequency separation

For UE category NB1/NB2 operation in stand-alone mode, the default TX-RX frequency separation shall be as specified in Table 5.4B.3-1 for the NB-IoT TX and RX channel bandwidth defined in Table 5.3B-1.

Table 5.4B.3-1: Default UE TX-RX frequency separation

| E-UTRA Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| 256 | 190 MHz1 160.2 to 219.8 MHz2 |
| 255 | -101.5 MHz1 -67.7 to -135.3 MHz2 |
| 254 | 873.5 MHz1 857.2 to 889.8 MHz2 |
| 253 | -150 MHz1 -143.2 to -156.8 MHz2 |
| NOTE 1: Default Tx-Rx separation. NOTE 2: The verification of flexible TX-RX frequency separation within this range is limited to reference sensitivity. Further details are specified in clause 7.3B. |  |

<End of change 2>

<Start of change 3>

7.1	General

The requirements in clause 7.1 of TS 36.101 [7] shall apply.

All requirements in this section are applicable to devices supporting GSO and/or NGSO satellites.

All RX requirements are verified with default TX-RX frequency separation specified in Table 5.4A.3-1. Additional TX-RX frequency separations specified in clauses 7.3A and 7.3B do not apply to other requirements.

<End of change 3>

<Start of change 4>

7.3A	Reference sensitivity power level for UE category M1

The reference sensitivity power level REFSENS is the minimum mean power applied to the single antenna port for UE category M1, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

The throughput for the REFSENS test is measured based on the Transmission Mode 1 unless specified otherwise.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes [A.2.2 and A.3.2] (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex [A.5.1.1]) with parameters specified in Table 7.3A-1 and Table 7.3A-2 for category M1.

Table 7.3A-1: Reference sensitivity for FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 256 | -102.2 | FDD |
| 255 | -102.7 | FDD |
| 254 | -102.2 | FDD |
| 253 | -102.7 | FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5- in TS 36.101 [7]. |  |  |

Table 7.3A-2: Reference sensitivity for HD-FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 256 | -103 | HD-FDD |
| 255 | -103.5 | HD-FDD |
| 254 | -103.1 | HD-FDD |
| 253 | -103.5 | HD-FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5 in TS 36.101 [7]. |  |  |

The reference receive sensitivity (REFSENS) requirement specified in Table 7.3A-1/Table 7.3A-2 shall be met for an uplink transmission bandwidth less than or equal to that specified in Table 7.3A-3 and with default TX-RX carrier center frequency separation except for cases specified in Table 7.3A-4.

NOTE:	Table 7.3A-3 is intended for conformance tests and does not necessarily reflect the operational conditions of the network, where the number of uplink and downlink allocated resource blocks will be practically constrained by other factors. Typical receiver sensitivity performance with HARQ retransmission enabled and using a residual BLER metric relevant for e.g. Speech Services is given in the Annex [G] (informative).

Table 7.3A-3: FDD UE category M1 Uplink configuration for reference sensitivity

| E-UTRA Band | NRB | Duplex Mode |
| --- | --- | --- |
| 256 | 61 | FDD and HD-FDD |
| 255 | 61 | FDD and HD-FDD |
| 254 | 61 | FDD and HD-FDD |
| 253 | 61 | FDD and HD-FDD |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3A-1). |  |  |

Table 7.3A-4: TX – RX carrier centre frequency separation for REFSENS verification

| E-UTRA Operating Band | Channel bandwidth | TX – RX carrier centre frequency separation for REFSENS verification |
| --- | --- | --- |
| 256 | 1.4 MHz | 161.4 MHz, 218.6 MHz |
| 255 | 1.4 MHz | -68.9 MHz, -134.1 MHz |
| 254 | 1.4 MHz | 858.4 MHz, 888.6 MHz |
| 253 | 1.4 MHz | -144.4 MHz, -155.6 MHz |

<End of change 4>

<Start of change 5>

7.3B	Reference sensitivity power level for UE category NB1 and NB2

The reference sensitivity power level REFSENS is the minimum mean power applied to the single antenna port for UE category NB1 and category NB2, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

The throughput for the REFSENS test is measured based on the Transmission Mode 1 unless specified otherwise.

The category NB1 and NB2 UE throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in TS 36.101 [7] Annex A.3.2 with received signal level as specified in Table 7.3B-1. Requirement in Table 7.3B-1 applies for any uplink configuration and with default TX-RX carrier center frequency separation except for cases specified in Table 7.3B-2.

Table 7.3B-1: Reference sensitivity for UE category NB1 and NB2

| Operating band | REFSENS [dBm] |
| --- | --- |
| According to subclause 5.2B | - 108.2 |

Table 7.3B-2: TX – RX carrier centre frequency separation for REFSENS verification

| E-UTRA Operating Band | Channel bandwidth | TX – RX carrier centre frequency separation for REFSENS verification |
| --- | --- | --- |
| 256 | 0.2 MHz | 160.2 MHz, 219.8 MHz |
| 255 | 0.2 MHz | -67.7 MHz, -135.3 MHz |
| 254 | 0.2 MHz | 857.2 MHz, 889.8 MHz |
| 253 | 0.2 MHz | -143.2 MHz, -156.8 MHz |

<End of change 5>