3GPP TSG-RAN Meeting #106	RP-250639

Incheon, Korea, 12th – 14th March 2025

Agenda Item:	15.3

Source:	Eutelsat Group

Title:	Motivation to add VSAT testability to NR FR2 OTA testing Phase 3

Document for:	Information

# 1	Background

This contribution was written for AI 9.3.6 but was moved to 15.3 at the request of the chair.

In May 2024 at RAN4 #111, the issue of VSAT testability was first formally presented in [1]. In parallel at RAN5 #103, a draft WID to develop VSAT test requirements was presented in [2]. Following offline discussions in Fukuoka, it was decided that RAN5 would not present [2] to RAN #104 but defer to the outcome of a draft SID on VSAT testability which was presented to RAN #104 in [3].

At RAN #104, the proposal in [3] was not approved. At RAN #105, and RAN #106, proposals in [4] and [5] to start the work on VSAT test methods were also not approved. The reason for not approving this study is that it has been treated in the same category as new features competing with limited resources. This is not how issues related to testability and the development of conformance test requirements have previously been treated. These requests are never debated by RAN and are not considered in competition with new features, rather they are built into the work that follows RAN approval of the core WI.

Once RAN approves a WI with UE core requirements, it is the default procedure that RAN5 follows up with a WI on conformance testing. The normal flow of approving conformance test WI has only been interrupted when the core requirements have involved the development of new test methods - which have typically been related to radiated testing. In these cases, RAN4 has started feasibility studies to develop the test methods before RAN5 takes over and follows up with the detailed test system design and resulting conformance test cases. This procedure has also been documented in changes to the terms of reference of RAN4 [8]. In the NR era, RAN4 started this testability FS process in 2016 to handle FR2 handsets, and this work is now on its third cycle of radiated test method development with the ongoing study FS_NR_FR2_OTA_Ph3.

This paper stresses the urgent need to start work on VSAT testability and proposes the approval of a revised SID to address the  VSAT testability  aspects of concern in Rel-18 and Rel-19, in the absence of which the set of specifications for the related bands and features would remain incomplete or inconsistent.

# 2	Discussion

When RAN approved the WI NR_NTN_enh for VSATs operating above 10 GHz there was an implicit and obvious consequence on test methods since the size of VSATs is much larger than the size of the FR2 handsets used to develop the original FR2 NR radiated test systems whose quiet zone was only 15 cm (now 40 cm). Also, the FR2-NTN frequency range has a lower limit of 17.3 GHz, well below the 24.25 GHz of existing approved FR2 radiated test methods. When RAN approved the WI Introduction of Ku bands for NR NTN in June 2024, the lower frequency dropped further to 10.7 GHz.

In addition to the frequency range and VSAT device size issues, the introduction of Ka band added the issue of polarization. Current radiated test systems are all based on cross polarized antennas, but Ka band operates with circular polarization. Unfortunately, there is no obvious way to convert existing test systems to use circular polarization without reverting to using a vector network analyzer approach which would mean the loss of the L3 signalling connectivity normally provided by a base station emulator, which lacks the phase control to generate stable circular polarization.

The need to address the large and growing gap in VSAT testability is further evidenced by a Feb 2025 industry press release [6] which announced the first successful connection between a Ku band VSAT using an NR air interface and a LEO constellation. This indicates the demand for 3GPP-based NTN standards since current commercial developments are actually ahead of 3GPP rather than being a couple of years behind the standards There has already been a 9-month delay with no progress since these testabilityissues were first formally raised at RAN #104. If 3GPP does not act very soon, then the test infrastructure to complete the work that 3GPP started for NTN above 10 GHz will not be in place when it is needed.

The key current and future VSAT testability issues are:

- VSAT device sizes exceed by far what is currently testable in existing 3GPP FR2 test systems

- The introduction of Ka band with its 17.3 GHz lower limit falls well below the range of existing FR2 test systems of 24.25 GHz.

- The introduction of Ku band (10.7 and 14.5 GHz) further challenges the operating range of existing test methods.

- The need to support circular polarization adds a significant challenge to existing test systems which may require regressing to using non-signalling methods.

The Revised SID in [7] is the proposal for how to progress these issues by extending the scope of the existing FS Study on NR FR2 OTA (Over the Air) testing Phase 3 [RAN4 SI: FS_NR_FR2_OTA_Ph3]. This scope extension Is supported by the moderator.

# 3	 Proposal

Proposal: It is proposed to approve at RAN #107 the revised SID in [7], which has broad industry support, to commence the work on developing VSAT test methods following the completion in June 2024 of the core requirements specification for Ka band VSAT, and in advance of the additional needs of the Rel-19 WI NR_NTN_Ku_bands.

# 4	References

[1] R4-2407232 VSAT performance requirements and testability (Eutelsat Group, RAN4 #111 Fukuoka May 2024)

[2] R5-243355 New WID on UE Conformance - NR NTN (Non-Terrestrial Networks) enhancements plus CT aspects (Qualcomm Inc, China Unicom, MediaTek, RAN5 #103 Fukuoka May May 2024)

[3] RP-241171 New SID on VSAT test methods (Eutelsat Group, RAN #104 Shanghai June 2024)

[4] RP-242112 New SID on VSAT test methods (Eutelsat Group, RAN #105 Melbourne Sep 2024)

[5] RP-243236 Revised SID to add VSAT testability to NR FR2 OTA (Over the Air) testing Phase 3 (Eutelsat Group, RAN #106 Madrid Dec 2024)

[6] https://www.eutelsat.com/en/news/press.html#/pressreleases/eutelsat-mediatek-and-airbus-announce-worlds-first-5g-non-terrestrial-network-connection-leveraging-oneweb-leo-satellites-3371519

[7] RP-250640 Revised SID to add VSAT testability to NR FR2 OTA (Over the Air) testing Phase 3 (Eutelsat Group, RAN #107 Incheon March 2025)

[8] RP-210786 Terms of Reference (ToR) for 3GPP TSG RAN WG4 (RAN4), RAN#91-e, Mar 2021