3GPP TSG-RAN WG2 Meeting #129 	R2-2501494

# Athens, Greece, 17 - 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 2029 | rev | 2 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on PDCCH monitoring |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung (Rapporteur), Huawei, HiSilicon, Ericsson, Sharp |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_2step_RACH-Core |  |  |  |  |  | Date: |  |  | 2025-02-17 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | MSGB reception related to msgB-ResponseWindow is introduced in section 5.1.4a since R16. In the current MAC specification, the PDCCH monitoring behaviour related to the RACH procedure are specified using the reference to 5.1.4 (RAR reception), 5.1.4a (MSGB reception) and 5.1.5 (Contention Resolution), e.g.: 2> if a MAC PDU is received in a configured downlink assignment: 3> if there is no ongoing Random Access procedure associated with this Serving Cell; or 3> if the ongoing Random Access procedure associated with this Serving Cell is successfully completed upon reception of this PDCCH addressed to C-RNTI (as specified in clauses 5.1.4, 5.1.4a and 5.1.5): 4> start or restart the bwp-InactivityTimer associated with the active DL BWP. … 1> if the ongoing Random Access procedure associated with this Serving Cell is successfully completed upon reception of this PDCCH addressed to C-RNTI (as specified in clauses 5.1.4, 5.1.4a, and 5.1.5): … If the MAC entity receives a PDCCH for BWP switching for a Serving Cell(s) or a dormancy SCell group(s) while a Random Access procedure associated with that Serving Cell is ongoing in the MAC entity, it is up to UE implementation whether to switch BWP or ignore the PDCCH for BWP switching, except for the PDCCH reception for BWP switching addressed to the C-RNTI for successful Random Access procedure completion (as specified in clauses 5.1.4, 5.1.4a, and 5.1.5) in which case the UE shall perform BWP switching to a BWP indicated by the PDCCH.. However, in the section 5.14 Handling of measurement gaps, this section 5.1.4a is missing, even though the msgB-ResponseWindow is also checked. 1> if the ra-ResponseWindow or the ra-ContentionResolutionTimer or the msgB-ResponseWindow is running: 2> monitor the PDCCH as specified in clauses 5.1.4 and 5.1.5.. It is proposed to add "5.1.4a" in clause 5.14. |
| Summary of change: |  | Add '5.1.4a' for 'monitor the PDCCH as specified'. Impact analysis Impacted functionality: PDCCH monitor for 2-step RACH Inter-operability: 1. if the network supports the change and the UE does not: No interoperability problems are foreseen. 2. if the UE supports the change and the network does not: No interoperability problems are foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PDCCH monitoring behaviour is unspecified in handling of measurement gaps (clause 5.14) for the 2-step RACH case about the MsgB reception. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | - |  |  |
| affected: |  |  | X | Test specifications |  |  |  | - |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | - |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | - |  |  |  |  |  |  |  |  |

## 5.14	Handling of measurement gaps

During an activated measurement gap, the MAC entity shall, on the Serving Cell(s) in the corresponding frequency range of the measurement gap configured by measGapConfig as specified in TS 38.331 [5]:

1>	not perform the transmission of HARQ feedback, SR, and CSI;

1>	not report SRS;

1>	not transmit on UL-SCH except for Msg3 or the MSGA payload as specified in clause 5.4.2.2;

1>	if the ra-ResponseWindow or the ra-ContentionResolutionTimer or the msgB-ResponseWindow is running, or if there is an ongoing RACH-less LTM cell switch, or if there is an ongoing RACH-less handover:

2>	monitor the PDCCH as specified in clauses 5.1.4, 5.1.4a, 5.1.5, and 5.7.

1>	else:

2>	not monitor the PDCCH;

2>	not receive on DL-SCH.

End of changes