3GPP TSG-RAN WG2 Meeting #129	R2-2501370

# Athens, Greece, 17 – 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 5264 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification to UL RRC segmentation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_QoE-Core, NR_QoE_enh-Core, TEI17 |  |  |  |  |  | Date: |  |  | 2025-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | UL capability segmentation according to the network indication rrc-MaxCapaSegAllowed was added in RP-243227. As part of the corresponding CR to TS 38.331, a clarification was added to clause 5.7.7.3 to state that a UE shall determine the number of UL RRC segments according to the network indicating either rrc-SegAllowed or rrc-MaxCapaSegAllowed. While rrc-SegAllowed and rrc-MaxCapaSegAllowed are used by the UE to determine how to segment UE capabilities (UECapabilityInformation), there is another scenario where the UE shall apply UL RRC segmentation according to other network indications: i.e. segmentation of SRB4/SRB5 (MeasurementReportAppLayer) according to the presence of rrc-SegAllowedSRB4 and rrc-SegAllowedSRB5. Hence, it is not entirely accurate to only state that the UE shall determine UL RRC segmentation according rrc-SegAllowed or rrc-MaxCapaSegAllowed, and a clarification should also be added related to the usage of rrc-SegAllowedSRB4 and rrc-SegAllowedSRB5 in UL RRC segmentation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify in clause 5.7.7.3 that rrc-SegAllowed and rrc-MaxCapaSegAllowed are applicable to UL segmentation of UECapabilityInformation. - Add to clause 5.7.7.3 that rrc-SegAllowedSRB4 and rrc-SegAllowedSRB5 are applicable to UL segmentation of MeasurementReportAppLayer. Impact analysis Impacted 5G architecture options: NR SA, NR-DC, NE-DC Impacted functionality: UL segmentation of UE capabilities and SRB4/SRB5. Inter-operability: - If the network is implemented according to the CR and the UE is not the UE might incorrectly assume that RRC segmentation on SRB4 or SRB5 is dependent on the presence of rrc-SegAllowed or rrc-MaxCapaSegAllowed. - If the UE is implemented according to the CR and the network is not no interoperability issues are forseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE might incorrectly assume that RRC segmentation on SRB4 or SRB5 is dependent on the presence of rrc-SegAllowed or rrc-MaxCapaSegAllowed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR also extends the clarification to the Rel-18 indication rrc-SegAllowedSRB5. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R2-2501189 |  |  |  |  |  |  |  |  |

First Modified Subclause

### 5.7.7	UL message segment transfer

#### 5.7.7.1	General

Figure 5.7.7.1-1: UL message segment transfer

The purpose of this procedure is to transfer segments of UL DCCH messages from UE to a NG-RAN in RRC_CONNECTED.

NOTE:	The segmentation of UL DCCH message is only applicable to UECapabilityInformation and MeasurementReportAppLayer in this release.

#### 5.7.7.2	Initiation

A UE capable of UL RRC message segmentation in RRC_CONNECTED will initiate the procedure when the following conditions are met:

1>	if the RRC message segmentation is enabled based on the field rrc-SegAllowed, rrc-MaxCapaSegAllowed, rrc-SegAllowedSRB4 or rrc-SegAllowedSRB5 received, and

1>	if the encoded RRC message is larger than the maximum supported size of a PDCP SDU specified in TS 38.323 [5];

Upon initiating the procedure, the UE shall:

1>	initiate transmission of the ULDedicatedMessageSegment message as specified in 5.7.7.3;

#### 5.7.7.3	Actions related to transmission of ULDedicatedMessageSegment message

The UE shall segment the encoded RRC PDU based on the maximum supported size of a PDCP SDU specified in TS 38.323 [5] and the maximum number of UL segments according to rrc-SegAllowed or rrc-MaxCapaSegAllowed (for segmentation of UECapabilityInformation) and rrc-SegAllowedSRB4 or rrc-SegAllowedSRB5 (for segmentation of MeasurementReportAppLayer), if received. UE shall minimize the number of segments and set the contents of the ULDedicatedMessageSegment messages as follows:

1>	For each new UL DCCH message, set the segmentNumber to 0 for the first message segment and increment the segmentNumber for each subsequent RRC message segment;

1>	set rrc-MessageSegmentContainer to include the segment of the UL DCCH message corresponding to the segmentNumber;

1>	if the segment included in the rrc-MessageSegmentContainer is the last segment of the UL DCCH message:

2>	set the rrc-MessageSegmentType to lastSegment;

1>	else:

2>	set the rrc-MessageSegmentType to notLastSegment;

1>	submit all the ULDedicatedMessageSegment messages generated for the segmented RRC message to lower layers for transmission in ascending order based on the segmentNumber, upon which the procedure ends.

End of Changes