3GPP TSG-RAN WG2 Meeting #129	R2-2501424

# Athens, Greece, February 17 – 21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.355 | CR | 0009 | rev | 2 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Missing additional measurements for SL-TDOA and SL-TOA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2025-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The SL-TDOA-ProvideCapabilities and SL-TOA-ProvideCapabilities can indicate a maximum number of measurements supported by the UE. For example: "sl-PRS-RSTD-Meas The value indicates the supported maximum number of SL RSTD measurement reporting for different SL-PRS reception for the same pair of UEs." However, the corresponding measurement request and report are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A request for additional measurement reporting is included in IEs SL-TDOA-RequestLocationInformation and SL-TOA-RequestLocationInformation. Up to 3 additional measurement results are added to the IEs SL-TDOA-ProvideLocationInformation and SL-TOA-ProvideLocationInformation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing signalling for a feature which can be indicated in the capabilities. Impact analysis Impacted 5G architecture options: NR SA Impacted functionality: SL-TDOA, SL-TOA Inter-operability: - If the NW is implemented according to this CR while the UE is not, the NW will not receive additional measurements when requested. - If UE is implemented according to this CR while the NW is not, there is no inter-operability issue foreseen. - If a Tx UE is implemented according to this CR while the Rx UE is not, the Tx UE will not receive additional measurements when requested. - If a Rx UE is implemented according to this CR while the Tx UE is not, there is no inter-operability issue foreseen. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.9, 6.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | RAN2#127bis (R2-2408513): CR was postponed and LS with questions has been sent to RAN1 in R2-2409256. RAN2#129: Response from RAN1 has been received in R2-2500006 (R1-2410665) indicating additional SL-TOA/SL-TDOA measurement reporting and requesting should be supported. In addition, there is no need to perform relative encoding of the additional SL-RSTD/SL-RTOA, SL-PRS RSRP/RSRPP measurements |  |  |  |  |  |  |  |  |

## 6.9	SLPP PDU SL-TDOA Contents

[…]

#### –	SL-TDOA-RequestLocationInformation

-- ASN1START

-- TAG-SL-TDOA-REQUESTLOCATIONINFORMATION-START

SL-TDOA-RequestLocationInformation ::= SEQUENCE {

sl-ARP-InfoRequest                     ENUMERATED { true }        OPTIONAL,

sl-LOS-NLOS-IndicatorRequest           ENUMERATED { true }        OPTIONAL,

sl-PRS-RSRP-Request                    ENUMERATED { true }        OPTIONAL,

sl-RSRPP-Request                       ENUMERATED { true }        OPTIONAL,

sl-AdditionalPathsRequest              ENUMERATED { true }        OPTIONAL,

measurementsForMultipleARP-IDs-Rx      SEQUENCE {

requestedARP-IDs-Rx                    BIT STRING (SIZE (4))  OPTIONAL

}                                                                 OPTIONAL,

...,

[[

maxNumberOfMeasurementsPerARP        INTEGER (2..4)             OPTIONAL

]]

}

-- TAG-SL-TDOA-REQUESTLOCATIONINFORMATION-STOP

-- ASN1STOP

SL-TDOA-RequestLocationInformation field descriptions

maxNumberOfMeasurementsPerARP This field, if present, indicates that the UE is requested to provide the indicated maximum number of SL-TDOA measurements per ARP for different SL-PRS reception for the same pair of UEs.

measurementsForMultipleARP-IDs-Rx This field, if present, indicates that the UE is requested to provide the requested SL-TDOA measurements for multiple SL-PRS Rx ARP-IDs.

requestedARP-IDs-Rx This field, if present, indicates that the UE is requested to provide the requested SL-TDOA measurements for indicated SL-PRS Rx ARP-IDs. Bit 1 in the bit string indicates ARP-ID = 1, bit 2 indicates ARP-ID = 2, and so on.

sl-AdditionalPathsRequest This field, if present, indicates that the UE is requested to provide sl-TDOA-AdditionalPathList.

sl-ARP-InfoRequest This field, if present, indicates that the UE is requested to provide sl-POS-ARP-ID-Rx.

sl-LOS-NLOS-IndicatorRequest This field, if present, indicates that the UE is requested to provide the estimated los-NLOS-Indicator.

sl-PRS-RSRP-Request This field, if present, indicates that the UE is requested to provide sl-PRS-RSRP-Result.

sl-RSRPP-Request This field, if present, indicates that the UE is requested to provide sl-RSRPP.

#### –	SL-TDOA-ProvideLocationInformation

-- ASN1START

-- TAG-SL-TDOA-PROVIDELOCATIONINFORMATION-START

SL-TDOA-ProvideLocationInformation ::= SEQUENCE {

sl-RSTD-ReferenceUE-Info               SEQUENCE {

applicationLayerID                     OCTET STRING

}                                                                              OPTIONAL,

sl-TDOA-SignalMeasurementInformation   SL-TDOA-SignalMeasurementInformation    OPTIONAL,

sl-TDOA-Error                          SL-TDOA-LocationInformationError        OPTIONAL,

...

}

SL-TDOA-SignalMeasurementInformation ::= SEQUENCE {

sl-TDOA-MeasList                         SEQUENCE (SIZE(1..maxNrOfUEs)) OF SL-TDOA-MeasElementPerARP-ID-Rx,

...

}

SL-TDOA-MeasElementPerARP-ID-Rx ::= SEQUENCE (SIZE(1..4)) OF SL-TDOA-MeasElement

SL-TDOA-MeasElement ::= SEQUENCE {

applicationLayerID                    OCTET STRING              OPTIONAL,  -- Cond FirstElement

los-NLOS-Indicator                    LOS-NLOS-Indicator        OPTIONAL,  -- sl-losNlosIndicator

sl-POS-ARP-ID-Rx                      INTEGER (1..4)            OPTIONAL,  -- sl-pos-arpID-Rx

sl-PRS-ResourceId                     INTEGER (0..16)           OPTIONAL,  -- sl-PRS-ResourceId

sl-PRS-RSRP-Result                    INTEGER (0..126)          OPTIONAL,  -- sl-PRS-RSRP

sl-PRS-RSRPP-Result                   INTEGER (0..126)          OPTIONAL,  -- sl-PRS-RSRPP

sl-RSTD-Result                        CHOICE {

k0                                    INTEGER (0..1970049),

k1                                    INTEGER (0..985025),

k2                                    INTEGER (0..492513),

k3                                    INTEGER (0..246257),

k4                                    INTEGER (0..123129),

k5                                    INTEGER (0..61565)

}                                                                OPTIONAL,  -- sl-PRS-RSTD

sl-TDOA-AdditionalPathList            SL-TDOA-AdditionalPathList OPTIONAL,

sl-TimeStamp                          SL-TimeStamp               OPTIONAL,  -- sl-Timestamp

sl-TimingQuality                      SL-TimingQuality           OPTIONAL,  -- sl-TimingQuality

...,

[[

sl-TDOA-AdditionalMeasurements      SL-TDOA-AdditionalMeasurements       OPTIONAL

]]

}

SL-TDOA-AdditionalMeasurements ::= SEQUENCE (SIZE (1..3)) OF SL-TDOA-AdditionalMeasurementElement

SL-TDOA-AdditionalMeasurementElement ::= SEQUENCE {

sl-PRS-ResourceId                     INTEGER (0..16)                      OPTIONAL,

sl-PRS-RSRP-Result                    INTEGER (0..126)                     OPTIONAL,

sl-PRS-RSRPP-Result                   INTEGER (0..126)                     OPTIONAL,

sl-RSTD-Result                        CHOICE {

k0                                    INTEGER (0..1970049),

k1                                    INTEGER (0..985025),

k2                                    INTEGER (0..492513),

k3                                    INTEGER (0..246257),

k4                                    INTEGER (0..123129),

k5                                    INTEGER (0..61565)

}                                                                          OPTIONAL,

los-NLOS-Indicator                    LOS-NLOS-Indicator                   OPTIONAL,

sl-TDOA-AdditionalPathList            SL-TDOA-AdditionalPathList           OPTIONAL,

sl-TimeStamp                          SL-TimeStamp                         OPTIONAL,

sl-TimingQuality                      SL-TimingQuality                     OPTIONAL,

...

}

SL-TDOA-AdditionalPathList ::= SEQUENCE (SIZE(1..8)) OF SL-TDOA-AdditionalPath

SL-TDOA-AdditionalPath  ::= SEQUENCE {

sl-RSTD-AdditionalPathResult          CHOICE {

k0                                    INTEGER (0..1970049),

k1                                    INTEGER (0..985025),

k2                                    INTEGER (0..492513),

k3                                    INTEGER (0..246257),

k4                                    INTEGER (0..123129),

k5                                    INTEGER (0..61565)

}                                                               OPTIONAL,  -- additionalPath-SL-PRS-RSTD

sl-PRS-AdditionalPathRSRPP-Result      INTEGER (0..126)         OPTIONAL,  -- additionalPath-SL-PRS-RSRPP

sl-TimingQuality                       SL-TimingQuality         OPTIONAL,  -- sl-TimingQuality

...

}

SL-TDOA-LocationInformationError ::= ENUMERATED { undefined, assistanceDataMissing, notAllRequestedMeasurementsPossible, ... }

-- TAG-SL-TDOA-PROVIDELOCATIONINFORMATION-STOP

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| FirstElement | The field is mandatory present in the first SL-TDOA-MeasElement in IE SL-TDOA-MeasElementPerARP-ID-Rx. Otherwise, it is not present. |

SL-TDOA-ProvideLocationInformation field descriptions

los-NLOS-Indicator This field specifies the UE's best estimate of the LOS or NLOS of the UE measurements (including RSTD, RTOA, RSRP, RSRPP, AoA and UE Rx-Tx time difference).

sl-POS-ARP-ID-Rx This field indicates ARP ID of an ARP used for reception for per-ARP measurement reporting. The ARP ID is used to uniquely identify an ARP associated with a UE.

sl-PRS-ResourceId This field specifies the sidelink PRS resource ID used for SL positioning measurements.

sl-PRS-RSRP-Result This field specifies the sidelink PRS reference signal received power (RSRP) measurement.

sl-PRS-RSRPP-Result This field specifies the SL-RSRPP measurement based on first path of arrival.

sl-TDOA-AdditionalPathList This field specifies the sidelink PRS measurements based on additional path of arrival.

sl-RSTD-Result This field specifies the SL-RSTD measurement based on first path of arrival. In this release, this field is always included.

sl-RSTD-ReferenceUE-Info This field indicates reference UE information for SL-PRS based RSTD measurement report.

sl-TimeStamp This field specifies the time instance at which the SL RSTD and SL-PRS RSRP (if included) measurement is performed.

[…]

## 6.10	SLPP PDU SL-TOA Contents

[…]

#### –	SL-TOA-RequestLocationInformation

-- ASN1START

-- TAG-SL-TOA-REQUESTLOCATIONINFORMATION-START

SL-TOA-RequestLocationInformation ::= SEQUENCE {

sl-ARP-InfoRequest                    ENUMERATED { true }        OPTIONAL,

sl-LOS-NLOS-IndicatorRequest          ENUMERATED { true }        OPTIONAL,

sl-PRS-RSRP-Request                   ENUMERATED { true }        OPTIONAL,

sl-RSRPP-Request                      ENUMERATED { true }        OPTIONAL,

sl-AdditionalPathsRequest             ENUMERATED { true }        OPTIONAL,

measurementsForMultipleARP-IDs-Rx     SEQUENCE {

requestedARP-IDs-Rx                   BIT STRING (SIZE (4))  OPTIONAL

}                                                                OPTIONAL,

...,

[[

maxNumberOfMeasurementsPerARP       INTEGER (2..4)             OPTIONAL

]]

}

-- TAG-SL-TOA-REQUESTLOCATIONINFORMATION-STOP

-- ASN1STOP

SL-TOA-RequestLocationInformation field descriptions

maxNumberOfMeasurementsPerARP This field, if present, indicates that the UE is requested to provide the indicated maximum number of SL-TOA measurements per ARP for different SL-PRS reception for the same pair of UEs.

measurementsForMultipleARP-IDs-Rx This field, if present, indicates that the UE is requested to provide the requested SL-TOA measurements for multiple SL-PRS Rx ARP-IDs.

requestedARP-IDs-Rx This field, if present, indicates that the UE is requested to provide the requested SL-TOA measurements for indicated SL-PRS Rx ARP-IDs. Bit 1 in the bit string indicates ARP-ID = 1, bit 2 indicates ARP-ID = 2, and so on.

sl-AdditionalPathsRequest This field, if present, indicates that the UE is requested to provide sl-TOA-AdditionalPathList.

sl-ARP-InfoRequest This field, if present, indicates that the UE is requested to provide sl-POS-ARP-ID-Rx.

sl-LOS-NLOS-IndicatorRequest This field, if present, indicates that the UE is requested to provide the estimated los-NLOS-Indicator.

sl-RSRPP-Request This field, if present, indicates that the UE is requested to provide sl-RSRPP.

sl-PRS-RSRP-Request This field, if present, indicates that the UE is requested to provide sl-PRS-RSRP-Result.

#### –	SL-TOA-ProvideLocationInformation

-- ASN1START

-- TAG-SL-TOA-PROVIDELOCATIONINFORMATION-START

SL-TOA-ProvideLocationInformation ::= SEQUENCE {

sl-TOA-SignalMeasurementInformation   SL-TOA-MeasElementPerARP-ID-Rx         OPTIONAL,

sl-TOA-Error                          SL-TOA-LocationInformationError        OPTIONAL,

...

}

SL-TOA-MeasElementPerARP-ID-Rx ::= SEQUENCE (SIZE(1..4)) OF SL-TOA-MeasElement

SL-TOA-MeasElement ::= SEQUENCE {

los-NLOS-Indicator                    LOS-NLOS-Indicator        OPTIONAL,  -- sl-losNlosIndicator

sl-RTOA-Result                        CHOICE {

k0                                    INTEGER (0..1970049),

k1                                    INTEGER (0..985025),

k2                                    INTEGER (0..492513),

k3                                    INTEGER (0..246257),

k4                                    INTEGER (0..123129),

k5                                    INTEGER (0..61565)

}                                                               OPTIONAL,  -- sl-PRS-RTOA

sl-POS-ARP-ID-Rx                      INTEGER (1..4)            OPTIONAL,  -- sl-pos-arpID-Rx

sl-PRS-ResourceId                     INTEGER (0..16)           OPTIONAL,  -- sl-PRS-ResourceId

sl-PRS-RSRP-Result                    INTEGER (0..126)          OPTIONAL,  -- sl-PRS-RSRP

sl-PRS-RSRPP-Result                   INTEGER (0..126)          OPTIONAL,  -- sl-PRS-RSRPP

sl-TOA-AdditionalPathList             SL-TOA-AdditionalPathList OPTIONAL,

sl-TimeStamp                          SL-TimeStamp              OPTIONAL,  -- sl-Timestamp

sl-TimingQuality                      SL-TimingQuality          OPTIONAL,  -- sl-TimingQuality

...,

[[

sl-TOA-AdditionalMeasurements       SL-TOA-AdditionalMeasurements      OPTIONAL

]]

}

SL-TOA-AdditionalMeasurements ::= SEQUENCE (SIZE (1..3)) OF SL-TOA-AdditionalMeasurementElement

SL-TOA-AdditionalMeasurementElement ::= SEQUENCE {

sl-PRS-ResourceId                     INTEGER (0..16)               OPTIONAL,

sl-PRS-RSRP-Result                    INTEGER (0..126)              OPTIONAL,

sl-PRS-RSRPP-Result                   INTEGER (0..126)              OPTIONAL,

sl-RTOA-Result                        CHOICE {

k0                                  INTEGER (0..1970049),

k1                                  INTEGER (0..985025),

k2                                  INTEGER (0..492513),

k3                                  INTEGER (0..246257),

k4                                  INTEGER (0..123129),

k5                                  INTEGER (0..61565)

}                                                                   OPTIONAL,

los-NLOS-Indicator                    LOS-NLOS-Indicator            OPTIONAL,

sl-TOA-AdditionalPathList             SL-TOA-AdditionalPathList     OPTIONAL,

sl-TimeStamp                          SL-TimeStamp                  OPTIONAL,

sl-TimingQuality                      SL-TimingQuality              OPTIONAL,

...

}

SL-TOA-AdditionalPathList ::= SEQUENCE (SIZE(1..8)) OF SL-TOA-AdditionalPath

SL-TOA-AdditionalPath  ::= SEQUENCE {

sl-RTOA-AdditionalPathResult               CHOICE {

k0                                         INTEGER (0..16351),

k1                                         INTEGER (0..8176),

k2                                         INTEGER (0..4088),

k3                                         INTEGER (0..2044),

k4                                         INTEGER (0..1022),

k5                                         INTEGER (0..511)

}                                                                OPTIONAL,  -- additionalPath-SL-PRS-RTOA

sl-PRS-AdditionalPathRSRPP-Result          INTEGER (0..126)      OPTIONAL,  -- additionalPath-SL-PRS-RSRPP

sl-TimingQuality                           SL-TimingQuality      OPTIONAL,  -- sl-TimingQuality

...

}

SL-TOA-LocationInformationError ::= ENUMERATED { undefined, assistanceDataMissing, notAllRequestedMeasurementsPossible, ... }

-- TAG-SL-TOA-PROVIDELOCATIONINFORMATION-STOP

-- ASN1STOP

SL-TOA-ProvideLocationInformation field descriptions

los-NLOS-Indicator This field specifies the UE's best estimate of the LOS or NLOS of the UE measurements (including RSTD, RTOA, RSRP, RSRPP, AoA and UE Rx-Tx time difference).

sl-TOA-AdditionalPathList This field specifies the sidelink PRS measurements based on additional path of arrival.

sl-POS-ARP-ID-Rx This field indicates ARP ID of an ARP used for reception for per-ARP measurement reporting. The ARP ID is used to uniquely identify an ARP associated with a UE.

sl-PRS-ResourceId This field specifies the PRS sidelink resource ID used for SL positioning measurements.

sl-PRS-RSRP-Result This field specifies the sidelink PRS reference signal received power (RSRP) measurement.

sl-PRS-RSRPP-Result This field specifies the SL-RSRPP measurement based on first path of arrival.

sl-RTOA-Result This field specifies the SL-RTOA measurement based on first path of arrival. In this release, this field is always included.

sl-TimeStamp This field specifies the time instance at which the SL RTOA and SL-PRS RSRP (if included) measurement is performed.