3GPP TSG-RAN5 Meeting #106	R5-251279

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4801 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to 5GMM TC 9.1.13.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, eNS_Ph2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-01-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Table 9.1.13.2.3.3-4, the encoding of ST is incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | ST value was updated in Table 9.1.13.2.3.3-4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect configuration is used in this test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.13.2.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This is the outcome of R5-250086 with one revision. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-250086 -> R5-251279 |  |  |  |  |  |  |  |  |

[Start of change]

9.1.13.2	NSSRG / Generic UE configuration update

9.1.13.2.1	Test Purpose (TP)

(1)

with { UE has received configured NSSAI }

ensure that {

when { UE receives new configured NSSAI in the CONFIGURATION UPDATE COMMAND message with NSSRG information }

then { UE stores the new configured NSSAI and the NSSRG information }

}

9.1.13.2.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.501 clauses 5.5.1.2.2. Unless otherwise stated these are Rel-17 requirements.

[TS 24.501 clause 5.5.1.2.2]

The UE in state 5GMM-DEREGISTERED shall initiate the registration procedure for initial registration by sending a REGISTRATION REQUEST message to the AMF,

a)	when the UE performs initial registration for 5GS services;

b)	when the UE performs initial registration for emergency services;

c)	when the UE performs initial registration for SMS over NAS;

d)	when the UE moves from GERAN to NG-RAN coverage or the UE moves from a UTRAN to NG-RAN coverage and the following applies:

1)	the UE initiated a GPRS attach or routing area updating procedure while in A/Gb mode or Iu mode; or

2)	the UE has performed 5G-SRVCC from NG-RAN to UTRAN as specified in 3GPP TS 23.216 [6A],

and since then the UE did not perform a successful EPS attach or tracking area updating procedure in S1 mode or registration procedure in N1 mode;

e)	when the UE performs initial registration for onboarding services in SNPN; and

f)	when the UE performs initial registration for disaster roaming services;

…

The subset of configured NSSAI provided in the requested NSSAI consists of one or more S-NSSAIs in the configured NSSAI applicable to the current PLMN or SNPN, if the S-NSSAI is neither in the rejected NSSAI nor associated to the S-NSSAI(s) in the rejected NSSAI. In addition, if the NSSRG information is available, the subset of configured NSSAI provided in the requested NSSAI shall be associated with at least one common NSSRG value. If the UE is in 5GMM-REGISTERED state over the other access and has already an allowed NSSAI for the other access, all the S-NSSAI(s) in the requested NSSAI for the current access shall share at least an NSSRG value common to all the S-NSSAI(s) of the allowed NSSAI for the other access. If the UE is simultaneously performing the registration procedure on the other access, the UE shall include S-NSSAIs that share at least a common NSSRG value across all access types.

If the UE supports the unavailability period, the UE shall set the UN-PER bit to "unavailability period supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

…

[TS 24.501 clause 5.4.4.3]

Upon receiving the CONFIGURATION UPDATE COMMAND message, the UE shall stop timer T3346 if running and use the contents to update appropriate information stored within the UE.

If "acknowledgement requested" is indicated in the Acknowledgement bit of the Configuration update indication IE in the CONFIGURATION UPDATE COMMAND message, the UE shall send a CONFIGURATION UPDATE COMPLETE message.

…

If the UE receives a new configured NSSAI in the CONFIGURATION UPDATE COMMAND message, the UE shall consider the new configured NSSAI for the registered PLMN or SNPN as valid and the old configured NSSAI for the registered PLMN or SNPN as invalid; otherwise, the UE shall consider the old configured NSSAI for the registered PLMN or SNPN as valid. The UE shall store the new configured NSSAI as specified in subclause 4.6.2.2. In addition, if the CONFIGURATION UPDATE COMMAND message contains an NSSRG information IE, the UE shall store the contents of the NSSRG information IE as specified in subclause 4.6.2.2. If the UE receives a new configured NSSAI in the CONFIGURATION UPDATE COMMAND message and no NSSRG information IE, the UE shall delete any stored NSSRG information, if any, as specified in subclause 4.6.2.2.

9.1.13.2.3	Test description

9.1.13.2.3.1	Pre-test conditions

System Simulator:

-	NGC Cell A belongs to Home PLMN and TAI1 and set as serving cell;

UE:

-	None;

Preamble:

-	The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1 [4].

9.1.13.2.3.2	Test procedure sequence

Table 9.1.13.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2-3 | Steps 2 to 3 of the generic procedure for NR RRC_IDLE specified in TS 38.508-1 subclause 4.5.2 are performed. | - | - | - | - |
| 4 | Check: Does UE transmit a REGISTRATION REQUEST message including NSSRG bit? | --> | REGISTRATION REQUEST | 1 | P |
| 5-13 | Steps 5 to 13 of the generic procedure for NR RRC_IDLE specified in TS 38.508-1 subclause 4.5.2 are performed. | - | - | - | - |
| 14 | The SS transmits a REGISTRATION ACCEPT message including Configured NSSAI with NSSRG information included. | <-- | REGISTRATION ACCEPT | - | - |
| 15-19a1 | Steps 15 to 19a1 of the generic procedure for NR RRC_IDLE specified in TS 38.508-1 subclause 4.5.2 are performed. | - | - | - | - |
| 20 | The SS transmits a CONFIGURATION UPDATE COMMAND message including NSSRG information. | <-- | CONFIGURATION UPDATE COMMAND | - | - |
| 21 | Check: Does UE transmit a CONFIGURATION UPDATE COMPLETE message? | --> | CONFIGURATION UPDATE COMPLETE | 1 | P |
| 22 | Switch off procedure in RRC_CONNECTED specified in TS 38.508-1 subclause 4.9.6.3 is performed. | - | - | - | - |
| 23 | UE is switched on. | - | - | - | - |
| 24-25 | Steps 2 to 3 of the generic procedure for NR RRC_IDLE specified in TS 38.508-1 subclause 4.5.2 are performed. | - | - | - | - |
| 26 | Check: Does UE transmit a REGISTRATION REQUEST message with requested NSSAI which are new configured NSSAI in CONFIGURATION UPDATE COMMAND message and associated with common NSSRG value? | --> | REGISTRATION REQUEST | 1 | P |
| 27-41a1 | Steps 5 to 19a1 of the generic procedure for NR RRC_IDLE specified in TS 38.508-1 subclause 4.5.2 are performed. | - | - | - | - |

9.1.13.2.3.3	Specific message contents

Table 9.1.13.2.3.3-1: REGISTRATION REQUEST (step 4, Table 9.1.13.2.3.2-1)

| Derivation path: TS 38.508-1 Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| 5GS registration type value | ‘001’B | Initial registration |  |
| 5GMM capability |  |  |  |
| NSSRG | ‘1’B | NSSRG supported |  |

Table 9.1.13.2.3.3-2: REGISTRATION ACCEPT (step 14, Table 9.1.13.2.3.2-1)

| Derivation path: TS 38.508-1 Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Allowed NSSAI |  |  |  |
| Length of NSSAI contents | 1 entry |  |  |
| S-NSSAI |  | S-NSSAI value 2 |  |
| Length of S-NSSAI contents | ‘00000001’B | SST |  |
| SST | ‘00000010’B | SST value 2 |  |
| SD | Not Present |  |  |
| Mapped configured SST | Not Present |  |  |
| Mapped configured SD | Not Present |  |  |
| Configured NSSAI |  |  |  |
| Length of NSSAI contents | 1 entry |  |  |
| S-NSSAI |  | S-NSSAI value 2 |  |
| Length of S-NSSAI contents | ‘00000001’B | SST |  |
| SST | ‘00000010’B | SST value 2 |  |
| SD | Not Present |  |  |
| Mapped configured SST | Not Present |  |  |
| Mapped configured SD | Not Present |  |  |
| NSSRG information |  |  |  |
| Length of NSSRG information contents | 1 entry |  |  |
| NSSRG values for S-NSSAI 2 |  |  |  |
| Length of NSSRG values for S-NSSAI | 2 entries |  |  |
| S-NSSAI value |  |  |  |
| S-NSSAI |  | S-NSSAI value 2 |  |
| Length of S-NSSAI contents | ‘00000001’B | SST |  |
| SST | ‘00000010’B | SST value 2 |  |
| SD | Not Present |  |  |
| Mapped configured SST | Not Present |  |  |
| Mapped configured SD | Not Present |  |  |
| NSSRG value 1 for the S-NSSAI | ‘00000010’B |  |  |
| NSSRG value 2 for the S-NSSAI | ‘00000011’B |  |  |

Table 9.1.13.2.3.3-3: CONFIGURATION UPDATE COMMAND (step 20, Table 9.1.13.2.3.2-1)

| Derivation path: TS 38.508-1 Table 4.7.1-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Configuration update indication | 0011 | Acknowledgement requested Registration Requested |  |
| Allowed NSSAI |  |  |  |
| Length of NSSAI contents | 2 entries |  |  |
| S-NSSAI |  | S-NSSAI value 4 |  |
| Length of S-NSSAI contents | ‘00000001’B | SST |  |
| SST | ‘00000100’B | SST value 4 |  |
| SD | Not Present |  |  |
| Mapped configured SST | Not Present |  |  |
| Mapped configured SD | Not Present |  |  |
| S-NSSAI |  | S-NSSAI value 5 |  |
| Length of S-NSSAI contents | ‘00000001’B | SST |  |
| SST | ‘00000101’B | SST value 5 |  |
| SD | Not Present |  |  |
| Mapped configured SST | Not Present |  |  |
| Mapped configured SD | Not Present |  |  |
| NSSRG information |  |  |  |
| Length of NSSRG information contents | 1 entry |  |  |
| NSSRG values for S-NSSAI 1 |  |  |  |
| Length of NSSRG values for S-NSSAI | 1 entry |  |  |
| S-NSSAI value |  |  |  |
| S-NSSAI |  | S-NSSAI value 4 |  |
| Length of S-NSSAI contents | ‘00000001’B | SST |  |
| SST | ‘00000100’B | SST value 4 |  |
| SD | Not Present |  |  |
| Mapped configured SST | Not Present |  |  |
| Mapped configured SD | Not Present |  |  |
| NSSRG value 1 for the S-NSSAI | ‘00000001’B |  |  |

Table 9.1.13.2.3.3-4: REGISTRATION REQUEST (step 26, Table 9.1.13.2.3.2-1)

| Derivation path: TS 38.508-1 Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| 5GS registration type value | ‘001’B | Initial registration |  |
| Requested NSSAI |  |  |  |
| Length of NSSAI contents | 1 entry |  |  |
| S-NSSAI |  | S-NSSAI value 4 |  |
| Length of S-NSSAI contents | ‘00000001’B | SST |  |
| SST | ‘00000100’B | SST value 4 |  |
| SD | Not Present |  |  |
| Mapped configured SST | Not Present |  |  |
| Mapped configured SD | Not Present |  |  |

[End of change]