3GPP TSG-RAN5 Meeting #106	R5-251694

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-3 | CR | 3081 | rev | 1 | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NPDCCH and NPDSCH repetition for nb-ntn DCQR and random access test cases in enhanced coverage |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | QUALCOMM Europe Inc. - Italy |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | NB NTN enhanced coverage DCQR tests procedure currently requires scheduling of 4500 NPDCCH for verifyng the DCQR requirements. These test cases are run in static channel. Hence there is no expected channel variation that will require running the test for such long duration. A much shorter test time is sufficient for this static channel case. For DCQR and random access NB-NTN test cases in enhanced coverage, requires paging the UE to trigger NPRACH. The configured SNR of -12 dB in these test cases is very low for UE to reliably detect the NPDCCH carrying the page. Increasing the NPDCCH repetition carrying the page to a high enough value 32 does not alter the actual purpose of the test case and at the same time ensures UE can reliably decode the page. |  |  |  |  |  |  |  |  |
| Summary of change: |  | -- Reduced the number of NPDCCH to be scheduled for test cases 13.6.2.2,13.6.2.4,13.6.2.5,13.6.2.6 -- Increased the NPDCCH repetition for P-RNTI to 32 for NB-NTN test cases DCQR and random access test cases in enhanced coverage |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test duration will unncessarily be long |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.3.2.2, 13.3.2.3, 13.6.2.2, 13.6.2.4, 13.6.2.5, 13.6.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: removing changes to npdcch repetitions for paging since CR in R5-250030r2 introduces same change via default message contents. R5-250860r1 R5-251694 |  |  |  |  |  |  |  |  |

#### 13.3.2.2	Contention Based Random Access Test for UE category NB1 UEs in Satellite Access - Standalone mode in Enhanced Coverage

##### 13.3.2.2.1	Test purpose

The purpose of this test is to verify whether the behavior of the random access procedure of a category NB1 UE in Enhanced Coverage is according to the requirements when connected to a NTN NB-IoT cell, whether the NPRACH power settings and timing are within specified limits, and whether the UE determines properly the enhanced coverage level based on the NRSRP measurement and the configured criterion in NRSRP-ThresholdsPrach[5]. This test will verify the requirements in TS36.133 Clause 6.6A.2, Clause 6.6A.3 and Clause 7.20A.2 in an AWGN model.

For this test a single NB-IoT cell is used. The test parameters are given in tables 13.3.2.1.5-1, 13.3.2.1.5-2 and 13.3.2.1.5-3. The UE shall perform timing pre-compensation before the initial NPRACH transmission using AT command-based test approach.

##### 13.3.2.2.2	Test applicability

This test case applies to all types of NB-IoT HD-FDD category NB1 UEs supporting GSO or NGSO or both from release 17 and forwards.

##### 13.3.2.2.3	Minimum conformance requirements

Contention based random access is triggered by not explicitly assigning a random access preamble via dedicated signalling in the downlink.

The UE may stop monitoring for Random Access Response(s) and shall transmit the msg3 if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble.

The UE shall re-select a preamble and transmit with the calculated NPRACH transmission power when the backoff time expires if all received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

The UE shall re-select a preamble and transmit with the calculated NPRACH transmission power when the backoff time expires if no Random Access Response is received within the RA Response window. The RA response window shall be started at the point in time indicated by clause 5.1.4 in TS 36.321[11].

The UE shall re-transmit the msg3 upon the reception of a NACK on msg3 until the maximum number of re-transmissions defined by maxNumPreambleAttemptCE in the table 13.3.2.2.5-3 is reached.

The UE shall re-select a preamble and transmit with the calculated NPRACH transmission power when the backoff time expires unless the received message includes a UE Contention Resolution Identity MAC control element and the UE Contention Resolution Identity included in the MAC control element matches the CCCH SDU transmitted in the uplink message.

The UE shall send ACK if the Contention Resolution is successful.

The UE shall re-select a preamble and transmit with the calculated NPRACH transmission power when the backoff time expires if the Contention Resolution Timer expires.

The UE shall select NPRACH resources and transmits or re- transmits NPRACH preambles using the NPRACH resources and NPRACH configuration corresponding to the coverage enhancement level 1. The rate of correct coverage enhancement level selection during repeated tests shall be at least 90%.

Note:	Correct coverage enhancement level selection is a prerequisite for testing the other NPRACH requirements.

The normative reference for this requirement is 3GPP TS 36.133 [4] clause A.13.3.2.2.

##### 13.3.2.2.4	Test description

13.3.2.2.4.1	Initial conditions

Test Environment: Normal,TL/VL, TL/VH, TH/VL, TH/VH;  as defined in 3GPP TS 36.508 [7] clause 8.1.1.

Frequencies to be tested: According to Annex E table E-4 and 3GPP TS 36.508 [7] clauses 8.1.3 and 8.1.4.2.

Channel Bandwidth to be tested: Ncell bandwidth is as specified in Table 13.3.2.2.5-1.

1.	Connect the SS and AWGN noise source to the UE antenna connectors as shown in 3GPP TS 36.508 [7] Annex A Figure A.18 using only UE main Tx/Rx antenna.

2.	Propagation conditions are set according to Annex B clause B.0.

3.	There is one NB-IoT cell specified in the test. Ncell 1 is the cell used for registration with the power level set according to Annex C.0 and C.1 for this test.

4.	UE location according to TS 36.508 [12] clause 8.4.6.1 is provided to the UE through any preconfigured means.

5.	Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 36.508 [12] clause 8.4.6.2.1 depending on the type of satellite under test. Test system shall send same SIB31-NB information during the duration of the test as defined in TS 36.508 [12] clause 8.4.6.3.1.

6.	Deactivate UE prediction of satellite trajectory through any preconfigured means.

13.3.2.2.4.2	Test procedure

The test scenario comprises of one NB-IoT carrier with 1 Ncell.

1.	Ensure the UE is in State 3A-NB with CP CIoT Optimisation according to TS 36.508 [7] clause 8.1.5 in Ncell 1.

2.	Set the parameters according to Tables 13.3.2.2.5-1 to 13.3.2.2.5-3 selecting values appropriate to the test condition of normal or extreme. Propagation conditions are set according to Annex B clause B.1.1.

3.	The UE shall establish a connection setup with SS, the random access procedure within the connection setup is used in the test.

4.	Test 1: Correct behaviour when receiving random access response reception.

4.1.	In Test 1, the UE shall send the preamble to the SS. In response to the first 4 preamble transmission attempts (the preamble is transmitted 8 times in each attempt), the SS shall transmit a random access response not corresponding to the transmitted random access preamble.

4.2.	The UE shall consider the random access response reception not successful then re-select a preamble and transmit with the calculated NPRACH transmission power when the backoff time expires if all received random access response contain random access preamble identifiers that do not match the transmitted random access preamble.

4.3.	The SS shall transmit a random access response containing a random access preamble identifier corresponding to the transmitted random access after 5 preamble transmission attempts have been received by the SS.

4.4.	The UE shall consider this random access response reception successful and transmit the msg3.

4.5.	Measure the power and timing of the first preamble transmission attempt. The power of the first preamble transmission attempt shall be 23 dBm for power class 3, 20 dBm for power class 5 with the accuracy specified in clause 6.3B.4 of TS 36.102. Measure the relative power and timing applied to additional preamble transmission attempts. The relative power of the last 4 preamble transmission attempts shall not exceed the values specified in clause 6.3B.4 of TS 36.102. The transmit timing of all NPRACH transmissions shall be within the accuracy specified in TS36.133 clause 7.20A.2. If the UE transmits or re-transmits NPRACH preambles using the NPRACH resources and NPRACH configuration corresponding to the coverage enhancement level 1, then for each preamble count a success for the event "CE selection". Otherwise count a fail for the event "CE selection". The power, relative power and timing are only measured if the coverage enhancement level selection was correct, and any results relating to a wrong selection are discarded.

5.	Test 2: Correct behaviour when not receiving random access response reception

5.1.	Repeat step 1-3.

5.2.	In Test 2, the UE shall send the preamble to the SS. The SS shall not respond to the first 4 preamble transmission attempts (the preamble is transmitted 8 times in each attempt).

5.3.	The UE shall consider the random access response reception not successful then re-select a preamble and transmit with the calculated NPRACH transmission power when the backoff time expires if no random access response is received within the RA Response window. The RA response window shall be started at the point in time indicated by clause 5.1.4 in TS 36.321.

5.4.	The SS shall transmit a random access response containing a random access preamble identifier corresponding to the transmitted random access after 5 preamble transmission attemps have been received by the SS.

5.5.	The UE shall consider this random access response reception successful and transmit the msg3.

5.6.	Measure the power and timing of the first preamble transmission attempt. The power of the first preamble transmission attempt shall be 23 dBm for power class 3, 20 dBm for power class 5 with the accuracy specified in clause 6.3B.4 of TS 36.102. Measure the relative power and timing applied to additional preamble transmission attempts. The relative power of the last 4 preamble transmission attempts shall not exceed the values specified in clause 6.3B.4 of TS 36.102. The transmit timing of all NPRACH transmissions shall be within the accuracy specified in TS36.133 clause 7.20A.2. If the UE transmits or re-transmits NPRACH preambles using the NPRACH resources and NPRACH configuration corresponding to the coverage enhancement level 1, then for each preamble count a success for the event "CE selection". Otherwise count a fail for the event "CE selection". The power, relative power and timing are only measured if the coverage enhancement level selection was correct, and any results relating to a wrong selection are discarded.

6.	Test 3:  Not applicable.

7.	Test 4: Correct behaviour when receiving an incorrect message over Temporary C-RNTI

7.1.	Repeat step 1-3.

7.2.	In Test 4, the UE shall send the preamble to the SS. If the UE transmits an NPRACH preamble using the NPRACH resources and NPRACH configuration corresponding to the coverage enhancement level 1, then count a success for the event "CE selection". Otherwise count a fail for the event "CE selection". The SS shall transmit a random access response containing a random access preamble identifier corresponding to the transmitted random access after the first preamble has been received by the SS.

7.3.	The UE shall consider this random access response reception successful and transmit the msg3.

7.4.	The SS shall send a message addressed to the Temporary C-RNTI with a UE contention resolution identity included in the MAC control element not matching the CCCH SDU transmitted in msg3 uplink message.

7.5.	The UE shall consider the contention resolution not successful then re-select a preamble and transmit with the calculated NPRACH transmission power when the backoff time expires. If the UE transmits NPRACH preambles using the NPRACH resources and NPRACH configuration corresponding to the coverage enhancement level 1, then for each preamble count a success for the event "CE selection". Otherwise count a fail for the event "CE selection". The power is only measured if the coverage enhancement level selection was correct, and any results relating to a wrong selection are discarded.

8.	Test 5: Correct behaviour when receiving a correct message over Temporary C-RNTI

8.1.	In Test 5, the SS shall transmit a random access response containing a random access preamble identifier corresponding to the transmitted random access after the first preamble has been received by the SS. If the UE transmits an NPRACH preamble using the NPRACH resources and NPRACH configuration corresponding to the coverage enhancement level 1, then count a success for the event "CE selection". Otherwise count a fail for the event "CE selection".

8.2.	The UE shall consider this random access response reception successful and transmit the msg3.

8.3.	The SS shall send a message addressed to the Temporary C-RNTI with a UE contention resolution identity included in the MAC control element matching the CCCH SDU transmitted in the msg3 uplink message.

8.4.	The UE shall send ACK and consider the contention resolution successful and the random access procedure successfully completed.

9.	Test 6: Correct behaviour when contention resolution timer expires

9.1.	Repeat step 1-3.

9.2.	In Test 6, the UE shall send the preamble to the SS. The SS shall transmit a random access response containing a random access preamble identifier corresponding to the transmitted random access after the first preamble has been received by the SS. If the UE transmits an NPRACH preamble using the NPRACH resources and NPRACH configuration corresponding to the coverage enhancement level 1, then count a success for the event "CE selection". Otherwise count a fail for the event "CE selection".

9.3.	The UE shall consider this random access response reception successful and transmit the msg3.

9.4.	The SS shall send an ACK for msg3 but not send msg4 allowing the contention resolution timer to expire.

9.5.	The UE shall consider the contention resolution not successful then re-select a preamble and transmit with the calculated PRACH transmission power when the backoff time expires. If the UE transmits an NPRACH preambles using the NPRACH resources and NPRACH configuration corresponding to the coverage enhancement level 1, then for each preamble count a success for the event "CE selection". Otherwise count a fail for the event "CE selection". The power is only measured if the coverage enhancement level selection was correct, and any results relating to a wrong selection are discarded.

9.6.	The SS shall transmit a random access response containing a random access preamble identifier corresponding to the transmitted random access after the first preamble has been received by the SS.

9.7.	The UE shall consider this random access response reception successful and transmit the msg3.

10.	Void.

13.3.2.2.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 8.1.5B and 8.1.6 with following exceptions.

<< several sections skipped >>

#### 13.3.2.3	Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs Standalone mode in Enhanced Coverage

##### 13.3.2.3.1	Test purpose

The purpose of this test is to verify whether the behavior of the random access procedure of a category NB1 UE in Enhanced Coverage is according to the requirements, whether the NPRACH power settings and timing are within specified limits, and whether the UE determines properly the enhanced coverage level based on the NRSRP measurement and the configured criterion in NRSRP-ThresholdsPrach [5]. This test will verify the requirements in TS36.133 Clause 6.6A.2, Clause 6.6A.3 and Clause 7.20A2 in an AWGN model.

For this test a single NB-IoT cell is used. The test parameters are given in tables 13.3.2.3.5-1, 13.3.2.3.5-2 and 13.3.2.3.5-3. The UE shall perform timing pre-compensation before the initial NPRACH transmission using AT command-based test approach.

##### 13.3.2.3.2	Test applicability

This test case applies to all types of NB-IoT HD-FDD category NB1 UEs supporting GSO or NGSO or both  from release 17 and forwards.

Applicability requires support for sending NPRACH on non-anchor carrier.

##### 13.3.2.3.3	Minimum conformance requirements

Contention based random access is triggered by not explicitly assigning a random access preamble via dedicated signalling in the downlink.

The UE may stop monitoring for Random Access Response(s) and shall transmit the msg3 if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble.

The UE shall re-select a preamble and transmit with the calculated NPRACH transmission power when the backoff time expires if all received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

The UE shall re-select a preamble and transmit with the calculated NPRACH transmission power when the backoff time expires if no Random Access Response is received within the RA Response window. The RA response window shall be started at the point in time indicated by clause 5.1.4 in TS 36.321[11].

The UE shall re-transmit the msg3 upon the reception of a NACK on msg3 until the maximum number of re-transmissions defined by maxNumPreambleAttemptCE in the table 13.3.2.3.5-3 is reached.

The UE shall re-select a preamble and transmit with the calculated NPRACH transmission power when the backoff time expires unless the received message includes a UE Contention Resolution Identity MAC control element and the UE Contention Resolution Identity included in the MAC control element matches the CCCH SDU transmitted in the uplink message.

The UE shall send ACK if the Contention Resolution is successful.

The UE shall re-select a preamble and transmit with the calculated NPRACH transmission power when the backoff time expires if the Contention Resolution Timer expires.

The UE shall select NPRACH resources and transmits or re- transmits NPRACH preambles using the NPRACH resources and NPRACH configuration corresponding to the coverage enhancement level 1. The rate of correct coverage enhancement level selection during repeated tests shall be at least 90%.

Note:	Correct coverage enhancement level selection is a prerequisite for testing the other NPRACH requirements.

The normative reference for this requirement is 3GPP TS 36.133 [4] clause A.13.3.2.3.

##### 13.3.2.3.4	Test description

13.3.2.3.4.1	Initial conditions

Test Environment: Normal, TL/VL, TL/VH, TH/VL, TH/VH as defined in 3GPP TS 36.508 [7] clause 8.1.1.

Frequencies to be tested: According to Annex E table E-4 and 3GPP TS 36.508 [7] clauses 8.1.3 and 8.1.4.2.

Channel Bandwidth to be tested: Ncell bandwidth is as specified in Table 13.3.2.3.5-1.

1.	Connect the SS and AWGN noise source to the UE antenna connectors as shown in 3GPP TS 36.508 [7] Annex A Figure A.18 using only UE main Tx/Rx antenna.

2.	Propagation conditions are set according to Annex B clause B.0.

3.	There is one NB-IoT cell specified in the test. Ncell 1 is the cell used for registration with the power level set according to Annex C.0 and C.1 for this test. In Ncell 1 one anchor carrier and one non-anchor carrier are configured.

4.	UE location according to TS 36.508 [12] clause 8.4.6.1 is provided to the UE through any preconfigured means.

5.	Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 36.508 [12] clause 8.4.6.2.1 depending on the type of satellite under test. Test system shall send same SIB31-NB information during the duration of the test as defined in TS 36.508 [12] clause 8.4.6.3.1.

6.	Deactivate UE prediction of satellite trajectory through any preconfigured means.

13.3.2.3.4.2	Test procedure

The test scenario comprises of 2 NB-IoT carriers with 1 Ncell.

The test procedures are the same as test procedures defined in 13.3.2.2.4.2, with step 4 and forward performed on the non-anchor carrier.

13.3.2.3.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 8.1.5B and 8.1.6 with following exceptions.

<< several sections skipped >>

#### 13.6.2.2	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under enhanced coverage

##### 13.6.2.2.1	Test purpose

The purpose of this test is to verify that the downlink channel quality reporting accuracy is within the specified limits. This test will verify the requirements in TS36.133[4] Section 9.1.22A.8 for NB-IoT SAN PCell.

##### 13.6.2.2.2	Test applicability

This test case applies to all types of NB-IoT HD-FDD category NB1 UEs supporting GSO or NGSO or both and DL channel quality reporting for anchor carrier for FDD in Msg3 from release 17 and forwards.

##### 13.6.2.2.3	Minimum conformance requirements

The requirements for accuracy of downlink channel quality reporting in this clause apply to the serving cell on the anchor carrier and non-anchor carrier for UE Category NB1.

The accuracy requirements in Table 13.6.2.2.3-1 are valid under the following conditions:

-	Cell specific reference signals are transmitted either from one port.

-	Conditions defined in TS 36.102 Clause 7.3B for reference sensitivity are fulfilled.

-	NRSRP|dBm according to TS36.133 Annex B.3.25A for a corresponding Band.

Table 13.6.2.2.3-1: Downlink channel quality reporting accuracy for UE Category NB1

| NPDCCH Repetition | Pm-Dsg (%) | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Ês/Iot | Io NOTE 2 range |  |  |  |
|  |  |  | E-UTRA/NR operating band groups NOTE 3 | Minimum Io |  | Maximum Io |
|  |  | dB |  | dBm/15kHz | dBm/BWChannel | dBm/BWChannel |
| R NOTE 1 | ≤1 | -6 dB | NFDD_SAB_G | -122.9 | N/A | -70 |
| R/4 NOTE 1 | >1 | -6 dB | NFDD_SAB_G | -122.9 | N/A | -70 |
| R NOTE 1 | ≤1 | -15 ≤ Ês/Iot ≤ -6 dB | NFDD_SAB_G | -122.9 | N/A | -70 |
| R/8 NOTE 1 | >1 | -15 ≤ Ês/Iot ≤ -6 dB | NFDD_SAB_G | -122.9 | N/A | -70 |
| NOTE 1: R is the reported NPDCCH repetition level that UE has reported in CQI-NPDCCH-NB or CQI-NPDCCH-Short-NB. NOTE 2: Io is assumed to have constant EPRE across the bandwidth. NOTE 3: E-UTRA/NR operating band groups are as defined in TS36.133 Section 3.5. |  |  |  |  |  |  |

The normative reference for this requirement is 3GPP TS 36.133 [4] clause A.13.6.2.2.

##### 13.6.2.2.4	Test description

13.6.2.2.4.1	Initial conditions

Test Environment: Normal, as defined in 3GPP TS 36.508 [7] clause 8.1.1.

Frequencies to be tested: According to Annex E table E-4 and 3GPP TS 36.508 [7] clauses 8.1.3 and 8.1.4.2.

Channel Bandwidth to be tested: Ncell bandwidth is as specified in Table 13.6.2.2.5-2.

1.	Connect the SS and AWGN noise source to the UE antenna connectors as shown in 3GPP TS 36.508 [7] Annex A Figure A.18 using only UE main Tx/Rx antenna.

2.	Propagation conditions are set according to Annex B clause B.0.

3.	There is one NB-IoT cell specified in the test. Ncell 1 is the cell used for registration with the power level set according to Annex C.0 and C.1 for this test.

4.	UE location according to TS 36.508 [12] clause 8.4.6.1 is provided to the UE through any preconfigured means.

5.	Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 36.508 [12] clause 8.4.6.2.1 depending on the type of satellite under test. Test system shall send same SIB31-NB information during the duration of the test as defined in TS 36.508 [12] clause 8.4.6.3.1.

6.	Deactivate UE prediction of satellite trajectory through any preconfigured means.

The UE shall be provided with the valid information about the SAN serving cells before the test.

Table 13.6.2.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, HD-FDD duplex mode |
| 2 | NGSO, HD-FDD duplex mode |
| Note: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. |  |

13.6.2.2.4.2	Test procedure

The test scenario comprises of one NB-IoT carrier with 1 Ncell.

1.	Set the parameters according to Tables 13.6.2.2.5-1 and 13.6.2.2.5-2. Propagation conditions are set according to Annex B clause B.1.1.

2.	 Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [7] clause 8.1.5 in Ncell 1. (The UE shall transmit msg3 with the NPDCCH repetition number R report according to TS36.133 Table 9.1.22.15-1 after successful reception of the RAR.)

3.	The SS config Rmax=R based on the UE report in step 2 ensuring the UE is in State 2A-NB with new Rmax value.

4.	The SS transmits NPDSCH via NPDCCH DCI format N1 according to the UE reported NPDCCH repetition number R. The SS sends downlink MAC padding bits on the DL RMC.

5.	Measure the Pm-dsg for a duration of 100 NPDCCH. SS counts the number of NACKs, ACKs and statDTXs on the UL NPUSCH. Pm-dsg is the ratio according to the formula (statDTX)/(NACK+ACK+statDTX). If Pm-dsg is ≤ 1%, continue with step 6. If R/8 is unconfigurable according to TS 36.213 [8] clause16.6, skip following test steps and pass the UE. Otherwise fail the UE.

6.	The SS transmits NPDSCH via NPDCCH DCI format N1 according to NPDCCH repetition number R/8. The SS sends downlink MAC padding bits on the DL RMC.

7.	Measure the Pm-dsg for a duration of 100 NPDCCH. SS counts the number of NACKs, ACKs and statDTXs on the UL NPUSCH. Pm-dsg is the ratio according to the formula (statDTX)/(NACK+ACK+statDTX). If Pm-dsg is > 1%, pass the UE. Otherwise fail the UE.

13.6.2.2.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 8.1.4, 8.1.5B and 8.1.6 with following exceptions.

Table 13.6.2.2.4.3-1: Common Exception messages for NB1 test requirements

| Default Message Contents |  |
| --- | --- |
| NPDCCH standalone message contents | Table A.10.1.2-1 |
| NPDSCH standalone message contents | Table A.10.2.2-1 |

Table 13.6.2.2.4.3-2: NPUSCH-ConfigDedicated-NB-DEFAULT

| Derivation Path: 36.508 clause Table 8.1.6.3-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPUSCH-ConfigDedicated-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| ack-NACK-NumRepetitions-r13 | r1 | Default |  |
| npusch-AllSymbols-r13 | TRUE | Default |  |
| groupHoppingDisabled-r13 | Not present | Default |  |
| } |  |  |  |

Table 13.6.2.2.4.3-3: SystemInformationBlockType2-NB

| Derivation Path: 36.508 Table 8.1.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2-NB-r13 ::= SEQUENCE { |  |  |  |
| lateNonCriticalExtension SEQUENCE { |  |  | FDD |
| cqi-Reporting-r14 | True |  |  |
| } |  |  |  |
| } |  |  |  |

##### 13.6.2.2.5	Test requirement

In this set of test cases all cells are on the same carrier frequency. The MSG3-based downlink channel quality reporting accuracy is tested by using the parameters in Tables 13.6.2.2.5-1 and 13.6.2.2.5-2. This test is divided into two parts. Firstly, UE should report the correct R value to ensure the pm-dsg under 1%. Secondly, the pm-dsg should larger than 1% when R/4 or R/8 is used.

Table 13.6.2.2.5-1: General Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN HD-FDD Category NB1 UE in Standalone mode under enhanced coverage

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| NB-IoT operational mode |  |  | Standalone |
| CP Length |  |  | Normal |
| DRX |  |  | OFF |
| NPUSCH repetition level |  |  | 1 |
| Satellite information | Config 1 | SSC.1 | GSO |
|  | Config 2 | SSC.2 | NGSO |

Table 13.6.2.2.5-2: nCell specific Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN HD-FDD Category NB1 UE in Standalone mode under enhanced coverage

| Parameter | Unit | Test 1 |
| --- | --- | --- |
| BWchannel | kHz | 200 |
| NPDCCH parameter |  | R.30 HD-FDD |
| NPDCCH repetition level for RAR |  | 16 |
| NPBCH_RB | dB | 0 |
| NPSS_RA | dB |  |
| NSSS_RA | dB |  |
| NPDCCH_RA | dB |  |
| NPDCCH_RB | dB |  |
| NPDSCH_RA | dB |  |
| NPDSCH_RB | dB |  |
| OCNG_RANote1 | dB |  |
| OCNG_RBNote1 | dB |  |
| Note2 | dBm/15 kHz | -98 |
| NRS | dB | -12 |
| Propagation condition |  | AWGN |
| Antenna Configuration |  | 1x1 |
| Scheduling delay in RAR (IDelay) Note3 |  | 0 |
| Channel quality IE Note4 |  | CQI-NPDCCH-NB |
| Note 1: OCNG shall be used such that active cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: See section 16.3.3 in TS 36.213 [23]. Note 4: See TS 36.331 [2]. |  |  |

The pass/fail decision is as specified in the test procedure in clause 13.6.2.2.4.2.

<< several sections skipped >>

#### 13.6.2.4	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under enhanced coverage

##### 13.6.2.4.1	Test purpose

The purpose of this test is to verify that the downlink channel quality reporting accuracy on non-anchor carrier is within the specified limits. This test will verify the requirements in TS36.133[4] Section 13.6.2.4.1 for NB-IoT SAN PCell.

##### 13.6.2.4.2	Test applicability

This test case applies to all types of NB-IoT HD-FDD category NB1 UEs supporting GSO or NGSO or both and DL channel quality reporting for non-anchor carrier for FDD in Msg3 from release 17 and forwards.

##### 13.6.2.4.3	Minimum conformance requirements

The requirements for accuracy of downlink channel quality reporting in this clause apply to the serving cell on the anchor carrier and non-anchor carrier for UE Category NB1.

The accuracy requirements in Table 13.6.2.4.3-1 are valid under the following conditions:

-	Cell specific reference signals are transmitted either from one port.

-	Conditions defined in TS 36.102 Clause 7.3B for reference sensitivity are fulfilled.

-	NRSRP|dBm according to TS36.133 Annex B.3.25A for a corresponding Band.

Table 13.6.2.4.3-1: Downlink channel quality reporting accuracy for UE Category NB1

| NPDCCH Repetition | Pm-Dsg (%) | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Ês/Iot | Io NOTE 2 range |  |  |  |
|  |  |  | E-UTRA/NR operating band groups NOTE 3 | Minimum Io |  | Maximum Io |
|  |  | dB |  | dBm/15kHz | dBm/BWChannel | dBm/BWChannel |
| R NOTE 1 | ≤1 | -6 dB | NFDD_SAB_G | -122.9 | N/A | -70 |
| R/4 NOTE 1 | >1 | -6 dB | NFDD_SAB_G | -122.9 | N/A | -70 |
| R NOTE 1 | ≤1 | -15 ≤ Ês/Iot ≤ -6 dB | NFDD_SAB_G | -122.9 | N/A | -70 |
| R/8 NOTE 1 | >1 | -15 ≤ Ês/Iot ≤ -6 dB | NFDD_SAB_G | -122.9 | N/A | -70 |
| NOTE 1: R is the reported NPDCCH repetition level that UE has reported in CQI-NPDCCH-NB or CQI-NPDCCH-Short-NB. NOTE 2: Io is assumed to have constant EPRE across the bandwidth. NOTE 3: E-UTRA/NR operating band groups are as defined in TS36.133 Section 3.5. |  |  |  |  |  |  |

The normative reference for this requirement is 3GPP TS 36.133 [4] clause A.13.6.2.4.

##### 13.6.2.4.4	Test description

13.6.2.4.4.1	Initial conditions

Test Environment: Normal, as defined in 3GPP TS 36.508 [7] clause 8.1.1.

Frequencies to be tested: According to Annex E table E-4 and 3GPP TS 36.508 [7] clauses 8.1.3 and 8.1.4.2.

Channel Bandwidth to be tested: Ncell bandwidth is as specified in Table 13.6.2.4.5-2.

1.	Connect the SS and AWGN noise source to the UE antenna connectors as shown in 3GPP TS 36.508 [7] Annex A Figure A.18 using only UE main Tx/Rx antenna and without fading.

2.	Propagation conditions are set according to Annex B clause B.0.

3.	There is one NB-IoT cell specified in the test. Ncell 1 is the cell used for registration with the power level set according to Annex C.0 and C.1 for this test. In Ncell 1 one anchor carrier and one non-anchor carrier are configured.

4.	UE location according to TS 36.508 [12] clause 8.4.6.1 is provided to the UE through any preconfigured means.

5.	Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 36.508 [12] clause 8.4.6.2.1 depending on the type of satellite under test. Test system shall send same SIB31-NB information during the duration of the test as defined in TS 36.508 [12] clause 8.4.6.3.1.

6.	Deactivate UE prediction of satellite trajectory through any preconfigured means.

The UE shall be provided with the valid information about the SAN serving cells before the test.

Table 13.6.2.4.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, HD-FDD duplex mode |
| 2 | NGSO, HD-FDD duplex mode |
| Note: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. |  |

13.6.2.4.4.2	Test procedure

The test scenario comprises of 2 NB-IoT carriers with 1 Ncell.

The test procedures are the same as test procedures defined in 13.6.2.2.4.2 but performed on the non-anchor carrier.

13.6.2.4.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 8.1.4, 8.1.5B and 8.1.6 with following exceptions.

Table 13.6.2.4.4.3-1: Common Exception messages for NB1 test requirements

| Default Message Contents |  |
| --- | --- |
| NPDCCH standalone message contents | Table A.10.1.2-1 |
| NPDSCH standalone message contents | Table A.10.2.2-1 |

Table 13.6.2.4.4.3-2: SystemInformationBlockType22-NB

| Derivation Path: 36.508 Table 8.1.4.3.3-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType22-NB-r14 ::= SEQUENCE { |  |  |  |
| ul-ConfigList-r14 SEQUENCE (SIZE (1.. maxNonAnchorCarriers-NB-r14)) OF UL-ConfigCommon-NB-r14 SEQUENCE { |  |  |  |
| UL-ConfigCommon-NB-r14[1] SEQUENCE { |  |  |  |
| nprach-ParametersList-r14 SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF NPRACH-Parameters-NB-r14 SEQUENCE { |  |  |  |
| NPRACH-Parameters-NB-r14[1] SEQUENCE { |  |  |  |
| nprach-Parameters-r14 SEQUENCE { |  |  |  |
| nprach-Periodicity-r14 | ms640 |  |  |
| nprach-StartTime-r14 | ms8 |  |  |
| nprach-SubcarrierOffset-r14 | n0 |  |  |
| nprach-NumSubcarriers-r14 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r14 | zero |  |  |
| npdcch-NumRepetitions-RA-r14 | r8 |  |  |
| npdcch-StartSF-CSS-RA-r14 | v2 |  |  |
| npdcch-Offset-RA-r14 | zero |  |  |
| nprach-NumCBRA-StartSubcarriers-r14 | n8 |  |  |
| npdcch-CarrierIndex-r14 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| NPRACH-Parameters-NB-r14[2] SEQUENCE { |  |  |  |
| nprach-Parameters-r14 SEQUENCE { |  |  |  |
| nprach-Periodicity-r14 | ms640 |  |  |
| nprach-StartTime-r14 | ms64 |  |  |
| nprach-SubcarrierOffset-r14 | n0 |  |  |
| nprach-NumSubcarriers-r14 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r14 | zero |  |  |
| npdcch-NumRepetitions-RA-r14 | r64 |  |  |
| npdcch-StartSF-CSS-RA-r14 | v2 |  |  |
| npdcch-Offset-RA-r14 | zero |  |  |
| nprach-NumCBRA-StartSubcarriers-r14 | n8 |  |  |
| npdcch-CarrierIndex-r14 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| NPRACH-Parameters-NB-r14[3] SEQUENCE { |  |  |  |
| nprach-Parameters-r14 SEQUENCE { |  |  |  |
| nprach-Periodicity-r14 | ms640 |  |  |
| nprach-StartTime-r14 | ms128 |  |  |
| nprach-SubcarrierOffset-r14 | n0 |  |  |
| nprach-NumSubcarriers-r14 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r14 | zero |  |  |
| npdcch-NumRepetitions-RA-r14 | r128 |  |  |
| npdcch-StartSF-CSS-RA-r14 | v2 |  |  |
| npdcch-Offset-RA-r14 | zero |  |  |
| nprach-NumCBRA-StartSubcarriers-r14 | n8 |  |  |
| npdcch-CarrierIndex-r14 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nprach-ProbabilityAnchorList-r14 SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF NPRACH-ProbabilityAnchor-NB-r14 SEQUENCE { |  |  |  |
| NPRACH-ProbabilityAnchor-NB-r14[1] SEQUENCE { |  |  |  |
| nprach-ProbabilityAnchor-r14 | zero |  |  |
| } |  |  |  |
| NPRACH-ProbabilityAnchor-NB-r14[2] SEQUENCE { |  |  |  |
| nprach-ProbabilityAnchor-r14 | zero |  |  |
| } |  |  |  |
| NPRACH-ProbabilityAnchor-NB-r14[3] SEQUENCE { |  |  |  |
| nprach-ProbabilityAnchor-r14 | zero |  |  |
| } |  |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

Table 13.6.2.4.4.3-3: NPUSCH-ConfigDedicated-NB-DEFAULT

| Derivation Path: 36.508 clause Table 8.1.6.3-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPUSCH-ConfigDedicated-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| ack-NACK-NumRepetitions-r13 | r1 | Default |  |
| npusch-AllSymbols-r13 | TRUE | Default |  |
| groupHoppingDisabled-r13 | Not present | Default |  |
| } |  |  |  |

Table 13.6.2.4.4.3-4: SystemInformationBlockType2-NB

| Derivation Path: 36.508 Table 8.1.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2-NB-r13 ::= SEQUENCE { |  |  |  |
| lateNonCriticalExtension SEQUENCE { |  |  | FDD |
| cqi-Reporting-r14 | True |  |  |
| } |  |  |  |
| } |  |  |  |

Table 13.6.2.4.4.3-5: NPRACH-ConfigSIB-NB-DEFAULT

| Derivation Path: 36.508 Table 8.1.6.3-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPRACH-ConfigSIB-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| rsrp-ThresholdsPrachInfoList-r13 ::= SEQUENCE (SIZE(1)) OF RSRP-Range { | 1 entry |  |  |
| RSRP-Range[1] | 18 | -122dBm | Normal |
| RSRP-Range[2] | 41 | -99dBm |  |
| } |  |  |  |
| nprach-ParametersList-r13 SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF NPRACH-Parameters-NB-r13 { | 3 entries |  |  |
| NPRACH-Parameters-NB-r13[1] ::= SEQUENCE { |  |  |  |
| nprach-Periodicity-r13 | ms640 |  |  |
| nprach-StartTime-r13 | ms8 |  |  |
| nprach-SubcarrierOffset-r13 | n0 |  |  |
| nprach-NumSubcarriers-r13 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r13 | zero |  |  |
| maxNumPreambleAttemptCE-r13 | n3 |  |  |
| numRepetitionsPerPreambleAttempt-r13 | n2 |  |  |
| npdcch-NumRepetitions-RA-r13 | r8 |  |  |
| npdcch-StartSF-CSS-RA-r13 | v2 |  |  |
| npdcch-Offset-RA-r13 | zero |  |  |
| } |  |  |  |
| NPRACH-Parameters-NB-r13[2] ::= SEQUENCE { |  |  |  |
| nprach-Periodicity-r13 | ms640 |  |  |
| nprach-StartTime-r13 | ms64 |  |  |
| nprach-SubcarrierOffset-r13 | n0 |  |  |
| nprach-NumSubcarriers-r13 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r13 | zero |  |  |
| maxNumPreambleAttemptCE-r13 | n6 |  |  |
| numRepetitionsPerPreambleAttempt-r13 | n8 |  |  |
| npdcch-NumRepetitions-RA-r13 | r64 |  |  |
| npdcch-StartSF-CSS-RA-r13 | v2 |  |  |
| npdcch-Offset-RA-r13 | zero |  |  |
| } |  |  |  |
| NPRACH-Parameters-NB-r13[3] ::= SEQUENCE { |  |  |  |
| nprach-Periodicity-r13 | ms640 |  |  |
| nprach-StartTime-r13 | ms128 |  |  |
| nprach-SubcarrierOffset-r13 | n0 |  |  |
| nprach-NumSubcarriers-r13 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r13 | zero |  |  |
| maxNumPreambleAttemptCE-r13 | n10 |  |  |
| numRepetitionsPerPreambleAttempt-r13 | n64 |  |  |
| npdcch-NumRepetitions-RA-r13 | r128 |  |  |
| npdcch-StartSF-CSS-RA-r13 | v2 |  |  |
| npdcch-Offset-RA-r13 | zero |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 13.6.2.4.4.3-6: RACH-ConfigCommon-NB-DEFAULT

| Derivation Path: 36.508 Table 8.1.6.3-8: RACH-ConfigCommon-NB-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| preambleTransMax-CE-r13 | n6 |  |  |
| powerRampingParameters-r13 SEQUENCE { |  |  |  |
| powerRampingStep | dB2 |  |  |
| preambleInitialReceivedTargetPower | dBm-104 |  |  |
| } |  |  |  |
| rach-InfoList-r13 (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF SEQUENCE { | 3 entry |  |  |
| { |  |  |  |
| ra-ResponseWindowSize-r13 | pp10 |  |  |
| mac-ContentionResolutionTimer-r13 | pp8 |  |  |
| } |  |  |  |
| { |  |  |  |
| ra-ResponseWindowSize-r13 | pp10 |  |  |
| mac-ContentionResolutionTimer-r13 | pp8 |  |  |
| } |  |  |  |
| { |  |  |  |
| ra-ResponseWindowSize-r13 | pp10 |  |  |
| mac-ContentionResolutionTimer-r13 | pp8 |  |  |
| } |  |  |  |
| connEstFailOffset-r13 | Not present |  |  |
| } |  |  |  |

Table 13.6.2.4.4.3-7: NPUSCH-ConfigCommon-NB-DEFAULT

| Derivation Path: 36.508 Table 8.1.6.3-6: NPUSCH-ConfigCommon-NB-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPUSCH-ConfigCommon-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| ack-NACK-NumRepetitions-Msg4-r13 SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF { |  | 3 entry |  |
| {ACK-NACK-NumRepetitions-NB-r13 } | r8 |  |  |
| {ACK-NACK-NumRepetitions-NB-r13 } | r8 |  |  |
| {ACK-NACK-NumRepetitions-NB-r13 } | r8 |  |  |
| } |  |  |  |
| srs-SubframeConfig-r13 | Not present |  |  |
| dmrs-Config-r13 SEQUENCE { |  |  |  |
| threeTone-BaseSequence-r13 | Not present |  |  |
| threeTone-CyclicShift-r13 | 0 |  |  |
| sixTone-BaseSequence-r13 | Not present |  |  |
| sixTone-CyclicShift-r13 | 0 |  |  |
| twelveTone-BaseSequence-r13 | Not present |  |  |
| } |  |  |  |
| ul-ReferenceSignalsNPUSCH-r13 SEQUENCE { |  |  |  |
| groupHoppingEnabled-r13 | FALSE |  |  |
| groupAssignmentPUSCH-13 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

##### 13.6.2.4.5	Test requirement

In this set of test cases all cells are on the same carrier frequency. The MSG3-based downlink channel quality reporting accuracy on non-anchor carrier is tested by using the parameters in Tables 13.6.2.4.5-1 and 13.6.2.4.5-2. This test is divided into two parts. Firstly, UE should report the correct R value to ensure the pm-dsg under 1%. Secondly, the pm-dsg should larger than 1% when R/4 or R/8 is used.

Table 13.6.2.4.5-1: General Test Parameters for Downlink channel quality reporting accuracy test on non-anchor carrier for E-UTRAN HD-FDD Category NB1 UE in Standalone mode under enhanced coverage

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| NB-IoT operational mode |  |  | Standalone |
| CP Length |  |  | Normal |
| DRX |  |  | OFF |
| NPUSCH repetition level |  |  | 1 |
| NPDCCH carrier index (npdcch-CarrierIndex-r14) |  |  | 1 (Note 1) |
| Satellite information | Config 1 | SSC.1 | GSO |
|  | Config 2 | SSC.2 | NGSO |
| Note 1: Index of non-anchor carrier configured in SystemInformationBlockType22-NB (See TS 36.331 [2]). |  |  |  |

Table 13.6.2.4.5-2: nCell specific Test Parameters for Downlink channel quality reporting accuracy test on non-anchor carrier for E-UTRAN HD-FDD Category NB1 UE in Standalone mode under enhanced coverage

| Parameter | Unit | Test 1 |
| --- | --- | --- |
| BWchannel | kHz | 200 |
| NPDCCH parameter |  | R.30 HD-FDD |
| NPDCCH repetition level for RAR on non-anchor |  | 16 |
| NPBCH_RB | dB | 0 |
| NPSS_RA | dB |  |
| NSSS_RA | dB |  |
| NPDCCH_RA | dB |  |
| NPDCCH_RB | dB |  |
| NPDSCH_RA | dB |  |
| NPDSCH_RB | dB |  |
| OCNG_RANote1 | dB |  |
| OCNG_RBNote1 | dB |  |
| Note2 | dBm/15 kHz | -98 |
| NRS | dB | -12 |
| Propagation condition |  | AWGN |
| Antenna Configuration |  | 1x1 |
| Scheduling delay in RAR (IDelay) Note3 |  | 0 |
| Channel quality IE Note4 |  | CQI-NPDCCH-NB |
| Note 1: OCNG shall be used such that active cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: See section 16.3.3 in TS 36.213 [23]. Note 4: See TS 36.331 [2]. |  |  |

The pass/fail decision is as specified in the test procedure in clause 13.6.2.4.4.2.

#### 13.6.2.5	E-UTRAN HD-FDD Downlink channel quality reporting accuracy in RRC_CONNECTED for UE Category NB1 Standalone mode under normal coverage

##### 13.6.2.5.1	Test purpose

The purpose of this test is to verify that the downlink channel quality reporting accuracy in connected mode is within the specified limits. This test will verify the requirements in TS36.133[4] Section 9.1.22A.8 for NB-IoT SAN PCell.

##### 13.6.2.5.2	Test applicability

This test case applies to all types of NB-IoT HD-FDD category NB1 UEs supporting GSO or NGSO or both and DL channel quality reporting in RRC_CONNECTED from release 17 and forwards.

##### 13.6.2.5.3	Minimum conformance requirements

The requirements for accuracy of downlink channel quality reporting in this clause apply to the serving cell on the anchor carrier and non-anchor carrier for UE Category NB1.

The accuracy requirements in Table 13.6.2.5.3-1 are valid under the following conditions:

-	Cell specific reference signals are transmitted either from one port.

-	Conditions defined in TS 36.102 Clause 7.3B for reference sensitivity are fulfilled.

-	NRSRP|dBm according to TS36.133 Annex B.3.25A for a corresponding Band.

Table 13.6.2.5.3-1: Downlink channel quality reporting accuracy for UE Category NB1

| NPDCCH Repetition | Pm-Dsg (%) | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Ês/Iot | Io NOTE 2 range |  |  |  |
|  |  |  | E-UTRA/NR operating band groups NOTE 3 | Minimum Io |  | Maximum Io |
|  |  | dB |  | dBm/15kHz | dBm/BWChannel | dBm/BWChannel |
| R NOTE 1 | ≤1 | -6 dB | NFDD_SAB_G | -122.9 | N/A | -70 |
| R/4 NOTE 1 | >1 | -6 dB | NFDD_SAB_G | -122.9 | N/A | -70 |
| R NOTE 1 | ≤1 | -15 ≤ Ês/Iot ≤ -6 dB | NFDD_SAB_G | -122.9 | N/A | -70 |
| R/8 NOTE 1 | >1 | -15 ≤ Ês/Iot ≤ -6 dB | NFDD_SAB_G | -122.9 | N/A | -70 |
| NOTE 1: R is the reported NPDCCH repetition level that UE has reported in CQI-NPDCCH-NB or CQI-NPDCCH-Short-NB. NOTE 2: Io is assumed to have constant EPRE across the bandwidth. NOTE 3: E-UTRA/NR operating band groups are as defined in TS36.133 Section 3.5. |  |  |  |  |  |  |

The normative reference for this requirement is 3GPP TS 36.133 [4] clause A.13.6.2.5.

##### 13.6.2.5.4	Test description

13.6.2.5.4.1	Initial conditions

Test Environment: Normal, as defined in 3GPP TS 36.508 [7] clause 8.1.1.

Frequencies to be tested: According to Annex E table E-4 and 3GPP TS 36.508 [7] clauses 8.1.3 and 8.1.4.2.

Channel Bandwidth to be tested: Ncell bandwidth is as specified in Table 13.6.2.5.5-2.

1.	Connect the SS and AWGN noise source to the UE antenna connectors as shown in 3GPP TS 36.508 [7] Annex A Figure A.18 using only UE main Tx/Rx antenna.

2.	Propagation conditions are set according to Annex B clause B.0.

3.	There is one NB-IoT cell specified in the test. Ncell 1 is the cell used for registration with the power level set according to Annex C.0 and C.1 for this test.

4.	UE location according to TS 36.508 [12] clause 8.4.6.1 is provided to the UE through any preconfigured means.

5.	Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 36.508 [12] clause 8.4.6.2.1 depending on the type of satellite under test. Test system shall send same SIB31-NB information during the duration of the test as defined in TS 36.508 [12] clause 8.4.6.3.1.

6.	Deactivate UE prediction of satellite trajectory through any preconfigured means.

The UE shall be provided with the valid information about the SAN serving cells before the test.

Table 13.6.2.5.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, HD-FDD duplex mode |
| 2 | NGSO, HD-FDD duplex mode |
| Note: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. |  |

13.6.2.5.4.2	Test procedure

The test scenario comprises of one NB-IoT carrier with 1 Ncell.

1.	Set the parameters according to Tables 13.6.2.5.5-1 and 13.6.2.5.5-2 T1. Propagation conditions are set according to Annex B clause B.1.1.

2.	 Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [7] clause 8.1.5 in Ncell 1. (The UE shall transmit msg3 with the NPDCCH repetition number R report according to TS36.133 Table 9.1.22.15-1 after successful reception of the RAR.)

3.	The SS configures Rmax=R based on the UE report in step 2 ensuring the UE is in State 2A-NB with new Rmax value. If the UE report in step 2 is <4, the SS configures Rmax=4.

4.	The SS triggers UE’s channel quality report measurement by the MAC-CE command as specified in TS36.321 clause 5.25.

5.	Set the parameters according to Tables 13.6.2.5.5-1 and 13.6.2.5.5-2 T2. The SS start to transmit NPDCCH DCI format N0(NPDCCH repetition number=4) as specified in TS36.133 clause 8.14.4.

6.	The reported NPDCCH repetition level shall be derived from the channel quality measured over the NPDCCH period which carries the uplink grant of channel quality report for measurement of DL channel quality of the configured carrier. The UE report the NPDCCH repetition number R by NPUSCH.

7.	The SS transmits NPDSCH via NPDCCH DCI format N1 according to the UE reported NPDCCH repetition number R. The SS sends downlink MAC padding bits on the DL RMC.

8.	Measure the Pm-dsg for a duration of 100 NPDCCH. SS counts the number of NACKs, ACKs and statDTXs on the UL NPUSCH. Pm-dsg is the ratio according to the formula (statDTX)/(NACK+ACK+statDTX). If Pm-dsg is ≤ 1%, continue with step 9. If R/4 is unconfigurable according to TS 36.213 [8] clause 16.6, skip following test steps and pass the UE. Otherwise fail the UE.

9.	The SS transmits NPDSCH via NPDCCH DCI format N1 according to NPDCCH repetition number R/4. The SS sends downlink MAC padding bits on the DL RMC.

10.	Measure the Pm-dsg for a duration of 100 NPDCCH.SS counts the number of NACKs, ACKs and statDTXs on the UL NPUSCH. Pm-dsg is the ratio according to the formula (statDTX)/(NACK+ACK+statDTX). If Pm-dsg is > 1%, pass the UE. Otherwise fail the UE.

13.6.2.5.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 8.1.5B and 8.1.6 with following exceptions.

Table 13.6.2.5.4.3-1: Common Exception messages for NB1 test requirements

| Default Message Contents |  |
| --- | --- |
| NPDCCH standalone message contents | Table A.10.1.2-1 |
| NPDSCH standalone message contents | Table A.10.2.2-1 |

Table 13.6.2.5.4.3-2: NPUSCH-ConfigDedicated-NB-DEFAULT

| Derivation Path: 36.508 clause Table 8.1.6.3-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPUSCH-ConfigDedicated-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| ack-NACK-NumRepetitions-r13 | r1 | Default |  |
| npusch-AllSymbols-r13 | TRUE | Default |  |
| groupHoppingDisabled-r13 | Not present | Default |  |
| } |  |  |  |

##### 13.6.2.5.5	Test requirement

In this set of test cases all cells are on the same carrier frequency. The tests consist of two successive time periods of length T1 and T2, respectively, at different SNR levels. The start of T2 coincides with the start of the channel quality measurement period specified in section TS36.133 8.14.4. The MAC CE-based downlink channel quality reporting accuracy is tested by using the parameters in Tables 13.6.2.5.5-1 and 13.6.2.5.5-2. This test is divided into two parts. Firstly, UE should report the correct R value to ensure the pm-dsg under 1%. Secondly, the pm-dsg should larger than 1% when R/4 or R/8 is used.

Table 13.6.2.5.5-1: General Test Parameters for Downlink channel quality reporting accuracy test in RRC_CONNECTED for E-UTRAN HD-FDD Category NB1 UE in Standalone mode under normal coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NB-IoT operational mode |  |  | Standalone |  |
| CP Length |  |  | Normal |  |
| DRX |  |  | OFF |  |
| NPUSCH repetition level |  |  | 1 |  |
| T1 |  | s | 15 | Initialization period |
| T2 Note 1 |  | s | - | Evaluation period |
| Satellite information | Config 1 | SSC.1 | GSO |  |
|  | Config 2 | SSC.2 | NGSO |  |
| Note 1: This time period starts at the beginning of the NPDCCH period that carries the uplink grant for the channel quality report (section 8.14.4 in TS36.133). |  |  |  |  |

Table 13.6.2.5.5-2: nCell specific Test Parameters for Downlink channel quality reporting accuracy test in RRC_CONNECTED for E-UTRAN HD-FDD Category NB1 UE in Standalone mode under normal coverage

| Parameter | Unit | Test 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| BWchannel | kHz | 200 |  |
| NPDCCH parameter |  | R.30 HD-FDD |  |
| NPDCCH repetition Note4 |  | 4 |  |
| NPBCH_RB | dB | 0 |  |
| NPSS_RA | dB |  |  |
| NSSS_RA | dB |  |  |
| NPDCCH_RA | dB |  |  |
| NPDCCH_RB | dB |  |  |
| NPDSCH_RA | dB |  |  |
| NPDSCH_RB | dB |  |  |
| OCNG_RANote1 | dB |  |  |
| OCNG_RBNote1 | dB |  |  |
| Note2 | dBm/15 kHz | -98 |  |
| NRS | dB | -5.4 | -5.4 |
| Propagation condition |  | AWGN |  |
| Antenna Configuration |  | 1x1 |  |
| Channel quality IE Note3 |  | CQI-NPDCCH-NB |  |
| Note 1: OCNG shall be used such that active cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: See TS 36.331 [2]. Note 4: The NPDCCH repetition level shall be adjusted during T2 based on the DL channel quality report so that the requirements in TS36.133 Table 9.1.22A.8-1 can be verified. |  |  |  |

The pass/fail decision is as specified in the test procedure in clause 13.6.2.5.4.2.

#### 13.6.2.6	E-UTRAN HD-FDD Downlink channel quality reporting accuracy in RRC_CONNECTED for UE Category NB1 Standalone mode under enhanced coverage

##### 13.6.2.6.1	Test purpose

The purpose of this test is to verify that the downlink channel quality reporting accuracy in connected mode is within the specified limits. This test will verify the requirements in TS36.133[4] Section 9.1.22A.8 for NB-IoT SAN PCell.

##### 13.6.2.6.2	Test applicability

This test case applies to all types of NB-IoT HD-FDD category NB1 UEs supporting GSO or NGSO or both and DL channel quality reporting in RRC_CONNECTED from release 17 and forwards.

##### 13.6.2.6.3	Minimum conformance requirements

The requirements for accuracy of downlink channel quality reporting in this clause apply to the serving cell on the anchor carrier and non-anchor carrier for UE Category NB1.

The accuracy requirements in Table 13.6.2.6.3-1 are valid under the following conditions:

-	Cell specific reference signals are transmitted either from one port.

-	Conditions defined in TS 36.102 Clause 7.3B for reference sensitivity are fulfilled.

-	NRSRP|dBm according to TS36.133 Annex B.3.25A for a corresponding Band.

Table 13.6.2.6.3-1: Downlink channel quality reporting accuracy for UE Category NB1

| NPDCCH Repetition | Pm-Dsg (%) | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Ês/Iot | Io NOTE 2 range |  |  |  |
|  |  |  | E-UTRA/NR operating band groups NOTE 3 | Minimum Io |  | Maximum Io |
|  |  | dB |  | dBm/15kHz | dBm/BWChannel | dBm/BWChannel |
| R NOTE 1 | ≤1 | -6 dB | NFDD_SAB_G | -122.9 | N/A | -70 |
| R/4 NOTE 1 | >1 | -6 dB | NFDD_SAB_G | -122.9 | N/A | -70 |
| R NOTE 1 | ≤1 | -15 ≤ Ês/Iot ≤ -6 dB | NFDD_SAB_G | -122.9 | N/A | -70 |
| R/8 NOTE 1 | >1 | -15 ≤ Ês/Iot ≤ -6 dB | NFDD_SAB_G | -122.9 | N/A | -70 |
| NOTE 1: R is the reported NPDCCH repetition level that UE has reported in CQI-NPDCCH-NB or CQI-NPDCCH-Short-NB. NOTE 2: Io is assumed to have constant EPRE across the bandwidth. NOTE 3: E-UTRA/NR operating band groups are as defined in TS36.133 Section 3.5. |  |  |  |  |  |  |

The normative reference for this requirement is 3GPP TS 36.133 [4] clause A.13.6.2.6.

##### 13.6.2.6.4	Test description

13.6.2.6.4.1	Initial conditions

Test Environment: Normal, as defined in 3GPP TS 36.508 [7] clause 8.1.1.

Frequencies to be tested: According to Annex E table E-4 and 3GPP TS 36.508 [7] clauses 8.1.3 and 8.1.4.2.

Channel Bandwidth to be tested: Ncell bandwidth is as specified in Table 13.6.2.6.5-2.

1.	Connect the SS and AWGN noise source to the UE antenna connectors as shown in 3GPP TS 36.508 [7] Annex A Figure A.18 using only UE main Tx/Rx antenna.

2.	Propagation conditions are set according to Annex B clause B.0.

3.	There is one NB-IoT cell specified in the test. Ncell 1 is the cell used for registration with the power level set according to Annex C.0 and C.1 for this test.

4.	UE location according to TS 36.508 [12] clause 8.4.6.1 is provided to the UE through any preconfigured means.

5.	Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 36.508 [12] clause 8.4.6.2.1 depending on the type of satellite under test. Test system shall send same SIB31-NB information during the duration of the test as defined in TS 36.508 [12] clause 8.4.6.3.1.

6.	Deactivate UE prediction of satellite trajectory through any preconfigured means.

The UE shall be provided with the valid information about the SAN serving cells before the test.

Table 13.6.2.6.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, HD-FDD duplex mode |
| 2 | NGSO, HD-FDD duplex mode |
| Note: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. |  |

13.6.2.6.4.2	Test procedure

The test scenario comprises of one NB-IoT carrier with 1 Ncell.

The test procedures are the same as test procedures defined in 13.6.2.5.4.2 with the following exceptions:

-	in step 3, if the UE report in step 2 is <16, the SS configures Rmax=16.

-	in step 5 NPDCCH repetition number=16.

-	in step 5 NPDCCH repetition number R/8.

13.6.2.6.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 8.1.5B and 8.1.6 with following exceptions.

Table 13.6.2.6.4.3-1: Common Exception messages for NB1 test requirements

| Default Message Contents |  |
| --- | --- |
| NPDCCH standalone message contents | Table A.10.1.2-1 |
| NPDSCH standalone message contents | Table A.10.2.2-1 |

Table 13.6.2.6.4.3-2: NPUSCH-ConfigDedicated-NB-DEFAULT

| Derivation Path: 36.508 clause Table 8.1.6.3-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPUSCH-ConfigDedicated-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| ack-NACK-NumRepetitions-r13 | r1 | Default |  |
| npusch-AllSymbols-r13 | TRUE | Default |  |
| groupHoppingDisabled-r13 | Not present | Default |  |
| } |  |  |  |

##### 13.6.2.6.5	Test requirement

In this set of test cases all cells are on the same carrier frequency. The tests consist of two successive time periods of length T1 and T2, respectively, at different SNR levels. The start of T2 coincides with the start of the channel quality measurement period specified in section TS36.133 8.14.4. The MAC CE-based downlink channel quality reporting accuracy is tested by using the parameters in Tables 13.6.2.6.5-1 and 13.6.2.6.5-2. This test is divided into two parts. Firstly, UE should report the correct R value to ensure the pm-dsg under 1%. Secondly, the pm-dsg should larger than 1% when R/4 or R/8 is used.

Table 13.6.2.6.5-1: General Test Parameters for Downlink channel quality reporting accuracy test in RRC_CONNECTED for E-UTRAN HD-FDD Category NB1 UE in Standalone mode under enhanced coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NB-IoT operational mode |  |  | Standalone |  |
| CP Length |  |  | Normal |  |
| DRX |  |  | OFF |  |
| NPUSCH repetition level |  |  | 60 |  |
| T1 |  | s | 1 | Initialization period |
| T2 Note 1 |  | s | - | Evaluation period |
| Satellite information | Config 1 | SSC.1 | GSO |  |
|  | Config 2 | SSC.2 | NGSO |  |
| Note 1: This time period starts at the beginning of the NPDCCH period that carries the uplink grant for the channel quality report (section 8.14.4). |  |  |  |  |

Table 13.6.2.6.5-2: nCell specific Test Parameters for Downlink channel quality reporting accuracy test in RRC_CONNECTED for E-UTRAN HD-FDD Category NB1 UE in Standalone mode under enhanced coverage

| Parameter | Unit | Test 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| BWchannel | kHz | 200 |  |
| NPDCCH parameter |  | R.30 HD-FDD |  |
| NPDCCH repetition Note4 |  | 16 |  |
| NPBCH_RB | dB | 0 |  |
| NPSS_RA | dB |  |  |
| NSSS_RA | dB |  |  |
| NPDCCH_RA | dB |  |  |
| NPDCCH_RB | dB |  |  |
| NPDSCH_RA | dB |  |  |
| NPDSCH_RB | dB |  |  |
| OCNG_RANote1 | dB |  |  |
| OCNG_RBNote1 | dB |  |  |
| Note2 | dBm/15 kHz | -98 |  |
| NRS | dB | -12 | -12 |
| Propagation condition |  | AWGN |  |
| Antenna Configuration |  | 1x1 |  |
| Channel quality IE Note3 |  | CQI-NPDCCH-NB |  |
| Note 1: OCNG shall be used such that active cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: See TS 36.331 [2]. Note 4: The NPDCCH repetition level shall be adjusted during T2 based on the DL channel quality report so that the requirements in Table 9.1.22A.8-1 can be verified. |  |  |  |

The pass/fail decision is as specified in the test procedure in clause 13.6.2.5.4.2.

<< several sections skipped >>