3GPP TSG RAN meeting #107										RP-250751

Incheon, Korea, March 12-14th, 2025

## Status Report to TSG

Agenda item:			9.4.1.5

| WI Name | Introduction of new NR NTN bands to support the Extended L-band (UL 1668-1675MHz, DL 1518-1525MHz) and the combined MSS L-band and Extended L-band ranges (DL 1518-1559 MHz, UL 1626.5-1660.5 MHz and 1668-1675 MHz) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_NTN_combinedLband |  |  |  |  |
| Unique ID | 1040134 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-241856 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 03/2025 06/2025 | Performance part: 06/2025 | Testing part: N/A |  |
| Overall Completion level | Study Item: N/A | Core part: 70% | Performance Part: 0% | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur | Name | Luca Lodigiani |
|  | Company | Aalyria |
|  | Email | luca@aalyria.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | Yes |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

Additional explanations/motivations for the time budget changes in the attached Excel table:

Some aspects related to treatment of regulatory radiated emission requirements and protection of radio astronomy required extensive discussions and slowed down the progress of the Core part.  In addition, TUs during online meetings were eroded by discussion in other work items.  There are some remaining open items on UE RF requirements that need to be addressed and formal CRs have not yet been agreed.

This WI requires a 1 Quarter extension of the Core Part to June 2025 (RAN#108). The Performance part can remain unchanged (minimal impact).

Revised TU budget is attached

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.4	RAN4

### 2.4.2	Agreements after RAN4#114

The discussion in RAN4#114 has mostly focused on how to capture radiated regulatory requirements, ETSI requirements and on the requirements and mechanisms for protection of Radio Astronomy service.

#### Issue 3-1-1: Capturing of ETSI EN Requirements

- Agreement:

  - Capture existing in-band and out-of-band emission requirements for L-bands n251, n250 and n253 as defined by ETSI EN 301 681 in a new table under a new sub-clause under Clause 6.5.2 (Conducted transmitter characteristics, Out-of-Band emissions) as Additional Regional Emission Requirements.

    - Introduce a NOTE specifying that the requirements are converted from radiated requirements (i.e. EIRP) in ETSI EN 301 681 to conducted requirements, assuming a 0dBi antenna with flat frequency response integral to the UE.

      - Detailed reference with specific version of EN 301 681 spec being referenced shall be included.

    - FFS for UE with high gain antenna or UE with non-integral antenna.

    - Come back in next meeting with specific NS values to be specified

#### Issue 3-1-2: Mechanisms for Protection of Radioastronomy

- Agreement:

  - For n251, n250, n253 protection of Radio Astronomy, consider the following two mechanisms as a baseline:

    - Network-controlled RB scheduling is considered as a baseline based on operator’s control

    - FFS whether a new NS value should be specified to signal the additional emission limit at the edge of the band, to signal cells in proximity or overlap to RA stations

      - Clarify the specific emission limits required and the frequency range applicability, and whether a specific value of geographical distance for the limit is considered, when defining proximity to a RA station

#### Sub-topic 3-2: UE RF

Sub-topic description: System parameters and UE RF aspects

#### Issue 3-2-1: UE Out-of-band blocking

  - Agreement:

    - Consider the following out-of-band blocking requirements for band n253, n251 and n250 as a starting point for further verification (come back in next meeting):

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| n251, n250 | Finterferer (CW) | MHz | [-60 < f – FDL_low < -15 or 15 < f – FDL_high < 60] | [-85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85] | [1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750] |
| n253 | Finterferer (CW) | MHz | [-60 < f – FDL_low < -15 or 15 < f – FDL_high < 94] | [-85 < f – FDL_low ≤ -60 or 94 ≤ f – FDL_high < 119] | [1 ≤ f ≤ FDL_low – 85 or FDL_high + 119 ≤ f ≤ 12750] |

### 2.4.3	Remaining Open issues after RAN4#114

UL Channel Bandwidths and final channel bandwidth table are still missing, alongside a number of other details on system parameters (e.g. actual TX-RX frequency separation range, SAN SS Raster)

Companies are expected to further study how to capture UE radiated regulatory requirements and further finalize how to specify requirements and mechanisms for protection of Radio Astronomy services.

Some UE RF requirements are still pending

Normative CRs are expected in the upcoming meetings

Performance has not started yet.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

- RP-241856 - Revised WID on New NR NTN bands to support Extended L-band and combined MSS L-band and Extended L-band ranges – Inmarsat, Viasat

- R4-2500292	Emission requirements for the UE operating in 1626.5-1660.5MHz and 1668-1675MHz UL frequency ranges	Apple

- R4-2501180	Draft CR to TS 38.133 Introduction of a new NTN L-band	ZTECorporation,Sanechips

- R4-2501309	Discussion on UE RF requirements for NR-NTN bands to support the Extended L-band and the combined MSS L-band and Extended L-band ranges	MediaTek (Hefei) Inc.

- R4-2501454	Discussion on the full NTN L band definition	Huawei, HiSilicon

- R4-2502011	On protection of radioastronomy and radiated regulatory requirements	Qualcomm Inc.

- R4-2500666	Topic summary for [114][114] NR_IoT_NTN_Bands	Moderator(Aalyria)

- R4-2502850	WF on NR and IoT NTN bands	Aalyria