3GPP TSG RAN #107			RP-250811

Incheon, Korea, March 12-14, 2025

Title:	Moderator's summary for RAN4 led REL-20 topics

Source:	RAN4 Chair (Huawei)

Introduction

This document summarizes the discussion of all the companies’ proposals for RAN4 Rel-20 5G-Adavanced. All the topics are categorized into 7 groups by basically following the Rel-19 approach,

- UE RF centric evolution

- BS RF centric evolution

- UE OTA enhancement

- RRM enhancement

- Demodulation performance evolution

- NTN enhancements

- Cross RF and baseband topics

Offline discussions will attempt to collect companies’ views on how to build Rel-20 RAN4 package for 5G-Advanced. All the related contributions are provided in Annex.

UE RF centric evolution

Summary of contributions

| NO. | Proposed objectives for Rel-20 and supporting companies | High level objectives |
| --- | --- | --- |
| 1 | FR1 HPUE enhancement Nokia (0085), OPPO (0165), LGE (0240), Huawei (0284), Apple (0321), Qualcomm (0458), Ericsson (0540), T-Mobile (0638) | Proposed objectives: - High power PC1.5/PC1 UE for TN single carrier in NR FR1 - PC1 with 4Tx UL for FWA. - PC1 with 2Tx and UL-MIMO - PC1.5 FDD with 2Tx - PC1.5 with 1Tx - PC1.5 TDD and FDD with 3Tx - High power [PC1]/PC1.5/PC2 UE for TN single carrier in EN-DC and NR CA - PC2/PC1.5 EN-DC with PC2 LTE band with 2Tx or 3Tx - PC1.5 with 4Tx or 8Tx for UL intra-band and inter-band CA - PC1.5 with DL CA - 3UL CC in two bands for CA/EN-DC with PC2 / PC1.5 (2Tx) with 2CC in NR - PC1.5(2Tx) FR1 PDE for intra-band CA NOTE: PDE power domain enhancement T-mobile proposal: - Continue with PC2 and PC1.5 band combinations - PC1.5 evolution - with 2Tx (TDD+TDD, TDD+FDD, FDD+FDD) - with 3Tx including uplink inter-band + intra-band CA (like CA_n25A-n41C) - with 4Tx (TDD+TDD, TDD+FDD, FDD+FDD) - FDD single band PC1.5 - PC1 with downlink Carrier Aggregation - PC1 with inter-band and intra-band UL CA (4Tx and 8Tx enhancements) - 4x26 dBm - 8x23 dBm - 2x29 dBm (New request for Rel-20) - Combination of 23 and 26 (and 29) dB |
| 2 | Power enhancement with relaxed UE Tx EVM and advanced BS receiver Samsung (0104), vivo (0209), Xiaomi (0353), China Telecom (0387), Huawei (0284) | Proposed objectives: - Reduce UE MPR with relaxed Tx EVM requirements and advanced BS receiver for 64QAM and 256QAM - Reduce UE MPR with relaxed Tx EVM requirements for QPSK and 16QAM |
| 3 | MSD optimization vivo (0209), Huawei (0284), Qualcomm (0458), ZTE (0566), T-Mobile (0638) | Proposed objectives: - Simplify MSD requirements for PC3 - Simplify MSD requirements for HPUE (Rel-19 leftover if any) - (T-Mobile) dynamic real time feedback for MSD issues, rather than a static UE capability |
| 4 | UL Tx switching enhancement OPPO (0165), Apple (0321), ZTE (0566), T-mobile (0638) | Proposed objectives: - Introduce non-zero switching period for 1Tx UE for SUL+NUL band combination - Uplink Tx switching for intra-band non-contiguous UL CA - 3Tx 3CC or 2CC Tx Switching T+F today 2Tx |
| 5 | Tx requirements enhancement for U6G Huawei (0284), China Telecom (0387), CATT (0518) | Proposed objectives: - Specify ACLR of 26dB and corresponding MPR requirements for U6GHz - Specify RF requirements for 200MHz CBW for U6GHz - (CATT) Support higher-order bandwidth class for intra-band uplink contiguous CA, such as introducing bandwidth class D. |
| 6 | Band/band combination requirements simplification OPPO (0165), Huawei (0284), Nokia (0085) | Proposed objectives: - Simplify the band/band combination specifications - Replace the current MS Word-based table listing for band combination specifications by using database. |
| 7 | Power classes framework Qualcomm (0458) | Proposed objectives: - High power class framework - Power class handling for CA (allowing maximum output power per band for different CA combos) |
| 8 | Optimize MPR/A-MPR Ericsson (0540) | Proposed objectives: - Overhaul UE TX model in order to selectively improve MPR/A-MPR in 5G-A and be prepared for 6G - Considering PC3, PC2, PC1 and multiple TX/MIMO |
| 9 | SRS enhancement Ericsson (0540) | Proposed objectives: - Further improvement to SRS performance for reciprocal DL MIMO including improved testing of SRS and IL reproting |
| 10 | SL-U enhancement LGE (0240) | Proposed objectives: - PC3 in single carrier and PC5 in intra-band contiguous CA for SL-U |
| 11 | MPR reduction by leveraging asymmetric ACLR Huawei (0284) | Proposed objectives: - Specify the UE RF requirements of MPR reduction by defining different regions corresponding to asymmetric ACLR |
| 12 | Basket WIs China Telecom (0387) | Proposed objectives: - HPUE basket WIs - Downlink interruption basket WI |
| 13 | Coherent UL MIMO ZTE (0566) | Proposed objectives: - Study how to make the UL-MIMO to be commercially available e.g., introducing certain mechanism by leveraging the current DPD feedback link to do the calibration between RF chains. |
| 14 | 256QAM evolution T-Mobile (0638) | Proposed objectives: - 8 layer DL 256QAM including performance requirements - 4-layer UL 256QAM |

Moderator’s observations

According to moderator’s observations, the following evolution/enhancement directions attract the relatively more common interests (≥4 sources):

- FR1 HPUE enhancement (PC2/PC1.5/PC1) for single carrier and EN-DC/NR-CA

- Power enhancement with relaxed UE Tx EVM and advanced BS receiver

- MSD optimization

- UL Tx switching enhancement

The other evolution/enhancement directions are also captured in this summary but seem get less supports and may need more discussions. Moderators would like to remind companies that the available TU for Rel-20 5G-Advanced would be relatively limited compared with the proposals, and would like to encourage companies to converge on a limited number of mainstream proposals, which would be more critical and will be commercialized.

Offline discussions and conclusions

From moderator’s point of view the following list of UE RF centric evolution and/or enhancement can be used as the starting point for further discussions to converge on Rel-20 5G-Advanced RAN4 package. It does not mean that the technique directions under this list must be part of a certain WI (e.g., UE RF enhancement in Rel-20), in other words, how to organize the final RAN4 project package will be further discussed separately.

The following list of high level candidate objectives can be used as the starting point for 5G-A UE RF centric evolution and/or enhancement in Rel-20:

- FR1 HPUE enhancement (PC2/PC1.5/PC1) for single carrier and EN-DC/NR-CA

- Power enhancement with relaxed UE Tx EVM and advanced BS receiver

- MSD optimization

- UL Tx switching enhancement

- Tx requirements enhancement for U6G

- Band/band combination requirements simplification

- Power classes framework

- Optimize MPR/A-MPR

- SRS enhancement

- SL-U enhancement

- MPR reduction by leveraging asymmetric ACLR

- Requirement enhancement to support coherent UL MIMO

- 256QAM evolution

Discussions

Comments:

Skywork: how to distinguish and what the relations between the first two bullets? We should make it clear.

Nokia: In Rel-20, the current Rel-19 WIs have too many objectives. We should only do HPUE enhancement in Rel-20. We need keep enough TU for 6G.

Ericsson: we should dramatically down-select. We are not OK to downselect the proposal with one company supporting now. Our proposal to optimize the MPR/A-MPR, we want to have some time to discuss the corresponding band combination.

Apple: For HPUE, we see a lot of PC1 proposals. Is it for FWA only or also consider handheld. 4Tx HPUE is 32dBm which is not aligned with 31dBm.

ZTE: High power UE is more popular. The test tolerance can support 32dB. Regarding relaxing Tx EVM, it is better to group it into the demodulation part. Downselection needs doublecheck.

OPPO: Now many topics are grouped into one. We support PC1.5. But Tx number would be our conern. Relaxing Tx EVM and Tx switching are our preference.

T-Mobile: Focus on HPUE. To Apple, our interest on PC1 is for FWA. We are interested in 2Tx or 4Tx FWA PC1. 2x29 or 4x26. We reuse PC1 or new power classes are fine.

Qualcomm: We should first understand the TU splitting between 5G and 6G. We should consider pre-6G such like power class framework. For PC1, we are ok with PC1 with modified tolerance. For Tx EVM relaxation, we need compare the uplink throughput between Tx EVM relaxation vs no Tx EVM with the same enhancement in the base station side. The base station enhancement also works when UE has no relaxed Tx EVM. You can also get the power gain.

Skyworks: What the MPR/A-MPR are? PC1 or PC1.5, what the emission requirement should be targeted at?

CATT: Echo Ericsson. This is too long list. We need dramatically down-selection. The Tx EVM relaxation should be cross are topic.

Huawei: We need consider the scope of HPUE with different number of Tx. Tend to agree with ZTE, we need consider the commercial feasibility such as the filter capability for HPUE. For FDD, 1Tx 26dBm and 1Tx 29dBm would be too early. Regarding Tx EVM, that is a good direction. The demodulation part can be captured in the potential WID. Power class MSD is also helpful to reduce the workload. For enhancement of U6G, for 6G it is very important. ACLR for U6G should be based on the co-existence study. With 26dB ACLR we can get better power.

Sony: We are aligned with Ericsson, Nokia, Qualcomm. We should first guarantee the TU. Some specific comment: for high power we see the strong interest. Based on current proposal, there are quite a lot. We need do downselection. For EVM relaxation, we need some study phase. How does BS behave to decode this? It could not be applied to all the modulation order.

Mediatek: We have concern on 29dBm with 1Tx, which is not commercially implemented. That has big impact on handheld. It should be limited to FWA. We can first focus on 4Tx PC1. For some proposal, we need the study phase for Tx EVM with the evaluation of power enhancement. Other AMPR/MPR feature is same like Rel-19 proposal. We support coherent UL-MIMO.

LGE: We need have TU analysis first. How much WI that we can accommodate. The max 3 objectives should be considered. Tx EVM should be necessary to have study phase. This is related to AI processing. We hope that AI study based topic can be deferred to 6G.

Nokia: Indeed, we need splitting TU between 6G and 5G. RAN4 splitting should be the same as other WGs. We have include RAN1~3 led items and spectrum performance. RAN4-led items should be very small. Merging into one item does not make much difference. We see the interest common for HPUE.

Xiaomi: Regarding TU splitting, we should be careful about the splitting between 5G and 6G should be balanced. We should reduce the workload for Rel-20 compared to Rel-19. We should focus on critical items. HPUE get more support. For HPUE, we have totally 10 items. We should consider which is more feasible. We have the same comment as Apple and Mediatek. We also need consider the filter impact. Some feasibility study is needed. For Tx EVM relaxation, we echo other companies that the study phase is needed.

Samsung: General question e.g., we do believe that some companies can support other feature, will we change the order. For TU splitting, we need follow the same TU as other WGs. We also need consider RAN4 spectrum item. For HPUE, we do receive a lot of interest and it is really important. But we also agree that 29dBm 1Tx needs more discussions. For Tx EVM, we also think that our proposal with AI receiver, we do have similar consequence as other companies’ proposal in this UE RF. We are interested in merging them. We are open to how to handle it, first in 5G track and then in 6G track. We are open to that.

Ericsson: Tx EVM, we do see some cross areas. We need expertise from demod to evaluate it. And also we do not think we should associated BS algorithm enhancement with Tx EVM and we have concern on using AI. On the U6G, the proposals for bandwidth, we do support 200MHz CBW and that should be discussed in RAN1. For ACLR, we should consider the input from ITU-R. For bandwidth combinations simplification, we need consider the data base. In general, we will have only small amount of TU.

Murata: Share the similar view as Huawei, Xiaomi… For HPUE, we need study. RF performance MPR/A-MPR should be based on such study. We do not have such commercial implementation to support some proposal.

Apple: RAN4 generally, we do not know how much capacity will be left. Without the clear understanding, it is very hard to proceed. We also need consider. The more efficient way to use the time is to figure out the examples. We have same TU splitting between 5G and 6G as the other WGs? Which project should be handled in 6G or 5G. For AI related, RAN4 is overloaded with AI. AI topics can be delayed to 6G.

OPPO: Agree with Apple on the TU issue. How many TUs can be used in RAN4. No matter, in the first half year, 5G would be dominant and in the second half year 6G. We can stagger the TU arrangement for 5G and 6G. For 6G, there are some propsoals which are more suitable for 6G.

Nokia: to OPPO, on 6G, it is true that for RAN1 things, RAN4 cannot start first. but RAN4 needs start what the requirements should be defined and how to define the band/band combinations at the beginning of Rel-20.

CATT: On band and band combination simplification, we support this work. One comment is that we cannot remove the table and just rely on the database only.

Xiaomi: Regarding TU, what is in the scope of 6G in RAN4? It cannot be discussed and decided now. We should figure out the items which are important. Some of them should not be urgent. Some requires the long term study. Regarding the detailed TU, we should have general guidance that we should be same as other WGs.

ZTE: Commercial interest and minimal workload.

Apple: Regarding the commercial interest, should we consider the counting the supporting companies?

Skywork: if you relax the Tx EVM how about the IBE? It seems 6G study.

CHTTL: MSD simplification for 5G is too early.

Summary/conclusions

In the section, the high level candidate objectives for UE RF centric evolution and/or enhancement in Rel-20 are captured, where companies share the common interests in the bullet(s) highlighted by blue. The candidate objectives below are listed without priority.

The following list of high level candidate objectives can be used as the starting point for 5G-A UE RF centric evolution and/or enhancement in Rel-20, considering the proper TU splitting between 5G and 6G.

- FR1 HPUE enhancement (PC2/PC1.5/PC1) for single carrier and EN-DC/NR-CA

  - This item receives common interest, but needs discussion on the feasibility of some use cases and also need down-selection.

- Power enhancement with relaxed UE Tx EVM and/or uplink throughput improvement with advanced BS receiver (AI based receiver is not precluded as the starting point)

  - TBD on objectives should include Tx EVM relaxation, uplink throughput improvement, or both.

  - Study phase is needed if it is considered and FFS whether to treat it in UE RF or demod package

  - There are still concern on whether Tx EVM should be part of Rel-20 package for 5G-A.

- Band/band combination requirements simplification and MSD simplification/optimization

  - TBD on whether MSD work is for PC3 and/or HPUE as Rel-19 leftover work.

- Tx requirements enhancement for U6G

- Power classes framework

- Optimize MPR/A-MPR

- Requirement enhancement to support coherent UL MIMO

- UL Tx switching enhancement

- 256QAM evolution

- SRS enhancement

- SL-U enhancement

- MPR reduction by leveraging asymmetric ACLR

BS RF centric evolution

Summary of contributions

| NO. | Proposed objectives for Rel-20 and supporting companies | High level objectives |
| --- | --- | --- |
| 1 | Enhancement of co-location and co-existence requirements Nokia (0085), Ericsson (0540), ZTE (0566) | Proposed objectives: - The OTA co-location requirement was designed for BS up to 3 GHz and is not suitable for higher frequencies - Rel-20 should aim to define an improved co-location concept that will enable real and relevant requirements to be defined and tested OTA for relevant frequency ranges and deployment scenarios above 3 GHz - Essential to support robust performance in multi-operator/vendor shared tower deployments |
| 2 | Additional Hybrid requirement sets for BS type 1-H/large antenna array Huawei (0284), Ericsson (540) | Proposed objectives: - Introduce additional Hybrid requirement sets with more OTA options for some TX and RX requirements for the BS type 1-H - Ensure specification support for hybrid beamforming - Define unambiguous BS types for bands / large array sizes |
| 3 | OTA requirements enhancement Nokia (0085), Ericsson (0540) | Proposed objectives: - In Rel-20 RAN4 should re-evaluate RX blocking levels with respect to technical background assumptions (e.g., UE power, BS antenna) - Ensure that other OTA requirements are properly defined such that OTA testing becomes feasible |
| 4 | NB-IoT to AAS BS specifications Huawei (0284) | Proposed objectives: - Define the transmitter and receiver requirements and conformance testing for introduction of NB-IoT in-band, NB-IoT guard band, and standalone NB-IoT operation to AAS BS specifications. |
| 5 | LP-WUS in TN MSR spec ZTE (0566) | Proposed objectives: - Specify the LP-WUS related requirement in MSR specification which is similar as NB-IoT operation in NR in-band in Rel-16. |
| 6 | OBUE requirements structure simplification Huawei (0284) | Proposed objectives: - (Huawei) Investigate OBUE requirements structure simplification solutions in (all network-nodes specifications; conducted and OTA), e.g. compacting tables, etc. |

Moderator’s observations

According to moderator’s observations, the following evolution/enhancement directions attract the relatively more common interests (≥2 sources):

- Enhancement of co-location and co-existence requirements

- Additional Hybrid requirement sets for BS type 1-H/large antenna array

- OTA requirements enhancement

The other evolution/enhancement directions are also captured in this summary but seem get less supports and may need more discussions. Moderators would like to remind companies that the available TU for Rel-20 5G-Advanced would be relatively limited compared with the proposals, and would like to encourage companies to converge on a limited number of mainstream proposals, which would be more critical and will be commercialized.

Offline discussions and conclusions

From moderator’s point of view the following list of BS RF centric evolution and/or enhancement can be used as the starting point for further discussions to converge on Rel-20 5G-Advanced RAN4 package. It does not mean that the technique directions under this list must be part of a certain WI (e.g., BS RF enhancement in Rel-20), in other words, how to organize the final RAN4 project package will be further discussed separately.

The following list of high level candidate objectives can be used as the starting point for 5G-A BS RF centric evolution and/or enhancement in Rel-20:

- 1) Enhancement of co-location and co-existence requirements

- 2) Additional Hybrid requirement sets for BS type 1-H/large antenna array

- 3) OTA requirements enhancement

- 4) NB-IoT to AAS BS specifications

- 5) LP-WUS in TN MSR spec

- 6) OBUE requirements structure simplification

Discussions

Comments:

ZTE: For the second one “Additional Hybrid…”, it is fine for us since the workload is small. The third one, I am sure whether it can improve the performance. We do not use measurement so far. For FR1 band, we have a lot of band supporting AAS. We are ok with forth one, i.e., NB-IoT to AAS BS spec.

Huawei: 1) is under discussion in Rel-19. The work is clear. Some companies seemed assume that we will delay Rel-19 work. I do not see how to give a blind check to say that we should start the Rel-20 work. For 2), we support it. For 3) receiver blocking we do not know the details. Study item? In Rel-19 we do work. The details to be addressed is not clear. Support 4). For 5) it could be good not in the priority.

Nokia: 1) and 3) are in high priority. 2), what is it intended? Do we follow OTA testing? Beamforming should be implementation?

Ericsson: 1) this is focusing on co-location requirements. We should see the Rel-19 work as the study item. 2), the new requirement set is interesting. There are many aspects to hybrid requirement. We can introduce the new requirement set and how to map the requirements to capability. 3) OTA for blocking this is in same …. If needed, we update the higher frequency requirement. For 5) we can deprioritize it.

CATT: 1) 2) and 4) should have more priority. For the 1) we agree that the title is misleading.

Samsung: pure study objective should not be included for 5G-A. For 1), we agree with Huawei. That is already in the scope. We should try best to complete it in Rel-19. 2) and 3) should be prioritized.

ZTE: If checking with Chinese operator, they have their own spec. Currently RAN4 only has EIRP and EIS requriements. But Operators have more.

Ericsson: we need general principle for OTA and conductive. Out-of-band should be OTA. In-band should be with conductive. That will make the work simple. There is a link between 1) and 2).

Huawei: 1) ~3) requires some clarification what we really do. 1) is related to colocation isolation, which is not going in Rel-19. How can we handle it as Rel-20.

Summary/conclusions

In the section, the high level candidate objectives for BS RF centric evolution and/or enhancement in Rel-20 are captured. The candidate objectives below are listed without priority.

The following list of high level candidate objectives can be used as the starting point for 5G-A BS RF centric evolution and/or enhancement in Rel-20, considering the TU splitting between 5G and 6G in RAN4

- 1) Workable and testable co-location requirements for FR1 high band(s)

  - Necessity of this objective depends on the progress in Rel-19 and the group will check it in September 2025.

  - NOTE: refinement of wording on this bullet is needed.

- 2) Additional Hybrid requirement sets for BS type 1-H/large antenna array

- 3) OTA requirements enhancement for receiver in-band blocking

- 4) NB-IoT to AAS BS specifications

- 5) LP-WUS in TN MSR spec

- 6) OBUE requirements structure simplification

  - FFS on whether to do it in Rel-20 WI or as maintenance

UE OTA enhancement

Summary of contributions

| NO. | Proposed objectives for Rel-20 and supporting companies | Details of objectives |
| --- | --- | --- |
| 1 | TRP/TRS and MIMO OTA test enhancement vivo (0209), Huawei (0284), Apple (0321), Xiaomi (0353), CAICT (0423), Thales (0457) | Proposed objectives: - Enhance test method - More UE types/advanced features, including - CA, 3T/6R, XR UE - NTN for Ku band, RedCap NTN, mobile VSAT - FR1 Dynamic MIMO OTA testing - Phantom-based dynamic MIMO OTA test method, Multi-TRP - Performance requirements - Specify TRP and TRS requirements for new bands: - Specify MIMO OTA requirements for new bands |

Moderator’s observations

The proposed evolution or enhancement technique directions for TRP/TRS and MIMO OTA are basically aligned. But moderators would like to remind companies that the available TU for Rel-20 5G-Advanced would be relatively limited compared with the proposals, and would like to encourage companies to converge on a limited number of mainstream proposals, which would be more critical and will be commercialized.

Offline discussions and conclusions

From moderator’s point of view the following list of UE OTA evolution and enhancement can be used as the starting point for further discussions to converge on Rel-20 5G-Advanced RAN4 package. It does not mean that the technique directions under this list must be part of a certain WI (e.g., UE OTA in Rel-20), in other words, how to organize the final RAN4 project package will be further discussed separately.

The following list of high level candidate objectives can be used as the starting point for 5G-A UE OTA evolution and/or enhancement in Rel-20

- Enhance test method

- More UE types/advanced features, including

  - CA, 3T/6R, XR UE

- NTN for Ku band, RedCap NTN, mobile VSAT

- FR1 Dynamic MIMO OTA testing

  - Phantom-based dynamic MIMO OTA test method, Multi-TRP

- Performance requirements

- Specify TRP and TRS requirements for new bands:

- Specify MIMO OTA requirements for new bands

Discussions

Xiaomi: Regarding NTN for Ku band and mobile VSAT, we need clarify whether it is target for radiated test method or for OTA TRP/TRS test method. Need clarification of scope on it.

Sony: for XR UE, we have on-going WI. Not clear to have it. For NTN, I am not sure if we need NTN RedCap UE.

OPPO: What is the phantom is?

Ericsson: XR UE should be in the perf scope.

Huawei: Not clear whether we should have some limitation for band number. We should consider the scope. XR UE -> Wearable UE.

Samsung: agree with Huawei to consider band number.

Sony: what is difference for DL CA and single carrier?

Vivo: we should keep UL and DL CA. We simply use single carrier but with different configurations.

Summary/conclusions

In the section, the high level candidate objectives for UE OTA RF enhancement in Rel-20 are captured. The candidate objectives below are listed without priority.

The following list of high level candidate objectives can be used as the starting point for 5G-A UE OTA evolution and/or enhancement in Rel-20, considering the TU splitting between 5G and 6G in RAN4

- Enhance test method

- More UE types/advanced features, including

  - CA, 3T with inter-band CA, 6R, wearable devices

- FFS on RedCap NTN

- Radiated test method should be developed for mobile VSAT on Ku and Ka band

- FR1 Dynamic MIMO OTA testing

  - Hand phantom-based dynamic MIMO OTA test method

  - FFS on Multi-TRP

- Performance requirements

- Specify TRP and TRS requirements for new bands:

  - Single carrier is prioritized

  - FFS on whether to cover wearable devices

- Specify MIMO OTA requirements for new bands based on the traditional static OTA test method

RRM enhancements

Summary of contributions

| NO. | Proposed objectives for Rel-20 and supporting companies | High level objectives |
| --- | --- | --- |
| 1 | RLM/BFD relaxation for RedCap UE OPPO (0165), vivo (0209), Huawei (0284), Xiaomi (0353), China Telecom (0387), Sony (0456), Mediatek (0483), CATT (0518), Ericsson (0540) | Proposed objectives: RLM/BFD relaxation requirements for RedCap UE |
| 2 | Fast SCell activation Nokia (0085), Samsung (0104), Huawei (0284), Apple (0321), China Telecom (0387) | Proposed objectives: - Further reduce the delay of measurement and procedure for SCell activation for intra-band CA - Reduce SCell activation delay based on Temporary RS and L3 measurement triggered by SCell activation command, and define specific requirements for unknown SCell - utilizing A-TRS for the enhancement to the corresponding UE activities for SCell activation |
| 3 | Uplink only SCell (NES SSB-less SCell enhancement) Samsung (0104), Intel (0465), ZTE (0566), CATT (0518) | Proposed objectives: - Support SCell with uplink only (SSB-less SCell without DL signals) - SSB-less SCell for intra-band NC CA |
| 4 | FR2 multi-Tx and multi-Rx Samsung (0104), LGE (0240) | Proposed objectives: - Multi-Tx: Enable UL STxMP (Simultaneous transmission across multiple panels) - Multi-Rx: Enhance simultaneous DL reception to support Rel-17/18 unified TCI state framework and inter-cell multi-TRP scenarios |
| 5 | Enhanced RRM requirements for NES OPPO (0165), Ericsson (0540) | Proposed objectives: - (OPPO) Specify the RRM requirements to support the combined OD-SBB and always on-SSB based L3 measurement (Ericsson) To support OD-SSB based multiple Scell activation delay requirements if not done in Rel-19 |
| 6 | Enhanced RedCap with LTM Ericsson (0540) | Proposed objectives: - Specify support for L1/L2 based mobility for RedCap - Based on the LTM requirements introduced in Rel-18 for non-RedCap UEs, define LTM requirements for RedCap UEs |
| 7 | R18 leftover on LTM RRM vivo (0209) | Proposed objectives: - Specify the RRM requirements to support the combined OD-SBB and always on-SSB based L3 measurement |
| 8 | Interruption at NR SRS antenna port switching CMCC (0262) | Proposed objectives: - Specify interruption requirements at NR SRS antenna port switching to cover the case that SRS resource(s) is not within the last 6 symbols of a slot |
| 9 | Parallel measurement CMCC (0262) | Proposed objectives: - Specify RRM requirements to support parallel measurements upon MG collision |
| 10 | Inter-frequency measurement enhancement for LTM CMCC (0262) | Proposed objectives: - Specify RRM requirements to support L1 inter-frequency measurement with NCSG and L1 inter-frequency measurement with NeedforGap |
| 11 | On-demand deactivated SCell measurement Huawei (0284) | Proposed objectives: - Introduce on-demand fast deactivated SCell measurement and specify corresponding SCell activation requirements |
| 12 | A-TRS based RRM enhancement Apple (0321) | Proposed objectives: - utilizing A-TRS for the enhancement to the corresponding UE activities - HO, - TCI switching - SCell activation - PSCell addition/change |
| 13 | Dynamic RTD/TTD status update Apple (0321) | Proposed objectives: - Dynamic RTD/TTD status update |
| 14 | AL/ML beam prediction testing Qualcomm (0458) | Proposed objectives: - AI/ML beam prediction testing to provide foundation to 6G |
| 15 | Handover interruption minimization Mediatek (0483) | Proposed objectives: - Early DL synchronization before handover - Introduce an enhanced UE behavior to start autonomous early pre-synchronization to neighboring cells - Study and identify the applicable conditions - Under the applicable conditions, define the reduced handover delay without additional time for synchronization - Postponed interruption (in case time for synchronization during handover is still needed) - Introduce an enhanced HO delay requirement by leveraging the waiting time of SSB to reduce interruption. - If needed, LS can be sent to RAN2 for necessary signaling change, based on RAN4 consensus RRM Handover interruption minimization [1/3] RP-250483 RRC Dec Proc. SS |
| 16 | Interruption-free DCI operation Mediatek (0483) | Proposed objectives: - Introduce a new UE type capable of interruption-free DCI reception with reduced RF re-tuning time - Identify which existing interruption requirements can be enhanced with the enhanced UE behavior - If needed, introduce new requirements or modify existing requirements for UEs capable of interruption-free DCI reception |
| 17 | Extend OD-SSB to PSCell in NR-DC ZTE (0566) | Proposed objectives: - Extend the application of OD-SSB to PSCell in NR-DC - To speed up the deactivated PSCell identification/measurement - To reduce the latency of PSCell addition, PSCell change, SCG activation |

Moderator’s observations

According to moderator’s observations, the following evolution/enhancement directions attract the relatively more common interests (≥4 sources):

- RLM/BFD relaxation for RedCap UE

- Fast SCell activation

The other evolution/enhancement directions are also captured in this summary but seem get less supports and may need more discussions. Moderators would like to remind companies that the available TU for Rel-20 5G-Advanced would be relatively limited compared with the proposals, and would like to encourage companies to converge on a limited number of mainstream proposals, which would be more critical and will be commercialized.

Offline discussions and conclusions

From moderator’s point of view the following list of RRM evolution and/or enhancement can be used as the starting point for further discussions to converge on Rel-20 5G-Advanced RAN4 package. It does not mean that the technique directions under this list must be part of a certain WI (e.g., RRM enhancement in Rel-20), in other words, how to organize the final RAN4 project package will be further discussed separately.

The following list of high level candidate objectives can be used as the starting point for 5G-A RRM centric evolution and/or enhancement in Rel-20:

- RLM/BFD relaxation for RedCap UE

- Fast SCell activation

- Uplink only SCell (NES SSB-less SCell enhancement)

- FR2 multi-Tx and multi-Rx

- Enhanced RRM requirements for NES

- Enhanced RedCap with LTM

- R18 leftover on LTM RRM

- Interruption at NR SRS antenna port switching

- Parallel measurement

- Inter-frequency measurement enhancement for LTM

- On-demand deactivated SCell measurement

- A-TRS based RRM enhancement

- Dynamic RTD/TTD status update

- AL/ML beam prediction testing

- Handover interruption minimization

- Interruption-free DCI operation

- Extend OD-SSB to PSCell in NR-DC

Discussions

Comments:

Ericsson: the RRM session is overloaded. For the fast SCell activation, we would like to see more specific solution and benefit of the solution before we see the solution.

LGE: consider there will be multiple candidate cross area, we should be careful about the objective. We prefer to select one objective, i.e., Cell activatioin.

Apple: We can reuse the REl-17 solution as baseline.

Nokia: be careful on the RRM objectives. We cannot approve more than 1 or 2 items. For RRM BFD/RLM it makes sense. Agree with Ericsson on fast SCell activation. We need more discussions on the principle.

Mediatek: focus on 1Rx UE for RLM objective. The solution for SCell activation is complicated already.

Huawei: We need double check 1Rx proposal.

Xiaomi: Support RLF/BFD relaxation. That is the only one objective which has consensus. 1Rx and 2Rx should be considered. For fast Scell activation, the detailed solution and target scenario are quite diverse. I wonder if the single

ZTE: We have no consensus on the common interests. We can remove the “unified solution” bullet.

CATT: For RLM/BFD, we see the strong interests. For 1Rx and 2Rx, we should keep it open. For fast SCell activation, considering EN-DC will be used for a long time, we can prioritize the PSCell.

Ericsson: RLM/BFD, we need 1Rx and 2Rx. 2Rx needs less work. We need see concrete proposal for fast scell activation.

Samsung: for RLM/BFD, we need consider 2Rx. For SCell activation, for PSCell addition enhancement, it is not mentioned by many companies. We can have SCell solution first and then see whether it can be reused for PSCell

Spreadtrum: it is tied to eRedCap. We doubt the necessity of SCell activation.

Vivo: Support RLM/BFD for RedCap. We have concern on the multiple solutions for fast Scell activation.

Qualcomm: Fine with the first one. For fast SCell, we have enh. in every release. We need see the solution. We should pick the simple one with benefit.

Sony: RLM/BFD is in high priority. SCell activation has diverse solution.

China Telecom: Consider 1Rx and 2Rx for the first one.

NTT DOCOMO: Fine with RLM/BFD relaxation. Should we discuss “AL/ML beam prediction testing” in RRM or cross area.

Furturewei: Support RedCap objective.

Intel: Support the redcap and consider 1Rx and 2Rx. Activation of SCell solution is diveres.

Nokia: We need check and come back to June for SCell activation,

Apple: For fast SCell activation, we do not want to design the new solution. We want to reuse the existing solution.

Anterix/Nokia: support RedCap and four companies supported it. less 5MHz redcap got more support.

OPPO: We support the Redcap RLM/BFD relaxation, and for Fast SCell activation which has been enhanced for several releases, benefits need to be shown if further enh it.

Summary/conclusions

In the section, the high level candidate objectives for RRM enhancement in Rel-20 are captured, where companies share the common interests in the bullet(s) highlighted by blue. The candidate objectives below are listed without priority.

The following list of high level candidate objectives can be used as the starting point for RRM enhancement in Rel-20, considering the proper TU splitting between 5G and 6G.

- RLM/BFD relaxation for RedCap UE

  - Consider to reuse Rel-17 solution as baseline

  - Consider both 1Rx and 2Rx.

- Fast SCell activation [and PSCell addition] enhancement

  - Consider the reduction of the delay of measurement and/or procedure

    - FFS on the detailed solutions

  - Whether to have this item depends on the detailed proposals of solution

    - One unified solution is expected

- Uplink only SCell (NES SSB-less SCell enhancement)

- FR2 multi-Tx and multi-Rx

- Enhanced RRM requirements for NES

- Enhanced RedCap with LTM

- R18 leftover on LTM RRM

- Interruption at NR SRS antenna port switching

- Parallel measurement

- Inter-frequency measurement enhancement for LTM

- On-demand deactivated SCell measurement

- A-TRS based RRM enhancement

- Dynamic RTD/TTD status update

- AL/ML beam prediction testing

- Handover interruption minimization

- Interruption-free DCI operation

- Extend OD-SSB to PSCell in NR-DC

Demodulation performance evolution

Summary of contributions

| NO. | Proposed objectives for Rel-20 and supporting companies | High level objectives |
| --- | --- | --- |
| 1 | SCM based requirements vs SCM further study Nokia (0085), Samsung (0104), CMCC (0262), Xiaomi (0353), China Telecom (0387), Qualcomm (0458), Intel (0465), Mediatek (0483), Ericsson (0540) | Proposed objectives: - (Nokia, CMCC, Ericsson) Introduce the following SCM based requirements for SU-MIMO and MU-MIMO - (Samsung) Further study on SCM impact on demodulation performance by considering testability and test repeatability for MU-MIMO, FR2, AI, ISAC and etc - (Qualcomm/Mediatek) If channel model can be aligned well in Rel-19, introduce the simple demodulation test focusing on SU-MIMO [with neighbour cell interference] - (Ericsson) Study spatial channel model for AI/ML |
| 2 | R-ML receiver for 8Rx SU-MIMO scenario Huawei (0284), Xiaomi (0353), China Telecom (0387), Mediatek (0483) | Proposed objectives: - Introduce the UE performance requirements for 8Rx SU-MIMO scenario with R-ML receiver |
| 3 | CSI Reporting Requirements Enhancement Apple (0321), China Telecom (0387) | Proposed objectives: - Study the framework and methodology for CRI reporting requirements - Use CRI reporting in LTE as a starting point - Define CRI reporting requirements for at least 32 TX ports for Type I codeboo |
| 4 | BS MMSE-IRC Xiaomi (0353), China Telecom (0387), ZTE (0566) | Proposed objectives: - MMSE-IRC for BS (async TDD) - Investigate the advanced receiver to address the co-channel CLI in the HetNet |
| 5 | 6-layer performance requirements for handheld UE Qualcomm (0458) | Proposed objectives: - 6-layer performance requirements for handheld devices |
| 6 | UE IC receiver ZTE (0566) | Proposed objectives: - Investigate the IC receiver for UE demod requirement |

Moderator’s observations

According to moderator’s observations, the following evolution/enhancement directions attract the relatively more common interests (≥4 sources):

- SCM based requirements vs SCM further study

- R-ML receiver for 8Rx SU-MIMO scenario

The other evolution/enhancement directions are also captured in this summary but seem get less supports and may need more discussions. Moderators would like to remind companies that the available TU for Rel-20 5G-Advanced would be relatively limited compared with the proposals, and would like to encourage companies to converge on a limited number of mainstream proposals, which would be more critical and will be commercialized.

Offline discussions and conclusions

From moderator’s point of view the following list of demodulation evolution and/or enhancement can be used as the starting point for further discussions to converge on Rel-20 5G-Advanced RAN4 package. It does not mean that the technique directions under this list must be part of a certain WI (e.g., demodulation in Rel-20), in other words, how to organize the final RAN4 project package will be further discussed separately.

The following list of high level candidate objectives can be used as the starting point for 5G-A demodulation evolution and/or enhancement in Rel-20:

- SCM based requirements vs SCM further study

- R-ML receiver for 8Rx SU-MIMO scenario

- CSI Reporting Requirements Enhancement

- BS MMSE-IRC

- 6-layer performance requirements for handheld UE

- UE IC receiver

Discussions

Comments:

Nokia: It should be SCM based requirements

OPPO: for 6-layer, we had difficult discussion in Rel-19. Should we reevaluate in Rel-20.

Samsung: for SCM, we have different view from Nokia. Further study is the more common understanding. We need study for the remaining part. For 6-layer, we share the view from OPPO.

Ericsson: For SCM, it would not be possible to conclude study. I do not see the study for AI. We need consider AI in SCM study.

ZTE: Need consider the TU balance between BS demod and UE demod.

Apple: for SCM RAN4 is under the study. It is better to let RAN4 continue study, although we understand the importance. We need it for 6G. For CSI reporting, we propose it based on understanding that in LTE we had such requirement but in NR we had no. It is meaningful test. We think it is better to have 3GPP specified requirements to make sure UE behave properly. For 6-layer, what is the change compared in REl-19.

Xiaomi: For SCM, echo comments from Samsung… We had no final decision in Rel-19. We suggest to add the checking point in SCM bullet. We did not touch RLM CSI requirement previously. We should be consistent.

NTT DOCOMO: For BS MMSE-IRC, it is better to add the enhancement scenario.

China Telecom: in Rel-19 we can work out the stable SCM channel and we can proceed with WI. Otherwise we need move the study to 6G.

CATT: For SCM, we are on the same page. In the case, we won’t get the conclusion on SCM. It makes no sense to list here. Xiaomi made the point to set up the checking point. For 6-layer the conclusion whether it can be applied to handheld is clear to us. We cannot develop requirements here.

Intel: Support SCM. Some work can be completed.

BT: Support SCM work. SU-MIMO and maybe MU-MIMO should be considered. The work should be completed.

SK telecom: Share the Qualomm view to consider 6-layer in Rel-20.

Vivo: Share the similar view as Samsung for SCM. We are OK to have the SCM in 6G Day1. We can consider moving it to 6G.

Huawei: We are trying to conclude on SCM and the study is on-going. We should first conclude the study. Should we have SI in parallel to normative work?

LGE: Support SCM. We should be careful to develop the SCM based testing because of test cost.

Nokia: It is business as usual, i.e., SI first and WI follows. There are huge operators to support this work.

ZTE: Can we have general principle that one UE objective and one BS objective?

Mediatek: SCM needs more time to see the progress and it is reasonable to have checking point. For 6-layer, we also consider EVM tighten in the same package.

Samsung: It is same for SBFD.

Xiaomi: For SCM we need focus on the technique aspects. We are ready to see that we must have requirements in Rel-20. For RML we should focus on demodulation.

Ericsson: I do not think we need do both BS and UE. We should focus on commercial.

Sony: For SCM, at least under Rel-20 we should focus on the scenario with convergence of Rel-19. For any new scenario, we need study, which can be put in 6G track.

Qualcomm: For SCM, it is better to come back in June or September. It is a good pre-6G. We should do SU-MIMO test,

Apple: Xiaomi, we can make it clear for CRI. For SCM, we agreed with Nokia. We can follow the normal procedure. Agree with Qualcomm, it is pre-6G. To make checking point in September. For reduced RML requirement, we do not have signalling. Are we going to have UE capability

AT&T: Many operators co-signed the proposal for SCM. We want SCM to be captured.

T-Mobile: support SCM.

Summary/conclusions

In the section, the high level candidate objectives for demodulation evolution and/or enhancement in Rel-20 are captured, where companies share the common interests in the bullet(s) highlighted by blue. The candidate objectives below are listed without priority.

The following list of high level candidate objectives can be used as the starting point for demodulation evolution and/or enhancement in Rel-20, considering the proper TU splitting between 5G and 6G.

- SCM based requirements and/or further study of SCM

  - FFS whether new requirements need be specified in Rel-20 depending on Rel-19 SI progress, and check the progress by September.

  - [At least for SU-MIMO requirement is expected in Rel-20].

- R-ML receiver for 8Rx SU-MIMO scenario

  - Focus on demodulation requirements rather than CSI reporting

  - FFS on whether the new UE capability is needed.

- CRI Reporting Requirements

- BS MMSE-IRC

  - For async TDD scenario and/or SBFD, and/or hetnet scenario with co-channel interference

- 6-layer performance requirements for handheld UE

- UE IC receiver

NTN enhancements

Summary of contributions

| NO. | Proposed objectives for Rel-20 and supporting companies | High level objectives |
| --- | --- | --- |
| 1 | NTN HPUE Samsung (0104), vivo (0209), CMCC (0262), Xiaomi (0353), China Telecom (0387), China Unicom (0423), ZTE (0566) | Proposed objectives: - PC1.5/PC1 for handheld NR-NTN and IoT-NTN devices - Higher power than PC1 for handheld NR-NTN and IoT-NTN devices - PC1/PC2 for RedCap NTN UE |
| 2 | NTN CA Samsung (0104), vivo, Xiaomi (0353), Thales (0457), Mediatek (0483), Visat, Inmarsat (0749) | Proposed objectives: - Intra-band and inter-band UL/DL CA for NR-NTN with necessary RF and RRM requirements being specified |
| 3 | Ku band NTN LGE (0240), Eutelsat (0291), CHTTL et al (0410), Mediatek (0483), CATT (0518), Sharp (0648) | Proposed objectives: - Fixed and other mobile types VSAT RF and RRM requirements for single carrier and intra-band CA in Ku band - Support intra-SAN CA in Ku band - (Mediatek) Specify requirements for HD-FDD devices for NR-NTN - (CHTTL, et al) Smaller channel bandwidth scenarios - Other Rel-19 leftover if needed. |
| 4 | NTN mobile VSAT Samsung (0104), Xiaomi (0353),Thales (0457), BMW Group, Fraunhofer IIS (0542), CATT (0518) | Proposed objectives: - NGSO mobile VSAT in Ku and Ka bands - Ultra-small mobile VSAT: definition of type 6 and/or additional type in above 10GHz band(s) - Specify requirements for HD-FDD devices for NR-NTN in Ka bands |
| 5 | Testing method for VSAT Thales (0457), vivo (0209), Xiaomi (0353), CAICT (0423) | Proposed objectives: - Investigate the applicability of current 3GPP test methods for the following example VSAT device types |
| 6 | NTN power domain enhancements Apple (0321), Xiaomi (0353) | Proposed objectives: - NTN power domain enhancements - MPR reduction with emission relaxation under NTN deployment |
| 7 | MSR for in-band IoT-NTN in NR Thales (0457), ZTE(0518) | Proposed objectives: - Create TS 37.181 (NTN equivalent of TS 37.141 NR, E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) conformance testing) and TS 37.108 (NTN equivalent of TS 37.104 NR, E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) radio transmission and reception) |
| 8 | Irregular CBW 7MHz for NR-NTN CATT (0518) | Proposed objectives: - Include 7MHz channel bandwidth for NR NTN |
| 9 | Mobility between TN and NR-NTN CATT (0518), ZTE (0566), BMW, Fraunhofer (0543), ViaSat (0749) | Proposed objectives: - Mobility enhancement between NR TN and NR NTN in the RRC_CONNECTED state. - Mobility enhancement between LTE TN and NR-NTN in the RRC_CONNECTED state. - Define the RRM requirement for NTN-TN handover |
| 10 | Repeater for NTN ZTE (0566) | Proposed objectives: - Mobility enhancement between TN and NR NTN in the RRC_CONNECTED state. |

Viasat, RP-250749

Moderator’s observations

According to moderator’s observations, the following evolution/enhancement directions attract the relatively more common interests (≥4 sources):

- NTN HPUE

- Ku band NTN

- NTN CA

- NTN mobile VSAT (testing method for VSAT)

The other evolution/enhancement directions are also captured in this summary but seem get less supports and may need more discussions. Moderators would like to remind companies that the available TU for Rel-20 5G-Advanced would be relatively limited compared with the proposals, and would like to encourage companies to converge on a limited number of mainstream proposals, which would be more critical and will be commercialized.

Offline discussions and conclusions

From moderator’s point of view the following list of NTN evolution and/or enhancement can be used as the starting point for further discussions to converge on Rel-20 5G-Advanced RAN4 package. It does not mean that the technique directions under this list must be part of a certain WI (e.g., NTN in Rel-20), in other words, how to organize the final RAN4 project package will be further discussed separately.

The following list of high level candidate objectives can be used as the starting point for 5G-A NTN evolution and/or enhancement in Rel-20:

- NTN HPUE

- Enhancements for Ku band NTN, including

- NTN CA including intra-band and inter-band UL/DL CA for NR-NTN

- NTN mobile VSAT

- Testing method for VSAT

- NTN power domain enhancements

- MSR for in-band IoT-NTN in NR

- Irregular CBW 7MHz for NR-NTN

- Mobility between NR-TN/IoT-NTN and NR-NTN in the RRC_CONNECTED state.

- Repeater for NTN

Discussions

Comments:

ZTE: Clarify the target RAT either NR_NTN or IoT-NTN. There is challenge on filter design. We need decide the target band.

CATT: Mobile VSAT receives more interest. About mobility enhancement of TN and NTN, we should make it general including E-UTRA NTN and NR NTN.

Xiaomi: I do see some overlapping between main bullets. We are talking about the non-spectrum. We can take Ka or Ku band as example bands for NTN CA work. We suggest the merge the second and third one. Enable CA for NTN with intra-band contiguous CA and take Ka or Ku band as example bands.

LGE: for Ku band NTN… for NTN, the other mobile VSAT can include the small antenna size. It is better to have example. There should be not RRM impact.

Viasat: on CA, we need consider L-band. we cannot agree with only intra-band.

Ericsson: We should remember that there is very limited TU available. We have concern to have all. We prefer the mobility. For Ku band, we are not sure.

Mediatek: even with the first two items, we have not enough TU. We cannot do all the proposals. On CA, according to online discussions, we can consider more generic CA framework. Regarding inter-band and intra-band, we prefer to do intra-band CA.

CSCN: Support HD-FDD in NTN band and VSAT. We support 29dBm handset NTN.

CHTTL: Ku band is non-spectrum related item in Rel-19. Due to time limited, Rel-19 focuses on the specific features. We think that there are a lot of features missing for example VSAT. For GSO operator, it is really important to have narrow band NTN.

Nokia: There is discussions on RAN2 package on the mobility between TN and NTN. The RAN4 impact is identified. Regarding the overlapping bulllets, it might be needed to consider the generic CA together.

OPPO: Regarding TU, we agree with Ericsson and MTK. Even for first two, it is large enough. For HPUE, we need different NR-NTN and IoT-NTN. CA has impact on RAN2 also.

Vivo: Need differentiate NR-NTN and IoT-NTN, At least for NB based NTN, HD is needed.

Eutelsat: Everything for Ku is important for us. CA is critical one.

Samsung: We agree with Nokia on CA part. General CA is better.

Thales: HPUE, Mobile VSAT in Ka and Ku band, CA for Ka/Ku and other band, mobility.

IntelSat: Support mobile VSAT, CA and narrow band services.

Sharp: To achieve the smaller antenna, we need HD-FDD is needed.

SES: we support the mobility between TN and NTN. Ka is also important. CA for Ka and Ku are important. Mobile VSAT is important.

His..: CA is important for Ka and Ku bands. Mobile VSAT.

BMW: Mobility between TN and NTN.

Skyworks: For CA, it is confusing. we need know whether it is DL CA or UL.

Nokia: need consider the other WG-led NTN item impacts on RAN4.

Summary/conclusions

In the section, the high level candidate objectives for NTN requirements evolution and/or enhancement in Rel-20 are captured, where companies share the common interests in the bullet(s) highlighted by blue. The candidate objectives below are listed without priority.

The following list of high level candidate objectives can be used as the starting point for NTN requirements evolution and/or enhancement in Rel-20, considering the proper TU splitting between 5G and 6G.

- NTN HPUE

  - Study whether the high power than PC1 for IoT-NTN and/or NR-NTN, and what high power classes should be considered.

  - FFS whether the proposed high power class can be applied to handheld + FWA or only FWA.

  - Need differentiate NR-NTN and IoT-NTN in terms of power class to be discussed.

- Enhancements for Ku band NTN, including

  - Intra-SAN intra-band CA in Ku band

  - Fixed and other mobile types VSAT (e.g., with small antenna size or LEO only) RF [and RRM] requirements for single carrier

    - For small antenna size, HD-FDD would be needed.

  - Smaller channel bandwidth support (e.g. 3, 5 MHz) and other Rel.19 leftovers

- NTN CA including intra-band and inter-band UL/DL CA for NR-NTN

- NTN mobile VSAT

- Testing method for VSAT

- NTN power domain enhancements

- MSR for in-band IoT-NTN in NR

- Irregular CBW 7MHz for NR-NTN

- Mobility between NR-TN/LTE TN and NR-NTN in the RRC_CONNECTED state.

  - Need check whether RAN2 package includes it.

- Repeater for NTN

NOTE: there are overlapping objectives above and FFS on how to merge them.

Cross RF and baseband topics

Summary of contributions

| NO. | Proposed objectives for Rel-20 and supporting companies | Motivation and High level objectives |
| --- | --- | --- |
| 1 | NR FR1 DL fragmented carrier Samsung(0104)*, vivo (0209), LGE (0240), Xiaomi (0353), Intel (0465), Mediatek (0483), ZTE (0566) * General principle: UE RF topics shall be selected to “address real and critical commercial deployment needs* ”. Identified demands from operators: (a) NR FR1 DL fragmented carriers and (b) low-low band CA via switching enhancement. | Motivation: - Follow-up WI of Rel-19 SI. Proposed objectives: - To introduce the mechanism and UE requirements to enable DL fragmented carrier with the proper assumptions - Specify necessary RF and RRM requirements to support fragmented DL carriers in FR1 following the outcome of Rel-19 SI - (Intel) limited to Rel-19 SI objectives/conclusions |
| 2 | Extension of low-low band CA via switching Samsung*(0104), vivo (0209), LGE (0240), Apple (0321), Xiaomi (0353), Qulalcomm (0458), ZTE (0566), Telus et al (0569) * General principle: UE RF topics shall be selected to “address real and critical commercial deployment needs* ”. Identified demands from operators: (a) NR FR1 DL fragmented carriers and (b) low-low band CA via switching enhancement. | Motivation: - Extend Rel-19 low-low band CA via switching to other cases Proposed objectives: - Extend Rel-19 Low-low band switching to other cases, for which the following candidates are proposed - To other FDD+SDL combo, if any, with dual DL being specified - To low band CA FDD+FDD - To switching between FDD UL/DL and FDD DL+SDL - Dynamic/rule based switching to improve performance |
| 3 | Irregular channel bandwidth (flexible channel bandwidth) Samsung (0104), OPPO (0165), China Telecom (0387), China Unicom (0423), Intel (0465), T-Mobile (0638) | Proposed objectives: - Support more irregular channel bandwidth - Support 6MHz for n29, n5, n12, n85, n28 with core and band specific requirements - Study and specify the support of flexible CBWs in TN and NTN with necessary guard bands and max transmission bandwidth being defined - Add support of 11MHz for Band n5 by Next-larger CBW Solution |
| 4 | 3MHz channel bandwidth for eRedcap devices Nokia (0085), Anterix (0093), Sony (0456), Intel (0465), Futurewei (0530), Ericsson (0540) | Motivation: Proposed objectives: - Introduce support of 3MHz channel bandwidth (15 RPB) for eRedcap devices in FR1 with 15KHz SCS [for both TN and NTN] - Example bands: n106, n85, n26, n28 - Introduce 3 MHz channel bandwidth support for n106 for 1Rx and 2Rx* eRedCap devices. - Utilize existing eRedcap capabilities defined in Rel-17, Rel-18 and Rel-19 unless otherwise specified. - Define the necessary RF and RRM Core and Performance requirements for UE and BS. - (Futurewei) Other migration aspects: - set optional features as mandatory for Rel-20 RedCap to guarantee power savings (e.g., LP-WUS) or performance benefits (e.g., PDSCH repetition) - RF bandwidth reduction to 3MHz, mandatory half-duplex, simplified cell search (only 15kHz SCS), reduced control channel processing - (Sony) Study the limiting factors in UE RF requirements in TS38.101-1 and specify the corresponding meachanism(s) to enable single SKU designs for HD-FDD eRedCap with baseband bandwidth reduction BW3/PR3 - Including 3MHz and 5MHz for SKU Note: SKU – stock-keeping unit |
| 5 | Intra-band non-collocated EN-DC/NR-CA deployment KDDI, et al (0277) | Proposed objectives: - Type 3 UE requirements assuming MRTD<CP and power imbalance < [xx] dB for handheld Without intended impacts on the network side |
| 6 | AI/ML assisted receiver Samsung (0104) | Proposed objectives: - AI/ML assisted non-linearity compensation (AI-NC) at BS - AI/ML channel estimation (AI-CE) at BS |
| 7 | ATG evolution LGE (0240) | Proposed objectives: - RF/RRM requirements of ATG for - FR1 intra/inter-band UL CA - FR2 in single carrier |
| 8 | UAV enhancement CMCC (0262) | Proposed objectives: - UAV enhancement - Use main lobe to enhance the UAV downlink coverage - Enhance the maximum output power of UAV - Beamforming for FR1 - Other RF/RRM requirements if needed |

Moderator’s observations

According to moderator’s observations, the following evolution/enhancement directions attract the relatively more common interests (≥4 sources):

- NR FR1 DL fragmented carrier

- Extension of low-low band CA via switching

- Irregular channel bandwidth (flexible channel bandwidth)

- 3MHz channel bandwidth for eRedcap devices

- Intra-band non-collocated EN-DC/NR-CA deployment

The other evolution/enhancement directions are also captured in this summary but seem get less supports and may need more discussions. Moderators would like to remind companies that the available TU for Rel-20 5G-Advanced would be relatively limited compared with the proposals, and would like to encourage companies to converge on a limited number of mainstream proposals, which would be more critical and will be commercialized.

Offline discussions and conclusions

From moderator’s point of view the following list of cross RF and baseband evolution and/or enhancement can be used as the starting point for further discussions to converge on Rel-20 5G-Advanced RAN4 package. It does not mean that the technique directions under this list must be part of certain WIs (e.g., fragmented DL in Rel-20), in other words, how to organize the final RAN4 project package will be further discussed separately.

The following list of high level candidate objectives can be used as the starting point for cross RF and baseband evolution and/or enhancement in Rel-20:

- NR FR1 DL fragmented carrier

- Extension of low-low band CA via switching

- Irregular channel bandwidth (flexible channel bandwidth)

- 3MHz channel bandwidth for eRedcap devices

- Intra-band non-collocated EN-DC/NR-CA deployment

- AI/ML assisted receiver

- ATG evolution

- UAV enhancement

Discussions

Comments: No time to have discussions in offline drafting session.

Summary/conclusions

In the section, the high level candidate objectives for cross-area evolution and/or enhancement in Rel-20 are captured. The candidate objectives below are listed without priority.

The following list of high level candidate objectives can be used as the starting point for cross-area evolution and/or enhancement in Rel-20, considering the proper TU splitting between 5G and 6G.

- NR FR1 DL fragmented carrier

  - Depending on the conclusion of Rel-19 SI

- Extension of low-low band CA via switching

  - FFS on the scenarios for Rel-20

- Irregular channel bandwidth (flexible channel bandwidth)

  - Check what the new aspects to be evaluated or new solutions to be introduced are? Need consider the other issues, e.g., guard band.

  - FFS whether it should be in 5G-A or 6G.

- 3MHz channel bandwidth for eRedcap devices, if any

  - Minimize the impact on RAN1/RAN2 impacts, if any

  - FFS on whether and what additional features should be supported by 3MHz eRedCap

- Intra-band non-collocated EN-DC/NR-CA deployment

- AI/ML assisted receiver

  - FFS whether it should be in 5G-A or 6G

- ATG evolution

- UAV enhancement

Conclusions

Below we list the candidate objectives for each potential RAN4 Rel-20 project as the starting point for the further discussions of RAN4 Rel-20 package. The candidate objectives below are listed without priority, and those highlighted by blue are the candidate objectives with the common interests shown among companies during RAN#107. The further discussions on the detailed objectives will be needed before making decisions on RAN4 Rel-20 package until September.

UE RF centric evolution

- FR1 HPUE enhancement (PC2/PC1.5/PC1) for single carrier and EN-DC/NR-CA

  - This item receives common interest, but needs discussion on the feasibility of some use cases and also need down-selection.

- Power enhancement with relaxed UE Tx EVM and/or uplink throughput improvement with advanced BS receiver (AI based receiver is not precluded as the starting point)

  - TBD on objectives should include Tx EVM relaxation, uplink throughput improvement, or both.

  - Study phase is needed if it is considered and FFS whether to treat it in UE RF or demod package

  - There are still concern on whether Tx EVM should be part of Rel-20 package for 5G-A.

- Band/band combination requirements simplification and MSD simplification/optimization

  - TBD on whether MSD work is for PC3 and/or HPUE as Rel-19 leftover work.

- Tx requirements enhancement for U6G

- Power classes framework

- Optimize MPR/A-MPR

- Requirement enhancement to support coherent UL MIMO

- UL Tx switching enhancement

- 256QAM evolution

- SRS enhancement

- SL-U enhancement

- MPR reduction by leveraging asymmetric ACLR

BS RF centric evolution

- 1) Workable and testable co-location requirements for FR1 high band(s)

  - Necessity of this objective depends on the progress in Rel-19 and the group will check it in September 2025.

  - NOTE: refinement of wording on this bullet is needed.

- 2) Additional Hybrid requirement sets for BS type 1-H/large antenna array

- 3) OTA requirements enhancement for receiver in-band blocking

- 4) NB-IoT to AAS BS specifications

- 5) LP-WUS in TN MSR spec

- 6) OBUE requirements structure simplification

  - FFS on whether to do it in Rel-20 WI or as maintenance

UE OTA enhancement

- Enhance test method

- More UE types/advanced features, including

  - CA, 3T with inter-band CA, 6R, wearable devices

- FFS on RedCap NTN

- Radiated test method should be developed for mobile VSAT on Ku and Ka band

- FR1 Dynamic MIMO OTA testing

  - Hand phantom-based dynamic MIMO OTA test method

  - FFS on Multi-TRP

- Performance requirements

- Specify TRP and TRS requirements for new bands:

  - Single carrier is prioritized

  - FFS on whether to cover wearable devices

- Specify MIMO OTA requirements for new bands based on the traditional static OTA test method

RRM enhancement

- RLM/BFD relaxation for RedCap UE

  - Consider to reuse Rel-17 solution as baseline

  - Consider both 1Rx and 2Rx.

- Fast SCell activation [and PSCell addition] enhancement

  - Consider the reduction of the delay of measurement and/or procedure

    - FFS on the detailed solutions

  - Whether to have this item depends on the detailed proposals of solution

    - One unified solution is expected

- Uplink only SCell (NES SSB-less SCell enhancement)

- FR2 multi-Tx and multi-Rx

- Enhanced RRM requirements for NES

- Enhanced RedCap with LTM

- R18 leftover on LTM RRM

- Interruption at NR SRS antenna port switching

- Parallel measurement

- Inter-frequency measurement enhancement for LTM

- On-demand deactivated SCell measurement

- A-TRS based RRM enhancement

- Dynamic RTD/TTD status update

- AL/ML beam prediction testing

- Handover interruption minimization

- Interruption-free DCI operation

- Extend OD-SSB to PSCell in NR-DC

Demodulation evolution

- SCM based requirements and/or further study of SCM

  - FFS whether new requirements need be specified in Rel-20 depending on Rel-19 SI progress, and check the progress by September.

  - [At least for SU-MIMO requirement is expected in Rel-20].

- R-ML receiver for 8Rx SU-MIMO scenario

  - Focus on demodulation requirements rather than CSI reporting

  - FFS on whether the new UE capability is needed.

- CRI Reporting Requirements

- BS MMSE-IRC

  - For async TDD scenario and/or SBFD, and/or hetnet scenario with co-channel interference

- 6-layer performance requirements for handheld UE

- UE IC receiver

NTN enhancements

- NTN HPUE

  - Study whether the high power than PC1 for IoT-NTN and/or NR-NTN, and what high power classes should be considered.

  - FFS whether the proposed high power class can be applied to handheld + FWA or only FWA.

  - Need differentiate NR-NTN and IoT-NTN in terms of power class to be discussed.

- Enhancements for Ku band NTN, including

  - Intra-SAN intra-band CA in Ku band

  - Fixed and other mobile types VSAT (e.g., with small antenna size or LEO only) RF [and RRM] requirements for single carrier

    - For small antenna size, HD-FDD would be needed.

  - Smaller channel bandwidth support (e.g. 3, 5 MHz) and other Rel.19 leftovers

- NTN CA including intra-band and inter-band UL/DL CA for NR-NTN

- NTN mobile VSAT

- Testing method for VSAT

- NTN power domain enhancements

- MSR for in-band IoT-NTN in NR

- Irregular CBW 7MHz for NR-NTN

- Mobility between NR-TN/LTE TN and NR-NTN in the RRC_CONNECTED state.

  - Need check whether RAN2 package includes it.

- Repeater for NTN

NOTE: there are overlapping objectives above and FFS on how to merge them.

Cross RF and baseband topics

- NR FR1 DL fragmented carrier

  - Depending on the conclusion of Rel-19 SI

- Extension of low-low band CA via switching

  - FFS on the scenarios for Rel-20

- Irregular channel bandwidth (flexible channel bandwidth)

  - Check what the new aspects to be evaluated or new solutions to be introduced are? Need consider the other issues, e.g., guard band.

  - FFS whether it should be in 5G-A or 6G.

- 3MHz channel bandwidth for eRedcap devices

  - Minimize the impact on RAN1/RAN2 impacts, if any

  - FFS on whether and what additional features should be supported by 3MHz eRedCap

- Intra-band non-collocated EN-DC/NR-CA deployment

- AI/ML assisted receiver

  - FFS whether it should be in 5G-A or 6G

- ATG evolution

- UAV enhancement

Annex reference contributions

The Tdocs submitted for RAN4 Rel-20 5G-Advanced are listed in the table below.

| TDoc | Title | Source | Type | Agenda ittem |
| --- | --- | --- | --- | --- |
| RP-250039 | Moderator's summary for RAN4 led REL-20 topics | RAN4 Chair (Huawei) | report | 15.3 |
| RP-250085 | RAN4-led topics for 5G-Advanced in Rel-20 | Nokia | discussion | 15.3 |
| RP-250093 | Proposal for <5 Mhz (e)RedCap Support in Rel-20 | Anterix | discussion | 15.3 |
| RP-250104 | Overview on Rel-20 RAN4-led 5G-Advanced Scope | Samsung | discussion | 15.3 |
| RP-250165 | View on RAN4 R20 | OPPO | other | 15.3 |
| RP-250209 | High level overview of 5G-A Rel-20 proposals - RAN4 | vivo | discussion | 15.3 |
| RP-250240 | Views on RAN4-led topics in Rel-20 | LG Electronics Deutschland | discussion | 15.3 |
| RP-250262 | Views on Rel-20 5G-A RAN4 led topics | CMCC | discussion | 15.3 |
| RP-250277 | Motivation on supporting Intra-band non-collocated EN-DC/NR-CA deployment in Rel-20 (Type 3 UE for handheld) | KDDI, NTT Docomo, SoftBank, KT, LG Uplus | discussion | 15.3 |
| RP-250284 | Views on Rel-20 RAN4-led topics | Huawei, HiSilicon | discussion | 15.3 |
| RP-250291 | Carrier Aggregation requirements for Ku band in 5G-Advanced | Eutelsat Group | discussion | 15.3 |
| RP-250321 | Apple’s view on Rel-20 RAN4 package | Apple | discussion | 15.3 |
| RP-250353 | Initial view for Rel-20 5G-A RAN4 scope | Xiaomi Technology | discussion | 15.3 |
| RP-250387 | Views on topics for RAN4 Rel-20 | China Telecom | discussion | 15.3 |
| RP-250410 | Views on the NTN Ku band related enhancement in Rel.20 | CHTTL | discussion | 15.3 |
| RP-250423 | Views on RAN4 Rel-20 OTA Topics | CAICT | discussion | 15.3 |
| RP-250430 | Views on Rel-20 5G-A RAN4 Topics | China Unicom | discussion | 15.3 |
| RP-250456 | (e)Redcap enhancement in Rel-20 | Sony | discussion | 15.3 |
| RP-250457 | Candidate RAN4 led NTN topics for Rel-20 5G Advanced | THALES | discussion | 15.3 |
| RP-250458 | Qualcomm Views on RAN4 Topics for Rel-20 | Qualcomm Incorporated | discussion | 15.3 |
| RP-250465 | Views on Rel-20 5G-Adv RAN4-led topics | Intel Corporation | discussion | 15.3 |
| RP-250483 | MediaTek views on Rel-20 RAN4 5G-A scope | MediaTek Inc. | discussion | 15.3 |
| RP-250518 | General views on Rel-20 RAN4 works | CATT, CSCN | discussion | 15.3 |
| RP-250530 | RedCap for Rel-20 | Futurewei | discussion | 15.3 |
| RP-250540 | 5G-Advanced RAN4 led topics in Rel-20 | Ericsson | discussion | 15.3 |
| RP-250542 | NTN in 5G-Advanced Rel-20 / Automotive Terminal / VSAT support | BMW AG, Fraunhofer IIS | discussion | 15.3 |
| RP-250566 | Views on Rel-20 RAN4-led topics | ZTE Corporation, Sanechips | discussion | 15.3 |
| RP-250569 | Motivation for low-band carrier aggregation via switching enhancement in Rel-20 | TELUS, Bell Mobility, Murata, AT&T, BT plc, Apple | discussion | 15.3 |
| RP-250638 | Priorities for RAN4-led 5G-A topics for Rel-20 | T-Mobile USA Inc. | discussion | 15.3 |
| RP-250648 | Motivation to specify RF requirements for ultra-small Mobile VSAT phased array antenna in Ku and Ka Band | SHARP Corporation | discussion | 15.3 |