3GPP TSG RAN Meeting #108	RP-251011
Prague, Czech Republic, June 9-13, 2025

Agenda item:	16.1

Source:	Reliance Jio, IIT-M, CEWiT, IIT-H, WiSig

Title:	Handling Remote Interference (Ducting / Tropospheric scattering) in 6G Studies

Document for:	Discussion

# 1	Introduction

Remote interference (RI) is a serious issue for TDD systems deployed in frequencies of 1.8 Ghz and above and typically refers to interference from remote base stations (gNBs) that can impact the performance of a local base station. This interference is caused by atmospheric ducting, where radio waves bend and propagate over long distances due to specific temperature and humidity conditions.

RI has been discussed in LTE and in 5G (Remote Interference Management - RIM) and the solutions so far has focused on RI detection, the mitigation part is yet to be fully accommodated in the specifications. Solutions such as Tilt management and power control can only go that far, they are also subject to implementation inefficiencies and are suboptimal. Subframe Blanking and other RIM mitigation solutions are also suboptimal. Also, the fact that RIM does not work when the interference is from other countries / operators makes the existing RIM solution suboptimal.

# 2	Discussion

During certain atmospheric conditions, layering of the air may form a waveguide in the troposphere, a phenomenon called tropospheric ducting. A tropospheric duct can last for anything between a few minutes to several hours. Radio signals that enter the duct can travel hundreds of kms with low propagation losses and hence cause significant interference far away from the transmitter. In TDD networks, sever degradation of coverage and connection success rate has been observed due to RI.

Tropospheric ducts, characterized by anomalous positive vertical refractive index gradients, significantly influence radio wave propagation by enhancing signal strength and extending communication ranges. While beneficial in some contexts, these phenomena can disrupt communication systems, particularly in maritime and coastal regions where ducting conditions are prevalent. The key challenges include accurately predicting and mitigating the effects of tropospheric ducts on radio propagation, which is critical for the reliability of next-generation communication systems.

Figure: Atmospheric Ducting (Source: Electronics desk)

3GPP Release 16 introduced a key feature for remote interference management (RIM) to provide solutions for remote uplink interference at base stations. RIM has been discussed in LTE and in 5G, but the discussions so far has focused on RI detection, the mitigation part is yet to be fully addressed in the specifications. Solutions such as Tilt management and power control can only go that far, they are also subject to implementation inefficiencies and are suboptimal. Subframe Blanking and other RIM mitigation solutions are also suboptimal. Also, the fact that RIM does not work when the interference is from other countries / operators makes the existing RIM solution suboptimal.

One effective way forward is to look at Interference cancellation (active) as the ducting holds for some time. This is non-trivial and could involve native support in the waveform design, possible physical layer enhancements to actively detect and mitigate the interference.

Support for Remote Interference cancellation would need native support in the waveform design and possible enhancements in the physical layer design. Noise due to Remote Interference is typically coloured, as such does not render itself for cancellation.

Also the issue of ducting is under discussion in ITU-R WP5D [2] with a focus on identifying a minimum set of parameters that operators / administrations who face interference can report. A standardized set of parameters that administrations should include to ensure timely identification, analysis, and resolution of such interference cases. Such parameters could also serve as the foundation for predictive modeling, technical assessment, and mitigation strategy formulation.

# 2	Conclusion

Proposal 1: The new RAT SI shall include study for handling Remote Interference Management which including detection across national / operator boundaries and possible active interference cancellation mechanisms supported by RI noise models.

Proposal 2: The new RAT SI shall study the possible use of AI mechanisms to accurately predict and mitigating the effects of tropospheric ducts on radio propagation.

# References

An overview of remote interference management for 5G, Ericsson Blog, ” available online: https://www.ericsson.com/en/blog/2019/9/overview-of-remote-interference-management

M.[IMT-TROPO DUCT MITIGATION] – MITIGATION OF INTERFERENCE FOR IMT NETWORK UNDER TROPOSPHERIC DUCTING EFFECT, ITU-R Draft Report