3GPP TSG RAN Meeting #108								RP-251014

Prague, Czech Republic, June 9-13, 2025

## Status Report to TSG

Agenda item:			9.3.1.1

| WI / SI Name | NR MIMO Phase 5 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_MIMO_Ph5 |  |  |  |  |
| Unique ID | 1020089 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-242394 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: n/a | Core part: 09/2025 | Performance part: 03/2026 | Testing part: n/a |  |
| Overall Completion level | Study Item: n/a | Core part: 95% | Performance Part: 10% | Testing part: n/a |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur (primary) | Name | Eko Onggosanusi |
|  | Company | Samsung |
|  | Email | eko.o@samsung.com |
| Rapporteur | Name | Darcy Tsai |
|  | Company | MediaTek |
|  | Email | darcy.tsai@mediatek.com |
| Rapporteur | Name | Chai Li |
|  | Company | China Mobile |
|  | Email | chaili@chinamobile.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

In RAN1#120bis, the following agreements were made.

UE-initiated/event-driven beam management

Agreement

Regarding triggering event determination for Event 2, on resetting the counting, the following modification on the agreed Candidate #2 in RAN1#120 is supported.

- Candidate#2: [The measured current beam RS is updated based on] indicated TCI state is updated;

  - In such case, the UE needs to reset the counting for all new beams.

  - FFS: Further details on the update (if necessary)

Agreement

On UE-initiated/event-driven beam reporting, support the following interpretation on each of the codepoints of the ‘1-bit’ condition met indicator as follows:

- ‘0’ – indicating that the Event-2 instances for corresponding CRI/SSBRI within the time window doesn’t reach the configured number M.

- ‘1’ – indicating that the Event-2 instances for corresponding CRI/SSBRI within the time window reach the configured number M.

FFS: whether/how to introduce the ‘1-bit’ condition met indicator for Event 7.

Agreement

On cross-CC beam report measurement for UE-initiated/event-driven beam reporting, regarding Event-2, introduce an RRC parameter to indicate ServCellIndex on which the indicated TCI state used to determine the current beam RS is applied

- FFS: Indication of the cell that corresponds to the configured first PUCCH resource in CSI report config including whether it is necessary

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, for the case the pre-configured Type-1 CG PUSCH carry the beam report, for the second UL channel in Mode-B, reuse the intra-UE multiplexing/prioritization rules of PUSCH with SP-CSI for Type-1 CG PUSCH with UEI-BR for Mode B

Agreement

Regarding triggering event determination for Event 2, on the measurement window for initiating the UE-initiated/event-driven beam reporting procedure, down-select one of the following options in RAN1#120bis.

- Option-1: The measurement window is from T_PUCCH – T_proc – T_window to T_PUCCH – T_proc, where T_PUCCH is a transmission occasion of a first PUCCH, and T_proc is RRC configured.

- Option-3: The length, slot offset and periodicity of a measurement window are configured per CSI report configuration by NW.

- Option-4: If an Event-2 instance for a new beam is obtained at the time t [and the time for the new beam is not running], UE (re)starts the timer for the new beam, where the expiry time of the timer is equal to the NW-configured length of the time window (T_window)

- Note: T_window is the agreed time window parameter for measurement.

Agreement

On UE-initiated/event-driven beam reporting, support the following for Event 1

- Regarding report format of L1-RSRP value of current beam,

  - Option-A2: L1-RSRP of current beam is absolute L1-RSRP (7-bit quantized, rather than 4-bit as in Event-2).

Agreement

On UE-initiated/event-driven beam reporting, regarding current beam report on Event 1, reuse RRC parameter enabledCurrentBeamReport-r19 to enable/disable the RSRP report of current beam.

Conclusion

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the case that one PUCCH resource of first PUCCH can be associated with one or multiple CSI report configurations, there is NO RAN1 consensus on supporting multiple UEI beam reports carrying in a second PUSCH.

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, support the following rule for the Case-4: the 1-bit first PUCCH is collided/overlapped with a PUCCH format 0/1 carrying HARQ.

- Option-1: Follow legacy SR multiplexing rule.

  - Note: There is no further enhancement on additional phase offset/constellation point(s).

Conclusion

Regarding resource mapping/configuration between first and second UL channel associated with a same CSI report configuration in Mode-B, there is NO consensus on further supporting the case that first PUCCH resource and pre-configured resource for second UL channel can have different periodicities (in ms).

Agreement

Regarding triggering event determination for Event 2, on the measurement window for initiating the UE-initiated/event-driven beam reporting procedure, support Option-4.

- Option-4: If an Event-2 instance for a new beam is obtained at the time t and the timer for the new beam is not running, UE starts the timer for the new beam, where the expiry time of the timer is equal to the NW-configured length of the time window (T_window)

  - Note: Timer is new beam specific.

- Note: T_window is the agreed time window parameter for measurement.

- Introduce separate UE capability to limit the number of timers. There is only 1 timer per new beam.

Above agreement is captured in RAN1 specifications.

Agreement

On UE-initiated/event-driven beam reporting, the procedure of triggering event determination is captured in RAN1 spec.

Conclusion

There is no RAN1 consensus on the following proposal:

On UE-initiated/event-driven beam reporting, support at least Option-1 of following for Event 7 as an extension on report format for Event-2

- Option-1: Additional field to indicate the codepoint of the activated TCI state with Q-th best quality.

  - FFS: Further report codepoints of other activated TCI state(s) [, e.g., from {Q+1}-th best quality to the worst one];

  - FFS: Details of the additional indication.

- Option-2: Extending the maximum number of reported RS(s) to 8.

  - [Note: If Option 2 is agreed, RAN1 can revisit the agreed maximum value of N for event-2.]

- Above is applicable only if time window and a configurable value M for event counting on Event-7 are not configured.

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting for a CSI report configuration, down-select at least one of the following candidates in RAN1#121:

- Candidate#1: To introduce a prohibit timer for mode-A and/or mode-B

  - Option-1A: In the case that triggering-event associated with the CSI report configuration is determined [by the same triggering beams(s) as the last transmitted PUCCH], if the prohibit timer is NOT running, the UE can transmit first PUCCH;

    - At the first symbol after the end of the PUSCH transmission, the UE starts the prohibit timer

  - Option-1B: In the case that triggering-event associated with the CSI report configuration is determined, if the prohibit timer is NOT running, the UE can transmit first PUCCH;

    - At the first symbol after the end of the first PUCCH transmission, the UE starts the prohibit timer

  - If the prohibit timer is running, the first PUCCH is not allowed to be transmitted.

- Candidate#2: To introduce a time interval for mode-A and/or mode-B

  - Option-2A: For a first PUCCH transmission occasion, if there is a transmission of another second PUSCH [corresponding to the CSI report determined by the same triggering beam(s) as the first PUCCH] within a configurable time interval before the first PUCCH transmission occasion, the UE should not transmit the first PUCCH.

  - Option-2B: For a first PUCCH transmission occasion, if there is a transmission of another first PUCCH(s) within a configurable time interval before the first PUCCH transmission occasion, the UE should not transmit the first PUCCH.

- Candidate#3: No further enhancement.

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, down-select one of the following rules for the Case-2: the 1-bit first PUCCH is collided/overlapped with a PUSCH, in RAN1#121

- Option-1: Prioritize first PUCCH over PUSCH, i.e., PUSCH is dropped.

  - FFS: If the PUSCH should be with UL-SCH or not for UEI beam report

- Option-3: Piggyback 1-bit indication of first PUCCH into the PUSCH.

  - FFS: The 1-bit indication is always multiplexed in the PUSCH, regardless that UEI beam report procedure is triggered.

  - FFS: If the PUSCH should be with UL-SCH or not for UEI beam report

- Option-4: Reuse the SR dropping rules.

- FFS: whether/how to handle the case of different PHY priorities.

Conclusion

There is no RAN1 concensus on the following. No spec change needed.

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, support one of the following rule for the Case-3: the 1-bit first PUCCHs corresponding to different CSI configuration for UE-initiated/event-driven beam reporting are overlapping in the time domain

- Option-1: Per CSI report configuration.

- Option-2: Up to implementation.

- Option-3: UE transmits the first PUCCH with higher priority (e.g., PUCCH resource with lower PUCCH resource ID, or event-type)

- Option-4: Multiplexing all 1-bit indications

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, confirming the following working assumption with the following modification.

- For Mode-A, the multiple CSI report configurations associated with the same PUCCH resource should be associated with a same CSI-AperiodicTriggerState.

- FFS: For Mode-A, the multiple CSI report configurations associated with the same CSI-AperiodicTriggerState should be associated with a same PUCCH resource.

Conclusion

There is no RAN1 concensus on the following. No spec change needed.

For event 2, when one PUCCH resource of first PUCCH can be associated with one or multiple CSI report configurations, and if multiple UE initiated beam report procedures occur, down-select one of the following options:

- Option-1: It is up to UE implementation to select one of configuration.

- Option-2: The UEI beam report with highest priority is reported

  - FFS: priority, e.g., lowest CSI-ReportConfigId, event-type or based on legacy CSI report priority rule

- Option-3: The report triggered in the latest measurement is reported in PUSCH

CSI

Agreement

For the Rel-19 Type-I multi-panel (MP) codebook refinement for a given P∈{48, 64, 128} CSI-RS ports, the mapping of i1,3 to (k1,k2) for RI=2-4 is the same as that used in the Rel-19 Type-I SP Scheme-A for RI=2-4 and N1>N2>1..

- Note: How this is captured is up to the TS38.214 editor.

Agreement

For the Rel-19 Type-I multi-panel (MP) codebook refinement for 48, 64, and 128 CSI-RS ports, support only the following (Ng, N1, N2) values (revision from the current version of editor draft CR on TS 38.214):

| Number of CSI-RS antenna ports, PCSI-RS | (Ng,N1,N2) |
| --- | --- |
| 48 | (2,4,3) |
|  | (2,6,2) |
|  | (2,12,1) |
| 64 | (2,4,4) |
|  | (2,8,2) |
|  | (2,16,1) |
|  | (4,4,2) |
|  | (4,8,1) |
| 128 | (4,4,4) |
|  | (4,8,2) |
|  | (4,16,1) |

Conclusion

For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, when the Rel-18 SD NES Type-I is configured for the Rel-19 Type-I SP codebook, the powerOffset parameter can be configured in all the respective subConfiguration IEs

- The supported values for powerOffset follow the legacy specification

Conclusion

There is no RAN1 consensus on the following proposal:

For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the dynamic range for delay offset reporting Dn,offset, i.e. AD, also support

AD = {0.1CP, 0.2CP}.

- Each of the above values of the dynamic ranges is subject to a separate UE capability.

- Each of the above values of the dynamic ranges is applicable when the bandwidth of configured TRS is no smaller than 52 PRBs

Conclusion

For the Rel-19 Type-I multi-panel (MP) codebook refinement for 48, 64, and 128 CSI-RS ports, there is no need to support any mapping from CSI-RS resource index and CSI-RS resource port index per resource to port index for CSI/PMI calculation

Conclusion

There is no consensus to support the following proposal:

- For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding aperiodic CSI-RS resource configuration, support configuring, via higher-layer (RRC) signalling, an available slot offset for each CSI-RS resource within the aperiodic CSI-RS resource set

Agreement

For the Rel-19 Type-I and Type-II (except for Type-II Doppler) codebook refinement for 48, 64, and 128 CSI-RS ports, when the K>1 aggregated NZP aperiodic CSI-RS resources for CMR are within two consecutive slots, the triggering offset of the associated aperiodic CSI-IM follows the associated NZP CSI-RS resource(s) for CMR in the first slot.

- FFS: How to resolve this for Rel-19 Type-II Doppler

3Tx uplink

Agreement

Adopt the following changes in Clause 6.3.1.5 in TS38.211.

#### 6.3.1.5 Precoding The block of vectors y0i…yυ-1iT shall be precoded according to z(p0)i⋮z(pρ-1)i=Wy(0)i⋮yυ-1i where i=0,1,…,Msymbap-1, Msymbap=Msymblayer. The set of antenna ports p0,…,pρ-1 shall be determined according to the procedure in [6, TS 38.214]. For non-codebook-based transmission, the precoding matrix W equals the identity matrix. For codebook-based transmission, the precoding matrix W depends on the number of antenna ports used for the transmission: - for single-layer transmission on a single antenna port, W=1; - for transmissions using 2, or 4 antenna ports, W is given by Tables 6.3.1.5-1 to 6.3.1.5-7; - for transmissions using 3 antenna ports when 4portSRS_3TX is configured, the UE does not transmit on antenna port 1003 and W is given by Tables 6.53.1.5-48 to 6.3.1.5-50;

Agreement

Adopt the following changes in Clause 6.2.1.2 in TS38.214.

#### 6.2.1.2 UE sounding procedure for DL CSI acquisition -------------------------------------------Unchanged parts are omitted------------------------------------------- - For 1T=1R, 2T=2R, 3T=3R, 4T=4R or 8T=8R, up to two SRS resource sets each with one SRS resource can be configured, where the number of SRS ports for each resource is equal to 1, 2, 4 with port 1003 disabled by higher layer parameter [RRC_parameter] is configured, 4 or 8 if the UE is not indicating srs-AntennaSwitching2SP-1Periodic or [srs-AntennaSwitching3T3R2SP-1Periodic]. Up to two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'semi-persistent' and one SRS resource set configured with resourceType in SRS-ResourceSet set to 'periodic' can be configured and the two SRS resource sets configured with 'semi-persistent' are not activated at the same time, or up to two SRS resource sets can be configured, if the UE is indicating srs-AntennaSwitching2SP-1Periodic, srs-AntennaSwitching8T8R2SP-1Periodic, or [srs-AntennaSwitching3T3R2SP-1Periodic], where each SRS resource set has one SRS resource, the number of SRS ports for each resource is equal to 1, 2, 4 with port 1003 disabled by higher layer parameter [RRC_parameter] is configured, 4, or 8 or -------------------------------------------Unchanged parts are omitted------------------------------------------- - For 3T6R, zero or one or two SRS resource sets configured with a different value of resourceType in SRS-ResourceSet set to 'periodic' or 'semi-persistent' and with higher layer parameter [RRC_parameter] is configured if the UE is not indicating [srs-AntennaSwitching2SP-1Periodic] or [srs-AntennaSwitching3T6R2SP-1Periodic], or up to two SRS resource sets configured with 'semi-persistent' and up to one SRS resource set configured with 'periodic' if the UE is indicating [srs-AntennaSwitching2SP-1Periodic] or [srs-AntennaSwitching3T6R2SP-1Periodic], where the two SRS resource sets configured with 'semi-persistent' are not activated at the same time. Each SRS resource set has two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a four SRS ports with port 1003 disabled, and the SRS ports of the resource in the set are associated with a different UE antenna ports, or - For 3T6R, zero or one or two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'aperiodic' and with higher layer parameter [RRC_parameter] is configured, where in the case of one resource set has two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a four SRS ports with port 1003 disabled, and the SRS ports of the resource in the set are associated with a different UE antenna ports. In the case of two resource sets a total of two SRS resources are transmitted in different symbols of two different slots, and where the SRS ports of each SRS resource in the given two sets are associated with a different UE antenna ports. -------------------------------------------Unchanged parts are omitted-------------------------------------------

Agreement

For a 3TX UE, when configured to ‘nonCodebook’, to distinguish UE behavior for PTRS-DMRS association between the Rel-19 and Rel-18 behavior,

- Do not introduce a new RRC parameter, but enhance the definition of the RRC parameter 4portSRS_3Tx, for example,

  - ‘1-bit RRC parameter to indicate that port 1003 is disabled for all the 4-port resources in an SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, or to indicate 3-port transmission is enabled for a configured SRS resource set with usage ‘nonCodebook’.

Agreement

Adopt the following changes in Clause 6.2.1 in TS38.214.

### 6.2.1 UE sounding procedure -------------------------------------------Unchanged parts are omitted------------------------------------------- [The UE does not transmit SRS on antenna port 1003 if [RRC_parameter] is configured.] where [RRC_parameter] is only applicable when SRS resource(s) are configured with 4 ports with usage as codebook or antenna switching. -------------------------------------------Unchanged parts are omitted-------------------------------------------

Agreement

Adopt the following changes in Clause 7.1 in TS38.213.

## 7.1 Physical uplink shared channel For a PUSCH transmission on active UL BWP b, as described in clause 12, of carrier f of serving cell c, a UE first calculates a linear value PPUSCH,b,f,c(i,j,qd,l) of the transmit power PPUSCH,b,f,c(i,j,qd,l), with parameters as defined in clause 7.1.1. For a PUSCH transmission scheduled by a DCI format other than DCI format 0_0, or configured by ConfiguredGrantConfig or semiPersistentOnPUSCH, if txConfig in PUSCH-Config is set to 'codebook', - if ul-FullPowerTransmission in PUSCH-Config is provided, the UE scales PPUSCH,b,f,c(i,j,qd,l) by s where: -------------------------------------------Unchanged parts are omitted------------------------------------------- - else, if each SRS resource in the SRS-ResourceSet with usage set to 'codebook' has more than one SRS port, the UE scales the linear value by the ratio of the number of antenna ports with a non-zero PUSCH transmission power to the maximum number of SRS ports supported by the UE in one SRS resource [if 4portSRS_3Tx is not provided, or to 3 if 4portSRS_3Tx is provided]. -------------------------------------------Unchanged parts are omitted-------------------------------------------

Asymmetric DL-sTRP/UL-mTRP

Agreement

Answer the question of RAN4 as follows:

- The RAN1 agreements in RAN1#118bis “One downlink reference timing is supported and applied to both TAGs” are NOT applied for the scenario that UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s).

Agreement

- For the scenario that the UE is configured with PL offset in joint/UL TCI state(s), when the UE is configured with rel-19 2 TAs, the UE maintains one single downlink reference timing

- For the scenario that the UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s), when the UE is configured with rel-19 2 TAs:

  - The UE maintains two downlink reference timings;

  - Baseline assumption for this feature is that Rx timing difference between two DL reference timings is no larger than one CP length, while it is subject to optional UE capability that the Rx timing difference between two DL reference timings can be assumed to be larger than CP length.

  - The reference point for PRACH transmission is indicated as follows:

    - if “PRACH association indicator” in DCI format 1_0 is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG.

    - if “PRACH association indicator” in DCI format 1_0 is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the second TAG

  - Above applies for the case UE is configured with SSB-MTC-additionalPCI

Agreement

For an SRS resource set not configured with followUnifiedTCI-StateSRS, regarding how to determine the PL offset and the CLPC adjustment state

- Alt1: The PL offset and CLPC adjustment state indicated by the TCI state configured to the SRS resource with lowest ID (i.e., reusing the rule specified in Rel-17) and no additional enhancement.

When the UE is configured with Rel-19 2 TA, the above design is also applied to TAG.

Agreement

Reply to RAN4 LS on DL timing reference to support two TA enhancement for asymmetric DL sTRP/ UL mTRP is agreed in R1-2503091.

Conclusion

There is no RAN1 consensus on the following proposal:

If the PL offset associated with the indicated joint/UL TCI state is included in the PRACH transmission power for PDCCH-order PRACH transmission, the pathlossReferenceRS-Id in the same indicated joint/UL TCI state is used to determine PL RS for the PDCCH-order PRACH transmission.

Companies are encouraged to consider the following for RAN1#121:

Regarding whether/how to reset the CLPC of PUSCH/PUCCH/SRS, further study and down-select one from the following alternatives:

- Updated Alt1: For a UE configured with 2 TAGs, when the UE receives a RAR of a PDCCH-order PRACH:

  - If PL offset is configured in joint/UL TCI state(s), according to the new 1-bit DCI field PL offset indication in DCI format 1_0:

    - When there is one indicated joint/UL TCI state in Rel-17 Unified TCI:

      - If the 1-bit DCI field is 0, the UE resets the CLPC that is different from the CLPC associated with the indicated joint/UL TCI state.

      - If the 1-bit DCI field is 1, the UE resets the CLPC associated with the indicated joint/UL TCI state.

    - When there are two indicated joint/UL TCI states in Rel-18 Unified TCI:

      - If the 1-bit DCI field is 0, the UE resets the CLPC associated with the first indicated joint/UL TCI state;

      - If the 1-bit DCI field is 1, the UE resets the CLPC associated with the second indicated joint/UL TCI state

  - If PL offset is not configured in joint/UL TCI state(s) and the UE is configured with SSB-MTC-AddtionalPCI, according to the PRACH association indicator in DCI format 1_0:

    - If the indicated joint/UL TCI state is associated with the same PCI as indicated by the PRACH association indicator, the UE resets the closed loop power control adjustment state associated with the indicated joint/UL TCI state.

    - If the indicated joint/UL TCI state is associated with a different PCI from the one indicated by the PRACH association indicator, the UE resets the closed loop power control adjustment state that is different from the closed loop power control adjustment state associated with the indicated joint/UL TCI state.

- Alt2: For a UE configured with two TAGs, one TAG ID is indicated in the RAR

  - The UE resets the closed loop power control adjustment state associated with a joint/UL TCI state associated with the indicated TAG.

- Alt3: Not introduce any enhancement.

In RAN1#121, the following agreements were made.

UE-initiated/event-driven beam management

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing a number of L (L>=1) PUCCH format 0/1 with UEIRIs are collided/overlapped with a PUCCH format 2/3/4 carrying HARQ/CSI, reuse the legacy SR multiplexing rule.

- The value of log2L+1 bits is based on an ascending order of the values of PUCCH resource ID associated with the first PUCCHs.

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing a number of L (L>=1) first PUCCH is collided/overlapped with a PUCCH format 2/3/4 carrying HARQ/CSI/SR.

- Option-1: Extend the SR field of  log2K+1 bits to a field of log2K+L+1 bits of representing at most one of positive SR or positive UEIRI

  - The value in the field is based the ascending order of SR ID first and then the ascending order of the PUCCH resource ID associated with the first PUCCHs.

  - If one of the SRs is a positive LRR, the value of the log2K+L+1 bits indicates the positive LRR, else, if one of the UEIBRs is a positive UEIBR, the value of the log2K+L+1 bits indicates the positive UEIBR.

  - An all-zero value for the log2K+L+1 bits represents a negative SR and UEIRI value across all K LRR/SRs and/or L UEIRIs.

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding occupied CPU(s), OCPU = 1 is occupied for a CSI report configuration.

- Note: That is the same number of occupied CPU for legacy L1-RSRP measurement.

Agreement

Regarding triggering event determination, on candidate values of supported RRC parameters,

- Support the following as RRC candidate values for a threshold value eventThreshold-r19 for trigger event detection regarding Event-2 or Event-7.

  - Option-2: 0, 1, …, 30, 31 dB

- Support the following as RRC candidate values for a threshold value eventThreshold-Event1-r19 for trigger event detection regarding Event-1.

  - Reusing RSRP-Range in RRC

    - Note: only values 14, …,113 in RSRP-Range are valid

- Support the following as RRC candidate values for the time window length for triggering event determination eventDetectionTimeWindowLength-r19

  - 4, 5, 8, 10, 16, 20, 40, 80, 160, 320, 640, 1280 ms

- Support the following as RRC candidate values for the counting threshold eventInstanceCount-r19

  - Option-1: 2, …, 16

Agreement

On cross-CC beam report measurement for UE-initiated/event-driven beam reporting, regarding Event-2, introduce an RRC parameter to indicate one from {SpCell, PUCCH-Scell} as the cell of the configured PUCCH.

- Note: It is up to NW implementation to configure same or different PUCCH resource IDs in different serving cells

Agreement

On cross-CC beam report measurement for UE-initiated/event-driven beam reporting, regarding the evaluation periodicity for determining event instance, the periodicity of the current beam RS should be the same as that of the new beam RS(s).

- The evaluation periodicity is the same as the periodicity of the current and new beam RS(s).

Above applies for all events.

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the value of X symbols for determining available transmission occasion of the second UL channel on Mode-B

- Support Option-1 of the following as RRC candidate values for X symbols

  - Option-1: 0, 1, 2, 4, 8, 16, 32, 64, 128, 256, 512

  - Note: X is based on the SCS of the first PUCCH.

  - Minimum value of X is subject to UE capability

- Regarding ‘available’ transmission occasion of the second UL channel, the transmission restriction for of the second UL channel reuses the legacy rule of PUSCH with SP-CSI.

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, support Option-3 of the following rules for the Case-2: the 1-bit first PUCCH is collided/overlapped with a PUSCH

- Option-3: Piggyback 1-bit indication of first PUCCH into the PUSCH.

  - The 1-bit indication is always multiplexed in the PUSCH, regardless that UEI beam report procedure is triggered.

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding CSI reference resource definition for a UEI beam report, the CSI reference resource for a CSI reporting is defined by a single downlink slot n-nCSI_ref-Koffset⋅2μDL2μKoffset, where slot n is determined according to the uplink slot n’ in which the second PUSCH is transmitted, and

- For mode-A, the legacy CSI reference definition of aperiodic CSI reporting is used.

- For mode-B, nCSI_ref is the smallest value greater than or equal to , such that slot n- nCSI_ref corresponds to a valid downlink slot, where Z' corresponds to the delay requirement as defined in Clause 5.4.

In the report, a condition met indicator for new beam RS is determined according to the measurement(s) triggering the first PUCCH transmission.

Note: Strong concern was raised by vivo on the necessity of the above proposal especially considering the RAN4 status.

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-1 and Event-7, one PUCCH resource of first PUCCH can be associated with one or multiple CSI report configurations.

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding priority rules for CSI report multiplexing/dropping, UEI beam report for both mode-A and mode-B is prioritized over Semi-persistent CSI reports on PUCCH and Periodic CSI reports on PUCCH

- UEI beam report for mode-A > Aperiodic CSI report > UEI beam report (for mode-B) > Semi-persistent CSI reports on PUSCH

Note-1: The intra-UE multiplexing/prioritization rules of PUSCH with A-CSI for PUSCH is reused for UEI-BR for Mode A.

Note-2: How to capture the above is up to Editor.

CSI

Agreement

For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when ReportQuantity is ‘cjtc-Dd’ (delay offset), add the following definition of the measurement quantity in TS38.215 as follows (analogous to the Rel-18 TDCP):

- For FR1, the reference point shall be the antenna connector of the UE.

Agreement

For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, when configured with port subset indication (from the Rel-18 SD NES Type-1), the actual number of ports (the number of 1s in the port-subsetIndicator bitmap) can correspond to a supported value of PCSI-RS for the Rel-19 Type-I SP codebook (i.e. either 48, 64, or 128), or to a supported value of PCSI-RS for the Rel-15 Type-I SP codebook (i.e., either 2, 4, 8, 12, 16, 24, 32), respectively

- The supported number of ports for port subset of the full PCSI-RS ports, is according to UE capability

Agreement

For the Rel-19 Type-II codebook refinement for 48, 64, and 128 CSI-RS ports based on the Rel-18 Type-II Doppler codebook, when the KDOPP CSI-RS resource groups are configured for aperiodic CMR, where each CSI-RS resource group comprises K>1 aggregated NZP aperiodic CSI-RS resources, the triggering offset of an associated aperiodic CSI-IM follows the associated NZP CSI-RS resource(s) for CMR in the first slot (in the time domain).

Agreement

For the Rel-19 Type-II codebook refinement based on Rel-18 Type-II Doppler for 48, 64, and 128 CSI-RS ports, for Capability 1, further clarify the previous agreement that OCPU=min⁡(Y1⋅KDOPP⋅P/32,8)

Conclusion

For the Rel-19 Type-I and Type-II (except for Type-II Doppler) codebook refinement for 48, 64, and 128 CSI-RS ports, when the K>1 aggregated NZP aperiodic CSI-RS resources for CMR are within two consecutive slots, following the legacy spec, the aperiodic triggering offset configured with the NZP CSI-RS resource set for interference measurement is the same as that of the associated NZP CSI-RS resource set for CMR.

Conclusion

For the Rel-19 Type-II codebook refinement for 48, 64, and 128 CSI-RS ports based on the Rel-18 Type-II Doppler codebook, when the KDOPP >1 NZP aperiodic CSI-RS resource groups for CMR are configured, following the legacy spec, the aperiodic triggering offset configured with the NZP CSI-RS resource set for interference measurement is the same as that of the associated NZP CSI-RS resource set for CMR.

Agreement

For the Rel-19 Type-I SP and Type-II codebook refinement for P=48, 64, and 128 CSI-RS ports with K>1 aggregated NZP aperiodic CSI-RS resources for CMR, to implement the previous agreements on the mapping from CSI-RS resource index/port index per resource and port index to CSI/PMI calculation

- In TS38.211, extend the enumeration of antenna port p=3000+p’ with p’=0 …P–1

- In TS38.214, remove the term ‘port index for CSI/PMI calculation’

3Tx uplink

Conclusion

For codebook-based PUSCH transmission by a 3TX UE, PUSCH scheduling by DCI format 0_3 is not supported.

- Agreement: Adopt the following TP in red text for TS 38.214 section 6.1.1.1.

-------------------------------------------Unchanged parts are omitted---------------------------------- For codebook based transmission with three antenna ports, the UE determines its codebook subsets based on TPMI(s) and upon the reception of higher layer parameter codebookSubset in pusch-Config for PUSCH associated with DCI format 0_1 or 0_3 and codebookSubsetDCI-0-2in pusch-Config for PUSCH associated with DCI format 0_2 which may be configured with 'nonCoherent'. -----------------------------------------Unchanged parts are omitted------------------------------------------

Asymmetric DL-sTRP/UL-mTRP

Conclusion

When the UE transmits a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the MAC CE command updating PL offset of TCI state(s), the updated PL offset(s) shall be applied starting from the first slot that is after n+3Nslotsubrfame,  μ where μ is the SCS configuration for the PUCCH.

Conclusion

Regarding whether/how to reset the CLPC of PUSCH/PUCCH/SRS, no further additional enhancement in Rel-19

#### 2.1.2	Remaining Open issues

UE-initiated/event-driven beam management:

- Maintenance on enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management

  - Maintenance on UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching

  - Maintenance on UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting

CSI:

- Maintenance on CSI support for up to 128 CSI-RS ports, targeting FR1

  - Maintenance on Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks

  - Maintenance on Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s)

  - Maintenance on extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design

  - Maintenance on SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook

    - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format

    - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI

- Maintenance on UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD

- Maintenance on inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH

3Tx uplink:

- Maintenance on non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource

Note: UL full power transmission mode 1 and 2 are not supported.

Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching.

Asymmetric DL-sTRP/UL-mTRP:

- Maintenance on enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2

  - Maintenance on two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP.

  - Maintenance on two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

## 2.2	RAN2

#### 2.2.1	Agreements

- RAN2#129bis

Agreements on Asymmetric DL sTRP/UL mTRP

No need to add a maximum number restriction of the TCI states indicated by the PL offset MAC CE.

RAN2 understand the PL offset update MAC CE is at least applicable to PUCCH, PUSCH, SRS, and PDCCH-order CFRA.

We will capture in a note to reflect the previous understanding ‘RAN2 understands that if a joint/UL TCI state is configured with a PL offset, PHR trigger is based on the PL change of the PL-RS associated to the joint/UL TCI, where the PL change takes into account the PL offset.’. FFS on exact wording.

From RAN2 point of view, UE applies the latest PL offset value received in RRC or MAC CE.

For 2TA in asymmetric DL sTRP/UL mTRP scenario with pathloss offset configured Rel-18 2TA operation is applied with the following RRC changes:

- •	remove the restriction that RRC field tag2 is configured only if coresetPoolIndex is configured with more than one value;

- •	a single n-TimingAdvanceoffset is configured, i.e., n-TimingAdvanceOffset2 is not configured for 2TA in asymmetric DL sTRP/UL mTRP scenario.

For PRACH transmission, PL offset is applicable only to PDCCH-order CFRA.

MAC impact

In Mode A of UE-initiated CSI reporting, the active time of a DRX operation includes the time after a new UCI for UE-initiated beam reporting is sent on first PUCCH.

Confirm the following RAN2 understandings:

      - The CG type-1 PUSCH carrying the beam report of Mode-B does not carry MAC PDU (i.e. UL-SCH).

      - The DG PUSCH carrying the beam report of Mode-A carries MAC PDU (i.e. UL-SCH) as legacy.

FFS if any other MAC impact for UL skipping

The UE continues to perform CSI measurements for the UEIBM procedure when the active BWP is the dormant BWP.

If the BWP in an SCell is a dormant BWP, the UE should not report mode-A beam measurement results. The UE cannot perform mode-B beam reporting on this BWP.

RAN2 understand the event evaluation and report triggering for UE-initiated beam report is captured by RAN1 spec.

RRC impact

enabledCurrentBeamReport-r19 is added as an optional need-R field.

Reuse resourcesForChannelMeasurement in CSI-ReportConfig. Clarify in the field description that for UEI BM, the new beam to be measured is either CSI-RS (nzp-CSI-RS-ResourceSetList) or SSB (csi-SSB-ResourceSetList).

ng-n1-n2-r19, cri-typeI-SinglePanel-ri-restriction-r19/cri-typeII-ri-restriction-r19 and cri-typeI-SinglePanel-CBSR-r19/cri-typeII-CBSR-r19 in the same way as corresponding legacy fields i.e.:

        - ng-n1-n2-r19 is defined in the same way as ng-n1-n2 in R15 typeI-multiPanel

        - cri-typeI-SinglePanel-ri-restriction-r19/cri-typeII-ri-restriction-r19 are defined in the same way as legacy RI restrictions

        - cri-typeI-SinglePanel-CBSR-r19/cri-typeII-CBSR-r19 are defined in the same way as n1-n2-codebookSubsetRestriction-r18.

mrSelectedResources is defined as a SEQUENCE structure containing two fields with integer values from one to eight.

delayOffsetCompensation can be located under CSI-AperiodicTriggerState and outside of CSI-AssociatedReportConfigInfo and that the parameter triggeringScheme is not needed.

Define numberofSubbandsPO as a list (with size up to the number of subbands) where each element is an integer value within the maximum size of a BWP.

The following is used the signaling of typeI-CBSR and typeII-CBSR and typeI-softScalingRank:

- (N1, N2) can be signaled as a separate parameter, and CBSR can be signaled as a CHOICE of (X1, X2) and a CHOICE of N1N2;

- RAN2#130

Agreements on MAC open issues

UL skipping is not applicable to mode-B type-1 CG event-triggered beam report (i.e., MAC PDU is not generated).

For Rel-16 UL skipping (enhancedSkipUplinkTxDynamic), the UCI for mode-A DG-based UE-initiated report follows the existing procedure (i.e., MAC PDU is generated), there is no MAC impact.

The existing rule is applied to handle the overlapping/prioritization between the PUSCH of mode-A UE-initiated report and SR/other PUSCH in MAC. No MAC impact is expected.

For Rel-15 UL skipping (skipUplinkTxDynamic is configured), same principle as legacy aperiodic CSI applies for multiplexing UCI of mode-A DG-based UE-initiated report in PUSCH. Exact MAC spec can be discussed further

Agreements on Asymmetric DL sTRP/UL mTRP

Introduce a new RRC parameter per BWP that explicitly enables the Rel-19 sDCI-mTRP 2TA, and clarify in FD of tag2 to include all cases where tag2 is configured that “it is optionally configured in a serving cell for mDCI mTRP 2TA if coresetPoolIndex for a BWP is configured with more than one value, and for sDCI mTRP 2TA if [the new parameter] is configured.”;

2TA operation is supported for Rel-19 single-DCI mTRP without the restriction that coresetPoolIndex needs to be configured with more than one value, and single-DCI mTRP 2TA is applied to both the scenarios that PL offset is configured and PL offset is not configured.

Regarding Rel-19 sDCI mTRP 2TA operation for the scenario PL offset is not configured (UE is configured with SSB-MTC-additionalPCI), RAN2 assumes both n-TimingAdvanceoffset and n-TimingAdvanceOffset2 are configured unless RAN1 has different agreement.

Rel-17/18 Unified TCI States A/D MAC CE is reused for asymmetric DL sTRP/UL mTRP deployment.

No MAC spec impact is expect, can confirm in the post meeting email discussion

RAN2 understands UE maintains the internal configuration for this element (including Need R) in case the parent element (element in elementsToAddList) is absent as legacy.

RAN2 to confirm that the PL offset value stored in the UE, i.e. in the internal UE configuration is not updated based on the MAC CE for PL update.

RAN2 to confirm that the PL offset value stored in the source gNB/anchor gNB, i.e. in the UE RRC AS configuration, is not updated based on the MAC CE for PL update.

It is up to network implementation whether to 1) apply the stored RRC configured PL offset value to the UE in case the associated tci-State (i.e. the parent parameter in tci-StatesToAddList) is absent, or 2) release the stored RRC configured PL offset value in case PL update is absent and the associated tci-State (i.e. the parent parameter in tci-StatesToAddList) is present, or 3) configure a new RRC value to the UE for the associated tci-State during the RRC resume procedure. No specification change is needed.

Need R is applied for PL offset.

Agreements on other aspects

When the TAT of the pTAG expires, UE releases PUCCH resource for mode-A/B UEI report and clears type-1 CG for mode-B UEI report. FFS for the case when the TAT expires on the sTAG.

As a baseline, the events defined for UEI BM should be captured as a new reportConfigType which can also include parameters applicable only for one event. This can be captured once more parameters for this objective are defined by RAN1.

#### 2.2.2	Remaining Open issues

MAC:

- Coexistence with LTM and details (e.g. PL offset, etc.)

- MAC procedure details.

RRC:

- RRC signaling design based on RAN1 RRC parameter list

## 2.3	RAN3

n/a

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

In RAN4#114-bis (2025-04), the following agreements were made.

RF

- RAN4 concluded there is no consensus for PC1.5 3Tx in RAN4 in Rel-19

- It is agreed to reuse 2Tx PC2 MPR for 3Tx

- It is agreed to introduce the beam correspondence requirement when the IE unifiedTCI-StateType is configured as ‘separate’ and different reference signals for QCL-TypeD are configured across the TCI states

RRM

Issue 2-1: Spec structure and work split of Rel-19 MIMO Phase 5

Agreement on CR split

UE-initiated/event-driven beam management

Issue 1-1: How to define measurement period in Event triggered measurement reporting delay for the basic feature (M instance = 1, Event-2)?

Agreement:

  - For T_RS (TSSB or TCSI-RS) in L1_measurement periods

    - T_RS (TSSB or TCSI-RS):

      - the value of current beam and new beams are the same.

  - For M in L1_measurement periods:

    - If timeRestrictionForChannelMeasurement configured, M= 1, event evaluation should be based on the most recent result from both new beam and current beam

    - If timeRestrictionForChannelMeasurement is not configured,

      - Further discuss on:

        - Option1: Reuse the same values of legacy, measurement period is based on 3 sample. No further restriction of UE implantation of which one is used

        - Option 2: No requirements even the NW can configure it.

        - Option 3: FFS

  - For P (sharing factor) in L1_measurement periods:

    - For SSB based or CSI-RS based,

      - The definition is the same as legacy

      - The value can be the same or different for the current beam and new beam

Issue 1-2: Components of Event triggered measurement reporting delay?

Agreement:

- The event triggered measurement reporting delay includes the Tfirst UL channel

  - Tfirst UL channel is from the time point at RS which triggered the L1 reporting to the time point at next first PUCCH

Further discuss the two options and select one option in the next meeting:

- The event triggered measurement reporting delay is TL1_measurement period + Tfirst UL channel

  - Option 1: if TL1_measurement period is different for current beam and new beam, the TL1_measurement period is maximum of the L1 measurement periods to measure both current beam and new beams. (Samsung, Xiaomi, Apple, MTK)

  - Option 2: If the event is triggered by the current beam measurement, the event triggered measurement delay is TL1_measurement periods_currentbeam. If the event is triggered by the new beam measurement, the event triggered measurement delay is TL1_measurement periods_newbeam.. (E///, Nokia, QC)

Issue 1-4: Whether/how to specify the UE behaviour if new L1-RSRP during the time period between first PUCCH and second PUSCH?

Agreement:

- No further discussion on this issue in RAN4.

CJT calibration (CJTC) reporting

Issue 1-1: On the simulation assumption for delay and frequency offset for CJT calibration

Agreement:

- Update the report resolution to (MD = 256, MFO = 256).

Issue 1-2: How to define RRM core requirements for delay and frequency offset reporting based on the simulation results for CJT calibration?

Agreement:

- Define core requirements for delay and frequency offset reporting for

  - Measurement period (FFS number of samples)

  - Scheduling restriction (if identified)

Asymmetric DL-sTRP/UL-mTRP

Issue 2-1: For Scenario#2, how to update the RRM requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP?

Agreement:

Wait for RAN1 reply to the LS on whether the existing agreement for a single downlink timing reference is extended to Scenario#2.

Issue 2-4: For Scenario#1 in FR2, whether to update the RRM requirements for 2TA enhancement for FR2?

Agreement:

- RAN4 to define timing requirements for FR1 as well as FR2.

Issue 2-5: Whether to define UL TCI state activation requirement based on SRS?

Agreement: No RRM impacts on TCI state switching requirement.

In RAN4#115 (2025-05), the following agreements were made.

RF

- 3-layer with single band is supported from Rel-19

- Follow the last RAN4 conclusion “No 3Tx PC1.5 requirements will be defined in RAN4 in Rel-19”

- CR to introduce FR1 3Tx RF requirements is agreed in R4-2507961 without remaining issues

RRM

UE-initiated/event-driven beam management

Draft CRs for UE-initiated/event-driven beam management are endorsed.

Issue 1-1: How to define measurement period in Event triggered measurement reporting delay for the basic feature (M instance = 1, Event-2)?

Agreement:

- RAN4 define different separate for measurement period for the UE is configured or not for “timeRestrictionForChannelMeasurement”

- To check with RAN1 whether the parameter can be configured if RAN1 identify any issue

Issue 1-2: Components of Event triggered measurement reporting delay?

Agreement:

- If the measurement periods are different for current beam and new beam, the TL1_measurement period is not larger than the maximum of the L1 measurement periods to measure both current beam and new beams.

  - Note: The agreement does not imply that all measurements on individual reference signals are after the event is met over air.

  - UE shall perform L1-RSRP measurement based on current beam and new beams L1-RSRP measurement period by assuming TReport =0.

Issue 1-4: CSI-RS based L1-RSRP for inter-cell BM?

Agreement:

- Reuse the same requirements structure as that in Rel-17 clause 9.5. No need to add sharing factor for serving cell CSI-RS and NSC CSI-RS. From the configuration of CSI-RS perspective, there is only one configuration. If this CSI-RS is QCLed to the SSB of serving cell or SSB of additionPCI cell, they use the same set of requirements.

Issue 1-5: Whether to specify new unified TCI switching delay requirements?

Agreement:

- The existing requirements for TCI state switch delay are applicable for UE-initiated/even-triggered beam management

  - Further discuss additional enhancements in the next meeting, and the discussion does not impact the WI core part completion.

Issue 1-7: Requirements for event-1 and event-7?

Agreement:

- For Event-1 and Even-7, reuse the same design as the requirements defined Event-2

Issue 2-1-1: Measurement accuracy

Agreement:

Confirm previous agreement in RAN4#114. No further discuss is needed.

CJT calibration (CJTC) reporting

Issue 1-1: How to define measurement period requirements for delay and frequency offset reporting for CJT calibration?

Agreement:

- RAN4 to discuss the achievable accuracy in terms of delay and frequency offsets based on 1 sample and [3 or 5] samples in the next meeting.

- RAN4 to conclude the delay requirement for delay and frequency offset reporting in the next meeting. The delay requirement for phase offset is deprioritized in Rel-19.

Asymmetric DL-sTRP/UL-mTRP

Draft CRs for Asymmetric DL-sTRP/UL-mTRP are endorsed.

Issue 2-4: For Scenario#2, how to update PRACH and PUCCH/PUSCH/SRS transmission requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP?

Agreement:

For Scenario#2, inter-cell operation, PRACH reference point:

  - reference point for PRACH transmission is

    - if “PRACH association indicator” in DCI format 1_0 is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG.

    - if “PRACH association indicator” in DCI format 1_0 is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL [or UL] TCI state(s) of the reference cell associated with the second TAG

For inter-cell multi-TRP, for PUCCH/PUSCH/SRS:

    - For each TAG, the reference point for PUCCH/PUSCH/SRS, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of the reference cell associated with one of the DLorJointTCIState [or UL TCI states] having the same TAG as the uplink signal. For each TAG the uplink transmission timing takes place (NTA + NTA offset) x Tc before the reception of the downlink reference point.

Issue 2-5: For Scenario#2, how to update gradual timing requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP?

Agreement:

- Autonomous timing adjustment applies to both TAGs or TRPs.

- Two independent gradual timing adjustments with potentially different values should be applied to both TAGs.

Issue 2-6: For Scenario#2, if the UE is configured with SSB-MTC-additionalPCI, how to define MRTD and MTTD requirements?

Agreement:

- Reuse Rel 18 values of MRTD and MTTD for Rel 19.

Issue 2-7: Whether to define UL TCI state activation requirement for Scenario#2?

Agreement:

The existing requirements for unified TCI state with mTRP are applicable to Scenario#2 and no new requirements to be introduced. FFS on spec impact.

Demod

WF approved as R4-2508627.zip

UE-initiated/event-driven beam management

Issue 1-1-1: Clarify if UE demodulation/CSI reporting requirements are needed for UE-initiated/event-driven beam management

Agreement:

-	RAN4 do not introduce UE demodulation/CSI reporting requirements for UE-initiated/event-driven beam management.

CSI

General scope:

Issue 1-1-3: Clarify if PDSCH/PDCCH/PBCH/SDR demodulation requirements are needed for CSI enhancement up to 128 CSI-RS ports

Agreement:

-	RAN4 do not introduce PDSCH/PDCCH/PBCH/SDR demodulation requirements for CSI enhancement up to 128 CSI-RS ports.

Issue 1-1-4: Clarify if CQI/RI reporting requirements are needed for CSI enhancement up to 128 CSI-RS ports

Agreement:

-	RAN4 do not introduce CQI/RI reporting requirements for CSI enhancement up to 128 CSI-RS ports.

Issue 1-1-5: Clarify if CRI reporting requirements are needed for extension of CRI-based CSI reporting

Agreement:

-	RAN4 do not introduce CRI reporting requirements for extension of CRI-based CSI reporting.

Issue 1-1-6: Clarify if PMI reporting requirements are needed for Type-I codebook refinement up to 128 ports

Agreement:

-	RAN4 introduce PMI reporting requirements for typeI-SinglePanel-r19.

Sub-topic 1-2 Test assumptions for typeI-SinglePanel-r19:

Issue 1-2-1: Test metric

Agreement:

-	Reuse the test metric for Type-I PMI reporting requirements as the starting point, throughput ratio between the following PMI and random Type-I PMI, where the same Type-I codebook scheme is used for both follow PMI and random PMI throughput measurements.

Issue 1-2-2: Duplex mode

Agreement:

-	Both FDD and TDD

Issue 1-2-3: Number of Rx antennas

Agreement:

-	Both 2Rx and 4Rx

Issue 1-2-4: Propagation channel

Agreement:

-	Use TDLA30-5 as the starting point.

Issue 1-2-5: Correlation configuration

Agreement:

-	Extend the antenna correlation matrix models for X-pol arrays in TS 38.101-4 to support greater than 32 ports in the following manner, capture it in NR_MIMO_demod related CR.

- For the first dimension (N1), define the vector r1: - r₁(k) = αk - 1N1- 12, for k = 1, 2, ..., N₁ - RgNBDim,1 = Toeplitz(r₁) - where Toeplitz(r₁) constructs an N1×N1 Toeplitz matrix from the vector r1. - For the second dimension (N2), define the vector r2: - r₂(k) = αk - 1N2- 12, for k = 1, 2, ..., N₂ - RgNBDim,2 = Toeplitzr2 - If N2=1, RgNBDim,2=1. - Then the correlation matrix at the gNB is RgNB=RgNBDim,1⨂ RgNBDim,2 - The correlation matrix at the UE side assumes a 1D cross-polar array: - rue(k) = βk - 1N- 12, for k = 1, 2, ..., N - RUE = Toeplitz(rue) - The calculation of Rspat, Rhigh, and Rmedium remains the same as currently specified in TS 38.101-4. - The values of α, β, and the scaling factor a for specific correlation levels and antenna configurations are FFS.

-	FFS on the correlation configuration.

Issue 1-2-6: typeI-SinglePanel-ri-restriction-r19

Agreement:

-	Rank 2

Issue 1-2-7: Number of CSI-RS ports

Agreement:

-	Agree on to define requirement for 64 ports. Not to define requirements for 48 ports. FFS on 128 ports.

Issue 1-2-9: N1 and N2 configuration for 128 CSI-RS ports (if introduced)

Agreement:

-	(16, 4)

Issue 1-2-10: (Number of aggregated NZP CSI-RS resources) K for 64 CSI-RS ports

Agreement:

-	K=2

Issue 1-2-11: (Number of aggregated NZP CSI-RS resources) K for 128 CSI-RS ports (if introduced)

Agreement:

-	K=4

Issue 1-2-12: typeI-codebookMode-r19

Agreement:

-	Scheme A

Issue 1-2-13: portMappingMethod

Agreement:

-	Select one for requirements. FFS on which one next meeting.

Issue 1-2-14: Number of slots for CSI-RS transmission

Agreement:

-	One slot for 64 ports. FFS for 128 ports.

Issue 1-2-16: MCS level

Agreement:

-	MCS20 (Table 1) as the starting point

Issue 1-2-17: Other parameters

Agreement:

-	Reuse the setting for Single PMI with 32TX TypeI-SinglePanel Codebook as the starting point.

Asymmetric DL-sTRP/UL-mTRP

Issue 1-1-2: Clarify if UE demodulation/CSI reporting requirements are needed for asymmetric DL sTRP/UL mTRP

Agreement:

-	RAN4 do not introduce UE demodulation/CSI reporting requirements for asymmetric DL sTRP/UL mTRP deployment.

#### 2.4.2	Remaining Open issues

RF

- Whether and how to introduce beam correspondence requirements for different UL and DL TCI states in FR2

RRM

- Whether RAN4 to specify requirements for no time window based or cover both no time window based and with time window based with eventInstanceCount-r19

- FFS on the achievable accuracy in terms of delay and frequency offsets based on 1 sample and [3 or 5] samples for delay and frequency offset reporting for CJT calibration

- How to define scheduling restriction/measurement restriction requirements for delay and frequency offset reporting for CJT calibration

- FFS on PRACH timing requirements and gradual timing requirements for 2 TA enhancement for asymmetric DL sTRP/UL mTRP for scenario#1

- FFS on Test cases for RRM tests for performance part.

Demod

FFS on if PMI reporting requirements are needed for eTypeII-r19

FFS on if PMI reporting requirements are needed for typeII-Doppler-r19

FFS on if UE demodulation/CSI reporting requirements are needed for SRS port grouping

FFS on if UE demodulation requirements are needed for UE reporting enhancement for CJT

FFS on if CSI reporting requirements are needed for UE reporting enhancements for CJT

FFS on if demodulation requirements are needed for 3-antenna-port codebook-based UL transmissions

FFS on the correlation configuration for typeI-SinglePanel-ri-restriction-r19

FFS on the N1 and N2 configuration for 64 CSI-RS ports for typeI-SinglePanel-ri-restriction-r19

FFS on the CQI/RI/PMI delay for typeI-SinglePanel-ri-restriction-r19

FFS on Test assumptions for eTypeII-r19 (if introduced)

FFS on Test assumptions for typeII-Doppler-r19 (if introduced)

FFS on Test assumptions for CJT calibration (if introduced)

FFS on Test assumptions for PUSCH demodulation requirements (if introduced)

## 2.5	RAN5

n/a

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

n/a

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

n/a

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template