3GPP TSG-RAN Meeting #108	RP-251027

Prague, Czech Republic, 9–13 June, 2025

Agenda item:	15.2.1

Source:	Samsung

Title:	Views on the scope of Rel-20 AI/ML for Mobility

Document for:	Discussion and Decision

Introduction

RAN discussed AI/ML for Mobility as a candidate of Rel-20 5G-Adv items, and then concluded as captured in RP-250803 [1]:

Conclusions on AI/Mobility Only the following objectives are considered as baseline: - Specify support for RRM measurement prediction for both UE and network sided models [RAN2]: - Specify signalling and protocol aspect to enable reporting inference outcome of UE sided model for RRM measurement prediction - Specify signalling and protocol aspect to enable reporting information from UE for inference operation of network sided model for RRM measurement prediction - At least the use cases studied so far in RAN2 SI are supported: Temporal domain prediction and Frequency domain prediction. FFS Spatial domain pending RAN2 Study progress. FFS Cell level vs beam level prediction NOTE: network sided models can be concluded in RAN#108 after RAN2 discussions - Specify support for measurement event prediction for UE sided models [RAN2]: - Specify signalling and protocol aspect to enable reporting based on inference outcome of UE sided model for measurement event prediction - For both RRM measurement prediction and measurement event prediction [RAN2]: - Specify signalling and protocol to facilitate functionality-based LCM procedures - To be based on the LCM framework for Rel-19 AI/ML for NR air interface WI as much as possible - For both RRM measurement prediction and measurement event prediction: [RAN4] - Specify core requirements identified in study item phase - Specify LCM-related requirements, if any TU estimation from moderator: 1.5 TU for RAN2, [0.5-1] TU for RAN4 (performance part to be confirmed with WG), RAN3 (no impact to RAN3 at this point) RLF/HOF, CHO and LTM enhancements are not considered in the initial Rel-20 AI/ML mobility WI objective

As highlighted in yellow above, there are several FFS points, so we share our views on these.

Discussion

Regarding spatial domain prediction, first of all, the evaluation was not sufficiently conducted in RAN2. In addition, when comparing this with the beam prediction case, which was addressed in the RAN1-led Rel-19 AI/ML for Air, there appears to be no significant technical differences. Additionally, due to the constrained TUs available during Rel-20 timeframe, it is necessary to adjust the scope to a more reasonable level. Given these considerations, it seems reasonable to conclude that including spatial domain prediction within the WI scope may not be essential or necessary.

Proposal 1:	Spatial domain prediction is not part of the scope of the WI.

Regarding cell level vs. beam level prediction, cell-level prediction should serve as the baseline from a mobility perspective. In the case of beam-level prediction, we understand that the primary objective of the beam-level prediction is mainly to optimize the dedicated resources, rather than mobility performance. That is, it is to enable efficient dedicated RACH resource allocation–instead of allocating dedicated RACH resources for more beams–during handovers by optimizing beam prediction to allocate resources exclusively for specific candidate beams. However, having such beam-level measurement results from UE would lead to additional wastage of resources, which contradicts the original intention of the beam-level prediction.

Proposal 2:	Cell-level prediction is supported for RRM measurement prediction in the WI.

From the RAN2 SI, two temporal domain prediction cases were identified. Case A is primarily focused on future prediction, while Case B is mainly aimed at skipping intermediate measurements, as shown the following figures.

Figure 2-1: Example of intra-frequency temporal domain case A from the draft TR 38.744 [2]

Figure 2-2: Skipping pattern example 2 of intra-frequency temporal domain case B from the draft TR 38.744 [2]

For temporal domain Case A, both UE-side and NW-side models are feasible. However, for temporal domain Case B, only the UE-side model should be supported in Rel-20. Implementing NW-side inference for Case B would require the UE to periodically report SSB measurement results to the gNB—a task that was not previously necessary. This continuous reporting process would significantly increase UE power consumption, as it involves additional measurements and transmissions that were not required in earlier scenarios. Unless we are targeting the reduction of RS by sacrificing UE power consumption which is very important factor from user experience perspective, the scope for Case B should be limited to UE-side model only.

Proposal 3: For Case A, both UE-side and NW-side models are supported in the WI. For Case B, only UE-side model is supported in the WI.

If they are agreeable the objectives can be modified as follows:

Conclusions on AI/Mobility Only the following objectives are considered as baseline: - Specify support for cell-level RRM measurement prediction for both UE and network sided models [RAN2]: - Specify signalling and protocol aspect to enable reporting inference outcome of UE sided model for RRM measurement prediction for both temporal domain cases A and B and frequency domain prediction - Specify signalling and protocol aspect to enable reporting information from UE for inference operation of network sided model for RRM measurement prediction for temporal domain case A and frequency domain prediction - Specify support for measurement event prediction for UE sided models [RAN2]: - Specify signalling and protocol aspect to enable reporting based on inference outcome of UE sided model for measurement event prediction - For both RRM measurement prediction and measurement event prediction [RAN2]: - Specify signalling and protocol to facilitate functionality-based LCM procedures - To be based on the LCM framework for Rel-19 AI/ML for NR air interface WI as much as possible - For both RRM measurement prediction and measurement event prediction: [RAN4] - Specify core requirements identified in study item phase - Specify LCM-related requirements, if any TU estimation from moderator: 1.5 TU for RAN2, [0.5-1] TU for RAN4 (performance part to be confirmed with WG), RAN3 (no impact to RAN3 at this point) RLF/HOF, CHO and LTM enhancements are not considered in the initial Rel-20 AI/ML mobility WI objective

Detailed objectives

Proposal 1:	Spatial domain prediction is not part of the scope of the WI.

Proposal 2:	Cell-level prediction is supported for RRM measurement prediction in the WI.

Proposal 3: For Case A, both UE-side and NW-side models are supported in the WI. For Case B, only UE-side model is supported in the WI.

Proposal 4: Update the objectives for the WI as in clause 2 above.

Reference

RP-250803, Moderator's summary for RAN2 led REL-20 topics, RAN2 Chair (InterDigital), March 2025

R2-2503541	Text proposal of TR 38.744	OPPO	pCR	Rel-19	38.744	0.0.8	FS_NR_AIML_Mob