3GPP TSG-RAN5 Meeting #107	R5-252214

# St Julian’s, Malta, 19th - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1595 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of annex A.2.4 and annex A.2.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2025-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As proposed in the RAN5-endorsed discussion document R5-250355 (Proposal 2.1.4-1) the specification of the Route header field in PRACK (A.2.4) and UPDATE (A.2.5) shall be reworded. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Wording changed for Route header field of PRACK (A.2.4) and UPDATE (A.2.5) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.2.4, A.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | no TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modification>

## A.2.4	PRACK

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Request-Line |  |  |  | RFC 3261 [15] |
| Method |  | PRACK |  |  |
| Request-URI |  | same URI value as the recipient of PRACK has earlier sent in its Contact header within the same dialog |  |  |
| SIP-Version |  | SIP/2.0 |  |  |
| Via |  |  |  | RFC 3261 [15] |
| sent-protocol |  | SIP/2.0/UDP (when using UDP) or SIP/2.0/TCP (when using TCP) |  |  |
| sent-by |  | same value as in preceding INVITE message |  |  |
| via-branch |  | value starting with ‘z9hG4bK’ (NOTE 1) |  |  |
| Route |  | A1,A2 |  |  |  |  |  | RFC 3261 [15] |  |
| route-param |  |  |  | MO call (dialog creating INVITE sent by the UE): URIs of the Record-Route header field as sent to the UE in the 183 Session Progress in reverse order (or in any other response creating the dialog according to RFC 3261 clause 12.1 [15]). MT call (dialog creating INVITE sent by the SS): URIs of the Route header field as sent to the UE in the dialog creating INVITE. |  |  |  |  |  |
| From |  |  |  | RFC 3261 [15] |
| addr-spec |  | SIP URI of the UE when PRACK is sent by the UE, but SIP URI of the SS when PRACK is sent by the SS. URI must be the same as used for the endpoint in the earlier requests within the dialog. |  |  |
| tag |  | local tag of the dialog ID |  |  |
| To |  |  |  | RFC 3261 [15] |
| addr-spec |  | SIP URI of the SS when PRACK is sent by the UE, but SIP URI of the UE when PRACK is sent by the SS. URI must be the same as used for the endpoint in the earlier requests within the dialog. |  |  |
| tag |  | remote tag of the dialog ID |  |  |
| Call-ID |  |  |  | RFC 3261 [15] |
| callid |  | same value as received in INVITE message |  |  |
| CSeq |  |  |  | RFC 3261 [15] |
| value |  | value of CSeq sent by the endpoint within its previous request in the same dialog but increased by one |  |  |
| method |  | PRACK |  |  |
| Max-Forwards |  |  |  | RFC 3261 [15] |
| value |  | non-zero value |  |  |
| RAck |  |  |  | RFC 3262 [33] |
| response-num |  | same value as in RSeq header of the reliable response |  |  |
| cseq-num |  | same value as in CSeq of the reliable response |  |  |
| method |  | same value as in CSeq of the reliable response |  |  |
| P-Access-Network-Info | A1,A2(o) |  |  | RFC 7315 [132] RFC 7913 [154] |
| access-net-spec | A6 | access network information and, if applicable, the cell ID |  |  |
|  | A7 | access network information for NR, containing access-class parameter with value "3GPP-NR" or access-type parameter with value "3GPP-NR-FDD" or "3GPP-NR-TDD", and also containing the cell ID | Rel-15 |  |
| Proxy-Authorization | A5 |  |  | RFC 2617 [16] RFC 3310 [17] |
| username | A5 | preconfigured in the UE |  |  |
| realm | A5 | same value as received in the realm directive in the Proxy-Authorization header sent by SS |  |  |
| nonce | A5 | same value as in Proxy-Authorization header sent by SS |  |  |
| digest-uri | A5 | preconfigured in the UE |  |  |
| qop-value | A5 | auth |  |  |
| cnonce-value | A5 | value assigned by UE affecting the response calculation |  |  |
| nonce-count | A5 | counter to indicate how many times UE has sent the same value of nonce within successive INVITEs, initial value shall be 1 |  |  |
| response | A5 | response calculated by UE |  |  |
| algorithm | A5 | MD5 |  |  |
| Content-Type |  | header shall be present only if there is SDP in Message-body |  | RFC 3261 [15] |
| media-type |  | application/sdp |  |  |
| Content-Length |  |  |  | RFC 3261 [15] |
| value |  | length of Message-body |  |  |
| Message-body |  | Optional SDP body. If included then the contents of the SDP shall be checked as described in the Test requirements section of the test case. |  | RFC 4566 [27] RFC 3264 [30] RFC 3312 [31] |

| Condition | Explanation |
| --- | --- |
| A1 | PRACK sent by the UE (IMS security, A.6a/2 3GPP TS 34.229-2 [5]) (NOTE 2) |
| A2 | PRACK sent by the UE (GIBA, A.6a/1 3GPP TS 34.229-2 [5]) (NOTE 2) |
| A3 | PRACK sent by the SS (IMS security, A.6a/2 3GPP TS 34.229-2 [5]) (NOTE 2) |
| A4 | PRACK sent by the SS (GIBA, A.6a/1 3GPP TS 34.229-2 [5]) (NOTE 2) |
| A5 | SIP Digest without TLS for Fixed Broadband Access (SIP Digest without TLS, A.6a/5 3GPP TS 34.229-2 [5]) |
| A6 | UE uses E-UTRAN access (A.18/1 3GPP TS 34.229-2 [5]) |
| A7 | UE uses NR access (A.18/5 3GPP TS 34.229-2 [5]) |

NOTE 1:	Branch parameter values sent by SS are different within a test case execution.

NOTE 2:	When the UE supports IMS security then condition A1 or A3 is applied unless condition A2 or A4 is specified explicitly by the test case.

<End of modification>

<Start of modification>

## A.2.5	UPDATE

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Request-Line |  |  |  | RFC 3261 [15] |
| Method |  | UPDATE |  |  |
| Request-URI |  | same URI value as the recipient of UPDATE has earlier sent in its Contact header within the same dialog |  |  |
| SIP-Version |  | SIP/2.0 |  |  |
| Via |  |  |  | RFC 3261 [15] |
| sent-protocol |  | SIP/2.0/UDP (when using UDP) or SIP/2.0/TCP (when using TCP) |  |  |
| sent-by | A1,A2 | MO call: same value as in INVITE message MT call: as defined in A.2.1 |  |  |
|  | A3,A4 | as defined in A.2.9 (There is more than one value) |  |  |
| via-branch |  | value starting with ‘z9hG4bK’ (NOTE 2) |  |  |
| Route | A1,A2 |  |  | RFC 3261 [15] |
| route-param |  | MO call (dialog creating INVITE sent by the UE): URIs of the Record-Route header field as sent to the UE in the 183 Session Progress in reverse order (or in any other response creating the dialog according to RFC 3261 clause 12.1 [15]). MT call (dialog creating INVITE sent by the SS): URIs of the Route header field as sent to the UE in the dialog creating INVITE. |  |  |
| From |  |  |  |  |
| addr-spec |  | SIP URI of the UE when UPDATE is sent by the UE, but SIP URI of the SS when UPDATE is sent by the SS. URI must be the same as used for the endpoint in the earlier requests within the dialog. |  | RFC 3261 [15] |
| tag |  | local tag of the dialog ID |  |  |
| To |  |  |  | RFC 3261 [15] |
| addr-spec |  | SIP URI of the SS when UPDATE is sent by the UE, but SIP URI of the UE when UPDATE is sent by the SS. URI must be the same as used for the endpoint in the earlier requests within the dialog. |  |  |
| tag |  | remote tag of the dialog ID |  |  |
| Call-ID |  |  |  | RFC 3261 [15] |
| callid |  | same value as received in INVITE message |  |  |
| Contact | A1,A2 | Must be present, value not checked |  | RFC 3261 [15] RFC 3311 [137] |
|  | A3,A4 | MO call: same Contact header as sent by SS in this dialog (e.g. in a 183 or 180 response) MT call: same value as used in MT INVITE |  |  |
| CSeq |  |  |  | RFC 3261 [15] |
| value |  | value of CSeq sent by the endpoint within its previous request in the same dialog but increased by one |  |  |
| method |  | UPDATE |  |  |
| Require | A1 |  |  | RFC 3261 [15] RFC 3329 [21] |
| option-tag | A1 | sec-agree |  |  |
| Proxy-Require | A1 |  |  | RFC 3261 [15] RFC 3329 [21] |
| option-tag | A1 | sec-agree |  |  |
| Max-Forwards |  |  |  | RFC 3261 [15] |
| value |  | non-zero value |  |  |
| Security-Verify | A1 |  |  | RFC 3329 [21] |
| sec-mechanism |  | same value as Security-Server header sent by SS |  |  |
| Security-Verify | A2 | not present |  | RFC 3329 [21] |
| sec-mechanism |  |  |  |  |
| P-Access-Network-Info | A1,A2(o) |  |  | RFC 7315 [132] RFC 7913 [154] |
| access-net-spec | A5 | access network information and, if applicable, the cell ID |  |  |
|  | A6 | access network information for NR, containing access-class parameter with value "3GPP-NR" or access-type parameter with value "3GPP-NR-FDD" or "3GPP-NR-TDD", and also containing the cell ID | Rel-15 |  |
| Content-Type |  |  |  | RFC 3261 [15] |
| media-type |  | application/sdp |  |  |
| Content-Length | A3,A4 |  |  | RFC 3261 [15] |
| value |  | length of message-body |  |  |
| Message-body |  | Contents of the SDP body shall be checked as described in the Test requirements section of the test case. |  | RFC 4566 [27] RFC 3264 [30] RFC 3312 [31] |

| Condition | Explanation |
| --- | --- |
| A1 | UPDATE sent by the UE (IMS security, A.6a/2 3GPP TS 34.229-2 [5]) (NOTE 3) |
| A2 | UPDATE sent by the UE (GIBA, A.6a/1 3GPP TS 34.229-2 [5]) (NOTE 3) |
| A3 | UPDATE sent by the SS (IMS security, A.6a/2 3GPP TS 34.229-2 [5]) (NOTE 3) |
| A4 | UPDATE sent by the SS (GIBA, A.6a/1 3GPP TS 34.229-2 [5]) (NOTE 3) |
| A5 | UE uses E-UTRAN access (A.18/1 3GPP TS 34.229-2 [5]) |
| A6 | UE uses NR access (A.18/5 3GPP TS 34.229-2 [5]) |

NOTE 1:	All choices for applicable conditions are described for each header.

NOTE 2:	Branch parameter values sent by SS are different within a test case execution.

NOTE 3:	When the UE supports IMS security then condition A1 or A3 is applied unless condition A2 or A4 is specified explicitly by the test case.

<End of modification>