3GPP TSG-RAN5 Meeting #107	R5-252631

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1530 | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to the PICS in Generic procedure and RRC messages |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the PICS defined in TS 36.523-2 and TS 38.508-2, below PICS items shall be corrected: pc_Provide_ProSe => pc_Provide_ProSe_APN pc_dynamicPowerSharing => pc_dynamicPowerSharingENDC pc_intraFreq-CE-NeedForGaps => pc_intraFreq_CE_NeedForGaps |  |  |  |  |  |  |  |  |
| Summary of change: |  | PICS items in the generic procedure and RRC messages were updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect PICS will be used. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5A.22.3, 4.5A.22A.3, 4.6.1, 4.6.6 and 6.5.1. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

4.5A.22.3	Procedure

The procedure is only applicable for pc_Provide_ProSe_APN == true.

The HTTP signalling between the UE and the ProSe function is done over TLS i.e. the UE connects to TCP port 443 (HTTPS) and starts TLS handshake.

[Skip unchanged text]

4.5A.22A.3	Procedure

The procedure is only applicable for pc_Provide_ProSe_APN == true.

The HTTP signalling between the UE and the ProSe function is done over an already established TLS connection.

[Skip unchanged text]

–	RRCConnectionReconfiguration

Table 4.6.1-8: RRCConnectionReconfiguration

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| measConfig | Not present |  |  |
|  | MeasConfig-DEFAULT |  | MEAS |
| mobilityControlInfo | Not present |  |  |
|  | MobilityControlInfo-HO |  | HO, HO-TO-EUTRA, HO-TO-EN-DC |
| dedicatedInfoNASList | Not present |  |  |
|  | Set according to specific message content |  | SRB2-DRB(n, m) DRB(n, m) SRB1-SRB2-DRB(n,m) AM-DRB-ADD(bid) UM-DRB-ADD(bid) DRB-REL(bid) DRB-Mod MCG_and_split EN-DC_SRB2-DRB |
| radioResourceConfigDedicated | Not present |  |  |
|  | RadioResourceConfigDedicated-SRB2-DRB(n, m) |  | SRB2-DRB(n, m) |
|  | RadioResourceConfigDedicated-DRB(n, m) |  | DRB(n, m) |
|  | RadioResourceConfigDedicated-AM-DRB-ADD(bid) |  | AM-DRB-ADD(bid) |
|  | RadioResourceConfigDedicated-UM-DRB-ADD(bid) |  | UM-DRB-ADD(bid) |
|  | RadioResourceConfigDedicated-DRB-REL(bid) |  | DRB-REL(bid) |
|  | RadioResourceConfigDedicated-HO |  | HO |
|  | RadioResourceConfigDedicated-HO-TO-EUTRA(n, m) |  | HO-TO-EUTRA(n,m), HO-TO-EN-DC(n,m) |
|  | RadioResourceConfigDedicated-DRB-Mod |  | DRB-Mod |
|  | RadioResourceConfigDedicated-SCell_AddMod |  | SCell_AddMod |
|  | RadioResourceConfigDedicated-PCell-PATTERN |  | PCell-PATTERN |
|  | RadioResourceConfigDedicated-EN-DC-Split |  | MCG_and_split |
|  | RadioResourceConfigDedicated-EN-DC_Split_DRB_Rel(bid) |  | EN-DC_Split_Rel(bid) |
|  | RadioResourceConfigDedicated-SRB4 |  | SRB4 |
|  | RadioResourceConfigDedicated-EN-DC-SRB2-DRB |  | EN-DC_SRB2-DRB |
| securityConfigHO | Not present |  |  |
|  | SecurityConfigHO-DEFAULT |  | HO, HO-TO-EUTRA, HO-TO-EN-DC |
| nonCriticalExtension SEQUENCE { |  | RRCConnectionReconfiguration-v890-IEs |  |
| lateNonCriticalExtension | Not present | RRCConnectionReconfiguration-v8m0-IEs |  |
| lateNonCriticalExtension SEQUENCE { |  | RRCConnectionReconfiguration-v8m0-IEs | 4Rx_PCC |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  | RRCConnectionReconfiguration-v10i0-IEs |  |
| antennaInfoDedicatedPCell-v10i0 SEQUENCE { |  |  |  |
| maxLayersMIMO-r10 | fourLayers |  |  |
| } |  |  |  |
| nonCriticalExtension | Not present | RRCConnectionReconfiguration-v10l0-IEs |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  | RRCConnectionReconfiguration-v920-IEs |  |
| otherConfig-r9 | Not present |  |  |
|  | OtherConfig-r9 |  | OtherConfig |
| fullConfig-r9 | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  | RRCConnectionReconfiguration-v1020-IEs |  |
| sCellToReleaseList-r10 | Not present |  |  |
| sCellToAddModList-r10 | Not present |  |  |
| sCellToAddModList-r10 SEQUENCE (SIZE (1..maxSCell-r10)) OF SCellToAddMod-r10 { | The same number of entries as the configured SCell(s) | n denotes the index of the entry | SCell_AddMod |
| SCellToAddMod-r10[n] | SCellToAddMod-r10-DEFAULT |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  | RRCConnectionReconfiguration-v1130-IEs |  |
| systemInfomationBlockType1Dedicated-r11 | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  | RRCConnectionReconfiguration-v1250-IEs |  |
| wlan-OffloadInfo-r12 | Not present |  |  |
| wlan-OffloadInfo-r12 CHOICE { |  |  | WLAN-OffloadSetup, WLAN-OffloadRelease |
| Release | NULL |  | WLAN-OffloadRelease |
| setup SEQUENCE { |  |  | WLAN-OffloadSetup |
| wlan-OffloadConfigDedicated-r12 | WLAN-OffloadConfig-DEFAULT |  |  |
| t350-r12 | min10 |  |  |
| } |  |  |  |
| } |  |  |  |
| scg-Configuration-r12 | Not present |  |  |
| scg-Configuration-r12 | SCG-Configuration-r12-DEFAULT |  | PSCell_Release, PSCell_Mod, PSCell_Add_Split_DRB, PSCell_Add_SCG_DRB |
| scg-Configuration-r12 | SCG-Configuration-r12-NE-DC |  | NE-DC |
| sl-SyncTxControl-r12 | Not present |  |  |
| sl-DiscConfig-r12 | Not present |  |  |
| sl-CommConfig-r12 | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  | RRCConnectionReconfiguration-v1310-IEs | LWA |
| lwa-configuration-r13 | Set as specified in test case |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  | RRCConnectionReconfiguration-v1310-IEs | LWIP |
| lwip-configuration-r13 | Set as specified in test case |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  | RRCConnectionReconfiguration-v1310-IEs | MCG_and_SCG, MCG_and_split, SCG-DRB(n,m), MCG_NR_PDCP, EN-DC_PSCell_Rel, SRB_NR_PDCP, EN-DC_EmbedNR_RRCRecon, EN-DC_Embed_RBConfig, EN-DC_PSCell_HO, CHO, CPA, Inter-SN CPC, DeactivatedSCG |
| nonCriticalExtension SEQUENCE { |  | RRCConnectionReconfiguration-v1430-IEs |  |
| nonCriticalExtension SEQUENCE { |  | RRCConnectionReconfiguration-v1510-IEs |  |
| nr-Config-r15 CHOICE { |  |  |  |
| release |  |  | EN-DC_PSCell_Rel |
| setup SEQUENCE { |  |  | MCG_and_SCG, MCG_and_split, SCG-DRB(n,m), MCG_NR_PDCP, EN-DC_EmbedNR_RRCRecon, EN-DC_Embed_RBConfig, EN-DC_PSCell_HO |
| endc-ReleaseAndAdd-r15 | false |  |  |
| nr-SecondaryCellGroupConfig-r15 | OCTET STRING including the RRCReconfiguration message and the IE secondaryCellGroup according TS 38.508-1 [67], table 4.6.1-13 with condition EN-DC. |  | MCG_and_SCG，HO-TO-EN-DC(n,m) |
|  | OCTET STRING including the RRCReconfiguration message, the IE secondaryCellGroup according TS 38.508-1 [67], table 4.6.1-13 with condition EN-DC and the IE PDCP-Config according TS 38.508-1 [67], table 4.6.3-99 with condition Split. |  | MCG_and_split |
|  | OCTET STRING including the RRCReconfiguration message and the IE secondaryCellGroup according TS 38.508-1 [67], table 4.8.1-2. |  | SCG-DRB(n,m) |
|  | OCTET STRING including the test case specific RRCReconfiguration |  | EN-DC_EmbedNR_RRCRecon |
|  | OCTET STRING including the RRCReconfiguration message and the IE secondaryCellGroup according TS 38.508-1 [67], table 4.6.1-13 with condition EN-DC_HO |  | EN-DC_PSCell_HO |
| p-MaxEUTRA-r15 | Not present |  |  |
|  | 23 |  | SIG AND EN-DC AND pc_dynamicPowerSharingENDC |
|  | 20 |  | SIG AND EN-DC AND (NOT pc_dynamicPowerSharingENDC) |
|  | 10 |  | RF AND EN-DC_PC3 |
|  | 13 |  | RF AND EN-DC_PC2 |
| } |  |  |  |
| } |  |  |  |
| sk-Counter-r15 | 0 |  | MCG_and_SCG, MCG_and_split, SCG-DRB(n,m), HO-TO-EN-DC(n,m) |
|  | Not Present |  |  |
| nr-RadioBearerConfig1-r15 | OCTET STRING including RadioBearerConfig according TS 38.508-1 [67], table 4.6.3-132 with condition EN-DC_DRB. |  | MCG_and_SCG, MCG_and_split，HO-TO-EN-DC(n,m) |
|  | OCTET STRING including RadioBearerConfig according TS 38.508-1 [67], table 4.8.1-3. |  | SCG-DRB(n,m) |
|  | OCTET STRING including RadioBearerConfig according TS 38.508-1 [67], table 4.6.3-132 with condition MCG_NR_PDCP |  | MCG_NR_PDCP |
|  | OCTET STRING including RadioBearerConfig according TS 38.508-1 [67], table 4.6.3-132 with condition SRB_NR_PDCP |  | SRB_NR_PDCP |
|  | OCTET STRING including the test case specific RadioBearerConfig |  | EN-DC_Embed_RBConfig |
|  | OCTET STRING including RadioBearerConfig according TS 38.508-1 [67], table 4.6.3-132 with conditions EN-DC_DRB and Re-establish_PDCP |  | EN-DC_PSCell_HO AND RBConfig_KeyChange |
|  | OCTET STRING including RadioBearerConfig according TS 38.508-1 [67], table 4.6.3-132 with conditions EN-DC_DRB and Recover_PDCP |  | EN-DC_PSCell_HO AND RBConfig_NoKeyChange |
| nr-RadioBearerConfig2-r15 | Not present |  |  |
| tdm-PatternConfig-r15 | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  | RRCConnectionReconfiguration-v1530-IEs | CHO, CPA, Inter-SN CPC, DeactivatedSCG |
| securityConfigHO-v1530 | Not present |  |  |
| sCellGroupToReleaseList-r15 | Not present |  |  |
| sCellGroupToAddModList-r15 | Not present |  |  |
| dedicatedInfoNASList-r15 | Not present |  |  |
| p-MaxUE-FR1-r15 | Not present |  |  |
| smtc-r15 | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  | RRCConnectionReconfiguration-v1610-IEs | CHO, CPA, Inter-SN CPC, DeactivatedSCG |
| conditionalReconfiguration-r16 | Not present |  |  |
|  | ConditionalReconfiguration-r16-HO |  | CHO |
|  | ConditionalReconfiguration-r16-CPA |  | CPA |
|  | ConditionalReconfiguration-r16-interSN_CPC |  | Inter-SN CPC |
| daps-SourceRelease-r16 | Not present |  |  |
| tdm-PatternConfig2-r16 | Not present |  |  |
| sl-ConfigDedicatedForNR-r16 | Not present |  |  |
| sl-SSB-PriorityEUTRA-r16 | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  | DeactivatedSCG |
| systemInformationBlockType31Dedicated-r17 | Not present |  |  |
| scg-State-r17 | Not present |  |  |
|  | deactivated |  | DeactivatedSCG |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

[Skip unchanged text]

-	MeasConfig-DEFAULT

Table 4.6.6-1: MeasConfig-DEFAULT

| Derivation Path: 36.331, clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| measObjectToRemoveList | Not present |  |  |
| measObjectToAddModList | Not present |  |  |
|  | MeasObjectEUTRA-GENERIC |  | RF |
| reportConfigToRemoveList | Not present |  |  |
| reportConfigToAddModList | Not present |  |  |
|  | ReportConfigToAddModList_DEFAULT |  | RF |
| measIdToRemoveList | Not present |  |  |
| measIdToAddModList | Not present |  |  |
|  | MeasIdToAddModList_DEFAULT |  | RF |
| quantityConfig | QuantityConfig-DEFAULT |  |  |
| measGapConfig | Not present |  |  |
|  | MeasGapConfig-GP1 |  | INTER-FREQ, UTRAN |
|  | MeasGapConfig-GP2 |  | GERAN, INTER-RAT |
|  | MeasGapConfig-CE |  | intraFreq-CE-NeedForGaps |
| s-Measure | Not present |  |  |
| preRegistrationInfoHRPD | Not present |  |  |
| speedStatePars | Not present |  |  |
| measObjectToAddModList-v9e0 SEQUENCE { |  |  | Band > 64 and RF |
| measObjectEUTRA-v9e0 SEQUENCE { | N entries | N needs to be the same number of entries as in measObjectToAddModList |  |
| carrierFreq-v9e0[n] | Downlink EARFCN for Freq |  |  |
| } |  |  |  |
| } |  |  |  |
| measScaleFactor-r12 | sf-EUTRA-cf1 | specifies the factor for scaling the measurement performance requirements in TS 36.133 | IncMon |
| heightThreshRef-r15 | Not present |  |  |
| } |  |  |  |
| Note: For signalling test, table 6.5.1-1 is applied. |  |  |  |

| Condition | Explanation |
| --- | --- |
| INTER-FREQ | For E-UTRA inter-freq measurements |
| INTER-RAT | For inter-RAT measurements with UTRAN and GERAN |
| Band > 64 | If band > 64 is selected |
| intraFreq-CE-NeedForGaps | For UE having set pc_intraFreq_CE_NeedForGaps to TRUE. |
| RF | For RF, performance and RRM testing |
| IncMon | For IncMon RRM testing |

[Skip unchanged text]

6.5.1	Measurement information elements

-	MeasConfig-DEFAULT

Table 6.5.1-1: MeasConfig-DEFAULT

| Derivation Path: 36.331, clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| measObjectToRemoveList | Not present |  |  |
| measObjectToAddModList | Not present |  |  |
| reportConfigToRemoveList | Not present |  |  |
| reportConfigToAddModList | Not present |  |  |
| measIdToRemoveList | Not present |  |  |
| measIdToAddModList | Not present |  |  |
| quantityConfig | QuantityConfig-DEFAULT |  |  |
| measGapConfig | Not present |  |  |
|  | MeasGapConfig-GP1 |  | INTER-FREQ, UTRAN |
|  | MeasGapConfig-GP2 |  | GERAN, INTER-RAT |
|  | MeasGapConfig-CE |  | intraFreq-CE-NeedForGaps |
| s-Measure | Not present |  |  |
| preRegistrationInfoHRPD | Not present |  |  |
| speedStatePars | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| INTER-FREQ | For E-UTRA inter-freq measurements |
| UTRAN | For inter-RAT measurements with UTRAN |
| GERAN | For inter-RAT measurements with GERAN |
| INTER-RAT | For inter-RAT measurements with UTRAN and GERAN |
| intraFreq-CE-NeedForGaps | For UE having set pc_intraFreq_CE_NeedForGaps to TRUE. |

[End of change]