3GPP TSG-RAN5 Meeting #107	R5-253062

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5466 | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of inter RAT handover test cases 8.4.2.2, 8.4.2.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2025-05-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | At step 2 of inter RAT handover test cases 8.4.2.2 and 8.4.2.4, UE is expected to perform a reselection from E-UTRA to high priority UTRA cell. This cannot be achieved on a UE supporting protection against improper reselection from E-UTRAN to GERAN/UTRAN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | E-UTRA power levels are updated for UE to select UTRAN cell in these test cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may unfairly fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.2.2, 8.4.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

#### 8.4.2.2	Inter-RAT handover / From UTRA PS to E-UTRA / Data

8.4.2.2.1	Test Purpose (TP)

(1)

with { UE in UTRA CELL_DCH(PS-DCCH+DTCH_DCH) state }

ensure that {

when { UE receives a HANDOVER FROM UTRAN COMMAND message including the eutra-Message }

then { UE transmits an RRCConnectionReconfigurationComplete message and enters E-UTRA RRC_CONNECTED state }

}

8.4.2.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.4.2.3.

[TS 36.331, clause 5.4.2.3]

If the UE is able to comply with the configuration included in the RRCConnectionReconfiguration message, the UE shall:

1>	apply the default physical channel configuration as specified in 9.2.4;

1>	apply the default semi-persistent scheduling configuration as specified in 9.2.3;

1>	apply the default MAC main configuration as specified in 9.2.2;

1>	start timer T304 with the timer value set to t304, as included in the mobilityControlInfo;

1>	consider the target cell to be one on the frequency indicated by the carrierFreq with a physical cell identity indicated by the targetPhysCellId;

1>	start synchronising to the DL of the target cell;

1>	set the C-RNTI to the value of the newUE-Identity;

1>	for the target cell, apply the downlink bandwidth indicated by the dl-Bandwidth;

1>	for the target cell, apply the uplink bandwidth indicated by (the absence or presence of) the ul-Bandwidth;

1>	perform the radio resource configuration procedure as specified in 5.3.10;

1>	forward the nas-SecurityParamToEUTRA to the upper layers;

1>	derive the KeNB key, as specified in TS 33.401 [32];

1>	store the nextHopChainingCount value;

1>	derive the KRRCint key associated with the integrityProtAlgorithm, as specified in TS 33.401 [32];

1>	derive the KRRCenc key and the KUPenc key associated with the cipheringAlgorithm, as specified in TS 33.401 [32];

1>	configure lower layers to apply the indicated integrity protection algorithm and the KRRCint key immediately, i.e. the indicated integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	configure lower layers to apply the indicated ciphering algorithm, the KRRCenc key and the KUPenc key immediately, i.e. the indicated ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration;

1>	use the default values specified in 9.2.5 for timer T310, T311 and constant N310, N311;

1>	if MAC successfully completes the random access procedure:

2>	stop timer T304;

2>	apply the parts of the configuration that do not require the UE to know the SFN of the target cell;

2>	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the target cell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of the target cell;

2>	enter E-UTRA RRC_CONNECTED, upon which the procedure ends;

8.4.2.2.3	Test description

8.4.2.2.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 5.

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 (serving cell) according to [18].

8.4.2.2.3.2	Test procedure sequence

Table 8.4.2.2.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Subsequent configurations marked "T1" and "T2" are applied at the points indicated in the Main behaviour description in Table 8.4.2.2.3.2-2.

Table 8.4.2.2.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 5 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -115 | - | The power level values are such that UE camps on Cell 5. |
|  | CPICH Ec (UTRA FDD) | dBm/3.84 MHz | - | -65 |  |
|  | PCCPCH Ec (UTRA LCR TDD) | dBm/1.28 MHz | - | -65 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -70 | - | The power level values are such that entering conditions for event 3a are satisfied. |
|  | CPICH Ec (UTRA FDD) | dBm/3.84 MHz | - | -85 |  |
|  | PCCPCH Ec (UTRA LCR TDD) | dBm/1.28 MHz | - | -85 |  |

Table 8.4.2.2.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | Wait for 6 s for UE to receive system information. | - | - | - | - |
| 1 | The SS changes Cell 1 and Cell 5 level according to the row "T1" in table 8.4.2.2.3.2-1. | - | - | - | - |
| 2 | Generic test procedure in TS 36.508 subclause 6.4.2.8 is performed on Cell 5. NOTE: The UE performs an RAU procedure and the RRC connection is released. | - | - | - | - |
| - | Exception: Steps 2Ba1 to 2Ba1b1 describe optional behaviour that depend on the UE capability. | - | - | - | - |
| 2Aa1-2Aa3Aa6 | Void | - | - | - | - |
| 2Ba1 | IF pc_IMS = TRUE THEN SS starts 6s timer |  |  |  |  |
| 2Ba1a1-2Ba1a3a6 | The procedure defined in TS 36.523-3 Table 10.1.2.1-2 Steps 1-4a6 is performed |  |  |  |  |
| 2Ba1a4 | SS releases RRC Connection |  |  |  |  |
| 2Ba1a5 | Stop 6s timer |  |  |  |  |
| 2Ba1b1 | The 6s timer expires |  |  |  |  |
| 3-4 | Void | - | - | - | - |
| 4A-4E | Step 7 to 11 of test procedure in TS 34.123-1 subclause 12.9.14.4 is performed on Cell 5 using the UTRA reference radio bearer parameters and combination “UTRA PS RB” according to TS 36.508 subclause 4.8.3 and Table 4.8.3-1. NOTE: The UE performs Network initiated RAB re-establishment in a UTRAN cell. | - | - | - | - |
| - | For UTRAN FDD, EXCEPTION: Steps 5a1 to 5a2 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that takes place if a capability is supported. For UTRAN TDD, goto step8. | - | - | - | - |
| 5a1 | IF pc_UTRA_CompressedModeRequired THEN the SS transmits a PHYSICAL CHANNEL RECONFIGURATION message on Cell 5 including the DPCH compressed mode info. | <-- | PHYSICAL CHANNEL RECONFIGURATION | - | - |
| 5a2 | The UE transmits a PHYSICAL CHANNEL RECONFIGURATION COMPLETE message on Cell 5. | --> | PHYSICAL CHANNEL RECONFIGURATION COMPLETE | - | - |
| 6-7 | Void | - | - | - | - |
| 8 | The SS transmits a MEASUREMENT CONTROL message to setup inter-RAT measurement on Cell 5. | <-- | MEASUREMENT CONTROL | - | - |
| 9 | The SS changes Cell 1 and Cell 5 level according to the row "T2" in table 8.4.2.2.3.2-1. | - | - | - | - |
| 10 | The UE transmits a MEASUREMENT REPORT message on Cell 5 including the E-UTRA event results. | --> | MEASUREMENT REPORT | - | - |
| 11 | The SS transmits a HANDOVER FROM UTRAN COMMAND message on Cell 5. | <-- | HANDOVER FROM UTRAN COMMAND | - | - |
| 12 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 1 using the security key derived from the new KeNB? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 12A | Generic test procedure in TS 36.508 subclause 6.4.2.10 is performed on Cell 1. NOTE: The UE performs tracking area updating procedure without ISR and security reconfiguration after successful completion of handover from UTRA. | - | - | - | - |
| 13-19 | Void | - | - | - | - |
|  | EXCEPTION: Steps 19Aa1 to 19Aa9 describe optional behaviour that depend on the UE capability | - | - | - | - |
| 19Aa1-19Aa2b1 | The procedure defined in TS 36.508 subclause 4.5A.3 Steps 10a2a1-10a2b1 is performed |  |  |  |  |
| 20 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC CONNECTED state on Cell 1? | - | - | 1 | - |

8.4.2.2.3.3	Specific message contents

Table 8.4.2.2.3.3-1: Void

Table 8.4.2.2.3.3-2: SystemInformationBlockType6 for Cell 1 (preamble, Table 8.4.2.2.3.2-2)

| Derivation Path: 36.508, Table 4.4.3.3-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType6 ::= SEQUENCE { |  |  |  |
| carrierFreqListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { |  |  | UTRA-FDD |
| carrierFreq[n] | Same downlink UARFCN as used for Cell 5 |  |  |
| cellReselectionPriority[n] | 5 |  |  |
| p-MaxUTRA[1] | 0 |  |  |
| } |  |  |  |
| carrierFreqListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { | The same number of entries as the configured UTRA TDD carriers |  | UTRA-TDD |
| carrierFreq[1] | Same downlink ARFCN as used for Cell 5 |  |  |
| cellReselectionPriority[n] | 5 |  |  |
| p-MaxUTRA[n] | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 8.4.2.2.3.3-3: Void

Table 8.4.2.2.3.3-4: Void

Table 8.4.2.2.3.3-5: HANDOVER FROM UTRAN COMMAND (step 11, Table 8.4.2.2.3.2-2)

Derivation Path: 36.508, Table 4.7B.1-2

Table 8.4.2.2.3.3-6: RRCConnectionReconfiguration (Table 8.4.2.2.3.3-5)

Derivation Path: 36.508, Table 4.6.1-8, condition HO-TO-EUTRA(1,0)

Table 8.4.2.2.3.3-7: MobilityControlInfo (Table 8.4.2.2.3.3-5)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 1. |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 1. |  |  |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 1. |  | Band 24 High range |
| } |  |  |  |
| carrierFreq | Not present |  | Band > 64 |
| carrierBandwidth SEQUENCE { |  |  |  |
| dl-Bandwidth | Downlink system bandwidth under test. |  |  |
| ul-Bandwidth | Uplink Bandwidth under test. |  | FDD |
| ul-Bandwidth | Not present |  | TDD |
| } |  |  |  |
| additionalSpectrumEmission | 1 |  |  |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |
| Band > 64 | If band > 64 is selected |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

Table 8.4.2.2.3.3-8: SecurityConfigHO (Table 8.4.2.2.3.3-5)

| Derivation Path: 36.508, Table 4.6.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SecurityConfigHO ::= SEQUENCE { |  |  |  |
| handoverType CHOICE { |  |  |  |
| interRAT SEQUENCE { |  |  |  |
| securityAlgorithmConfig SEQUENCE { |  |  |  |
| cipheringAlgorithm | Set according to PIXIT parameter for default ciphering protection algorithm |  |  |
| integrityProtAlgorithm | Set according to PIXIT parameter for default integrity algorithm |  |  |
| } |  |  |  |
| nas-SecurityParamToEUTRA | Octets 1 to 4 are arbitrarily selected. Bits 1 to 3 of octet 5 are set according to PIXIT parameter for default integrity protection algorithm. Bits 5 to 7 of octet 5 are set according to PIXIT parameter for default ciphering algorithm. Bits 1 to 3 of octet 6 are arbitrarily selected between '000'B and '110'B, different from the valid NAS key set identifier of the UE if such a value exists. Bit 4 of octet 6 is set to 1. | Octets 1 to 4 include the NonceMME value. Bits 1 to 3 of octet 5 include the Type of integrity protection algorithm Bits 5 to 7 of octet 5 include the Type of ciphering algorithm. Bits 1 to 4 of octet 6 include the NAS key set identifier. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.4.2.2.3.3-9: MEASUREMENT CONTROL (step 8, Table 8.4.2.2.3.2-2)

| Derivation Path: 36.508, clause 4.7B.1-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| - Inter-RAT measurement quantity |  |  |  |
| - Measurement quantity for UTRAN quality estimate |  |  |  |
| - Filter coefficient | 0 |  |  |
| - CHOICE mode | FDD |  |  |
| - Measurement quantity | CPICH RSCP |  |  |
| - CHOICE system | E-UTRA |  |  |
| - Measurement quantity | RSRP |  |  |
| - Filter coefficient | 0 |  |  |
| - Inter-RAT reporting quantity |  |  |  |
| - UTRAN estimated quality | FALSE |  |  |
| - CHOICE system | E-UTRA |  |  |
| - Reporting quantity | both |  |  |
| - Reporting cell status | Not present |  |  |
| - CHOICE report criteria | Inter-RAT measurement reporting criteria |  |  |
| - Parameters required for each event | 1 entry |  |  |
| - Inter-RAT event identity | 3a |  |  |
| - Threshold own system | -66 |  |  |
| - W | 0 |  |  |
| - Threshold other system | -80 |  |  |
| - Hysteresis | 0 |  |  |
| - Time to trigger | 10 ms |  |  |
| - Reporting cell status |  |  |  |
| - CHOICE reported cell | Report cells within active set or within virtual active set or of the other RAT |  |  |
| - Maximum number of reported cells | 2 |  |  |

Table 8.4.2.2.3.3-10: System Information Block type 19 for Cell 5 (preamble, Table 8.4.2.2.3.2-2)

| Derivation Path: 36.508 clause 4.4.4.1, Table 4.4.4.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList SEQUENCE { |  |  |  |
| utra-ServingCell SEQUENCE { |  |  |  |
| Priority | 5 | higher priority than E-UTRA |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of modified section>

<Start of modified section>

#### 8.4.2.4	Inter-RAT handover / From UTRA HSPA to E-UTRA / Data

8.4.2.4.1	Test Purpose (TP)

(1)

with { UE in UTRA CELL_DCH(PS-DCCH+DTCH_HS-DSCH) state }

ensure that {

when { UE receives a HANDOVER FROM UTRAN COMMAND message including the eutra-Message }

then { UE transmits an RRCConnectionReconfigurationComplete message and enters E-UTRA RRC_CONNECTED state }

}

8.4.2.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.4.2.3.

[TS 36.331, clause 5.4.2.3]

If the UE is able to comply with the configuration included in the RRCConnectionReconfiguration message, the UE shall:

1>	apply the default physical channel configuration as specified in 9.2.4;

1>	apply the default semi-persistent scheduling configuration as specified in 9.2.3;

1>	apply the default MAC main configuration as specified in 9.2.2;

1>	start timer T304 with the timer value set to t304, as included in the mobilityControlInfo;

1>	consider the target cell to be one on the frequency indicated by the carrierFreq with a physical cell identity indicated by the targetPhysCellId;

1>	start synchronising to the DL of the target cell;

1>	set the C-RNTI to the value of the newUE-Identity;

1>	for the target cell, apply the downlink bandwidth indicated by the dl-Bandwidth;

1>	for the target cell, apply the uplink bandwidth indicated by (the absence or presence of) the ul-Bandwidth;

1>	perform the radio resource configuration procedure as specified in 5.3.10;

1>	forward the nas-SecurityParamToEUTRA to the upper layers;

1>	derive the KeNB key, as specified in TS 33.401 [32];

1>	store the nextHopChainingCount value;

1>	derive the KRRCint key associated with the integrityProtAlgorithm, as specified in TS 33.401 [32];

1>	derive the KRRCenc key and the KUPenc key associated with the cipheringAlgorithm, as specified in TS 33.401 [32];

1>	configure lower layers to apply the indicated integrity protection algorithm and the KRRCint key immediately, i.e. the indicated integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	configure lower layers to apply the indicated ciphering algorithm, the KRRCenc key and the KUPenc key immediately, i.e. the indicated ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration;

1>	use the default values specified in 9.2.5 for timer T310, T311 and constant N310, N311;

1>	if MAC successfully completes the random access procedure:

2>	stop timer T304;

2>	apply the parts of the configuration that do not require the UE to know the SFN of the target cell;

2>	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the target cell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of the target cell;

2>	enter E-UTRA RRC_CONNECTED, upon which the procedure ends;

8.4.2.4.3	Test description

8.4.2.4.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 5.

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 (serving cell) according to [18].

8.4.2.4.3.2	Test procedure sequence

Table 8.4.2.4.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Subsequent configurations marked "T1" and "T2" are applied at the points indicated in the Main behaviour description in Table 8.4.2.4.3.2-2.

Table 8.4.2.4.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 5 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -115 | - | The power level values are such that UE camps on Cell 5. |
|  | CPICH Ec (UTRA FDD) | dBm/3.84 MHz | - | -65 |  |
|  | PCCPCH Ec(UTRA LCR TDD) | dBm/1.28 MHz | - | -65 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -70 | - | The power level values are such that entering conditions for event 3a are satisfied. |
|  | CPICH Ec (UTRA FDD) | dBm/3.84 MHz | - | -85 |  |
|  | PCCPCH Ec (UTRA LCR TDD) | dBm/1.28 MHz | - | -85 |  |

Table 8.4.2.4.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell 1 and Cell 5 level according to the row "T1" in table 8.4.2.4.3.2-1. | - | - | - | - |
| 2 | Generic test procedure in TS 36.508 subclause 6.4.2.8 is performed on Cell 5. NOTE: The UE performs an RAU procedure and the RRC connection is released. | - | - | - | - |
| - | Exception: Steps 2Aa1 to 2Aa1b1 describe optional behaviour that depend on the UE capability. | - | - | - | - |
| 2Aa1 | IF pc_IMS = TRUE THEN SS starts 6s timer | - | - | - | - |
| 2Aa1a1-2Aa1a3a6 | The procedure defined in TS 36.523-3 Table 10.1.2.1-2 Steps 1-4a6 is performed | - | - | - | - |
| 2Aa1a4 | SS releases RRC Connection | - | - | - | - |
| 2Aa1a5 | Stop 6s timer | - | - | - | - |
| 2Aa1b1 | The 6s timer expires | - | - | - | - |
| 3-4 | Void | - | - | - | - |
| 4A-4E | Step 7 to 11 of test procedure in TS 34.123-1 subclause 12.9.14.4 is performed on Cell 5 using the UTRA reference radio bearer parameters and combination “UTRA HSDPA RB” according to TS 36.508 subclause 4.8.3 and Table 4.8.3-1. NOTE: The UE performs Network initiated RAB re-establishment in a UTRAN cell. | - | - | - | - |
| - | UTRAN FDD: EXCEPTION: Steps 5a1 to 5a2 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that takes place if a capability is supported. UTRAN TDD : go to step 8 | - | - | - | - |
| 5a1 | IF pc_UTRA_CompressedModeRequired THEN the SS transmits a PHYSICAL CHANNEL RECONFIGURATION message on Cell 5 including the DPCH compressed mode info. | <-- | PHYSICAL CHANNEL RECONFIGURATION | - | - |
| 5a2 | The UE transmits a PHYSICAL CHANNEL RECONFIGURATION COMPLETE message on Cell 5. | --> | PHYSICAL CHANNEL RECONFIGURATION COMPLETE | - | - |
| 6-7 | Void | - | - | - | - |
| 8 | The SS transmits a MEASUREMENT CONTROL message to setup inter-RAT measurement on Cell 5. | <-- | MEASUREMENT CONTROL | - | - |
| 9 | The SS changes Cell 1 and Cell 5 level according to the row "T2" in table 8.4.2.4.3.2-1. | - | - | - | - |
| 10 | The UE transmits a MEASUREMENT REPORT message on Cell 5 including the E-UTRA event results. | --> | MEASUREMENT REPORT | - | - |
| 11 | The SS transmits a HANDOVER FROM UTRAN COMMAND message on Cell 5. | <-- | HANDOVER FROM UTRAN COMMAND | - | - |
| 12 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 1 using the security key derived from the new KeNB? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 12A | Generic test procedure in TS 36.508 subclause 6.4.2.10 is performed on Cell 1. NOTE: The UE performs tracking area updating procedure without ISR and security reconfiguration after successful completion of handover from UTRA. | - | - | - | - |
| 13-19 | Void | - | - | - | - |
|  | EXCEPTION: Steps 19Aa1 to 19Aa9 describe optional behaviour that depend on the UE capability | - | - | - | - |
| 19Aa1-19Aa2b1 | The procedure defined in TS 36.508 subclause 4.5A.3 Steps 10a2a1-10a2b1 is performed |  |  |  |  |
| 20 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC CONNECTED state on Cell 1? | - | - | 1 | - |

8.4.2.4.3.3	Specific message contents

Table 8.4.2.4.3.3-1: Void

Table 8.4.2.4.3.3-2: SystemInformationBlockType6 for Cell 1 (preamble, Table 8.4.2.4.3.2-2)

| Derivation Path: 36.508, Table 4.4.3.3-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType6 ::= SEQUENCE { |  |  |  |
| carrierFreqListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { |  |  | UTRA-FDD |
| carrierFreq[n] | Same downlink UARFCN as used for Cell 5 |  |  |
| cellReselectionPriority[n] | 5 |  |  |
| p-MaxUTRA[n] | 0 |  |  |
| } |  |  |  |
| carrierFreqListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { |  |  | UTRA-TDD |
| carrierFreq[n] | Same downlink UARFCN as used for Cell 5 |  |  |
| cellReselectionPriority[n] | 5 |  |  |
| p-MaxUTRA[n] | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 8.4.2.4.3.3-3: Void

Table 8.4.2.4.3.3-4: Void

Table 8.4.2.4.3.3-5: HANDOVER FROM UTRAN COMMAND (step 11, Table 8.4.2.4.3.2-2)

Derivation Path: 36.508, Table 4.7B.1-2

Table 8.4.2.4.3.3-6: RRCConnectionReconfiguration (Table 8.4.2.4.3.3-5)

Derivation Path: 36.508, Table 4.6.1-8, condition HO-TO-EUTRA(1,0)

Table 8.4.2.4.3.3-7: MobilityControlInfo (Table 8.4.2.4.3.3-5)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 1. |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 1. |  |  |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 1. |  | Band 24 High range |
| } |  |  |  |
| carrierFreq | Not present |  | Band > 64 |
| carrierBandwidth SEQUENCE { |  |  |  |
| dl-Bandwidth | Downlink system bandwidth under test. |  |  |
| ul-Bandwidth | Uplink Bandwidth under test. |  | FDD |
| ul-Bandwidth | Not present |  | TDD |
| } |  |  |  |
| additionalSpectrumEmission | 1 |  |  |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |
| Band > 64 | If band > 64 is selected |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

Table 8.4.2.4.3.3-8: SecurityConfigHO (Table 8.4.2.4.3.3-5)

| Derivation Path: 36.508, Table 4.6.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SecurityConfigHO ::= SEQUENCE { |  |  |  |
| handoverType CHOICE { |  |  |  |
| interRAT SEQUENCE { |  |  |  |
| securityAlgorithmConfig SEQUENCE { |  |  |  |
| cipheringAlgorithm | Set according to PIXIT parameter for default ciphering protection algorithm |  |  |
| integrityProtAlgorithm | Set according to PIXIT parameter for default integrity algorithm |  |  |
| } |  |  |  |
| nas-SecurityParamToEUTRA | Octets 1 to 4 set to 11223344. Bits 1 to 3 of octet 5 are set according to PIXIT parameter for default integrity protection algorithm. Bits 5 to 7 of octet 5 are set according to PIXIT parameter for default ciphering algorithm. Bits 1 to 3 of octet 6 are set to the NAS key set identifier of the current security context. Bit 4 of octet 6 is set to 1. | Octets 1 to 4 include the NonceMME value and are arbitrarily selected. Bits 1 to 3 of octet 5 include the Type of integrity protection algorithm Bits 5 to 7 of octet 5 include the Type of ciphering algorithm. Bits 1 to 4 of octet 6 include the NAS key set identifier. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.4.2.4.3.3-9: PHYSICAL CHANNEL RECONFIGURATION (step 5a1, Table 8.4.2.4.3.2-2)

| Derivation Path: 34.108 clause 9.1.1 (PHYSICAL CHANNEL RECONFIGURATION message) |  |
| --- | --- |
| Information Element | Value/remark |
| - CHOICE mode | FDD |
| - DPCH compressed mode info |  |
| - TGPSI | 1 |
| - TGPS Status Flag | deactivate |
| - TGCFN | Not Present |
| - Transmission gap pattern sequence configuration parameters |  |
| - TGMP | FDD Measurement |
| - TGPRC | Infinity |
| - TGSN | 8 |
| - TGL1 | 10 |
| - TGL2 | Not Present |
| - TGD | 270 |
| - TGPL1 | 12 |
| - TGPL2 | Not Present |
| - RPP | mode 0 |
| - ITP | mode 0 |
| - CHOICE UL/DL Mode | UL and DL, UL only, or DL only, depending on UE capability |
| - Downlink compressed mode method | SF/2 or Not present depending on UE capability |
| - Uplink compressed mode method | Higher Layer Scheduling or Not Present depending on UE capability |
| - Downlink frame type | B |
| - DeltaSIR1 | 20 (2.0) |
| - DeltaSIRAfter1 | 10 (1.0) |
| - DeltaSIR2 | Not Present |
| - DeltaSIRAfter2 | Not Present |
| - N identify abort | Not Present |
| - T Reconfirm abort | Not Present |

Table 8.4.2.4.3.3-10: System Information Block type 19 for Cell 5 (preamble, Table 8.4.2.4.3.2-2)

| Derivation Path: 36.508 clause 4.4.4.1, Table 4.4.4.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList SEQUENCE { |  |  |  |
| utra-ServingCell SEQUENCE { |  |  |  |
| Priority | 5 | higher priority than E-UTRA |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of modified section>