3GPP TSG-RAN5 Meeting #107	R5-252059

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5538 | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to test configuration of NS_05 in Clause 6.6.3.3A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek (Chengdu) Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, LTE_CA_Rel13-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The test frequency range of TC 6.6.3.3A.2.2_1 does not align with the definition in A-MPR configuration Table 6.2.4_2.4.1_3. Therefore,it needs to be corrected |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction to test frequency range of test configuration Table 6.6.3.3A.2.4.1-5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test configuration is incorrect |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.3.3A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change1>

Table 6.6.3.3A.2_1.4.1-5: Test Configuration Table (network signalled value NS_05/NS_01)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low range, Mid range for PCC In case of Low range: - For 5MHz channel bandwidth: 1927.2MHz (NUL = 18072) - For 10MHz channel bandwidth: 1934.7MHz (NUL = 18147) - For 20MHz channel bandwidth: 1930MHz (NUL = 18100) Mid range for SCC (Note 3) |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg(Note 2) |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_05) | SCC (NS_01) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 15 | N/A for this test | QPSK | QPSK | 6 | 4 |
| 2 | 25 | 15 |  | 64QAM | 64QAM | 25 | 6 |
| 3 | 25 | 15 |  | 64QAM | QPSK | 25 | 4 |
| 4 | 25 | 25 |  | QPSK | QPSK | 6 | 6 |
| 5 | 25 | 25 |  | 64QAM | 64QAM | 25 | 25 |
| 6 | 25 | 25 |  | 64QAM | QPSK | 25 | 6 |
| 7 | 25 | 50 |  | QPSK | QPSK | 6 | 12 |
| 8 | 25 | 50 |  | 64QAM | 64QAM | 25 | 25 |
| 9 | 25 | 50 |  | 64QAM | QPSK | 25 | 12 |
| 10 | 50 | 50 |  | QPSK | QPSK | 12 | 12 |
| 11 | 50 | 50 |  | 64QAM | 64QAM | 50 | 25 |
| 12 | 50 | 50 |  | 64QAM | QPSK | 50 | 12 |
| 13 | 100 | 50 |  | QPSK | QPSK | 18 | 12 |
| 14 | 100 | 50 |  | 64QAM | 64QAM | 50 | 25 |
| 15 | 100 | 50 |  | 64QAM | QPSK | 50 | 12 |
| 16 | 100 | 75 |  | QPSK | QPSK | 18 | 16 |
| 17 | 100 | 75 |  | 64QAM | 64QAM | 50 | 25 |
| 18 | 100 | 75 |  | 64QAM | QPSK | 50 | 16 |
| 19 | 100 | 100 |  | QPSK | QPSK | 18 | 18 |
| 20 | 100 | 100 |  | 64QAM | 64QAM | 50 | 25 |
| 21 | 100 | 100 |  | 64QAM | QPSK | 50 | 18 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg. Only the combination with the highest NRB_PCC is tested. Note 3: For Low range place test point with RBstart=0. For High range place test point with RBstart=RBend-LCRB+1. |  |  |  |  |  |  |  |

< End of Change1 >