3GPP TSG-RAN5 Meeting #107	R5-252637

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5460 | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NB-IoT TC 22.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, NB_IOT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The data service type in CONTROL PLANE SERVICE REQUEST message shall be Control plane service type according to TS 24.301. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The data service type was replaced by control plane service type. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Error will exist in this test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.1.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

22.1.1.3.2	Test procedure sequence

Table 22.1.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict | Module |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |  |
|  | Module 1 (M1): Attach for Control Plane CIoT EPS optimisation for EPS services with/without SMS-only |  |  |  |  |  |
| 1 | Switch-on the UE. | - | - | - | - | M1 sT07 |
| 2 | Check: Does the UE transmit an RRCConnectionRequest-NB message, establishmentCause=mo-Signalling? | --> | RRC: RRCConnectionRequest-NB | 1 | P | M1 sT07 |
| 3 | SS transmits an RRCConnectionSetup-NB message to establish SRB1 and SRB1bis. | <-- | RRC: RRCConnectionSetup-NB | - | - | M1 sT07 |
| - | EXCEPTION: Steps 4a1 and 4b1 describe behaviour that depends on UE capabilities; the "lower case letter" identifies a step sequence that take place depending on whether the UE is configured to do Attach Without PDN or not. | - | - | - | - | M1 sT01, sT05, sT06, sT07 |
| 4a1 | IF px_DoAttachWithoutPDN THEN Check: Does the UE transmit an RRCConnectionSetupComplete-NB message (attachWithoutPDN-Connectivity=TRUE) containing an ATTACH REQUEST (indicating UE's CIoT preferred and supported network behaviour) and an ESM DUMMY MESSAGE? | --> | RRC: RRCConnectionSetupComplete-NB NAS: ATTACH REQUEST NAS: ESM DUMMY MESSAGE | 1 | P | M1 sT01, sT05, sT07 |
| 4b1 | ELSE Check: Does the UE transmit an RRCConnectionSetupComplete-NB message containing an ATTACH REQUEST (indicating UE's CIoT preferred and supported network behaviour) and a PDN CONNECTIVITY REQUEST (type of PDN "IP" or "non-IP")? | --> | RRC: RRCConnectionSetupComplete-NB NAS: ATTACH REQUEST NAS: PDN CONNECTIVITY REQUEST | 1,3 | P | M1 sT01, sT06, sT07 |
| 5 | The SS transmits an IDENTITY REQUEST message. | <-- | RRC: DLInformationTransfer-NB NAS: IDENTITY REQUEST | - | - | M1 sT02 |
| 6 | Check: Does the UE transmit an IDENTITY RESPONSE message? | --> | RRC: ULInformationTransfer-NB NAS: IDENTITY RESPONSE | 2 | P | M1 sT02 |
| 7 | The SS transmits an AUTHENTICATION REQUEST message to initiate the EPS authentication and AKA procedure. | <-- | RRC: DLInformationTransfer-NB NAS: AUTHENTICATION REQUEST | - | - | M1 sT03 |
| 8 | Check: Does the UE transmit an AUTHENTICATION RESPONSE message and establishes mutual authentication? | --> | RRC: ULInformationTransfer-NB NAS: AUTHENTICATION RESPONSE | 2 | P | M1 sT03 |
| 9 | The SS transmits a NAS SECURITY MODE COMMAND message to activate NAS security. | <-- | RRC: DLInformationTransfer-NB NAS: SECURITY MODE COMMAND | - | - | M1 sT04 |
| 10 | Check: Does the UE transmit a NAS SECURITY MODE COMPLETE message and establishes the initial security configuration? | --> | RRC: ULInformationTransfer-NB NAS: SECURITY MODE COMPLETE | 2 | P | M1 sT04 |
| - | EXCEPTION: Steps 11a1-11a2 describe behaviour that depends on UE configuration; the "lower case letter" identifies a step sequence that take place if the UE has ESM information which needs to be transferred. | - | - | - | - | M1 sT06 |
| 11a1 | IF the UE sets the ESM information transfer flag in the PDN CONNECTIVITY REQUEST message sent in step 4b1 THEN the SS transmits an ESM INFORMATION REQUEST message to initiate exchange of protocol configuration options and/or APN. | <-- | RRC: DLInformationTransfer-NB NAS: ESM INFORMATION REQUEST | - | - | M1 sT06 |
| 11a2 | The UE transmits an ESM INFORMATION RESPONSE message to transfer protocol configuration options and/or APN. | --> | RRC: ULInformationTransfer-NB NAS: ESM INFORMATION RESPONSE | - | - | M1 sT06 |
| - | EXCEPTION: Steps 12a1 and 12b1 describe behaviour that depends on UE capabilities; the "lower case letter" identifies a step sequence that take place depending on whether the UE is configured to do Attach Without PDN or not. | - | - | - | - | M1 sT01 |
| 12a1 | IF px_DoAttachWithoutPDN THEN SS transmits an ATTACH ACCEPT message and an ESM DUMMY MESSAGE. The SS indicates support of CP CIoT only, even if the ATTACH REQUEST message sent by the UE indicated as CIoT preferred and supported network behaviour, support of UP CIoT. | <-- | RRC: DLInformationTransfer-NB NAS: ATTACH ACCEPT NAS: ESM DUMMY MESSAGE | - | - | M1 sT01, sT05 |
| 12b1 | ELSE SS transmits an ATTACH ACCEPT message and an ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message, The SS indicates support of CP CIoT only, even if the ATTACH REQUEST message sent by the UE indicated as CIoT preferred and supported network behaviour, support of UP CIoT. If PDN type "IP" was included in the PDN CONNECTIVITY REQUEST step 4b1 then the network shall include the PDN type and the PDN address information within the PDN address IE in the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message sent to the UE. NOTE: Settings in ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST to check PLMN rate control - PLMN Rate control set to max of 4 messages per 6 minutes; - APN control not provided; - MTU parameters not provided. | <-- | RRC: DLInformationTransfer-NB NAS: ATTACH ACCEPT NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST | - | - | M1 sT01, sT06 |
| - | EXCEPTION: Steps 13a1 and 13b1 describe behaviour that depends on UE capabilities; the "lower case letter" identifies a step sequence that take place depending on whether the UE is configured to do Attach Without PDN or not. | - | - | - | - | M1 sT01 |
| 13a1 | IF px_DoAttachWithoutPDN THEN Check: Does the UE transmit an ATTACH COMPLETE message and an ESM DUMMY MESSAGE? | --> | RRC: ULInformationTransfer-NB NAS: ATTACH COMPLETE NAS: ESM DUMMY MESSAGE | 1 | P | M1 sT01, sT05 |
| - | EXCEPTION: IF not all IP address information was allocated in the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message sent in step 12b1 TNEN In parallel to the events described in step 13b1 the Generic 'Procedure for IP address allocation in the CP CIoT' described in TS 36.508 [18], clause 8.1.5A.1 takes place. | - | - | - | - | M1 sT01, sT06 |
| 13b1 | ELSE Check: Does the UE transmit an ATTACH COMPLETE message and an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message? | --> | RRC: ULInformationTransfer-NB NAS: ATTACH COMPLETE NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT | 1 | P | M1 sT01, sT06 |
| 14 | The SS transmits an RRCConnectionRelease-NB message to release RRC connection. | <-- | RRC: RRCConnectionRelease-NB | - | - | M1 sT09 |
|  | Module 2 (M2): UE and Network transmission of user data via the control plane (non-SMS service) |  |  |  |  |  |
| - | EXCEPTION: Steps15a1 to 15a21 describe behaviour that depends on UE configuration; the "lower case letter" identifies a step sequence that takes place if the UE is configured to utilise non-SMS services as transport mechanism for user data. | - | - | - | - | M2 sT10-sT17 |
| 15a1 | IF px_nonSMSTransport_CP_CIoT THEN Check: Does the 'Test procedure to check UE response to Paging for Control Plane CIoT MT access' as described in TS 36.508 [18], clause 8.1.5A.2 take place? NOTE: Settings in ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST to check PLMN rate control - PLMN Rate control set to max of 4 messages per 6 minutes; - APN control not provided; - MTU parameters not provided. | - | - | 7,1,3 | P | M2 sT09, sT12, sT16, sT17 |
| - | EXCEPTION: Steps15a2a1-15a2a4 describe behaviour that depends on UE configuration; the "lower case letter" identifies a step sequence that takes place if the UE did not establish a PDN connection in step 15a1. | - | - | - | - | M2 sT10-sT17 |
| 15a2a1 | Cause the UE to request PDN connectivity. (see Note 1) | - | - | - | - | M2 sT10-sT17 |
| 15a2a2 | The UE transmits a PDN CONNECTIVITY REQUEST message. | --> | RRC: ULInformationTransfer-NB NAS: PDN CONNECTIVITY REQUEST | - | - | M2 sT10-sT17 |
| 15a2a3 | The SS transmits an ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message. NOTE: Settings in ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST to check PLMN rate control - PLMN Rate control set to max of 10 messages per 6 minutes; - APN control not provided; - MTU parameters not provided. | <-- | RRC: DLInformationTransfer-NB NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST | - | - | M2 sT10-sT17 |
| - | EXCEPTION: IF pc_IP_PDN AND not all IP address information was allocated in the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message sent in step 15a2a3 TNEN In parallel to the events described in step 15a2a4 the Generic 'Procedure for IP address allocation in the CP CIoT' described in TS 36.508 [18], clause 8.1.5A.1 takes place. | - | - | - | - | M2 sT10-sT17 |
| 15a2a4 | The UE transmits an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message. | --> | RRC: ULInformationTransfer-NB NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT | - | - | M2 sT10-sT17 |
| 15a2A | The SS transmits an ACTIVATE TEST MODE message to activate test mode G (loopback of the User data container content of any received downlink ESM DATA TRANSPORT). | <-- | RRC: DLInformationTransfer-NB TC: ACTIVATE TEST MODE | - | - | M2 sT10-sT14 |
| 15a2B | The UE transmits an ACTIVATE TEST MODE COMPLETE message. | --> | RRC: ULInformationTransfer-NB TC: ACTIVATE TEST MODE COMPLETE | - | - | M2 sT10-sT14 |
| 15a3 | The SS transmits a CLOSE UE TEST LOOP message to close the NB-IoT UE test loop mode for user data transfer (12 transmission; 60 sec delay). | <-- | RRC: DLInformationTransfer-NB TC: CLOSE UE TEST LOOP | - | - | M2 sT10-sT17 |
| 15a4 | The UE transmits a CLOSE UE TEST LOOP COMPLETE message to confirm that loopback is activated. | --> | RRC: ULInformationTransfer-NB TC: CLOSE UE TEST LOOP COMPLETE | - | - | M2 sT10-sT17 |
| 15a5 | SS transmits an ESM DATA TRANSPORT message containing downlink user data of 3 octets. | <-- | RRC: DLInformationTransfer-NB ESM DATA TRANSPORT | - | - | M2 sT11 |
| - | EXCEPTION: Step 15a6a1 describes behaviour that depends on UE implementation; the "lower case letter" identifies a step sequence which may take place depending on whether the UE is configured to do Attach Without PDN or not. |  |  | - | - | M2 sT12 |
| 15a6a1 | IF px_DoAttachWithoutPDN THEN The 'Test procedure to check release of PDN connectivity before leaving RRC-CONNECTED for attach without PDN' specified in TS 36.508 [18], clause 8.1.5A.4 takes place. | - | - | 4 | P | M2 sT12 |
| 15a7 | The SS transmits an RRCConnectionRelease-NB message. | <-- | RRC: RRCConnectionRelease-NB | - | - | M2 sT10-sT17 |
| 15a8 | Wait for the time set in the CLOSE UE TEST LOOP to expire. | - | - | - | - | M2 sT10-sT17 |
| 15a9 | SS starts timer 6 min PLMN Rate control | - | - | - | - | M2 sT13 |
| 15a10 | Check: Does the 'Test procedure to check UE initiation of Control Plane CIoT MO user data transfer non-SMS transport' as described in TS 36.508 [18], clause 8.1.5A.3 take place? NOTE: The UE will transmit one ESM DATA TRANSPORT message containing user data matching the message sent in step 15a5. | - | - | 5 | P | M2 sT10, sT12 sT16, sT17 |
| - | EXCEPTION: Step 15a11 is repeated 9 times. Note: The number of messages is set so that together with the message sent in step 15a10 it respects the PLMN data rate set in the latest ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST. | - | - | - | - | M2 sT13 |
| 15a11 | Check: Does the UE send an ESM DATA TRANSPORT message containing the same user data sent by the SS in step 15a5? | --> | RRC: ULInformationTransfer-NB TC: ESM DATA TRANSPORT | 5,6,7 | P | M2 sT13 |
| 15a12 | Wait until 6 min timer Expires | - | - | - | - | M2 sT13 |
| 15a13 | SS starts timer 6 min PLMN Rate control | - | - | - | - | M2 sT13 |
| - | EXCEPTION: Step 15a14 is repeated 2 times. Note: The number of messages is set so that it respects the PLMN data rate set in the latest ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST. | - | - | - | - | M2 sT13 |
| 15a14 | Check: Does the UE send an ESM DATA TRANSPORT message containing the same user data sent by the SS in step15a5? | --> | RRC: ULInformationTransfer-NB TC: ESM DATA TRANSPORT | 5,6,7 | P | M2 sT13 |
| 15a15 | SS stops timer 6 min PLMN Rate control | - | - | - | - | M2 sT13 |
| - | EXCEPTION: Steps 15a16a1 to 15a16a8 describe behaviour that depends on UE capabilities; the "lower case letter" identifies a step sequence that take place depending on whether the UE supports APN control. | - | - | - | - | M2 sT13, sT14 |
| 15a16a1 | IF pc_APN_RateControl THEN The SS sends a MODIFY EPS BEARER CONTEXT REQUEST message to modify APN, MTU and PLMN rate controls. NOTE: Settings in MODIFY EPS BEARER CONTEXT REQUEST to check APN rate control - APN control set for max 1 message per minute; - MTU parameters are not provided; - PLMN Rate control not provided (i.e. the set in step 15a1 or 15a2a3 values still apply). | <-- | RRC: DLInformationTransfer-NB NAS: MODIFY EPS BEARER CONTEXT REQUEST | - | - | M2 sT22, sT14 |
| 15a16a2 | Check: Does the UE transmit a MODIFY EPS BEARER CONTEXT ACCEPT message? | --> | RRC: ULInformationTransfer NAS: MODIFY EPS BEARER CONTEXT ACCEPT | 10 | P | M2 sT22, sT14 |
| 15a16a3 | The SS transmits a CLOSE UE TEST LOOP message to close the NB-IoT UE test loop mode for user data transfer (2 transmission; 0 sec delay). | <-- | RRC: DLInformationTransfer-NB TC: CLOSE UE TEST LOOP | - | - | M2 sT10-sT17 |
| 15a16a4 | The UE transmits a CLOSE UE TEST LOOP COMPLETE message to confirm that loopback is activated. | --> | RRC: ULInformationTransfer-NB TC: CLOSE UE TEST LOOP COMPLETE | - | - | M2 sT10-sT17 |
| 15a16a5 | SS transmits an ESM DATA TRANSPORT message containing downlink user data. | <-- | RRC: DLInformationTransfer-NB ESM DATA TRANSPORT | - | - | M2 sT14 |
| - | EXCEPTION: Steps 15a16a6-15a16a8 are repeated 2 times. Note: The number of repetitions is set so that it respects the APN rate controls set in step 15a16a1: Max of 1 message every minute. | - | - | - | - | M2 sT14 |
| 15a16a6 | SS starts timer 1 min APN Rate control | - | - | - | - | M2 sT14 |
| 15a16a7 | Check: Does the UE send an ESM DATA TRANSPORT message? | --> | RRC: ULInformationTransfer-NB TC: ESM DATA TRANSPORT | 5,6,7 | P | M2 sT14 |
| 15a16a8 | Wait until timer 1 min APN Rate control expires. | - | - | - | - | M2 sT14 |
| 15a17a1-15a18 | Void |  |  |  |  |  |
| 15a19 | The SS transmits DEACTIVATE TEST MODE message to deactivate the test mode G. | <-- | RRC: DLInformationTransfer-NB TC: DEACTIVATE TEST MODE | - | - | M2 sT10-sT14 |
| 15a20 | The UE transmits a DEACTIVATE TEST MODE COMPLETE message. | --> | RRC: ULInformationTransfer-NB TC: DEACTIVATE TEST MODE COMPLETE | - | - | M2 sT10-sT14 |
| 15a21 | The SS transmits an RRCConnectionRelease-NB message. | <-- | RRC: RRCConnectionRelease-NB | - | - | M2 sT10-sT14 |
|  | Module 3 (M3): UE and Network transmission of SMS via the control plane |  |  |  |  |  |
| - | EXCEPTION: Steps16a1 to 16a12 describe behaviour that depends on UE configuration; the "lower case letter" identifies a step sequence that takes place if the UE is configured to utilise SMS services as transport mechanism for user data. | - | - | - | - | M3 sT18-sT20 |
| 16a1 | IF pc_NB_S1_only AND px_SMSTransport_CP_CIoT THEN Check: Does the 'Test procedure to check UE response to Paging for Control Plane CIoT MT access' as described in TS 36.508 [18], clause 8.1.5A.2 take place? | - | - | 9,1 | P | M3 sT09, sT19, sT20, sT17 |
| 16a1A | The SS transmits an ACTIVATE TEST MODE message to activate test mode H (loopback the content of any downlink received SMS messages). | <-- | RRC: DLInformationTransfer-NB TC: ACTIVATE TEST MODE | - | - | M3 sT18-sT20 |
| 16a1B | The UE transmits an ACTIVATE TEST MODE COMPLETE message. | --> | RRC: ULInformationTransfer-NB TC: ACTIVATE TEST MODE COMPLETE | - | - | M3 sT18-sT20 |
| 16a2 | The SS transmits a CLOSE UE TEST LOOP message to close the NB-IoT UE test loop mode for SMS transfer (2 transmissions; 60 sec delay). | <-- | RRC: DLInformationTransfer-NB TC: CLOSE UE TEST LOOP | - | - | M3 sT18-sT20 |
| 16a3 | The UE transmits a CLOSE UE TEST LOOP COMPLETE message to confirm that loopback is activated. | --> | RRC: ULInformationTransfer-NB TC: CLOSE UE TEST LOOP COMPLETE | - | - | M3 sT18-sT20 |
| 16a4 | SS transmits a DOWNLINK NAS TRANSPORT message containing downlink user data (SMS).(Note 2) | <-- | RRC: DLInformationTransfer-NB NAS: DOWNLINK NAS TRANSPORT | - | - | M3 sT19 |
| 16a4A | The UE transmits a CP-ACK encapsulated in an UPLINK NAS TRANSPORT message. | --> | NAS: UPLINK NAS TRANSPORT | - | - |  |
| 16a4B | The UE transmits a CP-DATA containing an RP-ACK RPDU encapsulated in an UPLINK NAS TRANSPORT message. | --> | NAS: UPLINK NAS TRANSPORT | - | - |  |
| 16a4C | The SS transmits a CP-ACK encapsulated in a DOWNLINK NAS TRANSPORT message to the UE. | <-- | NAS: DOWNLINK NAS TRANSPORT | - | - |  |
| 16a5 | The SS transmits an RRCConnectionRelease-NB message. | <-- | RRC: RRCConnectionRelease-NB | - | - | M3 sT18-sT20 |
| 16a6 | Wait for the time set in the CLOSE UE TEST LOOP to expire. | - | - | - | - | M3 sT18-sT20 |
| 16a7 | Check: Does the 'Test procedure to check UE initiation of Control Plane CIoT MO user data transfer SMS transport' as described in TS 36.508 [18], clause 8.1.5A.3A take place? NOTE: The UE will transmit one CONTROL PLANE SERVICE REQUEST message, Control plane service type = "mobile originating request", integrity protected and partially ciphered and including SMS in the NAS message container IE, matching the TPDU sent in step 16a4. (Note 3) | - | - | 5,8 | P | M3 sT18, sT20 |
| 16a7A | The SS transmits a CP-ACK encapsulated in a DOWNLINK NAS TRANSPORT message. | <-- | NAS: DOWNLINK NAS TRANSPORT |  |  |  |
| 16a7B | The SS transmits a CP-DATA containing an RP-ACK RPDU encapsulated in a DOWNLINK NAS TRANSPORT message | <-- | NAS: DOWNLINK NAS TRANSPORT |  |  |  |
| 16a7C | The UE transmits a CP-ACK encapsulated in an UPLINK NAS TRANSPORT message. | --> | NAS: UPLINK NAS TRANSPORT |  |  |  |
| 16a8 | Check: Does the UE send an UPLINK NAS TRANSPORT message containing SMS, matching the TP-User-Data sent in step 16a4? (Note 3) | --> | RRC: ULInformationTransfer-NB NAS: UPLINK NAS TRANSPORT | 8 | P | M3 sT18 |
| 16a8A | The SS transmits a CP-ACK encapsulated in a DOWNLINK NAS TRANSPORT message. | <-- | NAS: DOWNLINK NAS TRANSPORT |  |  |  |
| 16a8B | The SS transmits a CP-DATA containing an RP-ACK RPDU encapsulated in a DOWNLINK NAS TRANSPORT message | <-- | NAS: DOWNLINK NAS TRANSPORT |  |  |  |
| 16a8C | The UE transmits a CP-ACK encapsulated in an UPLINK NAS TRANSPORT message. | --> | NAS: UPLINK NAS TRANSPORT |  |  |  |
| 16a9 | IF px_DoAttachWithoutPDN THEN The 'Test procedure to check release of PDN connectivity before leaving RRC-CONNECTED for attach without PDN' specified in TS 36.508 [18], clause 8.1.5A.4 takes place. | - | - | - | - | M3 sT18-sT17 |
| 16a10 | The SS transmits DEACTIVATE TEST MODE message to deactivate the test mode H. | <-- | RRC: DLInformationTransfer-NB TC: DEACTIVATE TEST MODE | - | - | M3 sT18-sT20 |
| 16a11 | The UE transmits an DEACTIVATE TEST MODE COMPLETE message. | --> | RRC: ULInformationTransfer-NB TC: DEACTIVATE TEST MODE COMPLETE | - | - | M3 sT18-sT20 |
| 16a12 | The SS transmits an RRCConnectionRelease-NB message. | <-- | RRCConnectionRelease-NB | - | - | M3 sT18-sT20 |
| 17a1-17a18 | Void | - | - | - | - | - |
| Note 1: The request of connectivity to a PDN may be performed by MMI or AT command. Note 2: TP-User-Data (SMS user data) in a downlink TPDU (SMS-DELIVER) as defined in subclause 5.1 in TS 36.509 [25]. Note 3: TP-User-Data (SMS user data) in an uplink TPDU (SMS-SUBMIT) as defined in subclause 5.1 in TS 36.509 [25]. |  |  |  |  |  |  |

[End of change]