3GPP TSG RAN Meeting #108 Meeting			RP-251181

Prague, Czech Republic, June 9-13, 2025

Agenda Item:	16.1	Study on 6G Scenarios and Requirements

Source:	Airbus, ESA

Title:	Ubiquitous Positioning for IMT-2030 TPRs

Document for:	Discussion

# 1	Introduction

This document discusses the ubiquitous positioning capability as part of International Mobile Telecommunications (IMT) for 2030 and beyond (IMT-2030) technical performance requirements (TPRs), by considering positioning also applicable to the Ubiquitous Connectivity usage scenario.

# 2	Background

The ITU-R recommendation ITU-R M.2160 [1] describes the usage scenarios and capabilities of IMT-2030, including the positioning capability with a potential target of the positioning accuracy around 1-10cm. Based on this recommendation, ITU-R Working Party 5D (WP 5D) is developing a new report on the minimum TPRs, and requested input in RAN#106 LS [2].

The candidate set of TPRs were further discussed in RAN#107 as summarized in [3] with the following outcomes related to the positioning capability:

- Positioning 16 contributions support defining positioning as a quantitative TPR, about 7 contributions support defining both vertical and horizontal accuracy, about 6 contributions support to define positioning accuracy for both indoor and outdoor scenarios Moderator Proposal 1-1: Define positioning accuracy as a quantitative TPR

These RAN#107 contributions (summarized in [3]) discuss the positioning capability mainly based on the work of 3GPP Release 16 TR 38.855 [4] and Release 17 TR 38.857 [5], as well as the positioning service levels specified in TS 22.261 [6]. According to some of these contributions, the positioning capability is applicable to the following usage scenarios: Immersive Communication; Hyper-Reliable and Low Latency Communication; and/or Integrated Sensing and Communication. Nonetheless, the positioning capability can also be used in Ubiquitous Connectivity scenarios. As it is envisaged in [7], non-terrestrial networks (NTN) can extend the coverage of the positioning capability beyond terrestrial networks (TN). Indeed, there are relevant Ubiquitous Connectivity use cases based on the positioning capability, as reported in TR 22.870 [8].

Observation 1: Positioning can also be used in Ubiquitous Connectivity scenarios.

# 3	Ubiquitous Positioning

The integration of TN and NTN is a key enabler of both global connectivity and ubiquitous positioning. Indeed, the ubiquitous positioning capability is crucial to extend the coverage of high-accuracy and resilient positioning, which is driven by regulatory requirements, ultra-precise location services, resilience to GNSS degraded or unavailable conditions, and the expansion of service areas.

Certainly, GNSS is the backbone non-3GPP technology to provide ubiquitous positioning in outdoor environments. However, the degradation or unavailability of GNSS positioning, such as due to jamming, spoofing or obstructions, can lead to major threats and risks on a wide range of sectors, such as transportation, critical infrastructure (e.g. 6G system), and regulatory services (e.g. emergency services). Therefore, the exploitation of 3GPP positioning capabilities in Ubiquitous Connectivity scenarios (i.e., resulting in ubiquitous positioning) is a key feature to enhance the resilience, accuracy and availability of positioning-enabled services.

Proposal 1: Positioning TPR should also be applicable to Ubiquitous Connectivity usage scenarios, in order to enable ubiquitous positioning capabilities.

In [1], the IMT-2030 research target of the positioning capability is based on sub-meter level positioning accuracy. As reported in TR 22.870 [8], positioning use cases for the Ubiquitous Connectivity scenario are considered to target meter-level positioning accuracy in outdoor environments.

Proposal 2: Positioning TPR should also target meter-level accuracy for Ubiquitous Connectivity usage scenarios in outdoor environments.

# 4	Conclusions

Observation 1: Positioning can also be used in Ubiquitous Connectivity scenarios.

Proposal 1: Positioning TPR should also be applicable to Ubiquitous Connectivity usage scenarios, in order to enable ubiquitous positioning capabilities.

Proposal 2: Positioning TPR should also target meter-level accuracy for Ubiquitous Connectivity usage scenarios in outdoor environments.

# 5	References

- Recommendation ITU-R M.2160-0 (11/2023), “Framework and overall objectives of the future development of IMT for 2030 and beyond”.

- RP-243202, “LS on Minimum requirements related to technical performance for IMT-2030 radio interface(s) (ITUR_WP5D_TEMP_167; to: RAN, SA, 5G Americas, ARIB, ATIS, CCSA, DECT-Forum, EBU, ETSI, GSMA, IEEE, ITRI, NGMN Alliance, Nufront, one6G, TIA, TSDSI, TTA, TTC, Wi-Fi Alliance, WiMax Forum, Wireless Innovation Forum, 6G Smart Networks and Services Industry Association (6G-IA); cc: -; contact: AT&T)”, Source: ITU-R WP5D.

- RP-250819, “Moderator's summary for IMT-2030 TPRs”, Source: Moderator (CMCC).

- 3GPP TR 38.855 V16.0.0 (2019-03), “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on NR positioning support (Release 16)”.

- 3GPP TR 38.857 V17.0.0 (2021-03), “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on NR Positioning Enhancements (Release 17)”.

- 3GPP TS 22.261 V20.2.0 (2025-03), “3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Service requirements for the 5G system; Stage 1 (Release 20)”.

- 6GWS-250131, “3GPP 6G RAN Workshop - Airbus views on 6G”, Source: Airbus.

- 3GPP TR 22.870 V0.2.1 (2025-03), Technical Report, “3rd Generation Partnership Project; Technical Specification Group TSG SA; Study on 6G Use Cases and Service Requirements; Stage 1 (Release 20)”.